| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0120 |
| --- | --- |

| Title: | CE2 related: Remove “depth_dc_flag” signalling in DMM cases |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xu Chen, Xiaozhen Zheng, Yongbing Lin, Jianhua Zheng HiSilicon technologies, Shenzhen, P. R. China, 518129 | Tel: Email: | +86-755-28425333 anderson.chen@huawei.com |
| Source: | HiSilicon Technologies |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, SDC DMM and DMM are available in depth intra coding and 4 DMM cases(depth_dc_flag=0 or 1, DMM1 DMM4) are used in DMM process. In this contribution, depth_dc_flag is removed in DMM process. Experimental results reportedly show the proposed simplifications bring no coding efficiency impact.

- Introduction

In DMM of the current 3D-HEVC[1], “depth_dc_flag” need to be signalled in both DMM and SDC DMM cases. In SDC DMM cases, “depth_dc_flag” value depends on the residual value of segments. In DMM cases, “depth_dc_flag” value depends on weather dc offset is exist or not. There are 2 SDC DMM cases and 4 DMM cases in current 3D-HEVC that is illustrated as below:

Figure 1. SDC DMM and DMM cases in depth intra coding

- Proposed methods

The main idea behind the proposed methods is to omit DMM cases of dcOffsetAvailFlag=0 in DMM process. Benefits are shown as follow.

- depth_dc_flag is removed in DMM process.

- Fewer restrictions for SDC DMM checking.

- Fewer DMM cases need to check in encoder side.

Figure 2. Proposed method for DMM in depth intra coding

- Proposed text modification

I.7.3.8.5.2

| … |  |
| --- | --- |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && sdc_flag[ x0 ][ y0 ]) { |  |
| depth_dc_flag[ x0 + k ][ y0 + j ] | ae(v) |
| } |  |
| dcNumSeg = DmmFlag[ x0 + k ][ y0 + j ] ? 2 : 1 |  |
| } else |  |
| dcNumSeg = 1 |  |
| … |  |

- I.8.4.4.2.9 Depth partition value derivation and assignment process

The flag dcOffsetAvailFlag is set equal to ( !sdc_flag[ xTb ][ yTb ] && depth_dc_flag[xTb][yTb] ).

- Experimental Results

The proposed methods were implemented on top of HTM-11.0[2] and were carried out on the common test condition [3]. The results are summarized in Table 1 and Table 2.

Table 1 shows the proposed method provides no coding loss (-0.02% bit-saving for synthesized views).

Table 1. Comparison results between CTC anchor and Method

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 99.5% | 114.0% | 101.7% |
| Kendo | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | 99.3% | 102.1% | 102.5% |
| Newspaper_CC | 0.0% | -0.1% | 0.2% | 0.0% | 0.1% | -0.1% | 99.3% | 88.5% | 101.1% |
| GT_Fly | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 99.3% | 92.4% | 99.5% |
| Poznan_Hall2 | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 99.4% | 98.3% | 103.1% |
| Poznan_Street | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 99.6% | 105.0% | 101.0% |
| Undo_Dancer | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 99.5% | 112.7% | 101.1% |
| Shark | 0.0% | -0.2% | 0.1% | 0.0% | 0.0% | 0.0% | 99.0% | 91.7% | 101.3% |
| 1024x768 | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | 99.4% | 101.5% | 101.8% |
| 1920x1088 | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 99.4% | 100.0% | 101.2% |
| average | 0.00% | -0.04% | 0.01% | 0.00% | -0.01% | -0.02% | 99.4% | 100.6% | 101.4% |

Table 2 shows the results of the proposed method provides no coding loss (0.00% bit-saving for AI).

Table 2. Comparison results between AI anchor and Method

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Shark | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| average | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |

- Conclusions

In this contribution, a method is proposed to simplify and reduce complexity of DMM coding. It proposes to remove depth_dc_flag in DMM process. The experiments results show no coding efficiency impact.

- Patent rights declaration(s)

HiSilicon Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

- References

- Y. Chen, G. Tech, K. Wegner, S. Yea, “Test Model 8 of 3D-HEVC and MV-HEVC,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-H1003, 8th Meeting: Valencia, ES, Mar. 29–Apr. 4, 2014.

- HTM-11.0, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-11.0

- K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-G1100, 7th Meeting: San José, US, Jan. 11–17, 2014.