| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 July – 9 July 2014 | Document: JCT3V-I0125 |
| --- | --- |

| Title: | Bug-fixing of depth-based block partitioning signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongbing Lin1, Jianhua Zheng1, Xin Wang2, Ce Zhu2 1 HiSilicon technologies, Beijing, P. R. China,100095 2 School of Electronic Engineering University of Electronic Science and Technology of China, Chengdu, P. R. China,611731 | Tel: Email: | +86-10-60612372 linyongbing@hisilicon.com zhengjianhua@hisilicon.com +1-822-808-8676 wangxin_chinaman@163.com eczhu@uestc.edu.cn |
| Source: | HiSilicon, UESTC |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, the block partition mode of depth-based block partitioning (DBBP) is selected among 2NxN and Nx2N. In the syntax design [1], an encoder signals a DBBP flag to a decoder in a condition that the coding CU block should not be coded with intra mode. However, this condition will lead to the DBBP flag transmission when the block is coded in inter mode with other kinds of partition modes. Moreover, in the corresponding software HTM-11.0 [2], this condition is missed which additionally lead to the DBBP flag transmission when the block is coded in intra mode. In this proposal, a solution has been proposed to fix this bug. The experiment results reportedly show the rate-saving.

# Introduction

Depth-based block partitioning (DBBP) is applied to a texture coding block. As shown in Table 1, the DBBP flag is signaled depending on a condition that the coding CU block should not be coded with intra mode.

Table 1.  The syntax design for the DBBP flag

| coding_unit( x0, y0, log2CbSize , ctDepth) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = MinCbLog2SizeY ) && !predPartModeFlag ) |  |
| part_mode | ae(v) |
| if( depth_based_blk_part_flag[ nuh_layer_id ] && CuPredMode[ x0 ][ y0 ] != MODE_INTRA ) |  |
| dbbp_flag[ x0 ][ y0 ] | ae(v) |

However, only with this condition, the DBBP flag is still signaled even when the block is coded in inter mode with other kinds of partition modes.

# Proposed method

To solve the problem, the condition mentioned above has been modified with determining whether the partition mode of the coding block is 2NxN or Nx2N. As shown in Table 2, the DBBP flag will be signaled when the block partition mode is 2NxN or Nx2N.

Table 2.  The proposed syntax design for the DBBP flag

| coding_unit( x0, y0, log2CbSize , ctDepth) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = MinCbLog2SizeY ) && !predPartModeFlag ) |  |
| part_mode | ae(v) |
| if( depth_based_blk_part_flag[ nuh_layer_id ] && (PartMode==PART_2NxN¦¦ PartMode==PART_Nx2N) ) |  |
| dbbp_flag[ x0 ][ y0 ] | ae(v) |

- Experimental results

The bug-fixing method for DBBP flag signaling is tested based on HTM-11.0. Table 3 and Table4 illustrates the result of this method in All-Intra configurations and random access (CTC), respectively.

Table 3.  Results of the bug-fixing method (ALL INTRA)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.12% | -0.11% | -0.08% | -0.07% | -0.05% | 100.1% | 97.7% | 99.6% |
| Kendo | 0.00% | -0.11% | -0.11% | -0.07% | -0.07% | -0.05% | 99.9% | 105.7% | 100.8% |
| Newspaper_CC | 0.00% | -0.09% | -0.08% | -0.06% | -0.05% | -0.07% | 99.9% | 99.7% | 100.3% |
| GT_Fly | 0.00% | -0.10% | -0.11% | -0.07% | -0.07% | -0.06% | 100.1% | 102.9% | 100.1% |
| Poznan_Hall2 | 0.00% | -0.19% | -0.14% | -0.11% | -0.10% | -0.09% | 100.2% | 96.4% | 100.7% |
| Poznan_Street | 0.00% | -0.10% | -0.09% | -0.06% | -0.06% | -0.05% | 100.2% | 107.4% | 102.1% |
| Undo_Dancer | 0.00% | -0.07% | -0.06% | -0.05% | -0.05% | -0.06% | 100.0% | 103.2% | 100.7% |
| Shark | 0.00% | -0.09% | -0.06% | -0.05% | -0.05% | -0.03% | 99.7% | 105.1% | 103.3% |
| 1024x768 | 0.00% | -0.10% | -0.10% | -0.07% | -0.06% | -0.06% | 100.0% | 101.0% | 100.2% |
| 1920x1088 | 0.00% | -0.11% | -0.09% | -0.07% | -0.07% | -0.06% | 100.0% | 103.0% | 101.4% |
| average | 0.00% | -0.11% | -0.10% | -0.07% | -0.06% | -0.06% | 100.0% | 102.3% | 100.9% |

Table 4.  Results of the bug-fixing method (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.13% | 0.05% | -0.03% | -0.03% | -0.04% | 100.4% | 94.7% | 98.7% |
| Kendo | 0.00% | -0.26% | -0.03% | -0.06% | -0.06% | -0.08% | 100.6% | 99.1% | 99.0% |
| Newspaper_CC | 0.00% | -0.17% | 0.03% | -0.03% | -0.01% | -0.02% | 100.5% | 106.4% | 97.6% |
| GT_Fly | 0.00% | -0.09% | -0.09% | -0.01% | 0.00% | -0.01% | 100.0% | 110.4% | 102.4% |
| Poznan_Hall2 | 0.00% | 0.18% | -0.07% | 0.02% | 0.03% | 0.10% | 100.6% | 108.9% | 99.8% |
| Poznan_Street | 0.00% | -0.02% | 0.05% | -0.01% | 0.00% | 0.01% | 99.7% | 100.3% | 100.9% |
| Undo_Dancer | 0.00% | -0.06% | -0.09% | -0.01% | 0.00% | -0.01% | 100.5% | 102.2% | 100.1% |
| Shark | 0.00% | -0.18% | -0.03% | -0.03% | -0.03% | -0.01% | 100.0% | 87.7% | 99.7% |
| 1024x768 | 0.00% | -0.19% | 0.02% | -0.04% | -0.04% | -0.05% | 100.5% | 100.1% | 98.4% |
| 1920x1088 | 0.00% | -0.03% | -0.05% | -0.01% | 0.00% | 0.02% | 100.1% | 101.9% | 100.6% |
| average | 0.00% | -0.09% | -0.02% | -0.02% | -0.01% | -0.01% | 100.3% | 101.2% | 99.8% |

- Conclusion

In this document, a bug-fixing method for DBBP signaling is presented and tested.

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, "3D-HEVC Draft Text 4," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-G1001, Apr. 2014

- HTM-11.0, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-11.0

# Patent rights declaration(s)

HiSilicon Technologies and Huawei technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

UESTC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).