| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0126r3 |
| --- | --- |

| Title: | Proposed Draft Text of New AVC Amendment: MFC plus Depth, and Texture and Depth View Packing SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Peng Yin, Taoran Lu, Hariharan Ganapathy, Tao Chen, Walt Husak 432 Lakeside Dr. Sunnyvale CA 94085 USA Masayuki Tanimoto, Noa Yotsuya Building 2F, Yotsuya-dori 1-13, Chikusa-ku, Nagoya, 464-0819 Japan Takanori Senoh, Koki Wakunami, Yasuyuki Ichihashi, Hisayuki Sasaki, Ryutaro Oi, Kenji Yamamoto 4-2-1 Nukui-kitamachi, Koganei, Tokyo 184-8795, Japan Ying Chen 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1 408 330 3251 {pyin,wjh}@dolby.com +81 52 781 1883 tanimoto@nagoya-u.jp +81 52 781 1883 senoh@nict.go.jp +1 858 845 6589 chen@qti.qualcomm.com |
| Source: | Dolby Laboratories Inc., NISRI , NICT and Qualcomm |  |  |

_____________________________

# Abstract

This contribution proposes a draft text of the new amendment to ITU-T Rec. H.264 | ISO/IEC 14496-10, including MFC plus depth, and texture and depth view packing SEI message. It is also suggested several errata items for 3D-AVC may be included in the new amendment.

# Proposed text for MFC plus depth, and texture and depth view packing SEI message

In 7.3.2.1.1, replace the syntax table with:

| seq_parameter_set_data( ) { | C | Descriptor |
| --- | --- | --- |
| profile_idc | 0 | u(8) |
| constraint_set0_flag | 0 | u(1) |
| constraint_set1_flag | 0 | u(1) |
| constraint_set2_flag | 0 | u(1) |
| constraint_set3_flag | 0 | u(1) |
| constraint_set4_flag | 0 | u(1) |
| constraint_set5_flag | 0 | u(1) |
| reserved_zero_2bits /* equal to 0 */ | 0 | u(2) |
| level_idc | 0 | u(8) |
| seq_parameter_set_id | 0 | ue(v) |
| if( profile_idc = = 100 ¦ ¦ profile_idc = = 110 ¦ ¦ profile_idc = = 122 ¦ ¦ profile_idc = = 244 ¦ ¦ profile_idc = = 44 ¦ ¦ profile_idc = = 83 ¦ ¦ profile_idc = = 86 ¦ ¦ profile_idc = = 118 ¦ ¦ profile_idc = = 128 ¦ ¦ profile_idc = = 138 ¦ ¦ profile_idc = = 139 ¦ ¦ profile_idc = = 134 ¦ ¦ profile_idc = = 135) { [Ed. (YP): Specify 135 as MFC+Depth profile] |  |  |
| chroma_format_idc | 0 | ue(v) |
| if( chroma_format_idc = = 3 ) |  |  |
| separate_colour_plane_flag | 0 | u(1) |
| bit_depth_luma_minus8 | 0 | ue(v) |
| bit_depth_chroma_minus8 | 0 | ue(v) |
| qpprime_y_zero_transform_bypass_flag | 0 | u(1) |
| seq_scaling_matrix_present_flag | 0 | u(1) |
| if( seq_scaling_matrix_present_flag ) |  |  |
| for( i = 0; i < ( ( chroma_format_idc != 3 ) ? 8 : 12 ); i++ ) { |  |  |
| seq_scaling_list_present_flag[ i ] | 0 | u(1) |
| if( seq_scaling_list_present_flag[ i ] ) |  |  |
| if( i < 6 ) |  |  |
| scaling_list( ScalingList4x4[ i ], 16, UseDefaultScalingMatrix4x4Flag[ i ]) | 0 |  |
| else |  |  |
| scaling_list( ScalingList8x8[ i − 6 ], 64, UseDefaultScalingMatrix8x8Flag[ i − 6 ] ) | 0 |  |
| } |  |  |
| } |  |  |
| log2_max_frame_num_minus4 | 0 | ue(v) |
| pic_order_cnt_type | 0 | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |  |
| log2_max_pic_order_cnt_lsb_minus4 | 0 | ue(v) |
| else if( pic_order_cnt_type = = 1 ) { |  |  |
| delta_pic_order_always_zero_flag | 0 | u(1) |
| offset_for_non_ref_pic | 0 | se(v) |
| offset_for_top_to_bottom_field | 0 | se(v) |
| num_ref_frames_in_pic_order_cnt_cycle | 0 | ue(v) |
| for( i = 0; i < num_ref_frames_in_pic_order_cnt_cycle; i++ ) |  |  |
| offset_for_ref_frame[ i ] | 0 | se(v) |
| } |  |  |
| max_num_ref_frames | 0 | ue(v) |
| gaps_in_frame_num_value_allowed_flag | 0 | u(1) |
| pic_width_in_mbs_minus1 | 0 | ue(v) |
| pic_height_in_map_units_minus1 | 0 | ue(v) |
| frame_mbs_only_flag | 0 | u(1) |
| if( !frame_mbs_only_flag ) |  |  |
| mb_adaptive_frame_field_flag | 0 | u(1) |
| direct_8x8_inference_flag | 0 | u(1) |
| frame_cropping_flag | 0 | u(1) |
| if( frame_cropping_flag ) { |  |  |
| frame_crop_left_offset | 0 | ue(v) |
| frame_crop_right_offset | 0 | ue(v) |
| frame_crop_top_offset | 0 | ue(v) |
| frame_crop_bottom_offset | 0 | ue(v) |
| } |  |  |
| vui_parameters_present_flag | 0 | u(1) |
| if( vui_parameters_present_flag ) |  |  |
| vui_parameters( ) | 0 |  |
| } |  |  |

In 7.3.2.1.3, replace the syntax table with:

| subset_seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| seq_parameter_set_data( ) | 0 |  |
| if( profile_idc = = 83 ¦ ¦ profile_idc = = 86 ) { |  |  |
| seq_parameter_set_svc_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| svc_vui_parameters_present_flag | 0 | u(1) |
| if( svc_vui_parameters_present_flag = = 1 ) |  |  |
| svc_vui_parameters_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| } else if( profile_idc = = 118 ¦ ¦ profile_idc = = 128 ¦ ¦ profile_idc = = 134 ) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 0 | f(1) |
| seq_parameter_set_mvc_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| mvc_vui_parameters_present_flag | 0 | u(1) |
| if( mvc_vui_parameters_present_flag = = 1 ) |  |  |
| mvc_vui_parameters_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| } else if( profile_idc = = 138 ¦ ¦ profile_idc = = 135) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 0 | f(1) |
| seq_parameter_set_mvcd_extension( ) /* specified in Annex Error: Reference source not found */ |  |  |
| } else if( profile_idc = = 139 ) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 0 | f(1) |
| seq_parameter_set_mvcd_extension( ) /* specified in Annex Error: Reference source not found */ |  |  |
| seq_parameter_set_3davc_extension( ) /* specified in Annex J */ | 0 |  |
| } |  |  |
| additional_extension2_flag | 0 | u(1) |
| if( additional_extension2_flag = = 1 ) |  |  |
| while( more_rbsp_data( ) ) |  |  |
| additional_extension2_data_flag | 0 | u(1) |
| rbsp_trailing_bits( ) | 0 |  |
| } |  |  |

Add a new subclause I.10.1.2 “MFC Depth High profile”

- I.10.1.2	 MFC Depth High profile

Bitstreams conforming to the MFC Depth High profile shall obey the following constraints:

–	The base view bitstream as specified in clause Error: Reference source not found shall obey all constraints of the High profile specified in clause Error: Reference source not found and all active sequence parameter sets shall fulfill one of the following conditions:

–	profile_idc is equal to 77 or constraint_set1_flag is equal to 1,

–	profile_idc is equal to 100.

–	The sub-bitstream of stereoscopic texture bitstream as specified in clause Error: Reference source not found shall obey all constraints of the MFC High profile specified in clause Error: Reference source not found and all active MVC sequence parameter sets shall fulfill one of the following conditions:

–	profile_idc is equal to 134,

–	profile_idc is equal to 100,

–	profile_idc is equal to 77 or constraint_set1_flag is equal to 1.

–	Only I, P, and B slice types may be present.

–	NAL unit streams shall not contain nal_unit_type values in the range of 2 to 4, inclusive.

–	Arbitrary slice order is not allowed.

–	Picture parameter sets shall have num_slice_groups_minus1 equal to 0 only.

–	Picture parameter sets shall have redundant_pic_cnt_present_flag equal to 0 only.

–	When frame_mbs_only_flag is equal to 1 in an active sequence parameter set for a texture view, frame_mbs_only_flag shall be equal to 1 in the active sequence parameter set for the depth view having the same view_id.

–	When frame_mbs_only_flag is equal to 0 in an active sequence parameter set for a depth view, mb_adaptive_frame_field_flag shall be equal to 0.

–	MVCD sequence parameter sets for the depth views shall have chroma_format_idc equal to 0 only.

–	MVCD sequence parameter sets shall have bit_depth_luma_minus8 equal to 0 only.

–	MVCD sequence parameter sets shall have bit_depth_chroma_minus8 equal to 0 only.

–	MVCD sequence parameter sets shall have qpprime_y_zero_transform_bypass_flag equal to 0 only.

–	For each access unit, the value of level_idc for all active view MVCD sequence parameter set RBSPs shall be the same as the value of level_idc for the active MVCD sequence parameter set RBSP.

–	The level constraints specified for the MFC Depth High profile in clause Error: Reference source not found shall be fulfilled.

Conformance of a bitstream to the MFC Depth High profile is indicated by profile_idc being equal to 135.

Decoders conforming to the MFC Depth High profile at a specific level shall be capable of decoding all bitstreams in which both of the following conditions are true:

- All active MVCD sequence parameter sets have one or more of the following conditions fulfilled:

–	profile_idc is equal to 135,

–	profile_idc is equal to 138,

–	profile_idc is equal to 134,

–	profile_idc is equal to 128,

–	profile_idc is equal to 118 and constraint_set5_flag is equal to 1,

–	profile_idc is equal to 100,

–	profile_idc is equal to 77 or constraint_set1_flag is equal to 1.

- All active MVCD sequence parameter sets have one or more of the following conditions fulfilled:

–	level_idc or (level_idc and constraint_set3_flag) represent a level less than or equal to the specific level,

–	level_idc[ i ] or (level_idc[ i ] and constraint_set3_flag) represent a level less than or equal to the specific level.

In Table D.1, add the following after “depth_view_info ( payloadSize )”:

| else if( payloadType = = 56 ) |  |  |
| --- | --- | --- |
| texture_and_depth_view_packing( payloadSize ) /* specified in Annex J */ | 5 |  |

Add a new subclause J.13.1.2 “Textyre and depth view packing SEI message syntax”

      - Texture and depth view packing SEI message syntax

| texture_and_depth_view_packing( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| depth_type | 5 | ue(v) |
| if( depth_type = = 0 ) { |  |  |
| num_residual_texture_views_minus1 | 5 | ue(v) |
| residual_depth_flag | 5 | u(1) |
| view_synthesis_parameter( ) | 5 |  |
| } |  |  |
| } |  |  |

Add a new subclause J.13.1.3 “view_synthesis_parameter syntax”

      - View synthesis parameter syntax

| view_synthesis_parameter( ) { | C | Descriptor |
| --- | --- | --- |
| num_views = num_residual_texture_views_minus1 + 2 |  |  |
| z_flag | 5 | u(1) |
| intrinsic_param_flag | 5 | u(1) |
| rotation_flag | 5 | u(1) |
| translation_flag | 5 | u(1) |
| if( z_flag ) { |  |  |
| for( i=0; i<num_views; i++ ) { |  |  |
| z_near_sign_ flag[ i ] | 5 | u(1) |
| z_near_exponent [ i ] | 5 | u(7) |
| z_near_mantissa_len_minus1[ i ] | 5 | u(5) |
| z_near mantissa[ i ] | 5 | u(v) |
| z_far_sign_flag[ i ] | 5 | u(1) |
| z_far_exponent[ i ] | 5 | u(7) |
| z_far_mantissa_len_minus1[ i ] | 5 | u(5) |
| z_far_mantissa[ i ] |  |  |
| } |  |  |
| } | 5 | f(1) |
| if( intrinsic_param_flag ) { |  |  |
| prec_focal_length | 5 | ue(v) |
| prec_principal_point | 5 | ue(v) |
| } |  |  |
| if( rotation_flag ) |  |  |
| prec_rotation_param | 5 | ue(v) |
| if ( translation_flag ) |  |  |
| prec_translation_param | 5 | ue(v) |
| for( i=0; i<num_views; i++ ) { |  |  |
| if( intrinsic_param_flag ) { |  |  |
| sign_focal_length_x[ i ] | 5 | u(1) |
| exponent_focal_length_x[ i ] | 5 | u(6) |
| mantissa_focal_length_x[ i ] | 5 | u(v) |
| sign_focal_length_y[ i ] | 5 | u(1) |
| exponent_focal_length_y[ i ] | 5 | u(6) |
| mantissa_focal_length_y[ i ] | 5 | u(v) |
| sign_principal_point_x[ i ] | 5 | u(1) |
| exponent_principal_point_x[ i ] | 5 | u(6) |
| mantissa_principal_point_x[ i ] | 5 | u(v) |
| sign_principal_point_y[ i ] | 5 | u(1) |
| exponent_principal_point_y[ i ] | 5 | u(6) |
| mantissa_principal_point_y[ i ] | 5 | u(v) |
| } |  |  |
| if( rotation_flag ) { |  |  |
| for ( j=1;j<=3;j++ ) { /* row */ |  |  |
| for( k=1;k<=3;k++ ) { /* column */ |  |  |
| sign_r[ i ][ j ][ k ] | 5 | u(1) |
| exponent_r[ i ][ j ][ k ] | 5 | u(6) |
| mantissa_r[ i ][ j ][ k ] | 5 | u(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| if ( translation_flag ) { |  |  |
| sign_t_x[ i ] | 5 | u(1) |
| exponent_t_x[ i ] | 5 | u(6) |
| mantissa_t_x[ i ] | 5 | u(v) |
| } |  |  |
| } |  |  |
| } |  |  |

Add a new subclause J.13.2.2 “Texture and depth view packing SEI message semantics”

      - Texture and depth view packing SEI message semantics

The texture and depth view packing SEI message informs the decoder that view components of one output view contain samples of multiple distinct spatially packed constituent residual pictures. The related output view is not suitable to be displayed directly and may be converted to the displayable views by using the view synthesis parameters. This information can be used after the decoder output to appropriately rearrange the samples to produce displayable views that are appropriate for display or to server for other purposes (which are outside the scope of this Recommendation | International Standard).

The texture and depth view packing SEI message, when present, shall be associated with an IDR access unit. The information signaled in the SEI message applies to all the access units in the coded video sequence. This SEI message conveys information associated with the coded video sequence that outputs views that may be further processed to generate displayable views.

In such a coded video sequence, there are at least two output views, wherein the base view is coded as displayable view and a non-base view is a residual view. A residual view is not a displayable view. Instead it contains a sequence of difference pictures which are decoded view components of the residual view. Each difference picture is created by frame packing up to four constituent residual pictures.

Each constituent residual picture represents a picture generated by 2:1 decimation from the residual picture horizontally and vertically, starting from the top-left most sample.

Each constituent residual picture is considered as produced by pictures of two displayable views, wherein one belongs to the view that is coded as the base view and used to predict the other picture in the other view of view A. The residual picture of such a process is considered as the residual picture associated with view A. The residual picture consists of the pixels which are not predictable by the base view, namely occluded pixels and out-of-picture pixels in the base picture. After decimation from the residual picture associated with view A, a constituent residual picture associated with view A is generated.

Coded depth views follow the same picture structure as described above.

depth_type specifies the arrangement how distinct spatially packed constituent residual pictures are packed into view components. The values of depth_type shall be 0.

num_residual_texture_views_minus1 + 1 specifies the number of constituent residual texture pictures packed into each of the non-base texture view components. num_residual_texture_views_minus1 shall be in the range of 0 to 3, inclusive.

residual_depth_flag equal to 1 specifies that residual depth pictures are packed into the depth components of the non-base view, with a packing scheme that is identical to the one used for the residual texture pictures. The depth components of the base view shall always be present, independent of the value of residual_depth_flag.

When this SEI message is present, num_anchor_refs_l0[ i ], num_anchor_refs_l1[ i ], num_non_anchor_refs_l0[ i ] and num_non_anchor_refs_l1[ i ] shall be equal to 0 in the active SPS RBSP for the coded video sequence and avc_3d_extension_flag shall be 0 in all VCL NAL units of the coded video sequence.

NOTE 3 – These constraints disable tools for inter-view and inter-component prediction.

Furthermore, the coded video sequence shall contain two texture views, wherein the non-base texture view is the only residual view containing texture information and is composited by spatially packed residual pictures for texture.

The number of coded depth views in coded video sequence shall be equal to 1 + residual_depth_flag.

The view number (i=0, i<num_views) in view synthesis parameter() indicates the location of the constituent residual pictures in the non-base texture view as shown in Table J-xx. In view synthesis parameter(), i=0 indicates the base texture view.

The horizontally and vertically 2:1 subsampled constituent residual pictures are packed in the view components of the non-base texture view as shown in Table J-xx. The non-base texture view hah width W and height xx, where W and H shall be even numberh. Each constituent residual depth picture of the non-base depth view component is associated with a constituent residual picture of the non-base texture view component in the same relative location.

- Table J-xx – Top-left corner co-ordinates of constituent residual pictures (i=1, i<5) packed in a residual view of width=W and height=H.

| i=1 | (0, 0) |
| --- | --- |
| i=2 | (H/2, 0) |
| i=3 | (0, W/2) |
| i=4 | (H/2, W/2) |

Add a new subclause J.13.2.3 “View synthesis parameter semantics”

      - View synthesis parameter semantics

The syntax structure specifies parameters of view synthesis parameters which are used to synthesize views from the decoded base view and constituent residual pictures.

z_flag equal to 1 indicates the presence of the nearest depth value (z_near) and the farthest depth value (z_far). z_flag equal to 0 indicates the absence of the nearest and the farthest depth values.

intrinsic_param_flag equal to 1 indicates the presence of intrinsic camera parameters. intrinsic_param_flag equal to 0 indicates the absence of intrinsic camera parameters.

rotation_flag equal to 1 indicates the presence of rotation camera parameters. rotation_flag equal to 0 indicates the absence of rotation camera parameters. If rotation_flag is 0, a default rotation camera parameter of unit matrix is used unless provided before.

translation_flag equal to 1 indicates the presence of horizontal translation camera parameters. translation_flag equal to 0 indicates the absence of horizontal translation camera parameters.

z_near_sign_flag [ i ] equal to 0 indicates that the sign of the nearest depth value of the i-th camera is positive. z_near _sign_flag [ i ] equal to 1 indicates that the sign is negative.

z_near_exponent [ i ] specifies the exponent part of the nearest depth value of the i-th camera. The value of z_near_exponent [ i ] shall be in the range of 0 to 27-2, inclusive. The value 27-1 is reserved for future use by ITU T | ISO/IEC. Decoders shall treat the value 27-1 as indicating an unspecified z near[ i ].

z_near_mantissa_len_minus1 [ i ] + 1 specifies the number of bits in the mantissa of the nearest depth value of the i-th camera. The value of z_near_mantissa_len_minus1 [ i ] shall be in the range of 0 to 31, inclusive.

z_near_mantissa [ i ] specifies the mantissa part of the nearest depth value of the i-th camera.

z_far_sign_flag [ i ] equal to 0 indicates that the sign of the farthest depth value of the i-th camera is positive. z_far_sign_flag [ i ] equal to 1 indicates that the sign is negative.

z_far_exponent [ i ] specifies the exponent part of the farthest depth value of the i-th camera. The value of z_far_exponent [ i ] shall be in the range of 0 to 27-2, inclusive. The value 27-1 is reserved for future use by ITU T | ISO/IEC. Decoders shall treat the value 27-1 as indicating an unspecified z far[ i ].

z_far_mantissa_len_minus1 [ i ] + 1 specifies the number of bits in the mantissa of the farthest depth value of the i-th camera. The value of mantissa_len_minus1_z_far[ i ] shall be in the range of 0 to 31, inclusive.

z_far_mantissa [ i ] specifies the mantissa part of the farthest depth value of the i-th camera.

prec_focal_length specifies the exponent of the maximum allowable truncation error for focal_length_x[ i ] and focal_length_y[ i ] as given by 2−prec_focal_length. The value of prec_focal_length shall be in the range of 0 to 31, inclusive.

prec_principal_point specifies the exponent of the maximum allowable truncation error for principal_point_x[ i ] and principal_point_y[ i ] as given by 2−prec_principal_point. The value of prec_principal_point shall be in the range of 0 to 31, inclusive.

prec_rotation_param specifies the exponent of the maximum allowable truncation error for r[ i ][ j ][ k ] as given by 2−prec_rotation_param. The value of prec_rotation_param shall be in the range of 0 to 31, inclusive.

prec_translation_param specifies the exponent of the maximum allowable truncation error for t[ i ][ j ] as given by 2−prec_translation_param. The value of prec_ translation_param shall be in the range of 0 to 31, inclusive.

sign_focal_length_x[ i ] equal to 0 indicates that the sign of the focal length of the i-th camera in the horizontal direction is positive. sign_focal_length_x [ i ] equal to 1 indicates that the sign is negative.

exponent_focal_length_x[ i ] specifies the exponent part of the focal length of the i-th camera in the horizontal direction. The value of exponent_focal_length_x[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified focal length x[ i ].

mantissa_focal_length_x[ i ] specifies the mantissa part of the focal length of the i-th camera in the horizontal direction. The length of the mantissa_focal_length_x[ i ] syntax element is variable and determined as follows:

–	If exponent_focal_length_x[ i ] = = 0, the length is Max( 0, prec_focal_length − 30 ).

–	Otherwise (0 < exponent_focal_length_x[ i ] < 63), the length is Max( 0, exponent_focal_length_x[ i ] + prec_focal_length − 31 ).

sign_focal_length_y[ i ] equal to 0 indicates that the sign of the focal length of the i-th camera in the vertical direction is positive. sign_focal_length_y [ i ] equal to 1 indicates that the sign is negative.

exponent_focal_length_y[ i ] specifies the exponent part of the focal length of the i-th camera in the vertical direction. The value of exponent_focal_length_y[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified focal length y[ i ].

mantissa_focal_length_y[ i ] specifies the mantissa part of the focal length of the i-th camera in the vertical direction.

The length of the mantissa_focal_length_y[ i ] syntax element is variable and determined as follows:

–	If exponent_focal_length_y[ i ] = = 0, the length is Max( 0, prec_focal_length − 30 ).

–	Otherwise (0 < exponent_focal_length_y[ i ] < 63), the length is Max( 0, exponent_focal_length_y[ i ] + prec_focal_length − 31 ).

sign_principal_point_x[ i ] equal to 0 indicates that the sign of the principal point of the i-th camera in the horizontal direction is positive. sign_ principal_point_x [ i ] equal to 1 indicates that the sign is negative.

exponent_principal_point_x[ i ] specifies the exponent part of the principal point of the i-th camera in the horizontal direction. The value of exponent_ principal_point_x[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified principal point x[ i ].

mantissa_principal_point_x[ i ] specifies the mantissa part of the principal point of the i-th camera in the horizontal direction The length of the mantissa_principal_point_x[ i ] syntax element in units of bits is variable and is determined as follows:

–	If exponent_principal_point_x[ i ] = = 0, the length is Max( 0, prec_principal_point − 30 ).

–	Otherwise (0 < exponent_principal_point_x[ i ] < 63), the length is Max( 0, exponent_principal_point_x[ i ] + prec_principal_point − 31 ).

sign_principal_point_y[ i ] equal to 0 indicates that the sign of the principal point of the i-th camera in the vertical direction is positive. sign_ principal_point_y [ i ] equal to 1 indicates that the sign is negative.

exponent_principal_point_y[ i ] specifies the exponent part of the principal point of the i-th camera in the vertical direction. The value of exponent_ principal_point_y[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified principal point y[ i ].

mantissa_principal_point_y[ i ] specifies the mantissa part of the principal point of the i-th camera in the vertical direction The length of the mantissa_principal_point_y[ i ] syntax element in units of bits is variable and is determined as follows:

–	If exponent_principal_point_y[ i ] = = 0, the length is Max( 0, prec_principal_point − 30 ).

–	Otherwise (0 < exponent_principal_point_y[ i ] < 63), the length is Max( 0, exponent_principal_point_y[ i ] + prec_principal_point − 31 ).

sign_r[ i ][ j ][ k ] equal to 0 indicates that the sign of (j, k) component of the rotation matrix for the i-th camera is positive. sign_r [ i ] [ j ][ k ] equal to 1 indicates that the sign is negative.

exponent_r[ i ][ j ][ k ] specifies the exponent part of (j, k) component of the rotation matrix for the i-th camera. The value of exponent_r [ i ][ j ][ k ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified rotation matrix.

mantissa_r[ i ][ j ][ k ] specifies the mantissa part of (j, k) component of the rotation matrix for the i-th camera.

The length of the mantissa_r[ i ][ j ][ k ] syntax element in units of bits is variable and determined as follows:

–	If exponent_r[ i ] = = 0, the length is Max( 0, prec_rotation_param − 30 ).

–	Otherwise (0 < exponent_r[ i ] < 63), the length is Max( 0, exponent_r[ i ] + prec_rotation_param − 31 ).

The rotation matrix R[ i ] for i-th camera is represented as follows:

rE[i][0][0]rE[i][0][1]rE[i][0][2]rE[ i ][ 1 ][ 0 ]rE[i][1][1]rE[i][1][2]rE[ i ][ 2 ][ 0 ]rE[i][2][1]rE[i][2][2] 	(J-XX)

sign_t_x[ i ] equal to 0 indicates that the sign of horizontal component of the translation vector for the i-th camera is positive. sign_trans_x[ i ] equal to 1 indicates that the sign is negative.

exponent_t_x[ i ] specifies the exponent part of horizontal component of the translation vector for the i-th camera. The value of exponent_t_x[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified translation vector.

mantissa_t_x[ i ] specifies the mantissa part of horizontal component of the translation vector for the i-th camera.

The length v of the mantissa_t[ i ][ j ] syntax element in units of bits is variable and is determined as follows:

–	If exponent_t_x[ i ] = = 0, the length v = Max( 0, prec_translation_param − 30 ).

–	Otherwise (0 < exponent_t[ i ] < 63), the length v = Max( 0, exponent_t[ i ] + prec_translation_param − 31 ).

The variable x={focal_length, principal_point, rotation, translation} is computed as follows. Where e=exponent, s=sign, n=mantissa and v=mantissa_len_minus1 + 1:

–	If 0 < e < 63, x = (−1)s * 2e−31 * (1 + n  2v).

–	Otherwise (e is equal to 0), x = (−1)s * 2−(30+v) * n.

NOTE 4 – The above specification is similar to that found in IEC 60559:1989, Binary floating-point arithmetic for microprocessor systems.

      - Reconstruction of displayable texture views (Informative)

A process to reconstruct displayable texture views from the constituent residual pictures in the non-base view is summarized in the following paragraph, where view A is one of the constituent residual views.

- If residual_depth_flag=1, the constituent residual depth picture associated with view A is extracted out of the non-base depth view and 1:2 upsampled horizontally and vertically to a residual depth view A. By using the view synthesis parameters, the base depth view (i=0) is projected to the coordinate positions of residual depth view A. Pixels of residual depth view A which have smaller depth value are overwritten by larger value of projected depth samples. Remaining holes are inpainted.

- If residual_depth_flag=0, the base depth view is projected to residual depth view A, by using the view synthesis parameters and the depth values of the base depth view. In case of duplicate projections, the larger depth value is retained. Remaining holes are inpainted. When num_views is an even number, the base depth view is projected to the coordinate positions of residual depth view A from its center location between view (i=0) and view (i=1).

- The resultant depth views and the base depth view are up-sampled to the same resolution of the base-texture view.

- The constituent residual texture picture associated with view A (i=1, i<num_views) is extracted from the non-base texture view and 1:2 up-sampled horizontally and vertically to a residual picture view A.

- By using the resultant depth view A (as from 3.) and the view synthesis parameters, the base texture view (i=0) is projected to the coordinates of the residual picture A. Remaining holes are inpainted to generate the displayable view.

# Errata items for the latest AVC specification

Several potential errata items (related to 3D-AVC) are documented here and they may be included in the same amendment of ITU-T Rec. H.264 | ISO/IEC 14496-10.

- The following row should be added in Table 7-1.

| 16 | Depth parameter set | 11 | non-VCL | non-VCL | non-VCL |
| --- | --- | --- | --- | --- | --- |

- The title of J.7.3.2.1.5 should be changed to “Sequence parameter set 3D-AVC extension syntax” from "Sequence parameter set MVCD extension syntax".

It is suggested that if other errata items in the current latest specification of ITU-T Rec. H.264 | ISO/IEC 14496-10 will be identified during the 9-th JCT-3V meeting, those items can be added in the new amendment.

# Patent rights declaration(s)

Dolby Laboratories Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NISRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NICT may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).