| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0129 |
| --- | --- |

| Title: | AHG4: Editorial improvement for ARP specification text in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang Hongbin Liu Ying Chen 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +86-10-57760696 hongbinl@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In current 3D-HEVC, the advanced residual prediction (ARP) working draft text is implemented in a way that reuses the sub-clauses of weighted sample prediction process in HEVC version 1 which invokes right shifting operations due to the increased bit-depth in interpolation process. However, the bilinear interpolation process is defined in a way that the internal bit-depth of the prediction signal in interpolation process is kept unchanged during the bilinear filtering process used by ARP process. Therefore, the reuse of the weighted sample prediction process results in wrong prediction signal. It is proposed to fix these unintended bugs in this contribution by modifying the bilinear filtering process.

# Introduction

In HEVC specification, to get a prediction block of one inter-coded prediction unit, two steps are required: 1) possible fractional interpolation with HEVC 8-tap/6-tap filters to get the intermediate prediction signal with increased bit depth, (details could be referred to sub-clause 8.5.3.3.3); 2) weighted sample prediction process wherein shifting the intermediate prediction signal back to original bit-depth precision and clipping operation are invoked to make sure there is no overflow (see sub-clause 8.5.3.3.4).

In current 3D-HEVC specification, when ARP is applied, the defined bilinear filtering process which is similar to H.264/AVC specification results in an intermediate prediction signal in the same bit-depth as the input signal, i.e. 8-bit under CTC. In the current specification as described in sub-clause I.8.5.3.3.7.2, this is done by the following equation:

  - predPartLX[ x ][ y ] = (	refPicLX[ xA ][ yA ] * ( 8 − xFrac ) * ( 8 − yFrac ) + 
	refPicLX[ xB ][ yB ] * ( 8 − yFrac ) * xFrac + 
	refPicLX[ xC ][ yC ] * ( 8 − xFrac ) * yFrac + 
	refPicLX[ xD ][ yD ] * xFrac * yFrac ) >> 6	(Error: Reference source not found-1)

However, followed by the filtering process, the weighted sample prediction process as in sub-clause 8.5.3.3.4 of HEVC specification is invoked and the right shifting operation (e.g., by 6 under CTC) causes the wrong prediction values. Here, we give some examples of the shifting operation in the weighted sample prediction process. The final prediction signal predSamples is defined as follows:

- For uni-predicted blocks in P slices with weighted_pred_flag equal to 0, the following applies:

predSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1, ( predSamplesLX[ x ][ y ] + offset1 )  >>  shift1 )

wherein the predSamplesLX indicates the prediction signal which is the output of bilinear filtering process and shift1 is set equal to (14 – bitDepth) and offset1 is equal to 1  <<  ( shift1 − 1 ) when shift1 is unequal to 0. The final prediction signal is expected to have a right shift of 6 in CTC case to normalize the sum of the magnitudes of filter coefficients.

- For bi-predicted blocks in B slices with weighted_bipred_flag equal to 0, the following applies:

predSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1,
								( predSamplesL0[ x ][ y ] + predSamplesL1[ x ][ y ] + offset2 )  >>  shift2 )	(8-2)

wherein shift2 is set equal to (15 – bitDepth) and offset2 is equal to 1  <<  ( shift2 − 1 ). The final prediction signal is expected to have a right shift of 7 in CTC case.

Note that the current bi-linear interpolation is done in a way that similar to non-seperable filter as in equation I-238, wheras the design intent is to allign with HEVC motion compesnation, which interpolates in the horizontal direction the fractional pixles first and uses them to interpolate the final signal.

- Proposal

To avoid unintended editorial bugs, we provide a different version of the ARP working draft text, such that the intermediate bilinear interpolated residual prediction signal is kept in the same unit (left shifted version) as the motion compensation signal in HEVC version 1, and only at the final stage as in the weighted sample prediction process, the prediction signal is right shifted back and clipped.

## Proposed WD text (1)

The changes to the specification on top of JCT3V-H1001_v2 [1] are described as follow:

- I.8.5.3.3.7.1	Bilinear sample interpolation process

The specifications in subclause 8.5.3.3.3.1 apply with the following modifications:

- All invocations of the process specified in subclause 8.5.3.3.3.2 are replaced with invocations of the process specified in subclause  with chromaFlag equal to 0 as additional input.

- All invocations of the process specified in subclause 8.5.3.3.3.3 are replaced with invocations of the process specified in subclause  with chromaFlag equal to 1 as additional input.

- I.8.5.3.3.7.2	Bilinear luma and chroma sample interpolation process

Inputs to this process are:

- a location in full-sample units ( xInt, yInt ),

- a location offset in fractional-sample units ( xFrac, yFrac),

- a sample reference array refPicLX,

- a flag chromaFlag.

- Output of this process is a predicted sample value predPartLX[ x ][ y ].

- In Figure Error: Reference source not found-1, the positions labelled with A, B, C, and D represent samples at full-sample locations inside the given two-dimensional array refPicLX of samples.

Figure Error: Reference source not found-1 Fractional sample position dependent variables in bi-linear interpolation 
and surrounding integer position samples A, B, C, and D

- The variable picWidthInSamples is set equal to pic_width_in_luma_samples and the variable picHeightInSamples is set equal to pic_height_in_luma_samples.

- If chromaFlag is equal 0, xFrac is set equal to ( xFrac << 1 ) and yFrac is set equal to ( yFrac << 1 ).

- Otherwise (chromaFlag is equal to 1), picWidthInSamples is set equal to ( picWidthInSamples / SubWidthC) and picHeightInSamples is set equal to ( picHeightInSamples / SubHeightC ).

The coordinates of positions A, B, C and D are derived as follows:.

  - xA = Clip3( 0, picWidthInSamples − 1, xInt )		(Error: Reference source not found-3)

  - xB = Clip3( 0, picWidthInSamples − 1, xInt + 1 )		(Error: Reference source not found-4)

  - xC = Clip3( 0, picWidthInSamples − 1, xInt )		(Error: Reference source not found-5)

  - xD = Clip3( 0, picWidthInSamples − 1, xInt + 1 ) 		(Error: Reference source not found-6)

  - yA = Clip3( 0, picHeightInSamples − 1, yInt )		(Error: Reference source not found-7)

  - yB = Clip3( 0, picHeightInSamples − 1, yInt )		(Error: Reference source not found-8)

  - yC = Clip3( 0, picHeightInSamples − 1, yInt + 1 )	 	(Error: Reference source not found-9)

  - yD = Clip3( 0, picHeightInSamples − 1, yInt + 1 )		(Error: Reference source not found-10)

In Figure x- of HEVC specification, the positions labelled with upper-case letters Bi, j within shaded blocks represent luma/chroma samples at full-sample locations inside the given two-dimensional array refPicLX of luma/chroma samples depending on chromaFlag. These samples may be used for generating the predicted luma/chroma sample value predSampleLX[ x, y ]. The location offset in fractional-sample units ( xFracC, yFracC ) specifies which of the generated samples at full-sample and fractional-sample locations is assigned to the predicted sample value predSampleLX[ x, y ]. This assignment is as specified in  with xFracC replaced by xFrac, yFracC replaced by yFrac, and predSampleLXC[ xC, yC ] by predSampleLX[ x, y ]. The output is the value of predSampleLX[ x, y ].

- The locations ( xBi, j, yBi, j ) for each of the corresponding samples Bi, j inside the given array refPicLX are derived as follows:

- xBi, j = Clip3( 0,  picWidthInSamples − 1, xIntC + i )		(x-xxx)

- yBi, j = Clip3( 0,  picHeightInSamples − 1, yIntC + j )		(x-xxx)

- Set variable BitDepth equal to chromaFlag ? BitDepthC : BitDepthY.

- The variables shift1, shift2, and shift3 are derived as follows:

- –	The variable shift1 is set equal to BitDepth − 8, the variable shift2 is set equal to 6, and the variable shift3 is set equal to 14 − BitDepth.

- Given the chroma samples Bi, j at full-sample locations ( xBi, j, yBi, j ), the chroma samples ab0,0 to hh0,0 at fractional sample positions are derived as follows:

- –	The samples labelled ab0,0, ac0,0, ad0,0, ae0,0, af0,0, ag0,0, and ah0,0 are derived by applying a 2-tap filter to the nearest integer position samples as follows:

- [Ed. (CY): the equation numbers below need to be adjusted.]

- ab0,0 = ( 56* B0,0 + 8 * B1,0 )  >>  shift1		(8-11)

- ac0,0 = ( 48 * B0,0 + 16 * B1,0 )  >>  shift1		(8-12)

- ad0,0 = ( 40 * B0,0 + 24 * B1,0 ) >>  shift1		(8-13)

- ae0,0 = ( 32 * B0,0 + 32 * B1,0 ) >>  shift1		(8-14)

- af0,0 = ( 24 * B0,0 + 40 * B1,0 ) >>  shift1		(8-15)

- ag0,0 = ( 16 * B0,0 + 48 * B1,0 )  >>  shift1		(8-16)

- ah0,0 = ( 8 * B0,0 + 56 * B1,0 )  >>  shift1		(8-17)

- –	The samples labelled ba0,0, ca0,0, da0,0, ea0,0, fa0,0, ga0,0, and ha0,0 are derived by applying a 2-tap filter to the nearest integer position samples as follows:

- ba0,0 = ( 56 * B0,0 + 8 * B0,1 )  >>  shift1		(8-18)

- ca0,0 = ( 48 * B0,0 + 16 * B0,1 )  >>  shift1		(8-19)

- da0,0 = ( 40 * B0,0 + 24 * B0,1 )  >>  shift1		(8-20)

- ea0,0 = ( 32 * B0,0 + 32 * B0,1 )  >>  shift1		(8-21)

- fa0,0 = ( 24 * B0,0 + 40 * B0,1 )  >>  shift1		(8-22)

- ga0,0 = ( 16 * B0,0 + 48 * B0,1 )  >>  shift1		(8-23)

- ha0,0 = ( 8 * B0,0 + 56 * B0,1 )  >>  shift1		(8-24)

- –	The samples labelled bX0,0, cX0,0, dX0,0, eX0,0, fX0,0, gX0,0, and hX0,0 for X being replaced by b, c, d, e, f, g, and h, respectively, are derived by applying an 2-tap filter to the intermediate values aX0,i with i = −1..2 in the vertical direction as follows:

- bX0,0 = ( 56 * aX0,0 + 8 * aX0,1 )  >>  shift2		(8-25)

- cX0,0 = ( 48 * aX0,0 + 16 * aX0,1 )  >>  shift2		(8-26)

- dX0,0 = ( 40* aX0,0 + 24 * aX0,1 )  >>  shift2		(8-27)

- eX0,0 = ( 32* aX0,0 + 32 * aX0,1 )  >>  shift2		(8-28)

- fX0,0 = ( 24* aX0,0 + 40 * aX0,1 )  >>  shift2		(8-29)

- gX0,0 = ( 16* aX0,0 + 48 * aX0,1 )  >>  shift2		(8-30)

- hX0,0 = ( 8* aX0,0 + 56 * aX0,1 )  >>  shift2		(8-31)

- Figure x-x – Integer samples (shaded blocks with upper-case letters) and fractional sample positions (un-shaded blocks with lower-case letters) for eighth sample interpolation [Ed. (CY): the figure can be removed in the current 3D-HEVC WD text since the text refers to the HEVC version 1 specification. Here it is kept for clarificaiton purposes.]

The value of predPartLX[ x ][ y ] is derived as specified in the following:

  - predPartLX[ x ][ y ] = (	refPicLX[ xA ][ yA ] * ( 8 − xFrac ) * ( 8 − yFrac ) + 
	refPicLX[ xB ][ yB ] * ( 8 − yFrac ) * xFrac + 
	refPicLX[ xC ][ yC ] * ( 8 − xFrac ) * yFrac + 
	refPicLX[ xD ][ yD ] * xFrac * yFrac ) >> 6	(Error: Reference source not found-32)

NOTE – : when chromaFlag is equal to 0, the two variables (i.e., xFrac and yFrac) are the modified inputs wherein both xFrac%2 and yFrac%2 shall be equal to 0.

## Proposed WD text (2)

It is also noted that if non-separable implementation of bi-linear filter is used, only the following modification of equation I-238 as in I.8.5.3.3.7.1 is needed.

predPartLX[ x ][ y ] = (	refPicLX[ xA ][ yA ] * ( 8 − xFrac ) * ( 8 − yFrac ) + 
	refPicLX[ xB ][ yB ] * ( 8 − yFrac ) * xFrac + 
	refPicLX[ xC ][ yC ] * ( 8 − xFrac ) * yFrac + 
	refPicLX[ xD ][ yD ] * xFrac * yFrac ) >> 6shift0

wherein shift0 is equal to BitDepth-8.

# Experimental results

Proposed fix is integrated in to 3D-HTM 11.0 software [2] based on the specification text as provided in section 2.1 and simulations are performed based on the common test conditions (CTC) [3].

The coding performance in CTC (anchor is HTM 11.0) is provided in Table 1. As reported in Table 1, the fix has negligible influence on coding performance.

The encoding, decoding and rendering estimates of the proposed scheme are calculated on the Linux clusters. And these estimates may not be accurate due to parallel simulations.

Table 1: Coding results for 3-view case

|  | Video 1 | Video 2 | Video PSNR / video bitrate |  | Video PSNR / total bitrate |  | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.03% | 0.01% | -0.02% | 0.00% |  |  | -0.01% | 90.8% | 92.9% | 93.1% |
| Kendo | -0.02% | -0.02% | -0.03% | -0.07% |  |  | 0.04% | 92.8% | 93.2% | 92.7% |
| Newspapercc | -0.01% | 0.02% | -0.01% | -0.01% |  |  | -0.01% | 89.3% | 94.5% | 89.2% |
| GhostTownFly | -0.01% | -0.02% | 0.00% | 0.00% |  |  | -0.03% | 89.7% | 91.9% | 90.1% |
| PoznanHall2 | -0.17% | -0.25% | -0.09% | -0.09% |  |  | -0.01% | 104.2% | 101.3% | 106.1% |
| PoznanStreet | 0.11% | 0.07% | 0.01% | 0.02% |  |  | 0.06% | 101.1% | 98.8% | 100.1% |
| UndoDancer | 0.02% | 0.02% | 0.01% | 0.02% |  |  | 0.04% | 88.7% | 90.2% | 92.2% |
| Shark | -0.04% | 0.02% | -0.01% | -0.01% |  |  | 0.00% | 91.5% | 95.6% | 91.2% |
| 1024x768 | -0.02% | 0.00% | -0.02% | -0.03% |  |  | 0.01% | 91.0% | 93.6% | 91.7% |
| 1920x1088 | -0.02% | -0.03% | -0.02% | -0.01% |  |  | 0.01% | 95.0% | 95.5% | 95.9% |
| average | -0.02% | -0.02% | -0.02% |  |  | -0.02% | 0.01% | 93.5% | 94.8% | 94.3% |

Note that the software is modified to be aligned with the working draft text such that clipping with the final shifting is only done in the weighted sample prediction process, similar as in HEVC version 1.

# References

- Gerhard Tech, Krzysztof Wegner, Ying Chen, Sehoon Yea, “3D-HEVC Draft Text 4,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-H1001_v2, 8th Meeting: Valencia, ES, 29 March – 4 April 2014.

- 3DV-HTM version 11.0: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-11.0/

- K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-G1100, 7th Meeting: San José, US, 11–17 Jan. 2014.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).The authors are not aware of Peking University having any current or pending patent rights relating to the technology described in this contribution.