        - Prediction unit syntax

The specifications in subclause 7.3.8.6 apply.

| prediction_unit( x0, y0, nPbW, nPbH ) { | Descriptor |
| --- | --- |
| if( cu_skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { /* MODE_INTER */ |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| if( !acsEnableflag ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding( x0, y0, 0 ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI ) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } else |  |
| mvd_coding( x0, y0, 1 ) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

          - Prediction unit extension semantics

…

The variable acsEnableFlag is set to 0.

If MergeFlag[ x ][ y ] is equal to 1 and slice_type is equal to B and layerId is greater than 0 and PartMode is equal to 2Nx2N and MaxNumMergeCand is greater than 1, the following applies:

  - Depending on the values of collocated_from_l0_flag, and collocated_ref_idx, the variable colPic, specifying the collocated picture, is derived as follows:

    - If collocated_from_l0_flag is equal to 0, colPic is set equal to RefPicList1[ collocated_ref_idx ].

    - Otherwise (collocated_from_l0_flag is equal to 1), colPic is set equal to RefPicList0[ collocated_ref_idx ].

If slice_type of colPic is equal to B, the following applies:

  - The variable acsCandCol is set equal to the variable of the collocated picture specified by colPic, acsCand[ x ][ y ].

For X in the range of 0 to 1, inclusive, the following applies:

–	The flag availableFlagLXN is set equal to 0.

–	The motion vector mvLXN is set equal to ( 0, 0 ).

–	The reference index refIdxLXN is set equal to −1.

The variables minSize, nSbW and nSbH are derived as:

  - minSize = DepthFlag ? MpiSubPbSize : SubPbSize[ nuh_layer_id ]

  - nSbW = ( nPbW / SubPbSize[ nuh_layer_id ]  <=  1 ) ? nPbW : minSize

  - nSbH = ( nPbH / SubPbSize[ nuh_layer_id ]  <=  1 ) ? nPbH : minSize

Depending on DepthFlag, the following applies:

  - If DepthFlag is equal to 0 and mpi_flag[ layerId ] is equal to 1, the following applies:

    - For X in the range of 0 to 1, inclusive, the derivation process for a temporal inter-view motion vector candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb + ( nPbW / nSbW / 2 ) * nSbW, yPb + ( nPbH / nSbH / 2 ) * nSbH ), the variables nSbW and nSbH, the prediction list indication X, the view order index refViewIdx, and the disparity vector mvDisp as inputs, and the outputs are the flag availableFlagLXN, the motion vector mvLXN and the reference index refIdxLXN.

    - The variable checkDefaultT is set equal to 1.

  - Otherwise, if DepthFlag is equal to 1 and iv_mv_pred_flag[ layerId ] is equal to 1, the following applies:

    - The derivation process for a texture merging candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb + ( nPbW / nSbW / 2 ) * nSbW, yPb + ( nPbH / nSbH / 2 ) * nSbH ), the variables nSbW and nSbH, and the variable mvAccFlag being equal to 1 as inputs, and the outputs are the flags availableFlagT, predFlagLXN, the reference index refIdxLXN, and the motion vector mvLXN (with X being replaced by 0 and 1).

    - The variable checkDefaultIvMC is set equal to 1.

When availableFlagL0N or availableFlagL1N is equal to 1, the following applies:

  - When one of the following conditions is true, MergeIdx[ x ][ y ] is set to 0 and acsEnableFlag is set to 1.

    - DepthFlag is equal to 0 and acsCandCol is equal to IvMC

    - DepthFlag is equal to 1 and acsCandCol is equal to T

          - Derivation process for luma motion vectors for merge mode

- …

- If all of the following conditions are true, acsCand[ xPb ][ yPb ] is set to N.

    - ( N  = =  IvMC ) || ( N  = =  T )

    - PartMode  = =  PART_2Nx2N

          - Derivation process for inter layer predicted sub prediction block motion vector candidates

This process is not invoked when iv_mv_pred_flag[ nuh_layer_id ] is equal to 0.

Inputs to this process are:

- a luma location ( xPb, yPb ) of the top-left sample of the current prediction luma block relative to the top-left luma sample of the current picture,

- two variables nPbW and nPbH specifying the width and the height of the current prediction block,

- a reference view index refViewIdx,

- a disparity vector mvDisp.

Outputs of this process are:

- the flags availableFlagLXN, with X in the range of 0 to 1, inclusive, specifying whether the inter-view or inter-component predicted merging candidate is available,

- the  inter-view or inter-component predicted merging candidate mvLXN, with X in the range of 0 to 1, inclusive,

- the reference index refIdxLXN, with X in the range of 0 to 1, inclusive, specifying a reference picture in the reference picture list RefPicListLX.

For X in the range of 0 to 1, inclusive, the following applies:

–	The flag availableFlagLXN is set equal to 0.

–	The motion vector mvLXN is set equal to ( 0, 0 ).

–	The reference index refIdxLXN is set equal to −1.

The variables minSize, nSbW and nSbH are derived as:

  - minSize = DepthFlag ? MpiSubPbSize : SubPbSize[ nuh_layer_id ]	(Error: Reference source not found-1)

  - nSbW = ( nPbW / SubPbSize[ nuh_layer_id ]  <=  1 ) ? nPbW : minSize	(Error: Reference source not found-2)

  - nSbH = ( nPbH / SubPbSize[ nuh_layer_id ]  <=  1 ) ? nPbH : minSize	(Error: Reference source not found-3)

Depending on DepthFlag, the following applies:

- If DepthFlag is equal to 0 and checkDefaultT is equal to 0, for X in the range of 0 to 1, inclusive, the derivation process for a temporal inter-view motion vector candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb + ( nPbW / nSbW / 2 ) * nSbW, yPb + ( nPbH / nSbH / 2 ) * nSbH ), the variables nSbW and nSbH, the prediction list indication X, the view order index refViewIdx, and the disparity vector mvDisp as inputs, and the outputs are the flag availableFlagLXN, the motion vector mvLXN and the reference index refIdxLXN.

- Otherwise (DepthFlag is equal to 1), if checkDefaultIvMC is equal to 0, the derivation process for a texture merging candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb + ( nPbW / nSbW / 2 ) * nSbW, yPb + ( nPbH / nSbH / 2 ) * nSbH ), the variables nSbW and nSbH, and the variable mvAccFlag being equal to 1 as inputs, and the outputs are the flags availableFlagT, predFlagLXN, the reference index refIdxLXN, and the motion vector mvLXN (with X being replaced by 0 and 1).