| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0131 |
| --- | --- |

| Title: | Adaptive candidate selection |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gun Bang, *Young Su Heo, *Gwang Hoon Park, Gwang Soon Lee, Namho Hur ETRI, 218 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, Korea *Media Lab., Kyung Hee University 1, Seochun, Kiheung, Youngin, Gyonggi, 446-701, Korea | Tel: Email: | +82-42-860-6791 {gbang, gslee, namho} @etri.re.kr +82-31-201-3680 {* medist, *ghpark} @khu.ac.kr |
| Source: | ETRI and Kyung Hee University |  |  |

_____________________________

# Abstract

In this contribution, adaptive selecting method for merging candidate is proposed. By referencing selection of merging candidate on temporal collocated PU which is on the previously coded picture, merging index for temporal inter-view and MPI candidates are no more signaled. The results reportedly shows -0.15% coding gain in total bitrate.

# Introduction

There are many merging candidates on 3D-HEVC, but Inter-view Merging Candidate (IVMC) or Motion Parameter Inheritance (MPI) is selected in most case for texture or depth, respectively. In these two most chosen candidates, the selection is highly correlated between the current and the temporal collocated PUs. For dependent texture (or depth), IVMC (or MPI) is mostly selected for current merging candidate when it was selected on temporal collocated PU. So, it can be exploited to reduce bits for signaling merging index.

# Proposed method

When one of the following conditions are true, the proposed adaptive candidate selection method is applied:

- Current PU is belong to the depth-map and temporal collocated PU used MPI for merging, and MPI is available for the current PU.

- Current PU is belong to the dependent texture and temporal collocated PU used IVMC for merging, and IVMC is available for the current PU.

Fig. 1 describes those two conditions for dependent texture and depth. If the condition 1 is true, MPI is forced to set to be used for the merge mode. If the condition 2 is true, IVMC is forced to set to be used for the merge mode. Otherwise, current process of merge mode is applied.

Fig. 1 Example of two conditions for adaptive candidate selection method

Fig. 2 Decision process of signaling merging index

When signaling merging index, the decision process for the proposed method is applied as shown in Fig. 2. If the merging index is adaptively set to zero, IVMC or MPI is used for merge mode because the priority of them is first in texture or depth, respectively.

Note that the proposed method utilizes the information that which candidate is selected before. Therefore, the symbol index of merging candidate of each PU is saved in order to be utilized for the temporal PU on future frame.

# Experimental results

The proposed method is implemented on the HTM 11.0, and only applied to 2Nx2N PUs. The simulation is performed without DBBP because the software is crashed when applying DBBP, but it might be only implementation issue. The results are shown in Table 1.

Table 1 Coding performance of the proposed method (DBBP off)

The experimental results reportedly show that the BD-rate gain is -0.15% in total bitrate, particularly high for depth (-5.2% in best case). In terms of synthesized results, BD-rate loss is 0.23%. Note that time results may not be accurate due to the parallel simulation.

# Conclusion

In this contribution, the adaptive candidate selection method is introduced to reduce bits for signalling merging index. The temporal inter-view and MPI candidate can be adaptively selected without signaling merging index. It is recommended to use the proposed method in the 3D-HEVC

# Patent rights declaration(s)

ETRI and Kyung Hee University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).