| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-I0134 |
| --- | --- |

| Title: | On the Stereo Main profile |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 10 4488000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The following items are proposed to change the specification of the Stereo Main profile:

- It is proposed to allow the use of external base layers with the Stereo Main profile.

- It is proposed to allow indicating scaled reference layer offsets between layers that do not have an inter-layer prediction relationship.

It is also remarked that since the AHG10 output text makes the level specifications independent of the number of layers, it could be considered to generalize the Stereo Main profile to allow more than two views. Such a generalized profile could e.g. suit autostereoscopic multiview displays.

# Proposal

The following items are proposed to change the specification of the Stereo Main profile:

- It is proposed to allow the use of external base layers with the Stereo Main profile.

It is asserted that with the possibility to use an external base layer, the Stereo Main profile could serve a greater variety of applications.

- It is proposed to allow indicating scaled reference layer offsets between layers that do not have an inter-layer prediction relationship.

The scaled reference layer offsets can be used for depth auxiliary layers to indicate their spatial relation to the respective (texture) primary layers. The depth pictures might not cover the same spatial area as the respective texture pictures of the same view for example if there were captured with a depth sensor with a different field of view than that of the color sensor.

The constraint on not allowing the use of scaled reference layer offsets between layers that have inter-layer prediction relation is kept in order to avoid resampling.

There are also minor asserted clarifications in the proposed text, indicated with change marks in the accompanying specification text document.

It is also remarked that since the AHG10 output text makes the level specifications independent of the number of layers, it could be considered to generalize the Stereo Main profile to allow more than two views. Such a generalized profile could e.g. suit autostereoscopic multiview displays.

It is remarked that this contribution has the following dependencies on other contributions:

- The specification text is relative to and the basis of the proposals is the AHG10 output text (included in JCTVC-R0010/JCT3V-I0010). If the AHG10 output text is not adopted, this contribution needs to be revised.

- It is assumed in this contribution that the pseudo-monochrome support proposed in JCTVC-R0062/JCT3V-I0031 is adopted. If not adopted, then this contribution needs to be revised to add the support of the Monochrome 8 profile in the Stereo Main profile.

- It is assumed that all the layers of a bitstream partition are decoded (even if they are not output or required for decoding of output layers). Consequently, the profile for the bitstream partition has to suit the decoding of "unnecessary" layers too. "Unnecessary" layers may be located in their own bitstream partition. For example, it is possible to have an OLS, with one bitstream partition conforming to the Stereo Main profile and another bitstream partition including a 12-bit alpha plane for the base view and hence indicated to conform to the Monochrome 12 profile. See also JCTVC-R0053/JCT3V-I0029 for a more detailed discussion on this topic.

If a decision is made that a bitstream partition can include "unnecessary" layers the decoding of which does not conform to the profile of the bitstream partition, this contribution needs to be revised to exclude such "unnecessary" layers from the requirements of the profile conformance.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).