| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-I0167 |
| --- | --- |

| Title: | Crosscheck of On complexity reduction of bi-prediction for advanced residual prediction (JCT3V-I0096) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +86-10-57760696 hongbinl@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This proposal reports crosscheck results of on complexity reduction of bi-prediction for advanced residual prediction (JCT3V-I0096). Simulation is conducted in CTC and three simulation sets are tested. Performance evaluation results perfectly matches with that provided in JCT3V-I0096.

# Introduction

In JCT3V-I0096 [1], two aspects are proposed to support conversion from conversion from bi-prediction to uni-prediction in ARP in a non-normative manner (bi-prediction can be performed as uni-prediction if two motion vectors and their associated reference pictures are the same).

- For both uni-prediction and bi-prediction, a clipping operation is applied on the motion compensated signal, the signal used for generating prediction residual, and sum of the motion compensated signal and the prediction residual.

- Use one single ARP reference picture for both prediction directions of current PU. Among the two ARP reference pictures from reference list 0 and reference list 1, the one closer (i.e., smaller POC difference) to the current picture is selected as the single ARP reference picture.

Software and working draft text are both provided.

For aspect 2, the working draft text matches with the software. However, for aspect 1, there is minor mismatch between working draft text and the software. Details are as follows.

- In the working draft text, the involved clippings are highlighted in green as follows:

- I.8.5.3.3.7 Bilinear sample interpolation and residual prediction process

- …

- The variable shiftVal is set equal to ( iv_res_pred_weight_idx − 1 ).

- The modified prediction samples predSamplesLXL[ x ][ y ] with x = 0..( nPbW ) − 1 and y = 0..( nPbH ) − 1 are derived as specified in the following:

- predSamplesLXL[ x ][ y ] =	 ( Clip3( 0, ( 1 << bitDepth ) − 1,predSamplesLXL[ x ][ y ] + 
	( ( rpSamplesLXL[ x ][ y ] − rpRefSamplesLXL[ x ][ y ] )  >>  shiftVal ) ) )	(Error: Reference source not found-1)

- The modified prediction samples predSamplesLXCb[ x ][ y ] with x = 0..( nPbW /2 ) − 1 and y = 0..( nPbH /2 )−1 are derived as specified in the following:

- predSamplesLXCb[ x ][ y ] =	 ( Clip3( 0, ( 1 << bitDepth ) − 1,predSamplesLXCb[ x ][ y ] + 
	( ( rpSamplesLXCb[ x ][ y ] − rpRefSamplesLXCb[ x ][ y ] )  >>  shiftVal ) ) )	(Error: Reference source not found-2)

- The modified prediction samples predSamplesLXCr[ x ][ y ] with x = 0..( nPbW /2 ) − 1 and y = 0..( nPbH /2 ) − 1 are derived as specified in the following:

- predSamplesLXCr[ x ][ y ] =	 ( Clip3( 0, ( 1 << bitDepth ) − 1,predSamplesLXCr[ x ][ y ] + 
	( ( rpSamplesLXCr[ x ][ y ] − rpRefamplesLXCr[ x ][ y ] )  >>  shiftVal ) ) )	(Error: Reference source not found-3)

- In the software, the involved clippings are actually as follows (highlighted in green and yellow). The difference between working draft text and software are highlighted in yellow.

- I.8.5.3.3.7 Bilinear sample interpolation and residual prediction process

- …

- The variable shiftVal is set equal to ( iv_res_pred_weight_idx − 1 ).

- The modified prediction samples predSamplesLXL[ x ][ y ] with x = 0..( nPbW ) − 1 and y = 0..( nPbH ) − 1 are derived as specified in the following:

- predSamplesLXL[ x ][ y ] =	 ( Clip3( 0, ( 1 << bitDepth ) − 1, Clip3( 0, ( 1 << bitDepth ) − 1, 	predSamplesLXL[ x ][ y ] ) + 
	( ( Clip3( 0, ( 1 << bitDepth ) − 1, rpSamplesLXL[ x ][ y ] ) −  	
Clip3( 0, ( 1 << bitDepth ) − 1, rpRefSamplesLXL[ x ][ y ] ) )  >>  shiftVal ) ) )	(Error: Reference source not found-4)

- The modified prediction samples predSamplesLXCb[ x ][ y ] with x = 0..( nPbW /2 ) − 1 and y = 0..( nPbH /2 )−1 are derived as specified in the following:

- predSamplesLXCb[ x ][ y ] =	 ( Clip3( 0, ( 1 << bitDepth ) − 1, Clip3( 0, ( 1 << bitDepth ) − 1, 	predSamplesLXCb[ x ][ y ] ) + 
	( ( Clip3( 0, ( 1 << bitDepth ) − 1, rpSamplesLXCb[ x ][ y ] ) −  	
Clip3( 0, ( 1 << bitDepth ) − 1, rpRefSamplesLXCb[ x ][ y ] ) )  >>  shiftVal ) ) )	(Error: Reference source not found-5)

- The modified prediction samples predSamplesLXCr[ x ][ y ] with x = 0..( nPbW /2 ) − 1 and y = 0..( nPbH /2 ) − 1 are derived as specified in the following:

- predSamplesLXCr[ x ][ y ] =	 ( Clip3( 0, ( 1 << bitDepth ) − 1, Clip3( 0, ( 1 << bitDepth ) − 1, 	predSamplesLXCr[ x ][ y ] ) + 
	( ( Clip3( 0, ( 1 << bitDepth ) − 1, rpSamplesLXCb[ x ][ y ] ) −  	
Clip3( 0, ( 1 << bitDepth ) − 1, rpRefSamplesLXCr[ x ][ y ] ) )  >>  shiftVal ) ) )	(Error: Reference source not found-6)

# Simulation results

Simulation is performed under common test condition [2] and the coding performance compared with HTM-11.0 [3] is shown in Table -3 respectively.

Table 1: Coding performance of unification of clipping operation

|  | Video 1 | Video 2 | Video PSNR / video bitrate |  | Video PSNR / total bitrate |  | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.47% | 0.67% | 0.23% | 0.23% |  |  | 0.14% | 109.1% | 117.6% | 111.4% |
| Kendo | 0.46% | 0.55% | 0.20% | 0.17% |  |  | 0.11% | 91.9% | 96.5% | 94.1% |
| Newspapercc | 0.29% | 0.32% | 0.12% | 0.15% |  |  | 0.10% | 95.9% | 97.3% | 96.7% |
| GhostTownFly | 0.03% | -0.01% | 0.02% | 0.01% |  |  | -0.06% | 84.0% | 85.2% | 85.0% |
| PoznanHall2 | 0.24% | 0.02% | 0.04% | 0.03% |  |  | 0.05% | 104.7% | 101.4% | 105.5% |
| PoznanStreet | 0.37% | 0.18% | 0.10% | 0.10% |  |  | 0.08% | 102.8% | 106.8% | 105.3% |
| UndoDancer | 0.09% | -0.06% | 0.02% | 0.02% |  |  | 0.06% | 86.7% | 82.3% | 84.2% |
| Shark | 0.13% | 0.30% | 0.05% | 0.04% |  |  | 0.04% | 98.9% | 101.1% | 97.5% |
| 1024x768 | 0.40% | 0.51% | 0.18% | 0.19% |  |  | 0.11% | 99.0% | 103.8% | 100.7% |
| 1920x1088 | 0.17% | 0.09% | 0.05% | 0.04% |  |  | 0.03% | 95.4% | 95.4% | 95.5% |
| average | 0.26% | 0.25% | 0.10% |  |  | 0.09% | 0.06% | 96.8% | 98.5% | 97.5% |

Table 2: Coding performance of modified ref. picture selection

|  | Video 1 | Video 2 | Video PSNR / video bitrate |  | Video PSNR / total bitrate |  | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.56% | -0.46% | -0.18% | -0.19% |  |  | -0.19% | 99.6% | 103.1% | 100.8% |
| Kendo | -0.71% | -0.62% | -0.23% | -0.23% |  |  | -0.21% | 102.7% | 103.8% | 103.0% |
| Newspapercc | -0.41% | -0.24% | -0.12% | -0.11% |  |  | -0.08% | 100.9% | 103.3% | 100.4% |
| GhostTownFly | -0.07% | 0.00% | 0.00% | 0.00% |  |  | -0.03% | 83.7% | 83.5% | 82.0% |
| PoznanHall2 | -0.05% | -0.06% | -0.02% | -0.03% |  |  | 0.03% | 103.5% | 103.8% | 104.9% |
| PoznanStreet | -0.04% | -0.04% | -0.01% | -0.01% |  |  | 0.00% | 103.0% | 99.8% | 101.2% |
| UndoDancer | -0.22% | -0.30% | -0.05% | -0.05% |  |  | -0.04% | 88.1% | 93.5% | 92.6% |
| Shark | -0.19% | -0.30% | -0.03% | -0.04% |  |  | -0.02% | 95.6% | 104.9% | 99.1% |
| 1024x768 | -0.56% | -0.44% | -0.18% | -0.18% |  |  | -0.16% | 101.1% | 103.4% | 101.4% |
| 1920x1088 | -0.12% | -0.14% | -0.02% | -0.03% |  |  | -0.01% | 94.8% | 97.1% | 95.9% |
| average | -0.28% | -0.25% | -0.08% |  |  | -0.08% | -0.07% | 97.1% | 99.5% | 98.0% |

Table 3: Coding performance of two modifications

|  | Video 1 | Video 2 | Video PSNR / video bitrate |  | Video PSNR / total bitrate |  | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.08% | 0.09% | 0.03% | 0.05% |  |  | -0.03% | 99.7% | 102.0% | 100.1% |
| Kendo | -0.19% | -0.19% | -0.03% | -0.06% |  |  | -0.15% | 95.3% | 96.5% | 96.4% |
| Newspapercc | 0.09% | 0.07% | 0.04% | 0.07% |  |  | 0.05% | 95.5% | 97.9% | 96.5% |
| GhostTownFly | -0.07% | -0.11% | 0.01% | 0.00% |  |  | -0.04% | 87.2% | 92.1% | 90.3% |
| PoznanHall2 | 0.31% | 0.26% | 0.11% | 0.08% |  |  | 0.11% | 106.6% | 100.4% | 102.0% |
| PoznanStreet | 0.27% | 0.02% | 0.07% | 0.07% |  |  | 0.06% | 103.5% | 107.5% | 107.3% |
| UndoDancer | -0.18% | -0.28% | -0.03% | -0.03% |  |  | 0.05% | 90.7% | 86.7% | 89.7% |
| Shark | -0.07% | 0.01% | 0.03% | 0.02% |  |  | 0.01% | 93.7% | 93.9% | 92.3% |
| 1024x768 | -0.06% | -0.01% | 0.01% | 0.02% |  |  | -0.04% | 96.8% | 98.8% | 97.7% |
| 1920x1088 | 0.05% | -0.02% | 0.04% | 0.03% |  |  | 0.04% | 96.3% | 96.1% | 96.3% |
| average | 0.01% | -0.02% | 0.03% |  |  | 0.03% | 0.01% | 96.5% | 97.1% | 96.8% |

# Conclusion

Crosscheck results perfectly match with that provided in JCT3V-I0096.

For the provided unification of clipping in bi-prediction ARP and uni-prediction ARP, there is minor mismatch between the working draft and the software.

# References

- Y.-W. Chen, J.-L. Lin, Y.-W. Huang, S. Lei, “On complexity reduction of bi-prediction for advanced residual prediction,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-I0096, 9th Meeting: Sapporo, JP, 3 – 9 July 2014

- K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-G1100, 7th Meeting: San José, US, 11–17 Jan. 2014.

- HTM-11.0, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-11.0