| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-I0175 |
| --- | --- |

| Title: | 3D-HEVC: Simplification of DMM method |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Houari Sabirin Kei Kawamura Sei Naito 2-1-15 Ohara Fujimino-shi, Saitama Japan | Tel: Email: | +81 49 278 7411 ho-sabirin@kddilabs.jp ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI R&D Laboratories, Inc.) |  |  |

_____________________________

# Abstract

This contribution presents the effect of using only one mode of DMM in depth coding. Since the first proposal time, the modes of DMM have been simplified into two modes: DMM1 and DMM4 by the last meeting. It is reported from the experiments that only one DMM mode is possible to maintain the coding performance instead of two modes.

# Introduction

The Depth Modeling Modes (DMM) had been utilized to appropriately represent the less-varying depth map data in 3D-HEVC. By the Geneva meeting (7th JCT3V meeting), it has been confirmed that only two modes are specified due to the simplification discussion:

- DMM1: Explicit wedgelet signalling;

- DMM4: Inter-component-predicted contour partitioning.

Several contributions in previous meetings have proposed the simplification of DMM by modifying or restricting the DMM4. This contribution reports the possibility of further simplification by completely removing DMM4. Experiments results support the idea.

# Experiments results

The experiments are performed in HTM-11.0. Table 1 and Table 2 show the results of using only DMM1 mode and remove DMM4 mode for both common test condition and all-intra condition, respectively. It is noted that running times under AI condition is not reliable due to the cluster PCs.

Table 1 – Utilizing DMM1 only (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 96.1% | 90.4% | 92.4% |
| Kendo | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 97.8% | 101.4% | 102.7% |
| Newspaper_CC | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.1% | 98.0% | 100.3% | 101.0% |
| GT_Fly | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.3% | 97.5% | 103.7% | 107.5% |
| Poznan_Hall2 | 0.0% | 0.0% | -0.2% | -0.1% | -0.1% | 0.0% | 97.1% | 88.7% | 90.0% |
| Poznan_Street | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 98.5% | 100.7% | 101.0% |
| Undo_Dancer | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 96.9% | 94.3% | 96.7% |
| Shark | 0.0% | -0.1% | 0.1% | 0.0% | 0.2% | 0.3% | 99.6% | 99.6% | 96.4% |
| 1024x768 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 97.3% | 97.4% | 98.7% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 97.9% | 97.4% | 98.3% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 97.7% | 97.4% | 98.4% |

Table 2 – Utilizing DMM1 only (AI)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 92.3% | 110.6% | 122.6% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 93.5% | 109.8% | 125.4% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 91.3% | 105.4% | 114.8% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.5% | 94.0% | 115.0% | 125.3% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 93.5% | 109.4% | 121.2% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 93.0% | 116.9% | 128.1% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% | 92.7% | 109.1% | 118.3% |
| Shark | 0.0% | 0.0% | 0.0% | 0.0% | 0.4% | 0.6% | 96.1% | 108.7% | 116.1% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 92.4% | 108.6% | 121.0% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.3% | 93.9% | 111.8% | 121.8% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% | 93.3% | 110.6% | 121.5% |

Table 3 and Table 4 show the results of using only DMM4 mode and remove DMM1 mode for both common test condition and all-intra condition, respectively. It is noted that running times under AI condition is not reliable due to the cluster PCs.

Table – 3 Utilizing DMM4 only (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.1% | 0.1% | 0.0% | -0.1% | 1.3% | 95.5% | 88.8% | 89.8% |
| Kendo | 0.0% | -0.1% | 0.2% | 0.0% | -0.1% | 1.1% | 97.6% | 104.2% | 104.6% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | -0.5% | 3.3% | 97.5% | 110.0% | 108.8% |
| GT_Fly | 0.0% | 0.6% | 0.6% | 0.1% | -0.1% | 0.7% | 97.0% | 104.2% | 109.6% |
| Poznan_Hall2 | 0.0% | -0.1% | 0.2% | 0.0% | -0.3% | 1.9% | 94.5% | 88.8% | 89.2% |
| Poznan_Street | 0.0% | 0.3% | 0.3% | 0.1% | -0.2% | 1.0% | 96.7% | 100.9% | 99.9% |
| Undo_Dancer | 0.0% | 0.8% | 0.9% | 0.2% | 0.3% | 2.0% | 97.2% | 94.2% | 94.7% |
| Shark | 0.0% | 1.0% | 0.9% | 0.2% | 0.1% | 1.9% | 98.1% | 95.5% | 93.3% |
| 1024x768 | 0.0% | 0.0% | 0.1% | 0.0% | -0.2% | 1.9% | 96.9% | 101.0% | 101.1% |
| 1920x1088 | 0.0% | 0.5% | 0.6% | 0.1% | 0.0% | 1.5% | 96.7% | 96.7% | 97.4% |
| average | 0.0% | 0.3% | 0.4% | 0.1% | -0.1% | 1.7% | 96.8% | 98.3% | 98.8% |

Table 4 – Utilizing DMM4 only (AI)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 3.3% | 79.0% | 106.5% | 116.7% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 3.5% | 79.3% | 101.9% | 112.7% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | -0.4% | 5.7% | 78.6% | 108.6% | 116.1% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | -0.3% | 2.2% | 79.0% | 115.3% | 123.5% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | 4.6% | 78.0% | 108.6% | 119.9% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | 1.6% | 79.2% | 108.2% | 115.0% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 3.6% | 78.5% | 117.4% | 126.8% |
| Shark | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | 3.7% | 80.9% | 108.6% | 111.7% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | 4.2% | 78.9% | 105.7% | 115.2% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | 3.2% | 79.1% | 111.6% | 119.4% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | 3.5% | 79.1% | 109.4% | 117.8% |

Based on the results shown in the above tables, it can be shown that utilizing only DMM1 mode is sufficient without a significant loss, especially for common test condition.

# Proposed text modifications

By removing DMM4 mode, the working draft text is modified as follows.

Table Error: Reference source not found-1 – Specification of DepthIntraMode and associated name

| DepthIntraMode | Associated name |
| --- | --- |
| −1 | INTRA_DEP_NONE |
| 0 | INTRA_DEP_DMM_WFULL |
| 1 | INTRA_DEP_DMM_CPREDTEX |

wedge_full_tab_idx[ x0 ][ y0 ] specifies the index of the wedgelet pattern in the corresponding pattern list when DepthIntraMode[ x0 ][ y0 ] is equal to INTRA_DEP_DMM_WFULL.

Table Error: Reference source not found-2 specifies the value for the intra prediction mode and the associated names.

Table Error: Reference source not found-2 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated name |
| --- | --- |
| 0 | INTRA_PLANAR |
| 1 | INTRA_DC |
| 2..34 | INTRA_ANGULAR2..INTRA_ANGULAR34 |
| 35 | INTRA_DMM_WFULL |
| 36 | INTRA_DMM_CPREDTEX |

IntraPredModeY[ xPb ][ yPb ] labelled 0..34 represents directions of predictions as illustrated in Figure 8 1.

- If DepthIntraMode[ xPb ][ yPb ] is equal to INTRA_DEP_DMM_WFULL, IntraPredModeY[ xPb ][ yPb ] is set equal to INTRA_DMM_WFULL.

- Otherwise if DepthIntraMode[ xPb ][ yPb ] is equal to INTRA_DEP_DMM_CPREDTEX, IntraPredModeY[ xPb ][ yPb ] is set equal to INTRA_DMM_CPREDTEX.

- Otherwise (DepthIntraMode[ xPb ][ yPb ] is equal to INTRA_DEP_NONE), IntraPredModeY[ xPb ][ yPb ] is derived as the following ordered steps:

        - Intra sample prediction

          - General intra sample prediction

- …

- Depending on the value of predModeIntra, the following ordered steps apply:

  - When cIdx is equal to 0 and predModeIntra is in the range of 0 to 34, the filtering process of neighbouring samples specified in subclause 8.4.4.2.3 is invoked with the sample array p and the transform block size nTbS as inputs, and the output is reassigned to the sample array p.

  - The intra sample prediction process according to predModeIntra applies as follows:

    - If predModeIntra is equal to INTRA_PLANAR, the corresponding intra prediction mode specified in subclause 8.4.4.2.4 is invoked with the sample array p and the transform block size nTbS as inputs, and the output is the predicted sample array predSamples.

    - Otherwise, if predModeIntra is equal to INTRA_DC, the corresponding intra prediction mode specified in subclause 8.4.4.2.5 is invoked with the sample array p, the transform block size nTbS, and the colour component index cIdx as inputs, and the output is the predicted sample array predSamples.

    - Otherwise, if predModeIntra is in the range of INTRA_ANGULAR2..INTRA_ANGULAR34, the corresponding intra prediction mode specified in subclause 8.4.4.2.6 is invoked with the intra prediction mode predModeIntra, the sample array p, the transform block size nTbS, and the colour component index cIdx as inputs, and the output is the predicted sample array predSamples.

    - Otherwise, if predModeIntra is equal to INTRA_DMM_WFULL, the corresponding intra prediction mode specified in subclause I.1.1.1.1.2 is invoked with the location ( xTbY, yTbY ), the sample array p and the transform block size nTbS as inputs ,and the outputs are the predicted sample array predSamples.

    - Otherwise, if predModeIntra is equal to INTRA_DMM_CPREDTEX, the corresponding intra prediction mode specified in subclause I.1.1.1.1.3 is invoked with the location ( xTbY, yTbY ), with the sample array p and the transform block size nTbS as inputs, and the outputs are the predicted sample array predSamples.

          - Specification of intra prediction mode INTRA_DMM_WFULL

Inputs to this process are:

- a sample location ( xTb, yTb ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

- the neighbouring samples p[ x ][ y ], with x = −1, y = − 1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

- a variable nTbS specifying the transform block size.

Output of this process is:

- the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1, are derived as specified by the following ordered steps:

  - The variable wedgePattern[ x ][ y ] with x, y =0..nTbS − 1, specifying a binary partition pattern is derived as.

      - wedgePattern = WedgePatternTable[ Log2( nTbS ) ][ wedge_full_tab_idx[ xTb ][ yTb ] ]	(Error: Reference source not found-1)

  - The depth partition value derivation and assignment process as specified in subclause Error: Reference source not found is invoked with the neighbouring samples p[ x ][ y ], the binary pattern wedgePattern[ xTb ][ yTb ], the sample location ( xTb, yTb ), and the transform size nTbS as inputs, and the output is assigned to predSamples[ x ][ y ].

          - Specification of intra prediction mode INTRA_DMM_CPREDTEX

Inputs to this process are:

- a sample location ( xTb, yTb ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

- the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

- a variable nTbS specifying the transform block size.

Output of this process is:

- the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1, are derived as specified by the following ordered steps:

  - The derivation process for a texture predicted contour pattern as specified in subclause Error: Reference source not found is invoked with the sample location ( xTb, yTb ), and the variable nTbS as inputs, and the output is the binary partition pattern wedgePattern[ x ][ y ], with x, y =0..nTbS − 1.

  - The depth partition value derivation and assignment process as specified in subclause Error: Reference source not found is invoked with the neighbouring samples p[ x ][ y ], the binary pattern wedgeletPattern[ x ][ y ], the sample location ( xTb, yTb ) and the transform size nT as inputs, and the output is assigned to predSamples[ x ][ y ].

        - Segmental depth intra coding process

…

Depending on predModeIntra the array wedgePattern[ x ][ y ] with x, y =0..nTbS − 1 specifying the binary segmentation pattern is derived as follows:

- If predModeIntra is equal to INTRA_DMM_WFULL, the following applies:

    - wedgePattern = WedgePatternTable[ Log2( nTbS ) ][ wedge_full_tab_idx[ xTb ][ yTb ] ]

- Otherwise, if predModeIntra is equal to INTRA_DMM_CPREDTEX, the derivation process for a texture predicted contour pattern as specified in subclause Error: Reference source not found is invoked with the sample location ( xTb, yTb ), and the variable nTbS as inputs, and the output is the binary partition pattern wedgePattern[ x ][ y ], with x, y =0..nTbS − 1.

- Otherwise (predModeIntra is not equal to INTRA_DMM_WFULL or INTRA_DMM_CPREDTEX), the following applies:

  - For x, y = 0..nTbS − 1 wedgePattern[ x ][ y ] is set equal to 0.

Depending on DltFlag[ nuh_layer_id ] the reconstructed depth value samples resSamples[ x ][ y ] are derived as specified in the following:

- If DltFlag[ nuh_layer_id ] is equal to 0, the following applies:

  - For x, y = 0..nTbS − 1, the reconstructed depth value samples resSamples[ x ][ y ] are derived as specified in the following:

      - SL[ xTb0 + x ][ yTb0 + y ] =  predSamples[ x ][ y ] + DcOffset[ xTb ][ yTb ][ wedgePattern[ x ][ y ] ]	(Error: Reference source not found-2)

- Otherwise (DltFlag[ nuh_layer_id ] is equal to 1), the following applies:

  - The variables dcPred[ 0 ] and dcPred[ 1 ] are derived as specified in the following:

    - If predModeIntra is not equal to INTRA_DMM_WFULL or INTRA_DMM_CPREDTEX, the following applies:

        - dcPred[ 0 ] = ( predSamples[ 0 ][ 0 ] + predSamples[ 0 ][ nTbS − 1 ] + predSamples[ nTbS − 1 ][ 0 ]
		+ predSamples[ nTbS − 1 ][ nTbS − 1 ] + 2 )  >>  2	(Error: Reference source not found-3)

    - Otherwise, (predModeIntra is equal to INTRA_DMM_WFULL or INTRA_DMM_CPREDTEX), dcPred is set equal to DcVal.

# References

- Karsten Müller, Anthony Vetro, “Common Test Conditions of 3DV Core Experiments,” Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G1100, Jan. 2014.

- Gerhard Tech, Krzysztof Wegner, Ying Chen, Sehoon Yea, “3D-HEVC Draft Text 4,” Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-H1101-v2, April 2014.

# Patent rights declaration(s)

KDDI Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).