| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-I0176 |
| --- | --- |

| Title: | 3D-HEVC: Quantization of Residual Depth Data |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Houari Sabirin Kei Kawamura Sei Naito 2-1-15 Ohara Fujimino-shi, Saitama Japan | Tel: Email: | +81 49 278 7411 ho-sabirin@kddilabs.jp ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI R&D Laboratories, Inc.) |  |  |

_____________________________

# Abstract

Residual depth signals by a depth intra prediction have been a target of quantization since the complexity of residual depth signals is not as much as that of texture signals. The depth look-up table is a quantization method with a high complexity. To overcome the complexity issue, in this contribution, the method of quantizing residual depth signals based on an angular prediction modes is introduced. The BD-rate gains under the common test conditions are 0.00%, 0.03%, and 0.01% for texture, video, and synthesis, respectively.

# Introduction

Depth map values for some kind sequences are very sparse, which is a motivation to introduce the depth look-up table (DLT) in the 3D-HEVC. In the past, DLT is applied for not only the segment-wise depth coding (SDC) but also residual depth signals generated by a depth intra prediction on the some specific modes. After discussion in the previous meeting, the DLT for the residual depth signal is removed due to the complexity issue.

However, a dynamic range compaction of residual depth signals, i.e. DLT, is still effective for the depth coding if a complexity is acceptable.

# Proposed method

To overcome the complexity issue of DLT for the depth residual signals, we propose a pre-quantization of residual depth signals based on a following assumption. A predicted signals from farther reference pixels are less correlated to the input signals compared with that from nearby reference pixels. Then the residual signal from father reference pixels still represents large value. A proposed distance is defined as an angle of the angular prediction mode.

In the proposed method, based on the selected prediction mode of the current depth intra coding block, residual signals are mapped by quantization function Q by using the bit-shift operation which is determined as

QpPred pPred>>1,if IntraPredModeY<10 or IntraPredModeY>26pPred>>0,otherwise.

# Experiments results

The proposed method is integrated into HTM 11.0 reference software and the simulation is conducted under the common test conditions [1, 2]. Table 1 and  present the results under the common test conditions and all intra conditions, respectively. The proposed method provides 0.00%, 0.03%, and 0.01% gain for texture, video, and synthesis, respectively.

Table 1 Performance of the proposed method in CTC

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.09% | 0.16% | 0.04% | -0.01% | 0.10% | 92.3% | 110.6% | 122.6% |
| Kendo | 0.00% | -0.09% | 0.02% | -0.01% | -0.09% | -0.05% | 93.5% | 109.8% | 125.4% |
| Newspaper_CC | 0.00% | -0.11% | 0.05% | -0.01% | -0.05% | -0.03% | 91.3% | 105.4% | 114.8% |
| GT_Fly | 0.00% | 0.11% | -0.07% | 0.00% | -0.01% | -0.07% | 94.0% | 115.0% | 125.3% |
| Poznan_Hall2 | 0.00% | -0.05% | -0.14% | -0.03% | -0.02% | 0.01% | 93.5% | 109.4% | 121.2% |
| Poznan_Street | 0.00% | 0.25% | 0.01% | 0.03% | 0.02% | 0.03% | 93.0% | 116.9% | 128.1% |
| Undo_Dancer | 0.00% | 0.02% | -0.05% | 0.01% | 0.01% | -0.04% | 92.7% | 109.1% | 118.3% |
| Shark | 0.00% | 0.01% | 0.05% | -0.01% | -0.07% | -0.04% | 96.1% | 108.7% | 116.1% |
| 1024x768 | 0.00% | -0.03% | 0.08% | 0.01% | -0.05% | 0.01% | 92.4% | 108.6% | 121.0% |
| 1920x1088 | 0.00% | 0.07% | -0.04% | 0.00% | -0.01% | -0.02% | 93.9% | 111.8% | 121.8% |
| average | 0.00% | 0.03% | 0.00% | 0.00% | -0.03% | -0.01% | 93.3% | 110.6% | 121.5% |

Table 2 Performance of the proposed method in AI conditions

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.00% | 0.00% | 0.00% | -0.03% | 0.11% | 96.2% | 102.9% | 103.0% |
| Kendo | 0.00% | 0.00% | 0.00% | 0.00% | -0.03% | 0.06% | 97.8% | 102.9% | 102.0% |
| Newspaper_CC | 0.00% | 0.00% | 0.00% | 0.00% | -0.04% | 0.07% | 93.6% | 101.1% | 101.1% |
| GT_Fly | 0.00% | 0.00% | 0.00% | 0.00% | -0.03% | 0.02% | 96.5% | 101.2% | 98.4% |
| Poznan_Hall2 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 97.8% | 103.2% | 96.7% |
| Poznan_Street | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.03% | 99.0% | 102.6% | 101.7% |
| Undo_Dancer | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.04% | 100.5% | 101.3% | 106.9% |
| Shark | 0.00% | 0.00% | 0.00% | 0.00% | -0.09% | -0.04% | 99.9% | 103.9% | 103.5% |
| 1024x768 | 0.00% | 0.00% | 0.00% | 0.00% | -0.03% | 0.08% | 95.9% | 102.3% | 102.0% |
| 1920x1088 | 0.00% | 0.00% | 0.00% | 0.00% | -0.03% | 0.01% | 98.8% | 102.4% | 101.4% |
| average | 0.00% | 0.00% | 0.00% | 0.00% | -0.03% | 0.04% | 97.7% | 102.4% | 101.6% |

# Conclusion

This contribution proposed the pre-quantization of residual depth signals generated by the depth intra prediction. The proposed method provides 0.00%, 0.03%, and 0.01% gain for texture, video, and synthesis, respectively. We recommend adoption of this method to the future 3D-HEVC specification.

# References

- Karsten Müller, Anthony Vetro, “Common Test Conditions of 3DV Core Experiments,” Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G1100, Jan. 2014.

- Gerhard Tech, Krzysztof Wegner, Ying Chen, Sehoon Yea, “3D-HEVC Draft Text 4,” Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-H1101-v2, April 2014.

# Acknowledgment

This work was supported by “R&D on Ultra-Realistic Communication Technology with Innovative 3D Video Technology”, the Commissioned Research of National Institute of Information and Communications Technology (NICT), Japan.

# Patent rights declaration(s)

KDDI Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).