| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0191 |
| --- | --- |

| Title: | Removal of the 2Nx2N restriction for sub-PU IVMP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jicheng An1, Jian-Liang Lin2, Kai Zhang1 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Jian-Liang Lin +886-3-5670766 ext. 25250 {jicheng.an, jl.lin, kai.zhang}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution raises the issue of the removal of the 2Nx2N restriction for the sub-PU IVMP, which has been discussed in the previous meeting, but there is unfortunately no clearly described in the notes on removing the restriction for PartMode unequal to 2Nx2N.

# Introduction

In JCT3V H meeting on Mar. 2014, there is a discussion on the sub-PU application as shown in the meeting notes:

Topic a) Sub PUs are currently used with

- VSP (always 4x8/8x4) where B pred is explicitly disallowed

- IVMP for texture where a 2Nx2N restriction is implemented to avoid 4x8/8x4 B prediction with sub PU

- MPI for depth hypothetically this could cause B prediction with 4x8/8x4 (which is the issue here)

Generally, it is desirable to restrict identically for the latter two cases.

It is pointed out that it would be sufficient to disable asymmetric partitioning (and still allow the Nx2N and 2NxN case) to avoid the occasional occurrence of B prediction with 8x4/4x8 cases. What is proposed here seems to be more restrictive than necessary.

Another option would be to generally disallow B prediction in case of 4x8 and 8x4 sub PU sizes.

Offline activity (Jicheng An) to provide text for the latter option which seems to be the simplest and least restrictive solution.

And finally, it was decided to disallow B prediction in the case of 4x8 and 8x4 sub PU sizes as shown in the meeting notes:

Follow up discussion: working draft text was provided and reviewed.

Decision: Adopt (disallow B prediction in case of 4x8 and 8x4 sub PU sizes as provided in H0205)

It is clearly that the meeting notes believe the 2Nx2N restriction for IVMP for texture is more restrictive than necessary and the disallowing 8x4/4x8 B prediction method is the simplest and least restrictive solution. Unfortunately, there is no clear description in the notes on removing the restriction for PartMode unequal to 2Nx2N.

# Proposed method

Since currently the HTM software has already implemented this proposed method, we provide the modified WD as follows (make the WD aligned with the software):

- Green part is newly added, strike part is deleted..

- I.8.5.3.2.10 Derivation process for inter-view merge candidates

…

When ic_flag is equal to 0, the temporal inter-view motion vector merging candidate is derived by the following ordered steps.

  - Depending on PartMode, the following applies:

    - If dbbp_flag[xPb][yPb] is equal to 0 PartMode is equal to PART_2Nx2N, the derivation process for inter layer predicted sub prediction block motion vector candidates as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, the view order index RefViewIdx[ xPb ][ yPb ] and the disparity vector MvRefinedDisp[ xPb ][ yPb ] as inputs, and the outputs are, with X being in the range of 0 to 1, inclusive, the flag availableFlagLXIvMC, the motion vector mvLXIvMC and the reference index refIdxLXIvMC.

    - Otherwise (dbbp_flag[xPb][yPb] is equal to 1 PartMode is not equal to PART_2Nx2N), the derivation process for a temporal inter-view motion vector candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, the prediction list indication X, the view order index RefViewIdx[ xPb ][ yPb ], and the disparity vector MvRefinedDisp[ xPb ][ yPb ] as inputs, and the outputs are the flag availableFlagLXIvMC, the motion vector mvLXIvMC and the reference index refIdxLXIvMC.

  - The availability flag availableFlagIvMC, and the prediction utilization flags predFlagL0IvMC and predFlagL1IvMC are derived by

      - availableFlagIvMC = availableFlagL0IvMC | | availableFlagL1IvMC	(Error: Reference source not found-1)

      - predFlagL0IvMC = availableFlagL0IvMC		(Error: Reference source not found-2)

      - predFlagL1IvMC = availableFlagL1IvMC		(Error: Reference source not found-3)

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).