| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-I0192 |
| --- | --- |

| Title: | Draft text for the adopted improvement on the signaling of DBBP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian-Liang Lin1, Yi-Wen Chen1, Tzu-Der Chuang1, Xianguo Zhang2, Yu-Wen Huang1, Shawmin Lei1 1MediaTek Hsinchu, No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 2MediaTek Beijing, Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jl.lin, yiwen.chen, peter.chuang, xianguo.zhang, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the 8th meeting, an improvement on the signaling of DBBP proposed in JCT3V-H0094 was adopted to remove the dependence on 2NxN partition mode in DBBP flag signaling. In this contribution, the detailed draft text for the adopted improvement is provided to be aligned with the reference software HTM-11.0.

# Introduction

In 3D-HEVC [1], a DBBP flag is signaled at CU level to indicate whether current CU applies DBBP prediction or other conventional inter partition modes. Table 1 shows the syntax design for the DBBP flag. Before the adoption of the proposal JCT3V-H0094 [1], the DBBP flag is signaled depending on partition mode equals to 2NxN which is quite unreasonable. To signal the DBBP mode, encoder firstly needs to signal a 2NxN partition mode to a decoder. Then a DBBP flag is transmitted to indicate whether this is a conventional 2NxN inter partition mode or a DBBP mode. If it is DBBP mode, the transmitted 2NxN partition mode is actually meaningless because the real partition mode will be implicitly derived from the corresponding depth block. Therefore the signaling of the partition mode is actually unnecessary when DBBP is enabled.

Table 1.  The syntax design for the DBBP flag

| coding_unit( x0, y0, log2CbSize , ctDepth) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = MinCbLog2SizeY ) && !predPartModeFlag ) |  |
| part_mode | ae(v) |
| if( depth_based_blk_part_flag[ nuh_layer_id ] && PartMode = = PART_2NxN ) |  |
| dbbp_flag[ x0 ][ y0 ] | u(1) |

# Draft text of the proposed methods

In the adopted method proposed in JCT3V-H0094[1], the dependence on 2NxN partition mode in DBBP flag signaling is removed. In this contribution, as shown in Table 3, the text is proposed to be aligned with the reference software HTM-11.0 [2].

Table 2.  Proposed syntax

| coding_unit( x0, y0, log2CbSize , ctDepth) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = MinCbLog2SizeY ) && !predPartModeFlag ) |  |
| part_mode | ae(v) |
| if( depth_based_blk_part_flag[ nuh_layer_id ] && CuPredMode[ x0 ][ y0 ] != MODE_INTRA ) |  |
| dbbp_flag[ x0 ][ y0 ] | ae(v) |

- dbbp_flag[ x0 ][ y0 ] equal to 1 specifies that depth based block partition is used for the current coding unit. dbbp_flag[ x0 ][ y0 ] equal to 0 specifies that depth based block partition is not used for the current coding unit. When not present, the value of dbbp_flag[ x0 ][ y0 ] is inferred to be equal to 0. When dbbp_flag is equal to 1, the PartMode is set to PART_2NxN.

Table Error: Reference source not found-1 – Values of initValue for dbbp_flag ctxIdx

| Initialization variable | ctxIdx of dbbp_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 161 154 | 161 154 | 161 154 |

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- J.-L. Lin, Y.-W. Chen, T.-D. Chuang, X. Zhang, Y.-W. Huang, S. Lei, “Improvement on the signaling of DBBP,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-H0094, Mar., 2014.

- HTM-11, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-11.0