| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-I 0195 |
| --- | --- |

| Title: | BoG report on merge list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Shinya Shimizu | Tel: Email: | +81-46-859-2703 shimizu.shinya@lab.ntt.co.jp |
| Source: | BoG coordinator |  |  |

_____________________________

# Abstract

The BoG met on 5th July 2014 (3:30 to 6 pm) in the room Small Hall. Approximately 20 experts attended the BoG meeting.

The BoG recommends the following:

- ...

# Introduction

The following five different aspects are discussed in the BoG:

- Interaction between HEVC and 3D-HEVC merge candidates list construction processes

JCT3V-I0053, JCT3V-I0087

- Pruning process on 3D-HEVC merge candidate list construction

JCT3V-I0086

- Disparity motion vector (DMV) search for shifted disparity candidates

JCT3V-I0053, JCT3V-I0086

- VSP inheritance/candidate

JCT3V-I0053, JCT3V-I0087, JCT3V-I0098, JCT3V-I0106, JCT3V-I0112, JCT3V-I0130

(Note: the usage of VSP inheritance implies the interaction on the merge candidate list construction)

- Implicit signalling of merge index

JCT3V-I0131

# Discussions

## Interaction in the merge candidate list construction processes

One aspect of interaction is knowledge of HEVC merge candidate list

- In the current design, it is necessary to know the source of the different candidates in the HEVC candidate list, e.g. A1, B1, etc.

- In the proposed method, output of HEVC candidate list can be black box. No knowledge is required.

Another aspect of interaction is parallelization on the pruning process.

- In the current design, all pruning processes can be performed in parallel.

- In the proposed method in I0053/I0087, 3D-HEVC merge candidate list construction process should wait for the derivation of first two HEVC merge candidates. In the worst case, it is necessary to wait for finishing whole HEVC merge candidate list construction process.

It was mentioned that the implementation of 3D-HEVC merge candidate list construction process can be optimized with the current design. It was also mentioned that it is important to optimize the implementation. Since merge candidate list construction is one of the major parts, interaction on the merge list construction could not be a problem.

It was asserted that there are other parts where interaction exists.

Several experts expressed that reusing the existing module in HEVC ver1 as much as possible is desirable.

BoG could not reach consensus on this aspect.

## Pruning process in 3D-HEVC merge list construction

See notes under JCT3V-I0087.

## DMV search for shifted disparity candidates

|  | DV candidate | Shift DV candidates | DMV search for shifted DV candidate |
| --- | --- | --- | --- |
| 0053 | No change | Yes (no change) | Yes (First 2 entries in HEVC list) |
| 0086 option1 | No change | No | n/a |
| 0086 option2 | DoNBDV (only when horizontal component of DoNBDV is equal to zero) | Yes (only when horizontal component of DoNBDV is equal to zero) | No Always DoNBDV+(4,0) |
| missing | No change | Yes (no change) | No Always DV candidate + (4,0) |

Several experts expressed that removing/reducing DMV search would be beneficial.

There is no separate result for just removing DMV search process.

Revisit upon additional result which will be provided by the proponents of I0086.

## VSP inheritance/candidate

|  | VSP inheritance | Default VSP |
| --- | --- | --- |
| Anchor | All spatial neighbours (A1, B1, B0, A0, or B2) | Between IvDC and A0 |
| 0053 | No | Prior to IvDC |
| 0087 op.3 | A1 | Insert at 4th position in the final list |
| 0087 op.4 | No | Insert at 4th position in the final list |
| 0098 | All spatial neighbours (only 8x8+ PU) | Between IvDC and A0 (same as anchor) |
| 0106 | A1 | Between IvDC and A0 (same as anchor) |
| 0112 | A1 | Insert at 4th position in the final list only when there is no inherited VSP |
| 0130 | A1 or B1 | Between IvDC and A0 only when there is no inherited VSP |

It was asked what the most major problem on VSP inheritance is, VSP flag storage or the number of operation on inheritance. If the major problem is storage, reducing sources of VSP inheritance is not help solving the problem.

Two contributions (I0053 and I0087) propose removal of VSP inheritance, but both contributions include other modifications. No separate result was available.

Several experts mentioned that this solution is preferable than the other proposals related to VSP inheritance if there is no coding loss.

Revisit upon additional results (I0053 and/or I0087).

## Implicit signalling of merge index

See notes under JCT3V-I0131

# Reviews of Contributions

JCT3V-I0053 Simplifications for 3D-HEVC merge candidate list generation [H. Liu, Y. Chen (Qualcomm)]

As discussed in previous JCT-3V meetings, in derivation process of 3D-HEVC merge list, there are several aspects that may not be optimal. First, the insertion of HEVC merge candidates into 3D-HEVC merge candidate list depends on spatial position and availability of spatial merge candidates. Second, the VSP (View Synthesis Prediction) candidate may be generated in a way that requiring a status to be stored (a VSP variable/flag) for spatial neighboring so that the merging process takes this variable into consideration. Third, when deriving shifted disparity merging candidate, up to 5 spatial merge candidates may be checked to find the first available DMV (Disparity Motion Vector).

This contribution provides solutions that can be considered by 3D-HEVC. First, when inserting the additional merge candidates, the dependency on spatial positions of the HEVC merge candidates is removed. Second, the VSP flag is not used to generate VSP candidates therefore storage and check of spatial neighboring blocks are not needed. Third, only allow check up to 2 HEVC merge candidates when searching the first available DMV for the first two HEVC merge candidates. It is reported that proposed method has negligible coding loss compared to HTM under the common test conditions and the condition when depth dependent texture coding is not enabled.

The method is claimed to reduce the number of additions from 6 to 3, and the number of comparisons from 42 to 10, concerning computation of shifted inter-view merge candidate. One expert expresses that the proposed solution has the disadvantage of introducing dependencies between original HEVC and new 3D-HEVC merge candidates.

Another expert expressed support, since the coding loss is small.

- VSP inheritance

Remove all VSP flag and put VSP candidate prior to IvDC candidate.

No separate result is available.

It was asserted that VSP candidates in the current list are identical, i.e. redundant.

See further discussion under section 2.4.

JCT3V-I0139 Crosscheck for JCT3V-I0053 Simplifications for 3D-HEVC merge candidate list generation [J. An (Mediatek)] [late]

JCT3V-I0154 Cross check of simplifications for 3D-HEVC merge candidate list generation (JCT3V-I0053) [J. Y. Lee, C. Kim (Samsung)] [late]

JCT3V-I0087 Simplification of 3D Merge list construction [J. Y. Lee, M. W. Park, Y. Cho, C. Kim (Samsung), Y. Zhang, L. Yu (Zhejiang University)]

At Geneva meeting, the simple Merge candidate list construction was adopted (F0093). It introduced a simple and low-complexity way to implement the Merge list construction module for 3D-HEVC by re-using the existing HEVC Merge list construction. As follow-up of H0097, this contribution proposes to remove pruning processes, VSP inheritance, availability checking processes of HEVC candidates such as candidate A0, A1, B0, B1, and B2 in the 3D Merge list construction. As a result, it can minimize the interaction between the HEVC candidates and 3D candidates. The result demonstrates that the proposed method has small coding loss with about 0.1%. In terms of simplification, the availability of the candidate A0, A1, B0, B1, and B2 is not needed to be checked in the Merge list construction, the number of pruning processes is reduced from 8 to 0, and the VSP flag storage is not required. In addition, this contribution proposes several alternative options for simplification.

Some common aspects with I0053. Indicates that higher losses (0.1% bit rate increase) occur when interaction between HEVC/new merge candidates is given up.

(Interaction means that knowledge is required what the source of the different candidates in the HEVC candidate list was, e.g. if they came from A1, B1 etc. Preference is expressed by several experts for such a solution.)

“Option 1” of the proposal also comes with practically no loss, but still requires interaction in pruning for the first list positions.

To sort those aspects out and possibly come to a unified solution, establish BoG (Shimizu), to further investigate aspects of merge list construction from I0053, I0086, I0087, I0098, I0106, I0112, I0130, I0131

Regarding the aspect on the interaction with the HEVC merge list construction, see notes under section 2.1.

This contribution also includes a proposal to remove/reduce pruning process (from 8 to 2/0). Each pruning requires 6+ operations/comparisons.

Partial removal of pruning is not worth considering. Only complete removal is considered.

Experiment results: minor losses

The benefit, i.e. reduction of the number of operations, is rather small although coding loss is also minor (0.1%). If there is no coding loss, this kind of minor simplification would be preferable.

One expert mentioned that the current pruning process is less important than the other problems in 3D-HEVC merge candidate list construction and those problems should be solved first. On the top of the new design, the proposed simplification will be evaluated again.

Several experts support this simplification.

BoG encourages further study on this aspect (removal of pruning process).

JCT3V-I0156 Cross-check of Simplification of 3D Merge list construction (JCT3V-I0087) [T. Ikai (Sharp)] [late]

JCT3V-I0086 Simplification of shift DV candidate [J. Y. Lee, M. W. Park, Y. Cho, C. Kim (Samsung)]

In 3D-HEVC, shift DV is used in merge mode. In order to find the shift DV, all the neighboring blocks are searched whether they have DV or not. After the DV searching process, it is generated by adding four to a horizontal component of the existing DV. However, during several meetings it was reported that the shift DV provides the small coding gain. Therefore, this contribution proposes to remove and simplify the shift DV as the first and second options, respectively. The third option removes a pruning process of DV on top of the second option. Based on WD, both the options reduce 30 condition checks in the worst case by removing a subclause H.8.5.3.2.15 (DV searching process). The experiment shows the negligible coding loss in the first option and no coding loss in the second option.

The benefits of shifted DV candidates is small (0.04% BR saving for synthesized views under CTC and less than 0.1% with VSP and DoNBDV off)

There are three aspects in this contribution:

- Make DV candidate available only when horizontal component of DoNBDV is not equal to zero

- Generate shifted DV candidates by adding 4 to the DV candidate instead of searching neighboring blocks

- Remove the pruning process on DV candidate

No action on the first aspect since this aspect considered as coding efficiency improvement proposal, but no notable gain was observed.

Removing search (the second aspect) is preferable; however no separate result was available.

One expert requests separate results (both CTC and VSP/DoNBDV/DBBP off) and corresponding working draft for the second aspect.

Third aspect will also be discussed after the separate results are provided.

See further discussion under section 2.3.

JCT3V-I0113 Crosscheck on simplification of shift DV candidate (JCT3V-I0086) [Y. Zhang (Zhejiang University)]

JCT3V-I0098 Unification of Merging Candidate List Construction in 3D-HEVC [J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek)

In current 3D-HEVC, three kinds of merge candidate sets are used for the merge mode in independent texture view, non-smallest PU in dependent texture view and depth views, smallest PU in dependent texture view and depth views, respectively. To further simply and unify the merge candidate sets, it is proposed to simply apply the HEVC merge candidates for the smallest PU and remove the VSP inheritance for the smallest PU. With this simplification, only two kinds of the merge candidate sets, HEVC merge candidate set and 3D-HEVC merge candidate set exist. The experimental results reportedly show no coding loss.

There are three kinds of merge candidate sets, HEVC candidate set, 3D-HEVC candidate set, and HEVC set with VSP inheritance.

|  | Texture | Depth |
| --- | --- | --- |
| Independent view with PU >= 8x8 | HEVC merge candidate set | 3D-HEVC merge candidate set |
| Independent view with 4x8/8x4 PU | HEVC merge candidate set | HEVC merge candidate set |
| Dependent view with PU >= 8x8 | 3D-HEVC merge candidate set | 3D-HEVC merge candidate set |
| Dependent view with 4x8/8x4 PU | HEVC merge candidate set with VSP inheritance | HEVC merge candidate set |

It is propose to use HEVC merge candidate set for 4x8/8x4 PU, i.e., to remove VSP inheritance for 4x8/8x4 PU.

Experiment results: 0.00/0.01% BR increase for the coded/synthesized views.

This contribution removes VSP inheritance partially.

It is mentioned that complete removal of VSP inheritance is better if there is no coding impact.

Revisit upon the additional results by I0053.

JCT3V-I0152 Cross check of unification of the merge candidate sets in 3D-HEVC (JCT3V-I0098) [J. Y. Lee, C. Kim (Samsung)] [late]

JCT3V-I0106 Further VSP inheritance simplification [T. Ikai, T. Tsukuba (Sharp)]

This contribution proposes to simplify VSP inheritance by referring VSP flag at only one candidate (A1). It is reported that BD rate change is -0.01, -0.01 and 0.00 % respectively.

It is proposed to allow inheriting VSP from A1 candidate (1st spatial candidate).

Default VSP candidate is kept used.

See the further discussion under section 2.4.

JCT3V-I0188 Crosscheck on Further VSP inheritance simplification (JCT3V-I0106) [S. Shimizu (NTT)] [late]

JCT3V-I0112 Simplification for VSP-related Merge Candidate List Construction [Y. Zhang, Q. Wang, L. Yu (Zhejiang Univ.)]

In this contribution, a simplified method for VSP-related merge candidate list (MCL) construction is proposed. VSP mode from only left spatial neighboring block (block A1) is inherited for current prediction unit and only when block A1 is not VSP-coded, a default VSP candidate is inserted to a fixed position in MCL. The simulation results show that, by reducing the number of blocks for VSP inheritance and inserting the default VSP candidate in a fixed position, the proposed method achieved 0.01% gain, especially on sequences which benefit from view synthesis prediction at maximum 0.38% gain on dependent view.

It is proposed to allow inheriting VSP from A1 candidate (same with I0106).

It is also proposed that VSP candidate is inserted at the fixed position in the final candidate list only when there is no inherited VSP candidate in the list.

Small gain (0.01% BR reduction) for synthesis view is observed.

See the further discussion under section 2.4.

JCT3V-I0136 Crosscheck results on simplification for VSP-related merge candidate list construction (JCT3V-I0112) [M. Li, P. Wu (ZTE)]

JCT3V-I0130 Simplification of VSP candidate derivation [G. Bang (ETRI), M. S. Lee, Y. S. Heo, G. H. Park (KHU), G. S. Lee, N. H. Hur (ETRI)]

The merging candidate list(MCL) construction process of current HTM uses VSP candidate. However, the current HTM adds VSP candidate to MCL no matter whether spatial candidates are VSP block (inherited VSP candidates). And to reduce depth-map memory access, each inherited VSP candidate should have the same DV components with the current CU. Therefore, when the inherited VSP candidates are available and also the default VSP candidate is available, MCL includes redundant VSP candidates. In order to remove such redundancy, this contribution proposes to constrain the position of inherited VSP candidates, and skipping process for the default VSP merging candidate when at least one inherited VSP candidates is available. The experimental result shows almost same performance with the current HTM.

There are two aspects:

- Skip default VSP candidate when inherited VSP candidates are available in the list

- Allow to inherit VSP from 1st available candidate A1 or B1

Small gain (0.01% BR reduction) for synthesis view is observed.

See the further discussion under section 2.4.

JCT3V-I0174 Crosscheck on Simplification of VSP Candidate Derivation (JCT3V-I0130) [Y. Zhang (Zhejiang University)] [late]

JCT3V-I0131 Adaptive candidate selection [G. Bang (ETRI), Y. S. Heo, G. H. Park (KHU), G. S. Lee, N. H. Hur (ETRI)]

In this contribution, adaptive selecting method for merging candidate is proposed. By referencing selection of merging candidate on temporal collocated PU which is on the previously coded picture, merging index for temporal inter-view and MPI candidates are no more signaled. The results reportedly shows -0.15% coding gain in total bitrate.

The proposal was based on the observation that IVMC/MPI candidate is often selected and the current block tends to select IVMC/MPI as a result of RDO when temporally collocated block uses IVMC/MPI.

Proposal is a conditional signalling of merge index. If temporal collocated block is coded by IVMC/MPI merge, merge index signalling is skipped and inferred to IVMC/MPI.

Experiment result (with DBBP off): 0.15% BR reduction for the coded view, 0.23% BR increases for the synthesis view. It is mentioned that some implementation bugs are remained.

It is noted that this method requires storing one flag, whether IVMC/MPI or not, in DPB.

One expert recommends applying this method as encoder optimization.

JCT3V-I0149 Cross check of Adaptive candidate selection (JCT3V-I0131) [M. W. Park, C. Kim (Samsung)] [late]