| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0196 |
| --- | --- |

| Title: | BoG report on MV-HEVC conformance test |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ying Chen Shinya Shimizu Teruhiko Suzuki | Tel: Email: | +1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

This document provides a report of the BoG on MV-HEVC conformance tests.

# Introduction

A BoG was held on Sunday July 6, 2014, 06:00 pm to approx. 07:00 pm to provide information for the first draft text for the MV-HEVC conformance tests.

- Test streams for MV-HEVC (stereo main profile)

The aspects that are requested by JCT-3V to be considered to generate multiple bitstreams are listed as follows:

- Picture/slice type

  - A picture may be random access picture, e.g., IDR or CRA, or just non-IRAP.

  - A slice can be I, P or B.

- Prediction type

  - Inter-view prediction, Intra prediction, Inter prediction and different combinations.

- Spatial resolution

  - Symmetric or asymmetric resolutions

- Presence of auxiliary depth pictures

  - Even though the MV-HEVC bitstream includes depth pictures, as auxiliary pictures, the decoder shall not crash by decoding such a bitstream. The decoder as conforming to the current profile is not required to output the depth views.

In addition, the BoG discussed the following two topics, w.r.t. single-view compatibility and hybrid codec, a.k.a. multi-standard scalability.

The case which has an AVC base view and HEVC enhancement view was discussed and it was concluded that such a case might not be relevant for MV-HEVC conformance, mainly because the AVC base view cannot be decoded by MV-HEVC.

Inter-operability between MV-HEVC coded bitstream and HM software and hypothetically any device that conforming to HEVC version 1: even no specific bitstreams are created for this specific purpose, it is generally requested that the conformance bitstreams should be tested by the HM decoder and the base view shall be correctly decoded, i.e., by outputting the same pictures as the output pictures (with layerId equal to 0) from the MV-HEVC reference software. In addition, it is suggested to go through the bitstream extraction process to generate a sub-bitstream without NAL units that are not recognizable by HEVC version 1.

Volunteers for conformance bitstreams: Qualcomm, Nokia, NTT, Sony, Sharp and LGE.

## Test bitstream #MVHEVCS-A

Specification: All slices are coded as I, P or B slices. Only the first picture of each view is coded as IDR picture. Each picture contains only one slice. NumViews is equal to 2. NumDirectRefLayers of the non-base view is equal to 1. For each picture in the non-base view, inter-view prediction is enabled. The two views are with the same spatial resolution. All NAL units are encapsulated into the byte stream format specified in ITU-T Rec. H.265 | ISO/IEC 23008-2.

Functional stage: Decoding of two views with inter-view prediction and inter-prediction.

Purpose: To conform the most normal case.

Tentative contributor: Qualcomm

## Test bitstream #MVHEVCS-B (all Intra)

Specification: All slices of the base view are coded as I slices and all slices of the non-base view are coded with only inter-view prediction, thus P slices. Only the first picture of each view is coded as IDR picture. Each picture contains only one slice. NumViews is equal to 2. NumDirectRefLayers of the non-base view is equal to 1. The two views are with the same spatial resolution. All NAL units are encapsulated into the byte stream format specified in ITU-T Rec. H.265 | ISO/IEC 23008-2.

Functional stage: Decoding of two views with only inter-view prediction and intra-prediction.

Purpose: To conform the all-Intra case.

Tentative contributor: Sharp.

## Test bitstream #MVHEVCS-C (simulcast with the same resolution)

Specification: All slices are coded as I, P or B slices. Only the first picture of each view is coded as IDR picture. Each picture contains only one slice. NumViews is equal to 2. NumDirectRefLayers is always equal to 0, meaning inter-view prediction is disabled. The two views are with the same spatial resolution. All NAL units are encapsulated into the byte stream format specified in ITU-T Rec. H.265 | ISO/IEC 23008-2.

Functional stage: Decoding of two views with only inter-prediction and intra-prediction.

Purpose: To conform the case of simulcast coding.

Tentative contributor: Sony.

## Test bitstream #MVHEVCS-D (different SPS, e.g., spatial resolutions)

Specification: All slices are coded as I, P or B slices. Only the first picture of each view is coded as IDR picture. Each picture contains only one slice. NumViews is equal to 2. NumDirectRefLayers is always equal to 0, meaning inter-view prediction is disabled. The two views are with different spatial resolutions. All NAL units are encapsulated into the byte stream format specified in ITU-T Rec. H.265 | ISO/IEC 23008-2.

Functional stage: Decoding of two views with only inter-prediction and intra-prediction.

Purpose: To conform the case that uses different spatial resolutions for two views.

Tentative contributor: NTT

## Test bitstream #MVHEVCS-E (random access)

Specification: All slices are coded as I, P or B slices. Only the first picture of each view is coded as IDR picture and thus an IRAP access unit. In addition, every two GOPs start with an access unit which contains pictures that are all CRA pictures. Each picture contains only one slice. NumViews is equal to 2. NumDirectRefLayers of the non-base view is equal to 1. For each picture in the non-base view, inter-view prediction is enabled. The two views are with the same spatial resolution. All NAL units are encapsulated into the byte stream format specified in ITU-T Rec. H.265 | ISO/IEC 23008-2.

Functional stage: Decoding of two views with inter-view prediction and inter-prediction.

Purpose: To conform the random access hierarchical B case.

Tentative contributor: Qualcomm

## Test bitstream #MVHEVCS-F (Inter-view pred. only for IRAP pictures)

Specification: All slices are coded as I, P or B slices. Only the first picture of each view is coded as IDR picture and thus an IRAP access unit. In addition, for each of every two following GOPs, an access unit which contains pictures that are all CRA pictures are requested. Other pictures are non-IRAP pictures. Each picture contains only one slice. NumViews is equal to 2. NumActiveRefLayerPics is equal to 1 for each picture in an IRAP access unit of the non-base view, and 0 otherwise, i.e., only for each picture in the non-base view in the IRAP access unit, inter-view prediction is enabled. The two views are with the same spatial resolution. All NAL units are encapsulated into the byte stream format specified in ITU-T Rec. H.265 | ISO/IEC 23008-2.

Functional stage: Decoding of two views with inter-view prediction only for random access points.

Purpose: To conform the flexibility of inter-view prediction applicability scope.

Tentative contributor: Qualcomm.

## Test bitstream #MVHEVCS-G (Hybrid scalability)

Specification: All slices are coded as I, P or B slices. For the non-base view, only the first picture of is coded as IDR picture and thus an IRAP access unit. In addition, every two GOPs start with an access unit which contains pictures that are all CRA pictures. Each picture contains only one slice. NumViews is equal to 2. NumDirectRefLayers of the non-base view is equal to 1. For each picture in the non-base view, inter-view prediction is enabled. The two views are with the same spatial resolution. The base view contains pictures derived by external means (may be set as dummy pictures). All NAL units are encapsulated into the byte stream format specified in ITU-T Rec. H.265 | ISO/IEC 23008-2.

## Test bitstream #MVHEVCS-H (random access 3-view case, PIP conf.)

Specification: All slices are coded as I, P or B slices. Only the first picture of each view is coded as IDR picture and thus an IRAP access unit. In addition, every two GOPs start with an access unit which contains pictures that are all CRA pictures. Each picture contains only one slice. NumViews is equal to 3. NumDirectRefLayers of the non-base view is equal to 1. For each picture in the non-base view, inter-view prediction is enabled. The three views are with the same spatial resolution. All NAL units are encapsulated into the byte stream format specified in ITU-T Rec. H.265 | ISO/IEC 23008-2.

Functional stage: Decoding of two views with inter-view prediction and inter-prediction.

Purpose: To conform the random access hierarchical B case.

Tentative contributor: NTT

## Test bitstream #MVHEVCS-I (random access 3-view case, IBP conf.)

Specification: All slices are coded as I, P or B slices. Only the first picture of each view is coded as IDR picture and thus an IRAP access unit. In addition, every two GOPs start with an access unit which contains pictures that are all CRA pictures. Each picture contains only one slice. NumViews is equal to 3. NumDirectRefLayers of the first non-base view is equal to 1 and NumDirectRefLayers of the first non-base view is equal to 2. For each picture in the non-base view, inter-view prediction is enabled. The three views are with the same spatial resolution. All NAL units are encapsulated into the byte stream format specified in ITU-T Rec. H.265 | ISO/IEC 23008-2.

Functional stage: Decoding of two views with inter-view prediction and inter-prediction.

Purpose: To conform the random access hierarchical B case.

Tentative contributor: LGE

## Test bitstream #MVHEVCS-J (random access 3-view case, IBP conf. MV-HEVC plus depth)

Specification: All slices are coded as I, P or B slices. Only the first picture of each view is coded as IDR picture and thus an IRAP access unit. In addition, every two GOPs start with an access unit which contains pictures that are all CRA pictures. Each picture contains only one slice. NumViews is equal to 3. NumDirectRefLayers of the first non-base view is equal to 1 and NumDirectRefLayers of the first non-base view is equal to 2. The P view is coded with viewOrderIdx equal to 1. For each picture in the non-base view, inter-view prediction is enabled. Each view contains both texture and depth and the views are with the same spatial resolution. All NAL units are encapsulated into the byte stream format specified in ITU-T Rec. H.265 | ISO/IEC 23008-2.

Functional stage: Decoding of three views with inter-view prediction and inter-prediction.

Purpose: To conform the random access hierarchical B case.

Tentative contributor: Nokia

Note that the vertical conostraint applies only to the P view (for both texture and depth of this P view).