| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-I1002-v7 |
| --- | --- |

| Title: | MV-HEVC Draft Text 9 |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-3V |  |  |
| Purpose: |  |  |  |
| Author(s) or Contact(s): | Gerhard Tech Fraunhofer HHI Krzysztof Wegner Poznan University of Technology Ying Chen Qualcomm Incorporated Miska Hannuksela Nokia Corporation Jill Boyce Vidyo | Email: | gerhard.tech@hhi.fraunhofer.de kwegner@multimedia.edu.pl cheny@qti.qualcomm.com miska.hannuksela@nokia.com jill@vidyo.com |
| Source: | Editors |  |  |

_____________________________

# Integrations

- ----------- Release v7-----------

- (Review YK26): General review changes

  - Removed the copied semantics of sps_max_num_reorder_pics and sps_max_latency_increase_plus1 from clause 7, and remvoed the inference rules for sps_max_num_reorder_pics and sps_max_latency_increase_plus1 that are not needed.

- (Review GT34) Added a missing bracket.

- (Review GT33) Added missing inference rules for sps_max_num_reorder_pics and sps_max_latency_increase_plus1.

- (Review GT32) Copied semantics of sps_max_num_reorder_pics and sps_max_latency_increase_plus1 from clause 7.

- (Review YK25): General review changes

  - Fixed a typo in clause H.8.1.4.1.3 (Equation H-62)

  - Aligned the index styles of some figures and tables

  - Updated all fields without change marks

  - Removed the remaining three editor's notes

- (Review YY08): general review of Annex H. Fixed variable naming in clause H.8.1.4.1.3 to disambiguate Ver (Y direction) from  luma component (Y).

- (Review MH26): changed semantics of sub_layers_vps_max_minus1 as per YY07's request and confirmed her other change in F.7.4.3.1.1.

- (Review YY07): geneal review of F.7.4 Semantics

- (Review YK24): General review changes

  - Reviewed MH25 and JB18 changes.

  - Various editorial changes suggested by Gary

  - Fixed two PTL syntax element inference issues: 1) the presence of the sub-layer syntax elements is not dependent on the profile presence flag; 2) the presence of general_level_idc does not depend on the profile presence flag, either.

  - Updated all fields without change marks and made sure there are no occurrences of "clause 0", "clauses 0", "Annex 0", "through 0" and "Error!" in the text due to cross-referencing issues that would be found after updating fields.

  - Reviewed MH25 and JB18 changes.

  - 1) the presence of the sub-layer syntax elements is not dependent on the profile presence flag; 2) the presence of general_level_idc does not depend on the profile presence flag, either.

- (Review JB18) General review

- (Review MH25)

  - Reviewed YK22, GT31, and YK23. Editorial fixes.

  - Updated the derivation of picSeq for the semantics of general_xx_flag in profile_tier_level( ).

- (Review YK23): General review changes

  - Reviewed GT31 changes and addressed the editor's notes by GT31

- (Review GT31)

  - Some editor's notes and a fixes related to VPS VUI bitstream partition HRD parameters

  - A minor formatting fix

- (Review YK22): General review changes

  - Reviewed all recent changes after YK21

  - Addressed/removed some editors notes, particularly those by GT related to inference of profile_tier_level( ) syntax elements, and fixed a note indexing issue (two index reset) in the clause of profile_tier_level( ) semantics

  - Simplified a bit the semantics of the three HRD SEI messages by using "the current bitstream partition" and fixed a typo in the semantics of the buffering period SEI message

- (Review MH24): GT28, GT29, GT30, and JCT10 reviewed. Editorial fixes and some responses to Editor's notes.

- (Review JC10):

  - Semantics improvement of scaled_ref_layer_offset and ref_region_offset.

- (Review GT30):

  - Added some editor's notes regarding the profile_tier_level( ) syntax structure.

  - Minor fixes.

- (Review GT29): Cleanup semantics of dpb_size syntax elements. In particular inference of max_vps_dec_pic_buffering.

- (Review GT28): Fix layer_set_idx_for_ols_minus1. Reason for fix: When NumLayerSets is equal to 2, the length of the layer_set_idx_for_ols_minus1[ i ] syntax element is 0 bits.

- (Review MH23): General review changes

  - A few cross-references fixed.

  - A few editorial fixes and one informative note further clarified.

  - YK20 and YK21 reviewed.

- (Review YK21): General review changes

- (Review YK20): General review changes, included the following changes related to HRD text:

  - Removed/resolved some editor's notes

  - Fixed some HRD issues, including the issue of always setting SubPicHrdFlag equal to 1, includig the following changes:

    - Changed the definition of decoding unit

    - Moved the derivation of SubPicHrdFlag from the HRD clause to the general decoding process

    - Changed in conformance tests description that SubPicHrdFlag can be equal to either 0 or 1

    - Resurrected the removal of the HRD processes for SubPicHrdFlag is equal to 0 by rejecting those changes by YK16

    - Added a contraint that the DPB output times derived for all pictures in on AU shall be the same

    - Replaced the constraint that requires the value of sub_pic_hrd_params_present_flag in all hrd_parameters( ) syntax structures to be equal to 1 in certain conditions to be that the value of sub_pic_hrd_params_present_flag shall be the same for all hrd_parameters( ) syntax structures that apply to at least one bitstream partition of a particular output layer set, and moved the updated contraint to the semantics of the hrd_parameters( ) syntax structure.

  - Changed the name of the syntax element all_ref_layers_active_flag to default_ref_layers_active_flag

  - Editorially improved the equation and the description for calculation of the arrays BpBitRate and BpbSize

  - Reviewed MH22 changes

- (Review YK19): General review changes, included some changes agreed at the 5th conference call among editors, including removal of some editor's notes, and included guidelines on how to proceed for some remaining editor's notes.

- (Review MH22)

  - The invocations of function getCurrMsb were changed to GetCurrMsb.

  - Changes in F.13.5.2.3: Marking of pictures as "needed for output" or "not needed for output" is performed after the last picture of an access unit has been decoded, because the final PicOutputFlag value for the pictures of an access unit may not be available earlier. Setting of PicLatencyCount at picture level was removed, as the variable is AU-level (as modified by AR earlier).

  - Clarified in F.8.3.2 (RPS decoding), which parts of 8.3.2 are replaced by the specifications in F.8.3.2 and which parts of F.8.3.2 are additional. Added a constraint for CRA pictures in F.8.3.2. Added the definition of IRAP picture in F.3, where the definition itself is identical to that in clause 3, but the associated NOTE clarifies the characteristics of IRAP pictures in predicted layers.

  - Added the association of auxiliary pictures with primary pictures to the depth representation information SEI message, similarly to other SEI messages for indicating characteristics of auxiliary pictures.

- (Review YK18): General review changes, including the following:

  - Fixed some equation indexing issues, made all equations use appropriate equation styles with equation indexes, and removed the relevant editor's notes

  - Made sure there is one empty line after each table

  - Added inference of depth_nonlinear_representation_model[ 0 ] and depth_nonlinear_representation_model[ depth_nonlinear_representation_num_minus1 + 2 ], to avoid setting values to syntax elements.

  - Updated all fields with no change marks, and then resolved a few cross-referencing issues

  - Aligned some other styles with the latest version of JCTVC-R1013.

  - Turned on tracking of format changes.

- (Review YK17): General review changes

- (Review MH21)

  - Proof-reading of the semantics of the inter-layer constrained tile sets SEI message.

  - Proof-reading of the text related to the LayerResetFlag.

- (Review YK16): General review changes

  - Addressed/removed several editor's notes, including the ones on HRD text.

  - Fixed the equation on BpbSize calculation and added two editor's notes for discussion.

- ----------- Release v6-----------

- (Review YK15): General review changes

- (Review MH20):

  - Reviewed JB17, JC09, YK13, AR14, and YK14. Minor fixes.

  - Added a few editor's notes related to reference layer location offsets requesting for further clarifications.

- (Review YY06): Reviewed F.7.4.3.3.3 and fixed a few typos

- (Review YK14): General review changes

- (Review AR14): Fix few typos

- (Review YK13): General review changes, including the following:

  - Reviewed recent changes, resolved/removed some editor's notes

  - Updated the definitions of "picture order count", "quality scalability", and "spatial scalability"

  - Avoided the signalling of the max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] for unnecessarily layers

  - Avoided the use of "and/or" in this document

- (Review JC09): Reference offset semintics rephrase

- (Review JB17): Tier and level constraints

- (Review MH19):

  - Clarified that it is allowed to directly nest HRD SEI messages in a scalable nesting SEI message included in a SEI NAL unit with nuh_layer_id greater than 0, when the message applies to a rewritable independent non-base layer.

  - Some editorial changes in the semantics of HRD SEI message to align with Annex D.

- (Review MH18):

  - Reviewed YK12. Removed some editor's notes that I considered fully dealt with.

  - Slightly modified the wording of the layer-specific uniqueness requirement of POC values within a CVS

  - Added a NOTE to warn about cautious use of poc_reset_idc equal to 0 with IDR pictures.

- (Review YK12): General review changes, including the following:

  - Reviewed recent changes

  - Responded to some editor's notes, particularly on Miska's notes related to POC

  - Resolved and removed some editor's notes

  - Added a NOTE as part of the semantics of discardable_flag

  - Modified the note that is part of the semantics of poc_msb_cycle_val

  - Modified the wording of the layer-specific uniqueness requirement of POC values within a CVS

  - Modified the wording of some descriptions of tier and level

  - Made the following alignments to Doc 1 and the ITU-T text:

    - Indices in Table and Figure capations (for Annexes)

    - Addition of comma after "e.g." and "i.e." (no change mark)

    - Replacement of most "subclause" with "clause" (no change mark)

    - Syntax table sytle - center-aligned the Descriptor coloumn (no change mark)

  - Updated all fields and resolved several cross-referencing issues (mostly those fields referring to clauses not present in this document)

  - Yellow-highlighted all references to clauses not present in this document, for which when merged cross references should be added

  - Added Added missing cross-references to references of clauses present in this document

- (Review MH17): The order of paragraphs related to the semantics of reference layer location offsets was changed.

- (Review MH16)

  - The semantics of HRD SEI messages for multi-layer extension were appended in clause F.14.2.2.

  - Some obsolete editor's notes were removed, some other editor's notes were responded to.

  - A new editor's note on the derivation of CpbSize[ i ] and BitRate[ i ] in the profile specifications was added.

- (Review JB16): Updated tier, level references based on RExt changes to Annex A.

- (Review MH15)

  - Reviewed AR13, JC08, and GT27

  - Changed "direct reference layer picture" to "decoded picture" in reference region offset semantics to reflect the decision of the editors' call that layer for which reference regions are specified does not need to be a direct reference layer

  - Rephrased the constraint that no two POCs of the same layer within the same CVS shall be equal.

  - Added POC related editor's notes.

  - Defined "source picture for inter-layer prediction" and used that definition in the semantics of max_tid_il_ref_pics_plus1 and discardable_flag.

  - Modifications in the VUI parameters semantics in F.15.3.1.

- (Review AR13): General review

  - Resolved few Ed. notes related to POC

- (Review JC08)

- (Review GT27) General Review

  - Fix target layer list derivation

  - Replaced indirect and direct predicted/reference layer by predicted/reference layer in some places

  - Improved definition of indirect reference layer.

  - Changed sentences "The specifications in xxx apply with the following modifications and additions" to end consistently with a ":".

  - Other Minor formatting style corrections

- (Review MH14):

  - Ed. notes requiring decisions related to the reference layer location offsets signalling.

  - Restructured the specifications of the scope to which the profile_tier_level( ) syntax structure applies in F.7.4.4

  - Added an ed. note pointing out that the semantics of buffering period, picture timing and decoding unit information SEI messages is out-of-date when in the context of Annex F

  - Modifications in the bitstream partition initial arrival time SEI message syntax and semantics

- (Review YY05): general review

- (Review JC07): general review

- (Review JB15): General review

- (Review YK11): General review changes, including the following:

  - Reviewed recent changes.

  - Added comments responding to some editor's notes.

  - Resolved some editor's notes and removed some editor's notes that are either resolved or considered no action needed.

  - Aligned the level constraints descriptions in Annex G and H with Annex A, for those related to CPB size and bit rate.

- (Review MH13)

  - Checked AR11 changes on poc_msb_cycle_val, made related editorial corrections and added more related editor's notes.

  - One change and one editor's note added in F.8.3.1 (POC derivation).

- (Review AR12): General review

  - Remove inference of VPS DPB parameter from SPS.

  - Added array VpsProfileTierLevel.

  - Replied to some Ed. notes

- (Review AR11): General review

  - Semantics of poc_msb_cycle_val

  - Changes to restriction on POC alignment restriction

  - Clarify semantics of BSP initial arrival time SEI

  - Corrected indentations related to AuOutputFlag and PicLatencyCount in F.8.1.2

- (Review JB14): Removed resolved editor’s notes. Added note to H.8.1.3.

- (Review JB13): General review. Addressed editor’s note related to discardable_flag and max_tid_il_ref_pics_plus1, and added inference values for conf_win_vps_left_offset, conf_win_vps_right_offset, conf_win_vps_top_offset.

- (Review GT26) General review

- (Review MH12): The definition of "sub-bitstream extraction process" was clarified with a note. The invocation of the sub-bitstream extraction process related to bitrate and picture rate VPS VUI parameters was corrected.

- (Review MH11): General review:

  - Editor's note questioning the correctness of the loop of PTL structures in the VPS extension.

  - Other modifications and editor's notes concerning PTL signalling in the VPS.

  - Editor's notes added and other small changes.

- (Review JB11): Addressed editor’s notes relating to missing profile constraints by copying some constraints in Main/Main 10 profiles which were needed with RExt

- (Review YY04): Reviewed colour mapping related spec. Addressed Ed (GT) note in the semantics in F.7.4.3.3.5.

- (Review GT25) Improved derivation of reference and predicted layers.

- (Review GT24) Modifed definitions related to reference and predicted layers.

- (Review GT23) Renaming of variables related to reference and predicted layers.

- (Review MH10), editor's notes tackled:

  - Ensured that FirstAddLayerSetIdx and LastAddLayerSetIdx are used only when they have been derived

  - The persistence of and a constraint in temporal motion vector prediction constraints SEI message modified

- (Review JC06): general review

- (Review YK10): General review changes, including the following:

  - Reviewed recent changes, particularly by MH09.

  - Resolved/removed some editor's notes, including the one on the semantics of profile_tier_level_idx, by slightly changing the syntax and the semantics of both profile_tier_level_idx and vps_num_profile_tier_level_minus1.

- (Review MH09): General review, including:

  - Resurrected the pre-YK09 version of the derivation of PicOutputFlag and BaseLayerPicOutputFlag and included the constraint on alt_output_layer_flag in the Multiview Main profile specification.

  - Reviewed particularly AR10, YK08 and the SEI part of YK09.

- (Review YK09): General review changes, including the following:

  - Resolved/removed some editors' notes.

  - Changed coding of the 4 bit depth related syntax elements in the colour_mapping_table( ) syntax structure from u(3)-coded to be ue(v)-coded.

  - Changed the derivation of PicOutputFlag and BaseLayerPicOutputFlag such that the alternate output process is not applied across layers with different view IDs.

  - Added update of the derivation of the list nestingLayerIdList[ i ] in the semantics of the scalable nesting SEI message in Annex F.

- (Review YK08): General review changes, including the following:

  - Restructured SEI message subclauses in Annexes F and G.

  - Moved/clarified multi-layer specific changes to the semantics of scalable nesting SEI messages to Annexes F and G.

- (Review GT22) Fixed COctant, etc.

- (Review GT21)

- (Review AR10): General review

  - Move derivation of AuOutputFlag to end of AU.

  - Making PicLatencyCount AU-specific.

  - Changes to derivation of BaseLayerPicOutputFlag.

  - Editorial simplification of picture flushing process.

  - Changed "SmallestLayerId" to "SmallestLayerId or 0" for nuh_layer_id of EOS NAL unit in derivation of NoClrasOutputFlag.

  - Added condition to set sub_pic_hrd_params_present_flag equal to 1 when a bitstream partition contains more than one layer.

  - Derive LayerResetFlag only of nuh_layer_id greater than SmallestLayerId.

- (Review MH08): General review, including the following changes:

  - Definitions updated: added the definitions of access unit, inter-layer reference picture, and output order. Changes the definition of reference layer picture.

  - Text that had been directly copied from 7.4.2.2 to F.7.4.2.2 was removed.

- (Review YK07): General review

  - Removed semantics of AUD, to align with latest changes to the AUD semantics in clause 7.

  - Made some clarficaiton changes and added some editors' notes on the PTL descriptions in both Annexes G and H, including align the profile description in Annex H to Annex G for involving of the sub-layer PTL information.

- (Review YK06): General review

- (Review JB09): General review

- (Review YY03): reviewed Annex H

- (Review JC05): General review

- (Review GT20) Aligned syntax table formatting with Doc1 (no changemarks).

- (Review GT19) General Review

- (Review JB08): Harmonization of the Multiview Main and Scalable Main/Scalable Main 10 profile descriptions, addressing editors notes.

- (Review AR09):

- (Review YY02): General review of Annex F and Annex H. Changes include fixing some substring matching problems in Annex F, bug fixes in Annex H, and other editorial improvements.

- (Review YK04): General review, made the following changes as well as some minor improvements:

  - Moved sps_multilayer_extension_flag and pps_multilayer_extension_flag from Annex F to clause 7, and thus removed the PPS syntax table.

  - Moved semantics update to num_extra_slice_header_bits from Annex F to clause 7.

  - Moved semantics of general_inbld_flag from Annex F to clause 7.

- (removal of semantics for PTL constraint flags)

- (Review MH06):

  - Separated the processing of an external base layer picture to i) operations done at the beginning of CVSG, ii) operations done at the beginning of an AU, iii) operations done after the decoding of the first slice segment header of the first coded picture of an AU (but before decoding any actual coded data), and iv) operations done after all pictures of an AU have been decoded (already mostly done by MH05).

  - General review.

- (Review YK03): General review, made the following changes as well as some minor improvements:

  - Resolved the editor's note on inference of cprms_add_present_flag[ 0 ] and num_sub_layer_hrd_minus1[ 0 ], and the editor's note on inference of direct_dependency_type[ i ][ j ].

  - Moved most updates of the base VPS semantics to clause 7.

- (Review MH05): General review

- (Review JC04): General review

- (Migrate profile_tier_level to clause 7)

- (Review JB07) Resolution of some editor’s notes.

- (Review GT18) Removed the "Marking process for ending the decoding of a coded picture with nuh_layer_id greater than 0" in Annex G and H and moved marking to the "Decoding process for ending the decoding of a coded picture with nuh_layer_id greater than 0" in Annex F.

- (Review GT17) General review

- (Review JC03)

- (Review GT16) General review

- (Review GT15) General review

- (Review GT14) General review

- (Review GT13) Improved requirements related to bsp_sched_idx.

- (Review GT12) General review

- ----------- Release v5-----------

- (Review JB06)

- (Review MH04): Various alignments between R1013 and R1008.

- (Review JB05) Review of MH03 modifications to profile specifications.

- (Review MH03)

  - Resurrected the definitions of output layer, OLS, output operation point and picture unit and related editor's notes, as they have not yet been integrated in clause 3 and there are editorial decisions that need to be made whether to include the three first definitions in clause 3 or clause F.3.

  - Related to GT10 and JB03: removed the inference of layer_id_included_flag[ ][ ], because the inferred syntax element appears not to be used anywhere.

  - Corrected the invocation of sub-bitstream extraction processes for the constraint on alt_output_layer_flag[ i ].

  - Some asserted corrections in the Multiview, Scalable Main and Scalable Main 10 profile specifications.

  - Some editorial corrections and new editor's notes.

- (Review JB04) Corrections to R0156 integration, review of GT10

- (Review GT11) Fix requirement of NumOutputLayersInOutputLayerSet[ i ]

- (Review GT10) Removal inference layer_id_included_flag) from highest_layer_idx_plus1 semantics, since inferred values are not used somewhere else in the spec.

- (Review GT09) General review.

- (Review GT08) Renamed Clause7CompatibleSPSFlag to MultiLayerExtSpsFlag and inverted logic.

- (Review GT07) Removed ViewScalExtLayerFlag, since used in three places only.

- (Review GT06) Fix ScalabilityId default value.

- (Review GT05) General clean up

- (Review GT04) Clean up semantics of sps_max_sub_layers_minus1 sps_ext_or_max_sub_layers_minus1

- (Review GT03) Fixed minor issues.

- (Review JB03) Resolution of a number of editors’ notes

- (Review GT02)

- (Review JB02) General review, separation of  pps_multilayer_extensions( ) syntax

- (Review JB01) Migration of definitions to clause 3, harmonization of Multiview Main profile language with Scalable Main profile

- (Review MH02)

  - Moved as many definitions and abbreviations as possible from clauses 3 and 4, respectively, to clauses F.3 and F.4, respectively.

  - Removed clause F.7.4.2.4.6 "Order of VCL NAL units and association to picture units", which was empty in content and had been taken care by the definition of the term picture unit.

  - Rephrasing of modified semantics of Annex D SEI messages applied in multi-layer context

  - Various small editorial fixes

- (Review JB0)

- (Review AR08)

- ----------- Release v4-----------

- (Review YK02): Added definition of "necessary layer"; Fixed semantics of PTL syntax elements; Added an inference rule for sub-layer PTL syntax elements when the PTL syntax structure is included in an SPS or is the first PTL syntax structure in the VPS; Modified the decoder capability requirement descriptions, the profile definitions, and the level definitions to take into account sub-layer PTL information (the same changes to Annex H are yet to be done).

- (Review JC02)

- Annex D: v1 - v2/BP SEI message: Remaining changes to BP SEI message.

- Annex D: v1 - Other SEI payload semantics/Split: Move remaining changes to SEI messages to Annex F.

- (Review AR07): General review

- (Review YK01): Removed changes to clauses before and up to Annex E that have been integrated into R1013 and Annex F.

- (M21: HRD changes) Moved multi-layer HRD operations to clause F.13, including changes made in the separate HRD text branch; Fixed the sub-bitstream extraction/rewriting processes and the requirement on what sub-bitstreams must be conforming bitstreams.

- (Review YK00): Made general_inbld_flag applicable to Annex A profiles only.

- Annex D: v1 - Other SEI payload semantics/Split: Remove text already moved to base part

- Annex D: v1 - v2/General SEI payload semantics/Split: More changes category

- (M24) Moving of multi-layer specific aspects of clause 8.1 of R1008 to F.8. The general decoding process using the VPS scope instead of using “bitstream” or “CVS”.

- V1CompatibleSPSFlag fix

- (#24) Define semantics of single_layer_for_non_irap_flag and higher_layer_irap_skip_flag in an equivalent way as they would be used with an internal base layer.

- (Editorial improvement)

- (inbld fix) Review of the implications of general_inbld_flag

- (#76 F.11.2) If the capabilities of one profile are "nested" within the capabilities of another profile, decoders should be required to recognize that and decode the bitstream.

- (Review JB) Moved SEI message numbering to Annex D, defined collocated sample

- (Review JB2)

- Version 3 release

- (Typo corrections)

- (Review YY01): Cross-checked R0156 and R0227. Editorial improvements.

- (M21: HRD changes) Changes per JCTVC-R0230 and JCTVC-R0272, as well as moving of the multi-layers-specific HRD operations into clause F.13 instead of putting all HRD operations in Annex C

- (Review JC01) Fix in semantics of min_horizontal_ctu_offset_plus1 in VPS VUI and cross-reference fix in Annex H

- (MH01) Some fixes in sequence parameter set semantics

- (#83) Remove constraint prohibiting 4:2:0 for alpha and depth and specify the the semantics of associated SEI messages, if not already specified, to refer to the luma component only.

- (#29/R0053) On invocation of a correct decoding process for the current picture

- (M11) Clause 7.4.2.4.2 (Semantics): Only the changes as in the latest joint SHVC/MV-HEVC text. Move the only paragraph that needs a variable derived in Annex F to Annex F. Probably OK to leave the parameter set activation text in RExt text unchanged – but check for absolutely needed minimum amount of changes.

- (Review GT01) Fix NecessaryLayerFlag.

- (Normative references) Migration of normative references to R1013 doc

- (Overlay SEI) Corrections to byte alignment in overlay info SEI message, #11 indication of persistence scope

- (#84 INBLD) Use one of reserved_44_zero_bits to indicate INBLD capability required

- (Review AR03): Addition of definition of CLVS.

- (Review AR04): Remove semantics of two VPS flags, and move constraint on single_layer_for_non_irap_flag.

- Annex D: v1 - Other SEI payload semantics/Split: Move changes made to some of the SEI messages to the base part.

- Annex D: v1 - v2/APS SEI message/Split: Move changes made to APS SEI message to base part.

- (#76) share general_profile_idc for naturally-nested profiles (e.g., Scalable Main 10 and Scalable Main) as was done in RExt

- M9 7.2 (Specification of syntax functions and descriptors)

# Abstract

Remarks: In this document, Annex F contains common syntax, semantics and decoding processes for multi-layer video coding extensions, Annex G contains Syntax, semantics and decoding processes for the multiview extensions and Annex H contains Syntax, semantics and decoding processes for the scalable extensions. Modifications in long sections copied from the HEVC spec are highlighted in turquoise. Open issues and editor's notes are highlighted in yellow.

Contents

Page

Integrations	i

Abstract	vii

Contents	viii

Annex F  Common specifications for multi-layer extensions	1

F.1	Scope	1

F.2	Normative references	1

F.3	Definitions	1

F.4	Abbreviations	3

F.5	Conventions	4

F.6	Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships	4

F.7	Syntax and semantics	4

F.7.1	Method of specifying syntax in tabular form	4

F.7.2	Specification of syntax functions, categories, and descriptors	4

F.7.3	Syntax in tabular form	4

F.7.3.1	NAL unit syntax	4

F.7.3.2	Raw byte sequence payloads and RBSP trailing bits syntax	5

F.7.3.3	Profile, tier, and level syntax	19

F.7.3.4	Scaling list data syntax	19

F.7.3.5	Supplemental enhancement information message syntax	19

F.7.3.6	Slice segment header syntax	20

F.7.3.7	Short-term reference picture set syntax	23

F.7.3.8	Slice segment data syntax	23

F.7.4	Semantics	23

F.7.4.1	General	23

F.7.4.2	NAL unit semantics	23

F.7.4.3	Raw byte sequence payloads, trailing bits, and byte alignment semantics	27

F.7.4.4	Profile, tier, and level semantics	52

F.7.4.5	Scaling list data semantics	54

F.7.4.6	Supplemental enhancement information message semantics	54

F.7.4.7	Slice segment header semantics	55

F.7.4.8	Short-term reference picture set semantics	59

F.7.4.9	Slice segment data semantics	59

F.8	Decoding process	60

F.8.1	General decoding process	60

F.8.1.1	General	60

F.8.1.2	CVSG decoding process	60

F.8.1.3	Common decoding process for a coded picture	62

F.8.1.4	Decoding process for a coded picture with nuh_layer_id equal to 0	65

F.8.1.5	Decoding process for starting the decoding of a coded picture with nuh_layer_id greater than 0	65

F.8.1.6	Decoding process for ending the decoding of a coded picture with nuh_layer_id greater than 0	65

F.8.1.7	Decoding process for generating unavailable reference pictures for pictures first in decoding order within a layer	66

F.8.1.8	Initialization process for an external base layer picture	67

F.8.1.9	Decoding process for an external base layer picture	67

F.8.2	NAL unit decoding process	68

F.8.3	Slice decoding processes	68

F.8.3.1	Decoding process for picture order count	68

F.8.3.2	Decoding process for reference picture set	70

F.8.3.3	Decoding process for generating unavailable reference pictures	70

F.8.3.4	Decoding process for reference picture lists construction	70

F.8.4	Decoding process for coding units coded in intra prediction mode	71

F.8.5	Decoding process for coding units coded in inter prediction mode	71

F.8.6	Scaling, transformation and array construction process prior to deblocking filter process	71

F.8.7	In-loop filter process	71

F.9	Parsing process	71

F.10	Specification of bitstream subsets	71

F.10.1	Sub-bitstream extraction process	71

F.10.2	Independent non-base layer rewriting process	72

F.10.3	Sub-bitstream extraction process for additional layer sets	72

F.11	Profiles, tiers, and levels	73

F.11.1	Independent non-base layer decoding capability	73

F.11.2	Decoder capabilities	73

F.12	Byte stream format	74

F.13	Hypothetical reference decoder	74

F.13.1	General	74

F.13.2	Operation of bitstream partition buffer (BPB)	79

F.13.2.1	General	79

F.13.2.2	Timing of decoding unit arrival	79

F.13.2.3	Timing of decoding unit removal and decoding of decoding unit	82

F.13.3	Operation of decoded picture buffer (DPB)	84

F.13.3.1	General	84

F.13.3.2	Removal of pictures from the DPB	85

F.13.3.3	Picture output	86

F.13.3.4	Current decoded picture marking and storage	86

F.13.4	Bitstream conformance	87

F.13.5	Decoder conformance	89

F.13.5.1	General	89

F.13.5.2	Operation of the output order DPB	90

F.13.6	Demultiplexing process for deriving a bitstream partition	92

F.14	Supplemental enhancement information	93

F.14.1	General	93

F.14.2	SEI payload syntax	93

F.14.2.1	General SEI payload syntax	93

F.14.2.2	Annex D SEI message syntax for multi-layer extensions	93

F.14.2.3	Layers not present SEI message syntax	93

F.14.2.4	Inter-layer constrained tile sets SEI message syntax	94

F.14.2.5	Bitstream partition nesting SEI message syntax	94

F.14.2.6	Bitstream partition initial arrival time SEI message syntax	95

F.14.2.7	Sub-bitstream property SEI message syntax	95

F.14.2.8	Alpha channel information SEI message syntax	95

F.14.2.9	Overlay information SEI message syntax	96

F.14.2.10	Temporal motion vector prediction constraints SEI message syntax	97

F.14.2.11	Frame-field information SEI message syntax	97

F.14.3	SEI payload semantics	97

F.14.3.1	General SEI payload semantics	97

F.14.3.2	Annex D SEI message semantics for multi-layer extensions	98

F.14.3.3	Layers not present SEI message semantics	106

F.14.3.4	Inter-layer constrained tile sets SEI message semantics	106

F.14.3.5	Bitstream partition nesting SEI message semantics	108

F.14.3.6	Bitstream partition initial arrival time SEI message semantics	108

F.14.3.7	Sub-bitstream property SEI message semantics	109

F.14.3.8	Alpha channel information SEI message semantics	110

F.14.3.9	Overlay information SEI message semantics	111

F.14.3.10	Temporal motion vector prediction constraints SEI message semantics	113

F.14.3.11	Frame-field information SEI message semantics	114

F.15	Video usability information	115

F.15.1	General	115

F.15.2	VUI syntax	115

F.15.3	VUI semantics	115

F.15.3.1	VUI parameters semantics	115

F.15.3.2	HRD parameters semantics	116

F.15.3.3	Sub-layer HRD parameters semantics	116

Annex G  Multiview high efficiency video coding	117

G.1	Scope	117

G.2	Normative references	117

G.3	Definitions	117

G.4	Abbreviations	117

G.5	Conventions	117

G.6	Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships	117

G.7	Syntax and semantics	117

G.8	Decoding processes	117

G.8.1	General decoding process	117

G.8.1.1	General	117

G.8.1.2	Decoding process for a coded picture with nuh_layer_id greater than 0	117

G.8.1.3	Decoding process for inter-layer reference picture set	118

G.8.2	NAL unit decoding process	118

G.8.3	Slice decoding processes	118

G.8.4	Decoding process for coding units coded in intra prediction mode	118

G.8.5	Decoding process for coding units coded in inter prediction mode	118

G.8.6	Scaling, transformation and array construction process prior to deblocking filter process	118

G.8.7	In-loop filter process	118

G.9	Parsing process	118

G.10	Specification of bitstream subsets	119

G.11	Profiles, tiers, and levels	119

G.11.1	Profiles	119

G.11.1.1	Multiview Main profile	119

G.11.2	Tiers and levels	121

G.11.2.1	General tier and level limits	121

G.11.2.2	Profile-specific tier and level limits for the Multiview Main profile	122

G.11.3	Decoder capabilities	123

G.12	Byte stream format	123

G.13	Hypothetical reference decoder	123

G.14	Supplemental enhancement information	123

G.14.1	General	123

G.14.2	SEI payload syntax	123

G.14.2.1	General SEI payload syntax	123

G.14.2.2	Annex D and Annex F SEI message syntax for multiview high efficiency video coding	123

G.14.2.3	3D reference displays information SEI message syntax	124

G.14.2.4	Depth representation information SEI message syntax	125

G.14.2.5	Multiview scene information SEI message syntax	125

G.14.2.6	Multiview acquisition information SEI message syntax	126

G.14.2.7	Multiview view position SEI message syntax	127

G.14.3	SEI payload semantics	127

G.14.3.1	General SEI payload semantics	127

G.14.3.2	Annex D and Annex F SEI message semantics for multiview high efficiency video coding	127

G.14.3.3	3D reference displays information SEI message semantics	127

G.14.3.4	Depth representation information SEI message semantics	130

G.14.3.5	Multiview scene information SEI message semantics	132

G.14.3.6	Multiview acquisition information SEI message semantics	133

G.14.3.7	Multiview view position SEI message semantics	136

G.15	Video usability information	136

Annex H  Scalable high efficiency video coding	137

H.1	Scope	137

H.2	Normative references	137

H.3	Definitions	137

H.4	Abbreviations	137

H.5	Conventions	137

H.6	Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships	137

H.7	Syntax and semantics	137

H.8	Decoding processes	137

H.8.1	General decoding process	137

H.8.1.1	General	137

H.8.1.2	Decoding process for a coded picture with nuh_layer_id greater than 0	137

H.8.1.3	Decoding process for inter-layer reference picture set	138

H.8.1.4	Derivation process for inter-layer reference pictures	138

H.8.2	NAL unit decoding process	150

H.8.3	Slice decoding processes	150

H.8.3.1	Decoding process for picture order count	150

H.8.3.2	Decoding process for reference picture set	150

H.8.3.3	Decoding process for generating unavailable reference pictures	150

H.8.3.4	Decoding process for reference picture lists construction	150

H.8.4	Decoding process for coding units coded in intra prediction mode	150

H.8.5	Decoding process for coding units coded in inter prediction mode	150

H.8.6	Scaling, transformation and array construction process prior to deblocking filter process	151

H.8.7	In-loop filter process	151

H.9	Parsing process	151

H.10	Specification of bitstream subsets	151

H.11	Profiles, tiers, and levels	151

H.11.1	Profiles	151

H.11.1.1	Scalable Main and Scalable Main 10 profiles	151

H.11.2	Tiers and levels	153

H.11.2.1	General tier and level limits	153

H.11.2.2	Profile specific tier and level limits for the Scalable Main and Scalable Main 10 profiles	155

H.11.3	Decoder capabilities	156

H.12	Byte stream format	156

H.13	Hypothetical reference decoder	156

H.14	Supplemental enhancement information	156

H.15	Video usability information	156

- Annex F

Common specifications for multi-layer extensions

(This annex forms an integral part of this Recommendation | International Standard.)

  - Scope

This annex specifies the common syntax, semantics, and decoding processes for multi-layer video coding extensions, with references made to clauses 2-9 and Annexes A-E, B, and A.

  - Normative references

The list of normative references in clause 2 applies.

  - Definitions

For the purpose of this annex, the following definitions apply in addition to the definitions in the subclauses of clause 3. These definitions are either not present in the subclauses of clause 3 or replace definitions in the subclauses of clause 3.

- access unit: A set of NAL units that are associated with each other according to a specified classification rule, are consecutive in decoding order, and contain at most one coded picture with any specific value of nuh_layer_id.

- additional layer set: A set of layers of a bitstream with a set of layers of one or more non-base layer subtrees.

- alternative output layer: A layer that is a reference layer of an output layer and which may include a picture that may be output when no picture of the output layer is present in the access unit containing the picture.

- associated IRAP picture: The previous IRAP picture in decoding order within the same layer (if present).

- auxiliary picture: A picture that has no normative effect on the decoding process of primary pictures, and with a nuh_layer_id value such that AuxId[ nuh_layer_id ] is greater than 0.

- auxiliary picture layer: A layer with a nuh_layer_id value such that AuxId[ nuh_layer_id ] is greater than 0.

- base bitstream partition: A bitstream partition that contains the layer with nuh_layer_id equal to SmallestLayerId.

- bitstream partition: A sequence of bits, in the form of a NAL unit stream or a byte stream, that is a subset of a bitstream according to a partitioning scheme.

- broken link access (BLA) access unit: An access unit in which the coded picture with nuh_layer_id equal to SmallestLayerId is a BLA picture.

- clean random access (CRA) access unit: An access unit in which the coded picture with nuh_layer_id equal to SmallestLayerId is a CRA picture.

- coded layer-wise video sequence (CLVS): A sequence of coded pictures, with the same nuh_layer_id value nuhLayerId, that consists, in decoding order, of an IRAP picture with NoRaslOutputFlag equal to 1 or a picture with FirstPicInLayerDecodedFlag[ nuhLayerId ] equal to 0, followed by all coded pictures, if any, up to but excluding the next IRAP picture with NoRaslOutputFlag equal to 1 or the next picture with FirstPicInLayerDecodedFlag[ nuhLayerId ] equal to 0.

- coded picture: A coded representation of a picture comprising VCL NAL units with a particular value of nuh_layer_id within an access unit and containing all coding tree units of the picture.

- coded picture buffer (CPB): A first-in first-out buffer containing decoding units in decoding order specified in the hypothetical reference decoder in Annex C or in clause A.13.

- coded video sequence (CVS): A sequence of access units that consists, in decoding order, of an initial IRAP access unit, followed by zero or more access units that are not initial IRAP access units, including all subsequent access units up to but not including any subsequent access unit that is an initial IRAP access unit.

- collocated sample: A reference picture sample located at a corresponding position to the current sample as determined through the resampling process, for use in inter-layer prediction.

- cross-layer random access skip (CL-RAS) picture: A picture with nuh_layer_id equal to layerId such that LayerInitializedFlag[ layerId ] is equal to 0 when the decoding process for starting the decoding of a coded picture with nuh_layer_id greater than 0 is invoked.

- decoded picture buffer (DPB): A buffer holding decoded pictures for reference, output reordering, or output delay specified for the hypothetical reference decoder in Annex C or in clause A.13.

- decoding unit: A partition unit if SubPicHrdFlag is equal to 0 or a subset of a partition unit otherwise, consisting of one or more VCL NAL units in a partition unit and the associated non-VCL NAL units.

- direct predicted layer: A layer for which another layer is a direct reference layer.

- direct reference layer: A layer that may be used for inter-layer prediction of another layer.

- direct reference layer picture: A picture in a direct reference layer that is used as input to derive an inter-layer reference picture for inter-layer prediction of the current picture and is in the same access unit as the current picture.

- independent layer: A layer that does not have direct reference layers.

- indirect predicted layer: A layer for which another layer is an indirect reference layer.

- indirect reference layer: A layer that is not a direct reference layer of a second layer but is a direct reference layer of a third layer that is a reference layer of the second layer.

- initial intra random access point (IRAP) access unit: An IRAP access unit in which the coded picture with nuh_layer_id equal to SmallestLayerId has NoRaslOutputFlag equal to 1.

- inter-layer prediction: A prediction in a manner that is dependent on data elements (e.g., sample values or motion vectors) of reference pictures with a different value of nuh_layer_id than that of the current picture.

- inter-layer reference picture: A reference picture that may be used for inter-layer prediction of the current picture and is in the same access unit as the current picture.

- intra random access point (IRAP) access unit: An access unit in which the coded picture with nuh_layer_id equal to SmallestLayerId is an IRAP picture.

- intra random access point (IRAP) picture: A coded picture for which each VCL NAL unit has nal_unit_type in the range of BLA_W_LP to RSV_IRAP_VCL23, inclusive.

NOTE – An IRAP picture belonging to an independent layer contains only I slices. An IRAP picture belonging to a predicted layer with nuh_layer_id value currLayerId may contain P, B, and I slices, cannot use inter prediction from other pictures with nuh_layer_id equal to currLayerId, and may use inter-layer prediction from its direct reference layers. The IRAP picture belonging to a predicted layer with nuh_layer_id value currLayerId and all subsequent non-RASL pictures with nuh_layer_id equal to currLayerId in decoding order can be correctly decoded without performing the decoding process of any pictures with nuh_layer_id equal to currLayerId that precede the IRAP picture in decoding order, when the necessary parameter sets are available when they need to be activated and LayerInitializedFlag[ refLayerId ] is equal to 1 for refLayerId equal to all nuh_layer_id values of the direct reference layers of the layer with nuh_layer_id equal to currLayerId.

- layer subtree: A subset of the layers of a layer tree including all the reference layers of the layers within the subset.

- layer tree: A set of layers such that each layer in the set of layers is a predicted layer or a reference layer of at least one other layer in the set of layers and no layer outside the set of layers is a predicted layer or a reference layer of any layer in the set of layers.

- leading picture: A picture that is in the same layer as the associated IRAP picture and precedes the associated IRAP picture in output order.

- necessary layer: A layer in an output operation point associated with an OLS, the layer being an output layer of the OLS, or a reference layer of an output layer of the OLS.

- non-base layer: A layer in which all VCL NAL units have the same nuh_layer_id value greater than 0.

- non-base layer subtree: A layer subtree that does not include the base layer.

- output layer: A layer of an output layer set that is output when TargetOlsIdx is equal to the index of the output layer set.

- output layer set (OLS): A set of layers consisting of the layers of one of the specified layer sets, where one or more layers in the set of layers are indicated to be output layers.

- output operation point: A bitstream that is created from an input bitstream by operation of the sub-bitstream extraction process with the input bitstream, a target highest TemporalId, and a target layer identifier list as inputs, and that is associated with a set of output layers.

- output order: The order in which the decoded pictures are output from the decoded picture buffer (for the decoded pictures that are to be output from the decoded picture buffer), or the order in which the access units are considered to be output from the decoded picture buffer (for the access units the decoding of which results into decoded pictures that are to be output from the decoded picture buffer).

- partitioning scheme: A partitioning of layers in an OLS into one or more bitstream partitions such that each layer in the OLS is included in exactly one bitstream partition of the partitioning scheme and each bitstream partition of the partitioning scheme contains one or more layers.

- partition unit: A subset of access unit consisting of the picture units of the layers in a bitstream partition.

- picture order count (POC): A variable that is associated with each picture and that, at any moment, uniquely identifies the associated picture among all pictures with the same value of nuh_layer_id in the CVS, and, when the associated picture is to be output from the decoded picture buffer, indicates the position of the associated picture in output order relative to the output order positions of the other pictures with the same value of nuh_layer_id in the same CVS that are to be output from the decoded picture buffer.

- picture order count (POC) resetting period: A sequence of access units in decoding order, starting with an access unit with poc_reset_idc equal to 1 or 2 and a particular value of poc_reset_period_id and including all access units that either have the same value of poc_reset_period_id or have poc_reset_idc equal to 0.

- picture order count (POC) resetting picture: A picture that is the first picture, in decoding order, of a layer of a POC resetting period.

- predicted layer: A layer that is a direct predicted layer or an indirect predicted layer of another layer.

- primary picture: A picture with nuh_layer_id value such that AuxId[ nuh_layer_id ] is equal to 0.

- primary picture layer: A layer with a nuh_layer_id value such that AuxId[ nuh_layer_id ] is equal to 0.

- quality scalability: A type of scalability, represented by a scalability dimension, that indicates that the coded pictures in all layers of a bitstream have the same values of luma width, luma height, chroma width, and chroma height, but may have different fidelity level or different values of luma or chroma bit depths.

- reference layer: A layer that is a direct reference layer or an indirect reference layer of another layer.

- reference picture list: A list of reference pictures that is used for inter prediction or inter-layer prediction of a P or B slice.

- scalability dimension: An indication of the type of non-base layers which may be present in a CVS.

- source picture for inter-layer prediction: A decoded picture that either is, or is used in deriving, an inter-layer reference picture that is included in an inter-layer reference picture set of the current picture.

- spatial scalability: A type of scalability, represented by a scalability dimension, that indicates that the coded pictures in different layers of a bitstream have different values of luma or chroma width or height.

- sub-bitstream extraction process: A specified process by which NAL units in a bitstream that do not belong to a target set, determined by a target highest TemporalId and a target layer identifier list, are removed from the bitstream, with the output sub-bitstream consisting of the NAL units in the bitstream that belong to the target set.

NOTE – Depending on whether the target layer identifier list includes nuh_layer_id equal to 0 and on the value of vps_base_layer_internal_flag, the sub-bitstream extraction process may refer to the process specified in clause 10, clause A.10.1, or clause A.10.3.

- target output layer set: The output layer set for which the index is equal to TargetOlsIdx.

- trailing picture: A picture that is in the same layer as the associated IRAP picture and follows the associated IRAP picture in output order.

- view: A sequence of pictures associated with the same value of ViewOrderIdx.

NOTE – A view typically represents a sequence of pictures captured by one camera.

  - Abbreviations

For the purpose of this annex, the following abbreviations apply in addition to the abbreviations in clause 4:

BPB	Bitstream Partition Buffer

HBPS	Hypothetical Bitstream Partition Scheduler

OLS	Output Layer Set

  - Conventions

The specifications in clause 5 and its subclauses apply.

  - Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships

The specifications in clause 6 and its subclauses apply.

  - Syntax and semantics

    - Method of specifying syntax in tabular form

The specifications in clause 7.1 apply.

    - Specification of syntax functions, categories, and descriptors

The specifications in clause 7.2 apply, with the following additions:

more_data_in_slice_segment_header_extension( ) is specified as follows:

–	If ( the current position in the slice_segment_header( ) syntax structure ) − ( the position immediately following slice_segment_header_extension_length ) is less than ( slice_segment_header_extension_length * 8 ), the return value of more_data_in_slice_segment_header_extension( ) is equal to TRUE.

–	Otherwise, the return value of more_data_in_slice_segment_header_extension( ) is equal to FALSE.

    - Syntax in tabular form

      - NAL unit syntax

        - General NAL unit syntax

The specifications in clause 7.3.1.1 apply.

        - NAL unit header syntax

The specifications in clause 7.3.1.2 apply.

      - Raw byte sequence payloads and RBSP trailing bits syntax

        - Video parameter set RBSP

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_video_parameter_set_id | u(4) |
| vps_base_layer_internal_flag | u(1) |
| vps_base_layer_available_flag | u(1) |
| vps_max_layers_minus1 | u(6) |
| vps_max_sub_layers_minus1 | u(3) |
| vps_temporal_id_nesting_flag | u(1) |
| vps_reserved_0xffff_16bits | u(16) |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| vps_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( vps_sub_layer_ordering_info_present_flag ? 0 : vps_max_sub_layers_minus1 ); i <= vps_max_sub_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering_minus1[ i ] | ue(v) |
| vps_max_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase_plus1[ i ] | ue(v) |
| } |  |
| vps_max_layer_id | u(6) |
| vps_num_layer_sets_minus1 | ue(v) |
| for( i = 1; i <= vps_num_layer_sets_minus1; i++ ) |  |
| for( j = 0; j <= vps_max_layer_id; j++ ) |  |
| layer_id_included_flag[ i ][ j ] | u(1) |
| vps_timing_info_present_flag | u(1) |
| if( vps_timing_info_present_flag ) { |  |
| vps_num_units_in_tick | u(32) |
| vps_time_scale | u(32) |
| vps_poc_proportional_to_timing_flag | u(1) |
| if( vps_poc_proportional_to_timing_flag ) |  |
| vps_num_ticks_poc_diff_one_minus1 | ue(v) |
| vps_num_hrd_parameters | ue(v) |
| for( i = 0; i < vps_num_hrd_parameters; i++ ) { |  |
| hrd_layer_set_idx[ i ] | ue(v) |
| if( i > 0 ) |  |
| cprms_present_flag[ i ] | u(1) |
| hrd_parameters( cprms_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| vps_extension_alignment_bit_equal_to_one | u(1) |
| vps_extension( ) |  |
| vps_extension2_flag | u(1) |
| if( vps_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

          - Video parameter set extension syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| if( vps_max_layers_minus1 > 0 && vps_base_layer_internal_flag ) |  |
| profile_tier_level( 0, vps_max_sub_layers_minus1 ) |  |
| splitting_flag | u(1) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask_flag[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask_flag[ i ] |  |
| } |  |
| for( j = 0; j < ( NumScalabilityTypes − splitting_flag ); j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 1; i <= MaxLayersMinus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| if( !splitting_flag ) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| view_id_len | u(4) |
| if( view_id_len > 0 ) |  |
| for( i = 0; i < NumViews; i++ ) |  |
| view_id_val[ i ] | u(v) |
| for( i = 1; i <= MaxLayersMinus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| if( NumIndependentLayers > 1 ) |  |
| num_add_layer_sets | ue(v) |
| for( i = 0; i < num_add_layer_sets; i++ ) |  |
| for( j = 1; j < NumIndependentLayers; j++ ) |  |
| highest_layer_idx_plus1[ i ][ j ] | u(v) |
| vps_sub_layers_max_minus1_present_flag | u(1) |
| if( vps_sub_layers_max_minus1_present_flag ) |  |
| for( i = 0; i <= MaxLayersMinus1; i++ ) |  |
| sub_layers_vps_max_minus1[ i ] | u(3) |
| max_tid_ref_present_flag | u(1) |
| if( max_tid_ref_present_flag ) |  |
| for( i = 0; i < MaxLayersMinus1; i++ ) |  |
| for( j = i + 1; j <= MaxLayersMinus1; j++ ) |  |
| if( direct_dependency_flag[ j ][ i ] ) |  |
| max_tid_il_ref_pics_plus1[ i ][ j ] | u(3) |
| default_ref_layers_active_flag | u(1) |
| vps_num_profile_tier_level_minus1 | ue(v) |
| for( i = vps_base_layer_internal_flag ? 2 : 1; i <= vps_num_profile_tier_level_minus1; i++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| if( NumLayerSets > 1 ) { |  |
| num_add_olss | ue(v) |
| default_output_layer_idc | u(2) |
| } |  |
| NumOutputLayerSets = num_add_olss + NumLayerSets |  |
| for( i = 1; i < NumOutputLayerSets; i++ ) { |  |
| if( NumLayerSets > 2 && i >= NumLayerSets ) |  |
| layer_set_idx_for_ols_minus1[ i ] | u(v) |
| if( i > vps_num_layer_sets_minus1 ¦ ¦ defaultOutputLayerIdc = = 2 ) |  |
| for( j = 0; j < NumLayersInIdList[ OlsIdxToLsIdx[ i ] ]; j++ ) |  |
| output_layer_flag[ i ][ j ] | u(1) |
| for( j = 0; j < NumLayersInIdList[ OlsIdxToLsIdx[ i ] ]; j++ ) |  |
| if( NecessaryLayerFlag[ i ][ j ] && vps_num_profile_tier_level_minus1 > 0 ) |  |
| profile_tier_level_idx[ i ][ j ] | u(v) |
| if( NumOutputLayersInOutputLayerSet[ i ] = = 1 && NumDirectRefLayers[ OlsHighestOutputLayerId[ i ] ] > 0 ) |  |
| alt_output_layer_flag[ i ] | u(1) |
| } |  |
| vps_num_rep_formats_minus1 | ue(v) |
| for( i = 0; i <= vps_num_rep_formats_minus1; i++ ) |  |
| rep_format( ) |  |
| if( vps_num_rep_formats_minus1 > 0 ) |  |
| rep_format_idx_present_flag | u(1) |
| if( rep_format_idx_present_flag ) |  |
| for( i = vps_base_layer_internal_flag ? 1 : 0; i <= MaxLayersMinus1; i++ ) |  |
| vps_rep_format_idx[ i ] | u(v) |
| max_one_active_ref_layer_flag | u(1) |
| vps_poc_lsb_aligned_flag | u(1) |
| for( i = 1; i <= MaxLayersMinus1; i++ ) |  |
| if( NumDirectRefLayers[ layer_id_in_nuh[ i ] ] = = 0 ) |  |
| poc_lsb_not_present_flag[ i ] | u(1) |
| dpb_size( ) |  |
| direct_dep_type_len_minus2 | ue(v) |
| direct_dependency_all_layers_flag | u(1) |
| if( direct_dependency_all_layers_flag ) |  |
| direct_dependency_all_layers_type | u(v) |
| else { |  |
| for( i = vps_base_layer_internal_flag ? 1 : 2; i <= MaxLayersMinus1; i++ ) |  |
| for( j = vps_base_layer_internal_flag ? 0 : 1; j < i; j++ ) |  |
| if( direct_dependency_flag[ i ][ j ] ) |  |
| direct_dependency_type[ i ][ j ] | u(v) |
| } |  |
| vps_non_vui_extension_length | ue(v) |
| for( i = 1; i <= vps_non_vui_extension_length; i++ ) |  |
| vps_non_vui_extension_data_byte | u(8) |
| vps_vui_present_flag | u(1) |
| if( vps_vui_present_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| vps_vui_alignment_bit_equal_to_one | u(1) |
| vps_vui( ) |  |
| } |  |
| } |  |

          - Representation format syntax

| rep_format( ) { | Descriptor |
| --- | --- |
| pic_width_vps_in_luma_samples | u(16) |
| pic_height_vps_in_luma_samples | u(16) |
| chroma_and_bit_depth_vps_present_flag | u(1) |
| if( chroma_and_bit_depth_vps_present_flag ) { |  |
| chroma_format_vps_idc | u(2) |
| if( chroma_format_vps_idc = = 3 ) |  |
| separate_colour_plane_vps_flag | u(1) |
| bit_depth_vps_luma_minus8 | u(4) |
| bit_depth_vps_chroma_minus8 | u(4) |
| } |  |
| conformance_window_vps_flag | u(1) |
| if( conformance_window_vps_flag ) { |  |
| conf_win_vps_left_offset | ue(v) |
| conf_win_vps_right_offset | ue(v) |
| conf_win_vps_top_offset | ue(v) |
| conf_win_vps_bottom_offset | ue(v) |
| } |  |
| } |  |

          - DPB size syntax

| dpb_size( ) { |  |
| --- | --- |
| for( i = 1; i < NumOutputLayerSets; i++ ) { |  |
| currLsIdx = OlsIdxToLsIdx[ i ] |  |
| sub_layer_flag_info_present_flag[ i ] | u(1) |
| for( j = 0; j <= MaxSubLayersInLayerSetMinus1[ currLsIdx ]; j++ ) { |  |
| if( j > 0 && sub_layer_flag_info_present_flag[ i ] ) |  |
| sub_layer_dpb_info_present_flag[ i ][ j ] | u(1) |
| if( sub_layer_dpb_info_present_flag[ i ][ j ] ) { |  |
| for( k = 0; k < NumLayersInIdList[ currLsIdx ]; k++ ) |  |
| if( NecessaryLayerFlag[ i ][ k ] && ( vps_base_layer_internal_flag ¦ ¦ ( LayerSetLayerIdList[ currLsIdx ][ k ] != 0 ) ) ) |  |
| max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] | ue(v) |
| max_vps_num_reorder_pics[ i ][ j ] | ue(v) |
| max_vps_latency_increase_plus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |

          - VPS VUI syntax

| vps_vui( ) { | Descriptor |
| --- | --- |
| cross_layer_pic_type_aligned_flag | u(1) |
| if( !cross_layer_pic_type_aligned_flag ) |  |
| cross_layer_irap_aligned_flag | u(1) |
| if( cross_layer_irap_aligned_flag ) |  |
| all_layers_idr_aligned_flag | u(1) |
| bit_rate_present_vps_flag | u(1) |
| pic_rate_present_vps_flag | u(1) |
| if( bit_rate_present_vps_flag ¦ ¦ pic_rate_present_vps_flag ) |  |
| for( i = vps_base_layer_internal_flag ? 0 : 1; i < NumLayerSets; i++ ) |  |
| for( j = 0; j <= MaxSubLayersInLayerSetMinus1[ i ]; j++ ) { |  |
| if( bit_rate_present_vps_flag ) |  |
| bit_rate_present_flag[ i ][ j ] | u(1) |
| if( pic_rate_present_vps_flag ) |  |
| pic_rate_present_flag[ i ][ j ] | u(1) |
| if( bit_rate_present_flag[ i ][ j ] ) { |  |
| avg_bit_rate[ i ][ j ] | u(16) |
| max_bit_rate[ i ][ j ] | u(16) |
| } |  |
| if( pic_rate_present_flag[ i ][ j ] ) { |  |
| constant_pic_rate_idc[ i ][ j ] | u(2) |
| avg_pic_rate[ i ][ j ] | u(16) |
| } |  |
| } |  |
| video_signal_info_idx_present_flag | u(1) |
| if( video_signal_info_idx_present_flag ) |  |
| vps_num_video_signal_info_minus1 | u(4) |
| for( i = 0; i <= vps_num_video_signal_info_minus1; i++ ) |  |
| video_signal_info( ) |  |
| if( video_signal_info_idx_present_flag && vps_num_video_signal_info_minus1 > 0 ) |  |
| for( i = vps_base_layer_internal_flag ? 0 : 1; i <= MaxLayersMinus1; i++ ) |  |
| vps_video_signal_info_idx[ i ] | u(4) |
| tiles_not_in_use_flag | u(1) |
| if( !tiles_not_in_use_flag ) { |  |
| for( i = vps_base_layer_internal_flag ? 0 : 1; i <= MaxLayersMinus1; i++ ) { |  |
| tiles_in_use_flag[ i ] | u(1) |
| if( tiles_in_use_flag[ i ] ) |  |
| loop_filter_not_across_tiles_flag[ i ] | u(1) |
| } |  |
| for( i = vps_base_layer_internal_flag ? 1 : 2; i <= MaxLayersMinus1; i++ ) |  |
| for( j = 0; j < NumDirectRefLayers[ layer_id_in_nuh[ i ] ]; j++ ) { |  |
| layerIdx = LayerIdxInVps[ IdDirectRefLayer[ layer_id_in_nuh[ i ] ][ j ] ] |  |
| if( tiles_in_use_flag[ i ] && tiles_in_use_flag[ layerIdx ] ) |  |
| tile_boundaries_aligned_flag[ i ][ j ] | u(1) |
| } |  |
| } |  |
| wpp_not_in_use_flag | u(1) |
| if( !wpp_not_in_use_flag ) |  |
| for( i = vps_base_layer_internal_flag ? 0 : 1; i <= MaxLayersMinus1; i++ ) |  |
| wpp_in_use_flag[ i ] | u(1) |
| single_layer_for_non_irap_flag | u(1) |
| higher_layer_irap_skip_flag | u(1) |
| ilp_restricted_ref_layers_flag | u(1) |
| if( ilp_restricted_ref_layers_flag ) |  |
| for( i = 1; i <= MaxLayersMinus1; i++ ) |  |
| for( j = 0; j < NumDirectRefLayers[ layer_id_in_nuh[ i ] ]; j++ ) |  |
| if( vps_base_layer_internal_flag ¦ ¦ IdDirectRefLayer[ layer_id_in_nuh[ i ] ][ j ] > 0 ) { |  |
| min_spatial_segment_offset_plus1[ i ][ j ] | ue(v) |
| if( min_spatial_segment_offset_plus1[ i ][ j ] > 0 ) { |  |
| ctu_based_offset_enabled_flag[ i ][ j ] | u(1) |
| if( ctu_based_offset_enabled_flag[ i ][ j ] ) |  |
| min_horizontal_ctu_offset_plus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| vps_vui_bsp_hrd_present_flag | u(1) |
| if( vps_vui_bsp_hrd_present_flag ) |  |
| vps_vui_bsp_hrd_params( ) |  |
| for( i = 1; i <= MaxLayersMinus1; i++ ) |  |
| if( NumDirectRefLayers[ layer_id_in_nuh[ i ] ] = = 0 ) |  |
| base_layer_parameter_set_compatibility_flag[ i ] | u(1) |
| } |  |

          - Video signal info syntax

| video_signal_info( ) { | Descriptor |
| --- | --- |
| video_vps_format | u(3) |
| video_full_range_vps_flag | u(1) |
| colour_primaries_vps | u(8) |
| transfer_characteristics_vps | u(8) |
| matrix_coeffs_vps | u(8) |
| } |  |

          - VPS VUI bitstream partition HRD parameters syntax

| vps_vui_bsp_hrd_params( ) { | Descriptor |
| --- | --- |
| vps_num_add_hrd_params | ue(v) |
| for( i = vps_num_hrd_parameters; i < vps_num_hrd_parameters + vps_num_add_hrd_params; i++ ) { |  |
| if( i > 0 ) |  |
| cprms_add_present_flag[ i ] | u(1) |
| num_sub_layer_hrd_minus1[ i ] | ue(v) |
| hrd_parameters( cprms_add_present_flag[ i ], num_sub_layer_hrd_minus1[ i ] ) |  |
| } |  |
| if( vps_num_hrd_parameters + vps_num_add_hrd_params > 0 ) |  |
| for( h = 1; h < NumOutputLayerSets; h++ ) { |  |
| num_signalled_partitioning_schemes[ h ] | ue(v) |
| for( j = 1; j < num_signalled_partitioning_schemes[ h ] + 1; j++ ) { |  |
| num_partitions_in_scheme_minus1[ h ][ j ] | ue(v) |
| for( k = 0; k <= num_partitions_in_scheme_minus1[ h ][ j ]; k++ ) |  |
| for( r = 0; r < NumLayersInIdList[ OlsIdxToLsIdx[ h ] ]; r++ ) |  |
| layer_included_in_partition_flag[ h ][ j ][ k ][ r ] | u(1) |
| } |  |
| for( i = 0; i < num_signalled_partitioning_schemes[ h ] + 1; i++ ) |  |
| for( t = 0; t <= MaxSubLayersInLayerSetMinus1[ OlsIdxToLsIdx[ h ] ]; t++ ) { |  |
| num_bsp_schedules_minus1[ h ][ i ][ t ] | ue(v) |
| for( j = 0; j <= num_bsp_schedules_minus1[ h ][ i ][ t ]; j++ ) |  |
| for( k = 0; k <= num_partitions_in_scheme_minus1[ h ][ i ]; k++ ) { |  |
| if( vps_num_hrd_parameters + vps_num_add_hrd_params > 1 ) |  |
| bsp_hrd_idx[ h ][ i ][ t ][ j ][ k ] | u(v) |
| bsp_sched_idx[ h ][ i ][ t ][ j ][ k ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |

        - Sequence parameter set RBSP syntax

          - General sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| if( nuh_layer_id = = 0 ) |  |
| sps_max_sub_layers_minus1 | u(3) |
| else |  |
| sps_ext_or_max_sub_layers_minus1 | u(3) |
| MultiLayerExtSpsFlag = ( nuh_layer_id != 0 && sps_ext_or_max_sub_layers_minus1 = = 7 ) |  |
| if( !MultiLayerExtSpsFlag ) { |  |
| sps_temporal_id_nesting_flag | u(1) |
| profile_tier_level( 1, sps_max_sub_layers_minus1 ) |  |
| } |  |
| sps_seq_parameter_set_id | ue(v) |
| if( MultiLayerExtSpsFlag ) { |  |
| update_rep_format_flag | u(1) |
| if( update_rep_format_flag ) |  |
| sps_rep_format_idx | u(8) |
| } else { |  |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| conformance_window_flag | u(1) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| } |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| if( !MultiLayerExtSpsFlag ) { |  |
| sps_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( sps_sub_layer_ordering_info_present_flag ? 0 : sps_max_sub_layers_minus1 ); i <= sps_max_sub_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering_minus1[ i ] | ue(v) |
| sps_max_num_reorder_pics[ i ] | ue(v) |
| sps_max_latency_increase_plus1[ i ] | ue(v) |
| } |  |
| } |  |
| log2_min_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_luma_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enabled_flag | u(1) |
| if( scaling_list_enabled_flag ) { |  |
| if( MultiLayerExtSpsFlag ) |  |
| sps_infer_scaling_list_flag | u(1) |
| if( sps_infer_scaling_list_flag ) |  |
| sps_scaling_list_ref_layer_id | u(6) |
| else { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| scaling_list_data( ) |  |
| } |  |
| } |  |
| amp_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| log2_min_pcm_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_luma_coding_block_size | ue(v) |
| pcm_loop_filter_disabled_flag | u(1) |
| } |  |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++ ) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_sps; i++ ) { |  |
| lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_temporal_mvp_enabled_flag | u(1) |
| strong_intra_smoothing_enabled_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_present_flag | u(1) |
| if( sps_extension_present_flag ) { |  |
| sps_range_extensions_flag | u(1) |
| sps_multilayer_extension_flag | u(1) |
| sps_extension_6bits | u(6) |
| } |  |
| if( sps_range_extensions_flag ) |  |
| sps_range_extensions( ) |  |
| if( sps_multilayer_extension_flag ) |  |
| sps_multilayer_extension( ) |  |
| if( sps_extension_6bits ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

          - Sequence parameter set range extensions syntax

The specifications in clause 7.3.2.2.2 apply.

          - Sequence parameter set multilayer extension syntax

| sps_multilayer_extension( ) { | Descriptor |
| --- | --- |
| inter_view_mv_vert_constraint_flag | u(1) |
| } |  |

        - Picture parameter set RBSP syntax

          - General picture parameter set RBSP syntax

The specifications in clause 7.3.2.3.1 apply.

          - Picture parameter set range extensions syntax

The specifications in clause 7.3.2.3.2 apply.

          - Picture parameter set multilayer extension syntax

| pps_multilayer_extension( ) { | Descriptor |
| --- | --- |
| poc_reset_info_present_flag | u(1) |
| pps_infer_scaling_list_flag | u(1) |
| if( pps_infer_scaling_list_flag ) |  |
| pps_scaling_list_ref_layer_id | u(6) |
| num_ref_loc_offsets | ue(v) |
| for( i = 0; i < num_ref_loc_offsets; i++ ) { |  |
| ref_loc_offset_layer_id[ i ] | u(6) |
| scaled_ref_layer_offset_present_flag[ i ] | u(1) |
| if( scaled_ref_layer_offset_present_flag[ i ] ) { |  |
| scaled_ref_layer_left_offset[ ref_loc_offset_layer_id[ i ] ] | se(v) |
| scaled_ref_layer_top_offset[ ref_loc_offset_layer_id[ i ] ] | se(v) |
| scaled_ref_layer_right_offset[ ref_loc_offset_layer_id[ i ] ] | se(v) |
| scaled_ref_layer_bottom_offset[ ref_loc_offset_layer_id[ i ] ] | se(v) |
| } |  |
| ref_region_offset_present_flag[ i ] | u(1) |
| if( ref_region_offset_present_flag[ i ] ) { |  |
| ref_region_left_offset[ ref_loc_offset_layer_id[ i ] ] | se(v) |
| ref_region_top_offset[ ref_loc_offset_layer_id[ i ] ] | se(v) |
| ref_region_right_offset[ ref_loc_offset_layer_id[ i ] ] | se(v) |
| ref_region_bottom_offset[ ref_loc_offset_layer_id[ i ] ] | se(v) |
| } |  |
| resample_phase_set_present_flag[ i ] | u(1) |
| if( resample_phase_set_prsent_flag[ i ] ) { |  |
| phase_hor_luma[ ref_loc_offset_layer_id[ i ] ] | ue(v) |
| phase_ver_luma[ ref_loc_offset_layer_id[ i ] ] | ue(v) |
| phase_hor_chroma_plus8[ ref_loc_offset_layer_id[ i ] ] | ue(v) |
| phase_ver_chroma_plus8[ ref_loc_offset_layer_id[ i ] ] | ue(v) |
| } |  |
| } |  |
| colour_mapping_enabled_flag | u(1) |
| if( colour_mapping_enabled_flag ) |  |
| colour_mapping_table( ) |  |
| } |  |

          - General colour mapping table syntax

| colour_mapping_table( ) { | Descriptor |
| --- | --- |
| num_cm_ref_layers_minus1 | ue(v) |
| for( i = 0; i <= num_cm_ref_layers_minus1; i++ ) |  |
| cm_ref_layer_id[ i ] | u(6) |
| cm_octant_depth | u(2) |
| cm_y_part_num_log2 | u(2) |
| luma_bit_depth_cm_input_minus8 | ue(v) |
| chroma_bit_depth_cm_input_minus8 | ue(v) |
| luma_bit_depth_cm_output_minus8 | ue(v) |
| chroma_bit_depth_cm_output_minus8 | ue(v) |
| cm_res_quant_bits | u(2) |
| cm_delta_flc_bits_minus1 | u(2) |
| if( cm_octant_depth = = 1 ) { |  |
| cm_adapt_threshold_u_delta | se(v) |
| cm_adapt_threshold_v_delta | se(v) |
| } |  |
| colour_mapping_octants( 0, 0, 0, 0, 1 << cm_octant_depth ) |  |
| } |  |

          - Colour mapping octants syntax

| colour_mapping_octants( inpDepth, idxY, idxCb, idxCr, inpLength ) { | Descriptor |
| --- | --- |
| if( inpDepth < cm_octant_depth ) |  |
| split_octant_flag | u(1) |
| if( split_octant_flag ) |  |
| for( k = 0; k < 2; k++ ) |  |
| for( m = 0; m < 2; m++ ) |  |
| for( n = 0; n < 2; n++ ) |  |
| colour_mapping_octants( inpDepth + 1, idxY + PartNumY * k * inpLength / 2, idxCb + m * inpLength / 2, idxCr + n * inpLength / 2, inpLength / 2 ) |  |
| else |  |
| for( i = 0; i < PartNumY; i++ ) { |  |
| idxShiftY = idxY + ( ( i << ( cm_octant_depth − inpDepth ) ) |  |
| for( j = 0; j < 4; j++ ) { |  |
| coded_res_flag[ idxShiftY ][ idxCb ][ idxCr ][ j ] | u(1) |
| if( coded_res_flag[ idxShiftY ][ idxCb ][ idxCr ][ j ] ) |  |
| for( c = 0; c < 3; c++ ) { |  |
| res_coeff_q[ idxShiftY ][ idxCb ][ idxCr ][ j ][ c ] | ue(v) |
| res_coeff_r[ idxShiftY ][ idxCb ][ idxCr ][ j ][ c ] | u(v) |
| if( res_coeff_q[ idxShiftY ][ idxCb ][ idxCr ][ j ][ c ] ¦ ¦ res_coeff_r[ idxShiftY ][ idxCb ][ idxCr ][ j ][ c ] ) |  |
| res_coeff_s[ idxShiftY ][ idxCb ][ idxCr ][ j ][ c ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |

        - Supplemental enhancement information RBSP syntax

The specifications in clause 7.3.2.4 apply.

        - Access unit delimiter RBSP syntax

The specifications in clause 7.3.2.5 apply.

        - End of sequence RBSP syntax

The specifications in clause 7.3.2.6 apply.

        - End of bitstream RBSP syntax

The specifications in clause 7.3.2.7 apply.

        - Filler data RBSP syntax

The specifications in clause 7.3.2.8 apply.

        - Slice segment layer RBSP syntax

The specifications in clause 7.3.2.9 apply.

        - RBSP slice segment trailing bits syntax

The specifications in clause 7.3.2.10 apply.

        - RBSP trailing bits syntax

The specifications in clause 7.3.2.11 apply.

        - Byte alignment syntax

The specifications in clause 7.3.2.12 apply.

      - Profile, tier, and level syntax

The specifications in clause 7.3.3 apply.

      - Scaling list data syntax

The specifications in clause 7.3.4 apply.

      - Supplemental enhancement information message syntax

The specifications in clause 7.3.5 apply.

      - Slice segment header syntax

        - General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| if( nal_unit_type >= BLA_W_LP && nal_unit_type <= RSV_IRAP_VCL23 ) |  |
| no_output_of_prior_pics_flag | u(1) |
| slice_pic_parameter_set_id | ue(v) |
| if( !first_slice_segment_in_pic_flag ) { |  |
| if( dependent_slice_segments_enabled_flag ) |  |
| dependent_slice_segment_flag | u(1) |
| slice_segment_address | u(v) |
| } |  |
| if( !dependent_slice_segment_flag ) { |  |
| i = 0 |  |
| if( num_extra_slice_header_bits > i ) { |  |
| i++ |  |
| discardable_flag | u(1) |
| } |  |
| if( num_extra_slice_header_bits > i ) { |  |
| i++ |  |
| cross_layer_bla_flag | u(1) |
| } |  |
| for( ; i < num_extra_slice_header_bits; i++ ) |  |
| slice_reserved_flag[ i ] | u(1) |
| slice_type | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( ( nuh_layer_id > 0 && !poc_lsb_not_present_flag[ LayerIdxInVps[ nuh_layer_id ] ] ) ¦ ¦ ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ) |  |
| slice_pic_order_cnt_lsb | u(v) |
| if( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) { |  |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else if( num_short_term_ref_pic_sets > 1 ) |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| if( num_long_term_ref_pics_sps > 0 ) |  |
| num_long_term_sps | ue(v) |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ ) { |  |
| if( i < num_long_term_sps ) { |  |
| if( num_long_term_ref_pics_sps > 1 ) |  |
| lt_idx_sps[ i ] | u(v) |
| } else { |  |
| poc_lsb_lt[ i ] | u(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| } |  |
| } |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| } |  |
| if( nuh_layer_id > 0 && !default_ref_layers_active_flag && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| if( NumActiveRefLayerPics != NumDirectRefLayers[ nuh_layer_id ] ) |  |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| if( lists_modification_present_flag && NumPicTotalCurr > 1 ) |  |
| ref_pic_lists_modification( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P ) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| } |  |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| if( pps_loop_filter_across_slices_enabled_flag && ( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ¦ ¦ !slice_deblocking_filter_disabled_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| if( tiles_enabled_flag ¦ ¦ entropy_coding_sync_enabled_flag ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| } |  |
| if( slice_segment_header_extension_present_flag ) { |  |
| slice_segment_header_extension_length | ue(v) |
| if( poc_reset_info_present_flag ) |  |
| poc_reset_idc | u(2) |
| if( poc_reset_idc != 0 ) |  |
| poc_reset_period_id | u(6) |
| if( poc_reset_idc = = 3 ) { |  |
| full_poc_reset_flag | u(1) |
| poc_lsb_val | u(v) |
| } |  |
| if( !PocMsbValRequiredFlag && vps_poc_lsb_aligned_flag ) |  |
| poc_msb_cycle_val_present_flag | u(1) |
| if( poc_msb_cycle_val_present_flag ) |  |
| poc_msb_cycle_val | ue(v) |
| while( more_data_in_slice_segment_header_extension( ) ) |  |
| slice_segment_header_extension_data_bit | u(1) |
| } |  |
| byte_alignment( ) |  |
| } |  |

        - Reference picture list modification syntax

The specifications in clause 7.3.6.2 apply.

        - Weighted prediction parameters syntax

The specifications in clause 7.3.6.3 apply.

      - Short-term reference picture set syntax

The specifications in clause 7.3.7 apply.

      - Slice segment data syntax

        - General slice segment data syntax

The specifications in clause 7.3.8.1 apply.

        - Coding tree unit syntax

The specifications in clause 7.3.8.2 apply.

        - Sample adaptive offset syntax

The specifications in clause 7.3.8.3 apply.

        - Coding quadtree syntax

The specifications in clause 7.3.8.4 apply.

        - Coding unit syntax

The specifications in clause 7.3.8.5 apply.

        - Prediction unit syntax

The specifications in clause 7.3.8.6 apply.

        - PCM sample syntax

The specifications in clause 7.3.8.7 apply.

        - Transform tree syntax

The specifications in clause 7.3.8.8 apply.

        - Motion vector difference syntax

The specifications in clause 7.3.8.9 apply.

        - Transform unit syntax

The specifications in clause 7.3.8.10 apply.

        - Residual coding syntax

The specifications in clause 7.3.8.11 apply.

        - Cross-component prediction syntax

The specifications in clause 7.3.8.12 apply.

    - Semantics

      - General

      - NAL unit semantics

        - General NAL unit semantics

The specifications in clause 7.4.2.1 apply.

        - NAL unit header semantics

The specifications in clause 7.4.2.2 apply with the replacement of references to Annex C with references to clause A.13 and with the following additions:

The variable CraOrBlaPicFlag is derived as follows:

CraOrBlaPicFlag = ( nal_unit_type  = =  BLA_W_LP  | |  nal_unit_type  = =  BLA_N_LP  | |	(F-1)

					nal_unit_type  = =  BLA_W_RADL  | |  nal_unit_type  = =  CRA_NUT )

- NOTE 1 – When a picture picA that is a CRA picture and belongs to a layer with nuh_layer_id equal to layerId is present in a bitstream and pictures belonging to the layer with nuh_layer_id equal to layerId and preceding, in decoding order, the picture picA are dropped due to layer down-switching followed by layer up-switching, the RASL pictures associated with the picture picA, if any, may have some reference pictures that may not be available for reference unless one of the following conditions is true:

–	The access unit auA containing the picture picA is an IRAP access unit, and the picture with nuh_layer_id equal to SmallestLayerId in the access unit auA, if any, has NoClrasOutputFlag equal to 1.

–	The value of HandleCraAsBlaFlag is equal to 1 for the CRA picture picA.

A NAL unit with nal_unit_type equal to EOS_NUT and with a particular nuh_layer_id value shall not be present in an access unit, unless a coded picture with that particular nuh_layer_id value is present in the same access unit.

- NOTE 2 – Let indepLayerId be nuh_layer_id value of any independent non-base layer with nuh_layer_id greater than SmallestLayerId. Let firstPic be the first picture, in decoding order, among the pictures with nuh_layer_id equal to indepLayerId or IdPredictedLayer[ indepLayerId ][ i ] for any value of i in the range of 0 to NumPredictedLayers[ indepLayerId ] − 1, inclusive, that follows an IRAP picture with NoClrasOutputFlag equal to 1. firstPic has to be an IRAP picture with nuh_layer_id equal to indepLayerId and with LayerResetFlag equal to 1.

When nal_unit_type is equal to AUD_NUT, the value of nuh_layer_id shall be equal to the minimum of the nuh_layer_id values of all VCL NAL units in the access unit.

        - Encapsulation of an SODB within an RBSP (informative)

The specifications in clause 7.4.2.3 apply.

        - Order of NAL units and association to coded pictures, access units, and coded video sequences

          - General

The specifications in clause 7.4.2.4.1 apply.

          - Order of VPS, SPS and PPS RBSPs and their activation

The specifications in clause 7.4.2.4.2 apply with the following additions:

- A PPS RBSP includes parameters that can be referred to by the coded slice segment NAL units of one or more coded pictures. Each PPS RBSP is initially considered not active for any layer at the start of the operation of the decoding process. At most one PPS RBSP is considered active for each layer at any given moment during the operation of the decoding process, and the activation of any particular PPS RBSP for a particular layer results in the deactivation of the previously-active PPS RBSP for the particular layer (if any).

- One PPS RBSP may be the active PPS RBSP for more than one layer. When not explicitly specified, the layer a PPS RBSP is active for is inferred to be the current layer in the context where the active PPS RBSP is referred to.

- When a PPS RBSP (with a particular value of pps_pic_parameter_set_id) is not active for a particular layer with nuh_layer_id activatingLayerId and it is referred to by a coded slice segment NAL unit (using a value of slice_pic_parameter_set_id equal to the pps_pic_parameter_set_id value) of the particular layer, it is activated for the particular layer. This PPS RBSP is called the active PPS RBSP for the particular layer until it is deactivated by the activation of another PPS RBSP for the particular layer. A PPS RBSP, with that particular value of pps_pic_parameter_set_id, shall be available to the decoding process prior to its activation, included in at least one access unit with TemporalId less than or equal to the TemporalId of the PPS NAL unit or provided through external means, and the PPS NAL unit containing the PPS RBSP shall have nuh_layer_id equal to 0, activatingLayerId, or IdRefLayer[ activatingLayerId ][ i ] with any value of i in the range of 0 to NumRefLayers[ activatingLayerId ] − 1, inclusive.

NOTE 1 – All PPSs, regardless of their values of nuh_layer_id or TemporalId, share the same value space for pps_pic_parameter_set_id. In other words, a PPS with nuh_layer_id equal to X, TemporalId equal to Y, and pps_pic_parameter_set_id equal to A would update the previously received PPS that has pps_pic_parameter_set_id equal to A and that has nuh_layer_id not equal to X or TemporalId not equal to Y.

- An SPS RBSP includes parameters that can be referred to by one or more PPS RBSPs or one or more SEI NAL units containing an active parameter sets SEI message. Each SPS RBSP is initially considered not active for any layer at the start of the operation of the decoding process. At most one SPS RBSP is considered active for each layer at any given moment during the operation of the decoding process, and the activation of any particular SPS RBSP for a particular layer results in the deactivation of the previously-active SPS RBSP for that particular layer (if any).

- One SPS RBSP may be the active SPS RBSP for more than one layer. When not explicitly specified, the layer an SPS RBSP is active for is inferred to be the current layer in the context where the active SPS RBSP is referred to.

- When an SPS RBSP (with a particular value of sps_seq_parameter_set_id) is not already active for a particular non-base layer with nuh_layer_id nuhLayerId and it is referred to by activation of a PPS RBSP (in which pps_seq_parameter_set_id is equal to the sps_seq_parameter_set_id value), or is referred to by an SEI NAL unit containing an active parameter sets SEI message (in which active_seq_parameter_set_id[ layer_sps_idx[ LayerIdxInVps[ nuhLayerId ] ] ] ] is equal to the sps_seq_parameter_set_id value), it is activated for the particular non-base layer. This SPS RBSP is called the active SPS RBSP for the particular non-base layer until it is deactivated by the activation of another SPS RBSP for the particular non-base layer. An SPS RBSP, with the particular value of sps_seq_parameter_set_id, shall be available to the decoding process prior to its activation, included in at least one access unit with TemporalId equal to 0 or provided through external means, and the SPS NAL unit containing the SPS RBSP shall have nuh_layer_id equal to 0, nuhLayerId, or IdRefLayer[ nuhLayerId ][ i ] with any value of i in the range of 0 to NumRefLayers[ nuhLayerId ] − 1, inclusive. An activated SPS RBSP for a particular layer shall remain active for the entire CLVS of that particular layer.

The contents of the hrd_parameters( ) syntax structure shall remain unchanged within a sequence of activated SPS RBSPs, in their activation order, from any activated SPS RBSP until the end of the bitstream or up to but excluding an SPS RBSP that is activated within the next access unit in which at least one of the following conditions is true:

- The access unit includes a picture for each nuh_layer_id value in TargetDecLayerIdList and each picture in the access unit is an IDR picture.

- The access unit includes an IRAP picture with nuh_layer_id equal to SmallestLayerId for which NoClrasOutputFlag is equal to 1.

- Any SPS NAL unit with any nuh_layer_id value containing the value of sps_seq_parameter_set_id for the active SPS RBSP for a particular layer shall have the same content as that of the active SPS RBSP for the particular layer unless it follows the access unit auA containing the last coded picture for which the active SPS RBSP for the particular layer is required to be active for the particular layer and precedes the first NAL unit succeeding auA in decoding order that activates an SPS RBSP with the same value of seq_parameter_set_id.

NOTE 2 – All SPSs, regardless of their values of nuh_layer_id, share the same value space for sps_seq_parameter_set_id. In other words, an SPS with nuh_layer_id equal to X and sps_seq_parameter_set_id equal to A would update the previously received SPS with nuh_layer_id not equal to X and sps_seq_parameter_set_id equal to A.

- When a VPS RBSP (with a particular value of vps_video_parameter_set_id) is not already active and it is referred to by activation of an SPS RBSP (in which sps_video_parameter_set_id is equal to the vps_video_parameter_set_id value), or is referred to by an SEI NAL unit containing an active parameter sets SEI message (in which active_video_parameter_set_id is equal to the vps_video_parameter_set_id value), it is activated. This VPS RBSP is called the active VPS RBSP until it is deactivated by the activation of another VPS RBSP. A VPS RBSP, with that particular value of vps_video_parameter_set_id, shall be available to the decoding process prior to its activation, included in at least one access unit with TemporalId equal to 0 or provided through external means, and the VPS NAL unit containing the VPS RBSP shall have nuh_layer_id equal to 0 or SmallestLayerId.

An activated VPS RBSP shall remain active until the end of the bitstream or until it is deactivated by another VPS RBSP in an access unit in which at least one of the following conditions is true:

- The access unit includes a picture for each nuh_layer_id value in TargetDecLayerIdList and each picture in the access unit is an IDR picture.

- The access unit includes an IRAP picture with nuh_layer_id equal to SmallestLayerId for which NoClrasOutputFlag is equal to 1.

For any VPS NAL unit within the CVS with any value of nuh_layer_id and the value of vps_video_parameter_set_id equal to that of the active VPS RBSP for a CVS, the VPS RBSP in the VPS NAL unit shall have the same content as that of the active VPS RBSP for the CVS.

All constraints that are expressed on the relationship between the values of the syntax elements and the values of variables derived from those syntax elements in VPSs, SPSs, and PPSs and other syntax elements are expressions of constraints that apply only to the active VPS RBSP, the active SPS RBSP for any particular layer, and the active PPS RBSP for any particular layer. If any VPS RBSP, SPS RBSP, and PPS RBSP is present that is never activated in the bitstream, its syntax elements shall have values that would conform to the specified constraints if it was activated by reference in an otherwise conforming bitstream.

During operation of the decoding process for NAL units of a non-base layer, the values of parameters of the active VPS RBSP, the active SPS RBSP for the non-base layer, and the active PPS RBSP for the non-base layer are considered in effect. For interpretation of SEI messages applicable to a coded picture of a non-base layer, the values of the active VPS RBSP, the active SPS RBSP for the non-base layer, and the active PPS RBSP for the non-base layer for the operation of the decoding process for the VCL NAL units of the coded picture are considered in effect unless otherwise specified in the SEI message semantics.

          - Order of access units association to CVSs

A bitstream conforming to this Specification consists of one or more CVSs.

A CVS consists of one or more access units. The order of NAL units and coded pictures and their association to access units is described in clause A.7.4.2.4.4.

The first access unit of a CVS is an IRAP access unit with NoRaslOutputFlag equal to 1.

It is a requirement of bitstream conformance that, when present, the next access unit after an access unit that contains an end of sequence NAL unit with nuh_layer_id equal to SmallestLayerId or an end of bitstream NAL unit shall be an IRAP access unit.

          - Order of NAL units and coded pictures and association to access units

This clause specifies the order of NAL units and coded pictures and their association to access unit for CVSs that contain NAL units with nuh_layer_id greater than 0 that are decoded using the decoding processes specified in Annex F, Annex G and Annex H.

An access unit consists of one or more coded pictures, each with a distinct value of nuh_layer_id, and zero or more non-VCL NAL units. The association of VCL NAL units to coded pictures is described in clause 7.4.2.4.5.

The first access unit in the bitstream starts with the first NAL unit of the bitstream.

A VCL NAL unit is the first VCL NAL unit of an access unit, when all of the following conditions are true:

–	first_slice_segment_in_pic_flag is equal to 1.

–	At least one of the following conditions is true:

–	The previous picture in decoding order belongs to a different POC resetting period than the picture containing the VCL NAL unit.

–	PicOrderCntVal derived for the VCL NAL unit differs from the PicOrderCntVal of the previous picture in decoding order.

NOTE 1 – For any two consecutive access units auA and auB that belong to the same POC resetting period, the PicOrderCntVal of pictures in auA cannot be the same as the PicOrderCntVal of pictures in auB.

NOTE 2 – Additionally, more conditions, e.g., the following conditions, could but does not need to be used:

–	The nuh_layer_id value of the VCL NAL unit is equal to SmallestLayerId.

–	vps_poc_lsb_aligned_flag is equal to 1 and the slice_pic_order_cnt_lsb value of the VCL NAL unit differs from the slice_pic_order_cnt_lsb value of the previous VCL NAL unit in decoding order.

The first of any of the following NAL units preceding the first VCL NAL unit firstVclNalUnitInAu and succeeding the last VCL NAL unit preceding firstVclNalUnitInAu, if any, specifies the start of a new access unit:

–	Access unit delimiter NAL unit (when present).

–	VPS NAL unit (when present)

–	SPS NAL unit (when present)

–	PPS NAL unit (when present)

–	Prefix SEI NAL unit (when present)

–	NAL units with nal_unit_type in the range of RSV_NVCL41..RSV_NVCL44 (when present)

–	NAL units with nal_unit_type in the range of UNSPEC48..UNSPEC55 (when present)

When there is none of the above NAL units preceding firstVclNalUnitInAu and succeeding the last VCL NAL unit preceding firstVclNalUnitInAu, if any, firstVclNalUnitInAu starts a new access unit.

A coded picture with nuh_layer_id equal to nuhLayerIdA shall precede, in decoding order, all coded pictures with nuh_layer_id greater than nuhLayerIdA in the same access unit.

The order of the coded pictures and non-VCL NAL units within an access unit shall obey the following constraints:

–	When an access unit delimiter NAL unit is present, it shall be the first NAL unit. There shall be at most one access unit delimiter NAL unit in any access unit.

–	When any VPS NAL units, SPS NAL units, PPS NAL units, prefix SEI NAL units, NAL units with nal_unit_type in the range of RSV_NVCL41..RSV_NVCL44, or NAL units with nal_unit_type in the range of UNSPEC48..UNSPEC55 are present, they shall not follow the last VCL NAL unit of the access unit.

–	NAL units having nal_unit_type equal to FD_NUT or SUFFIX_SEI_NUT, or in the range of RSV_NVCL45..RSV_NVCL47 or UNSPEC56..UNSPEC63 shall not precede the first VCL NAL unit of the access unit.

–	When an end of sequence NAL unit with nuh_layer_id nuhLayerId is present, it shall be the last NAL unit with nuh_layer_id equal to nuhLayerId in the access unit other than an end of bitstream NAL unit (when present).

–	When an end of bitstream NAL unit is present, it shall be the last NAL unit in the access unit.

          - Order of VCL NAL units and association to coded pictures

The specifications in clause 7.4.2.4.5 apply.

      - Raw byte sequence payloads, trailing bits, and byte alignment semantics

        - Video parameter set RBSP semantics

The specifications in clause 7.4.3.1 apply with the replacement of references to Annex C with references to clause A.13, with the replacement of the semantics of the syntax element vps_extension_flag with that specified below, and with the following additions:

Each output operation point is associated with an operation point, a list of nuh_layer_id values of the output layers, in increasing order of nuh_layer_id values, denoted as OptLayerIdList, and the OpTid of the associated operation point. The OpLayerIdList of the operation point associated with an output operation point is also referred to as the OpLayerIdList of the output operation point.

vps_extension_flag equal to 0 specifies that no vps_extension( ) syntax structure is present in the VPS RBSP syntax structure. vps_extension_flag equal to 1 specifies that the vps_extension( ) syntax structure is present in the VPS RBSP syntax structure. When MaxLayersMinus1 is greater than 0, vps_extension_flag shall be equal to 1.

vps_extension_alignment_bit_equal_to_one shall be equal to 1.

vps_extension2_flag equal to 0 specifies that no vps_extension_data_flag syntax elements are present in the VPS RBSP syntax structure. vps_extension2_flag shall be equal to 0 in bitstreams conforming to this version of this Specification. The value of 1 for vps_extension2_flag is reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this version of this Specification shall ignore all data that follow the value 1 for vps_extension2_flag in a VPS RBSP.

          - Video parameter set extension semantics

splitting_flag equal to 1 indicates that the dimension_id[ i ][ j ] syntax elements are not present and that the binary representation of the nuh_layer_id value in the NAL unit header are split into NumScalabilityTypes segments with lengths, in bits, according to the values of dimension_id_len_minus1[ j ] and that the values of dimension_id[ LayerIdxInVps[ nuh_layer_id ] ][ j ] are inferred from the NumScalabilityTypes segments. splitting_flag equal to 0 indicates that the syntax elements dimension_id[ i ][ j ] are present.

NOTE 1 – When splitting_flag is equal to 1, scalability identifiers of the present scalability dimensions can be derived from the nuh_layer_id syntax element in the NAL unit header by a bit masked copy. The respective bit mask for the i-th present scalability dimension is defined by the value of the dimension_id_len_minus1[ i ] syntax element and dimBitOffset[ i ] as specified in the semantics of dimension_id_len_minus1[ j ].

scalability_mask_flag[ i ] equal to 1 indicates that dimension_id syntax elements corresponding to the i-th scalability dimension in Table FF.1 are present. scalability_mask_flag[ i ] equal to 0 indicates that dimension_id syntax elements corresponding to the i-th scalability dimension are not present.

Table FF.1 – Mapping of ScalabiltyId to scalability dimensions

| scalability mask index | Scalability dimension | ScalabilityId mapping |
| --- | --- | --- |
| 0 | Reserved |  |
| 1 | Multiview | View Order Index |
| 2 | Spatial/quality scalability | DependencyId |
| 3 | Auxiliary | AuxId |
| 4-15 | Reserved |  |

NOTE 2 – It is anticipated that in future 3D extensions of this Specification, scalability mask index 0 will be used to indicate depth maps.

dimension_id_len_minus1[ j ] plus 1 specifies the length, in bits, of the dimension_id[ i ][ j ] syntax element.

When splitting_flag is equal to 1, the following applies:

–	The variable dimBitOffset[ 0 ] is set equal to 0 and for j in the range of 1 to NumScalabilityTypes − 1, inclusive, dimBitOffset[ j ] is derived as follows:

dimBitOffset[ j ]=dimIdx=0j−1dimension_id_len_minus1[ dimIdx ]+1	(F-1)

–	The value of dimension_id_len_minus1[ NumScalabilityTypes − 1 ] is inferred to be equal to 5 − dimBitOffset[ NumScalabilityTypes − 1 ].

–	The value of dimBitOffset[ NumScalabilityTypes ] is set equal to 6.

It is a requirement of bitstream conformance that when NumScalabilityTypes is greater than 0, dimBitOffset[ NumScalabilityTypes − 1 ] shall be less than 6.

vps_nuh_layer_id_present_flag equal to 1 specifies that layer_id_in_nuh[ i ] for i from 1 to MaxLayersMinus1, inclusive, are present. vps_nuh_layer_id_present_flag equal to 0 specifies that layer_id_in_nuh[ i ] for i from 1 to MaxLayersMinus1, inclusive, are not present.

layer_id_in_nuh[ i ] specifies the value of the nuh_layer_id syntax element in VCL NAL units of the i-th layer. When i is greater than 0, layer_id_in_nuh[ i ] shall be greater than layer_id_in_nuh[ i − 1 ]. For any value of i in the range of 0 to MaxLayersMinus1, inclusive, when not present, the value of layer_id_in_nuh[ i ] is inferred to be equal to i.

For i from 0 to MaxLayersMinus1, inclusive, the variable LayerIdxInVps[ layer_id_in_nuh[ i ] ] is set equal to i.

dimension_id[ i ][ j ] specifies the identifier of the j-th present scalability dimension type of the i-th layer. The number of bits used for the representation of dimension_id[ i ][ j ] is dimension_id_len_minus1[ j ] + 1 bits.

Depending on splitting_flag, the following applies:

–	If splitting_flag is equal to 1, for i from 0 to MaxLayersMinus1, inclusive, and j from 0 to NumScalabilityTypes − 1, inclusive, dimension_id[ i ][ j ] is inferred to be equal to ( ( layer_id_in_nuh[ i ] & ( ( 1  <<  dimBitOffset[ j + 1 ] ) − 1 ) )  >>  dimBitOffset[ j ] ).

–	Otherwise (splitting_flag is equal to 0), for j from 0 to NumScalabilityTypes − 1, inclusive, dimension_id[ 0 ][ j ] is inferred to be equal to 0.

The variable ScalabilityId[ i ][ smIdx ] specifying the identifier of the smIdx-th scalability dimension type of the i-th layer, and the variables ViewOrderIdx[ lId ], DependencyId[ lId ], and AuxId[ lId ] specifying the view order index, the spatial/quality scalability identifier, and the auxiliary identifier, respectively, of the layer with nuh_layer_id equal to lId  are derived as follows:

NumViews = 1
for( i = 0; i  <=  MaxLayersMinus1; i++ ) {
	lId = layer_id_in_nuh[ i ]
	for( smIdx= 0, j = 0; smIdx < 16; smIdx++ ) {
		if( scalability_mask_flag[ smIdx ] )
			ScalabilityId[ i ][ smIdx ] = dimension_id[ i ][ j++ ]
		else
			ScalabilityId[ i ][ smIdx ] = 0
	}
	ViewOrderIdx[ lId ] = ScalabilityId[ i ][ 1 ]
	DependencyId[ lId ] = ScalabilityId[ i ][ 2 ]		(F-2)
	AuxId[ lId ] = ScalabilityId[ i ][ 3 ]
	if( i > 0 ) {
		newViewFlag = 1
		for( j = 0; j < i; j++ )
			if( ViewOrderIdx[ lId ]  = =  ViewOrderIdx[ layer_id_in_nuh[ j ] ] )
				newViewFlag = 0
		NumViews  +=  newViewFlag
	}
}

AuxId[ lId ] equal to 0 specifies the layer with nuh_layer_id equal to lId does not contain auxiliary pictures. AuxId[ lId ] greater than 0 specifies the type of auxiliary pictures in layer with nuh_layer_id equal to lId as specified in Table FF.2.

Table FF.2 – Mapping of AuxId to the type of auxiliary pictures

| AuxId | Name of AuxId | Type of auxiliary pictures | SEI message describing interpretation of auxiliary pictures |
| --- | --- | --- | --- |
| 1 | AUX_ALPHA | Alpha plane | Alpha channel information |
| 2 | AUX_DEPTH | Depth picture | Depth representation information |
| 3..127 |  | Reserved |  |
| 128..159 |  | Unspecified |  |
| 160..255 |  | Reserved |  |

NOTE 3 – The interpretation of auxiliary pictures associated with AuxId in the range of 128 to 159, inclusive, is specified through means other than the AuxId value.

AuxId[ lId ] shall be in the range of 0 to 2, inclusive, or 128 to 159, inclusive, for bitstreams conforming to this version of this Specification. Although the value of AuxId[ lId ] shall be in the range of 0 to 2, inclusive, or 128 to 159, inclusive, in this version of this Specification, decoders shall allow values of AuxId[ lId ] in the range of 0 to 255, inclusive.

It is a requirement of bitstream conformance that when AuxId[ lId ] is equal to AUX_ALPHA or AUX_DEPTH, either of the following applies:

–	chroma_format_idc is equal to 0 in the active SPS for the layer with nuh_layer_id equal to lId.

–	The value of all decoded chroma samples is equal to 1  <<  ( BitDepthC − 1 ) in all pictures that have nuh_layer_id equal to lId and for which this VPS RBSP is the active VPS RBSP.

SEI messages may describe the interpretation of auxiliary pictures, including their possible association with one or more primary pictures.

NOTE 4 – Unless constrained by the semantics of the SEI messages specifying the interpretation of auxiliary pictures, it is allowed to have two layers with nuh_layer_id values layerIdA and layerIdB such that AuxId[ layerIdA ] is equal to AuxId[ layerIdB ], both being greater than 0, and to have all values of ScalabilityId[ LayerIdxInVps[ layerIdA ] ][ i ] equal to ScalabilityId[ LayerIdxInVps[ layerIdB ] ][ i ] for each value of i in the range of 0 to 15, inclusive. SEI messages specifying the interpretation of auxiliary pictures may specify that a picture with nuh_layer_id equal to layerIdA and a picture with nuh_layer_id equal to layerIdB in the same access unit may both be associated with the same primary picture.

view_id_len specifies the length, in bits, of the view_id_val[ i ] syntax element.

view_id_val[ i ] specifies the view identifier of the i-th view specified by the VPS. The length of the view_id_val[ i ] syntax element is view_id_len bits. When not present, the value of view_id_val[ i ] is inferred to be equal to 0.

For each layer with nuh_layer_id equal to nuhLayerId, the value ViewId[ nuhLayerId ] is set equal to view_id_val[ ViewOrderIdx[ nuhLayerId ] ].

direct_dependency_flag[ i ][ j ] equal to 0 specifies that the layer with index j is not a direct reference layer for the layer with index i. direct_dependency_flag[ i ][ j ] equal to 1 specifies that the layer with index j is a direct reference layer for the layer with index i. When direct_dependency_flag[ i ][ j ] is not present for i and j in the range of 0 to MaxLayersMinus1, it is inferred to be equal to 0.

The variable DependencyFlag[ i ][ j ] is derived as follows:

for( i = 0; i  <=  MaxLayersMinus1; i++ )
	for( j = 0; j  <=  MaxLayersMinus1; j++ ) {
		DependencyFlag[ i ][ j ] = direct_dependency_flag[ i ][ j ]
		for( k = 0; k < i; k++ )
			if( direct_dependency_flag[ i ][ k ]  &&  DependencyFlag[ k ][ j ] )	(F-3)
				DependencyFlag[ i ][ j ] = 1
	}

The variables NumDirectRefLayers[ iNuhLId ], IdDirectRefLayer[ iNuhLId ][ d ], NumRefLayers[ iNuhLId ], IdRefLayer[ iNuhLId ][ r ], NumPredictedLayers[ iNuhLId ], and IdPredictedLayer[ iNuhLId ][ p ] are derived as follows:

for( i = 0; i  <=  MaxLayersMinus1; i++ ) {
	iNuhLId = layer_id_in_nuh[ i ]
	for( j = 0, d = 0, r = 0, p = 0; j  <=  MaxLayersMinus1; j++ ) {
		jNuhLid = layer_id_in_nuh[ j ]
		if( direct_dependency_flag[ i ][ j ] )
			IdDirectRefLayer[ iNuhLId ][ d++ ] = jNuhLid
		if( DependencyFlag[ i ][ j ] )
			IdRefLayer[ iNuhLId ][ r++ ] = jNuhLid	(F-4)
		if( DependencyFlag[ j ][ i ] )
			IdPredictedLayer[ iNuhLId ][ p++ ] = jNuhLid
	}
	NumDirectRefLayers[ iNuhLId ] = d
	NumRefLayers[ iNuhLId ] = r
	NumPredictedLayers[ iNuhLId ] = p
}

The variables NumIndependentLayers, NumLayersInTreePartition[ i ], and TreePartitionLayerIdList[ i ][ j ] for i in the range of 0 to NumIndependentLayers − 1, inclusive, and j in the range of 0 to NumLayersInTreePartition[ i ] − 1, inclusive, are derived as follows:

for( i = 0; i  <=  63; i++ )
	layerIdInListFlag[ i ] = 0
for( i = 0, k = 0; i  <=  MaxLayersMinus1; i++ ) {
	iNuhLId = layer_id_in_nuh[ i ]
	if( NumDirectRefLayers[ iNuhLId ]  = =  0 ) {
		TreePartitionLayerIdList[ k ][ 0 ] = iNuhLId
		for( j = 0, h = 1; j < NumPredictedLayers[ iNuhLId ]; j++ ) {	(F-5)
			predLId = IdPredictedLayer[ iNuhLId ][ j ]
			if( !layerIdInListFlag[ predLId ] ) {
				TreePartitionLayerIdList[ k ][ h++ ] = predLId
				layerIdInListFlag[ predLId ] = 1
			}
		}
		NumLayersInTreePartition[ k++ ] = h
	}
}
NumIndependentLayers = k

It is a requirement of bitstream conformance that AuxId[ IdDirectRefLayer[ nuhLayerIdA ][ j ] ] for any values of nuhLayerIdA and j shall be equal to AuxId[ nuhLayerIdA ], when AuxId[ nuhLayerIdA ] is in the range of 0 to 2, inclusive.

NOTE 5 – In other words, no prediction takes place between layers with a different value of AuxId, when AuxId is in the range of 0 to 2, inclusive.

num_add_layer_sets specifies the number of additional layer sets. The value of num_add_layer_sets shall be in the range of 0 to 1023, inclusive. When vps_base_layer_available_flag is equal to 0, the value of num_add_layer_sets shall be greater than 0. When not present, the value of num_add_layer_sets is inferred to be equal to 0.

The variable NumLayerSets is derived as follows:

NumLayerSets = vps_num_layer_sets_minus1 + 1 + num_add_layer_sets	(F-6)

When num_add_layer_sets is greater than 0, the variables FirstAddLayerSetIdx and LastAddLayerSetIdx are derived as follows:

FirstAddLayerSetIdx = vps_num_layer_sets_minus1 + 1	(F-7)
LastAddLayerSetIdx = FirstAddLayerSetIdx + num_add_layer_sets − 1

highest_layer_idx_plus1[ i ][ j ] specifies the values of NumLayersInIdList[ vps_num_layer_sets_minus1 + 1 + i ] and LayerSetLayerIdList[ vps_num_layer_sets_minus1 + 1 + i ][ layerNum ] as follows:

layerNum = 0
lsIdx = vps_num_layer_sets_minus1 + 1 + i
for( treeIdx = 1; treeIdx < NumIndependentLayers; treeIdx++ )	(F-8)
	for( layerCnt = 0; layerCnt < highest_layer_idx_plus1[ i ][ treeIdx ]; layerCnt++ )
		LayerSetLayerIdList[ lsIdx ][ layerNum++ ] = TreePartitionLayerIdList[ treeIdx ][ layerCnt ]
NumLayersInIdList[ lsIdx ] = layerNum

The value of highest_layer_idx_plus1[ i ][ j ] shall be in the range of 0 to NumLayersInTreePartition[ j ], inclusive.

The length of the highest_layer_idx_plus1[ i ][ j ] syntax element is Ceil( Log2( NumLayersInTreePartition[ j ] + 1 ) ) bits.

It is a requirement of bitstream conformance that NumLayersInIdList[ vps_num_layer_sets_minus1 + 1 + i ] shall be greater than 0.

vps_sub_layers_max_minus1_present_flag equal to 1 specifies that the syntax elements sub_layers_vps_max_minus1[ i ] are present. vps_sub_layers_max_minus1_present_flag equal to 0 specifies that the syntax elements sub_layers_vps_max_minus1[ i ] are not present.

sub_layers_vps_max_minus1[ i ] plus 1 specifies, for each value of i in the range of ( vps_base_layer_internal_flag ? 0 : 1 ) to MaxLayersMinus1, inclusive, the maximum number of temporal sub-layers that may be present in the CVS for the layer with nuh_layer_id layerId equal to layer_id_in_nuh[ i ]. When vps_base_layer_internal_flag is equal to 0, sub_layers_vps_max_minus1[ 0 ] constrains the access units for which a decoded picture with nuh_layer_id equal to 0 may be provided by external means as follows: a decoded picture with nuh_layer_id equal to 0 cannot be provided by external means for decoding of an access unit with TemporalId greater than sub_layers_vps_max_minus1[ 0 ]. The value of sub_layers_vps_max_minus1[ i ] shall be in the range of 0 to vps_max_sub_layers_minus1, inclusive. When not present, the value of sub_layers_vps_max_minus1[ i ] is inferred to be equal to vps_max_sub_layers_minus1.

The variable MaxSubLayersInLayerSetMinus1[ i ] is derived as follows:

for( i = 0; i < NumLayerSets; i++ ) {
	maxSlMinus1 = 0
	for( k = 0; k < NumLayersInIdList[ i ]; k++ ) {
		lId = LayerSetLayerIdList[ i ][ k ]		(F-9)
		maxSlMinus1 = Max( maxSLMinus1, sub_layers_vps_max_minus1[ LayerIdxInVps[ lId ] ] )
	}
	MaxSubLayersInLayerSetMinus1[ i ] = maxSlMinus1
}

max_tid_ref_present_flag equal to 1 specifies that the syntax element max_tid_il_ref_pics_plus1[ i ][ j ] is present. max_tid_ref_present_flag equal to 0 specifies that the syntax element max_tid_il_ref_pics_plus1[ i ][ j ] is not present.

max_tid_il_ref_pics_plus1[ i ][ j ] equal to 0 specifies that non-IRAP pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] are not used as source pictures for inter-layer prediction for pictures with nuh_layer_id equal to layer_id_in_nuh[ j ]. max_tid_il_ref_pics_plus1[ i ][ j ] greater than 0 specifies that pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] and TemporalId greater than max_tid_il_ref_pics_plus1[ i ][ j ] − 1 are not used as source pictures for inter-layer prediction for pictures with nuh_layer_id equal to layer_id_in_nuh[ j ]. When not present, the value of max_tid_il_ref_pics_plus1[ i ][ j ] is inferred to be equal to 7.

default_ref_layers_active_flag equal to 1 specifies that for each picture referring to the VPS, the direct reference layer pictures that belong to all direct reference layers of the layer containing the picture, and that may be used for inter-layer prediction as specified by the values of sub_layers_vps_max_minus1[ i ] and max_tid_il_ref_pics_plus1[ i ][ j ] are present in the same access unit as the picture and are included in the inter-layer reference picture set of the picture. default_ref_layers_active_flag equal to 0 specifies that the above restriction may or may not apply.

vps_num_profile_tier_level_minus1 plus 1 specifies the number of profile_tier_level( ) syntax structures in the VPS. The value of vps_num_profile_tier_level_minus1 shall be in the range of 0 to 63, inclusive. When vps_max_layers_minus1 is greater than 0, the value of vps_num_profile_tier_level_minus1 shall be greater than or equal to 1.

vps_profile_present_flag[ i ] equal to 1 specifies that profile and tier information is present in the i-th profile_tier_level( ) syntax structure. vps_profile_present_flag[ i ] equal to 0 specifies that profile and tier information is not present in the i-th profile_tier_level( ) syntax structure and is inferred as specified in clause A.7.4.4.

num_add_olss specifies the number of OLSs in addition to the first NumLayerSets OLSs specified by the VPS. The value of num_add_olss shall be in the range of 0 to 1023, inclusive. When not present, the value of num_add_olss is inferred to be equal to 0.

default_output_layer_idc specifies the derivation of the output layers for the OLSs with index in the range of 1 to vps_num_layer_sets_minus1, inclusive. default_output_layer_idc equal to 0 specifies that all layers in each of the OLSs with index in the range of 1 to vps_num_layer_sets_minus1, inclusive, are output layers of their respective OLSs. default_output_layer_idc equal to 1 specifies that only the layer with the highest value of nuh_layer_id such that nuh_layer_id equal to nuhLayerIdA in each of the OLSs with index in the range of 1 to vps_num_layer_sets_minus1, inclusive, is an output layer of its OLS. default_output_layer_idc equal to 2 specifies that the output layers for the OLSs with index in the range of 1 to vps_num_layer_sets_minus1, inclusive, are specified with the syntax elements output_layer_flag[ i ][ j ]. The value of 3 for default_output_layer_idc is reserved for future use by ITU-T | ISO/IEC. Although the value of default_output_layer_idc is required to be less than 3 in this version of this Specification, decoders shall allow a value of default_output_layer_idc equal to 3 to appear in the syntax.

The variable defaultOutputLayerIdc is set equal to Min( default_output_layer_idc, 2 ).

layer_set_idx_for_ols_minus1[ i ] plus 1 specifies the index of the layer set for the i-th OLS. The value of layer_set_idx_for_ols_minus1[ i ] shall be in the range of 0 to NumLayerSets − 2, inclusive. The length of the layer_set_idx_for_ols_minus1[ i ] syntax element is Ceil( Log2( NumLayerSets − 1 ) ) bits. When not present, the value of layer_set_idx_for_ols_minus1[ i ] is inferred to be equal to 0.

For i in the range of 0 to NumOutputLayerSets − 1, inclusive, the variable OlsIdxToLsIdx[ i ] is derived as specified in the following:

OlsIdxToLsIdx[ i ] =  ( i < NumLayerSets ) ? i : ( layer_set_idx_for_ols_minus1[ i ] + 1 )		(F-10)

output_layer_flag[ i ][ j ] equal to 1 specifies that the j-th layer in the i-th OLS is an output layer. output_layer_flag[ i ][ j ] equal to 0 specifies that the j-th layer in the i-th OLS is not an output layer.

The value of output_layer_flag[ 0 ][ 0 ] is inferred to be equal to 1.

When defaultOutputLayerIdc is equal to 0 or 1, for i in the range of 0 to vps_num_layer_sets_minus1, inclusive, and j in the range of 0 to NumLayersInIdList[ OlsIdxToLsIdx[ i ] ] − 1, inclusive, the variable OutputLayerFlag[ i ][ j ] is derived as follows:

–	If defaultOutputLayerIdc is equal to 0 or LayerSetLayerIdList[ OlsIdxToLsIdx[ i ] ][ j ] is equal to nuhLayerIdA, with nuhLayerIdA being the highest value in LayerSetLayerIdList[ OlsIdxToLsIdx[ i ] ], OutputLayerFlag[ i ][ j ] is set equal to 1.

–	Otherwise, OutputLayerFlag[ i ][ j ] is set equal to 0.

For i in the range of ( defaultOutputLayerIdc  = =  2 ) ? 0 : ( vps_num_layer_sets_minus1 + 1 ) to NumOutputLayerSets − 1, inclusive, and j in the range of 0 to NumLayersInIdList[ OlsIdxToLsIdx[ i ] ] − 1, inclusive, the variable OutputLayerFlag[ i ][ j ] is set equal to output_layer_flag[ i ][ j ].

The variable NumOutputLayersInOutputLayerSet[ i ] is derived as follows:

NumOutputLayersInOutputLayerSet[ i ] = 0
for( j = 0; j < NumLayersInIdList[ OlsIdxToLsIdx[ i ] ]; j++ ) {
	NumOutputLayersInOutputLayerSet[ i ]  +=  OutputLayerFlag[ i ][ j ] 	(F-11)
	if( OutputLayerFlag[ i ][ j ] )
		OlsHighestOutputLayerId[ i ] = LayerSetLayerIdList[ OlsIdxToLsIdx[ i ] ][ j ]

It is a requirement of bitstream conformance that NumOutputLayersInOutputLayerSet[ i ] shall be greater than 0 for i in the range of 0 to NumOutputLayerSets − 1, inclusive.

The variables NumNecessaryLayers[ olsIdx ] and NecessaryLayerFlag[ olsIdx ][ lIdx ] are derived as follows:

for( olsIdx = 0; olsIdx < NumOutputLayerSets; olsIdx++ ) {
	lsIdx = OlsIdxToLsIdx[ olsIdx ]
	for( lsLayerIdx = 0; lsLayerIdx < NumLayersInIdList[ lsIdx ]; lsLayerIdx++ )
		NecessaryLayerFlag[ olsIdx ][ lsLayerIdx ] = 0
	for( lsLayerIdx = 0; lsLayerIdx < NumLayersInIdList[ lsIdx ]; lsLayerIdx++ )
		if( OutputLayerFlag[ olsIdx ][ lsLayerIdx ] ) {
			NecessaryLayerFlag[ olsIdx ][ lsLayerIdx ] = 1	(F-12)
			currLayerId = LayerSetLayerIdList[ lsIdx ][ lsLayerIdx ]
			for( rLsLayerIdx = 0; rLsLayerIdx < lsLayerIdx; rLsLayerIdx++ ) {
				refLayerId = LayerSetLayerIdList[ lsIdx ][ rLsLayerIdx ]
				if( DependencyFlag[ LayerIdxInVps[ currLayerId ] ][ LayerIdxInVps[ refLayerId ] ] )
					NecessaryLayerFlag[ olsIdx ][ rLsLayerIdx ] = 1
		}
	NumNecessaryLayers[ olsIdx ] = 0
	for( lsLayerIdx = 0; lsLayerIdx < NumLayersInIdList[ lsIdx ]; lsLayerIdx++ )
		NumNecessaryLayers[ olsIdx ]  +=  NecessaryLayerFlag[ olsIdx ][ lsLayerIdx ]
}

It is a requirement of bitstream conformance that for each layer index layerIdx in the range of ( vps_base_layer_internal_flag ? 0 : 1 ) to MaxLayersMinus1, inclusive, there shall be at least one OLS with index olsIdx such that NecessaryLayerFlag[ olsIdx ][ lsLayerIdx ] is equal to 1 for the value of lsLayerIdx for which LayerSetLayerIdList[ OlsIdxToLsIdx[ olsIdx ] ][ lsLayerIdx ] is equal to layer_id_in_nuh[ layerIdx ].

NOTE 6 – In other words, each layer has to be an output layer or a reference layer of an output layer in at least one OLS.

It is a requirement of bitstream conformance that when num_add_layer_sets is greater than 0 and olsIdx has any such value that OlsIdxToLsIdx[ olsIdx ] is in the range of FirstAddLayerSetIdx to LastAddLayerSetIdx, inclusive, and NumNecessaryLayers[ olsIdx ] is equal to 1, NecessaryLayerFlag[ olsIdx ][ 0 ] shall be equal to 1.

NOTE 7 – In other words, when an additional layer set has exactly one necessary layer, the necessary layer is required to be the layer with the smallest nuh_layer_id value within the additional layer set.

profile_tier_level_idx[ i ][ j ] specifies the index, into the list of profile_tier_level( ) syntax structures in the VPS, of the profile_tier_level( ) syntax structure that applies to the j-th layer of the i-th OLS as specified in clause A.7.4.4. The length of the profile_tier_level_idx[ i ][ j ] syntax element is Ceil( Log2( vps_num_profile_tier_level_minus1 + 1 ) ) bits.

When NecessaryLayerFlag[ i ][ j ] is equal to 1 and vps_num_profile_tier_level_minus1 is equal to 0, the value of profile_tier_level_idx[ i ][ j ] is inferred to be equal to 0.

When vps_base_layer_internal_flag is equal to 1, the following applies:

–	If vps_max_layers_minus1 is greater than 0, the value of profile_tier_level_idx[ 0 ][ 0 ] is inferred to be equal to 1.

–	Otherwise (vps_max_layers_minus1 is equal to 0), the value of profile_tier_level_idx[ 0 ][ 0 ] is inferred to be equal to 0.

When present, the value of profile_tier_level_idx[ i ][ j ] shall be in the range of ( vps_base_layer_internal_flag ? 0 : 1 ) to vps_num_profile_tier_level_minus1, inclusive.

alt_output_layer_flag[ i ] equal to 0 specifies that an alternative output layer is not used for any output layer in the i-th OLS. alt_output_layer_flag[ i ] equal to 1 specifies that an alternative output layer may be used for the output layer in the i-th OLS.

When not present, the value of alt_output_layer_flag[ i ] is inferred to be equal to 0.

NOTE 9 – When alt_output_layer_flag[ olsIdx ] is equal to 0, pictures that do not belong to the output layers of the OLS with index olsIdx are not output. When alt_output_layer_flag[ olsIdx ] is equal to 1 and a picture belonging to the output layer of the OLS with index olsIdx is not present in an access unit or has PicOutputFlag equal to 0, a picture with the highest nuh_layer_id among those pictures of the access unit for which PicOutputFlag is equal to 1 and having the nuh_layer_id value among the nuh_layer_id values of the reference layers of the output layer is output.

For each value of olsIdx in the range of 0 to NumOutputLayerSets − 1, inclusive, the following applies:

–	When alt_output_layer_flag[ olsIdx ] is equal to 1, the value of pic_output_flag shall be the same in the slice headers of an access unit that have nuh_layer_id value equal to OlsHighestOutputLayerId[ olsIdx ] or equal to the nuh_layer_id value of any reference layer of the layer with nuh_layer_id equal to OlsHighestOutputLayerId[ olsIdx ].

–	The variable olsBitstream is derived as follows:

–	Let lsIdx be equal to OlsIdxToLsIdx[ olsIdx ].

–	If lsIdx is less than or equal to vps_num_layer_sets_minus1, let olsBitstream be the output of the sub-bitstream extraction process specified in clause A.10.1 with the following inputs: the current bitstream, tIdTarget equal to 6 and layerIdListTarget equal to LayerSetLayerIdList[ lsIdx ].

–	Otherwise, let olsBitstream be the output of the sub-bitstream extraction process specified in clause A.10.3 with the following inputs: the current bitstream, tIdTarget equal to 6 and layerIdListTarget equal to LayerSetLayerIdList[ lsIdx ].

–	Let truncatedOlsBitstream be olsBitstream or be formed from the olsBitstream by removing access units preceding, in decoding order, any access unit with an IRAP picture having nuh_layer_id equal to SmallestLayerId.

–	It is a requirement of bitstream conformance that when alt_output_layer_flag[ olsIdx ] is equal to 1, a bitstream that is formed by removing, from the truncatedOlsBitstream, any coded picture that is not used as a reference for prediction for any other picture and is not the only coded picture of an access unit is a conforming bitstream.

NOTE 10 – When alt_output_layer_flag[ olsIdx ] is equal to 1, encoders are required to set the values of max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] such that these values suffice also when pictures of an alternative output layer are marked as "needed for output" in the HRD.

vps_num_rep_formats_minus1 plus 1 specifies the number of the following rep_format( ) syntax structures in the VPS. The value of vps_num_rep_formats_minus1 shall be in the range of 0 to 255, inclusive.

rep_format_idx_present_flag equal to 1 specifies that the syntax elements vps_rep_format_idx[ i ] are present. rep_format_idx_present_flag equal to 0 specifies that the syntax elements vps_rep_format_idx[ i ] are not present. When not present, the value of rep_format_idx_present_flag is inferred to be equal to 0.

vps_rep_format_idx[ i ] specifies the index, into the list of rep_format( ) syntax structures in the VPS, of the rep_format( ) syntax structure that applies to the layer with nuh_layer_id equal to layer_id_in_nuh[ i ]. When not present, the value of vps_rep_format_idx[ i ] is inferred to be equal to Min( i, vps_num_rep_formats_minus1 ). The value of vps_rep_format_idx[ i ] shall be in the range of 0 to vps_num_rep_formats_minus1, inclusive. The number of bits used for the representation of vps_rep_format_idx[ i ] is Ceil( Log2( vps_num_rep_formats_minus1 + 1 ) ).

max_one_active_ref_layer_flag equal to 1 specifies that at most one picture is used for inter-layer prediction for each picture in the CVS. max_one_active_ref_layer_flag equal to 0 specifies that more than one picture may be used for inter-layer prediction for each picture in the CVS.

vps_poc_lsb_aligned_flag equal to 0 specifies that the value of slice_pic_order_cnt_lsb may or may not be the same in different pictures of an access unit. vps_poc_lsb_aligned_flag equal to 1 specifies that the value of slice_pic_order_cnt_lsb is the same in all pictures of an access unit. Additionally, the value of vps_poc_lsb_aligned_flag affects the decoding process for picture order count in clause A.8.3.1. When not present, the value of vps_poc_lsb_aligned_flag is inferred to be equal to 0.

poc_lsb_not_present_flag[ i ] equal to 1 specifies that the slice_pic_order_cnt_lsb syntax element is not present in the slice headers of IDR pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] in the CVS. poc_lsb_not_present_flag[ i ] equal to 0 specifies that slice_pic_order_cnt_lsb syntax element may or may not be present in the slice headers of IDR pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] in the CVS. When not present, the value of poc_lsb_not_present_flag[ i ] is inferred to be equal to 0.

direct_dep_type_len_minus2 plus 2 specifies the number of bits of the direct_dependency_type[ i ][ j ] and the direct_dependency_all_layers_type syntax elements. In bitstreams conforming to this version of this Specification the value of direct_dep_type_len_minus2 shall be equal 0. Although the value of direct_dep_type_len_minus2 shall be equal to 0 in this version of this Specification, decoders shall allow other values of direct_dep_type_len_minus2 in the range of 0 to 30, inclusive, to appear in the syntax.

direct_dependency_all_layers_flag equal to 1 specifies that the syntax element direct_dependency_type[ i ][ j ] is not present and inferred from direct_dependency_all_layers_type. direct_dependency_all_layers_flag equal to 0 indicates that the syntax element direct_dependency_type[ i ][ j ] is present.

direct_dependency_all_layers_type, when present, specifies the inferred value of direct_dependency_type[ i ][ j ] for all combinations of i-th and j-th layers. The length of the direct_dependency_all_layers_type syntax element is direct_dep_type_len_minus2 + 2 bits. Although the value of direct_dependency_all_layers_type is required to be in the range of 0 to 2, inclusive, in this version of this Specification, decoders shall allow values of direct_dependency_all_layers_type in the range of 3 to 232 − 2, inclusive, to appear in the syntax.

direct_dependency_type[ i ][ j ] indicates the type of dependency between the layer with nuh_layer_id equal layer_id_in_nuh[ i ] and the layer with nuh_layer_id equal to layer_id_in_nuh[ j ]. direct_dependency_type[ i ][ j ] equal to 0 specifies that the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] may be used for inter-layer sample prediction but is not used for inter-layer motion prediction of the layer with nuh_layer_id equal layer_id_in_nuh[ i ]. direct_dependency_type[ i ][ j ] equal to 1 specifies that the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] may be  used for inter-layer motion prediction but is not used for inter-layer sample prediction of the layer with nuh_layer_id equal layer_id_in_nuh[ i ]. direct_dependency_type[ i ][ j ] equal to 2 specifies that the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] may be used for both inter-layer motion prediction and inter-layer sample prediction of the layer with nuh_layer_id equal layer_id_in_nuh[ i ]. The length of the direct_dependency_type[ i ][ j ] syntax element is direct_dep_type_len_minus2 + 2 bits. Although the value of direct_dependency_type[ i ][ j ] shall be in the range of 0 to 2, inclusive, in this version of this Specification, decoders shall allow values of direct_dependency_type[ i ][ j ] in the range of 3 to 232 − 2, inclusive, to appear in the syntax.

When direct_dependency_all_layers_flag is equal to 1, for any i in the range of ( 1 − vps_base_layer_internal_flag ) + 1 to MaxLayersMinus1, inclusive, and any j in the range of 1 − vps_base_layer_internal_flag to i – 1, inclusive, when direct_dependency_flag[ i ][ j ] is equal to 1, the value of direct_dependency_type[ i ][ j ] is inferred to be equal to direct_dependency_all_layers_type.

When vps_base_layer_internal_flag is equal to 0, for any i in the range of 1 to MaxLayersMinus1, inclusive, when direct_dependency_flag[ i ][ 0 ] is equal to 1, the value of direct_dependency_type[ i ][ 0 ] is inferred to be equal to 0.

The variables VpsInterLayerSamplePredictionEnabled[ i ][ j ] and VpsInterLayerMotionPredictionEnabled[ i ][ j ] are derived as follows:

if( direct_dependency_flag[ i ][ j ] )
	VpsInterLayerSamplePredictionEnabled[ i ][ j ] = ( direct_dependency_type[ i ][ j ] + 1 ) & 0x1	-
else
	VpsInterLayerSamplePredictionEnabled[ i ][ j ] = 0	(F-14)

if( direct_dependency_flag[ i ][ j ] )
	VpsInterLayerMotionPredictionEnabled[ i ][ j ] = ( ( direct_dependency_type[ i ][ j ] + 1 ) & 0x2 ) ? 1 : 0	-
else
	VpsInterLayerMotionPredictionEnabled[ i ][ j ] = 0

vps_non_vui_extension_length specifies the length of the non-VUI VPS extension data following this syntax element and before vps_vui_present_flag, in bytes. The value of vps_non_vui_extension_length shall be in the range of 0 to 4096, inclusive.

vps_non_vui_extension_data_byte may have any value. Decoders shall ignore the value of vps_non_vui_extension_data_byte. Its value does not affect decoder conformance to profiles specified in this version of this Specification.

vps_vui_present_flag equal to 1 specifies that the vps_vui( ) syntax structure is present in the VPS. vps_vui_present_flag equal to 0 specifies that the vps_vui( ) syntax structure is not present in the VPS.

vps_vui_alignment_bit_equal_to_one shall be equal to 1.

          - Representation format semantics

chroma_and_bit_depth_vps_present_flag equal to 1 specifies that the syntax elements chroma_format_vps_idc, bit_depth_vps_luma_minus8, and bit_depth_vps_chroma_minus8 are present and that the syntax element separate_colour_plane_vps_flag may be present in the rep_format( ) structure. chroma_and_bit_depth_vps_present_flag equal to 0 specifies that the syntax elements chroma_format_vps_idc, separate_colour_plane_vps_flag, bit_depth_vps_luma_minus8, and bit_depth_vps_chroma_minus8 are not present and are inferred from the previous rep_format( ) syntax structure in the VPS. The value of chroma_and_bit_depth_vps_present_flag of the first rep_format( ) syntax structure in the VPS shall be equal to 1.

pic_width_vps_in_luma_samples, pic_height_vps_in_luma_samples, chroma_format_vps_idc, separate_colour_plane_vps_flag, bit_depth_vps_luma_minus8, and bit_depth_vps_chroma_minus8 are used for inference of the values of the SPS syntax elements pic_width_in_luma_samples, pic_height_in_luma_samples, chroma_format_idc, separate_colour_plane_flag, bit_depth_luma_minus8, and bit_depth_chroma_minus8, respectively, for each SPS that refers to the VPS. When not present in the i-th rep_format( ) syntax structure in the VPS, the value of each of these syntax elements is inferred to be equal to the value of the corresponding syntax element in the (i − 1)-th rep_format( ) syntax structure in the VPS. pic_width_vps_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY. pic_height_vps_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY. The value of chroma_format_vps_idc shall be in the range of 0 to 3, inclusive. bit_depth_vps_luma_minus8 shall be in the range of 0 to 8, inclusive. bit_depth_vps_chroma_minus8 shall be in the range of 0 to 8, inclusive.

conformance_window_vps_flag equal to 1 specifies that the syntax elements conf_win_vps_left_offset, conf_win_vps_right_offset, conf_win_vps_top_offset, and conf_win_vps_bottom_offset follow next in the rep_format( ) structure. conformance_window_vps_flag equal to 0 specifies that the syntax elements conf_win_vps_left_offset, conf_win_vps_right_offset, conf_win_vps_top_offset, and conf_win_vps_bottom_offset are not present.

conf_win_vps_left_offset, conf_win_vps_right_offset, conf_win_vps_top_offset, and conf_win_vps_bottom_offset are used for inference of the values of the SPS syntax elements, conf_win_left_offset, conf_win_right_offset, conf_win_top_offset, and conf_win_bottom_offset, respectively, for each SPS that refers to the VPS. When not present, the values of conf_win_vps_left_offset, conf_win_vps_right_offset, conf_win_vps_top_offset, and conf_win_vps_bottom_offset are inferred to be equal to 0.

The value of SubWidthC * ( conf_win_vps_left_offset + conf_win_vps_right_offset ) shall be less than pic_width_vps_in_luma_samples, and the value of SubHeightC * ( conf_win_vps_top_offset + conf_win_vps_bottom_offset ) shall be less than pic_height_vps_in_luma_samples.

          - DPB size semantics

For the lsIdx-th layer set, the number of sub-DPBs is NumLayersInIdList[ lsIdx ], and for each layer with a particular value of nuh_layer_id in the layer set, the sub-DPB with index layerIdx is assigned, where LayerSetLayerIdList[ lsIdx ][ layerIdx ] is equal to nuh_layer_id.

- sub_layer_flag_info_present_flag[ i ] equal to 1 specifies that sub_layer_dpb_info_present_flag[ i ][ j ] is present for j in the range of 1 to MaxSubLayersInLayerSetMinus1[ OlsIdxToLsIdx[ i ] ], inclusive. sub_layer_flag_info_present_flag[ i ] equal to 0 specifies that, for each value of j greater than 0, sub_layer_dpb_info_present_flag[ i ][ j ] is not present.

- sub_layer_dpb_info_present_flag[ i ][ j ] equal to 1 specifies that max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] may be present for k in the range of 0 to NumLayersInIdList[ OlsIdxToLsIdx[ i ] ] − 1, inclusive, for the j-th sub-layer of the i-th OLS, and max_vps_num_reorder_pics[ i ][ j ] and max_vps_latency_increase_plus1[ i ][ j ] are present for the j-th sub-layer of the i-th OLS. sub_layer_dpb_info_present_flag[ i ][ j ] equal to 0 specifies that max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] for k in the range of 0 to NumLayersInIdList[ OlsIdxToLsIdx[ i ] ] − 1, inclusive, max_vps_num_reorder_pics[ i ][ j ], and max_vps_latency_increase_plus1[ i ][ j ] are not present. The value of sub_layer_dpb_info_present_flag[ i ][ 0 ] for any possible value of i is inferred to be equal to 1. When not present, the value of sub_layer_dpb_info_present_flag[ i ][ j ] for j greater than 0 and any possible value of i, is inferred to be equal to be equal to 0.

max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] plus 1, when NecessaryLayerFlag[ i ][ k ] is equal to 1, specifies the maximum number of decoded pictures, of the k-th layer for the CVS in the i-th OLS, that need to be stored in the DPB when HighestTid is equal to j.

When NecessaryLayerFlag[ i ][ k ] is equal to 1, the following applies for i in the range of 1 to NumOutputLayerSets − 1, inclusive:

–	When j is greater than 0, max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] shall be greater than or equal to max_vps_dec_pic_buffering_minus1[ i ][ k ][ j − 1 ].

–	When max_vps_dec_pic_buffering_minus1[ i ][ 0 ][ 0 ] is not present, it is inferred to be equal to 0.

–	When max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] is not present for j in the range of 1 to MaxSubLayersInLayerSetMinus1[ OlsIdxToLsIdx[ i ] ], inclusive, it is inferred to be equal to max_vps_dec_pic_buffering_minus1[ i ][ k ][ j − 1 ].

- max_vps_num_reorder_pics[ i ][ j ] specifies, when HighestTid is equal to j, the maximum allowed number of access units containing a picture with PicOutputFlag equal to 1 that can precede any access unit auA that contains a picture with PicOutputFlag equal to 1 in the i-th OLS in the CVS in decoding order and follow the access unit auA that contains a picture with PicOutputFlag equal to 1 in output order. When max_vps_num_reorder_pics[ i ][ j ] is not present for j in the range of 1 to MaxSubLayersInLayerSetMinus1[ OlsIdxToLsIdx[ i ] ], inclusive, due to sub_layer_dpb_info_present_flag[ i ][ j ] being equal to 0, it is inferred to be equal to max_vps_num_reorder_pics[ i ][ j − 1 ].

- max_vps_latency_increase_plus1[ i ][ j ] not equal to 0 is used to compute the value of VpsMaxLatencyPictures[ i ][ j ], which, when HighestTid is equal to j, specifies the maximum number of access units containing a picture with PicOutputFlag equal to 1 in the i-th OLS that can precede any access unit auA that contains a picture with PicOutputFlag equal to 1 in the CVS in output order and follow the access unit auA that contains a picture with PicOutputFlag equal to 1 in decoding order. When max_vps_latency_increase_plus1[ i ][ j ] is not present for j in the range of 1 to MaxSubLayersInLayerSetMinus1[ OlsIdxToLsIdx[ i ] ], inclusive, due to sub_layer_dpb_info_present_flag[ i ][ j ] being equal to 0, it is inferred to be equal to max_vps_latency_increase_plus1[ i ][ j − 1 ].

- When max_vps_latency_increase_plus1[ i ][ j ] is not equal to 0, the value of VpsMaxLatencyPictures[ i ][ j ] is specified as follows:

VpsMaxLatencyPictures[ i ][ j ] = max_vps_num_reorder_pics[ i ][ j ] +
	max_vps_latency_increase_plus1[ i ][ j ] − 1		(F-16)

- When max_vps_latency_increase_plus1[ i ][ j ] is equal to 0, no corresponding limit is expressed. The value of max_vps_latency_increase_plus1[ i ][ j ] shall be in the range of 0 to 232 − 2, inclusive.

          - VPS VUI semantics

NOTE 1 – When vps_vui_present_flag is equal to 0, some of the VPS VUI flags, such as cross_layer_pic_type_aligned_flag, tiles_not_in_use_flag, and wpp_not_in_use_flag, are not present and no value is inferred for them. In this case, the semantics of these flags are unspecified, and it should be interpreted as such that it is unknown whether the constraints associated with these flags being equal to 1 apply or not; in other words, in this case those constraints may or may not apply.

cross_layer_pic_type_aligned_flag equal to 1 specifies that within a CVS that refers to the VPS, all VCL NAL units that belong to an access unit have the same value of nal_unit_type. cross_layer_pic_type_aligned_flag equal to 0 specifies that within a CVS that refers to the VPS, all VCL NAL units in each access unit may or may not have the same value of nal_unit_type.

cross_layer_irap_aligned_flag equal to 1 specifies that IRAP pictures in the CVS are cross-layer aligned, i.e., when a picture pictureA of a layer layerA in an access unit is an IRAP picture, each picture pictureB in the same access unit that belongs to a direct reference layer of layerA or that belongs to a layer for which layerA is a direct reference layer of that layer is an IRAP picture and the VCL NAL units of pictureB have the same value of nal_unit_type as that of pictureA. cross_layer_irap_aligned_flag equal to 0 specifies that the above restriction may or may not apply. When not present, the value of cross_layer_irap_aligned_flag is inferred to be equal to vps_vui_present_flag.

all_layers_idr_aligned_flag equal to 1 indicates that within each access unit for which the VCL NAL units refer to the VPS, when one picture is an IRAP picture, all the pictures in the same access unit are IDR pictures and have the same value of nal_unit_type. all_layers_idr_aligned_flag equal to 0 specifies that the above restriction may or may not apply. When not present, the value of all_layers_idr_aligned_flag is inferred to be equal to 0.

bit_rate_present_vps_flag equal to 1 specifies that the syntax element bit_rate_present_flag[ i ][ j ] is present. bit_rate_present_vps_flag equal to 0 specifies that the syntax element bit_rate_present_flag[ i ][ j ] is not present.

pic_rate_present_vps_flag equal to 1 specifies that the syntax element pic_rate_present_flag[ i ][ j ] is present. pic_rate_present_vps_flag equal to 0 specifies that the syntax element pic_rate_present_flag[ i ][ j ] is not present.

bit_rate_present_flag[ i ][ j ] equal to 1 specifies that the bit rate information for the j-th subset of the i-th layer set is present. bit_rate_present_flag[ i ] equal to 0 specifies that the bit rate information for the j-th subset of the i-th layer set is not present. The j-th subset of a layer set is derived as follows:

–	If i is less than or equal to vps_num_layer_sets_minus1, the j-th subset of a layer set is the output of the sub-bitstream extraction process specified in clause A.10.1, when it is invoked with inBitstream equal to the layer set, tIdTarget equal to j, and layerIdListTarget equal to the layer identifier list associated with the layer set as inputs.

–	Otherwise (i is greater than vps_num_layer_sets_minus1), the j-th subset of a layer set is the output of the sub-bitstream extraction process specified in clause A.10.3, when it is invoked with inBitstream equal to the layer set, tIdTarget equal to j, and layerIdListTarget equal to the layer identifier list associated with the layer set as inputs.

When not present, the value of bit_rate_present_flag[ i ][ j ] is inferred to be equal to 0.

pic_rate_present_flag[ i ][ j ] equal to 1 specifies that picture rate information for the j-th subset of the i-th layer set is present. pic_rate_present_flag[ i ][ j ] equal to 0 specifies that picture rate information for the j-th subset of the i-th layer set is not present. When not present, the value of pic_rate_present_flag[ i ][ j ] is inferred to be equal to 0.

avg_bit_rate[ i ][ j ] indicates the average bit rate of the j-th subset of the i-th layer set, in bits per second. The value is given by BitRateBPS( avg_bit_rate[ i ][ j ] ) with the function BitRateBPS( ) being specified as follows:

BitRateBPS( x ) = ( x & ( 214 − 1 ) ) * 10( 2 + ( x  >>  14 ) )		(F-17)

The average bit rate is derived according to the access unit removal time specified in clause A.13. In the following, bTotal is the number of bits in all NAL units of the j-th subset of the i-th layer set, t1 is the removal time (in seconds) of the first access unit to which the VPS applies, and t2 is the removal time (in seconds) of the last access unit (in decoding order) to which the VPS applies. With x specifying the value of avg_bit_rate[ i ][ j ], the following applies:

–	If t1 is not equal to t2, the following condition shall be true:

( x & ( 214 − 1 ) )  = =  Round( bTotal ÷ ( ( t2 − t1 ) * 10( 2 + ( x  >>  14 ) ) ) )	(F-18)

–	Otherwise (t1 is equal to t2), the following condition shall be true:

( x & ( 214 − 1 ) )  = =  0		(F-19)

max_bit_rate_layer[ i ][ j ] indicates an upper bound for the bit rate of the j-th subset of the i-th layer set in any one-second time window of access unit removal time as specified in clause A.13. The upper bound for the bit rate in bits per second is given by BitRateBPS( max_bit_rate_layer[ i ][ j ] ). The bit rate values are derived according to the access unit removal time specified in clause A.13. In the following, t1 is any point in time (in seconds), t2 is set equal to t1 + 1 ÷ 100, and bTotal is the number of bits in all NAL units of access units with a removal time greater than or equal to t1 and less than t2. With x specifying the value of max_bit_rate_layer[ i ][ j ], the following condition shall be obeyed for all values of t1:

( x & ( 214 − 1 ) )  >=  bTotal ÷ ( ( t2 − t1 ) * 10( 2 + ( x  >>  14 ) ) )	(F-20)

constant_pic_rate_idc[ i ][ j ] indicates whether the picture rate of the j-th subset of the i-th layer set is constant. In the following, a temporal segment tSeg is any set of two or more consecutive access units, in decoding order, of the j-th subset of the i-th layer set, auTotal( tSeg ) is the number of access units in the temporal segment tSeg, t1( tSeg ) is the removal time (in seconds) of the first access unit (in decoding order) of the temporal segment tSeg, t2( tSeg ) is the removal time (in seconds) of the last access unit (in decoding order) of the temporal segment tSeg, and avgPicRate( tSeg ) is the average picture rate in the temporal segment tSeg, and is specified as follows:

avgPicRate( tSeg ) = Round( auTotal( tSeg ) * 256 ÷ ( t2( tSeg ) − t1( tSeg ) ) )	(F-21)

If the j-th subset of the i-th layer set only contains one or two access units or the value of avgPicRate( tSeg ) is constant over all the temporal segments, the picture rate is constant; otherwise, the picture rate is not constant.

constant_pic_rate_idc[ i ][ j ] equal to 0 indicates that the picture rate of the j-th subset of the i-th layer set is not constant. constant_pic_rate_idc[ i ][ j ] equal to 1 indicates that the picture rate of the j-th subset of the i-th layer set is constant. constant_pic_rate_idc[ i ][ j ] equal to 2 indicates that the picture rate of the j-th subset of the i-th layer set may or may not be constant. The value of constant_pic_rate_idc[ i ][ j ] shall be in the range of 0 to 2, inclusive.

avg_pic_rate[ i ] indicates the average picture rate, in units of picture per 256 seconds, of the j-th subset of the layer set. With auTotal being the number of access units in the j-th subset of the i-th layer set, t1 being the removal time (in seconds) of the first access unit to which the VPS applies, and t2 being the removal time (in seconds) of the last access unit (in decoding order) to which the VPS applies, the following applies:

–	If t1 is not equal to t2, the following condition shall be true:

avg_pic_rate[ i ]  = =  Round( auTotal * 256 ÷ ( t2 − t1 ) )	(F-22)

–	Otherwise (t1 is equal to t2), the following condition shall be true:

avg_pic_rate[ i ]  = =  0		(F-23)

video_signal_info_idx_present_flag equal to 1 specifies that the syntax elements vps_num_video_signal_info_minus1 is present and the syntax element vps_video_signal_info_idx[ i ] may be present. video_signal_info_idx_present_flag equal to 0 specifies that the syntax elements vps_num_video_signal_info_minus1 and vps_video_signal_info_idx[ i ] are not present.

vps_num_video_signal_info_minus1 plus 1 specifies the number of the following video_signal_info( ) syntax structures in the VPS. When not present, the value of vps_num_video_signal_info_minus1 is inferred to be equal to MaxLayersMinus1 − ( vps_base_layer_internal_flag ? 0 : 1 ).

vps_video_signal_info_idx[ i ] specifies the index, into the list of video_signal_info( ) syntax structures in the VPS, of the video_signal_info( ) syntax structure that applies to the layer with nuh_layer_id equal to layer_id_in_nuh[ i ]. If video_signal_info_idx_present_flag is equal to 0, the value of vps_video_signal_info_idx[ i ] for each value of i in the range of ( vps_base_layer_internal_flag ? 0 : 1 ) to MaxLayersMinus1, inclusive, is inferred to be equal to i. Otherwise, when vps_num_video_signal_info_minus1 is equal to 0, the value of vps_video_signal_info_idx[ i ] for each value of i in the range of vps_base_layer_internal_flag to MaxLayersMinus1, inclusive, is inferred to be equal to 0. The value of vps_video_signal_info_idx[ i ] for each value of i in the range of ( vps_base_layer_internal_flag ? 0 : 1 ) to MaxLayersMinus1, inclusive, shall be in the range of 0 to vps_num_video_signal_info_minus1, inclusive.

tiles_not_in_use_flag equal to 1 indicates that the value of tiles_enabled_flag is equal to 0 for each PPS that is referred to by at least one picture referring to the VPS. tiles_not_in_use_flag equal to 0 indicates that such a restriction may or may not apply.

tiles_in_use_flag[ i ] equal to 1 indicates that the value of tiles_enabled_flag is equal to 1 for each PPS that is referred to by at least one picture of the i-th layer specified by the VPS. tiles_in_use_flag[ i ] equal to 0 indicates that such a restriction may or may not apply. When not present, the value of tiles_in_use_flag[ i ] is inferred to be equal to 0.

loop_filter_not_across_tiles_flag[ i ] equal to 1 indicates that the value of loop_filter_across_tiles_enabled_flag is equal to 0 for each PPS that is referred to by at least one picture of the i-th layer specified by the VPS. loop_filter_not_across_tiles_flag[ i ] equal to 0 indicates that such a restriction may or may not apply. When not present, the value of loop_filter_not_across_tiles_flag[ i ] is inferred to be equal to 0.

tile_boundaries_aligned_flag[ i ][ j ] equal to 1 indicates that, when any two samples of one picture of the i-th layer specified by the VPS belong to one tile, the two collocated samples, when both present in the picture of the j-th direct reference layer of the i-th layer, belong to one tile, and when any two samples of one picture of the i-th layer belong to different tiles, the two collocated samples, when both present in the picture of the j-th direct reference layer of the i-th layer belong to different tiles. tile_boundaries_aligned_flag equal to 0 indicates that such a restriction may or may not apply. When not present, the value of tile_boundaries_aligned_flag[ i ][ j ] is inferred to be equal to 0.

wpp_not_in_use_flag equal to 1 indicates that the value of entropy_coding_sync_enabled_flag is equal to 0 for each PPS that is referred to by at least one picture referring to the VPS. wpp_not_in_use_flag equal to 0 indicates that such a restriction may or may not apply.

wpp_in_use_flag[ i ] equal to 1 indicates that the value of entropy_coding_sync_enabled_flag is equal to 1 for each PPS that is referred to by at least one picture of the i-th layer specified by the VPS. wpp_in_use_flag[ i ] equal to 0 indicates that such a restriction may or may not apply. When not present, the value of wpp_in_use_flag[ i ] is inferred to be equal to 0.

single_layer_for_non_irap_flag equal to 1 indicates the following:

–	If vps_base_layer_internal_flag is equal to 1, single_layer_for_non_irap_flag equal to 1 indicates that either one of the following is true for each access unit for which this VPS is the active VPS:

–	All the VCL NAL units of an access unit have the same nuh_layer_id value.

–	Two nuh_layer_id values are used by the VCL NAL units of an access unit and the picture with the greater nuh_layer_id value is an IRAP picture.

–	Otherwise (vps_base_layer_internal_flag is equal to 0), single_layer_for_non_irap_flag equal to 1 indicates that any one of the following is true for each access unit for which this VPS is the active VPS:

–	The decoded picture with nuh_layer_id equal to 0 is not provided for the access unit by external means and the access unit contains one coded picture.

–	The decoded picture with nuh_layer_id equal to 0 is not provided for the access unit by external means, the access unit contains two coded pictures, and the picture with the greater nuh_layer_id value is an IRAP picture.

–	The decoded picture with nuh_layer_id equal to 0 is provided for an access unit by external means and the access unit contains one coded picture that is an IRAP picture.

single_layer_for_non_irap_flag equal to 0 indicates that the above constraints may or may not apply. When not present, the value of single_layer_for_non_irap_flag is inferred to be equal to 0.

higher_layer_irap_skip_flag equal to 1 indicates that each IRAP picture currIrapPic is constrained as specified below. currIrapPic is derived as follows for each access unit currAu for which this VPS is the active VPS:

–	If vps_base_layer_internal_flag is equal to 1, currAu contains two coded pictures, and the picture with the greater nuh_layer_id value is an IRAP picture, let currIrapPic be that IRAP picture.

–	Otherwise, if vps_base_layer_internal_flag is equal to 0, a decoded picture with nuh_layer_id equal to 0 is not provided for currAu by external means, currAu contains two coded pictures, and the picture with the greater nuh_layer_id value is an IRAP picture, let currIrapPic be that IRAP picture.

–	Otherwise, if vps_base_layer_internal_flag is equal to 0, the decoded picture with nuh_layer_id equal to 0 is provided for currAu by external means and the access unit contains one coded picture that is an IRAP picture, let currIrapPic be that IRAP picture.

–	Otherwise, currIrapPic is not derived for currAu.

The following constraints apply for each picture currIrapPic:

–	For all slices of the IRAP picture:

–	slice_type shall be equal to P.

–	slice_sao_luma_flag and slice_sao_chroma_flag shall both be equal to 0.

–	five_minus_max_num_merge_cand shall be equal to 4.

–	weighted_pred_flag shall be equal to 0 in the PPS that is referred to by the slices.

–	For all coding units of the IRAP picture:

–	cu_skip_flag[ i ][ j ] shall be equal to 1.

higher_layer_irap_skip_flag equal to 0 indicates that the above constraints may or may not apply.

When single_layer_for_non_irap_flag is equal to 0, higher_layer_irap_skip_flag shall be equal to 0. When higher_layer_irap_skip_flag is not present it is inferred to be equal to 0.

NOTE 2 – When vps_base_layer_internal_flag is equal to 1, an encoder may set single_layer_for_non_irap_flag equal to 1 as an indication to a decoder that at most two pictures are present in any access unit and whenever there are two pictures in the same access unit, the one with the higher value of nuh_layer_id is an IRAP picture. The encoder may additionally set higher_layer_irap_skip_flag equal to 1 as an indication to a decoder that whenever there are two pictures in the same access unit, the one with the higher value of nuh_layer_id is an IRAP picture for which the decoded samples can be derived by applying the inter-layer reference picture derivation process specified in clause A.8.1.4 with the other picture with the lower value of nuh_layer_id as input.

ilp_restricted_ref_layers_flag equal to 1 indicates that additional restrictions on inter-layer prediction as specified below apply for each direct reference layer of each layer specified by the VPS. ilp_restricted_ref_layers_flag equal to 0 indicates that additional restrictions on inter-layer prediction may or may not apply. When not present, the value of ilp_restricted_ref_layers_flag is inferred to be equal to 0.

min_spatial_segment_offset_plus1[ i ][ j ] indicates the spatial region, in each picture of the j-th direct reference layer of the i-th layer, that is not used for inter-layer prediction for decoding of any picture of the i-th layer, by itself or together with min_horizontal_ctu_offset_plus1[ i ][ j ], as specified below.

The variables refLayerPicWidthInCtbsY[ i ][ j ] and refLayerPicHeightInCtbsY[ i ][ j ] are set equal to PicWidthInCtbsY and PicHeightInCtbsY, respectively, of the j-th direct reference layer of the i-th layer.

The value of min_spatial_segment_offset_plus1[ i ][ j ] shall be in the range of 0 to refLayerPicWidthInCtbsY[ i ][ j ] * refLayerPicHeightInCtbsY[ i ][ j ], inclusive. When not present, the value of min_spatial_segment_offset_plus1[ i ][ j ] is inferred to be equal to 0.

ctu_based_offset_enabled_flag[ i ][ j ] equal to 1 specifies that the spatial region, in units of CTUs, in each picture of the j-th direct reference layer of the i-th layer, that is not used for inter-layer prediction for decoding of any picture of the i-th layer is indicated by min_spatial_segment_offset_plus1[ i ][ j ] and min_horizontal_ctu_offset_plus1[ i ][ j ] together. ctu_based_offset_enabled_flag[ i ][ j ] equal to 0 specifies that the spatial region, in units of slice segments, tiles, or CTU rows, in each picture of the j-th direct reference layer of the i-th layer, that is not used for inter-layer prediction for decoding of any picture of the i-th layer is indicated by min_spatial_segment_offset_plus1[ i ] only. When not present, the value of ctu_based_offset_enabled_flag[ i ] is inferred to be equal to 0.

min_horizontal_ctu_offset_plus1[ i ][ j ], when ctu_based_offset_enabled_flag[ i ][ j ] is equal to 1, indicates the spatial region, in each picture of the j-th direct reference layer of the i-th layer, that is not used for inter-layer prediction for decoding of any picture of the i-th layer, together with min_spatial_segment_offset_plus1[ i ][ j ], as specified below. The value of min_horizontal_ctu_offset_plus1[ i ][ j ] shall be in the range of 0 to refLayerPicWidthInCtbsY[ i ][ j ], inclusive.

When ctu_based_offset_enabled_flag[ i ][ j ] is equal to 1, the variable minHorizontalCtbOffset[ i ][ j ] is derived as follows:

minHorizontalCtbOffset[ i ][ j ] = ( min_horizontal_ctu_offset_plus1[ i ][ j ] > 0 ) ?	(FF-24)
					( min_horizontal_ctu_offset_plus1[ i ][ j ] − 1 ) : ( refLayerPicWidthInCtbsY[ i ][ j ] − 1 )

For any i in the range of 1 to MaxLayersMinus1, inclusive, and any j in the range of 0 to NumDirectRefLayers[ layer_id_in_nuh[ i ] ], inclusive, when min_spatial_segment_offset_plus1[ i ][ j ] is greater than 0, it is a requirement of bitstream conformance that the following shall apply:

–	Let picA be a picture in the i-th layer, and let picB be the direct reference layer picture of picA with nuh_layer_id equal to IdDirectRefLayer[ layer_id_in_nuh[ i ] ][ j ]. The variable colCtbAddr[ i ][ j ] that denotes the raster scan address of the collocated CTU, in the direct reference layer picture picB, of the CTU with raster scan address equal to ctbAddr[ i ] in picA is derived as specified in the following:

–	The variables curPicWidthY, curPicHeightY, curCtbLog2SizeY and curPicWidthInCtbsY are set equal to pic_width_in_luma_samples, pic_height_in_luma_samples, CtbLog2SizeY and PicWidthInCtbsY, respectively, of the i-th layer picture picA.

–	The variables refPicWidthY, refPicHeightY, refCtbLog2SizeY, and refPicWidthInCtbsY are set equal to pic_width_in_luma_samples, pic_height_in_luma_samples, CtbLog2SizeY, and PicWidthInCtbsY, respectively, of the direct reference layer picture picB.

–	If scaled_ref_layer_offset_present_flag[ IdDirectRefLayer[ layer_id_in_nuh[ i ] ][ j ] ] in the PPS referred to by the i-th layer picture picA is equal to 1, the variables scaledLeftOffset, scaledTopOffset, scaledRightOffset and scaledBottomOffset are set equal to the left scaled reference layer offset ScaledRefLayerLeftOffset, the top scaled reference layer offset ScaledRefLayerTopOffset, the right scaled reference layer offset ScaledRefLayerRightOffset and the bottom scaled reference layer offset ScaledRefLayerBottomOffset, respectively, for the direct reference layer picture picB of the i-th layer picture picA.

–	Otherwise (scaled_ref_layer_offset_present_flag[ IdDirectRefLayer[ layer_id_in_nuh[ i ] ][ j ] ] is equal to 0), the variables scaledLeftOffset, scaledTopOffset, scaledRightOffset and scaledBottomOffset are set equal to 0.

–	If ref_region_offset_present_flag[ IdDirectRefLayer[ layer_id_in_nuh[ i ] ][ j ] ] in the PPS referred to by the i-th layer picture picA is equal to 1, the variables refRegionLeftOffset, refRegionTopOffset, refRegionRightOffset and refRegionBottomOffset are set equal to the left reference region offset RefLayerRegionLeftOffset, the top reference region offset RefLayerRegionTopOffset, the right reference region offset RefLayerRegionRightOffset and the bottom reference region offset RefLayerRegionBottomOffset, respectively, for the direct reference layer picture picB of the i-th layer picture picA.

–	Otherwise (ref_region_offset_present_flag[ IdDirectRefLayer[ layer_id_in_nuh[ i ] ][ j ] ] is equal to 0), the variables refRegionLeftOffset, refRegionTopOffset, refRegionRightOffset and refRegionBottomOffset are set equal to 0.

–	The variables ( xP, yP ) specifying the location of the top-left luma sample of the CTU with raster scan address equal to ctbAddr relative to top-left luma sample in picA are derived as follows:

xP = ( ctbAddr[ i ] % curPicWidthInCtbsY )  <<  curCtbLog2SizeY	(FF-25)

yP = ( ctbAddr[ i ] / curPicWidthInCtbsY )  <<  curCtbLog2SizeY	(FF-26)

–	The variables ( xPCol, yPCol ) specifying the collocated luma sample location in picB of the luma sample location ( xP, yP ) in picA are derived as follows:

refRegionWidth = refPicWidthY − refRegionLeftOffset – refRegionRightOffset 	(FF-27)

refRegionHeight = refPicHeightY − refRegionTopOffset – refRegionBottomOffset 	(FF-28)

scaledRefRegionWidth = curPicHeightY − scaledLeftOffset – scaledRightOffset 	(FF-29)

scaledRefRegionHeight = curPicHeightY − scaledTopOffset – scaledBottomOffset 	(FF-30)

scaleX = ( ( refRegionWidth  <<  16 ) + 	( scaledRefRegionWidth  >>  1 ) ) /
				scaledRefRegionWidth		(FF-31)

scaleY = ( ( refRegionHeight  <<  16 ) + 	( scaledRefRegionHeight >>  1 ) ) /
				scaledRefRegionHeight		(FF-32)

xPCol = Clip3( 0, ( refPicWidthY − 1 ),		(F-33)
			( ( ( ( xP − scaledLeftOffset ) * scaleX + ( 1  <<  15 ) )  >>  16 ) + refRegionLeftOffset ) )	-)

yPCol = Clip3( 0, ( refPicHeightY − 1 ),		(F-35)
			( ( ( ( yP − scaledTopOffset ) * scaleY + ( 1  <<  15 ) )  >>  16 ) + refRegionTopOffset ) )	-)

–	The variable colCtbAddr[ i ][ j ] is derived as follows:

xColCtb[ i ][ j ] = xPCol  >>  refCtbLog2SizeY		(FF-37)

yColCtb[ i ][ j ] = yPCol  >>  refCtbLog2SizeY		(FF-38)

colCtbAddr[ i ][ j ] = xColCtb[ i ][ j ] + ( yColCtb[ i ][ j ] * refPicWidthInCtbsY )	(FF-39)

- If ctu_based_offset_enabled_flag[ i ][ j ] is equal to 0, exactly one of the following applies:

- In each PPS referred to by a picture in the j-th direct reference layer of the i-th layer, tiles_enabled_flag is equal to 0 and entropy_coding_sync_enabled_flag is equal to 0, and the following applies:

- Let slice segment A be any slice segment of a picture of the i-th layer and ctbAddr[ i ] be the raster scan address of the last CTU in slice segment A. Let slice segment B be the slice segment that belongs to the same access unit as slice segment A, belongs to the j-th direct reference layer of the i-th layer, and contains the CTU with raster scan address colCtbAddr[ i ][ j ]. Let slice segment C be the slice segment that is in the same picture as slice segment B and follows slice segment B in decoding order, and between slice segment B and slice segment C there are min_spatial_segment_offset_plus1[ i ] − 1 slice segments in decoding order. When slice segment C is present, the syntax elements of slice segment A are constrained such that no sample or syntax elements values in slice segment C or any slice segment of the same picture following slice segment C in decoding order are used for inter-layer prediction in the decoding process of any samples within slice segment A.

- In each PPS referred to by a picture in the j-th direct reference layer of the i-th layer, tiles_enabled_flag is equal to 1 and entropy_coding_sync_enabled_flag is equal to 0, and the following applies:

- Let tile A be any tile in any picture picA of the i-th layer and ctbAddr[ i ] be the raster scan address of the last CTU in tile A. Let tile B be the tile that is in the picture picB belonging to the same access unit as picA and belonging to the j-th direct reference layer of the i-th layer and that contains the CTU with raster scan address colCtbAddr[ i ][ j ]. Let tile C be the tile that is also in picB and follows tile B in decoding order, and between tile B and tile C there are min_spatial_segment_offset_plus1[ i ] − 1 tiles in decoding order. When tile C is present, the syntax elements of tile A are constrained such that no sample or syntax elements values in tile C or any tile of the same picture following tile C in decoding order are used for inter-layer prediction in the decoding process of any samples within tile A.

- In each PPS referred to by a picture in the j-th direct reference layer of the i-th layer, tiles_enabled_flag is equal to 0 and entropy_coding_sync_enabled_flag is equal to 1, and the following applies:

- Let CTU row A be any CTU row in any picture picA of the i-th layer and ctbAddr[ i ] be the raster scan address of the last CTU in CTU row A. Let CTU row B be the CTU row that is in the picture picB belonging to the same access unit as picA and belonging to the j-th direct reference layer of the i-th layer and that contains the CTU with raster scan address colCtbAddr[ i ][ j ]. Let CTU row C be the CTU row that is also in picB and follows CTU row B in decoding order, and between CTU row B and CTU row C there are min_spatial_segment_offset_plus1[ i ] − 1 CTU rows in decoding order. When CTU row C is present, the syntax elements of CTU row A are constrained such that no sample or syntax elements values in CTU row C or row of the same picture following CTU row C are used for inter-layer prediction in the decoding process of any samples within CTU row A.

- Otherwise (ctu_based_offset_enabled_flag[ i ][ j ] is equal to 1), the following applies:

- The variable refCtbAddr[ i ][ j ] is derived as follows:

xOffset[ i ][ j ] = ( ( xColCtb[ i ][ j ] + minHorizontalCtbOffset[ i ][ j ] ) > ( refPicWidthInCtbsY[ i ][ j ] − 1 ) ) ? 
( refPicWidthInCtbsY[ i ][ j ] − 1 − xColCtb[ i ][ j ] ) : minHorizontalCtbOffset[ i ][ j ]	(FF-40)

yOffset[ i ][ j ] = ( min_spatial_segment_offset_plus1[ i ][ j ] − 1 ) * refPicWidthInCtbsY[ i ][ j ]	(FF-41)

refCtbAddr[ i ][ j ] = colCtbAddr[ i ][ j ] + xOffset[ i ][ j ] + yOffset[ i ][ j ]	(FF-42)

- Let CTU A be any CTU in any picture picA of the i-th layer, and ctbAddr[ i ] be the raster scan address ctbAddr of CTU A. Let CTU B be a CTU that is in the picture belonging to the same access unit as picA and belonging to the j-th direct reference layer of the i-th layer and that has raster scan address greater than refCtbAddr[ i ][ j ]. When CTU B is present, the syntax elements of CTU A are constrained such that no sample or syntax elements values in CTU B are used for inter-layer prediction in the decoding process of any samples within CTU A.

vps_vui_bsp_hrd_present_flag equal to 0 specifies that no bitstream partition HRD parameters are present in the VPS VUI. vps_vui_bsp_hrd_present_flag equal to 1 specifies that bitstream partition HRD parameters are present in the VPS VUI. When vps_timing_info_present_flag is equal to 0, the value of vps_vui_bsp_hrd_present_flag shall be equal to 0.

base_layer_parameter_set_compatibility_flag[ i ] equal to 1 specifies that the following constraints apply to the layer with nuh_layer_id equal to layer_id_in_nuh[ i ]. base_layer_parameter_set_compatibility_flag[ i ] equal to 0 specifies that the following constraints may or may not apply to the layer with nuh_layer_id equal to layer_id_in_nuh[ i ].

- Each coded slice segment NAL unit with nuh_layer_id value equal to layer_id_in_nuh[ i ] referring to the VPS shall refer to a PPS with nuh_layer_id value equal to 0.

- Each coded slice segment NAL unit with nuh_layer_id value equal to layer_id_in_nuh[ i ] referring to the VPS shall refer to a SPS with nuh_layer_id value equal to 0.

- The values of chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, bit_depth_chroma_minus8, conf_win_left_offset, conf_win_right_offset, conf_win_top_offset, and conf_win_bottom_offset, respectively, of the active SPS for the layer with nuh_layer_id equal to layer_id_in_nuh[ i ] shall be the same as the values of chroma_format_vps_idc, separate_colour_plane_vps_flag, pic_width_vps_in_luma_samples, pic_height_vps_in_luma_samples, bit_depth_vps_luma_minus8, bit_depth_vps_chroma_minus8, conf_win_vps_left_offset, conf_win_vps_right_offset, conf_win_vps_top_offset, and conf_win_vps_bottom_offset, respectively, of the vps_rep_format_idx[ i ]-th rep_format( ) syntax structure in the active VPS.

          - Video signal info semantics

video_vps_format, video_full_range_vps_flag, colour_primaries_vps, transfer_characteristics_vps, and matrix_coeffs_vps are used for inference of the values of the SPS VUI syntax elements video_format, video_full_range_flag, colour_primaries, transfer_characteristics, and matrix_coeffs, respectively, for each SPS that refers to the VPS.

For each of these syntax elements, all constraints, if any, that apply to the value of the corresponding SPS VUI syntax element also apply.

          - VPS VUI bitstream partition HRD parameters semantics

vps_num_add_hrd_params specifies the number of additional hrd_parameters( ) syntax structures present in the VPS. The value of vps_num_add_hrd_params shall be in the range of 0 to 1024 − vps_num_hrd_parameters, inclusive.

cprms_add_present_flag[ i ] equal to 1 specifies that the HRD parameters that are common for all sub-layers are present in the i-th hrd_parameters( ) syntax structure. cprms_add_present_flag[ i ] equal to 0 specifies that the HRD parameters that are common for all sub-layers are not present in the i-th hrd_parameters( ) syntax structure and are derived to be the same as the ( i − 1 )-th hrd_parameters( ) syntax structure. When vps_num_hrd_parameters is equal to 0, the value of cprms_add_present_flag[ 0 ] is inferred to be equal to 1.

num_sub_layer_hrd_minus1[ i ] plus 1 specifies the number of sub-layers for which HRD parameters are signalled in the i-th hrd_parameters( ) syntax structure. The value of num_sub_layer_hrd_minus1[ i ] shall be in the range of 0 to vps_max_sub_layers_minus1, inclusive.

num_signalled_partitioning_schemes[ h ] specifies the number of signalled partitioning schemes for the h-th OLS. The value of num_signalled_partitioning_schemes[ h ] shall be in the range of 0 to 16, inclusive. When vps_base_layer_internal_flag is equal to 1, the value of num_signalled_partitioning_schemes[ 0 ] is inferred to be equal to 0.

num_partitions_in_scheme_minus1[ h ][ j ] plus 1 specifies the number of bitstream partitions for the j-th partitioning scheme of the h-th OLS. The value of num_partitions_in_scheme_minus1[ h ][ j ] shall be in the range of 0 to NumLayersInIdList[ OlsIdxToLsIdx[ h ] ] – 1, inclusive. When vps_base_layer_internal_flag is equal to 1, the value of num_partitions_in_scheme_minus1[ 0 ][ 0 ] is inferred to be equal to 0. The value of num_partitions_in_scheme_minus1[ h ][ 0 ] is inferred to be equal to NumLayersInIdList[ h ] − 1.

NOTE – The 0-th partitioning scheme for each OLS is inferred to contain the number of bitstream partitions to be equal to the number of layers in the OLS and each bitstream partition contains one layer of the OLS.

layer_included_in_partition_flag[ h ][ j ][ k ][ r ] equal to 1 specifies that the r-th layer in the h-th OLS is included in the k-th bitstream partition of the j-th partitioning scheme of the h-th OLS. layer_included_in_partition_flag[ h ][ j ][ k ][ r ] equal to 0 specifies that the r-th layer in the h-th OLS is not included in the k-th bitstream partition of the j-th partitioning scheme of the h-th OLS. When vps_base_layer_internal_flag is equal to 1, the value of layer_included_in_partition_flag[ 0 ][ 0 ][ 0 ][ 0 ] is inferred to be equal to 1. The value of layer_included_in_partition_flag[ h ][ 0 ][ k ][ r ] for any value of h in the range of 1 to NumOutputLayerSets − 1, inclusive, is inferred to be equal to ( k  = =  r ).

It is a requirement of bitstream conformance that the following constraints apply:

–	For the j-th partitioning scheme of the h-th OLS, the bitstream partition with index k1 shall not include reference layers of any layers in the bitstream partition with index k2 for any values of k1 and k2 in the range of 0 to num_partitions_in_scheme_minus1[ h ][ j ], inclusive, such that k2 is less than k1.

–	When vps_base_layer_internal_flag is equal to 0 and layer_included_in_partition_flag[ h ][ j ][ k ][ 0 ] is equal to 1 for any value of h in the range of 1 to NumOutputLayerSets − 1, inclusive, any value of j in the range of 0 to num_signalled_partitioning_schemes[ h ], inclusive, and any value of k in the range of 0 to num_partitions_in_scheme_minus1[ h ][ j ], inclusive, the value of layer_included_in_partition_flag[ h ][ j ][ k ][ r ] for at least one value of r in the range of 1 to NumLayersInIdList[ OlsIdxToLsIdx[ h ] ] − 1, inclusive, shall be equal to 1.

–	For each partitioning scheme with index j of the h-th OLS, and for each layer with layer index r among the layers in the h-th OLS, there exists one and only one value of k in the range of 0 to num_partitions_in_scheme_minus1[ h ][ j ], inclusive, such that layer_included_in_partition_flag[ h ][ j ][ k ][ r ] is equal to 1.

num_bsp_schedules_minus1[ h ][ i ][ t ] plus 1 specifies the number of delivery schedules specified for bitstream partitions of the i-th partitioning scheme of the h-th OLS when HighestTid is equal to t. The value of num_bsp_schedules_minus1[ h ][ i ][ t ] shall be in the range of 0 to 31, inclusive.

The variable BspSchedCnt[ h ][ i ][ t ] is set equal to num_bsp_schedules_minus1[ h ][ i ][ t ] + 1.

bsp_hrd_idx[ h ][ i ][ t ][ j ][ k ] specifies the index of the hrd_parameters( ) syntax structure in the VPS for the j-th delivery schedule specified for the k-th bitstream partition of the i-th partitioning scheme for the h-th OLS when HighestTid is equal to t. The length of the bsp_hrd_idx[ h ][ i ][ t ][ j ][ k ] syntax element is Ceil( Log2( vps_num_hrd_parameters + vps_num_add_hrd_params ) ) bits. The value of bsp_hrd_idx[ h ][ i ][ t ][ j ][ k ] shall be in the range of 0 to vps_num_hrd_parameters + vps_num_add_hrd_params − 1, inclusive. When vps_num_hrd_parameters + vps_num_add_hrd_params is equal to 1, the value of bsp_hrd_idx[ h ][ i ][ t ][ j ][ k ] is inferred to be equal to 0.

bsp_sched_idx[ h ][ i ][ t ][ j ][ k ] specifies the index of the delivery schedule within the sub_layer_hrd_parameters( t ) syntax structure of the hrd_parameters( ) syntax structure with the index bsp_hrd_idx[ h ][ i ][ t ][ j ][ k ], that is used as the j-th delivery schedule specified for the k-th bitstream partition of the i-th partitioning scheme for the h-th OLS when HighestTid is equal to t. The value of bsp_sched_idx[ h ][ i ][ t ][ j ][ k ] shall be in the range of 0 to cpb_cnt_minus1[ t ], inclusive, where cpb_cnt_minus1[ t ] is found in the hrd_parameters( ) syntax structure corresponding to the index bsp_hrd_idx[ h ][ i ][ t ][ j ][ k ].

The following applies for any j greater than 0, any h in the range of 1 to NumOutputLayerSets − 1, inclusive, any i in the range 0 to num_signalled_partitioning_schemes[ h ], inclusive, any t in the range of 0 to MaxSubLayersInLayerSetMinus1[ OlsIdxToLsIdx[ h ] ], inclusive, and any k in the range of 0 to num_partitions_in_scheme_minus1[ h ][ i ], inclusive:

- For x in the range of 0 to 1, inclusive, the following applies:

- The variable bsIdx is set equal to bsp_sched_idx[ h ][ i ][ t ][ j − x ][ k ].

- The variables bitRateValueMinus1[ x ], bitRateDuValueMinus1[ x ], cpbSizeValueMinus1[ x ], and cpbSizeDuValueMinus1[ x ] are set equal to the values of the syntax elements bit_rate_value_minus1[ bsIdx ], bit_rate_du_value_minus1[ bsIdx ], cpb_size_value_minus1[ bsIdx ], and, cpb_size_du_value_minus1[ bsIdx ], respectively, found in the sub_layer_hrd_parameters( t ) syntax structure within the hrd_parameters( ) structure with index bsp_hrd_idx[ h ][ i ][ t ][ j − x ][ k ].

- It is a requirement of bitstream conformance that all of the following conditions shall be true:

- bitRateValueMinus1[ 0 ] is greater than bitRateValueMinus1[ 1 ].

- bitRateDuValueMinus1[ 0 ] is greater than bitRateDuValueMinus1[ 1 ].

- cpbSizeValueMinus1[ 0 ] is less than or equal to cpbSizeValueMinus1[ 1 ].

- cpbSizeDuValueMinus1[ 0 ] is less than or equal to cpbSizeDuValueMinus1[ 1 ].

The arrays BpBitRate and BpbSize for BPB are derived as follows:

for( h = 1; h < NumOutputLayerSets; h++ )
	for( i = 0; i  <=  num_signalled_partitioning_schemes[ h ]; i++ )
		for( t = 0; t  <=  MaxSubLayersInLayerSetMinus1[ OlsIdxToLsIdx[ h ] ]; t++ )
			for( j = 0; j  <=  num_bsp_schedules_minus1[ h ][ i ][ t ]; j++ )	-
				for( k = 0; k  <=  num_partitions_in_scheme_minus1[ h ][ i ]; k++ ) {
					bsIdx = bsp_sched_idx[ h ][ i ][ t ][ j ][ k ]
					brDu = ( bit_rate_du_value_minus1[ bsIdx ] + 1 ) * 2( 6 + bit_rate_scale )	(F-44)
					brPu = ( bit_rate_value_minus1[ bsIdx ] + 1 ) * 2( 6 + bit_rate_scale )
					cpbSizeDu = ( cpb_size_du_value_minus1[ bsIdx ] + 1 ) * 2( 4 + cpb_size_du_scale )
					cpbSizePu = ( cpb_size_value_minus1[ bsIdx ] + 1 ) * 2( 4 + cpb_size_scale )
					BpBitRate[ h ][ i ][ t ][ j ][ k ] = SubPicHrdFlag ? brDu : brPu
						
					BpbSize[ h ][ i ][ t ][ j ][ k ] = SubPicHrdFlag? cpbSizeDu : cpbSizePu
						
				}

where the syntax elements bit_rate_scale, cpb_size_du_scale, and cpb_size_scale values are found in the hrd_parameters( ) syntax structure corresponding to the index bsp_hrd_idx[ h ][ i ][ t ][ j ][ k ], and the syntax elements bit_rate_du_value_minus1[ bsIdx ], bit_rate_value_minus1[ bsIdx ], cpb_size_du_value_minus1[ bsIdx ], and cpb_size_value_minus1[ bsIdx ] are found in the sub_layer_hrd_parameters( t ) syntax structure within that hrd_parameters( ) syntax structure.

        - Sequence parameter set RBSP semantics

          - General sequence parameter set RBSP semantics

The specifications in clause 7.4.3.2.1 apply with following additions and modifications:

sps_max_sub_layers_minus1 plus 1 specifies the maximum number of temporal sub-layers that may be present in each CVS referring to the SPS. The value of sps_max_sub_layers_minus1 shall be in the range of 0 to 6, inclusive. The value of sps_max_sub_layers_minus1 shall be less than or equal to vps_max_sub_layers_minus1. When not present, the value of sps_max_sub_layers_minus1 is inferred to be equal to ( sps_ext_or_max_sub_layers_minus1  = =  7 ) ? vps_max_sub_layers_minus1 : sps_ext_or_max_sub_layers_minus1.

sps_ext_or_max_sub_layers_minus1 is used to infer the value of sps_max_sub_layers_minus1 and to derive the value of MultiLayerExtSpsFlag.

It is a requirement of bitstream conformance that the following applies:

–	If the SPS is referred to by any current picture that belongs to an independent non-base layer, the value of MultiLayerExtSpsFlag derived from the SPS shall be equal to 0.

–	Otherwise, the value of MultiLayerExtSpsFlag derived from the SPS shall be equal to 1.

sps_temporal_id_nesting_flag, when sps_max_sub_layers_minus1 is greater than 0, specifies whether inter prediction is additionally restricted for CVSs referring to the SPS. When vps_temporal_id_nesting_flag is equal to 1, sps_temporal_id_nesting_flag shall be equal to 1. When sps_max_sub_layers_minus1 is equal to 0, sps_temporal_id_nesting_flag shall be equal to 1. When not present, the value of sps_temporal_id_nesting_flag is inferred as follows:

–	If sps_max_sub_layers_minus1 is greater than 0, the value of sps_temporal_id_nesting_flag is inferred to be equal to vps_temporal_id_nesting_flag.

–	Otherwise, the value of sps_temporal_id_nesting_flag is inferred to be equal to 1.

NOTE 1 – The syntax element sps_temporal_id_nesting_flag is used to indicate that temporal up-switching, i.e., switching from decoding up to any TemporalId tIdN to decoding up to any TemporalId tIdM that is greater than tIdN, is always possible in the CVS.

update_rep_format_flag equal to 1 specifies that sps_rep_format_idx is present and that the sps_rep_format_idx-th rep_format( ) syntax structures in the active VPS applies to the layers that refer to this SPS. update_rep_format_flag equal to 0 specifies that sps_rep_format_idx is not present. When the value of vps_num_rep_formats_minus1 in the active VPS is equal to 0, it is a requirement of bitstream conformance that the value of update_rep_format_flag shall be equal to 0. When not present, the value of update_rep_format_flag is inferred to be equal to 0.

sps_rep_format_idx specifies the index, into the list of rep_format( ) syntax structures in the VPS, of the rep_format( ) syntax structure that applies to the layers that refer to this SPS. The value of sps_rep_format_idx shall be in the range of 0 to vps_num_rep_formats_minus1, inclusive.

When a current picture with nuh_layer_id layerIdCurr greater than 0 refers to an SPS and the layer with nuh_layer_id equal to layerIdCurr is not an independent non-base layer, the values of chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, bit_depth_chroma_minus8, conf_win_left_offset, conf_win_right_offset, conf_win_top_offset, and conf_win_bottom_offset are inferred or constrained as follows:

–	The variable repFormatIdx is derived as follows:

–	If update_rep_format_flag is equal to 0, the variable repFormatIdx is set equal to vps_rep_format_idx[ LayerIdxInVps[ layerIdCurr ] ].

–	Otherwise (update_rep_format_flag is equal to 1), repFormatIdx is set equal to sps_rep_format_idx.

–	If MultiLayerExtSpsFlag derived from the active SPS for the layer with nuh_layer_id equal to layerIdCurr is equal to 0, the values of chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, bit_depth_chroma_minus8, conf_win_left_offset, conf_win_right_offset, conf_win_top_offset, and conf_win_bottom_offset, are inferred to be equal to chroma_format_vps_idc, separate_colour_plane_vps_flag, pic_width_vps_in_luma_samples, pic_height_vps_in_luma_samples, bit_depth_vps_luma_minus8, bit_depth_vps_chroma_minus8, conf_win_vps_left_offset, conf_win_vps_right_offset, conf_win_vps_top_offset, and conf_win_vps_bottom_offset, respectively, of the repFormatIdx-th rep_format( ) syntax structure in the active VPS and the values of chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, bit_depth_chroma_minus8, conf_win_left_offset, conf_win_right_offset, conf_win_top_offset, and conf_win_bottom_offset signalled in the active SPS for the layer with nuh_layer_id equal to layerIdCurr are ignored.

NOTE 2 – The values are inferred from the VPS when a layer that is not an independent layer refers to an SPS that is also referred to by the base layer, in which case the SPS has nuh_layer_id equal to 0. For independent layers, the values of these parameters in the active SPS for the base layer apply.

–	Otherwise, the values of chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, bit_depth_chroma_minus8, conf_win_left_offset, conf_win_right_offset, conf_win_top_offset, and conf_win_bottom_offset are inferred to be equal to chroma_format_vps_idc, separate_colour_plane_vps_flag, pic_width_vps_in_luma_samples, pic_height_vps_in_luma_samples, bit_depth_vps_luma_minus8, bit_depth_vps_chroma_minus8, conf_win_vps_left_offset, conf_win_vps_right_offset, conf_win_vps_top_offset, and conf_win_vps_bottom_offset respectively, of the repFormatIdx-th rep_format( ) syntax structure in the active VPS.

It is a requirement of bitstream conformance that, when present, the value of chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, or bit_depth_chroma_minus8 shall be less than or equal to chroma_format_vps_idc, separate_colour_plane_vps_flag, pic_width_vps_in_luma_samples, pic_height_vps_in_luma_samples, bit_depth_vps_luma_minus8, or bit_depth_vps_chroma_minus8, respectively, of the vps_rep_format_idx[ j ]-th rep_format( ) syntax structure in the active VPS, where j is equal to LayerIdxInVps[ layerIdCurr ].

- sps_max_dec_pic_buffering_minus1[ i ] plus 1 specifies the maximum required size of the decoded picture buffer for the CVS in units of picture storage buffers when HighestTid is equal to i. The value of sps_max_dec_pic_buffering_minus1[ i ] shall be in the range of 0 to MaxDpbSize − 1 (as specified in clause A.4), inclusive. When i is greater than 0, sps_max_dec_pic_buffering_minus1[ i ] shall be greater than or equal to sps_max_dec_pic_buffering_minus1[ i − 1 ]. The value of sps_max_dec_pic_buffering_minus1[ i ] shall be less than or equal to vps_max_dec_pic_buffering_minus1[ i ] for each value of i. When sps_max_dec_pic_buffering_minus1[ i ] is not present for i in the range of 0 to sps_max_sub_layers_minus1 − 1, inclusive, due to sps_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to sps_max_dec_pic_buffering_minus1[ sps_max_sub_layers_minus1 ].

- When sps_max_dec_pic_buffering_minus1[ i ] is not present for i in the range of 0 to sps_max_sub_layers_minus1, inclusive, due to MultiLayerExtSpsFlag being equal to 1, for a layer that refers to the SPS and has nuh_layer_id equal to currLayerId, the value of sps_max_dec_pic_buffering_minus1[ i ] is inferred to be equal to max_vps_dec_pic_buffering_minus1[ TargetOlsIdx ][ layerIdx ][ i ] of the active VPS, where layerIdx is equal to the value such that LayerSetLayerIdList[ TargetDecLayerSetIdx ][ layerIdx ] is equal to currLayerId.

sps_infer_scaling_list_flag equal to 1 specifies that the syntax elements of the scaling list data syntax structure of the SPS are inferred to be equal to those of the SPS that is active for the layer with nuh_layer_id equal to sps_scaling_list_ref_layer_id. sps_infer_scaling_list_flag equal to 0 specifies that the syntax elements of the scaling list data syntax structure are not inferred. When not present, the value of sps_infer_scaling_list_flag is inferred to be 0.

sps_scaling_list_ref_layer_id specifies the value of the nuh_layer_id of the layer for which the active SPS is associated with the same scaling list data as the current SPS. The value of sps_scaling_list_ref_layer_id shall be in the range of 0 to 62, inclusive.

When vps_base_layer_internal_flag is equal to 0, it is a requirement of bitstream conformance that the value of sps_scaling_list_ref_layer_id, when present, shall be greater than 0.

It is a requirement of bitstream conformance that, when an SPS with nuh_layer_id equal to nuhLayerIdA is active for a layer with nuh_layer_id equal to nuhLayerIdB and sps_infer_scaling_list_flag in the SPS is equal to 1, sps_infer_scaling_list_flag shall be equal to 0 for the SPS that is active for the layer with nuh_layer_id equal to sps_scaling_list_ref_layer_id.

It is a requirement of bitstream conformance that, when an SPS with nuh_layer_id equal to nuhLayerIdA is active for a layer with nuh_layer_id equal to nuhLayerIdB and sps_infer_scaling_list_flag in the SPS equal to 1, the layer with nuh_layer_id equal to sps_scaling_list_ref_layer_id shall be a reference layer of the layer with nuh_layer_id equal to nuhLayerIdB.

          - Sequence parameter set range extensions semantics

The specifications in clause 7.4.3.2.2 apply.

          - Sequence parameter set multilayer extension semantics

inter_view_mv_vert_constraint_flag equal to 1 indicates that vertical component of motion vectors used for inter-layer prediction are constrained in the layers for which this SPS RBSP is the active SPS RBSP. When inter_view_mv_vert_constraint_flag is equal to 1, the vertical component of the motion vectors used for inter-layer prediction shall be equal to or less than 56 in units of luma samples. When inter_view_mv_vert_constraint_flag is equal to 0, no constraint on the vertical component of the motion vectors used for inter-layer prediction is signalled by this flag. When not present, the value of inter_view_mv_vert_constraint_flag is inferred to be equal to 0.

        - Picture parameter set RBSP semantics

          - General picture parameter set RBSP semantics

The specifications in clause 7.4.3.3.1 apply with the replacement of the semantics of pps_scaling_list_data_present_flag as follows:

pps_scaling_list_data_present_flag equal to 1 specifies that parameters are present in the PPS to modify the scaling lists specified by the active SPS. pps_scaling_list_data_present_flag equal to 0 specifies that the scaling list data used for the pictures referring to the PPS are inferred to be equal to those specified by the active SPS (when pps_infer_scaling_list_flag is equal to 0) or is specified by pps_scaling_list_ref_layer_id (when pps_infer_scaling_list_flag is equal to 1). When scaling_list_enabled_flag is equal to 0, the value of pps_scaling_list_data_present_flag shall be equal to 0. When scaling_list_enabled_flag is equal to 1, sps_scaling_list_data_present_flag is equal to 0, pps_scaling_list_data_present_flag is equal to 0, and pps_infer_scaling_list_flag is equal to 0, the default scaling list data are used to derive the array ScalingFactor as specified in clause 7.4.5.

          - Picture parameter set range extensions semantics

The specifications in clause 7.4.3.3.2 apply.

          - Picture parameter set multilayer extension semantics

poc_reset_info_present_flag equal to 0 specifies that the syntax element poc_reset_idc is not present in the slice segment headers of the slices referring to the PPS. poc_reset_info_present_flag equal to 1 specifies that the syntax element poc_reset_idc is present in the slice segment headers of the slices referring to the PPS. When not present, the value of poc_reset_info_present_flag is inferred to be equal to 0.

pps_infer_scaling_list_flag equal to 1 specifies that the syntax elements of the scaling list data syntax structure of the PPS are inferred to be equal to those of the PPS that is active for the layer with nuh_layer_id equal to pps_scaling_list_ref_layer_id. pps_infer_scaling_list_flag equal to 0 specifies that the syntax elements of the scaling list data syntax structure of the PPS are not inferred. When scaling_list_enabled_flag is equal to 0, nuh_layer_id of the layer referring to the PPS is equal to 0, or pps_scaling_list_data_present_flag is equal to 1, pps_infer_scaling_list_flag shall be equal to 0. When not present, the value of pps_infer_scaling_list_flag is inferred to be 0.

pps_scaling_list_ref_layer_id specifies the value of nuh_layer_id of the layer for which the active PPS has the same scaling list data as the current PPS.

The value of pps_scaling_list_ref_layer_id shall be in the range of 0 to 62, inclusive.

When vps_base_layer_internal_flag is equal to 0, it is a requirement of bitstream conformance that pps_scaling_list_ref_layer_id, when present, shall be greater than 0. It is a requirement of bitstream conformance that, when a PPS with nuh_layer_id equal to nuhLayerIdA is active for a layer with nuh_layer_id equal to nuhLayerIdB and pps_infer_scaling_list_flag in the PPS is equal to 1, pps_infer_scaling_list_flag shall be equal to 0 for the PPS that is active for the layer with nuh_layer_id equal to pps_scaling_list_ref_layer_id.

It is a requirement of bitstream conformance that, when a PPS with nuh_layer_id equal to nuhLayerIdA is active for a layer with nuh_layer_id equal to nuhLayerIdB and pps_infer_scaling_list_flag in the PPS equal to 1, the layer with nuh_layer_id equal to pps_scaling_list_ref_layer_id shall be a reference layer of the layer with nuh_layer_id equal to nuhLayerIdB.

num_ref_loc_offsets specifies the number of reference layer location offsets that are present in the PPS. The value of num_ref_loc_offsets shall be in the range of 0 to vps_max_layers_minus1, inclusive.

ref_loc_offset_layer_id[ i ] specifies the nuh_layer_id value for which the i-th reference layer location offset parameters are specified.

NOTE – ref_loc_offset_layer_id[ i ] need not be among the direct reference layers, for example when the spatial correspondence of an auxiliary picture to its associated primary picture is specified.

The i-th reference layer location offset parameters consist of the i-th scaled reference layer offset parameters, the i-th reference region offset parameters, and the i-th resampling phase set parameters.

scaled_ref_layer_offset_present_flag[ i ] equal to 1 specifies that the i-th scaled reference layer offset parameters are present in the PPS. scaled_ref_layer_offset_present_flag[ i ] equal to 0 specifies that the i-th scaled reference layer offset parameters are not present in the PPS. When not present, the value of scaled_ref_layer_offset_present_flag[ i ] is inferred to be equal to 0.

The i-th scaled reference layer offset parameters specify the spatial correspondence of a picture referring to this PPS relative to the reference region in a decoded picture with nuh_layer_id equal to ref_loc_offset_layer_id[ i ].

scaled_ref_layer_left_offset[ ref_loc_offset_layer_id[ i ] ] specifies the horizontal offset between the sample in the current picture that is collocated with the top-left luma sample of the reference region in a decoded picture with nuh_layer_id equal to ref_loc_offset_layer_id[ i ] and the top-left luma sample of the current picture in units of subWC luma samples, where subWC is equal to the SubWidthC of the picture that refers to this PPS. The value of scaled_ref_layer_left_offset[ ref_loc_offset_layer_id[ i ] ] shall be in the range of −214 to 214 − 1, inclusive. When not present, the value of scaled_ref_layer_left_offset[ ref_loc_offset_layer_id[ i ] ] is inferred to be equal to 0.

scaled_ref_layer_top_offset[ ref_loc_offset_layer_id[ i ] ] specifies the vertical offset between the sample in the current picture that is collocated with the top-left luma sample of the reference region in a decoded picture with nuh_layer_id equal to ref_loc_offset_layer_id[ i ] and the top-left luma sample of the current picture in units of subHC luma samples, where subHC is equal to the SubHeightC of the picture that refers to this PPS. The value of scaled_ref_layer_top_offset[ ref_loc_offset_layer_id[ i ] ] shall be in the range of −214 to 214 − 1, inclusive. When not present, the value of scaled_ref_layer_top_offset[ ref_loc_offset_layer_id[ i ] ] is inferred to be equal to 0.

scaled_ref_layer_right_offset[ ref_loc_offset_layer_id[ i ] ] specifies the horizontal offset between the sample in the current picture that is collocated with the bottom-right luma sample of the reference region in a decoded picture with nuh_layer_id equal to ref_loc_offset_layer_id[ i ] and the bottom-right luma sample of the current picture in units of subWC luma samples, where subWC is equal to the SubWidthC of the picture that refers to this PPS. The value of scaled_ref_layer_right_offset[ ref_loc_offset_layer_id[ i ] ] shall be in the range of −214 to 214 − 1, inclusive. When not present, the value of scaled_ref_layer_right_offset[ ref_loc_offset_layer_id[ i ] ] is inferred to be equal to 0.

scaled_ref_layer_bottom_offset[ ref_loc_offset_layer_id[ i ] ] specifies the vertical offset between the sample in the current picture that is collocated with the bottom-right luma sample of the reference region in a decoded picture with nuh_layer_id equal to ref_loc_offset_layer_id[ i ] and the bottom-right luma sample of the current picture in units of subHC luma samples, where subHC is equal to the SubHeightC of the picture that refers to this PPS. The value of scaled_ref_layer_bottom_offset[ ref_loc_offset_layer_id[ i ] ] shall be in the range of −214 to 214 − 1, inclusive. When not present, the value of scaled_ref_layer_bottom_offset[ ref_loc_offset_layer_id[ i ] ] is inferred to be equal to 0.

Let currTopLeftSample, currBotRightSample, colRefRegionTopLeftSample, and colRefRegionBotRightSample be the top-left luma sample of the current picture, the bottom-right luma sample of the current picture, the sample in the current picture that is collocated with the top-left luma sample of the reference region in a decoded picture with nuh_layer_id equal to ref_loc_offset_layer_id[ i ], and the sample in the current picture that is collocated with the bottom-right luma sample of the reference region in the decoded picture with nuh_layer_id equal to ref_loc_offset_layer_id[ i ], respectively.

When the value of scaled_ref_layer_left_offset[ ref_loc_offset_layer_id[ i ] ] is greater than 0, colRefRegionTopLeftSample is located to the right of currTopLeftSample. When the value of scaled_ref_layer_left_offset[ ref_loc_offset_layer_id[ i ] ] is less than 0, colRefRegionTopLeftSample is located to the left of currTopLeftSample.

When the value of scaled_ref_layer_top_offset[ ref_loc_offset_layer_id[ i ] ] is greater than 0, colRefRegionTopLeftSample is located below currTopLeftSample. When the value of scaled_ref_layer_top_offset[ ref_loc_offset_layer_id[ i ] ] is less than 0, colRefRegionTopLeftSample is located above currTopLeftSample.

When the value of scaled_ref_layer_right_offset[ ref_loc_offset_layer_id[ i ] ] is greater than 0, colRefRegionBotRightSample is located to the left of currBotRightSample. When the value of scaled_ref_layer_right_offset[ ref_loc_offset_layer_id[ i ] ] is less than 0, colRefRegionTopLeftSample is located to the right of currBotRightSample.

When the value of scaled_ref_layer_bottom_offset[ ref_loc_offset_layer_id[ i ] ] is greater than 0, colRefRegionBotRightSample is located above currBotRightSample. When the value of scaled_ref_layer_bottom_offset[ ref_loc_offset_layer_id[ i ] ] is less than 0, colRefRegionTopLeftSample is located below currBotRightSample.

ref_region_offset_present_flag[ i ] equal to 1 specifies that the i-th reference region offset parameters are present in the PPS. ref_region_offset_present_flag[ i ] equal to 0 specifies that the i-th reference region offset parameters are not present in the PPS. When not present, the value of ref_region_offset_present_flag[ i ] is inferred to be equal to 0.

The i-th reference region offset parameters specify the spatial correspondence of the reference region in the decoded picture with nuh_layer_id equal to ref_loc_offset_layer_id[ i ] relative to the same decoded picture.

ref_region_left_offset[ ref_loc_offset_layer_id[ i ] ] specifies the horizontal offset between the top-left luma sample of the reference region in the decoded picture with nuh_layer_id equal to ref_loc_offset_layer_id[ i ] and the top-left luma sample of the same decoded picture in units of subWC luma samples, where subWC is equal to the SubWidthC of the layer with nuh_layer_id equal to ref_loc_offset_layer_id[ i ]. The value of ref_region_left_offset[ ref_loc_offset_layer_id[ i ] ] shall be in the range of −214 to 214 − 1, inclusive. When not present, the value of ref_region_left_offset[ ref_loc_offset_layer_id[ i ] ] is inferred to be equal to 0.

ref_region_top_offset[ ref_loc_offset_layer_id[ i ] ] specifies the vertical offset between the top-left luma sample of the reference region in the decoded picture with nuh_layer_id equal to ref_loc_offset_layer_id[ i ] and the top-left luma sample of the same decoded picture in units of subHC luma samples, where subHC is equal to the SubHeightC of the layer with nuh_layer_id equal to ref_loc_offset_layer_id[ i ]. The value of ref_region_top_offset[ ref_loc_offset_layer_id[ i ] ] shall be in the range of −214 to 214 − 1, inclusive. When not present, the value of ref_region_top_offset[ ref_loc_offset_layer_id[ i ] ] is inferred to be equal to 0.

ref_region_right_offset[ ref_loc_offset_layer_id[ i ] ] specifies the horizontal offset between the bottom-right luma sample of the reference region in the decoded picture with nuh_layer_id equal to ref_loc_offset_layer_id[ i ] and the bottom-right luma sample of the same decoded picture in units of subWC luma samples, where subWC is equal to the SubWidthC of the layer with nuh_layer_id equal to ref_loc_offset_layer_id[ i ]. The value of ref_layer_right_offset[ ref_loc_offset_layer_id[ i ] ] shall be in the range of −214 to 214 − 1, inclusive. When not present, the value of ref_region_right_offset[ ref_loc_offset_layer_id[ i ] ] is inferred to be equal to 0.

ref_region_bottom_offset[ ref_loc_offset_layer_id[ i ] ] specifies the vertical offset between the bottom-right luma sample of the reference region in the decoded picture with nuh_layer_id equal to ref_loc_offset_layer_id[ i ] and the bottom-right luma sample of the same decoded picture in units of subHC luma samples, where subHC is equal to the SubHeightC of the layer with nuh_layer_id equal to ref_loc_offset_layer_id[ i ]. The value of ref_layer_bottom_offset[ ref_loc_offset_layer_id[ i ] ] shall be in the range of −214 to 214 − 1, inclusive. When not present, the value of ref_region_bottom_offset[ ref_loc_offset_layer_id[ i ] ] is inferred to be equal to 0.

Let refPicTopLeftSample, refPicBotRightSample, refRegionTopLeftSample, and refRegionBotRightSample be the top-left luma sample of the decoded picture with nuh_layer_id equal to ref_loc_offset_layer_id[ i ], the bottom-right luma sample of the decoded picture with nuh_layer_id equal to ref_loc_offset_layer_id[ i ], the top-left luma sample of the reference region in the decoded picture with nuh_layer_id equal to ref_loc_offset_layer_id[ i ], and the bottom-right luma sample of the reference region in the decoded picture with nuh_layer_id equal to ref_loc_offset_layer_id[ i ], respectively.

When the value of ref_region_left_offset[ ref_loc_offset_layer_id[ i ] ] is greater than 0, refRegionTopLeftSample is located to the right of refPicTopLeftSample. When the value of ref_region_left_offset[ ref_loc_offset_layer_id[ i ] ] is less than 0, refRegionTopLeftSample is located to the left of refPicTopLeftSample.

When the value of ref_region_top_offset[ ref_loc_offset_layer_id[ i ] ] is greater than 0, refRegionTopLeftSample is located below refPicTopLeftSample. When the value of ref_region_top_offset[ ref_loc_offset_layer_id[ i ] ] is less than 0, refRegionTopLeftSample is located above refPicTopLeftSample.

When the value of ref_region_right_offset[ ref_loc_offset_layer_id[ i ] ] is greater than 0, refRegionBotRightSample is located to the left of refPicBotRightSample. When the value of ref_region_right_offset[ ref_loc_offset_layer_id[ i ] ] is less than 0, refRegionBotRightSample is located to the right of refPicBotRightSample.

When the value of ref_region_bottom_offset[ ref_loc_offset_layer_id[ i ] ] is greater than 0, refRegionBotRightSample is located above refPicBotRightSample. When the value of ref_region_bottom_offset[ ref_loc_offset_layer_id[ i ] ] is less than 0, refRegionBotRightSample is located below refPicBotRightSample.

resample_phase_set_present_flag[ i ] equal to 1 specifies that the i-th resampling phase set is present in the PPS. resample_phase_set_present_flag[ i ] equal to 0 specifies that the i-th resampling phase set is not present in the PPS. When not present, the value of resample_phase_set_present_flag[ i ] is inferred to be equal to 0.

The i-th resampling phase set specifies the phase offsets used in resampling process of the direct reference layer picture with nuh_layer_id equal to ref_loc_offset_layer_id[ i ]. When the layer specified by ref_loc_offset_layer_id[ i ] is not a direct reference layer of the current layer, the values of the syntax elements phase_hor_luma[ ref_loc_offset_layer_id[ i ] ], phase_ver_luma[ ref_loc_offset_layer_id[ i ] ], phase_hor_chroma_plus8[ ref_loc_offset_layer_id[ i ] ] and phase_ver_chroma_plus8[ ref_loc_offset_layer_id[ i ] ] are unspecified and shall be ignored by decoders.

phase_hor_luma[ ref_loc_offset_layer_id[ i ] ] specifies the luma phase shift in the horizontal direction used in the resampling process of the direct reference layer picture with nuh_layer_id equal to ref_loc_offset_layer_id[ i ]. The value of phase_hor_luma[ ref_loc_offset_layer_id[ i ] ] shall be in the range of 0 to 31, inclusive. When not present, the value of phase_hor_luma[ ref_loc_offset_layer_id[ i ] ] is inferred to be equal to 0.

phase_ver_luma[ ref_loc_offset_layer_id[ i ] ] specifies the luma phase shift in the vertical direction used in the resampling of the direct reference layer picture with nuh_layer_id equal to ref_loc_offset_layer_id[ i ]. The value of phase_ver_luma[ ref_loc_offset_layer_id[ i ] ] shall be in the range of 0 to 31, inclusive. When not present, the value of phase_ver_luma[ ref_loc_offset_layer_id[ i ] ] is inferred to be equal to 0.

phase_hor_chroma_plus8[ ref_loc_offset_layer_id[ i ] ] minus 8 specifies the chroma phase shift in the horizontal direction used in the resampling of the direct reference layer picture with nuh_layer_id equal to ref_loc_offset_layer_id[ i ]. The value of phase_hor_chroma_plus8[ ref_loc_offset_layer_id[ i ] ] shall be in the range of 0 to 63, inclusive. When not present, the value of phase_hor_chroma_plus8[ ref_loc_offset_layer_id[ i ] ] is inferred to be equal to 8.

phase_ver_chroma_plus8[ ref_loc_offset_layer_id[ i ] ] minus 8 specifies the chroma phase shift in the vertical direction used in the resampling process of the direct reference layer picture with nuh_layer_id equal to ref_loc_offset_layer_id[ i ]. The value of phase_ver_chroma_plus8[ ref_loc_offset_layer_id[ i ] ] shall be in the range of 0 to 63, inclusive. When not present, the value of phase_ver_chroma_plus8[ ref_loc_offset_layer_id[ i ] ] is inferred to be equal to ( 4 * scaledRefRegHeight + refRegHeight / 2 ) / refRegHeight + 4, where the value of scaledRefRegHeight is equal to the value of ScaledRefRegionHeightInSamplesY derived for the direct reference layer picture with nuh_layer_id equal to ref_loc_offset_layer_id[ i ] of the picture that refers to this PPS, and the value of refRegHeight is equal to RefLayerRegionHeightInSamplesY derived for the direct reference layer picture with nuh_layer_id equal to ref_loc_offset_layer_id[ i ] ] of the picture that refers to this PPS.

colour_mapping_enabled_flag equal to 1 specifies the colour mapping process may be applied to the inter-layer reference pictures for the coded pictures referring to the PPS. colour_mapping_enabled_flag equal to 0 specifies that the colour mapping process is not applied to the inter-layer reference pictures for the coded pictures referring to the PPS. When not present, the value of colour_mapping_enabled_flag is inferred to be equal to 0.

It is a requirement of bitstream conformance that, when pps_pic_parameter_set_id is greater than or equal to 8, colour_mapping_enabled_flag shall be equal to 0.

          - General colour mapping table semantics

num_cm_ref_layers_minus1 specifies the number of the following cm_ref_layer_id[ i ] syntax elements. The value of num_cm_ref_layers_minus1 shall be in the range of 0 to 61, inclusive.

cm_ref_layer_id[ i ] specifies the nuh_layer_id value of the associated direct reference layer picture for which the colour mapping table is specified.

cm_octant_depth specifies the maximal split depth of the colour mapping table. In bitstreams conforming to this version of this Specification, the value of cm_octant_depth shall be in the range of 0 to 1, inclusive. Other values for cm_octant_depth are reserved for future use by ITU-T | ISO/IEC.

The variable OctantNumC is derived as follows:

OctantNumC = 1  <<  cm_octant_depth		(FF-45)

cm_y_part_num_log2 specifies the number of partitions of the smallest colour mapping table octant for the luma component. In bitstreams conforming to this version of this Specification, the value of ( cm_y_part_num_log2 + cm_octant_depth ) shall be in the range of 0 to 3, inclusive. Other values for ( cm_y_part_num_log2 + cm_octant_depth ) are reserved for future use by ITU-T | ISO/IEC.

The variables OctantNumY and PartNumY are derived as follows:

OctantNumY = 1  <<  ( cm_octant_depth + cm_y_part_num_log2 )	(FF-46)

PartNumY = 1  <<  cm_y_part_num_log2		(FF-47)

luma_bit_depth_cm_input_minus8 specifies the sample bit depth of the input luma component of the colour mapping process. The variable BitDepthCmInputY is derived as follows:

BitDepthCmInputY = 8 + luma_bit_depth_cm_input_minus8	(FF-48)

It is a requirement of bitstream conformance that the value of BitDepthCmInputY shall be equal to the bit depth of the luma component of any reference layer with nuh_layer_id equal to one of the cm_ref_layer_id[ i ], for all i in the range of 0 to num_cm_ref_layers_minus1, inclusive.

chroma_bit_depth_cm_input_minus8 specifies the sample bit depth of the input chroma components of the colour mapping process. The variable BitDepthCmInputC is derived as follows:

BitDepthCmInputC = 8 + chroma_bit_depth_cm_input_minus8	(FF-49)

It is a requirement of bitstream conformance that the value of BitDepthCmInputC shall be equal to the bit depth of the chroma components of any reference layer with nuh_layer_id equal to one of the cm_ref_layer_id[ i ], for all i in the range of 0 to num_cm_ref_layers_minus1, inclusive.

luma_bit_depth_cm_output_minus8 specifies the sample bit depth of the output luma component of the colour mapping process. The variable BitDepthCmOutputY is derived as follows:

BitDepthCmOutputY = 8 + luma_bit_depth_cm_output_minus8	(FF-50)

It is a requirement of bitstream conformance that the value of BitDepthCmOutputY shall be greater than or equal to BitDepthCmInputY and less than or equal to the bit depth of the luma components of any picture that refers to this PPS.

chroma_bit_depth_cm_output_minus8 specifies the sample bit depth of the output chroma components of the colour mapping process. The variable BitDepthCmOutputC is derived as follows:

BitDepthCmOutputC = 8 + chroma_bit_depth_cm_output_minus8	(FF-51)

It is a requirement of bitstream conformance that the value of BitDepthCmOutputC shall be greater than or equal to BitDepthCmInputC and less than or equal to the bit depth of the chroma components of any picture that refers to this PPS.

cm_res_quant_bits specifies the number of least significant bits to be added to the colour mapping coefficient residual values res_coeff_q and res_coeff_r.

cm_delta_flc_bits_minus1 specifies the delta value used to derive the number of bits used to code the syntax element res_coeff_r. The variable CMResLSBits is set equal to max( 0, ( 10 + BitDepthCmInputY − BitDepthCmOutputY − cm_res_quant_bits − ( cm_delta_flc_bits_minus1 + 1 ) ) ).

cm_adapt_threshold_u_delta specifies the partitioning threshold of the Cb component used in colour mapping process. When not present, the value of cm_adapt_threshold_u_delta is inferred to be equal to 0. The value of cm_adapt_threshold_u_delta shall be in the range of −2BitDepthCmInputC − 1 to 2BitDepthCmInputC − 1 − 1, inclusive.

The variable CMThreshU is derived as follows:

CMThreshU = cm_adapt_threshold_u_delta + ( 1  <<  ( BitDepthCmInputC − 1 ) )	(FF-52)

cm_adapt_threshold_v_delta specifies the partitioning threshold of the Cr component used in colour mapping process. When not present, the value of cm_adapt_threshold_v_delta is inferred to be equal to 0. The value of cm_adapt_threshold_v_delta shall be in the range of −2BitDepthCmInputC − 1 to 2BitDepthCmInputC − 1 − 1, inclusive.

The variable CMThreshV is derived as follows:

CMThreshV = cm_adapt_threshold_v_delta + ( 1  <<  ( BitDepthCmInputC − 1 ) )	(FF-53)

          - Colour mapping octants semantics

split_octant_flag equal to 1 specifies that the current colour mapping octant is further split into eight octants with half length in each of the three dimensions. split_octant_flag equal to 0 specifies that the current colour mapping octant is not further split into eight octants. When not present, the value of split_octant_flag is inferred to be equal to 0.

coded_res_flag[ idxShiftY ][ idxCb ][ idxCr ][ j ] equal to 1 specifies that the residuals for the j-th colour mapping coefficients of the octant with octant index equal to ( idxShiftY, idxCb, idxCr ) are present. coded_res_flag[ idxShiftY ][ idxCb ][ idxCr ][ j ] equal to 0 specifies that the residuals for the j-th colour mapping coefficients of the octant with octant index equal to ( idxShiftY, idxCb, idxCr ) are not present. When not present, the value of coded_res_flag[ idxShiftY ][ idxCb ][ idxCr ][ j ] is inferred to be equal to 0.

res_coeff_q[ idxShiftY ][ idxCb ][ idxCr ][ j ][ c ] specifies the quotient of the residual for the j-th colour mapping coefficient of the c-th colour component of the octant with octant index equal to ( idxShiftY, idxCb, idxCr ). When not present, the value of res_coeff_q[ idxShiftY ][ idxCb ][ idxCr ][ j ][ c ] is inferred to be equal to 0.

res_coeff_r[ idxShiftY ][ idxCb ][ idxCr ][ j ][ c ] specifies the remainder of the residual for the j-th colour mapping coefficient of the c-th colour component of the octant with octant index equal to ( idxShiftY, idxCb, idxCr ). The number of bits used to code res_coeff_r is equal to CMResLSBits. If CMResLSBits is equal to 0, res_coeff_r is not present. When not present, the value of res_coeff_r[ idxShiftY ][ idxCb ][ idxCr ][ j ][ c ] is inferred to be equal to 0.

res_coeff_s[ idxShiftY ][ idxCb ][ idxCr ][ j ][ c ] specifies the sign of the residual for the j-th colour mapping coefficient of the c-th colour component of the octant with octant index equal to ( idxShiftY, idxCb, idxCr ). When not present, the value of res_coeff_s[ idxShiftY ][ idxCb ][ idxCr ][ j ][ c ] is inferred to be equal to 0.

The variables cmResCoeff[ idxShiftY ][ idxCb ][ idxCr ][ j ][ c ], with idxShiftY in the range of 0 to OctantNumY − 1, inclusive, idxCb and idxCr both in the range of 0 to OctantNumC − 1, inclusive, j in the range of 0 to 3, inclusive, and c in the range of 0 to 2, inclusive, are derived as follows:

cmResCoeff[ idxShiftY ][ idxCb ][ idxCr ][ j ][ c ] =
				( 1 − 2 * res_coeff_s[ idxShiftY ][ idxCb ][ idxCr ][ j ][ c ] ) *
				( ( ( res_coeff_q[ idxShiftY ][ idxCb ][ idxCr ][ j ][ c ]  <<  CMResLSBits ) +	(F-54)
				res_coeff_r[ idxShiftY ][ idxCb ][ idxCr ][ j ][ c ] )  <<  cm_res_quant_bits )		-)

The colour mapping coefficients LutY[ idxY ][ idxCb ][ idxCr ][ j ], LutCb[ idxY ][ idxCb ][ idxCr ][ j ], LutCr[ idxY ][ idxCb ][ idxCr ][ j ] with idxY in the range of 0 and OctantNumY − 1, inclusive, idxCb in the range of 0 and OctantNumC − 1, inclusive, idxCr in the range of 0 and OctantNumC − 1, inclusive, and j in the range of 0 and 3, inclusive, are derived as follows:

if( idxY  = =  0 ) {
	predY[ idxY ][ idxCb ][ idxCr ][ j ] = ( j  = =  0 ) ? 1024 : 0
	predCb[ idxY ][ idxCb ][ idxCr ][ j ] = ( j  = =  1 ) ? 1024 : 0
	predCr[ idxY ][ idxCb ][ idxCr ][ j ] = ( j  = =  2 ) ? 1024 : 0
} else {
	predY[ idxY ][ idxCb ][ idxCr ][ j ] = LutY[ idxY − 1 ][ idxCb ][ idxCr ][ j ]
	predCb[ idxY ][ idxCb ][ idxCr ][ j ] = LutCb[ idxY − 1 ][ idxCb ][ idxCr ][ j ]
	predCr[ idxY ][ idxCb ][ idxCr ][ j ] = LutCr[ idxY − 1 ][ idxCb ][ idxCr ][ j ]		(FF-56)
}
LutY[ idxY ][ idxCb ][ idxCr ][ j ] =
				( cmResCoeff[ idxY ][ idxCb ][ idxCr ][ j ][ 0 ] ) +
				predY[ idxY ][ idxCb ][ idxCr ][ j ]
LutCb[ idxY ][ idxCb ][ idxCr ][ j ] =
				( cmResCoeff[ idxY ][ idxCb ][ idxCr ][ j ][ 1 ] ) +
				predCb[ idxY ][ idxCb ][ idxCr ][ j ]
LutCr[ idxY ][ idxCb ][ idxCr ][ j ] =
				( cmResCoeff[ idxY ][ idxCb ][ idxCr ][ j ][ 2 ] ) +
				predCr[ idxY ][ idxCb ][ idxCr ][ j ]

It is a requirement of bitstream conformance that the values of LutY[ idxY ][ idxCb ][ idxCr ][ i ], LutCb[ idxY ][ idxCb ][ idxCr ][ i ], and LutCr[ idxY ][ idxCb ][ idxCr ][ i ] shall be in the range of −211 to 211 − 1, inclusive.

NOTE – When BitDepthCmInputC is not equal to BitDepthCmInputY, the encoder should select values of LutY[ idxY ][ idxCb ][ idxCr ][ 1 ], LutY[ idxY ][ idxCb ][ idxCr ][ 2 ], LutCb[ idxY ][ idxCb ][ idxCr ][ 0 ] and LutCr[ idxY ][ idxCb ][ idxCr ][ 0 ] that compensate for the bit depth difference.

        - Supplemental enhancement information RBSP semantics

The specifications in clause 7.4.3.4 apply.

        - Access unit delimiter RBSP semantics

The specifications in clause 7.4.3.5 apply.

        - End of sequence RBSP semantics

The specifications in clause 7.4.3.6 apply with the following additions:

When included in a NAL unit with nuh_layer_id nuhLayerId, the end of sequence RBSP specifies that the next picture having nuh_layer_id equal to nuhLayerId or IdPredictedLayer[ nuhLayerId ][ i ] for any value of i in the range of 0 to NumPredictedLayers[ nuhLayerId ] − 1, inclusive, following the current access unit in the bitstream in decoding order (if any) is an IRAP picture with NoRaslOutputFlag equal to 1 and with nuh_layer_id equal to nuhLayerId.

        - End of bitstream RBSP semantics

The specifications in clause 7.4.3.7 apply.

        - Filler data RBSP semantics

The specifications in clause 7.4.3.8 apply.

        - Slice segment layer RBSP semantics

The specifications in clause 7.4.3.9 apply.

        - RBSP slice segment trailing bits semantics

The specifications in clause 7.4.3.10 apply.

        - RBSP trailing bits semantics

The specifications in clause 7.4.3.11 apply.

        - Byte alignment semantics

The specifications in clause 7.4.3.12 apply.

      - Profile, tier, and level semantics

The specifications in clause 7.4.4 apply with the replacement of each reference to "Annex A" with a reference to "Annex A, clause G.11, or clause H.11" and with the following additions and modifications:

The variable offsetVal is set equal to 0 and the variable VpsProfileTierLevel[ i ] is derived according to the following ordered steps:

–	VpsProfileTierLevel[ 0 ] is set equal to the profile_tier_level( ) syntax structure in the base VPS (i.e., in the VPS but not in the VPS extension syntax structure vps_extension( )), and offsetVal is set equal to 1.

–	When vps_max_layers_minus1 is greater than 0 and vps_base_layer_internal_flag is equal to 1, VpsProfileTierLevel[ offsetVal ] is set equal to the first profile_tier_level( ) syntax structure in the VPS extension, and offsetVal is incremented by 1.

–	For j in the range of 0 to ( vps_num_profile_tier_level_minus1 − (vps_base_layer_internal_flag ? 2 : 1) ), inclusive, VpsProfileTierLevel[ offsetVal + j ] is set equal to the j-th profile_tier_level( ) syntax structure signalled after the syntax element vps_num_profile_tier_level_minus1 in the VPS extension.

If vps_base_layer_internal_flag is equal to 0, all bits in the profile_tier_level( ) syntax structure VpsProfileTierLevel[ 0 ] shall be equal to 0 and decoders shall ignore the syntax structure VpsProfileTierLevel[ 0 ]. Otherwise, the semantics of the profile_tier_level( ) syntax structure VpsProfileTierLevel[ 0 ] are specified by the remaining part of the current clause.

The scope to which the profile_tier_level( ) syntax structure applies is specified as follows:

–	If the profile_tier_level( ) syntax structure is included in an active SPS for the base layer or is the profile_tier_level( ) syntax structure VpsProfileTierLevel[ 0 ], it applies to the OLS containing all layers in the bitstream but with only the base layer being the output layer.

–	Otherwise, if the profile_tier_level( ) syntax structure is included in an active SPS for an independent non-base layer with nuh_layer_id equal to layerId, it applies to the output bitstream of the independent non-base layer rewriting process of clause A.10.2 with the input variables assignedBaseLayerId equal to layerId and tIdTarget equal to 6.

–	Otherwise, if all of the following conditions are true, the profile_tier_level( ) syntax structure VpsProfileTierLevel[ profile_tier_level_idx[ olsIdx ][ 0 ] ] applies to the output bitstream of the independent non-base layer rewriting process of clause A.10.2 with the input variables assignedBaseLayerId equal to LayerSetLayerIdList[ OlsIdxToLsIdx[ olsIdx ] ][ 0 ] and tIdTarget equal to 6:

–	num_add_layer_sets is greater than 0.

–	OlsIdxToLsIdx[ olsIdx ] is in the range of FirstAddLayerSetIdx to LastAddLayerSetIdx, inclusive.

–	NumLayersInIdList[ OlsIdxToLsIdx[ olsIdx ] ] is equal to 1.

–	The profile_tier_level( ) syntax structure VpsProfileTierLevel[ profile_tier_level_idx[ olsIdx ][ 0 ] ] indicates a profile specified in Annex A.

–	The value of general_inbld_flag in the profile_tier_level( ) syntax structure VpsProfileTierLevel[ profile_tier_level_idx[ olsIdx ][ 0 ] ] is equal to 1.

–	Otherwise, the profile_tier_level( ) syntax structure provides profile, tier, and level to which a layer in an OLS conforms and the profile_tier_level_idx[ i ][ j ] syntax element specifies that the profile_tier_level( ) syntax structure VpsProfileTierLevel[ profile_tier_level_idx[ i ][ j ] ] applies to the j-th layer of the i-th OLS.

NOTE 1 – When vps_base_layer_internal_flag is equal to 1 and vps_max_layers_minus1 is greater than 0, the value of profile_tier_level_idx[ 0 ][ 0 ] is inferred to be equal to 1, such that VpsProfileTierLevel[ 1 ] applies to the 0-th OLS, which contains only the base layer that is the only output layer.

When the profile_tier_level( ) syntax structure is VpsProfileTierLevel[ k ] for k greater than 0 and profilePresentFlag is equal to 0, the syntax elements general_profile_space, general_tier_flag, general_profile_idc, general_profile_compatibility_flag[ j ], general_progressive_source_flag, general_interlaced_source_flag, general_non_packed_constraint_flag, general_frame_only_constraint_flag, general_max_12bit_‌constraint_flag, general_max_10bit_‌constraint_flag, general_max_8bit_‌constraint_flag, general_max_422chroma_‌constraint_flag, general_max_420chroma_‌constraint_flag, general_max_monochrome_‌constraint_flag, general_intra_‌constraint_flag, general_one_picture_only_‌constraint_flag, general_lower_bit_rate_‌constraint_flag, general_reserved_zero_34bits, general_reserved_zero_43bits, general_inbld_flag, and general_reserved_zero_1bit are not present in the profile_tier_level( ) syntax structure and each is inferred to be equal to the value of corresponding syntax element of the profile_tier_level( ) syntax structure VpsProfileTierLevel[ k − 1 ].

When the profile_tier_level( ) syntax structure is VpsProfileTierLevel[ k ] for k greater than 0 and any of the syntax elements sub_layer_profile_space[ i ], sub_layer_tier_flag[ i ], sub_layer_profile_idc[ i ], sub_layer_profile_compatibility_flag[ i ][ j ], sub_layer_progressive_source_flag[ i ], sub_layer_‌interlaced_source_flag[ i ], sub_layer_non_packed_‌constraint_flag[ i ], sub_layer_frame_only_‌constraint_flag[ i ], sub_layer_max_12bit_constraint_flag[ i ], sub_layer_max_10bit_‌constraint_flag[ i ], sub_layer_max_8bit_‌constraint_flag[ i ], sub_layer_max_422chroma_‌constraint_flag[ i ], sub_layer_max_420chroma_‌constraint_flag[ i ], sub_layer_max_monochrome_‌constraint_flag[ i ], sub_layer_intra_‌constraint_flag[ i ], sub_layer_one_picture_only_‌constraint_flag[ i ], sub_layer_lower_bit_rate_‌constraint_flag[ i ], sub_layer_reserved_zero_34bits[ i ], sub_layer_‌reserved_zero_43bits[ i ], sub_layer_inbld_flag[ i ], sub_layer_reserved_zero_1bit[ i ], and sub_layer_level_idc[ i ] is not present for any value of i in the range of 0 to maxNumSubLayersMinus1 − 1, inclusive, in the profile_tier_level( ) syntax structure, the value of the syntax element is inferred to be equal to the value of the corresponding syntax element of the profile_tier_level( ) syntax structure VpsProfileTierLevel[ k − 1 ].

: theyi) assumed that ii) iii) necessary  of an OLS

A sequence of pictures picSeq is derived as follows:

–	If the profile_tier_level( ) syntax structure is included in an SPS, picSeq consists of the pictures in the CLVS of the layer for which the SPS is the active SPS.

–	Otherwise, if the profile_tier_level( ) syntax structure is VpsProfileTierLevel[ 0 ], picSeq consists of the pictures with nuh_layer_id equal to 0 in the CVS.

–	Otherwise, if vps_max_layers_minus1 is greater than 0, vps_base_layer_internal_flag is equal to 1, and the profile_tier_level( ) syntax structure is VpsProfileTierLevel[ 1 ], picSeq consists of the pictures with nuh_layer_id equal to 0 in the CVS.

–	Otherwise (the profile_tier_level( ) syntax structure is VpsProfileTierLevel[ offsetVal + profile_tier_level_idx[ i ][ j ] ] for any values of i in the range of 0 to NumOutputLayerSets − 1, inclusive, and j in the range of 0 to NumLayersInIdList[ OlsIdxToLsIdx[ i ] ] − 1, inclusive, such that NecessaryLayerFlag[ i ][ j ] is equal to 1), picSeq consists of the pictures with nuh_layer_id equal to LayerSetLayerIdList[ OlsIdxToLsIdx[ i ] ][ j ] in the CVS.

general_progressive_source_flag and general_interlaced_source_flag are interpreted as follows:

–	If general_progressive_source_flag is equal to 1 and general interlaced_source_flag is equal to 0, the source scan type of the pictures in the picSeq should be interpreted as progressive only.

–	Otherwise, if general_progressive_source_flag is equal to 0 and general_interlaced_source_flag is equal to 1, the source scan type of the pictures in the picSeq should be interpreted as interlaced only.

–	Otherwise, if general_progressive_source_flag is equal to 0 and general_interlaced_source_flag is equal to 0, the source scan type of the pictures in the picSeq should be interpreted as unknown or unspecified.

–	Otherwise, general_progressive_source_flag is equal to 1 and general_interlaced_source_flag is equal to 1, the source scan type of each picture in the picSeq is indicated at the picture level using the syntax element source_scan_type in a picture timing SEI message or the syntax element ffinfo_source_scan_type in a frame-field information SEI message.

NOTE 3 – Decoders may ignore the values of general_progressive_source_flag and general_interlaced_source_flag for purposes other than determining the value to be inferred for frame_field_info_present_flag when vui_parameters_present_flag is equal to 0, as there are no other decoding process requirements associated with the values of these flags. Moreover, the actual source scan type of the pictures is outside the scope of this Specification, and the method by which the encoder selects the values of general_progressive_source_flag and general_interlaced_source_flag is unspecified.

general_non_packed_constraint_flag equal to 1 specifies that there are neither frame packing arrangement SEI messages nor segmented rectangular frame packing arrangement SEI messages present for the pictures of picSeq. general non_packed_constraint_flag equal to 0 indicates that there may or may not be one or more frame packing arrangement SEI messages or segmented rectangular frame packing arrangement SEI messages present for the pictures of picSeq.

NOTE 2 – Decoders may ignore the value of general_non_packed_constraint_flag, as there are no decoding process requirements associated with the presence or interpretation of frame packing arrangement SEI messages or segmented rectangular frame packing arrangement SEI messages.

general_frame_only_constraint_flag equal to 1 specifies that field_seq_flag in the active SPSs for the pictures of picSeq is equal to 0. general_frame_only_constraint_flag equal to 0 indicates that field_seq_flag in the active SPSs for the pictures of picSeq may or may not be equal to 0.

NOTE 3 – Decoders may ignore the value of general_frame_only_constraint_flag, as there are no decoding process requirements associated with the value of field_seq_flag.

NOTE 4 – When general_progressive_source_flag is equal to 1, general_frame_only_constraint_flag may or may not be equal to 1.

It is a requirement of bitstream conformance that when olsIdx is such that alt_output_layer_flag[ olsIdx ] is equal to 1, general_progressive_source_flag, general_interlaced_source_flag, general_non_packed_constraint_flag, and general_frame_only_constraint_flag in VpsProfileTierLevel[ offsetVal + profile_tier_level_idx[ olsIdx ][ j ] ] shall be equal to general_progressive_source_flag, general_interlaced_source_flag, general_non_packed_constraint_flag, and general_frame_only_constraint_flag, respectively, in VpsProfileTierLevel[ offsetVal + profile_tier_level_idx[ olsIdx ][ k ] ] for any values of j and k in the range of 0 to NumLayersInIdList[ OlsIdxToLsIdx[ olsIdx ] ] − 1, inclusive, such that NecessaryLayerFlag[ olsIdx ][ j ] is equal to 1 and NecessaryLayerFlag[ olsIdx ][ k ] is equal to 1.

      - Scaling list data semantics

The specifications in clause 7.4.5 apply.

      - Supplemental enhancement information message semantics

The specifications in clause 7.4.6 apply.

      - Slice segment header semantics

        - General slice segment header semantics

The specifications in clause 7.4.7.1 apply with the replacement of references to Annex C with references to clause A.13 and with the following additions:

When present, the value of the slice segment header syntax elements discardable_flag, cross_layer_bla_flag, inter_layer_pred_enabled_flag, num_inter_layer_ref_pics_minus1, poc_reset_idc, poc_reset_period_id, full_poc_reset_flag, poc_lsb_val, poc_msb_cycle_val_present_flag, and poc_msb_cycle_val shall be the same in all slice segment headers of a coded picture. When present, the value of the slice segment header syntax elements inter_layer_pred_layer_idc[ i ] shall be the same in all slice segment headers of a coded picture for each possible value of i.

When nal_unit_type has a value in the range of BLA_W_LP to RSV_IRAP_VCL23, inclusive, i.e., the picture is an IRAP picture, and NumDirectRefLayers[ nuh_layer_id ] is equal to 0, slice_type shall be equal to 2.

When sps_max_dec_pic_buffering_minus1[ TemporalId ] is equal to 0 and NumDirectRefLayers[ nuh_layer_id ] is equal to 0, slice_type shall be equal to 2.

discardable_flag equal to 1 specifies that the coded picture is not used as a reference picture for inter prediction and is not used as a source picture for inter-layer prediction in the decoding process of subsequent pictures in decoding order. discardable_flag equal to 0 specifies that the coded picture may be used as a reference picture for inter prediction and may be used as a source picture for inter-layer prediction in the decoding process of subsequent pictures in decoding order. When nal_unit_type is equal to TRAIL_R, TSA_R, STSA_R, RASL_R or RADL_R, the value of discardable_flag shall be equal to 0. When not present, the value of discardable_flag is inferred to be equal to 0.

NOTE 1 – When a coded picture has discardable_flag equal to 1 in its slice headers, the picture may be ignored by decoders withtout affecting the decoding result of any other picture, in the same layer or a different layer.

cross_layer_bla_flag equal to 1 affects the derivation of NoClrasOutputFlag and LayerResetFlag as specified in clause A.8.1.3. cross_layer_bla_flag shall be equal to 0 for pictures with nal_unit_type not equal to IDR_W_RADL or IDR_N_LP or with nuh_layer_id nuhLayerId not equal to any value for which NumDirectRefLayers[ nuhLayerId ] is equal to 0. When not present, the value of cross_layer_bla_flag is inferred to be equal to 0.

When vps_extension_flag is equal to 1, the sum of NumNegativePics[ CurrRpsIdx ], NumPositivePics[ CurrRpsIdx ], num_long_term_sps, and num_long_term_pics shall be less than or equal to MaxDpbSize − 1, where MaxDpbSize is as specified in clause B.11.2 or A.11.2.

inter_layer_pred_enabled_flag equal to 1 specifies that inter-layer prediction may be used in decoding of the current picture. inter_layer_pred_enabled_flag equal to 0 specifies that inter-layer prediction is not used in decoding of the current picture.

num_inter_layer_ref_pics_minus1 plus 1 specifies the number of pictures that may be used in decoding of the current picture for inter-layer prediction. The length of the num_inter_layer_ref_pics_minus1 syntax element is Ceil( Log2( NumDirectRefLayers[ nuh_layer_id ] ) ) bits. The value of num_inter_layer_ref_pics_minus1 shall be in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] − 1, inclusive.

The variables numRefLayerPics and refLayerPicIdc[ j ] are derived as follows:

for( i = 0, j = 0; i < NumDirectRefLayers[ nuh_layer_id ]; i++ ) {
	refLayerIdx = LayerIdxInVps[ IdDirectRefLayer[ nuh_layer_id ][ i ] ]
	if( sub_layers_vps_max_minus1[ refLayerIdx ]  >=  TemporalId  &&  ( TemporalId  = =  0  | |	(F-57)
					max_tid_il_ref_pics_plus1[ refLayerIdx ][ LayerIdxInVps[ nuh_layer_id ] ] > TemporalId ) )
		refLayerPicIdc[ j++ ] = i
}
numRefLayerPics = j

The variable NumActiveRefLayerPics is derived as follows:

if( nuh_layer_id  = =  0  | |  numRefLayerPics  = =  0 )
	NumActiveRefLayerPics = 0
else if( default_ref_layers_active_flag )
	NumActiveRefLayerPics = numRefLayerPics
else if( !inter_layer_pred_enabled_flag )		(F-58)
	NumActiveRefLayerPics = 0
else if( max_one_active_ref_layer_flag  | |  NumDirectRefLayers[ nuh_layer_id ]  = =  1 )
	NumActiveRefLayerPics = 1
else
	NumActiveRefLayerPics = num_inter_layer_ref_pics_minus1 + 1

All slices of a coded picture shall have the same value of NumActiveRefLayerPics.

inter_layer_pred_layer_idc[ i ] specifies the variable, RefPicLayerId[ i ], representing the nuh_layer_id of the i-th picture that may be used by the current picture for inter-layer prediction. The length of the inter_layer_pred_layer_idc[ i ] syntax element is Ceil( Log2( NumDirectRefLayers[ nuh_layer_id ] ) ) bits. The value of inter_layer_pred_layer_idc[ i ] shall be in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] − 1, inclusive. When i is greater than 0, inter_layer_pred_layer_idc[ i ] shall be greater than inter_layer_pred_layer_idc[ i − 1 ]. When not present, the value of inter_layer_pred_layer_idc[ i ] is inferred to be equal to refLayerPicIdc[ i ].

The variables RefPicLayerId[ i ] for all values of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive, are derived as follows:

for( i = 0, j = 0; i < NumActiveRefLayerPics; i++ )	(F-59)
	RefPicLayerId[ i ] = IdDirectRefLayer[ nuh_layer_id ][ inter_layer_pred_layer_idc[ i ] ]

It is a requirement of bitstream conformance that for each value of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive, either of the following two conditions shall be true:

–	The value of max_tid_il_ref_pics_plus1[ LayerIdxInVps[ RefPicLayerId[ i ] ] ][ LayerIdxInVps[ nuh_layer_id ] ] is greater than TemporalId.

–	The values of max_tid_il_ref_pics_plus1[ LayerIdxInVps[ RefPicLayerId[ i ] ] ][ LayerIdxInVps[ nuh_layer_id ] ] and TemporalId are both equal to 0 and the picture in the current access unit with nuh_layer_id equal to RefPicLayerId[ i ] is an IRAP picture.

poc_reset_idc equal to 0 specifies that neither the most significant bits nor the least significant bits of the picture order count value for the current picture are reset. poc_reset_idc equal to 1 specifies that only the most significant bits of the picture order count value for the current picture may be reset. poc_reset_idc equal to 2 specifies that both the most significant bits and the least significant bits of the picture order count value for the current picture may be reset. poc_reset_idc equal to 3 specifies that either only the most significant bits or both the most significant bits and the least significant bits of the picture order count value for the current picture may be reset and additional picture order count information is signalled. When not present, the value of poc_reset_idc is inferred to be equal to 0.

It is a requirement of bitstream conformance that the following constraints apply:

–	The value of poc_reset_idc shall not be equal to 1 or 2 for a RASL picture, a RADL picture, a sub-layer non-reference picture, or a picture that has TemporalId greater than 0, or a picture that has discardable_flag equal to 1.

–	The value of poc_reset_idc of all coded pictures that are present in the bitstream in an access unit shall be the same.

–	When the picture in an access unit with nuh_layer_id equal to 0 is an IRAP picture and vps_base_layer_internal_flag is equal to 1 and there is at least one other picture in the same access unit that is not an IRAP picture, the value of poc_reset_idc shall be equal to 1 or 2 for all pictures in the access unit.

–	When there is at least one picture that has nuh_layer_id greater than 0 and that is an IDR picture with a particular value of nal_unit_type in an access unit and there is at least one other coded picture that is present in the bitstream in the same access unit with a different value of nal_unit_type, the value of poc_reset_idc shall be equal to 1 or 2 for all pictures in the access unit.

–	The value of poc_reset_idc of a CRA or BLA picture shall be less than 3.

–	When the picture with nuh_layer_id equal to 0 in an access unit is an IDR picture and vps_base_layer_internal_flag is equal to 1 and there is at least one non-IDR picture in the same access unit, the value of poc_reset_idc shall be equal to 2 for all pictures in the access unit.

–	When the picture with nuh_layer_id equal to 0 in an access unit is not an IDR picture and vps_base_layer_internal_flag is equal to 1, the value of poc_reset_idc shall not be equal to 2 for any picture in the access unit.

–	When poc_lsb_not_present_flag[ LayerIdxInVps[ nuh_layer_id ] ] is equal to 1 and slice_pic_order_cnt_lsb is greater than 0, the value of poc_reset_idc shall not be equal to 2.

The value of poc_reset_idc of an access unit is the value of poc_reset_idc of the pictures in the access unit.

NOTE 2 – Encoders should be cautious in setting the value of poc_reset_idc of pictures when there is a picture of a layer not present in an access unit or there is a picture with discardable_flag equal to 1 present in an access unit, to ensure that the derived picture order count values of pictures within an access unit are cross-layer aligned. Basically, such cases should be treated similarly as access units with non-cross-layer-aligned IRAP pictures in terms of whether POC resetting is needed.

NOTE 3 – As specified in clause A.8.3.1, the most significant bits of the PicOrderCntVal of an IDR picture with poc_reset_idc equal to 0 are set equal to 0. As specified for the decoder conformance of output order DPB in clause A.13.5.2.2, an IDR picture with nuh_layer_id equal to SmallestLayerId does not cause the output of earlier access units, in decoding order, unless the IDR picture is a POC resetting picture, activates a new VPS, or causes NoClrasOutputFlag to be derived to be equal to 1. Encoders should be cautious when using poc_reset_idc equal to 0 with IDR pictures to maintain the desired picture output order and to obey the constraints on the consistent relation of output times to picture order counts as well as the constraints on the output order of two pictures in different CVSs.

poc_reset_period_id identifies a POC resetting period. There shall be no two pictures consecutive in decoding order in the same layer that have the same value of poc_reset_period_id and poc_reset_idc equal to 1 or 2. When not present, the value of poc_reset_period_id is inferred as follows:

–	If the previous picture picA that has poc_reset_period_id present in the slice segment header is present in the same layer of the bitstream as the current picture, the value of poc_reset_period_id is inferred to be equal to the value of the poc_reset_period_id of picA.

–	Otherwise, the value of poc_reset_period_id is inferred to be equal to 0.

NOTE 4 – It is not prohibited for multiple pictures in a layer to have the same value of poc_reset_period_id and to have poc_reset_idc equal to 1 or 2 unless such pictures occur in two consecutive access units in decoding order. To minimize the likelihood of such two pictures appearing in the bitstream due to picture losses, bitstream extraction, seeking, or splicing operations, encoders should set the value of poc_reset_period_id to be a random value for each POC resetting period (subject to the constraints specified above).

It is a requirement of bitstream conformance that the following constraints apply:

–	One POC resetting period shall not include more than one access unit with poc_reset_idc equal to 1 or 2.

–	An access unit with poc_reset_idc equal to 1 or 2 shall be the first access unit in a POC resetting period.

–	A picture that follows, in decoding order, the first POC resetting picture among all layers of a POC resetting period in decoding order shall not precede, in output order, another picture in any layer that precedes the first POC resetting picture in decoding order.

full_poc_reset_flag equal to 1 specifies that both the most significant bits and the least significant bits of the picture order count value for the current picture are reset when the previous picture in decoding order in the same layer does not belong to the same POC resetting period. full_poc_reset_flag equal to 0 specifies that only the most significant bits of the picture order count value for the current picture are reset when the previous picture in decoding order in the same layer does not belong to the same POC resetting period.

It is a requirement of bitstream conformance that when the value of poc_reset_idc of pictures in the first access unit in the same POC resetting period is equal to 1 or 2, the value of full_poc_reset_flag, when present, shall be equal to poc_reset_idc − 1.

poc_lsb_val specifies a value that may be used to derive the picture order count of the current picture. The length of the poc_lsb_val syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits.

When poc_lsb_not_present_flag[ LayerIdxInVps[ nuh_layer_id ] ] is equal to 1 and full_poc_reset_flag is equal to 1, the value of poc_lsb_val shall be equal to 0.

It is a requirement of bitstream conformance that, when poc_reset_idc is equal to 3, and the previous picture picA in decoding order that is in the same layer as the current picture, that has poc_reset_idc equal to 1 or 2, and that belongs to the same POC resetting period is present in the bitstream, picA shall be the same picture as the previous picture in decoding order that is in the same layer as the current picture, that is not a RASL picture, a RADL picture or a sub-layer non-reference picture, and that has TemporalId equal to 0 and discardable_flag equal to 0, and the value of poc_lsb_val of the current picture shall be equal to the value of slice_pic_order_cnt_lsb of picA.

The variable PocMsbValRequiredFlag is derived as follows:

PocMsbValRequiredFlag = CraOrBlaPicFlag  &&  	( !vps_poc_lsb_aligned_flag  | |	(FF-60)
				( vps_poc_lsb_aligned_flag  &&  NumDirectRefLayers[ nuh_layer_id ]  = =  0 ) )

poc_msb_cycle_val_present_flag equal to 1 specifies that poc_msb_cycle_val is present. poc_msb_cycle_val_present_flag equal to 0 specifies that poc_msb_cycle_val is not present. When not present, the value of poc_msb_cycle_val_present_flag is inferred as follows:

–	If slice_segment_header_extension_length is equal to 0, the value of poc_msb_cycle_val_present_flag is inferred to be equal to 0.

–	Otherwise, if PocMsbValRequiredFlag is equal to 1, the value of poc_msb_cycle_val_present_flag is inferred to be equal to 1.

–	Otherwise, the value of poc_msb_cycle_val_present_flag is inferred to be equal to 0.

NOTE 5 – When the current picture is an IDR picture with nuh_layer_id equal to 0 and is a POC resetting picture, vps_poc_lsb_aligned_flag is equal to 1, and NumPredictedLayers[ 0 ] is greater than 0, poc_msb_cycle_val_present_flag should be equal to 1.

poc_msb_cycle_val specifies the value that may be used to derive the picture order count value of the current picture or used to derive the value used to decrement the picture order count values of previously decoded pictures in the same layer as the current picture. When vps_poc_lsb_aligned_flag is equal to 1, poc_msb_cycle_val may also specify the value that is used to derive the value used to decrement the picture order count values of previously decoded pictures of the predicted layers of the current layer. The value of poc_msb_cycle_val shall be in the range of 0 to 232 − log2_max_pic_order_cnt_lsb_minus4 − 4, inclusive.

NOTE 6 – For a picture picA in layer layerA, let msbAnchorPic of the picture picA be the previous POC resetting picture in the layer layerA or the previous IDR picture that belongs to the layer layerA and has poc_msb_cycle_val_present_flag equal to 0, whichever is closer, in decoding order, to the picture picA. The value of poc_msb_cycle_val is set as follows:

–	If vps_poc_lsb_aligned_flag is equal to 0, the following applies:

–	If either of the following conditions is true, the value of poc_msb_cycle_val can be any value in the allowed range:

–	msbAnchorPic of the current picture is not present.

–	the current picture is the first picture in the current layer that belongs to or follows an access unit that contains an IRAP picture with nuh_layer_id equal to SmallestLayerId and NoClrasOutputFlag equal to 1.

–	Otherwise, if the current picture is a POC resetting picture, the value of poc_msb_cycle_val is equal to the difference between the values of the most significant bits of the picture order counts of the current picture, as if there was no POC reset operation for the current picture, and msbAnchorPic of the current picture.

–	Otherwise (the current picture is not a POC resetting picture), the value of poc_msb_cycle_val is equal to the difference between the values of the most significant bits of the picture order count values of the current picture and msbAnchorPic of the current picture.

–	Otherwise (vps_poc_lsb_aligned_flag is equal to 1), the following applies:

–	If the current picture is an IDR picture with nuh_layer_id equal to 0 and is a POC resetting picture, encoders should set the value of poc_msb_cycle_val as follows:

–	If picture picB other than the current picture is present in the current access unit, the difference between the most significant bits of the picture order counts of picB and msbAnchorPic of picB.

–	Otherwise (no picture picB other than the current picture is present in the current access unit), the difference between the most significant bits of the picture order counts of picB as if it would be present in the current access unit and msbAnchorPic of such a picB.

–	Otherwise, if the current picture is the first picture in the POC resetting period and has nuh_layer_id equal to SmallestLayerid, the value of poc_msb_cycle_val is equal to the difference between the values of the most significant bits of the picture order counts of the current picture, as if there was no POC reset operation for the current picture, and msbAnchorPic of the current picture.

–	Otherwise when the current picture is not a POC resetting picture, the value of poc_msb_cycle_val is equal to the difference between the values of the most significant bits of the picture order count values of the current picture and msbAnchorPic of the current picture.

slice_segment_header_extension_data_bit may have any value. Decoders shall ignore the value of slice_segment_header_extension_data_bit. Its value does not affect decoder conformance to profiles specified in this version of this Specification.

        - Reference picture list modification semantics

The specifications in clause 7.4.7.2 apply with following modifications:

–	Equation 7-43 specifying the derivation of NumPicTotalCurr is replaced by:

NumPicTotalCurr = 0
for( i = 0; i < NumNegativePics[ CurrRpsIdx ]; i++ )
	if( UsedByCurrPicS0[ CurrRpsIdx ][ i ] )
		NumPicTotalCurr++
for( i = 0; i < NumPositivePics[ CurrRpsIdx ]; i++ )		(FF-61)
	if( UsedByCurrPicS1[ CurrRpsIdx ][ i ] )
		NumPicTotalCurr++
for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ )
	if( UsedByCurrPicLt[ i ] )
		NumPicTotalCurr++
NumPicTotalCurr  +=  NumActiveRefLayerPics

        - Weighted prediction parameters semantics

The specifications in clause 7.4.7.3 apply.

      - Short-term reference picture set semantics

The specifications in clause 7.4.8 apply, with the following additions:

When vps_extension_flag is equal to 1, the value of num_negative_pics shall be in the range of 0 to MaxDpbSize − 1, inclusive, where MaxDpbSize is as specified in clause B.11.2 or A.11.2.

When vps_extension_flag is equal to 1, the value of num_positive_pics shall be in the range of 0 to MaxDpbSize − 1 − num_negative_pics, inclusive, where MaxDpbSize is as specified in clause B.11.2 or A.11.2.

      - Slice segment data semantics

        - General slice segment data semantics

The specifications in clause 7.4.9.1 apply.

        - Coding tree unit semantics

The specifications in clause 7.4.9.2 apply.

        - Sample adaptive offset semantics

The specifications in clause 7.4.9.3 apply.

        - Coding quadtree semantics

The specifications in clause 7.4.9.4 apply.

        - Coding unit semantics

The specifications in clause 7.4.9.5 apply.

        - Prediction unit semantics

The specifications in clause 7.4.9.6 apply.

        - PCM sample semantics

The specifications in clause 7.4.9.7 apply.

        - Transform tree semantics

The specifications in clause 7.4.9.8 apply.

        - Motion vector difference semantics

The specifications in clause 7.4.9.9 apply.

        - Transform unit semantics

The specifications in clause 7.4.9.10 apply.

        - Residual coding semantics

The specifications in clause 7.4.9.11 apply.

        - Cross-component prediction syntax

The specifications in clause 7.4.9.12 apply.

  - Decoding process

    - General decoding process

      - General

Input to this process is a bitstream. Output of this process is a list of decoded pictures.

The decoding process is specified such that all decoders capable of decoding an OLS where each of the output layers and the reference layers of the output layers is indicated to conform to a specified profile, tier, and level will produce numerically identical cropped decoded output pictures when invoking the decoding process associated with those profiles for the OLS. Any decoding process that produces identical cropped decoded output pictures to those produced by the process described herein (with the correct output order or output timing, as specified) conforms to the decoding process requirements of this Specification.

The following applies at the beginning of decoding a CVSG, after activating the VPS RBSP that is active for the entire CVSG and before decoding any VCL NAL units of the CVSG:

–	If vps_extension( ) is not present in the active VPS or a decoding process specified in Annex A is not in use, clause 8.1.2 is invoked with the CVSG as input.

–	Otherwise (vps_extension( ) is present in the active VPS and a decoding process specified in Annex A is in use), clause A.8.1.2 is invoked with the CVSG as input.

      - CVSG decoding process

Input to this process is a CVSG. Output of this process is a list of decoded pictures.

The variable TargetOlsIdx, which specifies the index to the list of the OLSs specified by the VPS, of the target OLS, is specified as follows:

–	If some external means, not specified in this Specification, is available to set TargetOlsIdx, TargetOlsIdx is set by the external means.

–	Otherwise, if the decoding process is invoked in a bitstream conformance test as specified in clause A.13.1, TargetOlsIdx is set as specified in clause A.13.1.

–	Otherwise, TargetOlsIdx is set equal to 0.

The variable TargetDecLayerSetIdx, the layer identifier list TargetOptLayerIdList, which specifies the list of nuh_layer_id values, in increasing order of nuh_layer_id values, of the pictures to be output, and the layer identifier list TargetDecLayerIdList, which specifies the list of nuh_layer_id values, in increasing order of nuh_layer_id values, of the NAL units to be decoded, are specified as follows:

TargetDecLayerSetIdx = OlsIdxToLsIdx[ TargetOlsIdx ]
	lsIdx = TargetDecLayerSetIdx
	for( i = 0, j = 0, k = 0; i < NumLayersInIdList[ lsIdx ]; i++ ) {
		if( NecessaryLayerFlag[ TargetOlsIdx ][ i ] )
			TargetDecLayerIdList[ j++ ] = LayerSetLayerIdList[ lsIdx ][ i ]									(F-62)-)
		if( OutputLayerFlag[ TargetOlsIdx ][ i ] )
			TargetOptLayerIdList[ k++ ] = LayerSetLayerIdList[ lsIdx ][ i ]
	}

When TargetOlsIdx is set by external means, it is a requirement for the external means that TargetOlsIdx is constrained as follows:

–	When vps_base_layer_available_flag is equal to 0, OlsIdxToLsIdx[ TargetOlsIdx ] shall be in the range of FirstAddLayerSetIdx to LastAddLayerSetIdx, inclusive.

–	When vps_base_layer_internal_flag is equal to 0, TargetOlsIdx shall be greater than 0.

The variable HighestTid, which identifies the highest temporal sub-layer to be decoded, is specified as follows:

–	If some external means, not specified in this Specification, is available to set HighestTid, HighestTid is set by the external means.

–	Otherwise, if the decoding process is invoked in a bitstream conformance test as specified in clause A.13.1, HighestTid is set as specified in clause A.13.1.

–	Otherwise, HighestTid is set equal to sps_max_sub_layers_minus1.

The variable SubPicHrdPreferredFlag is either specified by external means, or when not specified by external means, set equal to 0.

The variable SubPicHrdFlag is specified as follows:

–	If the decoding process is invoked in a bitstream conformance test as specified in clause A.13.1, SubPicHrdFlag is set as specified in clause A.13.1.

–	Otherwise, SubPicHrdFlag is set equal to ( SubPicHrdPreferredFlag  &&  sub_pic_hrd_params_present_flag ), where sub_pic_hrd_params_present_flag is found in any hrd_parameters( ) syntax structure that applies to at least one bitstream partition of the output layer set idenified by TargetOlsIdx.

A bitstream to be decoded, BitstreamToDecode, is specified as follows:

–	If TargetDecLayerSetIdx is less than or equal to vps_num_layer_sets_minus1 and vps_base_layer_internal_flag is equal to 1, the following applies:

–	The sub-bitstream extraction process as specified in clause 10 is applied with the CVSG, HighestTid, and TargetDecLayerIdList as inputs, and the output is assigned to a bitstream referred to as BitstreamToDecode.

–	The variable SmallestLayerId is set equal to 0.

–	Otherwise, if TargetDecLayerSetIdx is less than or equal to vps_num_layer_sets_minus1 and vps_base_layer_internal_flag is equal to 0, the following applies:

–	The sub-bitstream extraction process as specified in clause A.10.1 is applied with the CVSG, HighestTid, and TargetDecLayerIdList as inputs, and the output is assigned to a bitstream referred to as BitstreamToDecode.

–	The variable SmallestLayerId is set equal to 0.

–	Otherwise, if TargetDecLayerSetIdx is greater than vps_num_layer_sets_minus1 and NumLayersInIdList[ TargetDecLayerSetIdx ] is equal to 1, the following applies:

–	The independent non-base layer rewriting process of clause A.10.2 is applied with the CVSG, HighestTid, and TargetDecLayerIdList[ 0 ] as inputs, and the output is assigned to a bitstream referred to as BitstreamToDecode.

–	The variable SmallestLayerId is set equal to 0.

–	Otherwise, the following applies:

–	The sub-bitstream extraction process as specified in clause A.10.3 is applied with the CVSG, HighestTid, and TargetDecLayerIdList as inputs, and the output is assigned to a bitstream referred to as BitstreamToDecode.

–	The variable SmallestLayerId is set equal to TargetDecLayerIdList[ 0 ].

When vps_base_layer_internal_flag is equal to 0, vps_base_layer_available_flag is equal to 1, and TargetDecLayerSetIdx is in the range of 0 to vps_num_layer_sets_minus1, inclusive, the following applies:

–	The size of the sub-DPB for the layer with nuh_layer_id equal to 0 is set equal to 1.

–	The values of pic_width_in_luma_samples, pic_height_in_luma_samples, chroma_format_idc, separate_colour_plane_flag, bit_depth_luma_minus8, bit_depth_chroma_minus8, conf_win_left_offset, conf_win_right_offset, conf_win_top_offset, and conf_win_bottom_offset for decoded pictures with nuh_layer_id equal to 0 are set equal to the values of pic_width_vps_in_luma_samples, pic_height_vps_in_luma_samples, chroma_format_vps_idc, separate_colour_plane_vps_flag, bit_depth_vps_luma_minus8, bit_depth_vps_chroma_minus8, conf_win_vps_left_offset, conf_win_vps_right_offset, conf_win_vps_top_offset, and conf_win_vps_bottom_offset respectively, of the vps_rep_format_idx[ 0 ]-th rep_format( ) syntax structure in the active VPS.

–	The variable BaseLayerOutputFlag is set equal to ( TargetOptLayerIdList[ 0 ]  = =  0 ).

NOTE – The BaseLayerOutputFlag for each access unit is to be sent by an external means to the base layer decoder for controlling the output of base layer decoded pictures. BaseLayerOutputFlag equal to 1 indicates that the base layer is an output layer. BaseLayerOutputFlag equal to 0 indicates that the base layer is not an output layer.

–	The variable LayerInitializedFlag[ i ] is set equal to 0 for all values of i from 0 to vps_max_layer_id, inclusive, and the variable FirstPicInLayerDecodedFlag[ i ] is set equal to 0 for all values of i from 0 to vps_max_layer_id, inclusive.

If TargetOlsIdx is equal to 0, clause 8.1.3 is repeatedly invoked for each coded picture in BitstreamToDecode in decoding order and the decoding process for the current picture takes as inputs the syntax elements and upper-case variables from clause 7.

Otherwise (TargetOlsIdx is greater than 0), clause A.8.1.3 is repeatedly invoked for each coded picture in BitstreamToDecode in decoding order.

      - Common decoding process for a coded picture

The decoding processes specified in the remainder of this clause apply to each coded picture, referred to as the current picture and denoted by the variable CurrPic, in BitstreamToDecode.

Depending on the value of chroma_format_idc, the number of sample arrays of the current picture is as follows:

–	If chroma_format_idc is equal to 0, the current picture consists of 1 sample array SL.

–	Otherwise (chroma_format_idc is not equal to 0), the current picture consists of 3 sample arrays SL, SCb, SCr.

When interpreting the semantics of each syntax element in each NAL unit, the term "the bitstream" (or part thereof, e.g., a CVS of the bitstream) refers to BitstreamToDecode (or part thereof).

When vps_base_layer_internal_flag is equal to 0, vps_base_layer_available_flag is equal to 1, TargetDecLayerSetIdx is in the range of 0 to vps_num_layer_sets_minus1, inclusive, TemporalId is less than or equal to sub_layers_vps_max_minus1[ 0 ], and the current picture is the first coded picture of an access unit, clause A.8.1.8 is invoked prior to decoding the current picture.

When the current picture is an IRAP picture, the variable HandleCraAsBlaFlag is derived as specified in the following:

–	If some external means not specified in this Specification is available to set the variable HandleCraAsBlaFlag to a value for the current picture, the variable HandleCraAsBlaFlag is set equal to the value provided by the external means.

–	Otherwise, the variable HandleCraAsBlaFlag is set equal to 0.

When the current picture is an IRAP picture and has nuh_layer_id equal to SmallestLayerId, the following applies:

–	The variable NoClrasOutputFlag is specified as follows:

–	If the current picture is the first picture in the bitstream, NoClrasOutputFlag is set equal to 1.

–	Otherwise, if the current picture is included in the first access unit that follows an access unit including an end of sequence NAL unit with nuh_layer_id equal to SmallestLayerId or 0 in decoding order, NoClrasOutputFlag is set equal to 1.

–	Otherwise, if the current picture is a BLA picture or a CRA picture with HandleCraAsBlaFlag equal to 1, NoClrasOutputFlag is set equal to 1.

–	Otherwise, if the current picture is an IDR picture with cross_layer_bla_flag is equal to 1, NoClrasOutputFlag is set equal to 1.

–	Otherwise, if some external means, not specified in this Specification, is available to set NoClrasOutputFlag, NoClrasOutputFlag is set by the external means.

–	Otherwise, NoClrasOutputFlag is set equal to 0.

–	When NoClrasOutputFlag is equal to 1, the variable LayerInitializedFlag[ i ] is set equal to 0 for all values of i from 0 to vps_max_layer_id, inclusive, and the variable FirstPicInLayerDecodedFlag[ i ] is set equal to 0 for all values of i from 0 to vps_max_layer_id, inclusive.

The variables LayerResetFlag and dolLayerId are derived as follows:

–	If the current picture is an IRAP picture and has nuh_layer_id nuhLayerId greater than SmallestLayerId, the following applies:

–	If the current picture is the first picture, in decoding order, that follows an end of sequence NAL unit with nuh_layer_id equal to nuhLayerId, LayerResetFlag is set equal to 1 and dolLayerId is set equal to the nuh_layer_id value of the current NAL unit.

–	Otherwise, if the current picture is a CRA picture with HandleCraAsBlaFlag equal to 1, an IDR picture with cross_layer_bla_flag is equal to 1 or a BLA picture, LayerResetFlag is set equal to 1 and dolLayerId is set equal to the nuh_layer_id value of the current NAL unit.

–	Otherwise, LayerResetFlag is set equal to 0.

NOTE 1 – An end of sequence NAL unit, a CRA picture with HandleCraAsBlaFlag equal to 1, an IDR picture with cross_layer_bla_flag equal to 1, or a BLA picture, each with nuh_layer_id nuhLayerId greater than SmallestLayerId, may be present to indicate a discontinuity of the layer with nuh_layer_id equal to nuhLayerId and its predicted layers.

–	When LayerResetFlag is equal to 1, the following applies:

–	The values of LayerInitializedFlag and FirstPicInLayerDecodedFlag are updated as follows:

dolLayerIdx = LayerIdxInVps[ dolLayerId ]
for( i = 0; i < NumPredictedLayers[ dolLayerId ]; i++ ) {
	iLayerId = IdPredictedLayer[ dolLayerIdx ][ i ]	(F-63)
	LayerInitializedFlag[ iLayerId ] = 0
	FirstPicInLayerDecodedFlag[ iLayerId ] = 0
}

–	Each picture that is in the DPB and has nuh_layer_id equal to dolLayerId is marked as "unused for reference".

–	When NumPredictedLayers[ dolLayerId ] is greater than 0, each picture that is in the DPB and has nuh_layer_id equal to any value of IdPredictedLayer[ dolLayerId ][ i ] for the values of i in the range of 0 to NumPredictedLayers[ dolLayerId ] − 1, inclusive, is marked as "unused for reference".

–	Otherwise, LayerResetFlag is set equal to 0.

When the current picture is an IRAP picture, the following applies:

–	If the current picture with a particular value of nuh_layer_id is an IDR picture, a BLA picture, the first picture with that particular value of nuh_layer_id in the bitstream in decoding order, or the first picture with that particular value of nuh_layer_id that follows an end of sequence NAL unit with that particular value of nuh_layer_id in decoding order, the variable NoRaslOutputFlag is set equal to 1.

–	Otherwise, if LayerInitializedFlag[ nuh_layer_id ] is equal to 0 and LayerInitializedFlag[ refLayerId ] is equal to 1 for all values of refLayerId equal to IdDirectRefLayer[ nuh_layer_id ][ j ], where j is in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] − 1, inclusive, the variable NoRaslOutputFlag is set equal to 1.

–	Otherwise, the variable NoRaslOutputFlag is set equal to HandleCraAsBlaFlag.

When the current picture is an IRAP picture with NoRaslOutputFlag equal to 1 and one of the following conditions is true, LayerInitializedFlag[ nuh_layer_id ] is set equal to 1:

–	nuh_layer_id is equal to 0.

–	LayerInitializedFlag[ nuh_layer_id ] is equal to 0 and NumDirectRefLayers[ nuh_layer_id ] is equal to 0.

–	LayerInitializedFlag[ nuh_layer_id ] is equal to 0 and LayerInitializedFlag[ refLayerId ] is equal to 1 for all values of refLayerId equal to IdDirectRefLayer[ nuh_layer_id ][ j ], where j is in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] − 1, inclusive.

Depending on the value of separate_colour_plane_flag, the decoding process is structured as follows:

–	If separate_colour_plane_flag is equal to 0, the following decoding process is invoked a single time with the current picture being the output.

–	Otherwise (separate_colour_plane_flag is equal to 1), the following decoding process is invoked three times. Inputs to the decoding process are all NAL units of the coded picture with identical value of colour_plane_id. The decoding process of NAL units with a particular value of colour_plane_id is specified as if only a CVS with monochrome colour format with that particular value of colour_plane_id would be present in the bitstream. The output of each of the three decoding processes is assigned to one of the 3 sample arrays of the current picture, with the NAL units with colour_plane_id equal to 0, 1, and 2 being assigned to SL, SCb, and SCr, respectively.

NOTE 2 – The variable ChromaArrayType is derived as equal to 0 when separate_colour_plane_flag is equal to 1 and chroma_format_idc is equal to 3. In the decoding process, the value of this variable is evaluated resulting in operations identical to that of monochrome pictures (when chroma_format_idc is equal to 0).

The following applies for the decoding of the current picture:

–	If the current picture has nuh_layer_id equal to 0, the decoding process for the current picture takes as inputs the syntax elements and upper-case variables from clause A.7 and the decoding process for a coded picture with nuh_layer_id equal to 0 as specified in clause A.8.1.4 is invoked.

–	Otherwise, the following applies:

–	When vps_base_layer_internal_flag is equal to 0, vps_base_layer_available_flag is equal to 1, TargetDecLayerSetIdx is in the range of 0 to vps_num_layer_sets_minus1, inclusive, TemporalId is less than or equal to sub_layers_vps_max_minus1[ 0 ], the current picture is the first coded picture of an access unit, and a decoded picture with nuh_layer_id equal to 0 is provided by external means for the current access unit, clause A.8.1.9 is invoked after the decoding of the slice segment header of the first slice segment, in decoding order, of the current picture, but prior to decoding any slice segment of the first coded picture of the access unit.

–	For the decoding of the slice segment header of the first slice segment, in decoding order, of the current picture, the decoding process for starting the decoding of a coded picture with nuh_layer_id greater than 0 specified in clause A.8.1.5 is invoked.

–	Let lIdx be equal to such value for which the nuh_layer_id value of the current picture is equal to LayerSetLayerIdList[ OlsIdxToLsIdx[ TargetOlsIdx ] ][ lIdx ].

–	If general_profile_idc in the profile_tier_level( ) syntax structure VpsProfileTierLevel[ profile_tier_level_idx[ TargetOlsIdx ][ lIdx ] ] is equal to 6, the decoding process for the current picture takes as inputs the syntax elements and upper-case variables from clause B.7 and the decoding process of clause B.8.1.2 is invoked.

–	Otherwise, general_profile_idc in the profile_tier_level( ) syntax structure VpsProfileTierLevel[ profile_tier_level_idx[ TargetOlsIdx ][ lIdx ] ] is equal to 7, the decoding process for the current picture takes as inputs the syntax elements and upper-case variables from clause A.7 and the decoding process of clause A.8.1.2 is invoked.

–	After all slices of the current picture have been decoded, the decoding process for ending the decoding of a coded picture with nuh_layer_id greater than 0 specified in clause A.8.1.6 is invoked.

When the current picture is the last coded picture in an access unit in BitstreamToDecode, the following steps apply after the decoding of the current picture, prior to the decoding of the next picture:

–	PicOutputFlag is updated as follows:

–	If alt_output_layer_flag[ TargetOlsIdx ] is equal to 1 and the current access unit either does not contain a picture at the output layer or contains a picture at the output layer that has PicOutputFlag equal to 0, the following ordered steps apply:

–	The list nonOutputLayerPictures is set to be the list of the pictures of the access unit with PicOutputFlag equal to 1 and with nuh_layer_id values among the nuh_layer_id values of the reference layers of the output layer.

–	When the list nonOutputLayerPictures is not empty, the picture with the highest nuh_layer_id value among the list nonOutputLayerPictures is removed from the list nonOutputLayerPictures.

–	PicOutputFlag for each picture that is included in the list nonOutputLayerPictures is set equal to 0.

–	Otherwise, PicOutputFlag for pictures that are not included in an output layer is set equal to 0.

–	When vps_base_layer_internal_flag is equal to 0, vps_base_layer_available_flag is equal to 1, and TargetDecLayerSetIdx is in the range of 0 to vps_num_layer_sets_minus1, inclusive, the following applies:

–	If BaseLayerOutputFlag is equal to 0, the following applies:

–	If alt_output_layer_flag[ TargetOlsIdx ] is equal to 1, the base layer is a reference layer of the output layer, the access unit does not contain a picture at the output layer or contains a picture at the output layer that has PicOutputFlag equal to 0, and the access unit does not contain a picture at any other reference layer of the output layer, BaseLayerPicOutputFlag is set equal to 1.

–	Otherwise, BaseLayerPicOutputFlag is set equal to 0.

–	Otherwise, BaseLayerPicOutputFlag is set equal to 1.

NOTE 2 – The BaseLayerPicOutputFlag for each access unit is to be sent by an external means to the base layer decoder for controlling the output of base layer decoded pictures. BaseLayerPicOutputFlag equal to 1 for an access unit specifies that the base layer picture of the access unit is to be output. BaseLayerPicOutputFlag equal to 0 for an access unit specifies that the base layer picture of the access unit is not to be output.

–	The sub-DPB for the layer with nuh_layer_id equal to 0 is set to be empty.

–	The variable AuOutputFlag that is associated with the current access unit is derived as follows:

–	If at least one picture in the current access unit has PicOutputFlag equal to 1, AuOutputFlag is set equal to 1.

–	Otherwise, AuOutputFlag is set equal to 0.

–	The variable PicLatencyCount that is associated with the current access unit is set equal to 0.

–	When AuOutputFlag of the current access unit is equal to 1, for each access unit in the DPB that has at least one picture marked as "needed for output" and follows the current access unit in output order, the associated variable PicLatencyCount is set equal to PicLatencyCount + 1.

      - Decoding process for a coded picture with nuh_layer_id equal to 0

The specifications in clause 8.1.3 apply with the following changes:

–	Replace the references to clauses 8.2, 8.3, 8.3.1, 8.3.2, 8.3.3, 8.3.4, 8.4, 8.5, 8.6, and 8.7 with clauses F.8.2, F.8.3, F.8.3.1, F.8.3.2, F.8.3.3, F.8.3.4, F.8.4, F.8.5, F.8.6, and F.8.7, respectively.

–	At the end of the clause, add item 5 as follows:

5.	When FirstPicInLayerDecodedFlag[ 0 ] is equal to 0, FirstPicInLayerDecodedFlag[ 0 ] is set equal to 1.

      - Decoding process for starting the decoding of a coded picture with nuh_layer_id greater than 0

Each picture referred to in this clause is a complete coded picture.

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in clause A.8.2.

- The processes in clause A.8.3 specify the following decoding processes using syntax elements in the slice segment layer and above:

–	Variables and functions relating to picture order count are derived in clause A.8.3.1. This needs to be invoked only for the first slice segment of a picture. It is a requirement of bitstream conformance that PicOrderCntVal of each picture in an access unit shall have the same value during and at the end of decoding of the access unit.

NOTE – When the current picture is or succeeds, in decoding order, an IDR picture with nuh_layer_id equal to 0, PicOrderCntVal of a base layer picture picA preceding, in decoding order, the IDR picture with nuh_layer_id equal to 0 may or may not be equal to PicOrderCntVal of the pictures with nuh_layer_id greater than 0 in the access unit containing picA.

–	The decoding process for RPS in clause A.8.3.2 is invoked, wherein only reference pictures with nuh_layer_id equal to that of CurrPic may be marked as "unused for reference" or "used for long-term reference" and any picture with a different value of nuh_layer_id is not marked. This needs to be invoked only for the first slice segment of a picture.

–	When FirstPicInLayerDecodedFlag[ nuh_layer_id ] is equal to 0, the decoding process for generating unavailable reference pictures for pictures first in decoding order within a layer specified in clause A.8.1.7 is invoked, which needs to be invoked only for the first slice segment of a picture.

–	When FirstPicInLayerDecodedFlag[ nuh_layer_id ] is equal to 1 and the current picture is an IRAP picture with NoRaslOutputFlag equal to 1, the decoding process for generating unavailable reference pictures specified in clause A.8.3.3 is invoked, which needs to be invoked only for the first slice segment of a picture.

      - Decoding process for ending the decoding of a coded picture with nuh_layer_id greater than 0

The marking of decoded pictures is modified as specified in the following:

for( i = 0; i < NumActiveRefLayerPics0; i++ )
	RefPicSetInterLayer0[ i ] is marked as "used for short-term reference"	(F-64)

for( i = 0; i < NumActiveRefLayerPics1; i++ )
	RefPicSetInterLayer1[ i ] is marked as "used for short-term reference"

PicOutputFlag is set as follows:

–	If LayerInitializedFlag[ nuh_layer_id ] is equal to 0, PicOutputFlag is set equal to 0.

–	Otherwise, if the current picture is a RASL picture and NoRaslOutputFlag of the associated IRAP picture is equal to 1, PicOutputFlag is set equal to 0.

–	Otherwise, PicOutputFlag is set equal to pic_output_flag.

The decoded picture is marked as "used for short-term reference".

When FirstPicInLayerDecodedFlag[ nuh_layer_id ] is equal to 0, FirstPicInLayerDecodedFlag[ nuh_layer_id ] is set equal to 1.

      - Decoding process for generating unavailable reference pictures for pictures first in decoding order within a layer

- This process is invoked for a picture with nuh_layer_id equal to layerId, when FirstPicInLayerDecodedFlag[ layerId ] is equal to 0.

NOTE – The entire specification of the decoding process for CL-RAS pictures is included only for purposes of specifying constraints on the allowed syntax content of such CL-RAS pictures. During the decoding process, any CL-RAS pictures may be ignored, as these pictures are not specified for output and have no effect on the decoding process of any other pictures that are specified for output. However, in the HRD operations as specified in clause A.13, CL-RAS pictures may need to be taken into consideration in the derivation of CPB arrival and removal times.

When this process is invoked, the following applies:

–	For each RefPicSetStCurrBefore[ i ], with i in the range of 0 to NumPocStCurrBefore − 1, inclusive, that is equal to "no reference picture", a picture is generated as specified in clause 8.3.3.2, and the following applies:

–	The value of PicOrderCntVal for the generated picture is set equal to PocStCurrBefore[ i ].

–	The value of PicOutputFlag for the generated picture is set equal to 0.

–	The generated picture is marked as "used for short-term reference".

–	RefPicSetStCurrBefore[ i ] is set to be the generated reference picture.

–	The value of nuh_layer_id for the generated picture is set equal to nuh_layer_id.

–	For each RefPicSetStCurrAfter[ i ], with i in the range of 0 to NumPocStCurrAfter − 1, inclusive, that is equal to "no reference picture", a picture is generated as specified in clause 8.3.3.2, and the following applies:

–	The value of PicOrderCntVal for the generated picture is set equal to PocStCurrAfter[ i ].

–	The value of PicOutputFlag for the generated picture is set equal to 0.

–	The generated picture is marked as "used for short-term reference".

–	RefPicSetStCurrAfter[ i ] is set to be the generated reference picture.

–	The value of nuh_layer_id for the generated picture is set equal to nuh_layer_id.

–	For each RefPicSetStFoll[ i ], with i in the range of 0 to NumPocStFoll − 1, inclusive, that is equal to "no reference picture", a picture is generated as specified in clause 8.3.3.2, and the following applies:

–	The value of PicOrderCntVal for the generated picture is set equal to PocStFoll[ i ].

–	The value of PicOutputFlag for the generated picture is set equal to 0.

–	The generated picture is marked as "used for short-term reference".

–	RefPicSetStFoll[ i ] is set to be the generated reference picture.

–	The value of nuh_layer_id for the generated picture is set equal to nuh_layer_id.

–	For each RefPicSetLtCurr[ i ], with i in the range of 0 to NumPocLtCurr − 1, inclusive, that is equal to "no reference picture", a picture is generated as specified in clause 8.3.3.2, and the following applies:

–	The value of PicOrderCntVal for the generated picture is set equal to PocLtCurr[ i ].

–	The value of slice_pic_order_cnt_lsb for the generated picture is inferred to be equal to ( PocLtCurr[ i ] & ( MaxPicOrderCntLsb − 1 ) ).

–	The value of PicOutputFlag for the generated picture is set equal to 0.

–	The generated picture is marked as "used for long-term reference".

–	RefPicSetLtCurr[ i ] is set to be the generated reference picture.

–	The value of nuh_layer_id for the generated picture is set equal to nuh_layer_id.

–	For each RefPicSetLtFoll[ i ], with i in the range of 0 to NumPocLtFoll − 1, inclusive, that is equal to "no reference picture", a picture is generated as specified in clause 8.3.3.2, and the following applies:

–	The value of PicOrderCntVal for the generated picture is set equal to PocLtFoll[ i ].

–	The value of slice_pic_order_cnt_lsb for the generated picture is inferred to be equal to ( PocLtFoll[ i ] & ( MaxPicOrderCntLsb − 1 ) ).

–	The value of PicOutputFlag for the generated picture is set equal to 0.

–	The generated picture is marked as "used for long-term reference".

–	RefPicSetLtFoll[ i ] is set to be the generated reference picture.

–	The value of nuh_layer_id for the generated picture is set equal to nuh_layer_id.

      - Initialization process for an external base layer picture

A decoded picture with nuh_layer_id equal to 0 may be provided by external means. When not provided, no picture with nuh_layer_id equal to 0 is used for inter-layer prediction for the current access unit. When provided, the following applies:

–	The variable LayerInitializedFlag[ 0 ] is set equal to 1 and the variable FirstPicInLayerDecodedFlag[ 0 ] is set equal to 1.

–	The following information of the picture with nuh_layer_id equal to 0 for the access unit is provided by external means:

–	The decoded sample values (1 sample array SL if chroma_format_idc is equal to 0 or 3 sample arrays SL, SCb, and SCr otherwise)

–	The value of the variable BlIrapPicFlag, and when BlIrapPicFlag is equal to 1, the value of nal_unit_type of the decoded picture

–	BlIrapPicFlag equal to 1 specifies that the decoded picture is an IRAP picture. BlIrapPicFlag equal to 0 specifies that the decoded picture is a non-IRAP picture.

–	The provided value of nal_unit_type of the decoded picture shall be equal to IDR_W_RADL, CRA_NUT, or BLA_W_LP.

–	nal_unit_type equal to IDR_W_RADL specifies that the decoded picture is an IDR picture.

–	nal_unit_type equal to CRA_NUT specifies that the decoded picture is a CRA picture.

–	nal_unit_type equal to BLA_W_LP specifies that the decoded picture is a BLA picture.

–	When BlIrapPicFlag of the picture with nuh_layer_id equal to 0 is equal to 1, the following applies for the decoded picture with nuh_layer_id equal to 0 for the access unit:

–	The variable NoRaslOutputFlag is specified as follows:

–	If nal_unit_type is IDR_W_RADL or BLA_W_LP, the variable NoRaslOutputFlag is set equal to 1.

–	Otherwise, if the current access unit is the first access unit in the bitstream in decoding order, the variable NoRaslOutputFlag is set equal to 1.

–	Otherwise, the variable NoRaslOutputFlag is set equal to 0.

–	The variable NoClrasOutputFlag is specified as follows:

–	If the current access unit is the first access unit in the bitstream, NoClrasOutputFlag is set equal to 1.

–	Otherwise, if nal_unit_type is equal to BLA_W_LP, NoClrasOutputFlag is set equal to 1.

–	Otherwise, if some external means, not specified in this Specification, is available to set NoClrasOutputFlag, NoClrasOutputFlag is set by the external means.

–	Otherwise, NoClrasOutputFlag is set equal to 0.

–	When NoClrasOutputFlag is equal to 1, the variable LayerInitializedFlag[ i ] is set equal to 0 for all values of i from 1 to vps_max_layer_id, inclusive, and the variable FirstPicInLayerDecodedFlag[ i ] is set equal to 0 for all values of i from 1 to vps_max_layer_id, inclusive.

      - Decoding process for an external base layer picture

The following applies for the decoded picture with nuh_layer_id equal to 0 for the access unit:

–	TemporalId and PicOrderCntVal of the decoded picture with nuh_layer_id equal to 0 are set equal to the TemporalId and PicOrderCntVal, respectively, of any picture with nuh_layer_id greater than 0 in the access unit.

NOTE – The constraint on the value of TemporalId being required to be equal to 0 for IRAP pictures also applies to pictures with nuh_layer_id equal to 0 when vps_base_layer_internal_flag is equal to 0.

–	The decoded picture with nuh_layer_id equal to 0 is stored in the sub-DPB for the layer with nuh_layer_id equal to 0 and is marked as "used for long-term reference".

    - NAL unit decoding process

The specifications in clause 8.2 apply.

    - Slice decoding processes

      - Decoding process for picture order count

Output of this process is PicOrderCntVal, the picture order count of the current picture.

Picture order counts are used to identify pictures, for deriving motion parameters in merge mode and motion vector prediction, and for decoder conformance checking (see clause A.13.5).

Each coded picture is associated with a picture order count variable, denoted as PicOrderCntVal.

When the current picture is the first picture among all layers of a POC resetting period, the variable PocDecrementedInDPBFlag[ i ] is set equal to 0 for each value of i in the range of 0 to 62, inclusive.

The variable pocResettingFlag is derived as follows:

- If the current picture is a POC resetting picture, the following applies:

  - If vps_poc_lsb_aligned_flag is equal to 0, pocResettingFlag is set equal to 1.

  - Otherwise, if PocDecrementedInDPBFlag[ nuh_layer_id ] is equal to 1, pocResettingFlag is set equal to 0.

  - Otherwise, pocResettingFlag is set equal to 1.

- Otherwise, pocResettingFlag is set equal to 0.

The list affectedLayerList is derived as follows:

- If vps_poc_lsb_aligned_flag is equal to 0, affectedLayerList consists of the nuh_layer_id of the current picture.

- Otherwise, affectedLayerList consists of the nuh_layer_id of the current picture and the nuh_layer_id values equal to IdPredictedLayer[ currNuhLayerId ][ j ] for all values of j in the range of 0 to NumPredictedLayers[ currNuhLayerId ] − 1, inclusive, where currNuhLayerId is the nuh_layer_id value of the current picture.

Depending on pocResettingFlag, the following applies:

- If pocResettingFlag is equal to 1, the following applies:

  - When FirstPicInLayerDecodedFlag[ nuh_layer_id ] is equal to 1, the following applies:

  - The variables pocMsbDelta, pocLsbDelta and DeltaPocVal are derived as follows:

if( poc_reset_idc  = =  3 )
	pocLsbVal = poc_lsb_val
else
	pocLsbVal = slice_pic_order_cnt_lsb
if( poc_msb_cycle_val_present_flag )
	pocMsbDelta = poc_msb_cycle_val * MaxPicOrderCntLsb	(F-65)
else {
	prevPicOrderCntLsb = PrevPicOrderCnt[ nuh_layer_id ] & ( MaxPicOrderCntLsb − 1 )
	prevPicOrderCntMsb = PrevPicOrderCnt[ nuh_layer_id ] − prevPicOrderCntLsb
	pocMsbDelta = GetCurrMsb( pocLsbVal, prevPicOrderCntLsb, prevPicOrderCntMsb,
			MaxPicOrderCntLsb )
}
if( poc_reset_idc  = =  2  | |  ( poc_reset_idc  = =  3  &&  full_poc_reset_flag ) )
	pocLsbDelta = pocLsbVal
else
	pocLsbDelta = 0
DeltaPocVal = pocMsbDelta + pocLsbDelta

  - The PicOrderCntVal of each picture that has nuh_layer_id value nuhLayerId for which PocDecrementedInDPBFlag[ nuhLayerId ] is equal to 0 and that is equal to any value in affectedLayerList is decremented by DeltaPocVal.

  - PocDecrementedInDPBFlag[ nuhLayerId ] is set equal to 1 for each value of nuhLayerId included in affectedLayerList.

  - The PicOrderCntVal of the current picture is derived as follows:

if( poc_reset_idc  = =  1 )
	PicOrderCntVal = slice_pic_order_cnt_lsb
else if( poc_reset_idc  = =  2 )
	PicOrderCntVal = 0
else {
	PicOrderCntMsb =		(F-66)
		GetCurrMsb( slice_pic_order_cnt_lsb, full_poc_reset_flag ? 0 : poc_lsb_val,
				0, MaxPicOrderCntLsb )
	PicOrderCntVal = PicOrderCntMsb + slice_pic_order_cnt_lsb
}

- Otherwise (pocResettingFlag is equal to 0), the following applies:

  - The PicOrderCntVal of the current picture is derived as follows:

if( poc_msb_cycle_val_present_flag )
	PicOrderCntMsb = poc_msb_cycle_val * MaxPicOrderCntLsb
else if( !FirstPicInLayerDecodedFlag[ nuh_layer_id ]  | |
			nal_unit_type  = =  IDR_N_LP  | |  nal_unit_type  = =  IDR_W_RADL )
	PicOrderCntMsb = 0		(F-67)
else {
	prevPicOrderCntLsb = PrevPicOrderCnt[ nuh_layer_id ] & ( MaxPicOrderCntLsb − 1 )
	prevPicOrderCntMsb = PrevPicOrderCnt[ nuh_layer_id ] − prevPicOrderCntLsb
	PicOrderCntMsb = GetCurrMsb( slice_pic_order_cnt_lsb, prevPicOrderCntLsb,
			prevPicOrderCntMsb,
				 	MaxPicOrderCntLsb )
}
PicOrderCntVal = PicOrderCntMsb + slice_pic_order_cnt_lsb

The value of PrevPicOrderCnt[ lId ] for each of the lId values included in affectedLayerList is derived as follows:

  - If the current picture is not a RASL picture, a RADL picture or a sub-layer non-reference picture, and the current picture has TemporalId equal to 0 and discardable_flag equal to 0, PrevPicOrderCnt[ lId ] is set equal to PicOrderCntVal.

  - Otherwise, when poc_reset_idc is equal to 3 and one of the following conditions is true, PrevPicOrderCnt[ lId ] is set equal to ( full_poc_reset_flag ? 0 : poc_lsb_val ):

  - FirstPicInLayerDecodedFlag[ nuh_layer_id ] is equal to 0.

  - FirstPicInLayerDecodedFlag[ nuh_layer_id ] is equal to 1 and the current picture is a POC resetting picture.

The value of PicOrderCntVal shall be in the range of −231 to 231 − 1, inclusive.

It is a requirement of bitstream conformance that the current PicOrderCntVal values of any two pictures in the same layer and the same CVS shall not be the same.

NOTE 1 – PicOrderCntVal, as derived in this clause for the current picture, may be equal to initialPocVal, where initialPocVal is the PicOrderCntVal derived by this clause when an earlier picture, in decoding order, with nuh_layer_id equal to currNuhLayerId in the current CVS, was the current picture. However, the decoding processes applied subsequently have updated the PicOrderCntVal value for that earlier picture so that it no longer is equal to the PicOrderCntVal value derived in this clause for the current picture.

The function PicOrderCnt( picX ) is specified as follows:

PicOrderCnt( picX ) = PicOrderCntVal of the picture picX	(FF-68)

The function DiffPicOrderCnt( picA, picB ) is specified as follows:

DiffPicOrderCnt( picA, picB ) = PicOrderCnt( picA ) − PicOrderCnt( picB )	(FF-69)

The bitstream shall not contain data that result in values of DiffPicOrderCnt( picA, picB ) used in the decoding process that are not in the range of −215 to 215 − 1, inclusive.

NOTE 2 – Let X be the current picture and Y and Z be two other pictures in the same sequence, Y and Z are considered to be in the same output order direction from X when both DiffPicOrderCnt( X, Y ) and DiffPicOrderCnt( X, Z ) are positive or both are negative.

      - Decoding process for reference picture set

The specifications in clause 8.3.2 apply with the following changes:

- The references to clauses 7.4.7.2, 8.3.1, 8.3.3, and 8.3.4 are replaced with references to clauses A.7.4.7.2, A.8.3.1, A.8.3.3, and A.8.3.4, respectively.

- The following specifications are added:

- When the current picture is an IRAP picture with nuh_layer_id equal to SmallestLayerId, all reference pictures with any value of nuh_layer_id currently in the DPB (if any) are marked as "unused for reference" when at least one of the following conditions is true:

  - The current picture has NoClrasOutputFlag is equal to 1.

  - The current picture activates a new VPS.

- It is a requirement of bitstream conformance that the RPS is restricted as follows:

  - When the current picture is a CRA picture, there shall be no picture in RefPicSetStCurrBefore, RefPicSetStCurrAfter, or RefPicSetLtCurr.

- The constraints specified in clause 8.3.2 on the value of NumPicTotalCurr are replaced with the following:

- It is a requirement of bitstream conformance that the following applies to the value of NumPicTotalCurr:

  - If the current picture is a BLA or CRA picture and either currPicLayerId is equal to 0 or NumDirectRefLayers[ currPicLayerId ] is equal to 0, the value of NumPicTotalCurr shall be equal to 0.

  - Otherwise, when the current picture contains a P or B slice, the value of NumPicTotalCurr shall not be equal to 0.

      - Decoding process for generating unavailable reference pictures

The specifications in clause 8.3.3 and its subclauses apply with the replacement of references to Annex C with references to clause A.13.

      - Decoding process for reference picture lists construction

This process is invoked at the beginning of the decoding process for each P or B slice.

Reference pictures are addressed through reference indices as specified in clause 8.5.3.3.2. A reference index is an index into a reference picture list. When decoding a P slice, there is a single reference picture list RefPicList0. When decoding a B slice, there is a second independent reference picture list RefPicList1 in addition to RefPicList0.

At the beginning of the decoding process for each slice, the reference picture lists RefPicList0 and, for B slices, RefPicList1 are derived as follows:

- The variable NumRpsCurrTempList0 is set equal to Max( num_ref_idx_l0_active_minus1 + 1, NumPicTotalCurr ) and the list RefPicListTemp0 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList0 ) {
	for( i = 0; i < NumPocStCurrBefore  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrBefore[ i ]
	for( i = 0; i < NumActiveRefLayerPics0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetInterLayer0[ i ]
	for( i = 0;  i < NumPocStCurrAfter  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ )	(F-70)-)
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0; i < NumPocLtCurr  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetLtCurr[ i ]
	for( i = 0; i < NumActiveRefLayerPics1; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetInterLayer1[ i ]
}

- The list RefPicList0 is constructed as follows:

for( rIdx = 0; rIdx  <=  num_ref_idx_l0_active_minus1; rIdx++ )			(F-71)-)
		RefPicList0[ rIdx ] = ref_pic_list_modification_flag_l0 ? RefPicListTemp0[ list_entry_l0[ rIdx ] ] :
																			RefPicListTemp0[ rIdx ]

- When the slice is a B slice, the variable NumRpsCurrTempList1 is set equal to Max( num_ref_idx_l1_active_minus1 + 1, NumPicTotalCurr ) and the list RefPicListTemp1 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList1 ) {
	for( i = 0; i < NumPocStCurrAfter  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0; i< NumActiveRefLayerPics1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetInterLayer1[ i ]
	for( i = 0;  i < NumPocStCurrBefore  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ )	(F-72)-)
		RefPicListTemp1[ rIdx ] = RefPicSetStCurrBefore[ i ]
	for( i = 0; i < NumPocLtCurr  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetLtCurr[ i ]
	for( i = 0; i< NumActiveRefLayerPics0; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetInterLayer0[ i ]
}

- When the slice is a B slice, the list RefPicList1 is constructed as follows:

for( rIdx = 0; rIdx  <=  num_ref_idx_l1_active_minus1; rIdx++ )			(F-73)-)
		RefPicList1[ rIdx ] = ref_pic_list_modification_flag_l1 ? RefPicListTemp1[ list_entry_l1[ rIdx ] ] :
																			RefPicListTemp1[ rIdx ]

    - Decoding process for coding units coded in intra prediction mode

The specifications in clause 8.4 and its subclauses apply.

    - Decoding process for coding units coded in inter prediction mode

The specifications in clause 8.5 and its subclauses apply.

    - Scaling, transformation and array construction process prior to deblocking filter process

The specifications in clause 8.6 and its subclauses apply.

    - In-loop filter process

The specifications in clause 8.7 and its subclauses apply.

  - Parsing process

The specifications in clause 9 and its subclauses apply.

  - Specification of bitstream subsets

    - Sub-bitstream extraction process

Inputs to this process are a bitstream inBitstream, a target highest TemporalId value tIdTarget, and a target layer identifier list layerIdListTarget.

Output of this process is a sub-bitstream outBitstream.

When vps_base_layer_internal_flag is equal to 1 and vps_base_layer_available_flag is equal to 1, it is a requirement of bitstream conformance for inBitstream that any output sub-bitstream outBitstream that is the output of the process specified in this clause with inBitstream, tIdTarget equal to any value in the range of 0 to 6, inclusive, and layerIdListTarget equal to the layer identifier list associated with a layer set specified in the active VPS as inputs, and that satisfies both of the following conditions shall be a conforming bitstream:

–	The output sub-bitstream contains at least one VCL NAL unit with nuh_layer_id equal to each of the nuh_layer_id values in layerIdListTarget.

–	The output sub-bitstream contains at least one VCL NAL unit with TemporalId equal to tIdTarget.

NOTE 1 – When vps_base_layer_internal_flag is equal to 1 and vps_base_layer_available_flag is equal to 1, a conforming bitstream contains one or more coded slice segment NAL units with nuh_layer_id equal to 0 and TemporalId equal to 0.

The output sub-bitstream outBitstream is derived as follows:

–	The bitstream outBitstream is set to be identical to the bitstream inBitstream.

–	When one or more of the following two conditions are true, remove from outBitstream all SEI NAL units that have nuh_layer_id equal to 0 and that contain a non-nested buffering period SEI message, a non-nested picture timing SEI message, or a non-nested decoding unit information SEI message:

–	layerIdListTarget does not include all the values of nuh_layer_id in all NAL units in the bitstream.

–	tIdTarget is less than the greatest TemporalId in all NAL units in the bitstream.

NOTE 2 – A "smart" bitstream extractor may include appropriate non-nested buffering picture SEI messages, non-nested picture timing SEI messages, and non-nested decoding unit information SEI messages in the extracted sub-bitstream, provided that the SEI messages applicable to the sub-bitstream were present as nested SEI messages in the original bitstream.

–	Remove from outBitstream all NAL units with TemporalId greater than tIdTarget or nuh_layer_id not among the values included in layerIdListTarget.

    - Independent non-base layer rewriting process

Inputs to this process are a bitstream inBitstream, a target highest TemporalId value tIdTarget, and a nuh_layer_id value assignedBaseLayerId of an independent non-base layer of an additional layer set.

Output of this process is a sub-bitstream outBitstream.

It is a requirement of bitstream conformance for inBitstream that any output sub-bitstream outBitstream that is the output of the process specified in this clause shall otherwise be a conforming bitstream except that outBitstream does not contain any VPS NAL units, when all of the following conditions apply:

–	The output sub-bitstream outBitstream contains VCL NAL units with nuh_layer_id equal to assignedBaseLayerId.

–	The value of tIdTarget is equal to any value in the range of 0 to 6, inclusive, and outBitstream contains at least one VCL NAL unit with TemporalId equal to tIdTarget.

–	There is an OLS in the active VPS that consists of only the layer with nuh_layer_id equal to assignedBaseLayerId, the profile of that layer is a profile specified in Annex A, and the value of general_inbld_flag (when tIdTarget is equal to vps_max_sub_layers_minus1) or the value of sub_layer_inbld_flag[ tidTarget ] (when tidTarget is less than vps_max_sub_layers_minus1) in the profile_tier_level( ) syntax structure associated with that layer is equal to 1.

The output sub-bitstream outBitstream is derived from the bitstream inBitstream with the following ordered steps:

–	The bitstream outBitstream is set to be identical to the bitstream inBitstream.

–	NAL units with nal_unit_type not equal to SPS_NUT, PPS_NUT, and EOB_NUT and with nuh_layer_id not equal to the assignedBaseLayerId are removed from outBitstream.

–	NAL units with nal_unit_type equal to SPS_NUT or PPS_NUT with nuh_layer_id not equal to 0 or assignedBaseLayerId are removed from outBitstream.

–	NAL units with nal_unit_type equal to VPS_NUT are removed from outBitstream.

–	All NAL units with TemporalId greater than tIdTarget are removed from outBitstream.

–	nuh_layer_id is set equal to 0 in each NAL unit of outBitstream.

    - Sub-bitstream extraction process for additional layer sets

Inputs to this process are a bitstream inBitstream, a target highest TemporalId value tIdTarget, and a target layer identifier list layerIdListTarget of an additional layer set.

Output of this process is a sub-bitstream outBitstream.

It is a requirement of bitstream conformance for the input bitstream that the output sub-bitstream of the process specified in this clause shall be a conforming bitstream according to at least one profile in which vps_base_layer_available_flag may be equal to 0, when all of the following conditions apply:

–	The output sub-bitstream outBitstream contains VCL NAL units with each nuh_layer_id value in layerIdListTarget.

–	The value of tIdTarget is equal to any value in the range of 0 to 6, inclusive, and outBitstream contains at least one VCL NAL unit with TemporalId equal to tIdTarget.

–	layerIdListTarget is identical to LayerSetLayerIdList[ i ] for any value of i in the range of FirstAddLayerSetIdx to LastAddLayerSetIdx, inclusive.

The output sub-bitstream outBitstream is derived as follows:

–	The bitstream outBitstream is set to be identical to the bitstream inBitstream.

–	NAL units with nal_unit_type not equal to VPS_NUT, SPS_NUT, PPS_NUT, EOS_NUT, and EOB_NUT and with nuh_layer_id not equal to any value in layerIdListTarget are removed from outBitstream.

–	NAL units with nal_unit_type equal to VPS_NUT, SPS_NUT, PPS_NUT, or EOS_NUT with nuh_layer_id not equal to 0 or any value in layerIdListTarget are removed from outBitstream.

–	All NAL units with TemporalId greater than tIdTarget are removed from outBitstream.

–	vps_base_layer_available_flag in each VPS is set equal to 0.

  - Profiles, tiers, and levels

    - Independent non-base layer decoding capability

This clause specifies the independent non-base layer decoding (INBLD) capability, which is associated with the decoding capability of one or more of the profiles specified in Annex A. When expressing the capabilities of a decoder for one or more profiles specified in Annex A, whether the INBLD capability is supported for those profiles should also be expressed.

NOTE 1– The INBLD capability, when supported, indicates the capability of a decoder to decode an independent non-base layer that is indicated in the active VPSs and SPSs to conform to a profile specified in Annex A and is the layer with the smallest nuh_layer_id value in an additional layer set.

When the profile_tier_level( ) syntax structure is used for indicating of a decoder capability in systems, the INBLD capability may be indicated by setting the general_inbld_flag equal to 1 in the profile_tier_level( ) syntax structure used to express the profile, tier, and level that the decoder conforms to.

general_inbld_flag is set equal to 1 in the profile_tier_level( ) syntax structures in which a profile specified in Annex A is indicated and which are either specified in the VPS to be applicable for a non-base layer or included in an SPS activated for an independent non-base layer.

Decoders having the INBLD capability and conforming to a specific profile specified in Annex A at a specific level of a specific tier shall be capable of decoding any independent non-base layer or a sub-layer representation with TemporalId equal to i of the independent non-base layer for which all of the following condition applies for each active VPS:

–	There is an OLS that consists of the independent non-base layer and for which the associated profile_tier_level( ) syntax structure ptlStruct is constrained as follows:

–	ptlStruct indicates that the independent non-base layer or the sub-layer representation conforms to a profile specified in Annex A.

–	ptlStruct indicates that the independent non-base layer or the sub-layer representation conforms to a level lower than or equal to the specified level.

–	ptlStruct indicates that the independent non-base layer or the sub-layer representation conforms to a tier lower than or equal to the specified tier.

–	general_inbld_flag or sub_layer_inbld_flag[ i ] in ptlStruct is equal to 1.

NOTE 2– Let a derived bitstream outBitstream be a bitstream which is derived by invoking the independent non-base layer rewriting process specified in clause A.10.2 with a bitstream containing one or more independent non-base layers, tIdTarget equal to 6, and assignedBaseLayerId equal to the smallest nuh_layer_id value of additional layer set as inputs. As specified elsewhere in this Specification, it is a requirement of bitstream conformance that outBitstream is otherwise a conforming bitstream but does not contain VPSs. Consequently, decoders with INBLD capability may apply the independent non-base layer rewriting process specified in clause A.10.2 to obtain outBitstream and then apply a decoding process for a profile specified in Annex A with outBitstream as input.

NOTE 3– The following constraints are necessary to ensure that a sub-bitstream derived by invoking the independent non-base layer rewriting process specified in clause A.10.2 conforms to a profile specified in Annex A:

–	All active SPSs for the independent non-base layer have nuh_layer_id equal to 0 or MultiLayerExtSpsFlag equal to 0.

–	Each active VPS has poc_lsb_not_present_flag[ i ] equal to 1 for each value of i for which i is the layer index of the independent non-base layer.

    - Decoder capabilities

This clause specifies requirements for decoders having the capability of decoding an output operation point with one or more necessary layers.

NOTE 1 – For example, this clause specifies the requirements for a decoder supporting simultaneous decoding of the base layer and an independent non-base layer for which the INBLD capability is needed. Moreover, this clause specifies the requirements for a decoder supporting decoding of layers conforming to profiles specified in Annex B or A.

A decoder that conforms to a list ptliList consisting of profile, tier, level, and INBLD capability quadruplets ( Pi, Ti, Li, InbldFlagi ) for i in the range of 0 to d − 1, inclusive, where d is a positive integer, shall be capable of decoding any output operation point containing b necessary layers, where b is less than or equal to d, when the following condition applies:

–	There exists a reordered list, orderedPtliList, of the profile, tier, level, and INBLD capability quadruplets ( orderedPj, orderedTj, orderedLj, orderedInbldFlagj ) of the list ptliList such that all the following conditions apply for each value of j in the range of 0 to b − 1, inclusive, where the variable bLayer[ j ] represents the j-th necessary layer in the output operation point:

–	The profile to which bLayer[ j ] is indicated to conform is included in CompatibleProfileList for the profile orderedPj as specified in Table FF.3.

–	The tier to which bLayer[ j ] is indicated to conform is lower than or equal to the tier orderedTj.

–	The level to which bLayer[ j ] is indicated to conform is lower than or equal to the level orderedLj.

–	When the profile to which bLayer[ j ] is indicated to conform is a profile specified in Annex A, the value of general_inbld_flag (when OpTid of the output operation point is equal to vps_max_sub_layers_minus1) or sub_layer_inbld_flag[ OpTid ] (when OpTid of the output operation point is less than vps_max_sub_layers_minus1) of bLayer[ j ] is less than or equal to orderedInbldFlagj.

Table FF.3 – Specification of CompatibleProfileList

| - Profile to which the decoder conforms | - Profiles that the decoder shall support CompatibleProfileList |
| --- | --- |
| Scalable Main | - Scalable Main, Main, Main Still Picture |
| Scalable Main 10 | - Scalable Main 10, Main, Main Still Picture, Main 10, Scalable Main |
| Multiview Main | - Multiview Main, Main, Main Still Picture |

  - Byte stream format

The specifications in Annex B apply.

  - Hypothetical reference decoder

    - General

Three sets of bitstream conformance tests are needed for checking the conformance of a bitstream, which is referred to as the entire bitstream, denoted as entireBitstream. The first set of bitstream conformance tests are for testing the conformance of the entire bitstream and its temporal subsets, regardless of whether there is a layer set specified by the active VPS that contains all the nuh_layer_id values of VCL NAL units present in the entire bitstream. The second set of bitstream conformance tests are for testing the conformance of the layer sets specified by the active VPS and their temporal subsets. For all these tests, only the base layer pictures (i.e., pictures with nuh_layer_id equal to 0) are decoded and other pictures are ignored by the decoder when the decoding process is invoked.

The first and second sets of bitstream conformance tests are specified in clause C.1. Clause A.13 and its subclauses specify the HRD operations for the third sets of bitstream conformance tests.

The third set of bitstream conformance tests are for testing the conformance of the OLSs specified by the VPS extension part of the active VPS and their temporal subsets. For each test in the third set of bitstream conformance tests, the following steps apply in the order listed, followed by the processes described after these steps in this clause:

- An output operation point under test, denoted as TargetOp, is selected by selecting a value for TargetOlsIdx identifying a target OLS and selecting a target highest TemporalId value HighestTid. The value of TargetOlsIdx shall be in the range of 0 to NumOutputLayerSets − 1, inclusive, and the value of HighestTid shall be in the range of 0 to MaxSubLayersInLayerSetMinus1[ OlsIdxToLsIdx[ TargetOlsIdx ] ], inclusive. Additionally, the values of TargetOlsIdx and HighestTid are constrained as specified in the next step.

The variables TargetDecLayerSetIdx, TargetOptLayerIdList, and TargetDecLayerIdList are then derived as specified in clause A.8.1.2. The output operation point under test has OptLayerIdList equal to TargetOptLayerIdList, OpLayerIdList equal to TargetDecLayerIdList, and OpTid equal to HighestTid.

- A bitstream to be decoded, BitstreamToDecode, is specified as follows:

–	If TargetDecLayerSetIdx is less than or equal to vps_num_layer_sets_minus1 and vps_base_layer_internal_flag is equal to 1, the sub-bitstream extraction process as specified in clause 10 is applied with the bitstream entireBitstream, HighestTid, and TargetDecLayerIdList as inputs, and the output is assigned to BitstreamToDecode.

–	Otherwise, if TargetDecLayerSetIdx is less than or equal to vps_num_layer_sets_minus1 and vps_base_layer_internal_flag is equal to 0, the sub-bitstream extraction process as specified in clause A.10.1 is applied with the bitstream entireBitstream, HighestTid, and TargetDecLayerIdList as inputs, and the output is assigned to BitstreamToDecode.

–	Otherwise, if TargetDecLayerSetIdx is greater than vps_num_layer_sets_minus1, NumLayersInIdList[ TargetDecLayerSetIdx ] is equal to 1, and the profile of the layer in TargetOlsIdx is one of those specified in Annex A, the independent non-base layer rewriting process of clause A.10.2 is applied with the bitstream entireBitstream, HighestTid, and TargetDecLayerIdList[ 0 ] as inputs, and the output is assigned to BitstreamToDecode.

–	Otherwise, the sub-bitstream extraction process as specified in clause A.10.3 is applied with the bitstream entireBitstream, HighestTid, and TargetDecLayerIdList as inputs, and the output is assigned to BitstreamToDecode.

The values of TargetOlsIdx and HighestTid are additionally constrained as follows:

–	When vps_base_layer_available_flag is equal to 0, OlsIdxToLsIdx[ TargetOlsIdx ] shall be in the range of FirstAddLayerSetIdx to LastAddLayerSetIdx, inclusive.

–	When vps_base_layer_internal_flag is equal to 0, TargetOlsIdx shall be greater than 0.

–	The value of TargetOlsIdx shall be such that there is at least one VCL NAL unit in BitstreamToDecode with nuh_layer_id equal to LayerSetLayerIdList[ OlsIdxToLsIdx[ TargetOlsIdx ] ][ i ] for each value of i in the range of 0 to NumLayersInIdList[ OlsIdxToLsIdx[ TargetOlsIdx ] ] − 1, inclusive.

–	The value of HighestTid shall be such there is at least one VCL NAL unit with TemporalId equal to HighestTid in BitstreamToDecode.

- A partitioning scheme is selected from the list of partitioning schemes signalled in the active VPS for the selected OLS. The selected partitioning scheme is denoted as TargetPartitioningScheme with partitioning scheme index TargetPsIdx.

- The subsequent steps apply to each bitstream partition, referred to as the bitstream partition under test TargetBitstreamPartition, of the selected partitioning scheme of the target OLS. If there is only one bitstream partition for TargetPartitioningScheme, the TargetBitstreamPartition is identical to BitstreamToDecode. Otherwise, each bitstream partition is derived with the demultiplexing process for deriving a bitstream partition in clause A.13.6, with BitstreamToDecode, the list of layers in TargetBitstreamPartition, and the number of layers in TargetBitstreamPartition as inputs.

- The applicable hrd_parameters( ) syntax structures and the sub_layer_hrd_parameters( ) syntax structures are selected as follows:

–	A SchedSelCombIdx is selected for BitstreamToDecode and used for each TargetBitstreamPartition. The selected SchedSelCombIdx shall be in the range of 0 to num_bsp_schedules_minus1[ TargetOlsIdx ][ TargetPsIdx ][ HighestTid ], inclusive.

–	The applicable hrd_parameters( ) syntax structure is the bsp_hrd_idx[ TargetOlsIdx ][ TargetPsIdx ][ HighestTid ][ SchedSelCombIdx ][ j ]-th hrd_parameters( ) syntax structure in the active VPS, where j is the bitstream partition index of TargetBitstreamPartition.

Within the selected hrd_parameters( ) syntax structures, if BitstreamToDecode is a Type I bitstream, the sub_layer_hrd_parameters( HighestTid ) syntax structures that immediately follow the condition "if( vcl_hrd_parameters_present_flag )" are selected and the variable NalHrdModeFlag is set equal to 0; otherwise (BitstreamToDecode is a Type II bitstream), the sub_layer_hrd_parameters( HighestTid ) syntax structures that immediately follow either the condition "if( vcl_hrd_parameters_present_flag )" (in this case the variable NalHrdModeFlag is set equal to 0) or the condition "if( nal_hrd_parameters_present_flag )" (in this case the variable NalHrdModeFlag is set equal to 1) are selected. When BitstreamToDecode is a Type II bitstream and NalHrdModeFlag is equal to 0, all non-VCL NAL units except filler data NAL units, and all leading_zero_8bits, zero_byte, start_code_prefix_one_3bytes, and trailing_zero_8bits syntax elements that form a byte stream from the NAL unit stream (as specified in Annex B), when present, are discarded from TargetBitstreamPartition, and the remaining sub-bitstream is assigned to TargetBitstreamPartition.

- An access unit associated with a buffering period SEI message (present in a bitstream partition nesting SEI message in BitstreamToDecode or available through external means not specified in this Specification) applicable to TargetOp, TargetPartitioningScheme, and TargetBitstreamPartition is selected as the HRD initialization point and referred to as access unit 0.

The variable MultiLayerCpbOperationFlag is derived as follows:

–	If BitstreamToDecode contains only the base layer (i.e., TargetOlsIdx is equal to 0), MultiLayerCpbOperationFlag is set equal to 0.

–	Otherwise, MultiLayerCpbOperationFlag is set equal to 1.

- If sub_pic_hrd_params_present_flag in the selected hrd_parameters( ) syntax structure is equal to 1, the CPB is scheduled to operate either at the partition unit level (in which case the variable SubPicHrdFlag is set equal to 0) or at the sub-partition level (in which case the variable SubPicHrdFlag is set equal to 1). Otherwise, SubPicHrdFlag is set equal to 0 and the CPB is scheduled to operate at the partition unit level.

- For each access unit in TargetBitstreamPartition starting from access unit 0, the buffering period SEI message (present in a bitstream partition nesting SEI message in BitstreamToDecode or available through external means not specified in this Specification) that is associated with the access unit and applies to TargetOp, TargetPartitioningScheme, and TargetBitstreamPartition is selected, the picture timing SEI message (present in a bitstream partition nesting SEI message in BitstreamToDecode or available through external means not specified in this Specification) that is associated with the access unit and applies to TargetOp, TargetPartitioningScheme, and TargetBitstreamPartition is selected, and when SubPicHrdFlag is equal to 1 and sub_pic_cpb_params_in_pic_timing_sei_flag is equal to 0, the decoding unit information SEI messages (present in bitstream partition nesting SEI messages in BitstreamToDecode or available through external means not specified in this Specification) that are associated with decoding units in the access unit and apply to TargetOp, TargetPartitioningScheme, and TargetBitstreamPartition are selected.

- A value of SchedSelIdx for TargetBitstreamPartition is set equal to bsp_sched_idx[ TargetOlsIdx ][ TargetPsIdx ][ HighestTid ][ SchedSelCombIdx ][ j ], where j is the index of the bitstream partition index of TargetBitstreamPartition.

- The variable initialAltParamSelectionFlag is derived as follows:

–	If all of the following conditions are true, initialAltParamSelectionFlag is set equal to 1:

–	The coded picture with nuh_layer_id equal to SmallestLayerId in access unit 0 has nal_unit_type equal to CRA_NUT or BLA_W_LP.

–	MultiLayerCpbOperationFlag is equal to 0.

–	irap_cpb_params_present_flag in the selected buffering period SEI message is equal to 1.

–	Otherwise, if all of the following conditions are true, initialAltParamSelectionFlag is set equal to 1:

–	The coded picture with nuh_layer_id equal to SmallestLayerId in access unit 0 is an IRAP picture.

–	MultiLayerCpbOperationFlag is equal to 1.

–	irap_cpb_params_present_flag in the selected buffering period SEI message is equal to 1.

–	Otherwise, initialAltParamSelectionFlag is set equal to 0.

- When initialAltParamSelectionFlag is equal to 1, the following applies:

–	The variable skippedPictureList is set to consist of the CL-RAS pictures and the RASL pictures associated with the IRAP pictures with nuh_layer_id equal to nuhLayerId for which LayerInitializedFlag[ nuhLayerId ] is equal to 0 at the start of decoding the IRAP picture and for which nuhLayerId is among TargetDecLayerIdList.

–	Either of the following applies for selection of the initial CPB removal delay and delay offset:

–	If NalHrdModeFlag is equal to 1, the default initial CPB removal delay and delay offset represented by nal_initial_cpb_removal_delay[ SchedSelIdx ] and nal_initial_cpb_removal_offset[ SchedSelIdx ], respectively, in the selected buffering period SEI message are selected. Otherwise, the default initial CPB removal delay and delay offset represented by vcl_initial_cpb_removal_delay[ SchedSelIdx ] and vcl_initial_cpb_removal_offset[ SchedSelIdx ], respectively, in the selected buffering period SEI message are selected. The variable DefaultInitCpbParamsFlag is set equal to 1.

–	If NalHrdModeFlag is equal to 1, the alternative initial CPB removal delay and delay offset represented by nal_initial_alt_cpb_removal_delay[ SchedSelIdx ] and nal_initial_alt_cpb_removal_offset[ SchedSelIdx ], respectively, in the selected buffering period SEI message are selected. Otherwise, the alternative initial CPB removal delay and delay offset represented by vcl_initial_alt_cpb_removal_delay[ SchedSelIdx ] and vcl_initial_alt_cpb_removal_offset[ SchedSelIdx ], respectively, in the selected buffering period SEI message are selected. The variable DefaultInitCpbParamsFlag is set equal to 0, and all the pictures in skippedPictureList are discarded from BitstreamToDecode and the remaining bitstream is assigned to BitstreamToDecode.

Each conformance test consists of a combination of one option in each of the above steps. When there is more than one option for a step, for any particular conformance test only one option is chosen. All possible combinations of all the steps form the entire set of conformance tests.

When BitstreamToDecode is a Type II bitstream, the following applies:

–	If the sub_layer_hrd_parameters( HighestTid ) syntax structure that immediately follows the condition "if( vcl_hrd_parameters_present_flag )" is selected, the test is conducted at the Type I conformance point, and only VCL and filler data NAL units are counted for the input bit rate and CPB storage.

–	Otherwise (the sub_layer_hrd_parameters( HighestTid ) syntax structure that immediately follows the condition "if( nal_hrd_parameters_present_flag )" is selected), the test is conducted at the Type II conformance point, and all bytes of the Type II bitstream, which may be a NAL unit stream or a byte stream, are counted for the input bit rate and CPB storage.

NOTE 1 – NAL HRD parameters established by a value of SchedSelIdx for the Type II conformance point are sufficient to also establish VCL HRD conformance for the Type I conformance point for the same values of InitCpbRemovalDelay[ SchedSelIdx ], BitRate[ SchedSelIdx ], and CpbSize[ SchedSelIdx ] for the VBR case (cbr_flag[ SchedSelIdx ] equal to 0). This is because the data flow into the Type I conformance point is a subset of the data flow into the Type II conformance point and because, for the VBR case, the CPB is allowed to become empty and stay empty until the time a next picture is scheduled to begin to arrive. For example, when decoding a CVS conforming to one or more of the profiles specified in Annex B using the decoding process specified in clauses B.2 through B.10, when NAL HRD parameters are provided for the Type II conformance point that not only fall within the bounds set for NAL HRD parameters for profile conformance in item f) of clause B.11.2.2 but also fall within the bounds set for VCL HRD parameters for profile conformance in item e) of clause B.11.2.2, conformance of the VCL HRD for the Type I conformance point is also assured to fall within the bounds of item e) of clause B.11.2.2.

All VPSs, SPSs and PPSs referred to in the VCL NAL units, and the corresponding buffering period, picture timing and decoding unit information SEI messages shall be conveyed to the HRD, in a timely manner, either in the bitstream (by non-VCL NAL units), or by other means not specified in this Specification.

In Annexes C, D, and E, the specification for "presence" of non-VCL NAL units that contain VPSs, SPSs, PPSs, buffering period SEI messages, picture timing SEI messages, or decoding unit information SEI messages is also satisfied when those NAL units (or just some of them) are conveyed to decoders (or to the HRD) by other means not specified in this Specification. For the purpose of counting bits, only the appropriate bits that are actually present in the bitstream are counted.

NOTE 2 – As an example, synchronization of such a non-VCL NAL unit, conveyed by means other than presence in the bitstream, with the NAL units that are present in the bitstream, can be achieved by indicating two points in the bitstream, between which the non-VCL NAL unit would have been present in the bitstream, had the encoder decided to convey it in the bitstream.

When the content of such a non-VCL NAL unit is conveyed for the application by some means other than presence within the bitstream, the representation of the content of the non-VCL NAL unit is not required to use the same syntax as specified in this Specification.

NOTE 3 – When HRD information is contained within the bitstream, it is possible to verify the conformance of a bitstream to the requirements of this clause based solely on information contained in the bitstream. When the HRD information is not present in the bitstream, as is the case for all "stand-alone" Type I bitstreams, conformance can only be verified when the HRD data are supplied by some other means not specified in this Specification.

For the bitstream-partition-specific CPB operation as specified in this annex, the HRD contains a bitstream demultiplexer (optionally present), two or more bitstream partition buffers (BPB), two or more instantaneous decoding processes, a decoded picture buffer (DPB) that contains a sub-DPB for each layer, and output cropping as shown in Figure F.1.

Figure F.1 – Bitstream-partition-specific HRD buffer model

For each bitstream conformance test, the size of BPB (or CPB) CpbSize[ SchedSelIdx ] is BpbSize[ TargetOlsIdx ][ TargetPsIdx ][ HighestTid ][ SchedSelCombIdx ][ j ], and the BitRate[ SchedSelIdx ] is BpBitRate[ TargetOlsIdx ][ TargetPsIdx ][ HighestTid ][ SchedSelCombIdx ][ j ], where j is the bitstream partition index of TargetBitstreamPartition, and SchedSelIdx and the HRD parameters are specified above in this clause. The sub-DPB size of the sub-DPB for a layer with nuh_layer_id equal to currLayerId is max_vps_dec_pic_buffering_minus1[ TargetOlsIdx ][ layerIdx ][ HighestTid ] + 1, where layerIdx is equal to the value such that LayerSetLayerIdList[ TargetDecLayerSetIdx ][ layerIdx ] is equal to currLayerId.

If SubPicHrdFlag is equal to 0, the HRD operates at partition unit level and each decoding unit is a partition unit. Otherwise the HRD operates at sub-partition level and each decoding unit is a subset of a partition unit.

NOTE 4 – If the HRD operates at partition unit level, each time when some bits are removed from the CPB, a decoding unit that is an entire partition unit is removed from the CPB. Otherwise (the HRD operates at sub-partition level), each time when some bits are removed from the CPB, a decoding unit that is a subset of a partition unit is removed from the CPB. Regardless of whether the HRD operates at partition unit level or sub-partition level, each time when some picture is output from the DPB, an entire decoded picture is output from the DPB, though the picture output time is derived based on the differently derived CPB removal times and the differently signalled DPB output delays.

The following is specified for expressing the constraints in this annex:

–	Each access unit is referred to as access unit n, where the number n identifies the particular access unit. Access unit 0 is selected per step 6 above. The value of n is incremented by 1 for each subsequent access unit in decoding order.

–	Each decoding unit is referred to as decoding unit m, where the number m identifies the particular decoding unit. The first decoding unit in decoding order in access unit 0 is referred to as decoding unit 0. The value of m is incremented by 1 for each subsequent decoding unit in decoding order.

NOTE 5 – The numbering of decoding units is relative to the first decoding unit in access unit 0.

–	Picture n refers to a particular coded or decoded picture of access unit n.

The HRD operates as follows:

–	The HRD is initialized at decoding unit 0, with the CPB, each sub-DPB of the DPB, and each BPB being set to be empty (the sub-DPB fullness for each sub-DPB is set equal to 0).

NOTE 6 – After initialization, the HRD is not initialized again by subsequent buffering period SEI messages.

–	Data associated with decoding units that flow into the BPB according to a specified arrival schedule are delivered by an HBPS.

–	Each bitstream partition with index j is processed as specified in clause A.13.2 with the HSS replaced by the HBPS and with SchedSelIdx equal to bsp_sched_idx[ TargetOlsIdx ][ TargetPsIdx ][ HighestTid ][ SchedSelCombIdx ][ j ].

–	The data associated with each decoding unit are removed and decoded instantaneously by the instantaneous decoding process at the CPB removal time of the decoding unit.

–	Each decoded picture is placed in the DPB.

–	A decoded picture is removed from the DPB when it becomes no longer needed for inter prediction reference and no longer needed for output.

For each bitstream conformance test, the operation of the CPB and the BPB is specified in clause A.13.2, the instantaneous decoder operation is specified in clauses 2 through 10, clauses A.2 through A.10, and either clauses B.2 through B.10 or clauses A.2 through A.10, the operation of the DPB is specified in clause A.13.3, and the output cropping is specified in clauses A.13.3.3 and A.13.5.2.2.

HSS, HBPS, and HRD information concerning the number of enumerated delivery schedules and their associated bit rates and buffer sizes is specified in clauses E.2.2, E.3.2, A.7.3.2.1.6 and A.7.4.3.1.6. The HRD is initialized as specified by the buffering period SEI message specified in clauses D.2.2 and D.3.2. The removal timing of decoding units from the CPB and output timing of decoded pictures from the DPB is specified using information in picture timing SEI messages (specified in clauses D.2.3 and D.3.3) or in decoding unit information SEI messages (specified in clauses D.2.21 and D.3.21). All timing information relating to a specific decoding unit shall arrive prior to the CPB removal time of the decoding unit.

The requirements for bitstream conformance are specified in clause A.13.4, and the HRD is used to check conformance of bitstreams as specified above in this clause and to check conformance of decoders as specified in clause A.13.5.

    - Operation of bitstream partition buffer (BPB)

      - General

The specifications in this clause apply independently to each set of BPB parameters that is present and to both the Type I and Type II conformance points, and the set of BPB parameters is selected as specified in clause A.13.1. In clause A.13.2 and its subclauses, CPB is understood to be BPB, and access unit is understood to be partition unit.

      - Timing of decoding unit arrival

- The variable altParamSelectionFlag is derived as follows:

–	If all of the following conditions are true, altParamSelectionFlag is set equal to 1:

–	The current picture is a BLA picture that has nal_unit_type equal to BLA_W_LP and nuh_layer_id equal to SmallestLayerId or is a CRA picture that has nuh_layer_id equal to SmallestLayerId.

–	MultiLayerCpbOperationFlag is equal to 0.

–	Otherwise, if all of the following conditions are true, altParamSelectionFlag is set equal to 1:

–	The current picture is an IRAP picture with nuh_layer_id equal to SmallestLayerId and with NoClrasOutputFlag equal to 1.

–	MultiLayerCpbOperationFlag is equal to 1.

–	Otherwise, altParamSelectionFlag is set equal to 0.

- When altParamSelectionFlag is equal to 1, the following applies:

–	If some external means not specified in this Specification is available to set the variable UseAltCpbParamsFlag to a value, UseAltCpbParamsFlag is set equal to the value provided by the external means.

–	Otherwise, UseAltCpbParamsFlag is set equal to the value of use_alt_cpb_params_flag of the buffering period SEI message selected as specified in clause A.13.1.

- If SubPicHrdFlag is equal to 0, the variable subPicParamsFlag is set equal to 0, and the process specified in the remainder of this clause is invoked with a decoding unit being considered as an access unit, for derivation of the initial and final CPB arrival times for access unit n.

- Otherwise (SubPicHrdFlag is equal to 1), the process specified in the remainder of this clause is first invoked with the variable subPicParamsFlag set equal to 0 and a decoding unit being considered as an access unit, for derivation of the initial and final CPB arrival times for access unit n, and then invoked with subPicParamsFlag set equal to 1 and a decoding unit being considered as a subset of an access unit, for derivation of the initial and final CPB arrival times for the decoding units in access unit n.

The variables InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are derived as follows:

–	If one or more of the following conditions are true, InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are set equal to the values of the buffering period SEI message syntax elements nal_initial_alt_cpb_removal_delay[ SchedSelIdx ] and nal_initial_alt_cpb_removal_offset[ SchedSelIdx ], respectively, when NalHrdModeFlag is equal to 1, or vcl_initial_alt_cpb_removal_delay[ SchedSelIdx ] and vcl_initial_alt_cpb_removal_offset[ SchedSelIdx ], respectively, when NalHrdModeFlag is equal to 0, where the buffering period SEI message syntax elements are selected as specified in clause A.13.1:

–	Access unit 0 includes a BLA picture with nuh_layer_id equal to SmallestLayerId and nal_unit_type equal to BLA_W_RADL or BLA_N_LP, MultiLayerCpbOperationFlag is equal to 0 and the value of irap_cpb_params_present_flag of the buffering period SEI message is equal to 1.

–	Access unit 0 includes a BLA picture with nuh_layer_id equal to SmallestLayerId and nal_unit_type equal to BLA_W_LP or includes a CRA picture with nuh_layer_id equal to SmallestLayerId, MultiLayerCpbOperationFlag is equal to 0, and the value of irap_cpb_params_present_flag of the buffering period SEI message is equal to 1, and one or more of the following conditions are true:

–	UseAltCpbParamsFlag for access unit 0 is equal to 1.

–	DefaultInitCpbParamsFlag is equal to 0.

–	Access unit 0 includes an IRAP picture with nuh_layer_id equal to SmallestLayerId, MultiLayerCpbOperationFlag is equal to 1 and the value of irap_cpb_params_present_flag of the buffering period SEI message is equal to 1, and one or more of the following conditions are true:

–	UseAltCpbParamsFlag for access unit 0 is equal to 1.

–	DefaultInitCpbParamsFlag is equal to 0.

–	The value of subPicParamsFlag is equal to 1.

–	Otherwise, InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are set equal to the values of the buffering period SEI message syntax elements nal_initial_cpb_removal_delay[ SchedSelIdx ] and nal_initial_cpb_removal_offset[ SchedSelIdx ], respectively, when NalHrdModeFlag is equal to 1, or vcl_initial_cpb_removal_delay[ SchedSelIdx ] and vcl_initial_cpb_removal_offset[ SchedSelIdx ], respectively, when NalHrdModeFlag is equal to 0, where the buffering period SEI message syntax elements are selected as specified in clause A.13.1.

The time at which the first bit of decoding unit m begins to enter the CPB is referred to as the initial arrival time initArrivalTime[ m ].

Decoding units are indexed in decoding order within the bitstream.

The initial arrival time of decoding unit m is derived as follows:

–	If the decoding unit is decoding unit 0 (i.e., when m is equal to 0) and the decoding unit belongs to the base bitstream partition, initArrivalTime[ 0 ] is set equal to 0.

–	Otherwise, if the decoding unit is decoding unit 0 and the decoding unit does not belong to the base bitstream partition, initArrivalTime[ 0 ] is obtained from the bitstream partition initial arrival time SEI message that applies to TargetOp, TargetPartitioningScheme, and TargetBitstreamPartition and is associated with the access unit containing decoding unit 0. The applicable bitstream partition initial arrival time SEI message is either included in a bitstream partition nesting SEI message in an SEI NAL unit in the access unit containing decoding unit 0 in BitstreamToDecode or available through external means not specified in this Specification. If NalHrdModeFlag is equal to 0, initArrivalTime[ 0 ] is set equal to vcl_initial_arrival_delay[ SchedSelIdx ] in the applicable bitstream partition initial arrival time SEI message. Otherwise (NalHrdModeFlag is equal to 1), initArrivalTime[ 0 ] is set equal to nal_initial_arrival_delay[ SchedSelIdx ] in the applicable bitstream partition initial arrival time SEI message.

–	Otherwise, the following applies:

–	If cbr_flag[ SchedSelIdx ] is equal to 1, the initial arrival time for decoding unit m is equal to the final arrival time (which is derived below) of decoding unit m − 1, i.e.,

if( !subPicParamsFlag )
	initArrivalTime[ m ] = AuFinalArrivalTime[ m − 1 ]	(F-74)
else
	initArrivalTime[ m ] = DuFinalArrivalTime[ m − 1 ]

–	Otherwise (cbr_flag[ SchedSelIdx ] is equal to 0), the initial arrival time for decoding unit m is derived as follows:

if( !subPicParamsFlag )
	initArrivalTime[ m ] = Max( AuFinalArrivalTime[ m − 1 ], initArrivalEarliestTime[ m ] )	(F-75)
else
	initArrivalTime[ m ] = Max( DuFinalArrivalTime[ m − 1 ], initArrivalEarliestTime[ m ] )

where initArrivalEarliestTime[ m ] is derived as follows:

–	The variable tmpNominalRemovalTime is derived as follows:

if( !subPicParamsFlag )
	tmpNominalRemovalTime = AuNominalRemovalTime[ m ] 	(F-76)
else
	tmpNominalRemovalTime = DuNominalRemovalTime[ m ]

where AuNominalRemovalTime[ m ] and DuNominalRemovalTime[ m ] are the nominal CPB removal time of access unit m and decoding unit m, respectively, as specified in clause A.13.2.3.

–	If decoding unit m is not the first decoding unit of a subsequent buffering period, initArrivalEarliestTime[ m ] is derived as follows:

initArrivalEarliestTime[ m ] = tmpNominalRemovalTime − ( InitCpbRemovalDelay[ SchedSelIdx ]
		+ InitCpbRemovalDelayOffset[ SchedSelIdx ] )  90000	(F-77)

–	Otherwise (decoding unit m is the first decoding unit of a subsequent buffering period), initArrivalEarliestTime[ m ] is derived as follows:

initArrivalEarliestTime[ m ] = tmpNominalRemovalTime −
		( InitCpbRemovalDelay[ SchedSelIdx ]  90000 )	(F-78)

The final arrival time for decoding unit m is derived as follows:

if( !subPicParamsFlag )
		AuFinalArrivalTime[ m ] = initArrivalTime[ m ] + sizeInbits[ m ]  BitRate[ SchedSelIdx ]	(F-79)
else
		DuFinalArrivalTime[ m ] = initArrivalTime[ m ] + sizeInbits[ m ]  BitRate[ SchedSelIdx ]

- where sizeInbits[ m ] is the size in bits of decoding unit m, counting the bits of the VCL NAL units and the filler data NAL units for the Type I conformance point or all bits of the Type II bitstream for the Type II conformance point.

The values of SchedSelIdx, BitRate[ SchedSelIdx ], and CpbSize[ SchedSelIdx ] are constrained as follows:

–	If the lists of delivery schedules, including the HRD parameters associated with the individual delivery schedules, for the access unit containing decoding unit m and the previous access unit differ, the HSS selects a value SchedCombIdx1 in the range of 0 to num_bsp_schedules_minus1[ TargetOlsIdx ][ TargetPsIdx ][ HighestTid ], inclusive, and sets SchedSelIdx1 for TargetBitstreamPartition to be equal to bsp_sched_idx[ TargetOlsIdx ][ TargetPsIdx ][ HighestTid ][ SchedSelCombIdx ][ j ], where j is the index of the bitstream partition index of TargetBitstreamPartition, for the access unit containing decoding unit m that results in a BitRate[ SchedSelIdx1 ] or CpbSize[ SchedSelIdx1 ] for the access unit containing decoding unit m. The value of BitRate[ SchedSelIdx1 ] or CpbSize[ SchedSelIdx1 ] may differ from the value of BitRate[ SchedSelIdx0 ] or CpbSize[ SchedSelIdx0 ] for the value SchedSelIdx0 of SchedSelIdx that was in use for the previous access unit.

–	Otherwise, the HSS continues to operate with the previous values of SchedSelIdx, BitRate[ SchedSelIdx ], and CpbSize[ SchedSelIdx ].

When the HSS selects values of BitRate[ SchedSelIdx ] or CpbSize[ SchedSelIdx ] that differ from those of the previous access unit, the following applies:

–	The variable BitRate[ SchedSelIdx ] comes into effect at the initial CPB arrival time of the current access unit.

–	The variable CpbSize[ SchedSelIdx ] comes into effect as follows:

–	If the new value of CpbSize[ SchedSelIdx ] is greater than the old CPB size, it comes into effect at the initial CPB arrival time of the current access unit.

–	Otherwise, the new value of CpbSize[ SchedSelIdx ] comes into effect at the CPB removal time of the current access unit.

      - Timing of decoding unit removal and decoding of decoding unit

The variables InitCpbRemovalDelay[ SchedSelIdx ], InitCpbRemovalDelayOffset[ SchedSelIdx ], CpbDelayOffset, and DpbDelayOffset are derived as follows:

–	If one or more of the following conditions are true, CpbDelayOffset is set equal to the value of the buffering period SEI message syntax element cpb_delay_offset, DpbDelayOffset is set equal to the value of the buffering period SEI message syntax element dpb_delay_offset, and InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are set equal to the values of the buffering period SEI message syntax elements nal_initial_alt_cpb_removal_delay[ SchedSelIdx ] and nal_initial_alt_cpb_removal_offset[ SchedSelIdx ], respectively, when NalHrdModeFlag is equal to 1, or vcl_initial_alt_cpb_removal_delay[ SchedSelIdx ] and vcl_initial_alt_cpb_removal_offset[ SchedSelIdx ], respectively, when NalHrdModeFlag is equal to 0, where the buffering period SEI message containing the syntax elements is selected as specified in clause A.13.1:

–	Access unit 0 includes a BLA picture with nuh_layer_id equal to SmallestLayerId and nal_unit_type equal to BLA_W_RADL or BLA_N_LP, MultiLayerCpbOperationFlag is equal to 0 and the value of irap_cpb_params_present_flag of the buffering period SEI message is equal to 1.

–	Access unit 0 includes a BLA picture with nuh_layer_id equal to SmallestLayerId and nal_unit_type equal to BLA_W_LP or includes a CRA picture with nuh_layer_id equal to SmallestLayerId, MultiLayerCpbOperationFlag is equal to 0 and the value of irap_cpb_params_present_flag of the buffering period SEI message is equal to 1, and one or more of the following conditions are true:

–	UseAltCpbParamsFlag for access unit 0 is equal to 1.

–	DefaultInitCpbParamsFlag is equal to 0.

–	Access unit 0 includes an IRAP picture with nuh_layer_id equal to SmallestLayerId, MultiLayerCpbOperationFlag is equal to 1 and the value of irap_cpb_params_present_flag of the buffering period SEI message is equal to 1, and one or more of the following conditions are true:

–	UseAltCpbParamsFlag for access unit 0 is equal to 1.

–	DefaultInitCpbParamsFlag is equal to 0.

–	Otherwise, InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are set equal to the values of the buffering period SEI message syntax elements nal_initial_cpb_removal_delay[ SchedSelIdx ] and nal_initial_cpb_removal_offset[ SchedSelIdx ], respectively, when NalHrdModeFlag is equal to 1, or vcl_initial_cpb_removal_delay[ SchedSelIdx ] and vcl_initial_cpb_removal_offset[ SchedSelIdx ], respectively, when NalHrdModeFlag is equal to 0, where the buffering period SEI message containing the syntax elements is selected as specified in clause A.13.1, CpbDelayOffset and DpbDelayOffset are both set equal to 0.

The nominal removal time of the access unit n from the CPB is specified as follows:

–	If access unit n is the access unit with n equal to 0 (the access unit that initializes the HRD), the nominal removal time of the access unit from the CPB is specified by:

AuNominalRemovalTime[ 0 ] = InitCpbRemovalDelay[ SchedSelIdx ]  90000	(F-80)

–	Otherwise, the following applies:

–	When access unit n is the first access unit of a buffering period that does not initialize the HRD, the following applies:

The nominal removal time of the access unit n from the CPB is specified by:

if( !concatenationFlag ) {
	baseTime = AuNominalRemovalTime[ firstPicInPrevBuffPeriod ]
	tmpCpbRemovalDelay = AuCpbRemovalDelayVal
} else {
	baseTime = AuNominalRemovalTime[ prevNonDiscardablePic ]
	tmpCpbRemovalDelay =
		Max( ( auCpbRemovalDelayDeltaMinus1 + 1 ),	(F-81)
			Ceil( ( InitCpbRemovalDelay[ SchedSelIdx ]  90000 +
				AuFinalArrivalTime[ n − 1 ] − AuNominalRemovalTime[ n − 1 ] )  ClockTick ) )
}
AuNominalRemovalTime[ n ] = baseTime + ClockTick * ( tmpCpbRemovalDelay − CpbDelayOffset )

where AuNominalRemovalTime[ firstPicInPrevBuffPeriod ] is the nominal removal time of the first access unit of the previous buffering period, AuNominalRemovalTime[ prevNonDiscardablePic ] is the nominal removal time of the preceding access unit in decoding order, each picture of which is with TemporalId equal to 0 that is not a RASL, RADL or sub-layer non-reference picture, AuCpbRemovalDelayVal is the value of AuCpbRemovalDelayVal derived according to au_cpb_removal_delay_minus1 in the picture timing SEI message, selected as specified in clause A.13.1, associated with access unit n, and concatenationFlag and auCpbRemovalDelayDeltaMinus1 are the values of the syntax elements concatenation_flag and au_cpb_removal_delay_delta_minus1, respectively, in the buffering period SEI message, selected as specified in clause A.13.1, associated with access unit n.

After the derivation of the nominal CPB removal time and before the derivation of the DPB output time of access unit n, the values of CpbDelayOffset and DpbDelayOffset are updated as follows:

–	If one or more of the following conditions are true, CpbDelayOffset is set equal to the value of the buffering period SEI message syntax element cpb_delay_offset, and DpbDelayOffset is set equal to the value of the buffering period SEI message syntax element dpb_delay_offset, where the buffering period SEI message containing the syntax elements is selected as specified in clause A.13.1:

–	Access unit n includes a BLA picture with nuh_layer_id equal to SmallestLayerId and nal_unit_type equal to BLA_W_RADL or BLA_N_LP, MultiLayerCpbOperationFlag is equal to 0 and the value of irap_cpb_params_present_flag of the buffering period SEI message is equal to 1.

–	Access unit n includes a BLA picture with nuh_layer_id equal to SmallestLayerId and nal_unit_type equal to BLA_W_LP or includes a CRA picture with nuh_layer_id equal to SmallestLayerId, MultiLayerCpbOperationFlag is equal to 0, the value of irap_cpb_params_present_flag of the buffering period SEI message is equal to 1, and UseAltCpbParamsFlag for access unit n is equal to 1.

–	Access unit n includes an IRAP picture with nuh_layer_id equal to SmallestLayerId, MultiLayerCpbOperationFlag is equal to 1, the value of irap_cpb_params_present_flag of the buffering period SEI message is equal to 1, and UseAltCpbParamsFlag for access unit n is equal to 1.

–	Otherwise, CpbDelayOffset and DpbDelayOffset are both set equal to 0.

–	When access unit n is not the first access unit of a buffering period, the nominal removal time of the access unit n from the CPB is specified by:

AuNominalRemovalTime[ n ] = AuNominalRemovalTime[ firstPicInCurrBuffPeriod ] +
		ClockTick * ( AuCpbRemovalDelayVal − CpbDelayOffset )	(F-82)

where AuNominalRemovalTime[ firstPicInCurrBuffPeriod ] is the nominal removal time of the first access unit of the current buffering period, and AuCpbRemovalDelayVal is the value of AuCpbRemovalDelayVal derived according to au_cpb_removal_delay_minus1 in the picture timing SEI message, selected as specified in clause A.13.1, associated with access unit n.

When SubPicHrdFlag is equal to 1, the following applies:

–	The variable duCpbRemovalDelayInc is derived as follows:

–	If sub_pic_cpb_params_in_pic_timing_sei_flag is equal to 0, duCpbRemovalDelayInc is set equal to the value of du_spt_cpb_removal_delay_increment in the decoding unit information SEI message, selected as specified in clause A.13.1, associated with decoding unit m.

–	Otherwise, if du_common_cpb_removal_delay_flag is equal to 0, duCpbRemovalDelayInc is set equal to the value of du_cpb_removal_delay_increment_minus1[ i ] + 1 for decoding unit m in the picture timing SEI message, selected as specified in clause A.13.1, associated with access unit n, where the value of i is 0 for the first num_nalus_in_du_minus1[ 0 ] + 1 consecutive NAL units in the access unit that contains decoding unit m, 1 for the subsequent num_nalus_in_du_minus1[ 1 ] + 1 NAL units in the same access unit, 2 for the subsequent num_nalus_in_du_minus1[ 2 ] + 1 NAL units in the same access unit, etc.

–	Otherwise, duCpbRemovalDelayInc is set equal to the value of du_common_cpb_removal_delay_increment_minus1 + 1 in the picture timing SEI message, selected as specified in clause A.13.1, associated with access unit n.

–	The nominal removal time of decoding unit m from the CPB is specified as follows, where AuNominalRemovalTime[ n ] is the nominal removal time of access unit n:

–	If decoding unit m is the last decoding unit in access unit n, the nominal removal time of decoding unit m DuNominalRemovalTime[ m ] is set equal to AuNominalRemovalTime[ n ].

–	Otherwise (decoding unit m is not the last decoding unit in access unit n), the nominal removal time of decoding unit m DuNominalRemovalTime[ m ] is derived as follows:

if( sub_pic_cpb_params_in_pic_timing_sei_flag )
	DuNominalRemovalTime[ m ] = DuNominalRemovalTime[ m + 1 ] −
		ClockSubTick * duCpbRemovalDelayInc	(F-83)
else
	DuNominalRemovalTime[ m ] = AuNominalRemovalTime[ n ] −
		ClockSubTick * duCpbRemovalDelayInc

If SubPicHrdFlag is equal to 0, the removal time of access unit n from the CPB is specified as follows, where AuFinalArrivalTime[ n ] and AuNominalRemovalTime[ n ] are the final CPB arrival time and nominal CPB removal time, respectively, of access unit n:

if( !low_delay_hrd_flag[ HighestTid ]  | |  AuNominalRemovalTime[ n ]  >=  AuFinalArrivalTime[ n ] )
	AuCpbRemovalTime[ n ] = AuNominalRemovalTime[ n ]
else							(F-84)
	AuCpbRemovalTime[ n ] = AuNominalRemovalTime[ n ] + ClockTick *
		Ceil( ( AuFinalArrivalTime[ n ] − AuNominalRemovalTime[ n ] )  ClockTick )

NOTE 1 – When low_delay_hrd_flag[ HighestTid ] is equal to 1 and AuNominalRemovalTime[ n ] is less than AuFinalArrivalTime[ n ], the size of access unit n is so large that it prevents removal at the nominal removal time.

Otherwise (SubPicHrdFlag is equal to 1), the removal time of decoding unit m from the CPB is specified as follows:

–	The following applies:

if( !low_delay_hrd_flag[ HighestTid ]  | |  DuNominalRemovalTime[ m ]  >=  DuFinalArrivalTime[ m ] )
	DuCpbRemovalTime[ m ] = DuNominalRemovalTime[ m ]	(F-85)
else			-
	DuCpbRemovalTime[ m ] = DuFinalArrivalTime[ m ]

NOTE 2 – When low_delay_hrd_flag[ HighestTid ] is equal to 1 and DuNominalRemovalTime[ m ] is less than DuFinalArrivalTime[ m ], the size of decoding unit m is so large that it prevents removal at the nominal removal time.

–	When cbr_flag[ SchedSelIdx ] is equal to 0, the following applies:

–	Let refDuCpbRemovalTime be equal to the CPB removal time of the previous DU preceding the current DU in decoding order (regardless of the bitstream partitions to which the previous DU and the current DU belong).

–	The variable DuCpbRemovalTime[ m ] is modified as follows:

DuCpbRemovalTime[ m ] = Max( DuCpbRemovalTime[ m ], refDuCpbRemovalTime )	(F-87)

If SubPicHrdFlag is equal to 0, at the CPB removal time of access unit n, the access unit is instantaneously decoded.

Otherwise (SubPicHrdFlag is equal to 1), at the CPB removal time of decoding unit m, the decoding unit is instantaneously decoded, and when decoding unit m is the last decoding unit of access unit n, the following applies:

–	Access unit n is considered as decoded.

–	The final CPB arrival time of access unit n, i.e., AuFinalArrivalTime[ n ], is set equal to the final CPB arrival time of the last decoding unit in access unit n, i.e., DuFinalArrivalTime[ m ].

–	The nominal CPB removal time of access unit n, i.e., AuNominalRemovalTime[ n ], is set equal to the nominal CPB removal time of the last decoding unit in access unit n, i.e., DuNominalRemovalTime[ m ].

–	The CPB removal time of access unit n, i.e., AuCpbRemovalTime[ m ], is set equal to the CPB removal time of the last decoding unit in access unit n, i.e., DuCpbRemovalTime[ m ].

    - Operation of decoded picture buffer (DPB)

      - General

The specifications in this clause apply independently to each set of DPB parameters selected as specified in clause A.13.1.

The decoded picture buffer consists of sub-DPBs, and each sub-DPB contains picture storage buffers for storage of decoded pictures of one layer. Each of the picture storage buffers of a sub-DPB may contain a decoded picture that is marked as "used for reference" or is held for future output.

The processes specified in clauses A.13.3.2, A.13.3.3, and A.13.3.4 are sequentially applied as specified below, and are applied independently for each layer, starting from the base layer, in increasing order of nuh_layer_id values of the layers in the bitstream. When these processes are applied for a particular layer, only the sub-DPB for the particular layer is affected. In the descriptions of these processes, the DPB refers to the sub-DPB for the particular layer, and the particular layer is referred to as the current layer.

NOTE – In the operation of output timing DPB, decoded pictures with PicOutputFlag equal to 1 in the same access unit are output consecutively in ascending order of the nuh_layer_id values of the decoded pictures.

Let picture n and the current picture be the coded picture or decoded picture of the access unit n for a particular value of nuh_layer_id, wherein n is a non-negative integer number.

      - Removal of pictures from the DPB

When the current picture is not picture 0 in the current layer, the removal of pictures in the current layer, with nuh_layer_id equal to currLayerId, from the DPB before decoding of the current picture, i.e., picture n, but after parsing the slice header of the first slice of the current picture, happens instantaneously at the CPB removal time of the first decoding unit of the current picture and proceeds as follows:

–	The decoding process for RPS as specified in clause A.8.3.2 is invoked.

–	The variable listOfSubDpbsToEmpty is derived as follows:

–	If a new VPS is activated by the current access unit or the current picture is an IRAP picture with nuh_layer_id equal to SmallestLayerId, NoRaslOutputFlag equal to 1, and NoClrasOutputFlag equal to 1, listOfSubDpbsToEmpty is set to include all the sub-DPBs.

–	Otherwise, if the current picture is an IRAP picture with any nuh_layer_id value indepLayerId such that NumDirectRefLayers[ indepLayerId ] is equal to 0 and indepLayerId is greater than SmallestLayerId, and with NoRaslOutputFlag equal to 1, and LayerResetFlag is equal to 1, listOfSubDpbsToEmpty is set equal to the sub-DPBs containing the current layer and the sub-DPBs containing the predicted layers of the current layer.

–	Otherwise, listOfSubDpbsToEmpty is set equal to the sub-DPB containing the current layer.

–	When the current picture is an IRAP picture with NoRaslOutputFlag equal to 1 and any of the following conditions is true:

–	nuh_layer_id equal to SmallestLayerId,

–	nuh_layer_id of the current layer is greater than SmallestLayerId, and NumDirectRefLayers[ nuh_layer_id ] is equal to 0,

the following ordered steps are applied:

1.	The variable NoOutputOfPriorPicsFlag is derived for the decoder under test as follows:

–	If the current picture is a CRA picture, NoOutputOfPriorPicsFlag is set equal to 1 (regardless of the value of no_output_of_prior_pics_flag).

–	Otherwise, if the value of pic_width_in_luma_samples, pic_height_in_luma_samples, chroma_format_idc, bit_depth_luma_minus8, bit_depth_chroma_minus8, separate_colour_plane_flag, or sps_max_dec_pic_buffering_minus1[ HighestTid ] derived from the active SPS for the current layer is different from the value of pic_width_in_luma_samples, pic_height_in_luma_samples, chroma_format_idc, bit_depth_luma_minus8, bit_depth_chroma_minus8, separate_colour_plane_flag, or sps_max_dec_pic_buffering_minus1[ HighestTid ], respectively, derived from the SPS that was active for the current layer when decoding the preceding picture in the current layer, NoOutputOfPriorPicsFlag may (but should not) be set equal to 1 by the decoder under test, regardless of the value of no_output_of_prior_pics_flag.

NOTE – Although setting NoOutputOfPriorPicsFlag equal to no_output_of_prior_pics_flag is preferred under these conditions, the decoder under test is allowed to set NoOutputOfPriorPicsFlag to 1 in this case.

–	Otherwise, NoOutputOfPriorPicsFlag is set equal to no_output_of_prior_pics_flag.

2.	When the value of NoOutputOfPriorPicsFlag derived for the decoder under test is equal to 1, all non-empty picture storage buffers in all the sub-DPBs contained in listOfSubDpbsToEmpty are emptied without output of the pictures they contain, and the sub-DPB fullness of each sub-DPB in listOfSubDpbsToEmpty is set equal to 0.

–	When both of the following conditions are true for any pictures k in the DPB, all such pictures k in the DPB are removed from the DPB:

–	picture k is marked as "unused for reference"

–	picture k has PicOutputFlag equal to 0 or its DPB output time is less than or equal to the CPB removal time of the first decoding unit (denoted as decoding unit m) of the current picture n; i.e., DpbOutputTime[ k ] is less than or equal to CpbRemovalTime( m )

–	For each picture that is removed from the DPB, the DPB fullness is decremented by one.

      - Picture output

The processes specified in this clause happen instantaneously at the CPB removal time of access unit n, AuCpbRemovalTime[ n ].

When access unit n has AuOutputFlag equal to 1, its DPB output time DpbOutputTime[ n ] is derived as follows, where the variable firstPicInBufferingPeriodFlag is equal to 1 if access unit n is the first access unit of a buffering period and 0 otherwise:

if( !SubPicHrdFlag ) {
	DpbOutputTime[ n ] = AuCpbRemovalTime[ n ] + ClockTick * picDpbOutputDelay	(F-88)
	if( firstPicInBufferingPeriodFlag )
		DpbOutputTime[ n ]  −=  ClockTick * DpbDelayOffset
} else
	DpbOutputTime[ n ] = AuCpbRemovalTime[ n ] + ClockSubTick * picSptDpbOutputDuDelay	-

where picDpbOutputDelay is the value of pic_dpb_output_delay in the picture timing SEI message associated with access unit n, and picSptDpbOutputDuDelay is the value of pic_spt_dpb_output_du_delay, when present, in the decoding unit information SEI messages associated with access unit n, or the value of pic_dpb_output_du_delay in the picture timing SEI message associated with access unit n when there is no decoding unit information SEI message associated with access unit n or no decoding unit information SEI message associated with access unit n has pic_spt_dpb_output_du_delay present.

NOTE – When the syntax element pic_spt_dpb_output_du_delay is not present in any decoding unit information SEI message associated with access unit n, the value is inferred to be equal to pic_dpb_output_du_delay in the picture timing SEI message associated with access unit n.

The output of the current picture contained in access unit n is specified as follows:

–	If PicOutputFlag is equal to 1 and DpbOutputTime[ n ] is equal to AuCpbRemovalTime[ n ], the current picture is output.

–	Otherwise, if PicOutputFlag is equal to 0, the current picture is not output, but will be stored in the DPB as specified in clause A.13.3.4.

–	Otherwise (PicOutputFlag is equal to 1 and DpbOutputTime[ n ] is greater than AuCpbRemovalTime[ n ] ), the current picture is output later and will be stored in the DPB (as specified in clause A.13.3.4) and is output at time DpbOutputTime[ n ] unless indicated not to be output by the decoding or inference of NoOutputOfPriorPicsFlag equal to 1 at a time that precedes DpbOutputTime[ n ].

When output, a picture is cropped, using the conformance cropping window specified in the active SPS for the layer containing the picture.

When access unit n is an access unit that has AuOutputFlag equal to 1 and is not the last access unit of the bitstream that has AuOutputFlag equal to 1, the value of the variable DpbOutputInterval[ n ] is derived as follows:

DpbOutputInterval[ n ] = DpbOutputTime[ nextAuInOutputOrder ] − DpbOutputTime[ n ]	(F-90)

where nextAuInOutputOrder is the access unit that follows access unit n in output order and has AuOutputFlag equal to 1.

      - Current decoded picture marking and storage

The process specified in this clause happens instantaneously at the CPB removal time of the last decoding unit of the current picture.

The current decoded picture is stored in the DPB in an empty picture storage buffer, the DPB fullness is incremented by one, and the current picture is marked as "used for short-term reference".

NOTE – Decoders can start to store decoded parts of the current picture into the DPB before the decoding of the current picture is completed.

    - Bitstream conformance

A bitstream of coded data conforming to this Specification shall fulfil all requirements specified in this clause.

The bitstream shall be constructed according to the syntax, semantics, and constraints specified in this Specification outside of this annex.

The first access unit in a bitstream shall be an IRAP access unit.

When vps_base_layer_internal_flag is equal to 0, all the following bitstream conformance constraints apply only to coded pictures present in the bitstream, and do not apply to pictures with nuh_layer_id equal to 0 which are provided by external means.

Let currPicLayerId be equal to the nuh_layer_id of the current picture.

For each current picture, let the variables maxPicOrderCnt and minPicOrderCnt be set equal to the maximum and the minimum, respectively, of the PicOrderCntVal values of the following pictures with nuh_layer_id equal to currPicLayerId:

–	The current picture.

–	The previous picture in decoding order that has TemporalId equal to 0 and that is not a RASL picture, a RADL picture, or a sub-layer non-reference picture.

–	The short-term reference pictures in the RPS of the current picture.

–	All pictures m that have PicOutputFlag equal to 1, AuCpbRemovalTime[ m ] less than AuCpbRemovalTime[ currAu ], and DpbOutputTime[ m ] greater than or equal to AuCpbRemovalTime[ currAu ], where currAu is the access unit containing the current picture, and AuCpbRemovalTime[ m ] and DpbOutputTime[ m ] are the CPB removal time and the DPB output time, respectively, of the access unit containing picture m.

The bitstream and the sub-bitstream of each output layer set are tested by the HRD for conformance as specified in clause C.1 and clause A.13.1. All the conditions specified in clause C.4 shall be fulfilled for each of the first and second sets of conformance tests. All of the following conditions shall be fulfilled for each of the third set of conformance tests:

- For each partition unit n, with n greater than 0, associated with a buffering period SEI message, let the variable deltaTime90k[ n ] be specified as follows:

deltaTime90k[ n ] = 90000 * ( AuNominalRemovalTime[ n ] − AuFinalArrivalTime[ n − 1 ] )	(F-91)

The value of InitCpbRemovalDelay[ SchedSelIdx ] is constrained as follows:

–	If cbr_flag[ SchedSelIdx ] is equal to 0, the following condition shall be true:

InitCpbRemovalDelay[ SchedSelIdx ]  <=  Ceil( deltaTime90k[ n ] )	(F-92)

–	Otherwise (cbr_flag[ SchedSelIdx ] is equal to 1), the following condition shall be true:

Floor( deltaTime90k[ n ] )  <=  	InitCpbRemovalDelay[ SchedSelIdx ]
					<=  Ceil( deltaTime90k[ n ] )
			(F-93)

- NOTE 1 – The exact number of bits in the BPB at the removal time of each partition unit may depend on which buffering period SEI message is selected to initialize the HRD. Encoders must take this into account to ensure that all specified constraints must be obeyed regardless of which buffering period SEI message is selected to initialize the HRD, as the HRD may be initialized at any one of the buffering period SEI messages.

- A BPB overflow is specified as the condition in which the total number of bits in the BPB is greater than the BPB size. The BPB shall never overflow.

- A BPB underflow is specified as the condition in which the nominal BPB removal time of decoding unit m DuNominalRemovalTime( m ) is less than the final BPB arrival time of decoding unit m DuFinalArrivalTime( m ) for at least one value of m. When low_delay_hrd_flag[ HighestTid ] is equal to 0, the BPB shall never underflow.

- When SubPicHrdFlag is equal to 1, low_delay_hrd_flag[ HighestTid ] is equal to 1, and the nominal removal time of a decoding unit m of partition unit n is less than the final BPB arrival time of decoding unit m (i.e., DuNominalRemovalTime[ m ] < DuFinalArrivalTime[ m ]), the nominal removal time of partition unit n shall be less than the final BPB arrival time of partition unit n (i.e., AuNominalRemovalTime[ n ] < AuFinalArrivalTime[ n ]).

- The nominal removal times of partition units from the BPB (starting from the second partition unit in decoding order) shall satisfy the constraints on AuNominalRemovalTime[ n ] and AuCpbRemovalTime[ n ] expressed in clauses A.4.1 through A.4.2 for bitstreams or layers conforming to profiles defined in Annex A, or expressed in clauses B.11.2.1 through B.11.2.2 for bitstreams or layers conforming to profiles defined in Annex B, or expressed in clauses A.11.2.1 through A.11.2.2 for bitstreams or layers conforming to profiles defined in Annex A.

- For each current picture, after invocation of the process for removal of pictures from the sub-DPB as specified in clause A.13.3.2, the number of decoded pictures in the sub-DPB for the current layer, including all pictures n, where each picture n is contained in partition unit kn, in the current layer that are marked as "used for reference", or that have PicOutputFlag equal to 1 and AuCpbRemovalTime[ kn ] less than AuCpbRemovalTime[ currAu ], where currAu is the partition unit containing the current picture, shall be less than or equal to max_vps_dec_pic_buffering_minus1[ TargetOlsIdx ][ layerIdx ][ HighestTid ], where layerIdx is equal to the value such that LayerSetLayerIdList[ TargetDecLayerSetIdx ][ layerIdx ] is equal to currPicLayerId.

- All reference pictures shall be present in the DPB when needed for prediction. Each picture that has PicOutputFlag equal to 1 shall be present in the DPB at its DPB output time unless it is removed from the DPB before its output time by one of the processes specified in clause A.13.3.

- For each current picture that is not an IRAP picture with NoRaslOutputFlag equal to 1 or that is not the first picture of the current layer with nuh_layer_id greater than 0 that follows an IRAP picture that has nuh_layer_id equal to 0 and has NoClrasOutputFlag equal to 1, the value of maxPicOrderCnt − minPicOrderCnt shall be less than MaxPicOrderCntLsb / 2.

- The value of DpbOutputInterval[ n ] as given by Equation F-90, which is the difference between the output time of antpartition unit that has AuOutputFlag equal to 1 and that of the firsttpartition unit following it in output order and having AuOutputFlag equal to 1, shall satisfy the constraint on DpbOutputInterval[ n ] expressed in clause A.4.2 for the profile, tier, and level of the bitstream or layer using the decoding process specified in clauses 2 through 10, or expressed in clause 2 for the profile, tier, and level of the bitstream or layer using the decoding process specified in clauses 2 through 0 and either clauses Error: Reference source not found through B.10 or clauses A.2 through 0]

- For each current picture, when sub_pic_cpb_params_in_pic_timing_sei_flag is equal to 1, let tmpCpbRemovalDelaySum be derived as follows:

tmpCpbRemovalDelaySum = 0
for( i = 0; i < num_decoding_units_minus1; i++ )	(F-94)
	tmpCpbRemovalDelaySum  +=  du_cpb_removal_delay_increment_minus1[ i ] + 1

The value of ClockSubTick * tmpCpbRemovalDelaySum shall be equal to the difference between the nominal BPB removal time of the current partition unit and the nominal BPB removal time of the first decoding unit in the current partition unit in decoding order.

- Let picA be an IRAP picture with NoRaslOutputFlag equal to 1 and belonging to a layer layerA. Let auB be the earlier, in decoding order, of the first partition unit containing an IRAP picture with nuh_layer_id equal to 0 and NoClrasOutputFlag equal to 1 that succeeds picA in decoding order and the first partition unit containing an IRAP picture with NoRaslOutputFlag equal to 1 in layerA that succeeds picA in decoding order. For any two pictures picM and picN in the layer layerA contained in partition units m and n, respectively, that either are picA or succeed picA in decoding order and precede auB in decoding order, when DpbOutputTime[ m ] is greater than DpbOutputTime[ n ], PicOrderCnt( picM ) shall be greater than PicOrderCnt( picN ), where PicOrderCnt( picM ) and PicOrderCnt( picN ) are the PicOrderCntVal values of picM and picN, respectively, immediately after the invocation of the decoding process for picture order count of the latter of picM and picN in decoding order.

- NOTE 2 – All pictures of an earlier CVS in decoding order that are output are output before any pictures of a later CVS in decoding order. Within any particular CVS where all pictures have poc_reset_id not present or equal to 0, the pictures that are output are output in increasing PicOrderCntVal order. For any particular CVS where some pictures have poc_reset_id greater than 0, within each POC resetting period, the pictures that are output are output in increasing PicOrderCntVal order, and for any two pictures that are output and are in different POC resetting periods, the one that is earlier in decoding order is output earlier.

- For any decoding unit m, DuCpbRemovalTime[ m ] shall be greater than or equal to the BPB removal time of the previous DU that precedes the current DU in decoding order and that is in the same bitstream partition as the decoding unit m or includes one or more VCL NAL units of a picture that may be used as a direct or indirect reference picture for the picture containing the one or more VCL NAL units included in the decoding unit m.

- The DPB output times derived for all pictures in any particular access unit shall be the same.

    - Decoder conformance

      - General

A decoder conforming to this Specification shall fulfil all requirements specified in this clause.

A decoder claiming conformance to a specific profile, tier and level shall be able to successfully decode all bitstreams that conform to the bitstream conformance requirements specified in clause A.13.4, in the manner specified in Annexes A, B, and A for profile, tier, and level specified in Annexes A, B, and A, respectively, provided that all VPSs, SPSs and PPSs referred to by the VCL NAL units, appropriate buffering period, picture timing, and decoding unit information SEI messages are conveyed to the decoder, in a timely manner, either in the bitstream (by non-VCL NAL units), or by external means not specified in this Specification, and, when vps_base_layer_internal_flag is equal to 0, the decoded pictures with nuh_layer_id equal to 0 and their properties as specified in clause A.8.1 and its subclauses are conveyed to the decoder in a timely manner by external means not specified in this Specification.

When a bitstream contains syntax elements that have values that are specified as reserved and it is specified that decoders shall ignore values of the syntax elements or NAL units containing the syntax elements having the reserved values, and the bitstream is otherwise conforming to this Specification, a conforming decoder shall decode the bitstream in the same manner as it would decode a conforming bitstream and shall ignore the syntax elements or the NAL units containing the syntax elements having the reserved values as specified.

There are two types of conformance that can be claimed by a decoder: output timing conformance and output order conformance.

To check conformance of a decoder, test bitstreams containing one or more bitstream partitions conforming to the claimed profile, tier and level, as specified in clause A.13.4 are delivered by a hypothetical stream scheduler (HSS) both to the HRD and to the decoder under test (DUT). When vps_base_layer_internal_flag is equal to 0, decoded pictures with nuh_layer_id equal to 0 and their properties as specified in clause A.8.1 and its subclauses are also conveyed both to the HRD and to the DUT in a timely manner by external means not specified in this Specification. All cropped decoded pictures output by the HRD shall also be output by the DUT, each cropped decoded picture output by the DUT shall be a picture with PicOutputFlag equal to 1, and, for each such cropped decoded picture output by the DUT, the values of all samples that are output shall be equal to the values of the samples produced by the specified decoding process. The flag BaseLayerOutputFlag and all flags BaseLayerPicOutputFlag output by the HRD shall also be output by the DUT, and the values that are output shall be equal to the values produced by the specified decoding process.

For output timing decoder conformance, the HSS operates as described above, with delivery schedules selected only from the subset of values of SchedSelIdx for which the bit rate and CPB size are restricted as specified in Annexes A, B, and A for the specified profile, tier, and level in Annexes A, B, and A, respectively, or with "interpolated" delivery schedules as specified below for which the bit rate and CPB size are restricted as specified in Annexes A, B, and A for the specified profile, tier, and level in Annexes A, B, and A, respectively. The same delivery schedule is used for both the HRD and the DUT.

When the HRD parameters and the buffering period SEI messages are present with the number of signalled delivery schedules greater than 0, the decoder shall be capable of decoding each bitstream partition as delivered from the HSS operating using an "interpolated" delivery schedule specified as having peak bit rate r, CPB size c( r ), and initial CPB removal delay ( f( r )  r ) as follows:

 = ( r − BitRate[ SchedSelIdx − 1 ] )  ( BitRate[ SchedSelIdx ] − BitRate[ SchedSelIdx − 1 ] ),	(F-95)

c( r ) =  * CpbSize[ SchedSelIdx ] + ( 1 −   * CpbSize[ SchedSelIdx − 1 ],	(F-96)

f( r ) = InitCpbRemovalDelay[ SchedSelIdx ] * BitRate[ SchedSelIdx ] +
		( 1 −  InitCpbRemovalDelay[ SchedSelIdx − 1 ] * BitRate[ SchedSelIdx − 1 ]	(F-97)

for any SchedSelIdx > 0 and r such that BitRate[ SchedSelIdx − 1 ]  <=  r  <=  BitRate[ SchedSelIdx ] such that r and c( r ) are within the limits as specified in Annexes A, B, and A for the maximum bit rate and buffer size for the specified profile, tier and level defined in Annexes A, B, and A, respectively. The following applies for the specifications above:

–	Let combIdx be any value in the range of 0 to num_bsp_schedules_minus1[ TargetOlsIdx ][ TargetPsIdx ][ HighestTid ], inclusive.

–	SchedSelIdx shall be one of the values among bsp_sched_idx[ TargetOlsIdx ][ TargetPsIdx ][ HighestTid ][ combIdx ][ j ] for and j is the index of the bitstream partition index of TargetBitstreamPartition.

–	SchedSelIdx − 1 is to be understood as bsp_sched_idx[ TargetOlsIdx ][ TargetPsIdx ][ HighestTid ][ combIdx − 1 ][ j ] for any value of k in the range of 1 to combIdx, inclusive.

NOTE 1 – InitCpbRemovalDelay[ SchedSelIdx ] can be different from one buffering period to another and need to be recalculated.

For output timing decoder conformance, an HRD as described above is used and the timing (relative to the delivery time of the first bit) of picture output is the same for both the HRD and the DUT up to a fixed delay.

For output order decoder conformance, the following applies for each bitstream partition in TargetPartitioningScheme:

–	The HSS delivers the TargetBitstreamPartition to the DUT "by demand" from the DUT, meaning that the HSS delivers bits (in decoding order) only when the DUT requires more bits to proceed with its processing.

NOTE 2 – This means that for this test, the coded picture buffer of the DUT could be as small as the size of the largest decoding unit.

–	A modified HRD as described below is used, and the HSS delivers the bitstream to the HRD by one of the schedules specified in the TargetBitstreamPartition such that the bit rate and CPB size are restricted as specified in Annex A for profiles defined in Annex A, Annex B for profiles defined in Annex B, and Annex A for profiles defined in Annex A. The order of pictures output shall be the same for both the HRD and the DUT.

–	The HRD BPB size is given by CpbSize[ SchedSelIdx ] as specified in clause A.13.1. The sub-DPB size of the sub-DPB for a layer with nuh_layer_id equal to currLayerId is max_vps_dec_pic_buffering_minus1[ TargetOlsIdx ][ layerIdx ][ HighestTid ] + 1, where layerIdx is equal to the value such that LayerSetLayerIdList[ TargetDecLayerSetIdx ][ layerIdx ] is equal to currLayerId. The removal time from the CPB for the HRD is the final bit arrival time and decoding is immediate. The operation of the DPB of this HRD is as described in the subclauses of clause A.13.5.2.

      - Operation of the output order DPB

        - General

The decoded picture buffer consists of sub-DPBs, and each sub-DPB contains picture storage buffers for storage of decoded pictures of one layer. Each of the picture storage buffers of a sub-DPB contains a decoded picture that is marked as "used for reference" or is held for future output.

The process for output and removal of pictures from the DPB as specified in clause A.13.5.2.2 is invoked, followed by the invocation of the process for picture decoding, marking, additional bumping, and storage as specified in clause A.13.5.2.3. The "bumping" process is specified in clause A.13.5.2.4 and is invoked as specified in clauses A.13.5.2.2 and A.13.5.2.3.

These processes are applied independently for each layer, starting from the base layer, in increasing order of the nuh_layer_id values of the layers in the bitstream. When these processes are applied for a particular layer, only the sub-DPB for the particular layer is affected except for the "bumping" process, which may crop and output pictures, mark pictures as "not needed for output" and empty picture storage buffers for any layer.

NOTE – In the operation of output order DPB, same as in the operation of output timing DPB, decoded pictures with PicOutputFlag equal to 1 in the same access unit are also output consecutively in ascending order of the nuh_layer_id values of the decoded pictures.

Let picture n and the current picture be the coded picture or decoded picture of the access unit n for a particular value of nuh_layer_id, wherein n is a non-negative integer number.

When these processes are applied for a layer with nuh_layer_id equal to currLayerId, the variables MaxNumReorderPics, MaxLatencyIncreasePlus1, MaxLatencyValue, and MaxDecPicBufferingMinus1 are derived as follows:

–	MaxNumReorderPics is set equal to max_vps_num_reorder_pics[ TargetOlsIdx ][ HighestTid ] of the active VPS.

–	MaxLatencyIncreasePlus1 is set equal to the value of the syntax element max_vps_latency_increase_plus1[ TargetOlsIdx ][ HighestTid ] of the active VPS.

–	MaxLatencyValue is set equal to VpsMaxLatencyPictures[ TargetOlsIdx ][ HighestTid ] of the active VPS.

–	MaxDecPicBufferingMinus1 is set equal to the value of the syntax element max_vps_dec_pic_buffering_minus1[ TargetOlsIdx ][ layerIdx ][ HighestTid ] of the active VPS, where layerIdx is equal to the value such that LayerSetLayerIdList[ TargetDecLayerSetIdx ][ layerIdx ] is equal to currLayerId.

        - Output and removal of pictures from the DPB

When the current picture is not picture 0 in the current layer, the output and removal of pictures in the current layer, with nuh_layer_id equal to currLayerId, from the DPB before the decoding of the current picture, i.e., picture n, but after parsing the slice header of the first slice of the current picture and before the invocation of the decoding process for picture order count, happens instantaneously when the first decoding unit of the current picture is removed from the CPB and proceeds as follows:

–	When the current picture is a POC resetting picture, all pictures in the DPB that do not belong to the current access unit and that are marked as "needed for output" are output, starting with pictures with the smallest value of PicOrderCntVal of all pictures excluding those in the current access unit in the DPB, in ascending order of the PicOrderCntVal values, and pictures with the same value of PicOrderCntVal are output in ascending order of the nuh_layer_id values. When a picture is output, it is cropped using the conformance cropping window specified in the active SPS for the picture, the cropped picture is output, and the picture is marked as "not needed for output".

–	The decoding processes for picture order count and RPS are invoked. When decoding a CVS conforming to one or more of the profiles specified in Annex A using the decoding process specified in clauses 2 through 10, the decoding processes for picture order count and RPS that are invoked are as specified in clauses 8.3.1 and 8.3.2, respectively. When decoding a CVS conforming to one or more of the profiles specified in Annex B or A using the decoding process specified in Annex A, and Annex B or A, the decoding processes for picture order count and RPS that are invoked are as specified in clauses A.8.3.1 and A.8.3.2, respectively.

–	The variable listOfSubDpbsToEmpty is derived as follows:

–	If a new VPS is activated by the current access unit or the current picture is IRAP picture with nuh_layer_id equal to SmallestLayerId, NoRaslOutputFlag equal to 1, and NoClrasOutputFlag equal to 1, listOfSubDpbsToEmpty is set equal to all the sub-DPBs.

–	Otherwise, if the current picture is an IRAP picture with any nuh_layer_id value indepLayerId such that NumDirectRefLayers[ indepLayerId ] is equal to 0 and indepLayerId is greater than SmallestLayerId, and with NoRaslOutputFlag equal to 1, and LayerResetFlag is equal to 1, listOfSubDpbsToEmpty is set equal to the sub-DPBs containing the current layer and the sub-DPBs containing the predicted layers of the current layer.

–	Otherwise, crossLayerBufferEmptyFlag is set equal to 0.

–	If the current picture is an IRAP picture with NoRaslOutputFlag equal to 1 and any of the following conditions is true:

–	nuh_layer_id equal to SmallestLayerId,

–	nuh_layer_id of the current layer is greater than SmallestLayerId, and NumDirectRefLayers[ nuh_layer_id ] is equal to 0,

the following ordered steps are applied:

1.	The variable NoOutputOfPriorPicsFlag is derived for the decoder under test as follows:

–	If the current picture is a CRA picture, NoOutputOfPriorPicsFlag is set equal to 1 (regardless of the value of no_output_of_prior_pics_flag).

–	Otherwise, if the value of pic_width_in_luma_samples, pic_height_in_luma_samples, chroma_format_idc, bit_depth_luma_minus8, bit_depth_chroma_minus8, separate_colour_plane_flag, or sps_max_dec_pic_buffering_minus1[ HighestTid ] derived from the active SPS for the current layer is different from the value of pic_width_in_luma_samples, pic_height_in_luma_samples, chroma_format_idc, bit_depth_luma_minus8, bit_depth_chroma_minus8, separate_colour_plane_flag, or sps_max_dec_pic_buffering_minus1[ HighestTid ], respectively, derived from the SPS that was active for the current layer when decoding the preceding picture in the current layer, NoOutputOfPriorPicsFlag may (but should not) be set equal to 1 by the decoder under test, regardless of the value of no_output_of_prior_pics_flag.

NOTE – Although setting NoOutputOfPriorPicsFlag equal to no_output_of_prior_pics_flag is preferred under these conditions, the decoder under test is allowed to set NoOutputOfPriorPicsFlag to 1 in this case.

–	Otherwise, NoOutputOfPriorPicsFlag is set equal to no_output_of_prior_pics_flag.

2.	The value of NoOutputOfPriorPicsFlag derived for the decoder under test is applied for the HRD as follows:

–	If NoOutputOfPriorPicsFlag is equal to 0, all non-empty picture storage buffers in all the sub-DPBs included in listOfSubDpbsToEmpty are output by repeatedly invoking the "bumping" process specified in clause A.13.5.2.4 until all these pictures are marked as "not needed for output".

–	Otherwise (NoOutputOfPriorPicsFlag is equal to 1), all picture storage buffers containing a picture that is marked as "not needed for output" and "unused for reference" are emptied (without output), all pictures that are contained in a sub-DPB included in listOfSubDpbsToEmpty are emptied, and the sub-DPB fullness of each sub-DPB is decremented by the number of picture storage buffers emptied in that sub-DPB.

–	Otherwise, all picture storage buffers that contain a picture in the current layer and that are marked as "not needed for output" and "unused for reference" are emptied (without output). For each picture storage buffer that is emptied, the sub-DPB fullness is decremented by one. When one or more of the following conditions are true, the "bumping" process specified in clause A.13.5.2.4 is invoked repeatedly until none of the following conditions are true:

- The number of access units that contain at least one decoded picture in the DPB marked as "needed for output" is greater than MaxNumReorderPics.

- MaxLatencyIncreasePlus1 is not equal to 0 and there is at least one access unit that contains at least one decoded picture in the DPB marked as "needed for output" for which the associated variable PicLatencyCount is greater than or equal to MaxLatencyValue.

- The number of pictures in the sub-DPB is greater than or equal to MaxDecPicBufferingMinus1 + 1.

        - Picture decoding, marking, additional bumping, and storage

The processes specified in this clause happen instantaneously when the last decoding unit of picture n is removed from the CPB.

The current picture is considered as decoded after the last decoding unit of the picture is decoded. The current decoded picture is stored in an empty picture storage buffer in the sub-DPB.

When the current picture is the last picture in an access unit, the following applies for each decoded picture with nuh_layer_id greater than or equal to ( vps_base_layer_internal_flag ? 0 : 1 ) of the access unit:

–	If the decoded picture has PicOutputFlag equal to 1, it is marked as "needed for output".

–	Otherwise (the decoded picture has PicOutputFlag equal to 0), it is marked as "not needed for output".

NOTE – Prior to investigating the conditions above, PicOutputFlag of each picture of the access unit is updated as specified in clause A.8.1.3.

The current decoded picture is marked as "used for short-term reference".

When one or more of the following conditions are true, the "bumping" process specified in clause A.13.5.2.4 is invoked repeatedly until none of the following conditions are true:

–	The number of access units that contain at least one decoded picture in the DPB marked as "needed for output" is greater than MaxNumReorderPics.

–	MaxLatencyIncreasePlus1 is not equal to 0 and there is at least one access unit that contains at least one decoded picture in the DPB marked as "needed for output" for which the associated variable PicLatencyCount is greater than or equal to MaxLatencyValue.

        - "Bumping" process

The "bumping" process consists of the following ordered steps:

- The picture or pictures that are first for output are selected as the ones having the smallest value of PicOrderCntVal of all pictures in the DPB marked as "needed for output".

- Each of these pictures is, in ascending nuh_layer_id order, cropped, using the conformance cropping window specified in the active SPS for the picture, the cropped picture is output, and the picture is marked as "not needed for output".

- Each picture storage buffer that contains a picture marked as "unused for reference" and that was one of the pictures cropped and output is emptied and the fullness of the associated sub-DPB is decremented by one.

    - Demultiplexing process for deriving a bitstream partition

Inputs to this process are a bitstream, a layer identifier list bspLayerId[ bspIdx ], and the number of layer identifiers numBspLayers in the layer index list bspLayerId[ bspIdx ].

Output of this process is a bitstream partition.

Let the variable minBspLayerId be the smallest value of bspLayerId[ bspIdx ] with any value of bspIdx in the range of 0 to numBspLayers − 1, inclusive.

The output bitstream partition consists of selected NAL units of the input bitstream in the same order as they appear in the input bitstream. The following NAL units of the input bitstream are omitted from the output bitstream partition, while the remaining NAL units of the input bitstream are included in the output bitstream partition:

–	Omit all NAL units that have a nuh_layer_id value other than bspLayerId[ bspIdx ] with any value of bspIdx in the range of 0 to numBspLayers − 1, inclusive.

–	Omit all SEI NAL units containing a scalable nesting SEI message for which no derived nestingLayerIdList[ i ] contains any layer identifier value equal to bspLayerId[ bspIdx ] with any value of bspIdx in the range of 0 to numBspLayers − 1, inclusive.

–	Omit all SEI NAL units containing a scalable nesting SEI message for which a derived nestingLayerIdList[ i ] contains a layer identifier value less than minBspLayerId.

  - Supplemental enhancement information

    - General

The specifications in clause D.1 apply.

    - SEI payload syntax

      - General SEI payload syntax

The specifications in clause D.2.1 apply.

      - Annex D SEI message syntax for multi-layer extensions

The specifications in clauses D.2.2 through D.2.34 apply.

      - Layers not present SEI message syntax

| layers_not_present( payloadSize ) { | Descriptor |
| --- | --- |
| lnp_sei_active_vps_id | u(4) |
| for( i = 0; i <= MaxLayersMinus1; i++ ) |  |
| layer_not_present_flag[ i ] | u(1) |
| } |  |

      - Inter-layer constrained tile sets SEI message syntax

| inter_layer_constrained_tile_sets( payloadSize ) { | Descriptor |
| --- | --- |
| il_all_tiles_exact_sample_value_match_flag | u(1) |
| il_one_tile_per_tile_set_flag | u(1) |
| if( !il_one_tile_per_tile_set_flag ) { |  |
| il_num_sets_in_message_minus1 | ue(v) |
| if( il_num_sets_in_message_minus1 ) |  |
| skipped_tile_set_present_flag | u(1) |
| numSignificantSets = il_num_sets_in_message_minus1 − skipped_tile_set_present_flag + 1 |  |
| for( i = 0; i < numSignificantSets; i++ ) { |  |
| ilcts_id[ i ] | ue(v) |
| il_num_tile_rects_in_set_minus1[ i ] | ue(v) |
| for( j = 0; j <= il_num_tile_rects_in_set_minus1[ i ]; j++ ) { |  |
| il_top_left_tile_index[ i ][ j ] | ue(v) |
| il_bottom_right_tile_index[ i ][ j ] | ue(v) |
| } |  |
| ilc_idc[ i ] | u(2) |
| if ( !il_all_tiles_exact_sample_value_match_flag ) |  |
| il_exact_sample_value_match_flag[ i ] | u(1) |
| } |  |
| } else |  |
| all_tiles_ilc_idc | u(2) |
| } |  |

      - Bitstream partition nesting SEI message syntax

| bsp_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| sei_ols_idx | ue(v) |
| sei_partitioning_scheme_idx | ue(v) |
| bsp_idx | ue(v) |
| while( !byte_aligned( ) ) |  |
| bsp_nesting_zero_bit /* equal to 0 */ | u(1) |
| num_seis_in_bsp_minus1 | ue(v) |
| for( i = 0; i <= num_seis_in_bsp_minus1; i++ ) |  |
| sei_message( ) |  |
| } |  |

      - Bitstream partition initial arrival time SEI message syntax

| bsp_initial_arrival_time( payloadSize ) { | Descriptor |
| --- | --- |
| psIdx = sei_partitioning_scheme_idx |  |
| if( nalInitialArrivalDelayPresent ) |  |
| for( i = 0; i < BspSchedCnt[ sei_ols_idx ][ psIdx ][ MaxTemporalId[ 0 ] ]; i++ ) |  |
| nal_initial_arrival_delay[ i ] | u(v) |
| if( vclInitialArrivalDelayPresent ) |  |
| for( i = 0; i < BspSchedCnt[ sei_ols_idx ][ psIdx ][ MaxTemporalId[ 0 ] ]; i++ ) |  |
| vcl_initial_arrival_delay[ i ] | u(v) |
| } |  |

      - Sub-bitstream property SEI message syntax

| sub_bitstream_property( payloadSize ) { | Descriptor |
| --- | --- |
| sb_property_active_vps_id | u(4) |
| num_additional_sub_streams_minus1 | ue(v) |
| for( i = 0; i <= num_additional_sub_streams_minus1; i++ ) { |  |
| sub_bitstream_mode[ i ] | u(2) |
| ols_idx_to_vps[ i ] | ue(v) |
| highest_sublayer_id[ i ] | u(3) |
| avg_sb_property_bit_rate[ i ] | u(16) |
| max_sb_property_bit_rate[ i ] | u(16) |
| } |  |
| } |  |

      - Alpha channel information SEI message syntax

| alpha_channel_info( payloadSize ) { | Descriptor |
| --- | --- |
| alpha_channel_cancel_flag | u(1) |
| if( !alpha_channel_cancel_flag ) { |  |
| alpha_channel_use_idc | u(3) |
| alpha_channel_bit_depth_minus8 | u(3) |
| alpha_transparent_value | u(v) |
| alpha_opaque_value | u(v) |
| alpha_channel_incr_flag | u(1) |
| alpha_channel_clip_flag | u(1) |
| if( alpha_channel_clip_flag ) |  |
| alpha_channel_clip_type_flag | u(1) |
| } |  |
| } |  |

      - Overlay information SEI message syntax

| overlay_info( payloadSize ) { | Descriptor |
| --- | --- |
| overlay_info_cancel_flag | u(1) |
| if( !overlay_info_cancel_flag ) { |  |
| overlay_content_aux_id_minus128 | ue(v) |
| overlay_label_aux_id_minus128 | ue(v) |
| overlay_alpha_aux_id_minus128 | ue(v) |
| overlay_element_label_value_length_minus8 | ue(v) |
| num_overlays_minus1 | ue(v) |
| for( i = 0; i <= num_overlays_minus1; i++ ) { |  |
| overlay_idx[ i ] | ue(v) |
| language_overlay_present_flag[ i ] | u(1) |
| overlay_content_layer_id[ i ] | u(6) |
| overlay_label_present_flag[ i ] | u(1) |
| if( overlay_label_present_flag[ i ] ) |  |
| overlay_label_layer_id[ i ] | u(6) |
| overlay_alpha_present_flag[ i ] | u(1) |
| if( overlay_alpha_present_flag[ i ] ) |  |
| overlay_alpha_layer_id[ i ] | u(6) |
| if( overlay_label_present_flag[ i ] ) { |  |
| num_overlay_elements_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_overlay_elements_minus1[ i ]; j++ ) { |  |
| overlay_element_label_min[ i ][ j ] | u(v) |
| overlay_element_label_max[ i ][ j ] | u(v) |
| } |  |
| } |  |
| } |  |
| while( !byte_aligned( ) ) |  |
| overlay_zero_bit /* equal to 0 */ | f(1) |
| for( i = 0; i <= num_overlays_minus1; i++ ) { |  |
| if( language_overlay_present_flag[ i ] ) |  |
| overlay_language[ i ] | st(v) |
| overlay_name[ i ] | st(v) |
| if( overlay_label_present_flag[ i ] ) |  |
| for( j = 0; j <= num_overlay_elements_minus1[ i ]; j++ ) |  |
| overlay_element_name[ i ][ j ] | st(v) |
| } |  |
| overlay_info_persistence_flag | u(1) |
| } |  |
| } |  |

      - Temporal motion vector prediction constraints SEI message syntax

| temporal_mv_prediction_constraints( payloadSize ) { | Descriptor |
| --- | --- |
| prev_pics_not_used_flag | u(1) |
| no_intra_layer_col_pic_flag | u(1) |
| } |  |

      - Frame-field information SEI message syntax

| frame_field_info( payloadSize ) { | Descriptor |
| --- | --- |
| ffinfo_pic_struct | u(4) |
| ffinfo_source_scan_type | u(2) |
| ffinfo_duplicate_flag | u(1) |
| } |  |

    - SEI payload semantics

      - General SEI payload semantics

The general SEI payload semantics specified in clause D.3.1 apply with the following modifications and additions:

The list VclAssociatedSeiList is set to consist of the payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 128, 131, 132, and 134, 135, 136, 137, 138, 139, 140, 141, 142, 143, 161, 165, 167, and 168.

The list PicUnitRepConSeiList is set to consist of the payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 128, 129, 131, 132, 133, 135, 136, 137, 138, 139, 140, 141, 142, 143, 160, 161, 162, 163, 164, 165, 166, 167, and 168.

The semantics and persistence scope for each SEI message specified in this annex are specified in the semantics specification for each particular SEI message in clauses A.14.3.3 to A.14.3.11, inclusive.

NOTE – Persistence information for SEI messages specified in this annex is informatively summarized in Table FF.4.

Table FF.4 – Persistence scope of SEI messages (informative)

| - SEI message | - Persistence scope |
| --- | --- |
| Layers not present | - Specified by the semantics of the SEI message |
| Inter-layer constrained tile sets | - The CLVS associated with the SEI message |
| Bitstream partition nesting | - Depending on the nested SEI messages. Each nested SEI message has the same persistence scope as if the SEI message was not nested |
| Bitstream partition initial arrival time | - The remainder of the bitstream partition (specified by the containing bitstream partition nesting SEI message) |
| Sub-bitstream property | - The CVS containing the SEI message |
| Alpha channel information | - Specified by the syntax of the SEI message |
| Overlay information | - Specified by the syntax of the SEI message |
| Temporal motion vector prediction constraints | - Specified by the semantics of the SEI message |
| Frame-field information | - The access unit containing the SEI message |

Let prevVclNalUnitInAu of an SEI NAL unit or an SEI message be the preceding VCL NAL unit in decoding order, if any, in the same access unit, and nextVclNalUnitInAu of an SEI NAL unit or an SEI message be the next VCL NAL unit in decoding order, if any, in the same access unit. It is a requirement of bitstream conformance that the following restrictions apply:

–	When a buffering period SEI message, a picture timing SEI message, a decoding unit information SEI message, or a bitstream partition initial arrival time SEI message is present in a bitstream partition nesting SEI message contained in a scalable nesting SEI message, the scalable nesting SEI message shall not follow any other SEI message that follows the prevVclNalUnitInAu of the scalable nesting SEI message and precedes the nextVclNalUnitInAu of the scalable nesting SEI message, other than an active parameter sets SEI message, a non-nested buffering period SEI message, a non-nested picture timing SEI message, a non-nested decoding unit information SEI message, a scalable nesting SEI message including a buffering period SEI message, a picture timing SEI message, a decoding unit information SEI message, or another scalable nesting SEI message that contains a bitstream partition nesting SEI message including a buffering period SEI message, a picture timing SEI message, a decoding unit information SEI message, or a bitstream partition initial arrival time SEI message.

      - Annex D SEI message semantics for multi-layer extensions

        - General

The semantics of SEI messages specified in clauses D.3.2 through D.3.34 apply with the modifications specified in clauses A.14.3.2.2 through A.14.3.2.8.

        - Buffering period SEI message semantics for multi-layer extensions

The specifications of clause D.3.2 apply with the following modifications and additions:

When the buffering period SEI message is directly contained in a scalable nesting SEI message within an SEI NAL unit with a nuh_layer_id value targetNuhLayerId greater than 0, the semantics of clause D.3.2 apply to the output bitstream of the independent non-base layer rewriting process of clause A.10.2 with the input variables assignedBaseLayerId equal to nestingLayerIdList[ i ][ 0 ] and tIdTarget equal to MaxTemporalId[ i ] for each value of i in the range of 0 to nestingNumOps − 1, inclusive.

When the buffering period SEI message is contained in a bitstream partition nesting SEI message, the following applies:

–	A set of skipped leading pictures skippedPictureList consists of the CL-RAS pictures and the RASL pictures associated with the IRAP pictures with nuh_layer_id equal to nuhLayerId for which LayerInitializedFlag[ nuhLayerId ] is equal to 0 at the start of decoding the IRAP picture and for which nuhLayerId is among the nestingLayerIdList[ i ] for any value of i in the range of 0 to nesting_num_ops_minus1, inclusive.

–	A picture is said to be a notDiscardablePic picture when the picture has TemporalId equal to 0, has discardable_flag equal to 1, and is not a RASL, RADL, or SLNR picture.

–	The variables olsIdx, psIdx, and bspIdx are set equal to sei_ols_idx, sei_partitioning_scheme_idx, and bsp_idx, respectively, of the bitstream partition nesting SEI message containing this buffering period SEI message, and the bspIdx-th bitstream partition of the psIdx-th partitioning scheme of the olsIdx-th OLS is referred to as the current bitstream partition.

–	The variable maxTId is set equal to MaxTemporalId[ 0 ] for the scalable nesting SEI message containing the bitstream partition nesting SEI message containing this buffering period SEI message.

–	The variable CpbCnt is set equal to BspSchedCnt[ olsIdx ][ psIdx ][ maxTId ] – 1 for the syntax and semantics of the buffering period SEI message and for the constraints specified below.

–	The variable hrdParamIdx[ i ] is set equal to the value of bsp_hrd_idx[ olsIdx ][ psIdx ][ maxTId ][ i ][ bspIdx ] for each value of i in the range of 0 to CpbCnt, inclusive.

–	The syntax elements au_cpb_removal_delay_length_minus1, dpb_output_delay_length_minus1, and sub_pic_hrd_params_present_flag are found in or derived from the hrdParamIdx[ i ]-th hrd_parameters( ) syntax structure for any value of i in the range of 0 to CpbCnt. It is a requirement of bitstream conformance that the values of au_cpb_removal_delay_length_minus1, dpb_output_delay_length_minus1, and sub_pic_hrd_params_present_flag derived using a particular value of i shall be the same as the value of the respective syntax elements derived using any other allowed value of i.

–	The syntax element initial_cpb_removal_delay_length_minus1 is found in or derived from the hrdParamIdx[ i ]-th hrd_parameters( ) syntax structure for the semantics of nal_initial_cpb_removal_delay[ i ], nal_initial_alt_cpb_removal_delay[ i ], nal_initial_cpb_removal_offset[ i ], nal_initial_alt_cpb_removal_offset[ i ], vcl_initial_cpb_removal_delay[ i ], vcl_initial_alt_cpb_removal_delay[ i ], vcl_initial_cpb_removal_offset[ i ], and vcl_initial_alt_cpb_removal_offset[ i ].

–	The variable CpbSize[ i ] is set equal to BpbSize[ olsIdx ][ psIdx ][ maxTId ][ i ][ bspIdx ] for the semantics of nal_initial_cpb_removal_delay[ i ], nal_initial_alt_cpb_removal_delay[ i ], vcl_initial_cpb_removal_delay[ i ], and vcl_initial_alt_cpb_removal_delay[ i ].

–	The variable BitRate[ i ] is set equal to BpBitRate[ olsIdx ][ psIdx ][ maxTId ][ i ][ bspIdx ] for the semantics of nal_initial_cpb_removal_delay[ i ], nal_initial_alt_cpb_removal_delay[ i ], vcl_initial_cpb_removal_delay[ i ], and vcl_initial_alt_cpb_removal_delay[ i ].

–	The variables NalHrdBpPresentFlag and VclHrdBpPresentFlag are derived from the hrdParamIdx[ i ]-th hrd_parameters( ) syntax structure for any value of i in the range of 0 to CpbCnt. It is a requirement of bitstream conformance that the value of NalHrdBpPresentFlag shall be the same regardless of the value of i used in its derivation. It is a requirement of bitstream conformance that the value of VclHrdBpPresentFlag shall be the same regardless of the value of i used in its derivation.

The presence of buffering period SEI messages for the current bitstream partition is specified as follows on the basis of the variables NalHrdBpPresentFlag and VclHrdBpPresentFlag that are derived as specified above:

–	If NalHrdBpPresentFlag is equal to 1 or VclHrdBpPresentFlag is equal to 1, the following applies for each access unit in the CVS:

- If the access unit is an IRAP access unit, a buffering period SEI message applicable to the current bitstream partition shall be associated with the access unit.

- Otherwise, if the access unit contains a notDiscardablePic in at least one layer included in the current bitstream partition, a buffering period SEI message applicable to the current bitstream partition may or may not be associated with the access unit.

- Otherwise, the access unit shall not be associated with a buffering period SEI message applicable to the current bitstream partition.

–	Otherwise (NalHrdBpPresentFlag and VclHrdBpPresentFlag are both equal to 0), no access unit in the CVS shall be associated with a buffering period SEI message applicable to the current bitstream partition.

When the buffering period SEI message is contained in a bitstream partition nesting SEI message, the following semantics of concatenation_flag and au_cpb_removal_delay_delta_minus1 replace those specified in subcaluse D.3.2 and the following specifications apply to each picture currPic with nuh_layed_id currNuhLayerId equal to any value included in the current bitstream partition in the current access unit.

When the picture currPic with nuh_layer_id equal to targetLayerId in the current access unit is not the first picture with that nuh_layer_id value in the bitstream in decoding order, let prevNonDiscardablePic be the preceding picture in decoding order with that nuh_layer_id value and with TemporalId equal to 0 that is not a RASL, RADL or SLNR picture.

concatenation_flag indicates, when the picture currPic is not the first picture with nuh_layer_id equal to currNuhLayerId in the bitstream in decoding order, whether the nominal CPB removal time of the current picture is determined relative to the nominal CPB removal time of the preceding picture with a buffering period SEI message that applies to the current bitstream partition, or relative to the nominal CPB removal time of the picture prevNonDiscardablePic.

au_cpb_removal_delay_delta_minus1 plus 1, when the picture currPic is not the first picture with nuh_layer_id equal to currNuhLayerId in the bitstream in decoding order, specifies a CPB removal delay increment value relative to the nominal CPB removal time of the picture prevNonDiscardablePic. This syntax element has a length in bits given by au_cpb_removal_delay_length_minus1 + 1.

When the buffering period SEI message is contained in a bitstream partition nesting SEI message and concatenation_flag is equal to 0 and the picture currPic is not the first picture with nuh_layer_id equal to currNuhLayerId in the bitstream in decoding order, it is a requirement of bitstream conformance that the following constraint applies:

- If the picture prevNonDiscardablePic is not associated with a buffering period SEI message that applies the current bitstream partition, the au_cpb_removal_delay_minus1 of the picture currPic shall be equal to the au_cpb_removal_delay_minus1 of prevNonDiscardablePic, indicated in the picture timing SEI message applicable to the current bitstream partition, plus au_cpb_removal_delay_delta_minus1 + 1.

- Otherwise, au_cpb_removal_delay_minus1, indicated for the picture currPic in the picture timing SEI message applicable to the current bitstream partition, shall be equal to au_cpb_removal_delay_delta_minus1.

        - Picture timing SEI message semantics for multi-layer extensions

The specifications of clause D.3.3 apply with the following modifications and additions:

When the picture timing SEI message is directly contained in a scalable nesting SEI message within an SEI NAL unit with a nuh_layer_id value targetNuhLayerId greater than 0, the semantics of clause D.3.3 apply to the output bitstream of the independent non-base layer rewriting process of clause A.10.2 with the input variables assignedBaseLayerId equal to nestingLayerIdList[ i ][ 0 ] and tIdTarget equal to MaxTemporalId[ i ] for each value of i in the range of 0 to nestingNumOps − 1, inclusive.

When the picture timing SEI message is contained in a bitstream partition nesting SEI message, the following applies:

–	frame_field_info_present_flag is inferred to be equal to 0 for the syntax and semantics of the picture timing SEI message.

NOTE 1 – The frame-field information SEI message can be used to indicate the frame-field information.

–	The variables olsIdx, psIdx, and bspIdx are set equal to sei_ols_idx, sei_partitioning_scheme_idx, and bsp_idx, respectively, of the bitstream partition nesting SEI message containing this buffering period SEI message, and the bspIdx-th bitstream partition of the psIdx-th partitioning scheme of the olsIdx-th OLS is referred to as the current bitstream partition.

–	The variable maxTId is set equal to MaxTemporalId[ 0 ] for the scalable nesting SEI message containing the bitstream partition nesting SEI message containing this buffering period SEI message.

–	The variable hrdParamIdx is set equal to the value of bsp_hrd_idx[ olsIdx ][ psIdx ][ maxTId ][ i ][ bspIdx ] for any value of i in the range of 0 to BspSchedCnt[ olsIdx ][ psIdx ][ maxTId ] – 1, inclusive.

–	The following applies for the syntax and semantics of the picture timing SEI message:

–	The syntax elements sub_pic_hrd_params_present_flag, sub_pic_cpb_params_in_pic_timing_sei_flag, au_cpb_removal_delay_length_minus1, dpb_output_delay_length_minus1, dpb_output_delay_du_length_minus1, du_cpb_removal_delay_increment_length_minus1, and the variable CpbDpbDelaysPresentFlag are found in or derived from syntax elements found in the hrdParamIdx-th hrd_parameters( ) syntax structure.

–	It is a requirement of bitstream conformance that the values of sub_pic_hrd_params_present_flag, sub_pic_cpb_params_in_pic_timing_sei_flag, au_cpb_removal_delay_length_minus1, dpb_output_delay_length_minus1, dpb_output_delay_du_length_minus1, du_cpb_removal_delay_increment_length_minus1, and CpbDpbDelaysPresentFlag derived using a particular value of i for the derivation of hrdParamIdx shall be the same as the value of the respective syntax elements or variable derived using any other allowed value of i.

The presence of picture timing SEI messages for the current bitstream partition is specified as follows on the basis of the variable CpbDpbDelaysPresentFlag that is derived as specified above:

–	If CpbDpbDelaysPresentFlag is equal to 1, a picture timing SEI message applicable to the current bitstream partition shall be associated with every access unit in the CVS.

–	Otherwise, in the CVS there shall be no access unit that is associated with a picture timing SEI message applicable to the current bitstream partition.

The semantics of num_decoding_units_minus1 and num_nalus_in_du_minus1[ i ] are replaced with the following:

The variables targetUnit and totalSizeInCtbsY are derived as follows:

–	If the picture timing SEI message is associated with a partition unit, targetUnit is set to be the partition unit, and the value of totalSizeInCtbsY is set equal to the sum of the PicSizeInCtbsY of all pictures in the partition unit.

–	Otherwise (the picture timing SEI message is associated with an access unit), targetUnit is set to be the access unit, and the value of totalSizeInCtbsY is set equal to the sum of the PicSizeInCtbsY of all pictures in the access unit.

num_decoding_units_minus1 plus 1 specifies the number of decoding units in targetUnit. The value of num_decoding_units_minus1 shall be in the range of 0 to totalSizeInCtbsY − 1, inclusive.

num_nalus_in_du_minus1[ i ] plus 1 specifies the number of NAL units in the i-th decoding unit of targetUnit. The value of num_nalus_in_du_minus1[ i ] shall be in the range of 0 to totalSizeInCtbsY − 1, inclusive.

The first decoding unit of targetUnit consists of the first num_nalus_in_du_minus1[ 0 ] + 1 consecutive NAL units in decoding order in targetUnit. The i-th (with i greater than 0) decoding unit of targetUnit consists of the num_nalus_in_du_minus1[ i ] + 1 consecutive NAL units immediately following the last NAL unit in the previous decoding unit of targetUnit, in decoding order. There shall be at least one VCL NAL unit in each decoding unit. All non-VCL NAL units associated with a VCL NAL unit shall be included in the same decoding unit as the VCL NAL unit.

        - Recovery point SEI message semantics for multi-layer extensions

The recovery point SEI message assists a decoder in determining when the decoding process will produce acceptable pictures in the current layer for display after the decoder initiates random access, layer up-switching, or after the encoder indicates a broken link.

When all decoded pictures in earlier access units in decoding order are removed from the bitstream, the recovery point picture (defined below) and all the subsequent pictures in output order in the current layer can be correctly or approximately correctly decoded, the current picture is referred to as a layer random-accessing picture. When all the pictures that belong to the reference layers of the current layer and may be used for reference by the current picture or subsequent pictures in decoding order are correctly decoded, and the recovery point picture and all the subsequent pictures in output order in the current layer can be correctly or approximately correctly decoded when no picture prior to the current picture in decoding order in the current layer are present in the bitstream, the current picture is referred to as a layer up-switching picture.

When the recovery point SEI message applies to the current layer and all the reference layers of the current layer, the current picture is indicated as a layer random-accessing picture. When the recovery point SEI message applies to the current layer but not to all the reference layers of the current layer, the current picture is indicated as a layer up-switching picture.

Decoded pictures in the current layer produced by random access or layer up-switching at or before the current access unit does not need to be correct in content until the indicated recovery point, and the operation of the decoding process for pictures in the current layer starting at the current picture may contain references to pictures unavailable in the decoded picture buffer.

In addition, by use of the broken_link_flag, the recovery point SEI message can indicate to the decoder the location of some pictures in the current layer in the bitstream that can result in serious visual artefacts when displayed, even when the decoding process was begun at the location of a previous IRAP access unit in decoding order that contain IRAP pictures in all layers.

NOTE 1 – The broken_link_flag can be used by encoders to indicate the location of a point after which the decoding process for the decoding of some pictures in the current layer may cause references to pictures that, though available for use in the decoding process, are not the pictures that were used for reference when the bitstream was originally encoded (e.g., due to a splicing operation performed during the generation of the bitstream).

When the current picture is a layer random access-accessing picture and random access is performed to start decoding from the current access unit, the decoder operates as if the current access unit was the first access unit in the bitstream in decoding order, and the following applies:

–	If poc_msb_cycle_val is present for the current picture, the PicOrderCntVal of the current picture is derived as specified in the process for derivation of PicOrderCntVal in clause A.8.3.1.

–	Otherwise (poc_msb_cycle_val is not present for the current picture), the variable PrevPicOrderCnt[ nuh_layer_id ] used in derivation of PicOrderCntVal for each picture in the access unit is set equal to 0.

NOTE 2 – When HRD information is present in the bitstream, a buffering period SEI message should be associated with the access unit associated with the recovery point SEI message in order to establish initialization of the HRD buffer model after a random access.

When the current picture is either a layer random-accessing picture or a layer up-switching picture and layer up-switching is performed to start decoding of the current layer from the current access (while decoding of the reference layers of the current layer have started earlier and pictures of those layers in the current access unit are correctly decoded), the decoder operates as if the current picture was the first picture of the current layer in the bitstream in decoding order, and the PicOrderCntVal of the current picture is set equal to the PicOrderCntVal of any other decoded picture in the current access unit.

For a particular access unit auA, let auB be the first access unit that succeeds auA in decoding order such that the picture order count values of the pictures in auB can be derived without using the picture order count values of pictures preceding auB in decoding order. For a particular layer layerA, auA is not allowed to contain a recovery point SEI message that applies to a set of layers containing at least layerA and all reference layers of layerA when all of the following conditions are true:

–	All pictures in auA that are in the reference layers, if any, of layerA have both poc_msb_cycle_val_present_flag and poc_reset_idc equal to 0.

–	There is at least one picture picA with poc_msb_cycle_val_present_flag equal to 1 in the following access units:

–	Access units that follow auA in decoding order and precede auB, when present, in decoding order.

–	Access unit auB, when present and when picture in auB with nuh_layer_id equal to 0 is not an IRAP picture with NoClrasOutputFlag equal to 1.

Any SPS or PPS RBSP that is referred to by a picture of the access unit containing a recovery point SEI message or by any picture in a subsequent access unit in decoding order shall be available to the decoding process prior to its activation, regardless of whether or not the decoding process is started at the beginning of the bitstream or with the access unit, in decoding order, that contains the recovery point SEI message.

recovery_poc_cnt specifies the recovery point of decoded pictures in the current layer in output order. If there is a picture picB in the current layer that follows the current picture picA but precedes an access unit containing an IRAP picture in the current layer in decoding order and PicOrderCnt( picB ) is equal to PicOrderCnt( picA ) plus the value of recovery_poc_cnt, where PicOrderCnt( picA ) and PicOrderCnt( picB ) are the PicOrderCntVal values of picA and picB, respectively, immediately after the invocation of the decoding process for picture order count for picB, the picture picB is referred to as the recovery point picture. Otherwise, the first picture picC in the current layer in output order for which PicOrderCnt( picC ) is greater than PicOrderCnt( picA ) plus the value of recovery_poc_cnt is referred to as the recovery point picture, where PicOrderCnt( picA ) and PicOrderCnt( picC ) are the PicOrderCntVal values of picA and picC, respectively, immediately after the invocation of the decoding process for picture order count for picC. The recovery point picture shall not precede the current picture in decoding order. All decoded pictures in the current layer in output order are indicated to be correct or approximately correct in content starting at the output order position of the recovery point picture. The value of recovery_poc_cnt shall be in the range of −MaxPicOrderCntLsb / 2 to MaxPicOrderCntLsb / 2 − 1, inclusive.

exact_match_flag indicates whether decoded pictures in the current layer at and subsequent to the specified recovery point in output order derived by starting the decoding process at the access unit containing the recovery point SEI message will be an exact match to the pictures in the current layer that would be produced by starting the decoding process at the location of a previous access unit where the picture of the layer in the current layer and the pictures of all the reference layers are IRAP pictures, if any, in the bitstream. The value 0 indicates that the match may not be exact and the value 1 indicates that the match will be exact. When exact_match_flag is equal to 1, it is a requirement of bitstream conformance that the decoded pictures in the current layer at and subsequent to the specified recovery point in output order derived by starting the decoding process at the access unit containing the recovery point SEI message shall be an exact match to the pictures in the current layer that would be produced by starting the decoding process at the location of a previous access unit where the picture in the current layer and the pictures of all the reference layers are IRAP pictures, if any, in the bitstream.

NOTE 3 – When performing random access, decoders should infer all references to unavailable pictures as references to pictures containing only intra coding blocks and having sample values given by Y equal to ( 1  <<  ( BitDepthY − 1 ) ), Cb and Cr both equal to ( 1  <<  ( BitDepthC − 1 ) ) (mid-level grey), regardless of the value of exact_match_flag.

When exact_match_flag is equal to 0, the quality of the approximation at the recovery point is chosen by the encoding process and is not specified in this Specification.

broken_link_flag indicates the presence or absence of a broken link in the current layer at the location of the recovery point SEI message and is assigned further semantics as follows:

–	If broken_link_flag is equal to 1, pictures in the current layer produced by starting the decoding process at the location of a previous access unit where the picture in the current layer and the pictures of all the reference layers are IRAP pictures may contain undesirable visual artefacts to the extent that decoded pictures in the current layer at and subsequent to the access unit containing the recovery point SEI message in decoding order should not be displayed until the specified recovery point in output order.

–	Otherwise (broken_link_flag is equal to 0), no indication is given regarding any potential presence of visual artefacts.

When the current picture is a BLA picture, the value of broken_link_flag shall be equal to 1.

Regardless of the value of the broken_link_flag, pictures in the current layer subsequent to the specified recovery point in output order are specified to be correct or approximately correct in content.

        - Structure of pictures information SEI message semantics for multi-layer extensions

The specifications of clause D.3.18 apply by replacing the first paragraph specifying the persistence of the SEI message with the following:

The structure of pictures information SEI message provides information for a list of entries, some of which correspond to the target picture set consists of a series of pictures starting from the current picture until the last picture in decoding order in the current layer in the CLVS or the last picture in decoding order in the current POC resetting period, whichever is earlier.

        - Decoding unit information SEI message semantics for multi-layer extensions

The specifications of clause D.3.21 apply with the following modifications and additions:

When the decoding unit information SEI message is directly contained in a scalable nesting SEI message within an SEI NAL unit with a nuh_layer_id value targetNuhLayerId greater than 0, the semantics of clause D.3.21 apply to the output bitstream of the independent non-base layer rewriting process of clause A.10.2 with the input variables assignedBaseLayerId equal to nestingLayerIdList[ i ][ 0 ] and tIdTarget equal to MaxTemporalId[ i ] for each value of i in the range of 0 to nestingNumOps − 1, inclusive.

When the decoding unit information SEI message is contained in a bitstream partition nesting SEI message, the following applies:

–	The variables olsIdx, psIdx, and bspIdx are set equal to sei_ols_idx, sei_partitioning_scheme_idx, and bsp_idx, respectively, of the bitstream partition nesting SEI message containing this buffering period SEI message, and the bspIdx-th bitstream partition of the psIdx-th partitioning scheme of the olsIdx-th OLS is referred to as the current bitstream partition.

–	The variable maxTId is set equal to MaxTemporalId[ 0 ] for the scalable nesting SEI message containing the bitstream partition nesting SEI message containing this buffering period SEI message.

–	The variable hrdParamIdx is set equal to the value of bsp_hrd_idx[ olsIdx ][ psIdx ][ maxTId ][ i ][ bspIdx ] for any value of i in the range of 0 to BspSchedCnt[ olsIdx ][ psIdx ][ maxTId ] – 1, inclusive.

–	The following applies for the syntax and semantics of the decoding unit information SEI message:

–	The syntax elements sub_pic_hrd_params_present_flag, sub_pic_cpb_params_in_pic_timing_sei_flag, and dpb_output_delay_du_length_minus1, and the variable CpbDpbDelaysPresentFlag are found in or derived from syntax elements in the hrdParamIdx-th hrd_parameters( ) syntax structure.

–	It is a requirement of bitstream conformance that the values of sub_pic_hrd_params_present_flag, sub_pic_cpb_params_in_pic_timing_sei_flag, and dpb_output_delay_du_length_minus1, and CpbDpbDelaysPresentFlag derived using a particular value of i for the derivation of hrdParamIdx shall be the same as the value of the respective syntax elements or variable derived using any other allowed value of i.

The presence of decoding unit information SEI messages for the current bitstream partition is specified as follows on the basis of the syntax elements sub_pic_hrd_params_present_flag and sub_pic_cpb_params_in_pic_timing_sei_flag and the variable CpbDpbDelaysPresentFlag that are found or derived as specified above:

–	If CpbDpbDelaysPresentFlag is equal to 1, sub_pic_hrd_params_present_flag is equal to 1, and sub_pic_cpb_params_in_pic_timing_sei_flag is equal to 0, one or more decoding unit information SEI messages applicable to the current bitstream partition shall be associated with each decoding unit of the current bitstream partition in the CVS.

–	Otherwise, if CpbDpbDelaysPresentFlag is equal to 1, sub_pic_hrd_params_present_flag is equal to 1, and sub_pic_cpb_params_in_pic_timing_sei_flag is equal to 1, one or more decoding unit information SEI messages applicable to the current bitstream partition may or may not be associated with each decoding unit of the current bitstream partition in the CVS.

–	Otherwise (CpbDpbDelaysPresentFlag is equal to 0 or sub_pic_hrd_params_present_flag is equal to 0), in the CVS there shall be no decoding unit of the current bitstream partition that is associated with a decoding unit information SEI message applicable to the current bitstream partition.

The set of NAL units associated with a decoding unit information SEI message contained in a bitstream partition nesting SEI message consists, in decoding order, of the SEI NAL unit containing the decoding unit information SEI message and all subsequent NAL units in the partition unit up to but not including any subsequent SEI NAL unit containing a decoding unit information SEI message that is contained in a bitstream partition nesting SEI message applying to the current bitstream partition and that has a different value of decoding_unit_idx.

It is a requirement of bitstream conformance that all decoding unit information SEI messages that are contained in a bitstream partition nesting SEI message, are associated with the same access unit, apply to the current bitstream partition, and have dpb_output_du_delay_present_flag equal to 1 shall have the same value of pic_spt_dpb_output_du_delay.

        - Scalable nesting SEI message semantics for multi-layer extensions

The specifications of clause D.3.23 apply with the following modifications:

It is a requirement of bitstream conformance that the following restrictions apply on nesting of SEI messages:

–	An SEI message that has payloadType equal to 129 (active parameter sets), 132 (decoded picture hash), 133 (scalable nesting), 160 (layers not present), 163 (bitstream partition initial arrival time), 164 (sub-bitstream property), or 166 (overlay information) shall not be directly contained in a scalable nesting SEI message.

–	When a scalable nesting SEI message contains a buffering period SEI message, a picture timing SEI message, or a decoding unit information SEI message, the scalable nesting SEI message shall not contain any other SEI message with payloadType not equal to 0 (buffering period), 1 (picture timing), or 130 (decoding unit information).

–	When present, an SEI message that has payloadType equal to 162 (bitstream partition nesting) shall be contained within a scalable nesting SEI message.

–	When a scalable nesting SEI message contains a bitstream partition nesting SEI message, the scalable nesting SEI message shall not contain any other SEI message.

It is a requirement of bitstream conformance that the following restrictions apply on the value of bitstream_subset_flag:

–	When the scalable nesting SEI message contains a buffering period SEI message, a picture timing SEI message, a decoding unit information SEI message, or a bitstream partition nesting SEI message, bitstream_subset_flag shall be equal to 1.

–	When the scalable nesting SEI message contains an SEI message that has payloadType equal to any value among SingleLayerSeiList, 161, 165, 167, or 168, bitstream_subset_flag shall be equal to 0.

When nesting_op_idx[ i ] is present, the list nestingLayerIdList[ i ] is set equal to LayerSetLayerIdList[ nesting_op_idx[ i ] ] derived from the active VPS.

When a scalable nesting SEI message contains a bitstream partition nesting SEI message, the following applies:

–	nesting_op_flag shall be equal to 1, default_op_flag shall be equal to 0, nesting_num_ops_minus1 shall be equal to 0, and nesting_op_idx[ 0 ] shall not be equal to 0.

–	The nuh_layer_id of the SEI NAL unit containing the scalable nesting SEI message shall be equal to the highest value within the list nestingLayerIdList[ 0 ].

When a scalable nesting SEI message directly contains a buffering period SEI message, a picture timing SEI message, or a decoding unit information SEI message, and the nuh_layer_id value of the SEI NAL unit containing the scalable nesting SEI messge is greater than 0, it is a requirement of bitstream conformance that for any value of i in the range of default_op_flag to nesting_num_ops_minus1, inclusive, the nesting_op_idx[ i ]-th OLS consists of only the layer with nuh_layer_id equal to nestingLayerIdList[ i ][ 0 ], the profile of that layer is a profile specified in Annex A, and the value of general_inbld_flag (when MaxTemporalId[ i ] is equal to vps_max_sub_layers_minus1) or the value of sub_layer_inbld_flag[ tidTarget ] (when MaxTemporalId[ i ] is less than vps_max_sub_layers_minus1) in the profile_tier_level( ) syntax structure associated with that layer is equal to 1.

NOTE – In other words, buffering period, picture timing, or decoding unit inofrmaiton SEI messages that are directly nested in an SEI NAL unit with nuh_layer_id greater than 0 can only be present for rewritable independent non-base layers.

        - Region refresh information SEI message semantics for multi-layer extensions

The region refresh information SEI message indicates whether the slice segments that the current SEI message applies to belong to a refreshed region of the current picture.

The variable targetLayerIdList is derived as follows:

–	If the region refresh information SEI message applies to the current layer and all the reference layers, targetLayerIdList contains the nuh_layer_id of the current layer and all the reference layers.

–	Otherwise, targetLayerIdList contains the nuh_layer_id of the current layer.

The region refresh SEI message is associated with a recovery point SEI message that applies to targetLayerIdList.

A picture that is not an IRAP picture, that belongs to a layer, and that is contained in an access unit containing a recovery point SEI message where the recovery point SEI message applies to that layer is referred to as a gradual decoding refresh (GDR) picture, and the access unit containing the picture is referred to as a GDR access unit. The access unit corresponding to the indicated recovery point picture is referred to as the recovery point access unit.

If there is a picture picB in the current layer that follows the GDR picture picA in the current layer in decoding order in the CVS and PicOrderCnt( picB ) is equal to PicOrderCnt( picA ) plus the value of recovery_poc_cnt in the recovery point SEI message, where PicOrderCnt( picA ) and PicOrderCnt( picB ) are the PicOrderCntVal values of picA and picB, respectively, immediately after the invocation of the decoding process for picture order count for picB, let the variable lastPicInSet be the recovery point picture. Otherwise, let lastPicInSet be the picture in targetLayerIdList that immediately precedes the recovery point picture in output order. The picture lastPicInSet shall not precede the GDR access unit in decoding order.

Let gdrPicSet be the set of pictures in targetLayerIdList starting from a GDR access unit to the access unit containing lastPicInSet, inclusive, in output order. When the decoding process for the current layer is started from a GDR access unit, the refreshed region in each picture of the gdrPicSet is indicated to be the region of the picture that is correct or approximately correct in content, and, when lastPicInSet is contained in the recovery point access unit, the refreshed region in lastPicInSet covers the entire picture.

The slice segments of the current picture to which a region refresh information SEI message applies consist of all slice segments within the picture that follow the SEI NAL unit containing the region refresh information SEI message and precede the next SEI NAL unit, in decoding order, containing a region refresh information SEI message (if any) that has the same targetLayerIdList as the current SEI message. These slice segments are referred to as the slice segments associated with the region refresh information SEI message.

Let gdrAuSet be the set of access units corresponding to gdrPicSet. A gdrAuSet and the corresponding gdrPicSet are referred to as being associated with the recovery point SEI message contained in the GDR access unit.

Region refresh information SEI messages shall not be present in an access unit unless the access unit is included in a gdrAuSet associated with a recovery point SEI message. When any picture that is included in a gdrPicSet is associated with one or more region refresh information SEI messages, all pictures in the gdrPicSet shall be associated with one or more region refresh information SEI messages.

refreshed_region_flag equal to 1 indicates that the slice segments associated with the current SEI message belong to the refreshed region in the current picture. refreshed_region_flag equal to 0 indicates that the slice segments associated with the current SEI message may not belong to the refreshed region in the current picture.

When one or more region refresh information SEI messages are associated with a picture belonging to gdrPicSet and the first slice segment of the picture in decoding order does not have an associated region refresh information SEI message, the value of refreshed_region_flag for the slice segments of the picture that precede the first region refresh information SEI message is inferred to be equal to 0.

When lastPicInSet is the recovery point picture, and any region refresh SEI message is associated with the recovery point access unit, the first slice segment of the picture in decoding order shall have an associated region refresh SEI message, and the value of refreshed_region_flag shall be equal to 1 in all region refresh SEI messages associated with the picture.

When one or more region refresh information SEI messages are associated with a picture, the refreshed region in the picture is specified as the set of CTUs in all slice segments of the picture that are associated with region refresh information SEI messages that have refreshed_region_flag equal to 1. Other slice segments belong to the non-refreshed region of the picture.

It is a requirement of bitstream conformance that when a dependent slice segment belongs to the refreshed region, the preceding slice segment in decoding order shall also belong to the refreshed region.

Let gdrRefreshedSliceSegmentSet be the set of all slice segments that belong to the refreshed regions in the gdrPicSet. The variable upSwitchingRefreshedSliceSegmentSet is derived as follows:

–	If targetLayerIdList contains only one non-zero nuh_layer_id, upSwitchingRefreshedSliceSegmentSet is defined as the set inclusive of the following:

–	all slice segments of all pictures of the reference layers that precede, in decoding order, the current picture and that may be used for reference by the current picture or subsequent pictures of the reference layers.

–	all slice segments of all pictures of the reference layers that succeed, in decoding order, the current picture and that belong to gdrAuSet.

–	Otherwise, upSwitchingRefreshedSliceSegmentSet is defined as an empty set.

When a gdrPicSet contains one or more pictures associated with region refresh information SEI messages, it is a requirement of bitstream conformance that the following constraints all apply:

–	For each layer in targetLayerIdList, the refreshed region in the first picture, in decoding order, that belongs to the layer and that is included in gdrPicSet that contains any refreshed region shall contain only coding units that are coded in an intra coding mode or inter-layer prediction from slice segments belonging to the union of gdrRefreshedSliceSegmentSet and upSwitchingRefreshedSliceSegmentSet.

–	For each picture included in the gdrPicSet, the syntax elements in gdrRefreshedSliceSegmentSet shall be constrained such that no samples or motion vector values outside of the union of gdrRefreshedSliceSegmentSet and upSwitchingRefreshedSliceSegmentSet are used for inter prediction or inter-layer prediction in the decoding process of any samples within gdrRefreshedSliceSegmentSet.

–	For any picture that follows the picture lastPicInSet in output order, the syntax elements in the slice segments of the picture shall be constrained such that no samples or motion vector values outside of the union of gdrRefreshedSliceSegmentSet and upSwitchingRefreshedSliceSegmentSet are used for inter prediction or inter-layer prediction in the decoding process of the picture other than those of the other pictures that follow the picture lastPicInSet in output order.

      - Layers not present SEI message semantics

The layers not present SEI message provides a mechanism for signalling that VCL NAL units of particular layers indicated by the VPS are not present in a particular set of access units.

The target access units are defined as the set of access units starting from the access unit containing the layers not present SEI message up to but not including the next access unit, in decoding order, that contains a layers not present change SEI message or the end of the CVS, whichever is earlier in decoding order.

When present, the layers not present SEI message applies to the target access units.

A layers not present SEI message shall not be included in an SEI NAL unit with TemporalId greater than 0.

lnp_sei_active_vps_id identifies the active VPS of the CVS containing the layers not present SEI message. The value of lnp_sei_active_vps_id shall be equal to the value of vps_video_parameter_set_id of the active VPS for the VCL NAL units of the access unit containing the SEI message.

layer_not_present_flag[ i ] equal to 1 indicates that there are no VCL NAL units with nuh_layer_id equal to layer_id_in_nuh[ i ] present in the target access units. layer_not_present_flag[ i ] equal to 0 indicates that there may or may not be VCL NAL units with nuh_layer_id equal to layer_id_in_nuh[ i ] present in the target access units.

When layer_not_present_flag[ i ] is equal to 1 and i is less than MaxLayersMinus1, layer_not_present_flag[ LayerIdxInVps[ IdPredictedLayer[ layer_id_in_nuh[ i ] ][ j ] ] ] shall be equal to 1 for all values of j in the range of 0 to NumPredictedLayers[ layer_id_in_nuh[ i ] ] − 1, inclusive.

      - Inter-layer constrained tile sets SEI message semantics

The scope of the inter-layer constrained tile sets SEI message is a complete CLVS of the layer with nuh_layer_id equal to targetLayerId. When an inter-layer constrained tile sets SEI message is present for any coded picture of a CLVS and the first coded picture of the CLVS in decoding order is an IRAP picture, the inter-layer constrained tile sets SEI message shall be present for the first coded picture of the CLVS in decoding order and may also be present for other coded pictures of the CLVS.

The inter-layer constrained tile sets SEI message shall not be present for the layer with nuh_layer_id equal to targetLayerId when tiles_enabled_flag is equal to 0 for any PPS that is active for the pictures of the CLVS of the layer with nuh_layer_id equal to targetLayerId.

The inter-layer constrained tile sets SEI message shall not be present for the layer with nuh_layer_id equal to targetLayerId unless every PPS that is active for the pictures of the CLVS of the layer with nuh_layer_id equal to targetLayerId has tile_boundaries_aligned_flag equal to 1 or fulfills the conditions that would be indicated by tile_boundaries_aligned_flag being equal to 1.

An identified tile set is defined as a set of tiles within a picture with nuh_layer_id equal to targetLayerId that contains the tiles specified below on the basis of the values of the syntax elements contained in the inter-layer constrained tile sets SEI message.

An associated reference tile set for the iPred-th identified tile set with ilcts_id[ iPred ] equal to ilctsId, if any, is defined as the identified tile set present in the same access unit as the iPred-th identified tile set and associated with ilcts_id[ iRef ] equal to ilctsId and nuh_layer_id equal to IdDirectRefLayer[ targetLayerId ][ j ] for any value of j in the range of 0 to NumDirectRefLayers[ targetLayerId ] − 1, inclusive.

The presence of the inter-layer constrained tile sets SEI message indicates that the inter-layer prediction process is constrained such that no sample value outside each associated reference tile set, and no sample value at a fractional sample position that is derived using one or more sample values outside each associated reference tile set, is used for inter-layer prediction of any sample within the identified tile set.

NOTE 1 – When loop filtering and resampling filter is applied across tile boundaries, inter-layer prediction of any samples within an identified tile set that refers to samples within 8 samples from an associated reference tile set boundary that is not also a picture boundary may result in propagation of mismatch error. An encoder can avoid such potential error propagation by avoiding the use of motion vectors that cause such references.

When more than one inter-layer constrained tile sets SEI message is present for a CLVS, they shall contain identical content.

The number of inter-layer constrained tile sets SEI messages for the same CLVS in each access unit shall not exceed 5.

il_all_tiles_exact_sample_value_match_flag equal to equal to 1 indicates that, within the CLVS, when the coding tree blocks that are outside of any tile in any identified tile set specified in this inter-layer constrained tile sets SEI message are not decoded and the boundaries of the tile is treated as picture boundaries for purposes of the decoding process, the value of each sample in the tile would be exactly the same as the value of the sample that would be obtained when all the coding tree blocks of all pictures in the CLVS are decoded. il_all_tiles_exact_sample_value_match_flag equal to 0 indicates that, within the CLVS, when the coding tree blocks that are outside of any tile in any identified tile set specified in this inter-layer constrained tile sets SEI message are not decoded and the boundaries of the tile is treated as picture boundaries for purposes of the decoding process, the value of each sample in the tile may or may not be exactly the same as the value of the same sample when all the coding tree blocks of all pictures in the CLVS are decoded.

il_one_tile_per_tile_set_flag equal to 1 indicates that each identified tile set contains one tile, and il_num_sets_in_message_minus1 is not present.

It is a requirement of bitstream conformance that when il_one_tile_per_tile_set_flag is equal to 1, the value of

( num_tile_columns_minus1 + 1 ) * ( num_tile_rows_minus1 + 1 ) shall be less than or equal to 256.

When il_one_tile_per_tile_set_flag is equal to 0, the identified tile sets are signalled explicitly.

il_num_sets_in_message_minus1 plus 1 specifies the number of the identified tile sets in the SEI message. The value of il_num_sets_in_message_minus1 shall be in the range of 0 to 255, inclusive.

skipped_tile_set_present_flag equal to 1 indicates that, within the CLVS, the il_num_sets_in_message_minus1-th identified tile set consists of those remaining tiles that are not included in any earlier identified tile sets in the same message and all the prediction blocks that are inside the il_num_sets_in_message_minus1-th identified tile set are inter-layer predicted from inter-layer reference pictures with nuh_layer_id equal to IdDirectRefLayer[ targetLayerId ][ NumDirectRefLayers[ targetLayerId ] − 1 ] and no residual_coding( ) syntax structure is present in any transform unit of the il_num_sets_in_message_minus1-th identified tile set. skipped_tile_set_present_flag equal to 0 does not indicate a bitstream constraint within the CLVS. When not present, the value of skipped_tile_set_present_flag is inferred to be equal to 0.

ilcts_id[ i ] contains an identifying number that may be used to identify the purpose of the i-th identified tile set (for example, to identify an area to be extracted from the coded video sequence for a particular purpose). The value of ilcts_id[ i ] shall be in the range of 0 to 232 − 2, inclusive.

Values of ilcts_id[ i ] from 0 to 255, inclusive, and from 512 to 231 – 1, inclusive, may be used as determined by the application. If il_one_tile_per_tile_set_flag is equal to 1, values of ilcts_id[ i ] from 256 to 511, inclusive are used for the inferred ilcts_id[ i ] values as specified above. Otherwise, values of ilcts_id[ i ] from 256 to 511, inclusive, are reserved for future use by ITU-T | ISO/IEC. Values of ilcts_id[ i ] from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders encountering an indicated value of ilcts_id[ i ] in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore (remove from the bitstream and discard) it.

When ilcts_id[ i ] is not present for i in the range of 0 to ( num_tile_columns_minus1 + 1 ) * ( num_tile_rows_minus1 + 1 ) − 1, inclusive, due to il_one_tile_per_tile_set_flag being equal to 1, it is inferred to be equal to 256 + i.

il_num_tile_rects_in_set_minus1[ i ] plus 1 specifies the number of rectangular regions of tiles in the i-th identified tile set. The value of il_num_tile_rects_in_set_minus1[ i ] shall be in the range of 0 to ( num_tile_columns_minus1 + 1 ) * ( num_tile_rows_minus1 + 1 ) − 1, inclusive.

il_top_left_tile_index[ i ][ j ] and il_bottom_right_tile_index[ i ][ j ] identify the tile position of the top-left tile and the tile position of the bottom-right tile in a rectangular region of the i-th identified tile set, respectively, in tile raster scan order.

ilc_idc[ i ] equal to 1 indicates that no samples outside of any associated reference tile set and no samples at a fractional sample position that is derived using one or more samples outside of any associated reference tile set are used for inter-layer prediction of any sample within the i-th identified tile set. ilc_idc[ i ] equal to 2 indicates that no prediction block in the i-th identified tile set is predicted from an inter-layer reference picture. ilc_idc[ i ] equal to 0 indicates that the inter-layer prediction process may or may not be constrained for the prediction block in the i-th identified tile set. The value of ilc_idc[ i ] equal to 3 is reserved.

il_exact_sample_value_match_flag[ i ] equal to 1 indicates that, within the CLVS, when the coding tree blocks that do not belong to the i-th identified tile set are not decoded and the boundaries of the i-th identified tile set are treated as picture boundaries for the purposes of the decoding process, the value of each sample in the i-th identified tile set would be exactly the same as the value of the sample that would be obtained when all the coding tree blocks of all pictures in the CLVS are decoded. il_exact_sample_value_match_flag[ i ] equal to 0 indicates that, within the CLVS, when the coding tree blocks that are outside of the i-th identified tile set are not decoded and the boundaries of the i-th identified tile set are treated as picture boundaries for the purposes of the decoding process, the value of each sample in the i-th identified tile set may or may not be exactly the same as the value of the same sample when all the coding tree blocks of all pictures in the CLVS are decoded.

NOTE 2 – It is feasible to use il_exact_sample_value_match_flag equal to 1 when using certain combinations of loop_filter_across_tiles_enabled_flag, pps_loop_filter_across_slices_enabled_flag, pps_deblocking_filter_disabled_flag, slice_loop_filter_across_slices_enabled_flag, slice_deblocking_filter_disabled_flag, sample_adaptive_offset_enabled_flag, slice_sao_luma_flag, and slice_sao_chroma_flag.

all_tiles_ilc_idc equal to 1 indicates that, for each identified tile set within the CLVS, no sample value outside of each associated reference tile set and no sample value at a fractional sample position that is derived using one or more samples outside of each associated reference tile set is used for inter-layer prediction of any sample within the identified tile set. all_tiles_ilc_idc equal to 2 indicates that, within the CLVS, no prediction block in each identified tile set is predicted from an inter-layer reference picture. all_tiles_ilc_idc equal to 0 indicates that, within the CLVS, the inter-layer prediction process may or may not be constrained for the identified tile sets. The value of all_tiles_ilc_idc equal to 3 is reserved. When all_tiles_ilc_idc is not present, it is inferred to be equal to 0.

      - Bitstream partition nesting SEI message semantics

The bitstream partition nesting SEI message provides a mechanism to associate SEI messages with a bitstream partition of a partitioning scheme of an OLS.

NOTE – The bitstream partition nesting SEI message must be contained within a scalable nesting SEI message. Constraints on the scalable nesting SEI message containing a bitstream partition nesting SEI message are specified in clause A.14.3.2.7.

A bitstream partition nesting SEI message contains one or more SEI messages.

sei_ols_idx specifies the index of the OLS to which the contained SEI messages apply. The value of sei_ols_idx shall be in the range of 0 to NumOutputLayerSets − 1, inclusive.

It is a requirement of bitstream conformance that OlsIdxToLsIdx[ sei_ols_idx ] shall be equal to nesting_op_idx[ 0 ] of the scalable nesting SEI message that contains the bitstream partition nesting SEI message.

sei_partitioning_scheme_idx specifies the index of the partitioning scheme to which the contained SEI messages apply. The value of sei_partitioning_scheme_idx shall be in the range of 0 to num_signalled_partitioning_schemes[ sei_ols_idx ], inclusive.

bsp_idx specifies the index of the bitstream partition to which the contained SEI messages apply. The value of bsp_idx shall be in the range of 0 to num_partitions_in_scheme_minus1[ sei_ols_idx ][ sei_partitioning_scheme_idx ], inclusive.

bsp_nesting_zero_bit shall be equal to 0.

num_seis_in_bsp_minus1 plus 1 specifies the number of sei_message( ) syntax structures contained in the bsp_nesting( ) syntax structure. The value of num_seis_in_bsp_minus1 shall be in the range of 0 to 63, inclusive.

      - Bitstream partition initial arrival time SEI message semantics

The bitstream partition initial arrival time SEI message specifies the initial arrival times to be used in the bitstream-partition-specific CPB operation.

When present, this SEI message shall be contained within a bitstream partition nesting SEI message that is contained in a scalable nesting SEI message, and the same bitstream partition nesting SEI message shall also contain a buffering period SEI message.

The following applies for each value of i in the range of 0 to BspSchedCnt[ olsIdx ][ psIdx ][ MaxTemporalId[ 0 ] ] − 1, inclusive:

–	The variable hrdParamIdx[ i ] is set equal to the value of bsp_hrd_idx[ olsIdx ][ psIdx ][ maxTemporalId[ 0 ] ][ i ][ bspIdx ], where olsIdx, psIdx, and bspIdx are equal to sei_ols_idx, sei_partitioning_scheme_idx, and bsp_idx, respectively, of the bitstream partition nesting SEI message containing this bitstream partition initial arrival time SEI message, and maxTemporalId[ 0 ] is the value of MaxTemporalId[ 0 ] for the scalable nesting SEI message containing the bitstream partition nesting SEI message containing this bitstream partition initial arrival time SEI message.

–	The variable initialCpbRemovalDelayLength[ i ] is set equal to initial_cpb_removal_delay_length_minus1 + 1, where initial_cpb_removal_delay_length_minus1 is found in the hrdParamIdx[ i ]-th hrd_parameters( ) syntax structure in the active VPS.

–	The variable nalHrdParamsPresent[ i ] is set equal to the value of nal_hrd_parameters_present_flag in the hrdParamIdx[ i ]-th hrd_parameters( ) syntax structure in the active VPS.

–	The variable vclHrdParamsPresent[ i ] is set equal to the value of vcl_hrd_parameters_present_flag in the hrdParamIdx[ i ]-th hrd_parameters( ) syntax structure in the active VPS.

It is a requirement of bitstream conformance that the value of nalHrdParamsPresent[ i ] shall be the same for all values of i in the range of 0 to BspSchedCnt[ olsIdx ][ psIdx ][ MaxTemporalId[ 0 ] ] − 1, inclusive.

It is a requirement of bitstream conformance that the value of vclHrdParamsPresent[ i ] shall be the same for all values of i in the range of 0 to BspSchedCnt[ olsIdx ][ psIdx ][ MaxTemporalId[ 0 ] ] − 1, inclusive.

The variable nalInitialArrivalDelayPresent is set equal to nalHrdParamsPresent[ i ] of any value of i in the range of 0 to BspSchedCnt[ olsIdx ][ psIdx ][ MaxTemporalId[ 0 ] ] − 1, inclusive.

The variable vclInitialArrivalDelayPresent is set equal to vclHrdParamsPresent[ i ] of any value of i in the range of 0 to BspSchedCnt[ olsIdx ][ psIdx ][ MaxTemporalId[ 0 ] ] − 1, inclusive.

nal_initial_arrival_delay[ i ] specifies the initial arrival time for the i-th delivery schedule of the bitstream partition to which this SEI message applies, when NAL HRD parameters are in use. The length, in bits, of the nal_initial_arrival_delay[ i ] syntax element is equal to initialCpbRemovalDelayLength[ i ].

vcl_initial_arrival_delay[ i ] specifies the initial arrival time for the i-th delivery schedule of the bitstream partition to which this SEI message applies, when VCL HRD parameters are in use. The length, in bits, of the vcl_initial_arrival_delay[ i ] syntax element is equal to initialCpbRemovalDelayLength[ i ].

      - Sub-bitstream property SEI message semantics

The sub-bitstream property SEI message, when present, provides the bit rate information for a sub-bitstream created by discarding those pictures in the layers that do not belong to the output layers of the OLSs specified by the active VPS and that do not affect the decoding of the output layers.

When present, the sub-bitstream property SEI message shall be associated with an initial IRAP access unit, and the information provided by the SEI messages applies to the bitstream corresponding to the CVS containing the associated initial IRAP access unit.

sb_property_active_vps_id identifies the active VPS. The value of sb_property_active_vps_id shall be equal to the value of vps_video_parameter_set_id of the active VPS referred to by the VCL NAL units of the associated access unit.

num_additional_sub_streams_minus1 plus 1 specifies the number of the sub-bitstreams for which the bit rate information may be provided by this SEI message. The value of num_additional_sub_streams_minus1 shall be in the range of 0 to 210 − 1, inclusive.

sub_bitstream_mode[ i ] specifies how the i-th sub-bitstream is generated. The value of sub_bitstream_mode[ i ] shall be equal to 0 or 1, inclusive. The values 2 and 3 are reserved for future use by ITU-T and ISO/IEC. When sub_bitstream_mode[ i ] is the greater than 1, decoders shall ignore the syntax elements ols_idx_to_vps[ i ], highest_sublayer_id[ i ], avg_sb_property_bit_rate[ i ], and max_sb_property_bit_rate[ i ].

When sub_bitstream_mode[ i ] is equal to 0, the i-th sub-bitstream is generated as specified by the following steps:

–	Let lsIdx be equal to OlsIdxToLsIdx[ ols_idx_to_vps[ i ] ].

–	A sub-bitstream subBitstream[ i ] is first created as follows:

–	If lsIdx is less than or equal to vps_num_layer_sets_minus1 and vps_base_layer_internal_flag is equal to 1, the sub-bitstream extraction process as specified in clause 10 is invoked with the bitstream corresponding to the CVS containing the sub-bitstream property SEI message, highest_sublayer_id[ i ], and LayerSetLayerIdList[ lsIdx ] as inputs, and the output is assigned to subBitstream[ i ].

–	Otherwise, if lsIdx is less than or equal to vps_num_layer_sets_minus1 and vps_base_layer_internal_flag is equal to 0, the sub-bitstream extraction process as specified in clause A.10.1 is invoked with the bitstream corresponding to the CVS containing the sub-bitstream property SEI message, highest_sublayer_id[ i ], and LayerSetLayerIdList[ lsIdx ] as inputs, and the output is assigned to subBitstream[ i ].

–	Otherwise, the sub-bitstream extraction process as specified in clause A.10.3 is invoked with the bitstream corresponding to the CVS containing the sub-bitstream property SEI message, highest_sublayer_id[ i ], and LayerSetLayerIdList[ lsIdx ] as inputs, and the output is assigned to subBitstream[ i ].

–	Remove all NAL units for which the nuh_layer_id is not included in TargetOptLayerIdList and either of the following conditions is true:

–	The value of nal_unit_type is not in the range of BLA_W_LP to RSV_IRAP_VCL23, inclusive, and max_tid_il_ref_pics_plus1[ LayerIdxInVps[ nuh_layer_id ] ][ LayerIdxInVps[ layerId ] ] is equal to 0 for layerId values included in TargetOptLayerIdList.

–	TemporalId is greater than the maximum value of max_tid_il_ref_pics_plus1[ LayerIdxInVps[ nuh_layer_id ] ][ LayerIdxInVps[ layerId ] ] − 1 for all layerId values included in TargetOptLayerIdList.

When sub_bitstream_mode[ i ] is equal to 1, the i-th sub-bitstream is generated as specified by the above steps followed by:

–	Remove all NAL units with nuh_layer_id not among the values included in TargetOptLayerIdList and with discardable_flag equal to 1.

ols_idx_to_vps[ i ] specifies the index of the OLS corresponding to the i-th sub-bitstream. The value of ols_idx_to_vps[ i ] shall be in the range of 0 to NumOutputLayerSets − 1, inclusive.

highest_sublayer_id[ i ] specifies the highest TemporalId of access units in the i-th sub-bitstream.

avg_sb_property_bit_rate[ i ] indicates the average bit rate of the i-th sub-bitstream, in bits per second. The value is given by BitRateBPS( avg_sb_property_bit_rate[ i ] ) with the function BitRateBPS( ) being specified as follows:

BitRateBPS( x ) = ( x & ( 214 − 1 ) ) * 10( 2 + ( x  >>  14 ) )		(FF-98)

The average bit rate is derived according to the access unit removal time specified in clause A.13. In the following, bTotal is the number of bits in all NAL units of the i-th sub-bitstream, t1 is the removal time (in seconds) of the first access unit to which the VPS applies, and t2 is the removal time (in seconds) of the last access unit (in decoding order) to which the VPS applies. With x specifying the value of avg_sb_property_bit_rate[ i ], the following applies:

–	If t1 is not equal to t2, the following condition shall be true:

( x & ( 214 − 1 ) )  = =  Round( bTotal ÷ ( ( t2 − t1 ) * 10( 2 + ( x  >>  14 ) ) ) )	(FF-99)

–	Otherwise (t1 is equal to t2), the following condition shall be true:

( x & ( 214 − 1 ) )  = =  0		(FF-100)

max_sb_property_bit_rate[ i ] indicates an upper bound for the bit rate of the i-th sub-bitstream in any one-second time window of access unit removal time as specified in clause A.13. The upper bound for the bit rate in bits per second is given by BitRateBPS( max_sb_property_bit_rate[ i ] ). The bit rate values are derived according to the access unit removal time specified in clause A.13. In the following, t1 is any point in time (in seconds), t2 is set equal to t1 + 1 ÷ 100, and bTotal is the number of bits in all NAL units of access units with a removal time greater than or equal to t1 and less than t2. With x specifying the value of max_sb_property_bit_rate[ i ], the following condition shall be obeyed for all values of t1:

( x & ( 214 − 1 ) )  >=  bTotal ÷ ( ( t2 − t1 ) * 10( 2 + ( x  >>  14 ) ) )	(FF-101)

      - Alpha channel information SEI message semantics

The alpha channel information SEI message provides information about alpha channel sample values and post-processing applied to the decoded alpha planes coded in auxiliary pictures of type AUX_ALPHA, and one or more associated primary pictures.

For an auxiliary picture with nuh_layer_id equal to nuhLayerIdA and AuxId[ nuhLayerIdA ] equal to AUX_ALPHA, an associated primary picture, if any, is a picture in the same access unit having AuxId[ nuhLayerIdB ] equal to 0 such that ScalabilityId[ LayerIdxInVps[ nuhLayerIdA ] ][ j ] is equal to ScalabilityId[ LayerIdxInVps[ nuhLayerIdB ] ][ j ] for all values of j in the range of 0 to 2, inclusive, and 4 to 15, inclusive.

When an access unit contains an auxiliary picture picA with nuh_layer_id equal to nuhLayerIdA and AuxId[ nuhLayerIdA ] equal to AUX_ALPHA, the alpha channel sample values of picA persist in output order until one or more of the following conditions are true:

–	The next picture, in output order, with nuh_layer_id equal to nuhLayerIdA.

–	A CLVS containing the auxiliary picture picA ends.

–	The bitstream ends.

–	A CLVS of any associated primary layer of the auxiliary picture layer with nuh_layer_id equal to nuhLayerIdA ends.

The following semantics apply separately to each nuh_layer_id targetLayerId among the nuh_layer_id values to which the alpha channel information SEI message applies.

alpha_channel_cancel_flag equal to 1 indicates that the alpha channel information SEI message cancels the persistence of any previous alpha channel information SEI message in output order that applies to the current layer. alpha_channel_cancel_flag equal to 0 indicates that alpha channel information follows.

Let currPic be the picture which the alpha channel information SEI message is associated with. The semantics of alpha channel information SEI message persist for the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB with nuh_layer_id equal to targetLayerId in an access unit containing an alpha channel information SEI message with nuh_layer_id equal to targetLayerId is output having PicOrderCnt( picB ) greater than PicOrderCnt( currPic ) and alpha_channel_cancel_flag in the alpha channel information SEI message is equal to 1, where PicOrderCnt( picB ) and PicOrderCnt( currPic ) are the PicOrderCntVal values of picB and currPic, respectively, immediately after the invocation of the decoding process for picture order count for picB.

alpha_channel_use_idc equal to 0 indicates that for alpha blending purposes the decoded samples of the associated primary picture should be multiplied by the interpretation sample values of the auxiliary coded picture in the display process after output from the decoding process. alpha_channel_use_idc equal to 1 indicates that for alpha blending purposes the decoded samples of the associated primary picture should not be multiplied by the interpretation sample values of the auxiliary coded picture in the display process after output from the decoding process. alpha_channel_use_idc equal to 2 indicates that the usage of the auxiliary picture is unspecified. Values greater than 2 for alpha_channel_use_idc are reserved for future use by ITU-T | ISO/IEC. When not present, the value of alpha_channel_use_idc is inferred to be equal to 2.

alpha_channel_bit_depth_minus8 plus 8 specifies the bit depth of the samples of the luma sample array of the auxiliary picture. alpha_channel_bit_depth_minus8 shall be in the range 0 to 7 inclusive. alpha_channel_bit_depth_minus8 shall be equal to bit_depth_luma_minus8 of the associated primary picture.

alpha_transparent_value specifies the interpretation sample value of an auxiliary coded picture luma sample for which the associated luma and chroma samples of the primary coded picture are considered transparent for purposes of alpha blending. The number of bits used for the representation of the alpha_transparent_value syntax element is alpha_channel_bit_depth_minus8 + 9.

alpha_opaque_value specifies the interpretation sample value of an auxiliary coded picture luma sample for which the associated luma and chroma samples of the primary coded picture are considered opaque for purposes of alpha blending. The number of bits used for the representation of the alpha_opaque_value syntax element is alpha_channel_bit_depth_minus8 + 9.

alpha_channel_incr_flag equal to 0 indicates that the interpretation sample value for each decoded auxiliary picture luma sample value is equal to the decoded auxiliary picture sample value for purposes of alpha blending. alpha_channel_incr_flag equal to 1 indicates that, for purposes of alpha blending, after decoding the auxiliary picture samples, any auxiliary picture luma sample value that is greater than Min( alpha_opaque_value, alpha_transparent_value ) should be increased by one to obtain the interpretation sample value for the auxiliary picture sample, and any auxiliary picture luma sample value that is less than or equal to Min( alpha_opaque_value, alpha_transparent_value ) should be used without alteration as the interpretation sample value for the decoded auxiliary picture sample value. When not present, the value of alpha_channel_incr_flag is inferred to be equal to 0.

alpha_channel_clip_flag equal to 0 indicates that no clipping operation is applied to obtain the interpretation sample values of the decoded auxiliary picture. alpha_channel_clip_flag equal to 1 indicates that the interpretation sample values of the decoded auxiliary picture are altered according to the clipping process described by the alpha_channel_clip_type_flag syntax element. When not present, the value of alpha_channel_clip_flag is inferred to be equal to 0.

alpha_channel_clip_type_flag equal to 0 indicates that, for purposes of alpha blending, after decoding the auxiliary picture samples, any auxiliary picture luma sample that is greater than ( alpha_opaque_value − alpha_transparent_value ) / 2 is set equal to alpha_opaque_value to obtain the interpretation sample value for the auxiliary picture luma sample, and any auxiliary picture luma sample that is less or equal than ( alpha_opaque_value − alpha_transparent_value ) / 2 is set equal to alpha_transparent_value to obtain the interpretation sample value for the auxiliary picture luma sample. alpha_channel_clip_type_flag equal to 1 indicates that, for purposes of alpha blending, after decoding the auxiliary picture samples, any auxiliary picture luma sample that is greater than alpha_opaque_value is set equal to alpha_opaque_value to obtain the interpretation sample value for the auxiliary picture luma sample, and any auxiliary picture luma sample that is less than or equal to alpha_transparent_value is set equal to alpha_transparent_value to obtain the interpretation sample value for the auxiliary picture luma sample.

NOTE – When both alpha_channel_incr_flag and alpha_channel_clip_flag are equal to one, the clipping operation specified by alpha_channel_clip_type_flag should be applied first followed by the alteration specified by alpha_channel_incr_flag to obtain the interpretation sample value for the auxiliary picture luma sample.

      - Overlay information SEI message semantics

The overlay information SEI message provides information about overlay pictures coded as auxiliary pictures. Overlay auxiliary pictures have nuh_layer_id equal to nuhLayerIdA and AuxId[ nuhLayerIdA ] in the range of 128 to 159, inclusive. Each overlay auxiliary picture layer is associated with one or more primary picture layers as specified below.

overlay_info_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous overlay information SEI message in output order that is associated with one or more primary picture layers to which this SEI applies. overlay_info_cancel_flag equal to 0 indicates that overlay information follows.

overlay_content_aux_id_minus128 plus 128 indicates the value of AuxId of auxiliary pictures containing overlay content. overlay_content_aux_id_minus128 shall be in the range of 0 to 31, inclusive.

overlay_label_aux_id_minus128 plus 128 indicates the value of AuxId of auxiliary pictures containing overlay label. overlay_label_aux_id_minus128 shall be in the range of 0 to 31, inclusive.

overlay_alpha_aux_id_minus128 plus 128 indicates the value of AuxId of auxiliary pictures containing overlay alpha. overlay_alpha_aux_id_minus128 shall be in the range of 0 to 31, inclusive.

overlay_element_label_value_length_minus8 plus 8 indicates the number of bits used for coding the overlay_element_label_min[ i ][ j ] and overlay_element_label_max[ i ][ j ] syntax elements.

num_overlays_minus1 plus 1 specifies the number of overlays described by the overlay information SEI message. num_overlays_minus1 shall be in the range of 0 to 15, inclusive.

overlay_idx[ i ] indicates the index of the i-th overlay. overlay_idx[ i ] shall be in the range of 0 to 255, inclusive.

language_overlay_present_flag[ i ] equal to 1 indicates that overlay_language[ i ] is present. language_overlay_present_flag[ i ] equal to 0 indicates that overlay_language[ i ] is not present and that the language of the overlay is unspecified.

overlay_content_layer_id[ i ] indicates the nuh_layer_id value of the NAL units of the overlay content of the i-th overlay. AuxId[ overlay_content_layer_id[ i ] ] shall be equal to overlay_content_aux_id_minus128 + 128 for all values of i in the range of 0 to num_overlays_minus1, inclusive.

The value of the variable pLid, which identifies the nuh_layer_id value of the primary picture which the i-th overlay is associated with, is derived as follows:

pLid = −1
for( j = 0; j < 63; j++ )
	if( ViewOrderIdx[ j ]  = =  ViewOrderIdx[ overlay_content_layer_id[ i ] ]  &&	(F-102)
		DependencyId[ j ]  = =  DependencyId[ overlay_content_layer_id[ i ] ]  &&  AuxId[ j ]  = =  0 )
		pLid = j

The value of pLid shall be in the range of 0 to 62, inclusive.

overlay_label_present_flag[ i ] equal to 1 specifies that overlay_label_layer_id[ i ] is present. overlay_label_present_flag[ i ] equal to 0 specifies that overlay_label_layer_id[ i ] is not present.

overlay_label_layer_id[ i ] indicates the nuh_layer_id value of NAL units in the overlay label of the i-th overlay. AuxId[ overlay_label_layer_id[ i ] ] shall be equal to overlay_label_aux_id_minus128 + 128 for all values of i in the range of 0 to num_overlays_minus1, inclusive.

overlay_alpha_present_flag[ i ] equal to 1 specifies that overlay_alpha_layer_id[ i ] is present. overlay_alpha_present_flag[ i ] equal to 0 specifies that overlay_alpha_layer_id[ i ] is not present.

overlay_alpha_layer_id[ i ] indicates the nuh_layer_id value of NAL units in the overlay alpha of the i-th overlay. AuxId[ overlay_alpha_layer_id[ i ] ] shall be equal to overlay_alpha_aux_id_minus128 + 128 for all values of i in the range of 0 to num_overlays_minus1, inclusive.

num_overlay_elements_minus1[ i ] indicates the number of overlay elements in the i-th overlay. num_overlay_elements_minus1[ i ] shall be in the range of 0 to 255, inclusive. When not present, the value of num_overlay_elements_minus1[ i ] is inferred to be equal to 0.

overlay_element_label_min[ i ][ j ] and overlay_element_label_max[ i ][ j ] indicate the minimum and maximum values, respectively, of the range of sample values corresponding to the j-th overlay element of the i-th overlay. The length of the overlay_element_label_min[ i ][ j ] and the overlay_element_label_max[ i ][ j ] syntax elements is overlay_element_label_min_max_length_minus8 + 8 bits.

The variable overlayElementId[ i ][ x ][ y ] specifying the overlay element identifier of the i-th overlay for the sample location ( x, y ) relative to the top-left sample is derived as follows, where p[ i ][ x ][ y ] refers to the luma sample at location ( x, y ) in the decoded label auxiliary picture of the i-th overlay:

for( y = 0; y < pic_height_in_luma_samples; y++ )
	for( x = 0; x < pic_width_in_luma_samples; x++ )
		for( i = 0; i  <=  number_overlays_minus1[ i ] ) {
			overlayElementId[ i ][ x ][ y ] = 0
			for( j = 0; j  <=  num_overlay_elements_minus1[ i ]; j++ ) 	(F-103)
				if( p[ i ][ x ][ y ]  >=  overlay_element_label_min[ i ][ j ]  &&
							p[ i ][ x ][ y ]  <=  overlay_element_label_max[ i ][ j ] )
					overlayElementId[ i ][ x ][ y ] = j
		}

overlay_zero_bit shall be equal to 0.

overlay_language[ i ] contains a language tag as specified by RFC 5646 followed by a null termination byte equal to 0x00. The length of the overlay_language[ i ] syntax element shall be less than or equal to 255 bytes, not including the null termination byte.

overlay_name[ i ] indicates the name of the i-th overlay. The length of the overlay_name[ i ] syntax element shall be less than or equal to 255 bytes, not including the null termination byte.

overlay_element_name[ i ][ j ] indicates the name of the j-th overlay element of the i-th overlay. The length of the overlay_element_name[ i ][ j ] syntax element shall be less than or equal to 255 bytes, not including the null termination byte.

overlay_info_persistence_flag specifies the persistence of the overlay information SEI message. overlay_info_persistence_flag equal to 0 specifies that the overlay information SEI message applies to the current decoded picture only.

When an access unit contains an auxiliary picture picA with nuh_layer_id equal to nuhLayerIdA and AuxId[ nuhLayerIdA ] in the range of 128..159, and nuhLayerIdA is equal to any of overlay_content_layer_id[ i ], overlay_label_layer_id[ i ], overlay_label_layer_id[ i ], and overlay_alpha_layer_id[ i ], overlay_info_persistence_flag equal to 1 specifies that the overlay information SEI message persists for the CLVS containing the auxiliary picture picA in output order until one or more of the following conditions are true:

–	A new CLVS begins for the layer containing the auxiliary picture picA.

–	A new CLVS begins for the layer containing the primary picture associated with the auxiliary picture picA.

–	The bitstream ends.

–	A picture picB in an access unit containing an overlay information SEI message is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

      - Temporal motion vector prediction constraints SEI message semantics

The temporal motion vector prediction constraints SEI message indicates constraints on collocated pictures for temporal motion vector prediction. The SEI message may be used to determine whether the motion vectors of earlier pictures in decoding order no longer need to be stored and whether the motion vectors of the current picture and subsequent pictures need to be stored.

The temporal motion vector prediction constraints SEI message is a prefix SEI message. The temporal motion vector prediction constraints SEI message may be present in an access unit with TemporalId equal to 0 and shall not be present in an access unit with TemporalId greater than 0.

The following semantics apply separately to each nuh_layer_id targetLayerId among the nuh_layer_id values to which the temporal motion vector constraints SEI message applies. Let associatedLayerIdList consist of each targetLayerId value to which this temporal motion vector constraints SEI message applies.

Let a set of pictures associatedPicSet be the pictures with nuh_layer_id equal to targetLayerId from the access unit containing the SEI message, inclusive, up to but not including the first of any of the following in decoding order:

–	The next access unit, in decoding order, that contains a temporal motion vector prediction constraints SEI message with an associatedLayerIdList that contains targetLayerId.

–	The first picture of the next CLVS, in decoding order, of the layer with nuh_layer_id equal to targetLayerId.

prev_pics_not_used_flag equal to 1 indicates that the syntax elements for all coded pictures that are within or follow the access unit containing the current picture in decoding order are constrained such that no temporal motion vector from any picture that has nuh_layer_id equal to any value in associatedLayerIdList and precedes the access unit containing the current picture in decoding order is used directly or indirectly in decoding of any coded picture that is within or follows the access unit containing the current picture in decoding order. prev_pics_not_used_flag equal to 0 indicates that the bitstream may or may not fulfill the constraints indicated by prev_pics_not_used_flag equal to 1.

NOTE 1 – When prev_pics_not_used_flag is equal to 1, decoders may empty the "motion vector storage" for all reference pictures with nuh_layer_id equal to targetLayerId in the decoded picture buffer.

prev_pics_not_used_flag shall be equal to 1 when both of the following conditions are true:

–	no_intra_layer_col_pic_flag is equal to 1 in the previous temporal motion vector prediction constraints SEI message applying to nuh_layer_id equal to targetLayerId.

–	The previous temporal motion vector prediction constraints SEI message applying to nuh_layer_id equal to targetLayerId and the current temporal motion vector prediction constraints SEI message apply to the same CLVS of the layer with nuh_layer_id equal to targetLayerId.

no_intra_layer_col_pic_flag equal to 1 indicates the following:

–	If NumDirectRefLayers[ targetLayerId ] is equal to 0, slice_temporal_mvp_enabled_flag is not present or is equal to 0 in each picture in associatedPicSet.

–	Otherwise, all the pictures in associatedPicSet do not use temporal motion vector prediction or use collocated pictures with nuh_layer_id different from targetLayerId.

When no_intra_layer_col_pic_flag is equal to 0, no constraint on the collocated picture of the pictures with nuh_layer_id equal to targetLayerId is indicated.

Let NoIntraLayerColPicFlag[ targetLayerId ] be equal to no_intra_layer_col_pic_flag.

NOTE 2 – The motion vectors of the current picture with nuh_layer_id equal to layerId have to be stored when they may be used for temporal motion vector prediction of other pictures in the same layer or when they may be used for inter-layer motion prediction. In other words, the motion vectors of the current picture have to be stored when at least one of the following is true:

- sps_temporal_mvp_enabled_flag in the active SPS for the current picture is equal to 1 and NoIntraLayerColPicFlag[ layerId ] is equal 0.

- NoIntraLayerColPicFlag[ layerId ] is equal to 1 and there is a nuh_layer_id value nuhLayerIdA such that VpsInterLayerMotionPredictionEnabled[ LayerIdxInVps[ nuhLayerIdA ] ][ LayerIdxInVps[ layerId ] ] is equal to 1.

NOTE 3 – The motion vectors of a picture with nuh_layer_id equal to layerId need no longer be stored when the picture is marked as "unused for reference", or the picture is not used for temporal motion vector prediction of other pictures in the same layer and all pictures in the same access unit that may use the picture as a reference for inter-layer motion prediction have been decoded, or the access unit containing the picture precedes the current access unit in decoding order, where this SEI message is present with associatedLayerIdList including the nuh_layer_id of the picture and prev_pics_not_used_flag equal to 1.

      - Frame-field information SEI message semantics

The frame-field information SEI message may be used to indicate how the associated picture should be displayed, the source scan type of the associated picture, and whether the associated picture is a duplicate of a previous picture, in output order, of the same layer.

The following semantics apply separately to each nuh_layer_id targetLayerId among the nuh_layer_id values to which the frame-field information SEI message applies.

A frame-field information SEI message associated with nuh_layer_id equal to targetLayerId shall be present in an access unit, when all of the following conditions are true:

–	A picture with nuh_layer_id equal to targetLayerId is present in the access unit.

–	frame_field_info_present_flag is equal to 1 in the active SPS for the layer with nuh_layer_id equal to targetLayerId.

–	targetLayerId is greater than 0 or none of the following conditions is true:

–	A non-nested picture timing SEI message is present in the access unit.

–	A picture timing SEI message directly contained in a scalable nesting SEI message is present in the access unit.

A frame-field information SEI message that applies to a particular set of layers shall not be present when one or more of the following conditions are true:

–	The value of field_seq_flag is not the same for all active SPSs for the particular set of layers.

–	The values of general_progressive_source_flag and general_interlaced_source_flag are not identical, respectively, for all the profile_tier_level( ) syntax structures that apply to the layers to which the frame-field information SEI message applies.

The semantics of ffinfo_pic_struct, ffinfo_source_scan_type and ffinfo_duplicate_flag apply layer-wise to each value of targetLayerId.

ffinfo_pic_struct has the same semantics as the pic_struct syntax element in the picture timing SEI message.

ffinfo_source_scan_type has the same semantics as the source_scan_type syntax element in the picture timing SEI message.

ffinfo_duplicate_flag has the same semantics as the duplicate_flag syntax element in the picture timing SEI message.

  - Video usability information

    - General

The specifications in clause E.1 apply.

    - VUI syntax

The specifications in clause E.2 and its subclauses apply.

    - VUI semantics

      - VUI parameters semantics

The specifications in clause E.3.1 apply by replacing the semantics of field_seg_flag, frame_field_info_present_flag and vui_timing_info_present_flag with those below and with the following additions:

–	It is a requirement of bitstream conformance that, when nuh_layer_id layerId is greater than 0 and NumDirectRefLayers[ layerId ] is greater than 0, video_signal_type_present_flag present in or inferred for the active SPS for nuh_layer_id equal to layerId shall be equal to 0.

–	When the current picture has nuh_layer_id layerIdCurr greater than 0, either NumDirectRefLayers[ layerIdCurr ] is greater than 0 or MultiLayerExtSpsFlag derived from the active SPS for the nuh_layer_id equal to layerIdCurr is equal to 1, and the active SPS for nuh_layer_id equal to layerIdCurr contains the VUI parameters syntax structure, the following applies:

–	The values of video_format, video_full_range_flag, colour_primaries, transfer_characteristics, and matrix_coeffs are inferred to be equal to video_vps_format, video_full_range_vps_flag, colour_primaries_vps, transfer_characteristics_vps, and matrix_coeffs_vps, respectively, of the vps_video_signal_info_idx[ j ]-th video_signal_info( ) syntax structure in the active VPS where j is equal to LayerIdxInVps[ layerIdCurr ].

–	The values of video_format, video_full_range_flag, colour_primaries, transfer_characteristics, and matrix_coeffs signalled in the active SPS for the layer with nuh_layer_id equal to layerIdCurr are ignored.

NOTE 1 – The values are inferred from the VPS when a non-base layer refers to an SPS that is also referred to by the base layer, in which case the SPS has nuh_layer_id equal to 0. For the base layer, the values of these parameters in the active SPS for the base layer apply.

- field_seq_flag equal to 1 indicates that the layers for which the SPS is an active SPS within the CVS convey pictures that represent fields, and specifies the following:

–	When the SPS is an active SPS for nuh_layer_id equal to 0, a picture timing SEI message that is not nested or that is directly contained in a scalable nesting SEI message and applies to nuh_layer_id equal to 0 shall be present in every such access unit of the current CVS that contains a coded picture with nuh_layer_id equal to 0.

–	When the SPS is an active SPS for the nuh_layer_id value nuhLayerId greater than 0, a frame-field information SEI message shall be present for nuh_layer_id equal to nuhLayerId in every access unit containing a picture for the current CLVS of the layer with nuh_layer_id equal to nuhLayerId.

- field_seq_flag equal to 0 indicates that the layers for which the SPS is an active SPS within the CVS convey pictures that represent frames and that a picture timing SEI message or a frame-field information SEI message may or may not be present for the pictures within the CVS belonging to any layer for which the SPS is an active SPS. When field_seq_flag is not present, it is inferred to be equal to 0. When general_frame_only_constraint_flag is present in the SPS and is equal to 1, the value of field_seq_flag shall be equal to 0. When general_frame_only_constraint_flag is present in the active VPS, applies for a layer for which the SPS is an active SPS, and is equal to 1, the value of field_seq_flag shall be equal to 0.

NOTE 2 – The specified decoding process does not treat access units conveying pictures that represent fields or frames differently. A sequence of pictures that represent fields would therefore be coded with the picture dimensions of an individual field. For example, access units containing pictures that represent 1080i fields would commonly have cropped output dimensions of 1920x540, while the sequence picture rate would commonly express the rate of the source fields (typically between 50 and 60 Hz), instead of the source frame rate (typically between 25 and 30 Hz).

frame_field_info_present_flag equal to 1 specifies that picture timing SEI messages or frame-field information SEI messages are present for every picture for which this SPS is the active SPS and the picture timing SEI messages, when present, include the pic_struct, source_scan_type, and duplicate_flag syntax elements. frame_field_info_present_flag equal to 0 specifies that the pic_struct syntax element is not present in picture timing SEI messages associated with pictures for which the SPS is the active SPS.

When frame_field_info_present_flag is present and either or both of the following conditions are true, frame_field_info_present_flag shall be equal to 1:

–	field_seq_flag is equal to 1.

–	general_progressive_source_flag and general_interlaced_source_flag are present in this SPS, general_progressive_source_flag is equal to 1, and general_interlaced_source_flag is equal to 1.

When frame_field_info_present_flag is not present, its value is inferred as follows:

–	If general_progressive_source_flag and general_interlaced_source_flag are present in this SPS, general_progressive_source_flag is equal to 1, and general_interlaced_source_flag is equal to 1, frame_field_info_present_flag is inferred to be equal to 1.

–	Otherwise, frame_field_info_present_flag is inferred to be equal to 0.

vui_timing_info_present_flag equal to 1 specifies that vui_num_units_in_tick, vui_time_scale, vui_poc_proportional_to_timing_flag, and vui_hrd_parameters_present_flag are present in the vui_parameters( ) syntax structure. vui_timing_info_present_flag equal to 0 specifies that vui_num_units_in_tick, vui_time_scale, vui_poc_proportional_to_timing_flag, and vui_hrd_parameters_present_flag are not present in the vui_parameters( ) syntax structure. It is a requirement of bitstream conformance that, when MultiLayerExtSpsFlag is equal to 1, vui_timing_info_present_flag shall be equal to 0.

      - HRD parameters semantics

The specifications in clause E.3.2 apply with the replacement of each instance of "access unit level" and "sub-picture level" with "partition unit level" and "sub-partition level", respectively, and with the following additions:

initial_cpb_removal_delay_length_minus1 plus 1 within the j-th hrd_parameters( ) syntax structure in the VPS specifies the length, in bits, of the nal_initial_arrival_delay[ k ] and vcl_initial_arrival_delay[ k ] syntax elements of the bitstream partition initial arrival time SEI message that is contained in a bitstream partition nesting SEI message within a scalable nesting SEI message with values of MaxTemporalId[ 0 ], sei_ols_idx, sei_partitioning_scheme_idx, and bsp_idx such that bsp_hrd_idx[ sei_ols_idx ][ sei_partitioning_scheme_idx ][ MaxTemporalId[ 0 ] ][ k ][ bsp_idx ] is equal to j. When the initial_cpb_removal_delay_length_minus1 syntax element is not present, it is inferred to be equal to 23.

It is a requirement of bitstream conformance that the value of sub_pic_hrd_params_present_flag shall be the same for all hrd_parameters( ) syntax structures that apply to at least one bitstream partition of a particular output layer set.

      - Sub-layer HRD parameters semantics

The specifications in clause E.3.3 apply.

- Annex G

Multiview high efficiency video coding

(This annex forms an integral part of this Recommendation | International Standard.)

  - Scope

This annex specifies syntax, semantics, and decoding processes for multiview high efficiency video coding that use the syntax, semantics, and decoding processes specified in clauses 2-10 and Annexes A-A. This annex also specifies profiles, tiers, and levels for multiview high efficiency video coding.

  - Normative references

The list of normative references in clause A.2 applies.

  - Definitions

The specifications in clause A.3 and its subclauses apply.

  - Abbreviations

The specifications in clause A.4 apply.

  - Conventions

The specifications in clause A.5 apply.

  - Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships

The specifications in clause A.6 apply.

  - Syntax and semantics

The specifications in clause A.7 and its subclauses apply.

  - Decoding processes

    - General decoding process

      - General

The specifications of clause A.8.1.1 apply.

      - Decoding process for a coded picture with nuh_layer_id greater than 0

The decoding process for the current picture CurrPic is as follows:

- The decoding of NAL units is specified in clause B.8.2.

- The processes in clauses B.8.1.3 and A.8.3.4 specify the following decoding processes using syntax elements in the slice segment layer and above:

–	Prior to decoding the first slice of the current picture, clause B.8.1.3 is invoked.

–	At the beginning of the decoding process for each P or B slice, the decoding process for reference picture lists construction specified in clause A.8.3.4 is invoked for derivation of reference picture list 0 (RefPicList0), and when decoding a B slice, reference picture list 1 (RefPicList1).

- The processes in clauses A.8.5, B.8.5, B.8.6, and B.8.7 specify decoding processes using syntax elements in all syntax structure layers. It is a requirement of bitstream conformance that the coded slices of the picture shall contain slice segment data for every coding tree unit of the picture, such that the division of the picture into slices, the division of the slices into slice segments, and the division of the slice segments into coding tree units each form a partitioning of the picture.

      - Decoding process for inter-layer reference picture set

Outputs of this process are updated lists of inter-layer reference pictures RefPicSetInterLayer0 and RefPicSetInterLayer1 and the variables NumActiveRefLayerPics0 and NumActiveRefLayerPics1.

The variable currLayerId is set equal to nuh_layer_id of the current picture.

The lists RefPicSetInterLayer0 and RefPicSetInterLayer1 are first emptied, NumActiveRefLayerPics0 and NumActiveRefLayerPics1 are set equal to 0 and the following applies:

for( i = 0; i < NumActiveRefLayerPics; i++ ) {
	refPicSet0Flag =
		( ( ViewId[ currLayerId ]  <=  ViewId[ 0 ]  &&  	ViewId[ currLayerId ]  <=  ViewId[ RefPicLayerId[ i ] ] )  | |
		( ViewId[ currLayerId ]  >=  ViewId[ 0 ]  &&  	ViewId[ currLayerId ]  >=  ViewId[ RefPicLayerId[ i ] ] ) )
	if( there is a picture picX in the DPB that is in the same access unit as the current picture and has
				nuh_layer_id equal to RefPicLayerId[ i ] ) {
		if( refPicSet0Flag ) {
			RefPicSetInterLayer0[ NumActiveRefLayerPics0 ] = picX
			RefPicSetInterLayer0[ NumActiveRefLayerPics0++ ] is marked as "used for long-term reference"
		} else {
			RefPicSetInterLayer1[ NumActiveRefLayerPics1 ] = picX
			RefPicSetInterLayer1[ NumActiveRefLayerPics1++ ] is marked as "used for long-term reference"
		}
	} else {
		if( refPicSet0Flag )
			RefPicSetInterLayer0[ NumActiveRefLayerPics0++ ] = "no reference picture"	(G-6)
		else
			RefPicSetInterLayer1[ NumActiveRefLayerPics1++ ] = "no reference picture"
	}
}

There shall be no entry equal to "no reference picture" in RefPicSetInterLayer0 or RefPicSetInterLayer1.

There shall be no picture that has discardable_flag equal to 1 in RefPicSetInterLayer0 or RefPicSetInterLayer1.

If the current picture is a RADL picture, there shall be no entry in RefPicSetInterLayer0 or RefPicSetInterLayer1 that is a RASL picture.

NOTE – An access unit may contain both RASL and RADL pictures.

    - NAL unit decoding process

The specifications in clause A.8.2 apply.

    - Slice decoding processes

The specifications in clause A.8.3 and its subclauses apply.

    - Decoding process for coding units coded in intra prediction mode

The specifications in clause A.8.4 apply.

    - Decoding process for coding units coded in inter prediction mode

The specifications in clause A.8.5 apply.

    - Scaling, transformation and array construction process prior to deblocking filter process

The specifications in clause A.8.6 apply.

    - In-loop filter process

The specifications in clause A.8.7 apply.

  - Parsing process

The specifications in clause A.9 apply.

  - Specification of bitstream subsets

The specifications in clause A.10 and its subclauses apply.

  - Profiles, tiers, and levels

    - Profiles

      - Multiview Main profile

For a layer in an output operation point associated with an OLS in a bitstream, the layer being conforming to the Multiview Main profile, the following applies:

–	Let olsIdx be the OLS index of the OLS.

–	The sub-bitstream subBitstream is derived as follows:

–	If OlsIdxToLsIdx[ olsIdx ] is less than or equal to vps_num_layer_sets_minus1, subBitstream is derived by invoking the sub-bitstream extraction process as specified in clause A.10.1 with the following inputs: the bitstream, tIdTarget equal to OpTid of the output operation point, and layerIdListTarget containing the nuh_layer_id value layerId of the layer and all the reference layers of the layer.

–	Otherwise, subBitstream is derived by invoking the sub-bitstream extraction process as specified in clause A.10.3 with tIdTarget equal to OpTid of the output operation point and with layerIdListTarget containing the nuh_layer_id value of the layer and all the reference layers of the layer.

–	The base layer sub-bitstream baseBitstream is derived as follows:

–	If VCL NAL units with nuh_layer_id equal to 0 are included in subBitstream, baseBitstream is derived by invoking the sub-bitstream extraction process as specified in clause A.10.1 with the subBitstream, tIdTarget equal to OpTid of the output operation point, and layerIdListTarget containing only one nuh_layer_id value that is equal to 0 as inputs.

–	Otherwise, baseBitstream is derived by invoking the independent non-base layer rewriting process as specified in clause A.10.2 with subBitstream, tIdTarget equal to OpTid of the output operation point, and layerIdListTarget containing only the smallest nuh_layer_id value of the VCL NAL units of subBitstream as inputs.

The base layer sub-bitstream baseBitstream shall obey the following constraints:

- The base layer sub-bitstream baseBitstream shall be indicated to conform to the Main profile.

The sub-bitstream subBitstream shall obey the following constraints:

- All active VPSs shall have vps_num_rep_formats_minus1 in the range of 0 to 15, inclusive.

- All active SPSs for layers in subBitstream shall have chroma_format_idc equal to 1 only.

- All active SPSs for layers in subBitstream shall have transform_skip_rotation_enabled_flag, transform_skip_context_enabled_flag, implicit_rdpcm_enabled_flag, explicit_rdpcm_enabled_flag, extended_precision_processing_flag, intra_smoothing_disabled_flag, high_precision_offsets_enabled_flag, persistent_rice_adaptation_enabled_flag, and cabac_bypass_alignment_enabled_flag, when present, equal to 0 only.

- CtbLog2SizeY derived from all active SPSs for layers in subBitstream shall be in the range of 4 to 6, inclusive.

- All active PPSs for layers in subBitstream shall have log2_max_transform_skip_block_size_minus2 and chroma_qp_offset_list_enabled_flag, when present, equal to 0 only.

- ScalabilityId[ j ][ smIdx ] derived according to any active VPS shall be equal to 0 for any smIdx value not equal to 1 or 3 and for any value of j such that layer_id_in_nuh[ j ] is among layerIdListTarget that was used to derive subBitstream.

- When NumLayersInIdList[ OlsIdxToLsIdx[ olsIdx ] ] is equal to 2, output_layer_flag[ olsIdx ][ j ] derived according to any active VPS shall be equal to 1 for j in the range of 0 to 1, inclusive, for subBitstream.

- All active VPSs shall have alt_output_layer_flag[ olsIdx ] equal to 0 only.

- When ViewOrderIdx[ i ] derived according to any active VPS is equal to 1 for the layer with nuh_layer_id equal to i in subBitstream, inter_view_mv_vert_constraint_flag shall be equal to 1 in the sps_multilayer_extension( ) syntax structure in each active SPS for that layer.

- When ViewOrderIdx[ i ] derived according to any active VPS is greater than to 0 for the layer with nuh_layer_id equal to i in subBitstream, num_ref_loc_offsets shall be equal to 0 in each active PPS for that layer.

- When ViewOrderIdx[ i ] derived according to any active VPS is greater than 0 for the layer with nuh_layer_id equal to i in subBitstream, the values of pic_width_in_luma_samples and pic_height_in_luma_samples in each active SPS for that layer shall be equal to the values of pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, in each active SPS for all reference layers of that layer.

- For a layer with nuh_layer_id iNuhLId equal to any value included in layerIdListTarget that was used to derive subBitstream, the value of NumRefLayers[ iNuhLId ], which specifies the total number of direct and indirect dependent layers and is derived as specified in A.7.4.3.1, shall be less than or equal to 4.

- All active SPSs for layers in subBitstream shall have sps_range_extensions_flag equal to 0 only and sps_extension_6bits equal to 0 only.

- All active PPSs for layers in subBitstream shall have pps_range_extensions_flag equal to 0 only and pps_extension_6bits equal to 0 only.

- All active SPSs for layers in subBitstream shall have bit_depth_luma_minus8 equal to 0 only.

- All active SPSs for layers in subBitstream shall have bit_depth_chroma_minus8 equal to 0 only.

- All active PPSs for layers in subBitstream shall have colour_mapping_enabled_flag equal to 0 only.

- When an active PPS for any layer in subBitstream has tiles_enabled_flag equal to 1, it shall have entropy_coding_sync_enabled_flag equal to 0.

- When an active PPS for any layer in subBitstream has tiles_enabled_flag equal to 1, ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 256 for all values of i in the range of 0 to num_tile_columns_minus1, inclusive, and RowHeightInLumaSamples[ j ] shall be greater than or equal to 64 for all values of j in the range of 0 to num_tile_rows_minus1, inclusive.

- The number of times read_bits( 1 ) is called in clauses 9.3.4.3.3 and 9.3.4.3.4 when parsing coding_tree_unit( ) data for any coding tree unit shall be less than or equal to 5 * RawCtuBits / 3.

- general_level_idc and sub_layer_level_idc[ i ] for all values of i in active SPSs for any layer in subBitstream shall not be equal to 255 (which indicates level 8.5).

- The level constraints specified for the Multiview Main profile in clause B.11.2 shall be fulfilled.

- For any active VPS, ViewOrderIdx[ i ] shall be greater than ViewOrderIdx[ j ] for any values of i and j among layerIdListTarget that was used to derive subBitstream such that AuxId[ i ] is equal to AuxId[ j ] and i is greater than j.

In the remainder of this clause and clause B.11.2.1, all syntax elements in the profile_tier_level( ) syntax structure refer to those in the profile_tier_level( ) syntax structure associated with the layer.

Conformance of a layer in an output operation point associated with an OLS in a bitstream to the Multiview Main profile is indicated as follows:

- If OpTid of the output operation point is equal to vps_max_sub_layer_minus1, the conformance is indicated by general_profile_idc being equal to 6 or general_profile_compatibility_flag[ 6 ] being equal to 1, and general_max_12bit_constraint_flag being equal to 1, general_max_10bit_constraint_flag being equal to 1, general_max_8bit_constraint_flag being equal to 1, general_max_422chroma_constraint_flag being equal to 1, general_max_420chroma_constraint_flag being equal to 1, general_max_monochrome_constraint_flag being equal to 0, general_intra_constraint_flag being equal to 0, and general_one_picture_only_constraint_flag being equal to 0, and general_lower_bit_rate_constraint_flag being equal to 1.

- Otherwise (OpTid of the output operation point is less than vps_max_sub_layer_minus1), the conformance is indicated by sub_layer_profile_idc[ OpTid ] being equal to 6 or sub_layer_profile_compatibility_flag[ OpTid ][ 6 ] being equal to 1, and sub_layer_max_12bit_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_10bit_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_8bit_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_422chroma_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_420chroma_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_monochrome_constraint_flag[ OpTid ] being equal to 0, sub_layer_intra_constraint_flag[ OpTid ] being equal to 0, and sub_layer_one_picture_only_constraint_flag[ OpTid ] being equal to 0, and sub_layer_lower_bit_rate_constraint_flag[ OpTid ] being equal to 1.

    - Tiers and levels

      - General tier and level limits

For purposes of comparison of tier capabilities, the tier with general_tier_flag or sub_layer_tier_flag[ i ] equal to 0 is considered to be a lower tier than the tier with general_tier_flag or sub_layer_tier_flag[ i ] equal to 1.

For purposes of comparison of level capabilities, a particular level of a specific tier is considered to be a lower level than some other level of the same tier when the value of the general_level_idc or sub_layer_level_idc[ i ] of the particular level is less than that of the other level.

The following is specified for expressing the constraints in this clause and clause B.11.2.2:

–	The value of each of the variables CpbVclFactor, CpbNalFactor, FormatCapabilityFactor, and MinCrScaleFactor is the same as that specified in Table A.6 for the Main profile.

–	Let access unit n be the n-th access unit in decoding order, with the first access unit being access unit 0 (i.e., the 0-th access unit).

–	Let the variable fR be set equal to 1  300.

–	Let the variable olsIdx be the index of the OLS.

–	For each layer with nuh_layer_id equal to currLayerId, let the variable layerSizeInSamplesY be derived as follows:

layerSizeInSamplesY = pic_width_vps_in_luma_samples * pic_height_vps_in_luma_samples	(G-7)	-

where pic_width_vps_in_luma_samples and pic_height_vps_in_luma_samples are found in the vps_rep_format_idx[ LayerIdxInVps[ currLayerId ] ]-th rep_format( ) syntax structure in the VPS.

Each layer with nuh_layer_id equal to currLayerId conforming to a profile at a specified tier and level shall obey the following constraints for each bitstream conformance test as specified in clause A.13, where "access unit" is used to denote the picture unit in the layer and the CPB is understood to be the BPB:

- The value of layerSizeInSamplesY shall be less than or equal to MaxLumaPs, where MaxLumaPs is specified in Table A.4 for the tier and level of the layer.

- The value of pic_width_vps_in_luma_samples of the vps_rep_format_idx[ LayerIdxInVps[ currLayerId ] ]-th rep_format( ) syntax structure in the VPS shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

- The value of pic_height_vps_in_luma_samples of the vps_rep_format_idx[ LayerIdxInVps[ currLayerId ] ]-th rep_format( ) syntax structure in the VPS shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

- The value of max_vps_dec_pic_buffering_minus1[ olsIdx ][ LayerIdxInVps[ currLayerId ] ][ HighestTid ] shall be less than or equal to MaxDpbSize as derived by Equation A-2, with PicSizeInSamplesY being replaced with layerSizeInSamplesY, for the tier and level of the layer.

- For level 5 and higher levels, the value of CtbSizeY for the layer shall be equal to 32 or 64.

- The value of NumPicTotalCurr for each picture in the layer shall be less than or equal to 8.

- When decoding each coded picture in the layer, the value of num_tile_columns_minus1 shall be less than MaxTileCols and num_tile_rows_minus1 shall be less than MaxTileRows, where MaxTileCols and MaxTileRows are specified in Table A.4 for the tier and level of the layer.

- For the VCL HRD parameters of the layer, CpbSize[ i ] shall be less than or equal to CpbVclFactor * MaxCPB for at least one of the delivery schedules identified by bsp_sched_idx[ olsIdx ][ 0 ][ HighestTid ][ combIdx ][ LayerIdxInVps[ currLayerId ] ] for combIdx ranging from 0 to num_bsp_schedules_minus1[ olsIdx ][ 0 ][ HighestTid ], inclusive, where CpbSize[ i ] is specified in clause A.13.1 and MaxCPB is specified in Table A.4 for the tier and level of the layer in units of CpbVclFactor bits.

- For the NAL HRD parameters of the layer, CpbSize[ i ] shall be less than or equal to CpbNalFactor * MaxCPB for at least one of the delivery schedules identified by bsp_sched_idx[ olsIdx ][ 0 ][ HighestTid ][ combIdx ][ LayerIdxInVps[ currLayerId ] ] for combIdx ranging from 0 to num_bsp_schedules_minus1[ olsIdx ][ 0 ][ HighestTid ], inclusive, where CpbSize[ i ] is specified in clause A.13.1 and MaxCPB is specified in Table A.4 for the tier and level of the layer in units of CpbNalFactor bits.

Table A.4 specifies the limits for each level of each tier for levels other than level 8.5.

A tier and level to which a layer in an output operation point associated with an OLS in a bitstream conforms are indicated by the syntax elements general_tier_flag and general_level_idc if OpTid of the output layer set is equal to vps_max_sub_layer_minus1, and by the syntax elements sub_layer_tier_flag[ OpTid ] and sub_layer_level_idc[ OpTid ] otherwise, as follows:

–	If the specified level is not level 8.5, general_tier_flag or sub_layer_tier_flag[ OpTid ] equal to 0 indicates conformance to the Main tier, and general_tier_flag or sub_layer_tier_flag[ OpTid ] equal to 1 indicates conformance to the High tier, according to the tier constraints specified in Table A.4, and general_tier_flag and sub_layer_tier_flag[ OpTid ] shall be equal to 0 for levels below level 4 (corresponding to the entries in Table A.4 marked with "-"). Otherwise (the specified level is level 8.5), it is a requirement of bitstream conformance that general_tier_flag and sub_layer_tier_flag[ OpTid ] shall be equal to 1 and the value 0 for general_tier_flag and sub_layer_tier_flag[ OpTid ] is reserved for future use by ITU-T | ISO/IEC, and decoders shall ignore the value of general_tier_flag and sub_layer_tier_flag[ OpTid ].

–	general_level_idc and sub_layer_level_idc[ OpTid ] shall be set equal to a value of 30 times the level number specified in Table A.4.

      - Profile-specific tier and level limits for the Multiview Main profile

The following is specified for expressing the constraints in this clause:

–	The variable HbrFactor is set equal to 1.

–	The variable BrVclFactor is set equal to CpbVclFactor * HbrFactor.

–	The variable BrNalFactor is set equal to CpbNalFactor * HbrFactor.

–	The variable MinCr is set equal to MinCrBase * MinCrScaleFactor ÷ HbrFactor, where MinCrBase is specified in Table A.5.

Each layer conforming to the Multiview Main profile at a specified tier and level shall obey the following constraints for each conformance test as specified in clause A.13, where "access unit" is used to denote the picture unit in the layer, and the CPB is understood to be the BPB:

- The nominal removal time of access unit n (with n greater than 0) from the CPB, as specified in clause A.13.2.3, shall satisfy the constraint that AuNominalRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] is greater than or equal to Max( layerSizeInSamplesY  MaxLumaSr, fR ), where layerSizeInSamplesY is the value of layerSizeInSamplesY for access unit n − 1 and MaxLumaSr is the value specified in Table A.5 that applies to access unit n − 1 for the tier and level of the layer.

- The difference between consecutive output times of pictures in different access units, as specified in clause A.13.3.3, shall satisfy the constraint that DpbOutputInterval[ n ] is greater than or equal to Max( layerSizeInSamplesY  MaxLumaSr, fR ), where layerSizeInSamplesY is the value of layerSizeInSamplesY of access unit n and MaxLumaSr is the value specified in Table A.5 for access unit n for the tier and level of the layer, provided that access unit n is an access unit that has a picture that is output and is not the last of such access units.

- The removal time of access unit 0 shall satisfy the constraint that the number of coded slice segments in access unit 0 is less than or equal to Min( Max( 1, MaxSliceSegmentsPerPicture * MaxLumaSr / MaxLumaPs * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) + MaxSliceSegmentsPerPicture * layerSizeInSamplesY / MaxLumaPs ), MaxSliceSegmentsPerPicture ), for the value of layerSizeInSamplesY of access unit 0, where MaxSliceSegmentsPerPicture, MaxLumaPs, and MaxLumaSr are the values specified in Table A.4 and Table A.5 for the tier and level of the layer.

- The difference between consecutive CPB removal times of access units n and n − 1 (with n greater than 0) shall satisfy the constraint that the number of slice segments in access unit n is less than or equal to Min( ( Max( 1, MaxSliceSegmentsPerPicture * MaxLumaSr / MaxLumaPs * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ), MaxSliceSegmentsPerPicture ), where MaxSliceSegmentsPerPicture, MaxLumaPs, and MaxLumaSr are the values specified in Table A.4 and Table A.5 that apply to access unit n for the tier and level of the layer.

- For the VCL HRD parameters for the layer, BitRate[ i ] shall be less than or equal to BrVclFactor * MaxBR for at least one of the delivery schedules identified by bsp_sched_idx[ olsIdx ][ 0 ][ HighestTid ][ combIdx ][ LayerIdxInVps[ currLayerId ] ] for combIdx ranging from 0 to num_bsp_schedules_minus1[ olsIdx ][ 0 ][ HighestTid ], inclusive, where BitRate[ i ] is specified in clause A.13.1 and MaxBR is specified in Table A.5 in units of BrVclFactor bits/s for the tier and level of the layer.

- For the NAL HRD parameters for the layer, BitRate[ i ] shall be less than or equal to BrNalFactor * MaxBR for at least one of the delivery schedules identified by bsp_sched_idx[ olsIdx ][ 0 ][ HighestTid ][ combIdx ][ LayerIdxInVps[ currLayerId ] ] for combIdx ranging from 0 to num_bsp_schedules_minus1[ olsIdx ][ 0 ][ HighestTid ], inclusive, where BitRate[ i ] is specified in clause A.13.1 and MaxBR is specified in Table A.5 in units of BrNalFactor bits/s for the tier and level of the layer.

- The sum of the NumBytesInNalUnit variables for access unit 0 shall be less than or equal to FormatCapabilityFactor * ( Max( layerSizeInSamplesY, fR * MaxLumaSr ) + MaxLumaSr * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) ) ÷ MinCr for the value of layerSizeInSamplesY of access unit 0, where MaxLumaSr is specified in Table A.5, and both MaxLumaSr and FormatCapabilityFactor are the values that apply to access unit 0 for the tier and level of the layer.

- The sum of the NumBytesInNalUnit variables for access unit n (with n greater than 0) shall be less than or equal to FormatCapabilityFactor * MaxLumaSr * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ÷ MinCr, where MaxLumaSr is specified in Table A.5, and both MaxLumaSr and FormatCapabilityFactor are the values that apply to access unit n for the tier and level of the layer.

- The removal time of access unit 0 shall satisfy the constraint that the number of tiles in coded pictures in access unit 0 is less than or equal to Min( Max( 1, MaxTileCols * MaxTileRows * 120 * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) + MaxTileCols * MaxTileRows * PicSizeInSamplesY / MaxLumaPs ), MaxTileCols * MaxTileRows ), for the value of layerSizeInSamplesY of access unit 0, where MaxTileCols and MaxTileRows are the values specified in Table A.4 that apply to access unit 0 for the tier and level of the layer.

- The difference between consecutive CPB removal times of access units n and n − 1 (with n greater than 0) shall satisfy the constraint that the number of tiles in coded pictures in access unit n is less than or equal to Min( Max( 1, MaxTileCols * MaxTileRows * 120 * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ), MaxTileCols * MaxTileRows ), where MaxTileCols and MaxTileRows are the values specified in Table A.4 that apply to access unit n for the tier and level of the layer.

    - Decoder capabilities

When a decoder conforms to any profile specified in Annex B, it shall also have the INBLD capability specified in clause A.11.1.

Clause A.11.2 specifies requirements for a decoder conforming to any profile specified in Annex B.

  - Byte stream format

The specifications in clause A.12 apply.

  - Hypothetical reference decoder

The specifications in clause A.13 and its subclauses apply.

  - Supplemental enhancement information

    - General

The specifications in A.14.1 apply.

    - SEI payload syntax

      - General SEI payload syntax

The specifications in clause A.14.2.1 apply.

      - Annex D and Annex F SEI message syntax for multiview high efficiency video coding

The specifications in clauses A.14.2.2 through A.14.2.11 apply.

      - 3D reference displays information SEI message syntax

| three_dimensional_reference_displays_info( payloadSize ) { | Descriptor |
| --- | --- |
| prec_ref_display_width | ue(v) |
| ref_viewing_distance_flag | u(1) |
| if( ref_viewing_distance_flag ) |  |
| prec_ref_viewing_dist | ue(v) |
| num_ref_displays_minus1 | ue(v) |
| for( i = 0; i <= num_ref_displays_minus1; i++ ) { |  |
| left_view_id[ i ] | ue(v) |
| right_view_id[ i ] | ue(v) |
| exponent_ref_display_width[ i ] | u(6) |
| mantissa_ref_display_width[ i ] | u(v) |
| if( ref_viewing_distance_flag ) { |  |
| exponent_ref_viewing_distance[ i ] | u(6) |
| mantissa_ref_viewing_distance[ i ] | u(v) |
| } |  |
| additional_shift_present_flag[ i ] | u(1) |
| if( additional_shift_present_flag[ i ] ) |  |
| num_sample_shift_plus512[ i ] | u(10) |
| } |  |
| three_dimensional_reference_displays_extension_flag | u(1) |
| } |  |

      - Depth representation information SEI message syntax

        - General

| depth_representation_info( payloadSize ) { | Descriptor |
| --- | --- |
| z_near_flag | u(1) |
| z_far_flag | u(1) |
| d_min_flag | u(1) |
| d_max_flag | u(1) |
| depth_representation_type | ue(v) |
| if( d_min_flag ¦ ¦ d_max_flag ) |  |
| disparity_ref_view_id | ue(v) |
| if( z_near_flag ) |  |
| depth_rep_info_element( ZNearSign, ZNearExp, ZNearMantissa, ZNearManLen ) |  |
| if( z_far_flag ) |  |
| depth_rep_info_element( ZFarSign, ZFarExp, ZFarMantissa, ZFarManLen ) |  |
| if( d_min_flag ) |  |
| depth_rep_info_element( DMinSign, DMinExp, DMinMantissa, DMinManLen ) |  |
| if( d_max_flag ) |  |
| depth_rep_info_element( DMaxSign, DMaxExp, DMaxMantissa, DMaxManLen ) |  |
| if( depth_representation_type = = 3 ) { |  |
| depth_nonlinear_representation_num_minus1 | ue(v) |
| for( i = 1; i <= depth_nonlinear_representation_num_minus1 + 1; i++ ) |  |
| depth_nonlinear_representation_model[ i ] |  |
| } |  |
| } |  |

        - Depth representation information element syntax

| depth_rep_info_element( OutSign, OutExp, OutMantissa, OutManLen ) { | Descriptor |
| --- | --- |
| da_sign_flag | u(1) |
| da_exponent | u(7) |
| da_mantissa_len_minus1 | u(5) |
| da_mantissa | u(v) |
| } |  |

      - Multiview scene information SEI message syntax

| multiview_scene_info( payloadSize ) { | Descriptor |
| --- | --- |
| min_disparity | se(v) |
| max_disparity_range | ue(v) |
| } |  |

      - Multiview acquisition information SEI message syntax

| multiview_acquisition_info( payloadSize ) { | Descriptor |
| --- | --- |
| intrinsic_param_flag | u(1) |
| extrinsic_param_flag | u(1) |
| if( instrinsic_param_flag ) { |  |
| intrinsic_params_equal_flag | u(1) |
| prec_focal_length | ue(v) |
| prec_principal_point | ue(v) |
| prec_skew_factor | ue(v) |
| for( i = 0; i <= intrinsic_params_equal_flag ? 0 : numViewsMinus1; i++ ) { |  |
| sign_focal_length_x[ i ] | u(1) |
| exponent_focal_length_x[ i ] | u(6) |
| mantissa_focal_length_x[ i ] | u(v) |
| sign_focal_length_y[ i ] | u(1) |
| exponent_focal_length_y[ i ] | u(6) |
| mantissa_focal_length_y[ i ] | u(v) |
| sign_principal_point_x[ i ] | u(1) |
| exponent_principal_point_x[ i ] | u(6) |
| mantissa_principal_point_x[ i ] | u(v) |
| sign_principal_point_y[ i ] | u(1) |
| exponent_principal_point_y[ i ] | u(6) |
| mantissa_principal_point_y[ i ] | u(v) |
| sign_skew_factor[ i ] | u(1) |
| exponent_skew_factor[ i ] | u(6) |
| mantissa_skew_factor[ i ] | u(v) |
| } |  |
| } |  |
| if( extrinsic_param_flag ) { |  |
| prec_rotation_param | ue(v) |
| prec_translation_param | ue(v) |
| for( i = 0; i <= numViewsMinus1; i++ ) |  |
| for( j = 1; j <= 3; j++ ) { /* row */ |  |
| for( k = 1; k <= 3; k++ ) { /* column */ |  |
| sign_r[ i ][ j ][ k ] | u(1) |
| exponent_r[ i ][ j ][ k ] | u(6) |
| mantissa_r[ i ][ j ][ k ] | u(v) |
| } |  |
| sign_t[ i ][ j ] | u(1) |
| exponent_t[ i ][ j ] | u(6) |
| mantissa_t[ i ][ j ] | u(v) |
| } |  |
| } |  |
| } |  |

      - Multiview view position SEI message syntax

| multiview_view_position( payloadSize ) { | Descriptor |
| --- | --- |
| num_views_minus1 | ue(v) |
| for( i = 0; i <= num_views_minus1; i++ ) |  |
| view_position[ i ] | ue(v) |
| } |  |

    - SEI payload semantics

      - General SEI payload semantics

The specifications in clause A.14.3.1 apply with the following modifications and additions:

The list VclAssociatedSeiList is set to consist of payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 128, 131, 132, and 134, 135, 136, 137, 138, 139, 140, 141, 142, 143, 161, 165, 167, 168, 177, 178, and 179.

The list PicUnitRepConSeiList is set to consist of payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 128, 129, 131, 132, 133, 135, 136, 137, 138, 139, 140, 141, 142, 143, 160, 161, 162, 163, 164, 165,, 166, 167, 168, 176, 177, 178, 179, and 180.

The semantics and persistence scope for each SEI message specified in this annex are specified in the semantics specification for each particular SEI message in the subclauses of this clause.

NOTE – Persistence information for SEI messages specified in this annex is informatively summarized in Table G 1.

Table G 1 – Persistence scope of SEI messages (informative)

| - SEI message | - Persistence scope |
| --- | --- |
| 3D reference displays information | - Specified by the semantics of the SEI message |
| Depth representation information | - Specified by the semantics of the SEI message. |
| Multiview scene information | - The CVS containing the SEI message |
| Multiview acquisition information | - The CVS containing the SEI message |
| Multiview view position SEI message | - The CVS containing the SEI message |

      - Annex D and Annex F SEI message semantics for multiview high efficiency video coding

        - General

The specifications of clause A.14.3.2 and its subclauses apply with the modifications specified in clause B.14.3.2.2.

        - Scalable nesting SEI message semantics for multiview high efficiency video coding

The specifications of A.14.3.2.7 apply with the following additions:

An SEI message that has payloadType equal to 176 (3D reference displays information) or 180 (multiview view position) shall not be directly contained in a scalable nesting SEI message.

When the scalable nesting SEI message contains an SEI message that has payloadType equal to 177, 178, or 179, bitstream_subset_flag shall be equal to 0.

      - 3D reference displays information SEI message semantics

A 3D reference displays information SEI message contains information about the reference display width(s) and reference viewing distance(s) as well as information about the corresponding reference stereo pair(s), i.e., the pair(s) of views to be displayed for the viewer's left and right eyes on the reference display at the reference viewing distance. This information enables a view renderer to generate a proper stereo pair for the target screen width and the viewing distance. The reference display width and viewing distance values are signalled in units of centimetres. The reference pair of view specified in this SEI message can be used to extract or infer parameters related to the distance between the camera centers in the reference stereo pair, which can be used for generation of views for the target display. For multi-view displays, the reference stereo pair corresponds to a pair of views that can be simultaneously observed by the viewer's left and right eyes.

When present, this SEI message shall be associated with an IRAP access unit or with a non-IRAP access unit, when all access units that follow this access unit in the decoding order also follow it in output order. The 3D reference display information SEI message applies to the current access unit and all the access units which follow this access unit in both the output and decoding order until but not including the next IRAP access unit or the next access unit containing a 3D reference displays information SEI message.

NOTE 1 – The 3D reference displays information SEI message specifies display parameters for which the 3D sequence was optimized and the corresponding reference parameters. Each reference display (i.e., a reference display width and possibly a corresponding viewing distance) is associated with one reference pair of views by signalling their ViewId. The difference between the values of ViewId is referred to as the baseline distance (i.e., the distance between the centers of the cameras used to obtain the video sequence).

The following equations can be used for determining the baseline distance and horizontal shift for the receiver's display when the ratio between the receiver's viewing distance and the reference viewing distance is the same as the ratio between the receiver screen width and the reference screen width:

baseline[ i ] = refBaseline[ i ] * ( refDisplayWidth[ i ] ÷ displayWidth )	(G-8)

shift[ i ] = refShift[ i ] * ( refDisplayWidth[ i ] ÷ displayWidth )	(G-9)

where refBaseline[ i ] is equal to right_view_id[ i ] − left_view_id[ i ] signalled in this SEI message. Other parameters related to the view generation may be obtained determined by using a similar equation.

parameter[ i ] = refParameter[ i ] * ( refDisplayWidth[ i ] ÷ displayWidth )	(G-10)

where refParameter[ i ] is a parameter related to view generation that corresponds to the reference pair of views signalled by left_view_id[ i ] and right_view_id[ i ]. In the above equations, the width of the visible part of the display used for showing the video sequence should be understood under "display width". The same equations can also be used for determining the pair of views and horizontal shift or other view synthesis parameters when the viewing distance is not scaled proportionally to the screen width compared to the reference display parameters. In this case, the effect of applying the above equations would be to keep the perceived depth in the same proportion to the viewing distance as in the reference setup.

When the view synthesis related parameters that correspond to the reference stereo pair change from one access unit to another, they should be scaled with the same scaling factor as the parameters in the access unit that the SEI message is associated with. Therefore, the above equation should also be applied to obtain the parameters for a following access unit, where the refParameter is the parameter related to the reference stereo pair associated the following access unit.

The horizontal shift for the receiver's display should also be modified by scaling it with the same factor as that used to scale the baseline distance (or other view synthesis parameters).

prec_ref_display_width specifies the exponent of the maximum allowable truncation error for refDisplayWidth[ i ] as given by 2−prec_ref_display_width. The value of prec_ref_display_width shall be in the range of 0 to 31, inclusive.

ref_viewing_distance_flag equal to 1 indicates the presence of reference viewing distance. ref_viewing_distance_flag equal to 0 indicates that the reference viewing distance is not present.

prec_ref_viewing_dist specifies the exponent of the maximum allowable truncation error for refViewingDist[ i ] as given by 2−prec_ref_viewing_dist. The value of prec_ref_viewing_dist shall be in the range of 0 to 31, inclusive.

num_ref_displays_minus1 plus 1 specifies the number of reference displays that are signalled in this SEI message. The value of num_ref_displays_minus1 shall be in the range of 0 to 31, inclusive.

left_view_id[ i ] indicates the ViewId of the left view of a stereo pair corresponding to the i-th reference display.

right_view_id[ i ] indicates the ViewId of the right view of a stereo-pair corresponding to the i-th reference display.

exponent_ref_display_width[ i ] specifies the exponent part of the reference display width of the i-th reference display. The value of exponent_ref_display_width[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified reference display width.

mantissa_ref_display_width[ i ] specifies the mantissa part of the reference display width of the i-th reference display. The variable refDispWidthBits specifying the number of bits of the mantissa_ref_display_width[ i ] syntax element is derived as follows:

- If exponent_ref_display_width[ i ] is equal to 0, refDispWidthBits is set equal to Max( 0, prec_ref_display_width − 30 ).

- Otherwise ( 0 < exponent_ref_display_width[ i ] < 63 ), refDispWidthBits is set equal to Max( 0, exponent_ref_display_width[ i ] + prec_ref_display_width − 31 ).

exponent_ref_viewing_distance[ i ] specifies the exponent part of the reference viewing distance of the i-th reference display. The value of exponent_ref_viewing_distance[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified reference display width.

mantissa_ref_viewing_distance[ i ] specifies the mantissa part of the reference viewing distance of the i-th reference display. The variable refViewDistBits specifying the number of bits of the mantissa_ref_viewing_distance[ i ] syntax element is derived as follows:

- If exponent_ref_viewing_distance[ i ] is equal to 0, the refViewDistBits is set equal to Max( 0, prec_ref_viewing_distance − 30 ).

- Otherwise ( 0 < exponent_ref_viewing_distance[ i ] < 63 ), refViewDistBits is set equal to Max( 0, exponent_ref_viewing_distance[ i ] + prec_ref_viewing_distance − 31 ).

The variables in the x row of Table G.2 are derived from the respective variables or values in the e, n, and v rows of Table G.2 as follows:

- If e is not equal to 0, the following applies:

x = 2( e − 31 ) * ( 1 + n ÷2v )	(GG-11)

- Otherwise (e is equal to 0), the following applies:

x = 2−( 30 + v ) * n	(GG-12)

NOTE 2 – The above specification is similar to that found in IEC 60559:1989, Binary floating-point arithmetic for microprocessor systems.

Table G.2 – Association between camera parameter variables and syntax elements

| x | refDisplayWidth[ i ] | refViewingDistance[ i ] |
| --- | --- | --- |
| e | exponent_ref_display_width[ i ] | exponent_ref_viewing_distance[ i ] |
| n | mantissa_ref_display_width[ i ] | mantissa_ref_viewing_distance[ i ] |
| v | refDispWidthBits | refViewDistBits |

additional_shift_present_flag[ i ] equal to 1 indicates that the information about additional horizontal shift of the left and right views for the i-th reference display is present in this SEI message. additional_shift_present_flag[ i ] equal to 0 indicates that the information about additional horizontal shift of the left and right views for the i-th reference display is not present in this SEI message.

num_sample_shift_plus512[ i ] indicates the recommended additional horizontal shift for a stereo pair corresponding to the i-th reference baseline and the i-th reference display.

- If ( num_sample_shift_plus512[ i ] − 512 ) is less than 0, it is recommended that the left view of the stereo pair corresponding to the i-th reference baseline and the i-th reference display is shifted in the left direction by ( 512 − num_sample_shift_plus512[ i ] ) samples with respect to the right view of the stereo pair.

- Otherwise, if num_sample_shift_plus512[ i ] is equal to 512, it is recommended that shifting is not applied.

- Otherwise, ( ( num_sample_shift_plus512[ i ] − 512 ) is greater than 0 ), it is recommended that the left view in the stereo pair corresponding to the i-th reference baseline and the i-th reference display should be shifted in the right direction by ( 512 − num_sample_shift_plus512[ i ] ) samples with respect to the right view of the stereo pair.

The value of num_sample_shift_plus512[ i ] shall be in the range of 0 to 1023, inclusive.

NOTE 3 – Shifting the left view in the left (or right) direction by x samples with respect to the right view can be performed by the following two-step processing:

- Shift the left view by x / 2 samples in the left (or right) direction, and shift the right view by x / 2 samples in the right (or left) direction.

- Fill the left and right image margins of x / 2 samples in width in both the left and right views in background colour.

The following pseudo code explains the recommended shifting processing in the case of shifting the left view in the left direction by x samples with respect to the right view:

for( i = x / 2; i < width − x / 2; i++ )
		for( j = 0; j < height; j++ ) {
			leftView[ j ][ i ] = leftView[ j ][ i + x / 2 ]
			rightView[ j ][ width − 1 − i ] = rightView[ j ][ width − 1 − i − x / 2 ]	(G-13)
		}
for( i = 0; i < x / 2; i++ )
		for( j = 0; j < height; j++ ) {
			leftView[ j ][ width − 1 − i ] = leftView[ j ][ i ] = backgroundColour
			rightView[ j ][ width − 1 − i ] = rightView[ j ][ i ] = backgroundColour
	}

The following pseudo code explains the recommended shifting processing in the case of shifting the left view in the right direction by x samples with respect to the right view:

for( i = x / 2; i < width − x / 2; i++ )
		for( j = 0; j < height; j++ ){
			leftView[ j ][ width − 1 − i ] = leftView[ j ][ width − 1 − i − x / 2 ]
			rightView[ j ][ i ] = rightView[ j ][ i + x/2 ] 	(G-14)
	}
for( i=0; i < x / 2; i++ )
		for( j = 0; j < height; j++ ) {
			leftView[ j ][ width − 1 − i ] = leftView[ j ][ i ] = backgroundColour
			rightView[ j ][ width − 1 − i ] = rightView[ j ][ i ] = backgroundColour
	}

The variable backgroundColour may take different values in different systems, for example black or gray.

three_dimensional_reference_displays_extension_flag equal to 0 indicates that no additional data follows within the reference displays SEI message. The value of three_dimensional_reference_displays_extension_flag shall be equal to 0 in bitstreams conforming to this version of this Specification. The value of 1 for three_dimensional_reference_displays_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follows the value of 1 for three_dimensional_reference_displays_extension_flag in a reference displays SEI message.

      - Depth representation information SEI message semantics

        - General

The syntax elements in the depth representation information SEI message specify various parameters for auxiliary pictures of type AUX_DEPTH for the purpose of processing decoded primary and auxiliary pictures prior to rendering on a 3D display, such as view synthesis. Specifically, depth or disparity ranges for depth pictures are specified.

When present, the depth representation information SEI message shall be associated with one or more layers with AuxId value equal to AUX_DEPTH. The following semantics apply separately to each nuh_layer_id targetLayerId among the nuh_layer_id values to which the depth representation information SEI message applies.

When present, the depth representation information SEI message may be included in any access unit. It is recommended that, when present, the SEI message is included for the purpose of random access in an access unit in which the coded picture with nuh_layer_id equal to targetLayerId is an IRAP picture.

For an auxiliary picture with AuxId[ targetLayerId ] equal to AUX_DEPTH, an associated primary picture, if any, is a picture in the same access unit having AuxId[ nuhLayerIdB ] equal to 0 such that ScalabilityId[ LayerIdxInVps[ targetLayerId ] ][ j ] is equal to ScalabilityId[ LayerIdxInVps[ nuhLayerIdB ] ][ j ] for all values of j in the range of 0 to 2, inclusive, and 4 to 15, inclusive.

The information indicated in the SEI message applies to all the pictures with nuh_layer_id equal to targetLayerId from the access unit containing the SEI message up to but excluding the next picture, in decoding order, associated with a depth representation information SEI message applicable to targetLayerId or to the end of the CLVS of the nuh_layer_id equal to targetLayerId, whichever is earlier in decoding order.

z_near_flag equal to 0 specifies that the syntax elements specifying the nearest depth value are not present in the syntax structure. z_near_flag equal to 1 specifies that the syntax elements specifying the nearest depth value are present in the syntax structure.

z_far_flag equal to 0 specifies that the syntax elements specifying the farthest depth value are not present in the syntax structure. z_far_flag equal to 1 specifies that the syntax elements specifying the farthest depth value are present in the syntax structure.

d_min_flag equal to 0 specifies that the syntax elements specifying the minimum disparity value are not present in the syntax structure. d_min_flag equal to 1 specifies that the syntax elements specifying the minimum disparity value are present in the syntax structure.

d_max_flag equal to 0 specifies that the syntax elements specifying the maximum disparity value are not present in the syntax structure. d_max_flag equal to 1 specifies that the syntax elements specifying the maximum disparity value are present in the syntax structure.

depth_representation_type specifies the representation definition of decoded luma samples of auxiliary pictures as specified in Table G.3. In Table G.3, disparity specifies the horizontal displacement between two texture views and Z value specifies the distance from a camera.

The variable maxVal is set equal to ( 1  <<  ( 8 + bit_depth_luma_minus8 ) ) − 1, where bit_depth_luma_minus8 is the value included in or inferred for the active SPS of the layer with nuh_layer_id equal to targetLayerId.

Table G.3 – Definition of depth_representation_type

| depth_representation_type | Interpretation |
| --- | --- |
| 0 | Each decoded luma sample value of an auxiliary picture represents an inverse of Z value that is uniformly quantized into the range of 0 to maxVal, inclusive. When z_far_flag is equal to 1, the luma sample value equal to 0 represents the inverse of ZFar (specified below). When z_near_flag is equal to 1, the luma sample value equal to maxVal represents the inverse of ZNear (specified below). |
| 1 | Each decoded luma sample value of an auxiliary picture represents disparity that is uniformly quantized into the range of 0 to maxVal, inclusive. When d_min_flag is equal to 1, the luma sample value equal to 0 represents DMin (specified below). When d_max_flag is equal to 1, the luma sample value equal to maxVal represents DMax (specified below). |
| 2 | Each decoded luma sample value of an auxiliary picture represents a Z value uniformly quantized into the range of 0 to maxVal, inclusive. When z_far_flag is equal to 1, the luma sample value equal to 0 corresponds to ZFar (specified below). When z_near_flag is equal to 1, the luma sample value equal to maxVal represents ZNear (specified below). |
| 3 | Each decoded luma sample value of an auxiliary picture represents a nonlinearly mapped disparity, normalized in range from 0 to maxVal, as specified by depth_nonlinear_representation_num_minus1 and depth_nonlinear_representation_model[ i ]. When d_min_flag is equal to 1, the luma sample value equal to 0 represents DMin (specified below). When d_max_flag is equal to 1, the luma sample value equal to maxVal represents DMax (specified below). |
| Other values | Reserved for future use |

disparity_ref_view_id specifies the ViewId value against which the disparity values are derived.

NOTE 1 – disparity_ref_view_id is present only if d_min_flag is equal to 1 or d_max_flag is equal to 1 and is useful for depth_representation_type values equal to 1 and 3.

The variables in the x column of Table G.4 are derived from the respective variables in the s, e, n, and v columns of Table G.4 as follow.]

–	If the value of e is in the range of 0 to 127, exclusive, x is set equal to ( −1 )s * 2e − 31 * ( 1 + n ÷ 2v ).

–	Otherwise (e is equal to 0), x is set equal to ( −1 )s * 2−( 30 + v ) * n.

NOTE 1 – The above specification is similar to that found in IEC 60559:1989, Binary floating-point arithmetic for microprocessor systems.

Table G.4 – Association between depth parameter variables and syntax elements

| x | S | e | n | v |
| --- | --- | --- | --- | --- |
| ZNear | ZNearSign | ZNearExp | ZNearMantissa | ZNearManLen |
| ZFar | ZFarSign | ZFarExp | ZFarMantissa | ZFarManLen |
| DMax | DMaxSign | DMaxExp | DMaxMantissa | DMaxManLen |
| DMin | DMinSign | DMinExp | DMinMantissa | DMinManLen |

The DMin and DMax values, when present, are specified in units of a luma sample width of the coded picture with ViewId equal to ViewId of the auxiliary picture.

The units for the ZNear and ZFar values, when present, are identical but unspecified.

depth_nonlinear_representation_num_minus1 plus 2 specifies the number of piecewise linear segments for mapping of depth values to a scale that is uniformly quantized in terms of disparity.

depth_nonlinear_representation_model[ i ] for i ranging from 0 to depth_nonlinear_representation_num_minus1 + 2, inclusive, specify the piecewise linear segments for mapping of decoded luma sample values of an auxiliary picture to a scale that is uniformly quantized in terms of disparity. The values of depth_nonlinear_representation_model[ 0 ] and depth_nonlinear_representation_model[ depth_nonlinear_representation_num_minus1 + 2 ] are both inferred to be equal to 0.

NOTE 2 – When depth_representation_type is equal to 3, an auxiliary picture contains nonlinearly transformed depth samples. The variable DepthLUT[ i ], as specified below, is used to transform decoded depth sample values from the nonlinear representation to the linear representation, i.e., uniformly quantized disparity values. The shape of this transform is defined by means of line-segment approximation in two-dimensional linear-disparity-to-nonlinear-disparity space. The first ( 0, 0 ) and the last ( maxVal, maxVal ) nodes of the curve are predefined. Positions of additional nodes are transmitted in form of deviations (depth_nonlinear_representation_model[ i ]) from the straight-line curve. These deviations are uniformly distributed along the whole range of 0 to maxVal, inclusive, with spacing depending on the value of nonlinear_depth_representation_num_minus1.

The variable DepthLUT[ i ] for i in the range of 0 to maxVal, inclusive, is specified as follows:

for( k = 0; k  <=  depth_nonlinear_representation_num_minus1 + 1; k++ ) {
	pos1 = ( maxVal * k ) / (depth_nonlinear_representation_num_minus1 + 2 )
	dev1 = depth_nonlinear_representation_model[ k ]
	pos2 = ( maxVal * ( k + 1 ) ) / (depth_nonlinear_representation_num_minus1 + 2 )
	dev2 = depth_nonlinear_representation_model[ k + 1 ]	(G-15)

	x1 = pos1 − dev1
	y1 = pos1 + dev1
	x2 = pos2 − dev2
	y2 = pos2 + dev2

	for( x = Max( x1, 0 ); x  <=  Min( x2, maxVal ); x++ )
			DepthLUT[ x ] = Clip3( 0, maxVal, Round( ( ( x − x1 ) * ( y2 − y1 ) ) ÷ ( x2 − x1 ) + y1 ) )
}

When depth_representation_type is equal to 3, DepthLUT[ dS ] for all decoded luma sample values dS of an auxiliary picture in the range of 0 to maxVal, inclusive, represents disparity that is uniformly quantized into the range of 0 to maxVal, inclusive.

        - Depth representation information element semantics

The syntax structure specifies the value of an element in the depth representation information SEI message.

The syntax structure sets the values of the OutSign, OutExp, OutMantissa, and OutManLen variables that represent a floating-point value. When the syntax structure is included in another syntax structure, the variable names OutSign, OutExp, OutMantissa, and OutManLen are to be interpreted as being replaced by the variable names used when the syntax structure is included.

da_sign_flag equal to 0 indicates that the sign of the floating-point value is positive. da_sign_flag equal to 1 indicates that the sign is negative. The variable OutSign is set equal to da_sign_flag.

da_exponent specifies the exponent of the floating-point value. The value of da_exponent shall be in the range of 0 to 27 − 2, inclusive. The value 27 − 1 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 27 − 1 as indicating an unspecified value. The variable OutExp is set equal to da_exponent.

da_mantissa_len_minus1 plus 1 specifies the number of bits in the da_mantissa syntax element. The value of da_mantissa_len_minus1 shall be in the range of 0 to 31, inclusive. The variable OutManLen is set equal to da_mantissa_len_minus1 + 1.

da_mantissa specifies the mantissa of the floating-point value. The variable OutMantissa is set equal to da_mantissa.

      - Multiview scene information SEI message semantics

The multiview scene information SEI message indicates the minimum disparity and the maximum disparity among multiple views in an access unit. The minimum disparity and the maximum disparity could be used for processing the decoded views prior to rendering on a 3D display. When present, the multiview scene information SEI message shall be associated with an IRAP access unit. The information signalled in the SEI message applies to the coded video sequence. If the SEI message is not nested within a scalable nesting SEI message, it applies to the views with nuh_layer_id greater than or equal to the nuh_layer_id value of the SEI NAL unit that contains the SEI message. Otherwise (the SEI message is nested within a scalable nesting SEI message), the SEI message applies to the view with nuh_layer_id values identified in the scalable nesting SEI message. The views to which the SEI message applies are referred to as applicable views below.

The actual minimum disparity value may be greater than the one signalled in the multiview scene information SEI message and the actual maximum disparity value may be less than the one signalled in the multiview scene information SEI message, due to that some views in the coded video sequence may have been removed from the original bitstream to produce an extracted sub-bitstream, for example according to the process specified in clause 10.

Let xR be a luma sample position within a luma sample array of the right-side picture of any spatially adjacent views among the applicable views in an access unit. Let xL be the respective luma sample position within a luma sample array of the left-side picture of the same spatially adjacent view such that sample in the luma sample position xL in the left-side view represents the same content as luma sample position xR in the right-side picture. When pic_width_in_luma_samples for the left-side picture, picWidthL, is not equal to pic_width_in_luma_samples for the right-side picture, picWidthR, xL is normalized to the horizontal luma sample resolution of the right-side picture, i.e., xL is set equal to Round( xL * ( picWidthR ÷ picWidthL ) ). The disparity between xR and xL is specified to be equal to ( xR − xL ). The maximum disparity between two pictures is defined to be the maximum of the disparity between any sample position pairs xR and xL. The minimum disparity between two pictures is defined to be the minimum of the disparity between any sample position pairs xR and xL.

min_disparity specifies the minimum disparity, in units of luma samples, between pictures of any spatially adjacent views among the applicable views in an access unit. The value of min_disparity shall be in the range of −1024 to 1023, inclusive.

max_disparity_range specifies that the maximum disparity, in units of luma samples, between pictures of any spatially adjacent views among the applicable views in an access unit. The value of max_disparity_range shall be in the range of 0 to 2047, inclusive.

NOTE – The minimum disparity and the maximum disparity depend on the baseline distance between spatially adjacent views and the spatial resolution of each view. Therefore, if either the number of views or spatial resolution is changed, the minimum disparity and the maximum disparity should also be changed accordingly.

      - Multiview acquisition information SEI message semantics

The multiview acquisition information SEI message specifies various parameters of the acquisition environment. Specifically, intrinsic and extrinsic camera parameters are specified. These parameters could be used for processing the decoded views prior to rendering on a 3D display.

The following semantics apply separately to each nuh_layer_id targetLayerId among the nuh_layer_id values to which the multiview acquisition information SEI message applies as specified in clause D.3.1.

When present, the multiview acquisition information SEI message that applies to the current layer shall be included in an access unit that contains an IRAP picture that is the first picture of a CLVS of the current layer. The information signalled in the SEI message applies to the CLVS.

When the multiview acquisition information SEI message is included in a scalable nesting SEI message, the syntax elements bitstream_subset_flag, nesting_op_flag, and all_layers_flag in the scalable nesting SEI message shall be equal to 0.

The variable numViewsMinus1 is derived as follows:

- If the multiview acquisition information SEI message is not included in a scalable nesting SEI message, numViewsMinus1 is set equal to 0.

- Otherwise (the multiview acquisition information SEI message is included in a scalable nesting SEI message), numViewsMinus1 is set equal to nesting_num_layers_minus1.

Some of the views for which the multiview acquisition information is included in a multiview acquisition information SEI message may not be present.

In the semantics below, index i refers to the syntax elements and variables that apply to the layer with nuh_layer_id equal to nestingLayerIdList[ 0 ][ i ].

The extrinsic camera parameters are specified according to a right-handed coordinate system, where the upper left corner of the image is the origin, i.e., the ( 0, 0 ) coordinate, with the other corners of the image having non-negative coordinates. With these specifications, a 3-dimensional world point, wP = [ x y z ] is mapped to a 2-dimensional camera point, cP[ i ] = [ u v 1 ], for the i-th camera according to:

s * cP[ i ] = A[ i ] * R−1[ i ] * ( wP − T[ i ] )	(GG-16)

where A[ i ] denotes the intrinsic camera parameter matrix, R−1[ i ] denotes the inverse of the rotation matrix R[ i ], T[ i ] denotes the translation vector, and s (a scalar value) is an arbitrary scale factor chosen to make the third coordinate of cP[ i ] equal to 1. The elements of A[ i ], R[ i ], T[ i ] are determined according to the syntax elements signalled in this SEI message and as specified below.

intrinsic_param_flag equal to 1 indicates the presence of intrinsic camera parameters. intrinsic_param_flag equal to 0 indicates the absence of intrinsic camera parameters.

extrinsic_param_flag equal to 1 indicates the presence of extrinsic camera parameters. extrinsic_param_flag equal to 0 indicates the absence of extrinsic camera parameters.

intrinsic_params_equal_flag equal to 1 indicates that the intrinsic camera parameters are equal for all cameras and only one set of intrinsic camera parameters are present. intrinsic_params_equal_flag equal to 0 indicates that the intrinsic camera parameters are different for each camera and that a set of intrinsic camera parameters are present for each camera.

prec_focal_length specifies the exponent of the maximum allowable truncation error for focal_length_x[ i ] and focal_length_y[ i ] as given by 2−prec_focal_length. The value of prec_focal_length shall be in the range of 0 to 31, inclusive.

prec_principal_point specifies the exponent of the maximum allowable truncation error for principal_point_x[ i ] and principal_point_y[ i ] as given by 2−prec_principal_point. The value of prec_principal_point shall be in the range of 0 to 31, inclusive.

prec_skew_factor specifies the exponent of the maximum allowable truncation error for skew factor as given by 2−prec_skew_factor. The value of prec_skew_factor shall be in the range of 0 to 31, inclusive.

sign_focal_length_x[ i ] equal to 0 indicates that the sign of the focal length of the i-th camera in the horizontal direction is positive. sign_focal_length_x[ i ] equal to 1 indicates that the sign is negative.

exponent_focal_length_x[ i ] specifies the exponent part of the focal length of the i-th camera in the horizontal direction. The value of exponent_focal_length_x[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified focal length.

mantissa_focal_length_x[ i ] specifies the mantissa part of the focal length of the i-th camera in the horizontal direction. The length of the mantissa_focal_length_x[ i ] syntax element is variable and determined as follows:

- If exponent_focal_length_x[ i ] is equal to 0, the length is Max( 0, prec_focal_length − 30 ).

- Otherwise (exponent_focal_length_x[ i ] is in the range of 0 to 63, exclusive), the length is Max( 0, exponent_focal_length_x[ i ] + prec_focal_length − 31 ).

sign_focal_length_y[ i ] equal to 0 indicates that the sign of the focal length of the i-th camera in the vertical direction is positive. sign_focal_length_y[ i ] equal to 1 indicates that the sign is negative.

exponent_focal_length_y[ i ] specifies the exponent part of the focal length of the i-th camera in the vertical direction. The value of exponent_focal_length_y[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified focal length.

mantissa_focal_length_y[ i ] specifies the mantissa part of the focal length of the i-th camera in the vertical direction.

The length of the mantissa_focal_length_y[ i ] syntax element is variable and determined as follows:

- If exponent_focal_length_y[ i ] is equal to 0, the length is Max( 0, prec_focal_length − 30 ).

- Otherwise (exponent_focal_length_y[ i ] is in the range of 0 to 63, exclusive), the length is Max( 0, exponent_focal_length_y[ i ] + prec_focal_length − 31 ).

sign_principal_point_x[ i ] equal to 0 indicates that the sign of the principal point of the i-th camera in the horizontal direction is positive. sign_principal_point_x[ i ] equal to 1 indicates that the sign is negative.

exponent_principal_point_x[ i ] specifies the exponent part of the principal point of the i-th camera in the horizontal direction. The value of exponent_principal_point_x[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified principal point.

mantissa_principal_point_x[ i ] specifies the mantissa part of the principal point of the i-th camera in the horizontal direction. The length of the mantissa_principal_point_x[ i ] syntax element in units of bits is variable and is determined as follows:

- If exponent_principal_point_x[ i ] is equal to 0, the length is Max( 0, prec_principal_point − 30 ).

- Otherwise (exponent_principal_point_x[ i ] is in the range of 0 to 63, exclusive), the length is Max( 0, exponent_principal_point_x[ i ] + prec_principal_point − 31 ).

sign_principal_point_y[ i ] equal to 0 indicates that the sign of the principal point of the i-th camera in the vertical direction is positive. sign_principal_point_y[ i ] equal to 1 indicates that the sign is negative.

exponent_principal_point_y[ i ] specifies the exponent part of the principal point of the i-th camera in the vertical direction. The value of exponent_principal_point_y[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified principal point.

mantissa_principal_point_y[ i ] specifies the mantissa part of the principal point of the i-th camera in the vertical direction. The length of the mantissa_principal_point_y[ i ] syntax element in units of bits is variable and is determined as follows:

- If exponent_principal_point_y[ i ] is equal to 0, the length is Max( 0, prec_principal_point − 30 ).

- Otherwise (exponent_principal_point_y[ i ] is in the range of 0 to 63, exclusive), the length is Max( 0, exponent_principal_point_y[ i ] + prec_principal_point − 31 ).

sign_skew_factor[ i ] equal to 0 indicates that the sign of the skew factor of the i-th camera is positive.

sign_skew_factor[ i ] equal to 1 indicates that the sign is negative.

exponent_skew_factor[ i ] specifies the exponent part of the skew factor of the i-th camera. The value of exponent_skew_factor[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified skew factor.

mantissa_skew_factor[ i ] specifies the mantissa part of the skew factor of the i-th camera. The length of the mantissa_skew_factor[ i ] syntax element is variable and determined as follows:

- If exponent_skew_factor[ i ] is equal to 0, the length is Max( 0, prec_skew_factor − 30 ).

- Otherwise (exponent_skew_factor[ i ] is in the range of 0 to 63, exclusive), the length is Max( 0, exponent_skew_factor[ i ] + prec_skew_factor − 31 ).

The intrinsic matrix A[ i ] for i-th camera is represented by

focalLengthX[ i ]skewFactor[ i ]principalPointX[ i ]0 focalLengthY[ i ]principalPointY[ i ]001	(GG-17)

prec_rotation_param specifies the exponent of the maximum allowable truncation error for r[ i ][ j ][ k ] as given by 2−prec_rotation_param. The value of prec_rotation_param shall be in the range of 0 to 31, inclusive.

prec_translation_param specifies the exponent of the maximum allowable truncation error for t[ i ][ j ] as given by 2−prec_translation_param. The value of prec_translation_param shall be in the range of 0 to 31, inclusive.

sign_r[ i ][ j ][ k ] equal to 0 indicates that the sign of ( j, k ) component of the rotation matrix for the i-th camera is positive. sign_r[ i ][ j ][ k ] equal to 1 indicates that the sign is negative.

exponent_r[ i ][ j ][ k ] specifies the exponent part of ( j, k ) component of the rotation matrix for the i-th camera. The value of exponent_r[ i ][ j ][ k ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified rotation matrix.

mantissa_r[ i ][ j ][ k ] specifies the mantissa part of ( j, k ) component of the rotation matrix for the i-th camera. The length of the mantissa_r[ i ][ j ][ k ] syntax element in units of bits is variable and determined as follows:

- If exponent_r[ i ] is equal to 0, the length is Max( 0, prec_rotation_param − 30 ).

- Otherwise (exponent_r[ i ] is in the range of 0 to 63, exclusive), the length is Max( 0, exponent_r[ i ] + prec_rotation_param − 31 ).

The rotation matrix R[ i ] for i-th camera is represented as follows:

rE[ i ][ 0 ][ 0 ]rE[ i ][ 0 ][1]rE[ i ][ 0 ][ 2 ]rE[ i ][ 1 ][ 0 ]rE[ i ][ 1 ][ 1 ]rE[ i ][ 1 ][ 2 ]rE[ i ][ 2 ][ 0 ]rE[ i ][ 2 ][1]rE[ i ][ 2 ][ 2 ]	(GG-18)

sign_t[ i ][ j ] equal to 0 indicates that the sign of the j-th component of the translation vector for the i-th camera is positive. sign_t[ i ][ j ] equal to 1 indicates that the sign is negative.

exponent_t[ i ][ j ] specifies the exponent part of the j-th component of the translation vector for the i-th camera. The value of exponent_t[ i ][ j ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified translation vector.

mantissa_t[ i ][ j ] specifies the mantissa part of the j-th component of the translation vector for the i-th camera. The length v of the mantissa_t[ i ][ j ] syntax element in units of bits is variable and is determined as follows:

- If exponent_t[ i ] is equal to 0, the length v is set equal to Max( 0, prec_translation_param − 30 ).

- Otherwise (0 < exponent_t[ i ] < 63), the length v is set equal to Max( 0, exponent_t[ i ] + prec_translation_param − 31 ).

The translation vector T[ i ] for the i-th camera is represented by:

tE[ i ][ 0 ]tE[ i ][ 1 ]tE[ i ][ 2 ]	(GG-19)

The association between the camera parameter variables and corresponding syntax elements is specified by Table G.5. Each component of the intrinsic and rotation matrices and the translation vector is obtained from the variables specified in Table G.5 as the variable x computed as follows:

- If e is in the range of 0 to 63, exclusive, x is set equal to ( −1 )s * 2e − 31 * ( 1 + n ÷ 2v ).

- Otherwise (e is equal to 0), x is set equal to ( −1 )s * 2−( 30 + v ) * n.

NOTE – The above specification is similar to that found in IEC 60559:1989, Binary floating-point arithmetic for microprocessor systems.

Table G.5 – Association between camera parameter variables and syntax elements.

| x | s | e | n |
| --- | --- | --- | --- |
| focalLengthX[ i ] | sign_focal_length_x[ i ] | exponent_focal_length_x[ i ] | mantissa_focal_length_x[ i ] |
| focalLengthY[ i ] | sign_focal_length_y[ i ] | exponent_focal_length_y[ i ] | mantissa_focal_length_y[ i ] |
| principalPointX[ i ] | sign_principal_point_x[ i ] | exponent_principal_point_x[ i ] | mantissa_principal_point_x[ i ] |
| principalPointY[ i ] | sign_principal_point_y[ i ] | exponent_principal_point_y[ i ] | mantissa_principal_point_y[ i ] |
| skewFactor[ i ] | sign_skew_factor[ i ] | exponent_skew_factor[ i ] | mantissa_skew_factor[ i ] |
| rE[ i ][ j ][ k ] | sign_r[ i ][ j ][ k ] | exponent_r[ i ][ j ][ k ] | mantissa_r[ i ][ j ][ k ] |
| tE[ i ][ j ] | sign_t[ i ][ j ] | exponent_t[ i ][ j ] | mantissa_t[ i ][ j ] |

      - Multiview view position SEI message semantics

The multiview view position SEI message specifies the relative view position along a single horizontal axis of views within a CVS. When present, the multiview view position SEI message shall be associated with an IRAP access unit. The information signalled in this SEI message applies to the entire CVS.

num_views_minus1 plus 1 shall be equal to NumViews derived from the active VPS for the CVS. The value of num_views_minus1 shall be in the range of 0 to 62, inclusive.

view_position[ i ] indicates the order of the view with ViewOrderIdx equal to i among all the views from left to right for the purpose of display, with the order for the left-most view being equal to 0 and the value of the order increasing by 1 for next view from left to right. The value of view_position[ i ] shall be in the range of 0 to 62, inclusive.

  - Video usability information

The specifications in clause A.15 and its subclauses apply.

- Annex H

Scalable high efficiency video coding

(This annex forms an integral part of this Recommendation | International Standard.)

  - Scope

This annex specifies syntax, semantics and decoding processes for scalable high efficiency video coding that use the syntax, semantics, and decoding process specified in clauses 2-10 and Annexes A-A. This annex also specifies profiles, tier, and levels for scalable high efficiency video coding.

  - Normative references

The list of normative references in clause A.2 applies.

  - Definitions

The specifications in clause A.3 and its subclauses apply.

  - Abbreviations

The specifications in clause A.4 apply.

  - Conventions

The specifications in clause A.5 apply.

  - Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships

The specifications in clause A.6 apply.

  - Syntax and semantics

The specifications in clause A.7 and its subclauses apply.

  - Decoding processes

    - General decoding process

      - General

The specifications of clause A.8.1.1 apply.

      - Decoding process for a coded picture with nuh_layer_id greater than 0

The decoding process for the current picture CurrPic is as follows:

- The decoding of NAL units is specified in clause A.8.2.

- The processes in clause A.8.1.3 and A.8.3.4 specify the following decoding processes using syntax elements in the slice segment layer and above:

–	At the beginning of the decoding process for the first slice of the current picture, the process specified in clause A.8.1.3 is invoked.

–	At the beginning of the decoding process for each P or B slice, the decoding process for reference picture lists construction specified in clause A.8.3.4 is invoked for derivation of reference picture list 0 (RefPicList0), and when decoding a B slice, reference picture list 1 (RefPicList1).

- The processes in clauses A.8.4, A.8.5, A.8.6, and A.8.7 specify decoding processes using syntax elements in all syntax structure layers. It is a requirement of bitstream conformance that the coded slices of the picture shall contain slice segment data for every coding tree unit of the picture, such that the division of the picture into slices, the division of the slices into slice segments, and the division of the slice segments into coding tree units each form a partitioning of the picture.

      - Decoding process for inter-layer reference picture set

Outputs of this process are updated lists of inter-layer reference pictures RefPicSetInterLayer0 and RefPicSetInterLayer1 and the variables NumActiveRefLayerPics0 and NumActiveRefLayerPics1.

The variable currLayerId is set equal to nuh_layer_id of the current picture.

The variables NumRefLayerPicsProcessing, NumRefLayerPicsSampleProcessing, and NumRefLayerPicsMotionProcessing are set equal to 0.

The lists RefPicSetInterLayer0 and RefPicSetInterLayer1 are first emptied, NumActiveRefLayerPics0 and NumActiveRefLayerPics1 are set equal to 0 and the following applies:

for( i = 0; i < NumActiveRefLayerPics; i++ ) {
	refPicSet0Flag = 
		( ( ViewId[ currLayerId ]  <=  ViewId[ 0 ]  &&ViewId[ currLayerId ]  <=  ViewId[ RefPicLayerId[ i ] ] )  | |
		( ViewId[ currLayerId ]  >=  ViewId[ 0 ]  &&  	ViewId[ currLayerId ]  >=  ViewId[ RefPicLayerId[ i ] ] ) )
	if( there is a picture picX in the DPB that is in the same access unit as the current picture and has
				nuh_layer_id equal to RefPicLayerId[ i ] ) {
		an inter-layer reference picture ilRefPic is derived by invoking the process specified in clause A.8.1.4
				with picX and RefPicLayerId[ i ] given as inputs
		if( refPicSet0Flag ) {							(H-1)
			RefPicSetInterLayer0[ NumActiveRefLayerPics0 ] = ilRefPic
			RefPicSetInterLayer0[ NumActiveRefLayerPics0++ ] is marked as "used for long-term reference"
		} else {
			RefPicSetInterLayer1[ NumActiveRefLayerPics1 ] = ilRefPic
			RefPicSetInterLayer1[ NumActiveRefLayerPics1++ ] is marked as "used for long-term reference"
		}
	} else {
		if( refPicSet0Flag )
			RefPicSetInterLayer0[ NumActiveRefLayerPics0++ ] = "no reference picture"
		else
			RefPicSetInterLayer1[ NumActiveRefLayerPics1++ ] = "no reference picture"
	}
}

There shall be no entry equal to "no reference picture" in RefPicSetInterLayer0 or RefPicSetInterLayer1.

There shall be no picture that has discardable_flag equal to 1 in RefPicSetInterLayer0 or RefPicSetInterLayer1.

NOTE 1 – For the profiles defined in Annex A, RefPicSetInterLayer1 is always empty since the value of ViewId[ i ] is equal to zero for all layers.

If the current picture is a RADL picture, there shall be no entry in the RefPicSetInterLayer0 or RefPicSetInterLayer1 that is a RASL picture.

NOTE 2 – An access unit may contain both RASL and RADL pictures.

      - Derivation process for inter-layer reference pictures

Inputs to this process are:

–	a decoded direct reference layer picture rlPic,

–	a variable rLId specifying the value of nuh_layer_id of the direct reference layer picture.

Output of this process is the inter-layer reference picture ilRefPic.

The variables PicWidthInSamplesCurrY, PicHeightInSamplesCurrY, BitDepthCurrY, BitDepthCurrC, SubWidthCurrC and SubHeightCurrC are set equal to pic_width_in_luma_samples, pic_height_in_luma_samples, BitDepthY, BitDepthC, SubWidthC and SubHeightC of the current layer, respectively.

The variables PicWidthInSamplesRefLayerY and PicHeightInSamplesRefLayerY are set equal to the width and height of the decoded direct reference layer picture rlPic in units of luma samples, respectively. The variables BitDepthRefLayerY, BitDepthRefLayerC, SubWidthRefLayerC and SubHeightRefLayerC are set equal to the values of BitDepthY, BitDepthC, SubWidthC and SubHeightC of the direct reference layer picture rlPic, respectively.

NOTE – This clause and its subclauses support all possible values of SubWidthCurrC, SubHeightCurrC, SubWidthRefLayerC, SubHeightRefLayerC. When the value of chroma_format_idc of the current layer and the value of chroma_format_idc of the direct reference layer are both equal to 1, the values of SubWidthCurrC, SubHeightCurrC, SubWidthRefLayerC, SubHeightRefLayerC are all equal to 2, and the resampling process of picture sample values in clause A.8.1.4.1 and the colour mapping process of picture sample values in clause A.8.1.4.3 can be simplified.

The variables RefLayerRegionLeftOffset, RefLayerRegionTopOffset, RefLayerRegionRightOffset and RefLayerRegionBottomOffset are derived as follows:

RefLayerRegionLeftOffset = ref_region_left_offset[ rLId ] * SubWidthRefLayerC	(H-2)

RefLayerRegionTopOffset = ref_region_top_offset[ rLId ] * SubHeightRefLayerC	(H-2)

RefLayerRegionRightOffset = ref_region_right_offset[ rLId ] * SubWidthRefLayerC	(H-3)

RefLayerRegionBottomOffset = ref_region_bottom_offset[ rLId ] * SubHeightRefLayerC	(H-4)

The variables RefLayerRegionWidthInSamplesY and RefLayerRegionHeightInSamplesY are the width and height of the reference region in the decoded direct reference layer picture rlPic in units of luma samples, respectively, and are derived as follows:

RefLayerRegionWidthInSamplesY = PicWidthInSamplesRefLayerY −
	RefLayerRegionLeftOffset − RefLayerRegionRightOffset	(H-5)

RefLayerRegionHeightInSamplesY = PicHeightInSamplesRefLayerY −
	RefLayerRegionTopOffset − RefLayerRegionBottomOffset	(H-6)

The variables PicWidthInSamplesCurrC, PicHeightInSamplesCurrC, PicWidthInSamplesRefLayerC, and PicHeightInSamplesRefLayerC are derived as follows:

PicWidthInSamplesCurrC = PicWidthInSamplesCurrY / SubWidthCurrC	(H-7)

PicHeightInSamplesCurrC = PicHeightInSamplesCurrY / SubHeightCurrC	(H-8)

PicWidthInSamplesRefLayerC = PicWidthInSamplesRefLayerY / SubWidthRefLayerC	(H-9)

PicHeightInSamplesRefLayerC = PicHeightInSamplesRefLayerY / SubHeightRefLayerC	(H-10)

The variables ScaledRefLayerLeftOffset, ScaledRefLayerTopOffset, ScaledRefLayerRightOffset and ScaledRefLayerBottomOffset are derived as follows:

ScaledRefLayerLeftOffset = scaled_ref_layer_left_offset[ rLId ] * SubWidthCurrC	(H-11)

ScaledRefLayerTopOffset = scaled_ref_layer_top_offset[ rLId ] * SubHeightCurrC	(H-12)

ScaledRefLayerRightOffset = scaled_ref_layer_right_offset[ rLId ] * SubWidthCurrC	(H-13)

ScaledRefLayerBottomOffset = scaled_ref_layer_bottom_offset[ rLId ] * SubHeightCurrC	(H-14)

The variables ScaledRefRegionWidthInSamplesY and ScaledRefRegionHeightInSamplesY are derived as follows:

ScaledRefRegionWidthInSamplesY = PicWidthInSamplesCurrY −
		ScaledRefLayerLeftOffset − ScaledRefLayerRightOffset	(H-15)

ScaledRefRegionHeightInSamplesY = PicHeightInSamplesCurrY −
		ScaledRefLayerTopOffset − ScaledRefLayerBottomOffset	(H-16)

The variables SpatialScaleFactorHorY, SpatialScaleFactorVerY, SpatialScaleFactorHorC and SpatialScaleFactorVerC are derived as follows:

SpatialScaleFactorHorY = ( ( RefLayerRegionWidthInSamplesY  <<  16 ) +
		( ScaledRefRegionWidthInSamplesY  >>  1 ) ) / ScaledRefRegionWidthInSamplesY	(H-17)

SpatialScaleFactorVerY = ( ( RefLayerRegionHeightInSamplesY  <<  16 ) +
		( ScaledRefRegionHeightInSamplesY  >>  1 ) ) / ScaledRefRegionHeightInSamplesY	(H-18)

SpatialScaleFactorHorC = ( ( ( RefLayerRegionWidthInSamplesY / SubWidthRefLayerC )  <<  16 ) +
		( ( ScaledRefRegionWidthInSamplesY / SubWidthCurrC )  >>  1 ) )
		/ ( ScaledRefRegionWidthInSamplesY / SubWidthCurrC )	(H-19)

SpatialScaleFactorVerC = ( ( ( RefLayerRegionHeightInSamplesY / SubHeightRefLayerC )  <<  16 ) +
		( ( ScaledRefRegionHeightInSamplesY / SubHeightCurrC )  >>  1 ) )
		/ ( ScaledRefRegionHeightInSamplesY / SubHeightCurrC )	(H-20)

The variables PhaseHorY, PhaseVerY, PhaseHorC and PhaseVerC are set as follows:

PhaseHorY = phase_hor_luma[ rLId ]		(H-21)

PhaseVerY = phase_ver_luma[ rLId ]		(H-22)

PhaseHorC = phase_hor_chroma_plus8[ rLId ] − 8		(H-23)

PhaseVerC = phase_ver_chroma_plus8[ rLId ] − 8		(H-24)

It is a requirement of bitstream conformance that BitDepthRefLayerY shall be less than or equal to BitDepthCurrY and BitDepthRefLayerC shall be less than or equal to BitDepthCurrC.

It is a requirement of bitstream conformance that the values of RefLayerRegionWidthInSamplesY, RefLayerRegionHeightInSamplesY, ScaledRefRegionWidthInSamplesY and ScaledRefRegionHeightInSamplesY shall be greater than 0.

It is a requirement of bitstream conformance that ScaledRefRegionWidthInSamplesY shall be greater than or equal to RefLayerRegionWidthInSamplesY and ScaledRefRegionHeightInSamplesY shall be greater than or equal to RefLayerRegionHeightInSamplesY.

It is a requirement of bitstream conformance that, when ScaledRefRegionWidthInSamplesY is equal to RefLayerRegionWidthInSamplesY, PhaseHorY shall be equal to 0, when ScaledRefRegionWidthInSamplesC is equal to RefLayerRegionWidthInSamplesC, PhaseHorC shall be equal to 0, when ScaledRefRegionHeightInSamplesY is equal to RefLayerRegionHeightInSamplesY, PhaseVerY shall be equal to 0, and when ScaledRefRegionHeightInSamplesC is equal to RefLayerRegionHeightInSamplesC, PhaseVerC shall be equal to 0.

The following ordered steps are applied to derive the inter-layer reference picture ilRefPic.

–	The variables sampleProcessingFlag and motionProcessingFlag are initialized to 0.

–	The variable equalPictureSizeAndOffsetFlag is derived as follows:

–	If all of the following conditions are true, equalPictureSizeAndOffsetFlag is set equal to 1:

–	PicWidthInSamplesCurrY is equal to PicWidthInSamplesRefLayerY

–	PicHeightInSamplesCurrY is equal to PicHeightInSamplesRefLayerY

–	ScaledRefLayerLeftOffset is equal to RefLayerRegionLeftOffset

–	ScaledRefLayerTopOffset is equal to RefLayerRegionTopOffset

–	ScaledRefLayerRightOffset is equal to RefLayerRegionRightOffset

–	ScaledRefLayerBottomOffset is equal to RefLayerRegionBottomOffset

–	PhaseHorY, PhaseVerY, PhaseHorC and PhaseVerC are all equal to 0

–	Otherwise, equalPictureSizeAndOffsetFlag is set equal to 0.

–	The variable currColourMappingEnableFlag is derived as follows:

–	If colour_mapping_enabled_flag is equal to 1 and if there exists a value of i, with i in the range of 0 to num_cm_ref_layers_minus1, inclusive, for which cm_ref_layer_id[ i ] is equal to rLId, currColourMappingEnableFlag is set equal to 1.

–	Otherwise, currColourMappingEnableFlag is set equal to 0.

–	If equalPictureSizeAndOffsetFlag is equal to 1, BitDepthRefLayerY is equal to BitDepthCurrY, BitDepthRefLayerC is equal to BitDepthCurrC, SubWidthRefLayerC is equal to SubWidthCurrC, SubHeightRefLayerC is equal to SubHeightCurrC, and currColourMappingEnableFlag is equal to 0,

–	ilRefPic is set equal to rlPic.

–	Otherwise, the following steps apply:

–	The inter-layer reference picture ilRefPic is generated as follows:

–	The PicOrderCntVal value of ilRefPic is set equal to the PicOrderCntVal value of rlPic.

–	The variable currLayerId is set equal to the value of nuh_layer_id of the current picture.

–	When VpsInterLayerSamplePredictionEnabled[ LayerIdxInVps[ currLayerId ] ][ LayerIdxInVps[ rLId ] ] is equal to 1, the following steps apply:

–	If currColourMappingEnableFlag is equal to 1, the following ordered steps apply:

–	The colour mapping process as specified in clause A.8.1.4.3 is invoked with the picture sample arrays, rlPicSampleL, rlPicSampleCb and rlPicSampleCr, of the direct reference layer picture rlPic as inputs, and with the colour mapped picture sample arrays, cmPicSampleL, cmPicSampleCb and cmPicSampleCr of the colour mapped reference picture cmRefPic as outputs.

–	BitDepthRefLayerY and BitDepthRefLayerC are set equal to BitDepthCmOutputY and BitDepthCmOutputC, respectively.

–	If equalPictureSizeAndOffsetFlag is equal to 1, BitDepthRefLayerY is equal to BitDepthCurrY, BitDepthRefLayerC is equal to BitDepthCurrC, SubWidthRefLayerC is equal to SubWidthCurrC, and SubHeightRefLayerC is equal to SubHeightCurrC, the following applies:

–	rsPicSampleL, rsPicSampleCb and rsPicSampleCr are set equal to cmPicSampleL, cmPicSampleCb and cmPicSampleCr, respectively.

–	Otherwise, the following applies:

–	the picture sample resampling process as specified in clause A.8.1.4.1 is invoked with the picture sample arrays, cmPicSampleL, cmPicSampleCb and cmPicSampleCr, of the colour mapped direct reference layer picture cmPic as inputs, and with the resampled picture sample arrays, rsPicSampleL, rsPicSampleCb and rsPicSampleCr of the inter-layer reference picture ilRefPic as outputs.

–	Otherwise, the following applies:

–	the picture sample resampling process as specified in clause A.8.1.4.1 is invoked with the picture sample arrays, rlPicSampleL, rlPicSampleCb and rlPicSampleCr, of the direct reference layer picture rlPic as inputs, and with the resampled picture sample arrays, rsPicSampleL, rsPicSampleCb and rsPicSampleCr of the inter-layer reference picture ilRefPic as outputs.

–	sampleProcessingFlag is set equal to 1.

–	When VpsInterLayerMotionPredictionEnabled[ LayerIdxInVps[ currLayerId ] ][ LayerIdxInVps[ rLId ] ] is equal to 1, the following steps apply:

–	A single slice ilRefSlice of the inter-layer reference picture ilRefPic is generated as follows:

–	The values of slice_type, num_ref_idx_l0_active_minus1 and num_ref_idx_l1_active_minus1 for the generated slice ilRefSlice are set equal to the values of slice_type, num_ref_idx_l0_active_minus1 and num_ref_idx_l1_active_minus1, respectively, of the first slice of rlPic.

–	When ilRefSlice is a P or B slice, for i in the range of 0 to num_ref_idx_l0_active_minus1 of ilRefSlice, inclusive, the reference picture associated with index i in reference picture list 0 of ilRefSlice is set equal to the reference picture associated with index i in reference picture list 0 of the first slice of rlPic.

–	When ilRefSlice is a B slice, for i in the range of 0 to num_ref_idx_l1_active_minus1 of ilRefSlice, inclusive, the reference picture associated with index i in reference picture list 1 of ilRefSlice is set equal to the reference picture associated with index i in reference picture list 1 of the first slice of rlPic.

NOTE – When the inter-layer reference picture ilRefPic is used as the collocated picture for temporal motion vector prediction, all slices of rlPic are constrained to have the same values of slice_type, num_ref_idx_l0_active_minus1 and num_ref_idx_l1_active_minus1.

–	If equalPictureSizeAndOffsetFlag is equal to 1, the following applies:

–	The motion and mode parameters of the inter-layer reference picture ilRefPic, including an array CuPredMode specifying the prediction modes, two arrays RefIdxL0 and RefIdxL1 specifying the reference indices, two arrays MvL0 and MvL1 specifying the luma motion vectors, and two arrays PredFlagL0 and PredFlagL1 specifying the prediction list utilization flags, are set equal to those of the decoded direct reference layer picture rlPic, respectively.

–	Otherwise, the following applies:

–	The picture motion and mode parameters resampling process as specified in clause A.8.1.4.2 is invoked with the direct reference layer picture rlPic, an array rlPredMode specifying the prediction modes CuPredMode of rlPic, two arrays rlRefIdxL0 and rlRefIdxL1 specifying the reference indices of rlPic, two arrays rlMvL0 and rlMvL1 specifying the luma motion vectors of rlPic, and two arrays rlPredFlagL0 and rlPrefFlagL1 specifying the prediction list utilization flags of rlPic as inputs, and an array rsPredMode specifying the prediction modes CuPredMode of ilRefPic, two arrays rsRefIdxL0 and rsRefIdxL1 specifying the reference indices of ilRefPic, two arrays rsMvL0 and rsMvL1 specifying the luma motion vectors of ilRefPic, and two arrays rsPredFlagL0 and rsPredFlagL1 specifying the prediction list utilization flags of ilRefPic as outputs.

–	motionProcessingFlag is set equal to 1.

–	The following applies:

NumRefLayerPicsSampleProcessing  +=  sampleProcessingFlag	(H-25)

NumRefLayerPicsMotionProcessing  +=  motionProcessingFlag	(H-26)

NumRefLayerPicsProcessing  +=  sampleProcessingFlag  | |  motionProcessingFlag	(H-27)

        - Resampling process of picture sample values

Inputs to this process are:

–	a (PicWidthInSamplesRefLayerY)x(PicHeightInSamplesRefLayerY) array rlPicSampleL of luma samples,

–	a (PicWidthInSamplesRefLayerC )x(PicHeightInSamplesRefLayerC) array rlPicSampleCb of chroma samples of the component Cb,

–	a (PicWidthInSamplesRefLayerC )x(PicHeightInSamplesRefLayerC) array rlPicSampleCr of chroma samples of the component Cr.

Outputs of this process are:

–	a (PicWidthInSamplesCurrY)x(PicHeightInSamplesCurrY) array rsPicSampleL of luma samples,

–	a (PicWidthInSamplesCurrC)x(PicHeightInSamplesCurrC) array rsPicSampleCb of chroma samples of the component Cb,

–	a (PicWidthInSamplesCurrC)x(PicHeightInSamplesCurrC) array rsPicSampleCr of chroma samples of the component Cr.

The resampled luma sample value rsPicSampleL[ xP ][ yP ], with xP = 0..PicWidthInSamplesCurrY − 1, yP = 0..PicHeightInSamplesCurrY − 1, is derived by invoking the luma sample resampling process specified in clause A.8.1.4.1.1 with luma sample location ( xP, yP ) and the reference luma sample array rlPicSampleL given as inputs.

The resampled chroma sample value rsPicSampleCb[ xPC ][ yPC ], with xPC = 0..PicWidthInSamplesCurrC − 1, yPC = 0..PicHeightInSamplesCurrC − 1, of the chroma component Cb is derived by invoking the chroma sample resampling process specified in clause A.8.1.4.1.2 with chroma sample location ( xPC, yPC ) and the reference chroma sample array rlPicSampleCb given as inputs.

The resampled chroma sample value rsPicSampleCr[ xPC ][ yPC ], with xPC = 0..PicWidthInSamplesCurrC − 1, yPC = 0..PicHeightInSamplesCurrC − 1, of the chroma component Cr is derived by invoking the chroma sample resampling process specified in clause A.8.1.4.1.2 with chroma sample location ( xPC, yPC ) and the reference sample array rlPicSampleCr given as inputs.

          - Resampling process of luma sample values

Inputs to this process are

–	a luma sample location ( xP, yP ) relative to the top-left luma sample of the current picture,

–	the luma reference sample array rlPicSampleL.

Output of this process is the resampled luma sample value rsLumaSample.

- Table H.1 specifies the 8-tap filter coefficients fL[ p, x ] with p = 0..15 and x = 0..7 used for the luma resampling process.

Table H.1 – 16-phase luma resampling filter

| phase p | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p, 0 ] | fL[ p, 1 ] | fL[ p, 2 ] | fL[ p, 3 ] | fL[ p, 4 ] | fL[ p, 5 ] | fL[ p, 6 ] | fL[ p, 7 ] |
| 0 | 0 | 0 | 0 | 64 | 0 | 0 | 0 | 0 |
| 1 | 0 | 1 | −3 | 63 | 4 | −2 | 1 | 0 |
| 2 | −1 | 2 | −5 | 62 | 8 | −3 | 1 | 0 |
| 3 | −1 | 3 | −8 | 60 | 13 | −4 | 1 | 0 |
| 4 | −1 | 4 | −10 | 58 | 17 | −5 | 1 | 0 |
| 5 | −1 | 4 | −11 | 52 | 26 | −8 | 3 | −1 |
| 6 | −1 | 3 | −9 | 47 | 31 | −10 | 4 | −1 |
| 7 | −1 | 4 | −11 | 45 | 34 | −10 | 4 | −1 |
| 8 | −1 | 4 | −11 | 40 | 40 | −11 | 4 | −1 |
| 9 | −1 | 4 | −10 | 34 | 45 | −11 | 4 | −1 |
| 10 | −1 | 4 | −10 | 31 | 47 | −9 | 3 | −1 |
| 11 | −1 | 3 | −8 | 26 | 52 | −11 | 4 | −1 |
| 12 | 0 | 1 | −5 | 17 | 58 | −10 | 4 | −1 |
| 13 | 0 | 1 | −4 | 13 | 60 | −8 | 3 | −1 |
| 14 | 0 | 1 | −3 | 8 | 62 | −5 | 2 | −1 |
| 15 | 0 | 1 | −2 | 4 | 63 | −3 | 1 | 0 |

The value of the resampled luma sample rsLumaSample is derived by applying the following ordered steps:

- The derivation process for reference layer sample location used in resampling as specified in clause A.8.1.4.1.3 is invoked with chromaFlag equal to 0 and luma sample location ( xP, yP ) given as the inputs and luma sample location ( xRef16, yRef16 ) in units of 1/16-th luma sample as output.

- The variables xRef and xPhase are derived as follows:

xRef = ( xRef16  >>  4 )		(H-28)

xPhase = ( xRef16 ) % 16		(H-29)

- The variables yRef and yPhase are derived as follows:

yRef = ( yRef16  >>  4 )		(H-30)

yPhase = ( yRef16 ) % 16		(H-31)

- The variables shift1, shift2 and offset are derived as follows:

shift1 = BitDepthRefLayerY − 8		(H-32)

shift2 = 20 − BitDepthCurrY		(H-33)

offset = 1  << ( shift2 − 1 )		(H-34)

- The sample value tempArray[ n ] with n = 0..7, is derived as follows:

yPosRL = Clip3( 0, PicHeightInSamplesRefLayerY − 1, yRef + n − 3 )	(H-35)

refW = PicWidthInSamplesRefLayerY		(H-36)

tempArray[ n ] = ( fL[ xPhase, 0 ] * rlPicSampleL[ Clip3( 0, refW − 1, xRef − 3 ), yPosRL ] +
					fL[ xPhase, 1 ] * rlPicSampleL[ Clip3( 0, refW − 1, xRef − 2 ), yPosRL ] +
					fL[ xPhase, 2 ] * rlPicSampleL[ Clip3( 0, refW − 1, xRef − 1 ), yPosRL ] +
					fL[ xPhase, 3 ] * rlPicSampleL[ Clip3( 0, refW − 1, xRef ), yPosRL ] +
					fL[ xPhase, 4 ] * rlPicSampleL[ Clip3( 0, refW − 1, xRef + 1 ), yPosRL ] +	(H-37)
					fL[ xPhase, 5 ] * rlPicSampleL[ Clip3( 0, refW − 1, xRef + 2 ), yPosRL ] +
					fL[ xPhase, 6 ] * rlPicSampleL[ Clip3( 0, refW − 1, xRef + 3 ), yPosRL ] +
					fL[ xPhase, 7 ] * rlPicSampleL[ Clip3( 0, refW − 1, xRef + 4 ), yPosRL ] )  >>  shift1

- The resampled luma sample value rsLumaSample is derived as follows:

rsLumaSample = ( fL[ yPhase, 0 ] * tempArray[ 0 ] +	
					fL[ yPhase, 1 ] * tempArray[ 1 ] +	
					fL[ yPhase, 2 ] * tempArray[ 2 ] +	
					fL[ yPhase, 3 ] * tempArray[ 3 ] +	
					fL[ yPhase, 4 ] * tempArray[ 4 ] +	(H-38)
					fL[ yPhase, 5 ] * tempArray[ 5 ] +	
					fL[ yPhase, 6 ] * tempArray[ 6 ] +	
					fL[ yPhase, 7 ] * tempArray[ 7 ] + offset )  >>  shift2

rsLumaSample = Clip3( 0, ( 1  <<  BitDepthCurrY ) − 1, rsLumaSample )	(H-39)

          - Resampling process of chroma sample values

Inputs to this process are:

–	a chroma sample location ( xPC, yPC ) relative to the top-left chorma sample of the current picture,

–	the chroma reference sample array rlPicSampleC.

Output of this process is the resampled chroma sample value rsChromaSample.

- Table H.2 specifies the 4-tap filter coefficients fC[ p, x ] with p = 0..15 and x = 0..3 used for the chroma resampling process.

Table H.2 – 16-phase chroma resampling filter

| phase p | interpolation filter coefficients |  |  |  |
| --- | --- | --- | --- | --- |
|  | fC[ p, 0 ] | fC[ p, 1 ] | fC[ p, 2 ] | fC[ p, 3 ] |
| 0 | 0 | 64 | 0 | 0 |
| 1 | −2 | 62 | 4 | 0 |
| 2 | −2 | 58 | 10 | −2 |
| 3 | −4 | 56 | 14 | −2 |
| 4 | −4 | 54 | 16 | −2 |
| 5 | −6 | 52 | 20 | −2 |
| 6 | −6 | 46 | 28 | −4 |
| 7 | −4 | 42 | 30 | −4 |
| 8 | −4 | 36 | 36 | −4 |
| 9 | −4 | 30 | 42 | −4 |
| 10 | −4 | 28 | 46 | −6 |
| 11 | −2 | 20 | 52 | −6 |
| 12 | −2 | 16 | 54 | −4 |
| 13 | −2 | 14 | 56 | −4 |
| 14 | −2 | 10 | 58 | −2 |
| 15 | 0 | 4 | 62 | −2 |

The value of the resampled chroma sample value rsChromaSample is derived by applying the following ordered steps:

- The derivation process for reference layer sample location in resampling as specified in clause A.8.1.4.1.3 is invoked with chromaFlag equal to 1 and chroma sample location ( xPC, yPC ) given as the inputs and chroma sample location ( xRef16, yRef16 ) in units of 1/16-th chroma sample as output.

- The variables xRef and xPhase are derived as follows:

xRef = ( xRef16  >>  4 )		(H-40)

xPhase = ( xRef16 ) % 16		(H-41)

- The variables yRef and yPhase are derived as follows:

yRef = ( yRef16  >>  4 )		(H-42)

yPhase = ( yRef16 ) % 16		(H-43)

- The variables shift1, shift2 and offset are derived as follows:

shift1 = BitDepthRefLayerC − 8		(H-44)

shift2 = 20 − BitDepthCurrC		(H-45)

offset =1  <<  ( shift2 − 1 )		(H-46)

- The sample value tempArray[ n ] with n = 0..3, is derived as follows:

yPosRL = Clip3( 0, PicHeightInSamplesRefLayerC − 1, yRef + n − 1 )	(H-47)

refWC = PicWidthInSamplesRefLayerC		(H-48)

tempArray[ n ] = ( fC[ xPhase, 0 ] * rlPicSampleC[ Clip3( 0, refWC − 1, xRef − 1 ), yPosRL ] +
				fC[ xPhase, 1 ] * rlPicSampleC[ Clip3( 0, refWC − 1, xRef ), yPosRL ] +
				fC[ xPhase, 2 ] * rlPicSampleC[ Clip3( 0, refWC − 1, xRef + 1 ), yPosRL ] +	(H-49)
				fC[ xPhase, 3 ] * rlPicSampleC[ Clip3( 0, refWC − 1, xRef + 2 ), yPosRL ] )  >>  shift1

- The resampled chroma sample value rsChromaSample is derived as follows:

rsChromaSample = ( fC[ yPhase, 0 ] * tempArray[ 0 ] +
				fC[ yPhase, 1 ] * tempArray[ 1 ] +
				fC[ yPhase, 2 ] * tempArray[ 2 ] +	(H-50)
				fC[ yPhase, 3 ] * tempArray[ 3 ] + offset )  >>  shift2

rsChromaSample = Clip3( 0, ( 1  <<  BitDepthCurrC ) − 1, rsChromaSample )	(H-51)

          - Derivation process for reference layer sample location in units of 1/16-th sample

Inputs to this process are:

–	a variable chromaFlag specifying whether the sample location being derived is that of the luma component or that of one of the chroma colour components.

–	a sample location ( xP, yP ) relative to the top-left sample of the luma component or the top-left sample of one of the chroma components of the current picture depending on the value of chromaFlag.

Output of this process is a sample location ( xRef16, yRef16 ) specifying the reference layer sample location in units of 1/16-th sample relative to the top-left sample of the luma component or the top-left sample of one of the chroma components of the direct reference layer picture depending on the value of chromaFlag.

The variables currOffsetLeft, currOffsetTop, refOffsetLeft and refOffsetTop are derived as follows:

currOffsetLeft = ScaledRefLayerLeftOffset / ( chromaFlag ? SubWidthCurrC : 1)	(H-52)

currOffsetTop = ScaledRefLayerTopOffset / ( chromaFlag ? SubHeightCurrC : 1)	(H-53)

refOffsetLeft = ( RefLayerRegionLeftOffset / ( chromaFlag ? SubWidthRefLayerC : 1) )  <<  4	(H-54)

refOffsetTop = ( RefLayerRegionTopOffset / ( chromaFlag ? SubHeightRefLayerC : 1) )  <<  4	(H-55)

The variables phaseHor, phaseVer, scaleHor and scaleVer are derived as follows:

phaseHor = chromaFlag ? PhaseHorC : PhaseHorY		(H-56)

phaseVer = chromaFlag ? PhaseVerC : PhaseVerY		(H-57)

scaleHor = chromaFlag ? SpatialScaleFactorHorC : SpatialScaleFactorHorY	(H-58)

scaleVer = chromaFlag? SpatialScaleFactorVerC : SpatialScaleFactorVerY	(H-59)

The variables addHor and addVer are derived as follows:

addHor = −( ( scaleHor * phaseHor + 8 )  >>  4 )		(H-60)

addVer = −( ( scaleVer * phaseVer + 8 )  >>  4 )		(H-61)

The variables xRef16 and yRef16 are derived as follows:

xRef16 = ( ( ( xP − currOffsetLeft ) * scaleHor + addHor + ( 1  <<  11 ) )  >>  12 ) + refOffsetLeft	(H-62)

yRef16 = ( ( ( yP − currOffsetTop ) * scaleVer + addVer + ( 1  <<  11 ) )  >>  12 ) + refOffsetTop	(H-63)

        - Resampling process of picture motion and mode parameters

Inputs to this process are:

–	a decoded direct reference layer picture rlPic,

–	a (PicWidthInSamplesRefLayerY)x(PicHeightInSamplesRefLayerY) array rlPredMode specifying the prediction modes of the direct reference layer picture,

–	two (PicWidthInSamplesRefLayerY)x(PicHeightInSamplesRefLayerY) arrays rlRefIdxL0 and rlRefIdxL1 specifying the reference indices of the direct reference layer picture,

–	two (PicWidthInSamplesRefLayerY)x(PicHeightInSamplesRefLayerY) arrays rlMvL0 and rlMvL1 specifying the luma motion vectors of the direct reference layer picture,

–	two (PicWidthInSamplesRefLayerY)x(PicHeightInSamplesRefLayerY) arrays rlPredFlagL0 and rlPredFlagL1 specifying the prediction list utilization flags of the direct reference layer picture.

Outputs of this process are:

–	a (PicWidthInSamplesCurrY)x(PicHeightInSamplesCurrY) array rsPredMode specifying the prediction modes of the resampled picture,

–	two (PicWidthInSamplesCurrY)x(PicHeightInSamplesCurrY) arrays rsRefIdxL0 and rsRefIdxL1 specifying the reference indexes of the resampled picture,

–	two (PicWidthInSamplesCurrY)x(PicHeightInSamplesCurrY) arrays rsMvL0 and rsMvL1 specifying the luma motion vectors of the resampled picture,

–	two (PicWidthInSamplesCurrY)x(PicHeightInSamplesCurrY) arrays rsPredFlagL0 and rsPredFlagL1 specifying the prediction list utilization flags of the resampled picture.

For each 16x16 prediction block of the resampled picture with the top-left luma sample location at ( xPb, yPb ), where xPb = xB  <<  4 and xPb = xB  <<  4, for xB = 0..( ( PicWidthInSamplesCurrY + 15 )  >>  4 ) − 1 and yB = 0..( ( PicHeightInSamplesCurrY + 15 )  >>  4 ) − 1, its motion and mode parameters rsPredMode[ xPb ][ yPb ], rsMvLX[ xPb ][ yPb ], rsRefIdxLX[ xPb ][ yPb ], and rsPredFlagLX[ xPb ][ yPb ], with X being equal to 0 and 1, PicHeightInSamplesCurrYare derived by applying the following ordered steps:

- The center location ( xPCtr, yPCtr ) of the luma prediction block is derived as follows:

xPCtr = xPb + 8		(H-64)

yPCtr = yPb + 8		(H-65)

- The variables xRef and yRef are derived as follows:

xRef = ( ( ( xPCtr − ScaledRefLayerLeftOffset ) * SpatialScaleFactorHorY + ( 1  <<  15 ) )  >>  16 )
		+ RefLayerRegionLeftOffset		(H-66)

yRef = ( ( ( yPCtr − ScaledRefLayerTopOffset ) * SpatialScaleFactorVerY + ( 1  <<  15 ) )  >>  16 )
		+ RefLayerRegionTopOffset		(H-67)

- The rounded reference layer luma sample location ( xRL, yRL ) is derived as follows:

xRL = ( ( xRef + 4 )  >>  4 )  <<  4		(H-68)

yRL = ( ( yRef + 4 )  >>  4 )  <<  4		(H-69)

- The variable rsPredMode[ xP ][ yP ] is derived as follows:

if( xRL < 0  | |  xRL  >=  PicWidthInSamplesRefLayerY  | |  yRL < 0  | |
		yRL  >=  PicHeightInSamplesRefLayerY ),	(H-70)
	rsPredMode[ xP ][ yP ] = MODE_INTRA
else
	rsPredMode[ xP ][ yP ] = rlPredMode[ xRL ][ yRL ]	-

- For X being each of 0 and 1, the variables rsMvLX[ xP ][ yP ], rsRefIdxLX[ xP ][ yP ] and rsPredFlagLX[ xP ][ yP ] are derived as follows:

–	If rsPredMode[ xP ][ yP ] is equal to MODE_INTER, the following applies:

–	rsRefIdxLX[ xP ][ yP ] and rsPredFlagLX[ xP ][ yP ] are derived as follows:

rsRefIdxLX[ xP ][ yP ]	 = rlRefIdxLX[ xRL ][ yRL ]	(H-72)

rsPredFlagLX[ xP ][ yP ] = rlPredFlagLX[ xRL ][ yRL ]	(H-73)

–	rsMvLX[ xP ][ yP ][ 0 ] is derived as follows:

–	If ScaledRefRegionWidthInSamplesY is not equal to RefLayerRegionWidthInSamplesY, the following applies:

scaleMVX = Clip3( −4096, 4095, ( ( ScaledRefRegionWidthInSamplesY  <<  8 ) 	(H-74)
	+ ( RefLayerRegionWidthInSamplesY  >>  1 ) ) / RefLayerRegionWidthInSamplesY )	-

rsMvLX[ xP ][ yP ][ 0 ] = Clip3( −32768, 32767, Sign( scaleMVX *
	rlMvLX[ xRL ][ yRL ][ 0 ] ) * ( ( Abs ( scaleMVX * rlMvLX[ xRL ][ yRL ][ 0 ] ) 	(H-76)
	+ 127 )  >>  8 ) )		-

–	Otherwise, the following applies:

rsMvLX[ xP ][ yP ][ 0 ] = rlMvLX[ xRL ][ yRL ][ 0 ]	(H-78)

–	rsMvLX[ xP ][ yP ][ 1 ] is derived as follows:

–	If ScaledRefRegionHeightInSamplesY is not equal to RefLayerRegionHeightInSamplesY, the following applies:

scaleMVY = Clip3( −4096, 4095, ( ( ScaledRefRegionHeightInSamplesY  <<  8 ) 	(H-79)
	+ ( RefLayerRegionHeightInSamplesY  >>  1 ) ) / RefLayerRegionHeightInSamplesY )	-

rsMvLX[ xP ][ yP ][ 1 ] = Clip3( −32768, 32767, Sign( scaleMVY * 
	rlMvLX[ xRL ][ yRL ][ 1 ] ) * ( ( Abs ( scaleMVY * rlMvLX[ xRL ][ yRL ][ 1 ] ) 	(H-81)
	+ 127 )  >>  8 ) )		-

–	Otherwise, the following applies:

rsMvLX[ xP ][ yP ][ 1 ] = rlMvLX[ xRL ][ yRL ][ 1 ]	(H-83)

–	Otherwise (rsPredMode[ xP ][ yP ] is equal to MODE_INTRA), the following applies:

–	rsMvL0[ xP ][ yP ][ 0 ], rsMvL0[ xP ][ yP ][ 1 ], rsMvL1[ xP ][ yP ][ 0 ], and rsMvL1[ xP ][ yP ][ 1 ] are set equal to 0.

–	rsRefIdxL0[ xP ][ yP ] and rsRefIdxL1[ xP ][ yP ] are set equal to −1.

–	rsPredFlagL0[ xP ][ yP ] and rsPredFlagL1[ xP ][ yP ] are set equal to 0.

        - Colour mapping process of picture sample values

Inputs to this process are:

–	a (PicWidthInSamplesRefLayerY)x(PicHeightInSamplesRefLayerY) array rlPicSampleL of luma samples,

–	a (PicWidthInSamplesRefLayerC)x(PicHeightInSamplesRefLayerC) array rlPicSampleCb of chroma samples of the component Cb,

–	a (PicWidthInSamplesRefLayerC)x(PicHeightInSamplesRefLayerC) array rlPicSampleCr of chroma samples of the component Cr.

Outputs of this process are:

–	a (PicWidthInSamplesRefLayerY)x(PicHeightInSamplesRefLayerY) array cmPicSampleL of luma samples,

–	a (PicWidthInSamplesRefLayerC)x(PicHeightInSamplesRefLayerC) array cmPicSampleCb of chroma samples of the component Cb,

–	a (PicWidthInSamplesRefLayerC)x(PicHeightInSamplesRefLayerC) array cmPicSampleCr of chroma samples of the component Cr.

- The colour mapped luma sample cmPicSampleL[ xP ][ yP ] with xP = 0..PicWidthInSamplesRefLayerY − 1, yP = 0..PicHeightInSamplesRefLayerY − 1 is derived by invoking the colour mapping process of luma sample values as specified in clause A.8.1.4.3.1 with luma sample location ( xP, yP ), sample arrays rlPicSampleL, rlPicSampleCb and rlPicSampleCr given as inputs.

- The colour mapped chroma sample cmPicSampleCb[ xC ][ yC ] with xC = 0..PicWidthInSamplesRefLayerC − 1, yC = 0..PicHeightInSamplesRefLayerC − 1 is derived by invoking the colour mapping process of chroma sample values as specified in clause A.8.1.4.3.2 with chroma sample location ( xC, yC ), sample arrays rlPicSampleL, rlPicSampleCb rlPicSampleCr, and cIdx equal to 0 given as inputs.

- The colour mapped chroma sample cmPicSampleCr[ xC ][ yC ] with xC = 0..PicWidthInSamplesRefLayerC − 1, yC = 0..PicHeightInSamplesRefLayerC − 1 is derived by invoking the colour mapping process of chroma sample values as specified in clause A.8.1.4.3.2 with chroma sample location ( xC, yC ), sample arrays rlPicSampleL, rlPicSampleCb, rlPicSampleCr, and cIdx equal to 1 given as inputs.

          - Colour mapping process of luma sample values

Inputs to this process are:

–	a luma sample location ( xP, yP ) specifying the luma sample location relative to the top-left luma sample of the direct reference layer picture,

–	the luma reference layer sample array rlPicSampleY

–	the chroma reference layer sample array rlPicSampleCb of the Cb component

–	the chroma reference layer sample array rlPicSampleCr of the Cr component

Output of the process is a colour mapped luma sample value cmLumaSample

The chroma sample location ( xPC, yPC ) is set equal to ( xP / SubWidthRefLayerC, yP / SubHeightRefLayerC ).

The value of cmLumaSample is derived by applying the following ordered steps:

- The variables yShift2Idx, cShift2Idx are derived as follows:

yShift2Idx = BitDepthCmInputY − cm_octant_depth − cm_y_part_num_log2	(H-84)

cShift2Idx = BitDepthCmInputC − cm_octant_depth	(H-85)

- The variables nMappingShift, and nMappingOffset are derived as follows:

nMappingShift = 10 + BitDepthCmInputY − BitDepthCmOutputY	(H-86)

nMappingOffset = 1  <<  ( nMappingShift − 1 )		(H-87)

- The variables tempCb and tempCr are derived as follows:

–	If SubWidthRefLayerC is equal to 2 and SubHeightRefLayerC is equal to 2, the following applies:

–	If xP % 2 is equal to 0 and yP % 2 is equal to 0, the following applies:

yP2C = Max( 0, yPC − 1 )		(H-88)

tempCb = ( rlPicSampleCb[ xPC ][ yPC ] * 3 ) + rlPicSampleCb[ xPC ][ yP2C ] ) + 2 )  >>  2	(H-89)

tempCr = ( rlPicSampleCr[ xPC ][ yPC ] * 3 ) + rlPicSampleCr[ xPC ][ yP2C ] ) + 2 )  >>  2	(H-90)

–	Otherwise, if xP % 2 is equal to 0 and yP % 2 is equal to 1, the following applies:

yP2C = Min( yPC + 1, PicHeightInSamplesRefLayerC − 1 )	(H-91)

tempCb = ( rlPicSampleCb[ xPC ][ yPC ] * 3 + rlPicSampleCb[ xPC ][ yP2C ] + 2 )  >>  2	(H-92)

tempCr = ( rlPicSampleCr[ xPC ][ yPC ] * 3 + rlPicSampleCr[ xPC ][ yP2C ] + 2 )  >>  2	(H-93)

–	Otherwise, if xP % 2 is equal to 1 and yP % 2 is equal to 0, the following applies:

xP2C = Min( xPC + 1, PicWidthInSamplesRefLayerC − 1 )	(H-94)

yP2C = Max( 0, yPC − 1 )		(H-95)

tempCb = ( rlPicSampleCb[ xPC ][ yP2C ] + rlPicSampleCb[ xP2C ][ yP2C ] +
	( rlPicSampleCb[ xPC ][ yPC ] + rlPicSampleCb[ xP2C ][ yPC ] ) * 3 + 4 )  >>  3	(H-96)

tempCr = ( rlPicSampleCr[ xPC ][ yP2C ] + rlPicSampleCr[ xP2C ][ yP2C ] +
	( rlPicSampleCr[ xPC ][ yPC ] + rlPicSampleCr[ xP2C ][ yPC ] ) * 3 + 4 )  >>  3	(H-97)

–	Otherwise (xP % 2 is equal to 1 and yP % 2 is equal to 1), the following applies:

xP2C = Min( xPC + 1, PicWidthInSamplesRefLayerC − 1 )	(H-98)

yP2C = Min( yPC + 1, PicHeightInSamplesRefLayerC − 1 )	(H-99)

tempCb = ( ( rlPicSampleCb[ xPC ][ yPC ] + rlPicSampleCb[ xP2C ][ yPC ] ) * 3 +
	rlPicSampleCb[ xPC ][ yP2C ] + rlPicSampleCb[ xP2C ][ yP2C ] + 4 )  >>  3	(H-100)

tempCr = ( ( rlPicSampleCr[ xPC ][ yPC ] + rlPicSampleCr[ xP2C ][ yPC ] ) * 3 +
	rlPicSampleCr[ xPC ][ yP2C ] + rlPicSampleCr[ xP2C ][ yP2C ] + 4 )  >>  3	(H-101)

–	Otherwise, if SubWidthRefLayerC is equal to 2, the following applies:

–	If xP % 2 is equal to 1, the following applies:

xP2C = Min( xPC + 1, PicWidthInSamplesRefLayerC − 1 )	(H-102)

tempCb = ( rlPicSampleCb[ xPC ][ yPC ] + rlPicSampleCb[ xP2C ][ yPC ] + 1 )  >>  1	(H-103)

tempCr = ( rlPicSampleCr[ xPC ][ yPC ] + rlPicSampleCr[ xP2C ][ yPC ] + 1 )  >>  1	(H-104)

–	Otherwise (xP % 2 is equal to 0), the following applies:

tempCb = rlPicSampleCb[ xPC ][ yPC ]	(H-105)

tempCr = rlPicSampleCr[ xPC ][ yPC ]	(H-106)

–	Otherwise (SubWidthRefLayerC is equal to 1 and SubHeightRefLayerC is equal to 1), the following applies:

tempCb = rlPicSampleCb[ xPC ][ yPC ]	(H-107)

tempCr = rlPicSampleCr[ xPC ][ yPC ]	(H-108)

- The value of cmLumaSample is derived as follows:

idxY = rlPicSampleY[ xP ][ yP ]  >>  yShift2Idx		(H-109)

idxCb = ( cm_octant_depth  = =  1 ) ? ( tempCb  >=  CMThreshU ) : ( tempCb  >>  cShift2Idx )	(H-110)

idxCr = ( cm_octant_depth  = =  1 ) ? ( tempCr  >=  CMThreshV ) : ( tempCr  >>  cShift2Idx )	(H-111)

cmLumaSample = ( ( LutY[ idxY ][ idxCb ][ idxCr ][ 0 ] * rlPicSampleY[ xP ][ yP ]
		 + LutY[ idxY ][ idxCb ][ idxCr ][ 1 ] * tempCb + LutY[ idxY ][ idxCb ][ idxCr ][ 2 ] * tempCr
		 + nMappingOffset )  >>  nMappingShift ) + LutY[ idxY ][ idxCb ][ idxCr ][ 3 ]	(H-112)

cmLumaSample = Clip3( 0, (1  <<  BitDepthCmOutputY ) − 1, cmLumaSample )	(H-113)

          - Colour mapping process of chroma sample values

Inputs to this process are:

–	a chroma sample location ( xPC, yPC ) specifying the chroma sample location relative to the top-left chroma sample of the direct reference layer picture,

–	the luma reference sample array rlPicSampleY,

–	the chroma reference sample array rlPicSampleCb of the Cb component,

–	the chroma reference sample array rlPicSampleCr of the Cr component,

–	a variable cIdx specifying the chroma component index.

Output of the process is a colour mapped chorma sample value cmChromaSample.

The luma sample location ( xP, yP ) is set equal to ( xPC * SubWidthRefLayerC, yPC * SubHeightRefLayerC ).

The colour mapping table LutC is set equal to LutCb if cIdx is equal to 0, and set equal to LutCr otherwise.

The value of cmChromaSample is derived by applying the following ordered steps:

- The variables yShift2Idx, cShift2Idx are derived as follows:

yShift2Idx = BitDepthCmInputY − cm_octant_depth − cm_y_part_num_log2	(H-114)

cShift2Idx = BitDepthCmInputC − cm_octant_depth	(H-115)

- The variables nMappingShift, and nMappingOffset are derived as follows:

nMappingShift = 10 + BitDepthCmInputY − BitDepthCmOutputY	(H-116)

nMappingOffset = 1  <<  ( nMappingShift − 1 )		(H-117)

- The variable tempY is derived as follows:

–	If SubWidthRefLayerC is equal to 2 and SubHeightRefLayerC is equal to 2, the following applies:

tempY = ( rlPicSampleY[ xP ][ yP ] + rlPicSampleY[ xP ][ yP + 1 ] + 1 )  >>  1	(H-118)

–	Otherwise (SubWidthRefLayerC is not equal to 2 or SubHeightRefLayerC is not equal to 2), the following applies:

tempY = rlPicSampleY[ xP ][ yP ]		(H-119)

- The value of cmChromaSample is derived as follows:

idxY = tempY  >>  yShift2Idx		(H-120)

idxCb = ( cm_octant_depth  = =  1 ) ?		(H-121)
( rlPicSampleCb[ xPC ][ yPC ]  >=  CMThreshU ) : ( rlPicSampleCb[ xPC ][ yPC ]  >>  cShift2Idx )	-

idxCr = ( cm_octant_depth  = =  1 ) ?		(H-123)
( rlPicSampleCr[ xPC ][ yPC ]  >=  CMThreshV ) : ( rlPicSampleCr[ xPC ][ yPC ]  >>  cShift2Idx )	-

cmChromaSample = ( ( LutC[ idxY ][ idxCb ][ idxCr ][ 0 ] * tempY
		 + LutC[ idxY ][ idxCb ][ idxCr ][ 1 ] * rlPicSampleCb[ xPC ][ yPC ]	(H-125)
		 + LutC[ idxY ][ idxCb ][ idxCr ][ 2 ] * rlPicSampleCr[ xPC ][ yPC ]
		 + nMappingOffset )  >>  nMappingShift ) + LutC[ idxY ][ idxCb ][ idxCr ][ 3 ]	-

cmChromaSample = Clip3( 0, ( 1  <<  BitDepthCmOutputC ) − 1, cmChromaSample )	(H-127)

    - NAL unit decoding process

The specification in clause A.8.2 apply.

    - Slice decoding processes

      - Decoding process for picture order count

The specifications in clause A.8.3.1 apply.

      - Decoding process for reference picture set

The specifications in clause A.8.3.2 apply.

      - Decoding process for generating unavailable reference pictures

The specifications in clause A.8.3.3 apply.

      - Decoding process for reference picture lists construction

The specifications in clause A.8.3.4 apply with the following additions:

NOTE – Because bitstreams conforming to this annex are constrained to allow only zero-valued motion vectors for inter prediction using inter-layer reference pictures, it is suggested that a scalable encoder should disable temporal motion vector prediction for the current picture (by setting slice_temporal_mvp_enabled_flag to zero) when the reference picture lists of all slices in the current picture include only inter-layer reference pictures. This way, the encoder would be able to avoid the need to send the slice segment header syntax elements collocated_from_l0_flag and collocated_ref_idx.

    - Decoding process for coding units coded in intra prediction mode

The specifications in clause A.8.4 apply.

    - Decoding process for coding units coded in inter prediction mode

The specifications in clause A.8.5 apply with the following additions:

It is a requirement of bitstream conformance that, for X being replaced by either 0 or 1, the variables mvLX[ 0 ] and mvLX[ 1 ] as an output of clause 8.5.3.1 shall be equal to 0 if the value of refIdxLX as an output of clause 8.5.3.1 corresponds to an inter-layer reference picture. That is, in any conforming bitstream, for X being replaced by either 0 or 1, upon invoking the decoding process in clause 8.5.3.1, the values of the syntax elements merge_idx, mvp_lX_flag, ref_idx_lX, MvdLX, and mvd_l1_zero_flag shall always result in zero values for mvLX[ 0 ] and mvLX[ 1 ] when the value of refIdxLX of the reference picture list RefPicListX indicates an inter-layer reference picture.

The variable currLayerId is set equal to nuh_layer_id of the current picture.

It is a requirement of bitstream conformance that when the reference picture represented by the variable refIdxLX and derived by invoking clause 8.5.3.2, for X being replaced by either 0 or 1, is an inter-layer reference picture, VpsInterLayerSamplePredictionEnabled[ LayerIdxInVps[ currLayerId ] ][ LayerIdxInVps[ rLId ] ] shall be equal to 1, where rLId is set equal to nuh_layer_id of the direct reference layer picture from which the inter-layer reference picture is derived.

It is a requirement of bitstream conformance that when the collocated picture colPic, used for temporal motion vector prediction and derived by invoking clause 8.5.3.2.7, is an inter-layer reference picture, VpsInterLayerMotionPredictionEnabled[ LayerIdxInVps[ currLayerId ] ][ LayerIdxInVps[ rLId ] ] shall be equal to 1, where rLId is set equal to nuh_layer_id of the direct reference layer picture from which the inter-layer reference picture is derived.

It is a requirement of bitstream conformance that the collocated picture colPic, used for temporal motion vector prediction and derived by invoking clause 8.5.3.2.7, shall not be an inter-layer reference picture if the direct reference layer picture, from which the inter-layer reference picture is derived, is coded using two or more slice segments and any of the following conditions is true:

–	The slice segment header syntax element slice_type of at least one of the slice segments of the direct reference layer picture is different from the slice segment header syntax element slice_type of another slice segment of the direct reference layer picture;

–	For X being replaced by either 0 or 1, the slice segment header syntax element num_ref_idx_lX_active_minus1 of at least one of the slice segments of the direct reference layer picture is different from the slice segment header syntax element num_ref_idx_lX_active_minus1 of another slice segment of the direct reference layer picture;

–	For X being replaced by either 0 or 1, the reference picture list RefPicListX of at least one of the slice segments of the direct reference layer picture is different from the reference picture list RefPicListX of another slice segment of the direct reference layer picture.

    - Scaling, transformation and array construction process prior to deblocking filter process

The specifications in clause A.8.6 apply.

    - In-loop filter process

The specifications in clause A.8.7 apply.

  - Parsing process

The specifications in clause A.9 apply.

  - Specification of bitstream subsets

The specifications in clause A.10 and its subclauses apply.

  - Profiles, tiers, and levels

    - Profiles

      - Scalable Main and Scalable Main 10 profiles

For a layer in an output operation point associated with an OLS in a bitstream, the layer being conforming to the Scalable Main or Scalable Main 10 profile, the following applies:

–	Let olsIdx be the OLS index of the OLS.

–	The sub-bitstream subBitstream is derived as follows:

–	If OlsIdxToLsIdx[ olsIdx ] is less than or equal to vps_num_layer_sets_minus1, subBitstream is derived by invoking the sub-bitstream extraction process as specified in clause A.10.1 with the following inputs: the bitstream, tIdTarget equal to OpTid of the output operation point, and layerIdListTarget containing the nuh_layer_id value layerId of the layer and all the reference layers of the layer.

–	Otherwise, subBitstream is derived by invoking the sub-bitstream extraction process as specified in clause A.10.3 with tIdTarget equal to OpTid of the output operation point and with layerIdListTarget containing the nuh_layer_id value of the layer and all the reference layers of the layer.

–	The base layer sub-bitstream baseBitstream is derived as follows:

–	If VCL NAL units with nuh_layer_id equal to 0 are included in subBitstream, baseBitstream is derived by invoking the sub-bitstream extraction process as specified in clause A.10.1 with subBitstream, tIdTarget equal to OpTid of the output operation point, and layerIdListTarget containing only one nuh_layer_id value that is equal to 0 as inputs.

–	Otherwise, baseBitstream is derived by invoking the independent non-base layer rewriting process as specified in clause A.10.2 with subBitstream, tIdTarget equal to OpTid of the output operation point, and layerIdListTarget containing only the smallest nuh_layer_id value of the VCL NAL units of subBitstream as inputs.

The base layer sub-bitstream baseBitstream shall obey the following constraints:

- When the layer conforms to the Scalable Main profile, the base layer sub-bitstream baseBitstream shall be indicated to conform to the Main profile.

- When the layer conforms to the Scalable Main 10 profile, the base layer sub-bitstream baseBitstream shall be indicated to conform to the Main 10 profile or the Main profile.

The sub-bitstream subBitstream shall obey the following constraints:

- All active VPSs shall have vps_num_rep_formats_minus1 in the range of 0 to 15, inclusive.

- All active SPSs for layers in subBitstream shall have chroma_format_idc equal to 1 only.

- All active SPSs for layers in subBitstream shall have transform_skip_rotation_enabled_flag, transform_skip_context_enabled_flag, implicit_rdpcm_enabled_flag, explicit_rdpcm_enabled_flag, extended_precision_processing_flag, intra_smoothing_disabled_flag, high_precision_offsets_enabled_flag, persistent_rice_adaptation_enabled_flag, and cabac_bypass_alignment_enabled_flag, when present, equal to 0 only.

- CtbLog2SizeY derived from all active SPSs for layers in subBitstream shall be in the range of 4 to 6, inclusive.

- All active PPSs for layers in subBitstream shall have log2_max_transform_skip_block_size_minus2 and chroma_qp_offset_list_enabled_flag, when present, equal to 0 only.

- The variables NumRefLayerPicsProcessing, NumRefLayerPicsSampleProcessing, and NumRefLayerPicsMotionProcessing shall be less than or equal to 1 for each decoded picture with nuh_layer_id included in layerIdListTarget that was used to derive subBitstream.

- ScalabilityId[ j ][ smIdx ] derived according to any active VPS shall be equal to 0 for any smIdx value not equal to 2 or 3 and for any value of j such that layer_id_in_nuh[ j ] is among layerIdListTarget that was used to derive subBitstream.

- For a layer with nuh_layer_id iNuhLId equal to any value included in layerIdListTarget that was used to derive subBitstream, the value of NumRefLayers[ iNuhLId ], which specifies the total number of direct and indirect dependent layers and is derived as specified in A.7.4.3.1, shall be less than or equal to 4.

- All active SPSs for layers in subBitstream shall have sps_range_extensions_flag equal to 0 only and sps_extension_6bits equal to 0 only.

- All active PPSs for layers in subBitstream shall have pps_range_extensions_flag equal to 0 only and pps_extension_6bits equal to 0 only.

- When an active PPS for any layer in subBitstream has tiles_enabled_flag equal to 1, it shall have entropy_coding_sync_enabled_flag equal to 0.

- When an active PPS for any layer in subBitstream has tiles_enabled_flag equal to 1, ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 256 for all values of i in the range of 0 to num_tile_columns_minus1, inclusive, and RowHeightInLumaSamples[ j ] shall be greater than or equal to 64 for all values of j in the range of 0 to num_tile_rows_minus1, inclusive.

- The number of times read_bits( 1 ) is called in clauses 9.3.4.3.3 and 9.3.4.3.4 when parsing coding_tree_unit( ) data for any coding tree unit shall be less than or equal to 5 * RawCtuBits / 3.

- general_level_idc and sub_layer_level_idc[ i ] for all values of i in active SPSs for any layer in subBitstream shall not be equal to 255 (which indicates level 8.5).

- For any active VPS, DependencyId[ i ] shall be greater than DependencyId[ j ] for any values of i and j among layerIdListTarget that was used to derive subBitstream such that AuxId[ i ] is equal to AuxId[ j ] and i is greater than j.

When the layer conforms to the Scalable Main profile, the sub-bitstream subBitstream shall obey the following constraints:

- All active SPSs for layers in subBitstream shall have bit_depth_luma_minus8 equal to 0 only.

- All active SPSs for layers in subBitstream shall have bit_depth_chroma_minus8 equal to 0 only.

- All active PPSs for layers in subBitstream shall have colour_mapping_enabled_flag equal to 0 only.

- The level constraints specified for the Scalable Main profile in clause A.11.2 shall be fulfilled.

When the layer conforms to the Scalable Main 10 profile, the sub-bitstream subBitstream shall obey the following constraints:

- All active SPSs for layers in subBitstream shall have bit_depth_luma_minus8 in the range of 0 to 2, inclusive.

- All active SPSs for layers in subBitstream shall have bit_depth_chroma_minus8 in the range of 0 to 2, inclusive.

- The level constraints specified for the Scalable Main profile in clause A.11.2 shall be fulfilled.

In the remainder of this clause and clause A.11.2.1, all syntax elements in the profile_tier_level( ) syntax structure refer to those in the profile_tier_level( ) syntax structure associated with the layer.

Conformance of a layer in an output operation point associated with an OLS in a bitstream to the Scalable Main profile is indicated as follows:

- If OpTid of the output operation point is equal to vps_max_sub_layer_minus1, the conformance is indicated by general_profile_idc being equal to 7 or general_profile_compatibility_flag[ 7 ] being equal to 1, and general_max_12bit_constraint_flag being equal to 1, general_max_10bit_constraint_flag being equal to 1, general_max_8bit_constraint_flag being equal to 1, general_max_422chroma_constraint_flag being equal to 1, general_max_420chroma_constraint_flag being equal to 1, general_max_monochrome_constraint_flag being equal to 0, general_intra_constraint_flag being equal to 0, general_one_picture_only_constraint_flag being equal to 0, and general_lower_bit_rate_constraint_flag being equal to 1.

- Otherwise (OpTid of the output operation point is less than vps_max_sub_layer_minus1), the conformance is indicated by sub_layer_profile_idc[ OpTid ] being equal to 7 or sub_layer_profile_compatibility_flag[ OpTid ][ 7 ] being equal to 1, and sub_layer_max_12bit_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_10bit_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_8bit_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_422chroma_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_420chroma_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_monochrome_constraint_flag[ OpTid ] being equal to 0, sub_layer_intra_constraint_flag[ OpTid ] being equal to 0, and sub_layer_one_picture_only_constraint_flag[ OpTid ] being equal to 0, and sub_layer_lower_bit_rate_constraint_flag[ OpTid ] being equal to 1.

Conformance of a layer in an output operation point associated with an OLS in a bitstream to the Scalable Main 10 profile is indicated as follows:

- If OpTid of the output operation point is equal to vps_max_sub_layer_minus1, the conformance is indicated by general_profile_idc being equal to 7 or general_profile_compatibility_flag[ 7 ] being equal to 1, and general_max_12bit_constraint_flag being equal to 1, general_max_10bit_constraint_flag being equal to 1, general_max_8bit_constraint_flag being equal to 0, general_max_422chroma_constraint_flag being equal to 1, general_max_420chroma_constraint_flag being equal to 1, general_max_monochrome_constraint_flag being equal to 0, general_intra_constraint_flag being equal to 0, general_one_picture_only_constraint_flag being equal to 0, and general_lower_bit_rate_constraint_flag being equal to 1.

- Otherwise (OpTid of the output operation point is less than vps_max_sub_layer_minus1), the conformance is indicated by sub_layer_profile_idc[ OpTid ] being equal to 7 or sub_layer_profile_compatibility_flag[ OpTid ][ 7 ] being equal to 1, and sub_layer_max_12bit_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_10bit_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_8bit_constraint_flag[ OpTid ] being equal to 0, sub_layer_max_422chroma_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_420chroma_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_monochrome_constraint_flag[ OpTid ] being equal to 0, sub_layer_intra_constraint_flag[ OpTid ] being equal to 0, and sub_layer_one_picture_only_constraint_flag[ OpTid ] being equal to 0, and sub_layer_lower_bit_rate_constraint_flag[ OpTid ] being equal to 1.

    - Tiers and levels

      - General tier and level limits

For purposes of comparison of tier capabilities, the tier with general_tier_flag or sub_layer_tier_flag[ i ] equal to 0 is considered to be a lower tier than the tier with general_tier_flag or sub_layer_tier_flag[ i ] equal to 1.

For purposes of comparison of level capabilities, a particular level of a specific tier is considered to be a lower level than some other level of the same tier when the value of the general_level_idc or sub_layer_level_idc[ i ] of the particular level is less than that of the other level.

The following is specified for expressing the constraints in this clause and clause A.11.2.2:

–	For the Scalable Main profile, the value of each of the variables CpbVclFactor, CpbNalFactor, FormatCapabilityFactor, and MinCrScaleFactor is the same as that specified in Table A.6 for the Main profile. For the Scalable Main 10 profile, the value of each of these variables is the same as that specified in Table A.6 for the Main 10 profile.

- Let access unit n be the n-th access unit in decoding order, with the first access unit being access unit 0 (i.e., the 0-th access unit).

- Let the variable fR be set equal to 1  300.

- Let the variable olsIdx be the index of the OLS.

- For each layer with nuh_layer_id equal to currLayerId, let the variable layerSizeInSamplesY be derived as follows:

layerSizeInSamplesY = pic_width_vps_in_luma_samples * pic_height_vps_in_luma_samples	(H-128)

where pic_width_vps_in_luma_samples and pic_height_vps_in_luma_samples are found in the vps_rep_format_idx[ LayerIdxInVps[ currLayerId ] ]-th rep_format( ) syntax structure in the VPS.

Each layer with nuh_layer_id equal to currLayerId conforming to a profile at a specified tier and level shall obey the following constraints for each bitstream conformance test as specified in clause A.13, where "access unit" is used to denote the picture unit in the layer and the CPB is understood to be the BPB:

- The value of layerSizeInSamplesY shall be less than or equal to MaxLumaPs, where MaxLumaPs is specified in Table A.4 for the tier and level of the layer.

- The value of pic_width_vps_in_luma_samples of the vps_rep_format_idx[ LayerIdxInVps[ currLayerId ] ]-th rep_format( ) syntax structure in the VPS shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

- The value of pic_height_vps_in_luma_samples of the vps_rep_format_idx[ LayerIdxInVps[ currLayerId ] ]-th rep_format( ) syntax structure in the VPS shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

- The value of max_vps_dec_pic_buffering_minus1[ olsIdx ][ LayerIdxInVps[ currLayerId ] ][ HighestTid ] shall be less than or equal to MaxDpbSize as derived by Equation A-2, with PicSizeInSamplesY being replaced with layerSizeInSamplesY, for the tier and level of the layer.

- For level 5 and higher levels, the value of CtbSizeY for the layer shall be equal to 32 or 64.

- The value of NumPicTotalCurr for each picture in the layer shall be less than or equal to 8.

- When decoding each coded picture in the layer, the value of num_tile_columns_minus1 shall be less than MaxTileCols and num_tile_rows_minus1 shall be less than MaxTileRows, where MaxTileCols and MaxTileRows are specified in Table A.4 for the tier and level of the layer.

- For the VCL HRD parameters of the layer, CpbSize[ i ] shall be less than or equal to CpbVclFactor * MaxCPB for at least one of the delivery schedules identified by bsp_sched_idx[ olsIdx ][ 0 ][ HighestTid ][ combIdx ][ LayerIdxInVps[ currLayerId ] ] for combIdx ranging from 0 to num_bsp_schedules_minus1[ olsIdx ][ 0 ][ HighestTid ], inclusive, where CpbSize[ i ] is specified in clause A.13.1 and MaxCPB is specified in Table A.4 for the tier and level of the layer in units of CpbVclFactor bits.

- For the NAL HRD parameters of the layer, CpbSize[ i ] shall be less than or equal to CpbNalFactor * MaxCPB for at least one of the delivery schedules identified by bsp_sched_idx[ olsIdx ][ 0 ][ HighestTid ][ combIdx ][ LayerIdxInVps[ currLayerId ] ] for combIdx ranging from 0 to num_bsp_schedules_minus1[ olsIdx ][ 0 ][ HighestTid ], inclusive, where CpbSize[ i ] is specified in clause A.13.1 and MaxCPB is specified in Table A.4 for the tier and level of the layer in units of CpbNalFactor bits.

Table A.4 specifies the limits for each level of each tier for levels other than level 8.5.

A tier and level to which a layer in an output operation point associated with an OLS in a bitstream conforms are indicated by the syntax elements general_tier_flag and general_level_idc if OpTid of the output layer set is equal to vps_max_sub_layer_minus1, and by the syntax elements sub_layer_tier_flag[ OpTid ] and sub_layer_level_idc[ OpTid ] otherwise, as follows:

–	If the specified level is not level 8.5, general_tier_flag or sub_layer_tier_flag[ OpTid ] equal to 0 indicates conformance to the Main tier, and general_tier_flag or sub_layer_tier_flag[ OpTid ] equal to 1 indicates conformance to the High tier, according to the tier constraints specified in Table A.4, and general_tier_flag and sub_layer_tier_flag[ OpTid ] shall be equal to 0 for levels below level 4 (corresponding to the entries in Table A.4 marked with "-"). Otherwise (the specified level is level 8.5), it is a requirement of bitstream conformance that general_tier_flag and sub_layer_tier_flag[ OpTid ] shall be equal to 1 and the value 0 for general_tier_flag and sub_layer_tier_flag[ OpTid ] is reserved for future use by ITU-T | ISO/IEC, and decoders shall ignore the value of general_tier_flag and sub_layer_tier_flag[ OpTid ].

–	general_level_idc and sub_layer_level_idc[ OpTid ] shall be set equal to a value of 30 times the level number specified in Table A.4.

      - Profile specific tier and level limits for the Scalable Main and Scalable Main 10 profiles

The following is specified for expressing the constraints in this clause:

–	The variable HbrFactor is set equal to 1.

–	The variable BrVclFactor is set equal to CpbVclFactor * HbrFactor.

–	The variable BrNalFactor is set equal to CpbNalFactor * HbrFactor.

–	The variable MinCr is set equal to MinCrBase * MinCrScaleFactor ÷ HbrFactor, where MinCrBase is specified in Table A.5.

Each layer conforming to the Scalable Main or Scalable Main 10 profiles at a specified tier and level shall obey the following constraints for each conformance test as specified in clause A.13, where "access unit" is used to denote the picture unit in the layer, and the CPB is understood to be the BPB:

- The nominal removal time of access unit n (with n greater than 0) from the CPB, as specified in clause A.13.2.3, shall satisfy the constraint that AuNominalRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] is greater than or equal to Max( layerSizeInSamplesY  MaxLumaSr, fR ), where layerSizeInSamplesY is the value of layerSizeInSamplesY for access unit n − 1 and MaxLumaSr is the value specified in Table A.5 that applies to access unit n − 1 for the tier and level of the layer.

- The difference between consecutive output times of pictures in different access units, as specified in clause A.13.3.3 shall satisfy the constraint that DpbOutputInterval[ n ] is greater than or equal to Max( layerSizeInSamplesY  MaxLumaSr, fR ), where layerSizeInSamplesY is the value of layerSizeInSamplesY of access unit n and MaxLumaSr is the value specified in Table A.5 for access unit n for the tier and level of the layer, provided that access unit n is an access unit that has a picture that is output and is not the last of such access units.

- The removal time of access unit 0 shall satisfy the constraint that the number of coded slice segments in access unit 0 is less than or equal to Min( Max( 1, MaxSliceSegmentsPerPicture * MaxLumaSr / MaxLumaPs * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) + MaxSliceSegmentsPerPicture * layerSizeInSamplesY / MaxLumaPs ), MaxSliceSegmentsPerPicture ), for the value of layerSizeInSamplesY of access unit 0, where MaxSliceSegmentsPerPicture, MaxLumaPs, and MaxLumaSr are the values specified in Table A.4 and Table A.5 for the tier and level of the layer.

- The difference between consecutive CPB removal times of access units n and n − 1 (with n greater than 0) shall satisfy the constraint that the number of slice segments in access unit n is less than or equal to Min( ( Max( 1, MaxSliceSegmentsPerPicture * MaxLumaSr / MaxLumaPs * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ), MaxSliceSegmentsPerPicture ), where MaxSliceSegmentsPerPicture, MaxLumaPs, and MaxLumaSr are the values specified in Table A.4 and Table A.5 that apply to access unit n for the tier and level of the layer.

- For the VCL HRD parameters for the layer, BitRate[ i ] shall be less than or equal to BrVclFactor * MaxBR for at least one of the delivery schedules identified by bsp_sched_idx[ olsIdx ][ 0 ][ HighestTid ][ combIdx ][ LayerIdxInVps[ currLayerId ] ] for combIdx ranging from 0 to num_bsp_schedules_minus1[ olsIdx ][ 0 ][ HighestTid ], inclusive, where BitRate[ i ] is specified in clause A.13.1 and MaxBR is specified in Table A.5 in units of BrVclFactor bits/s for the tier and level of the layer.

- For the NAL HRD parameters for the layer, BitRate[ i ] shall be less than or equal to BrNalFactor * MaxBR for at least one of the delivery schedules identified by bsp_sched_idx[ olsIdx ][ 0 ][ HighestTid ][ combIdx ][ LayerIdxInVps[ currLayerId ] ] for combIdx ranging from 0 to num_bsp_schedules_minus1[ olsIdx ][ 0 ][ HighestTid ], inclusive, where BitRate[ i ] is specified in clause A.13.1 and MaxBR is specified in Table A.5 in units of BrNalFactor bits/s for the tier and level of the layer.

- The sum of the NumBytesInNalUnit variables for access unit 0 shall be less than or equal to FormatCapabilityFactor * ( Max( layerSizeInSamplesY, fR * MaxLumaSr ) + MaxLumaSr * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) ) ÷ MinCr for the value of layerSizeInSamplesY of access unit 0, where MaxLumaSr is specified in Table A.5, and both MaxLumaSr and FormatCapabilityFactor are the values that apply to access unit 0 for the tier and level of the layer.

- The sum of the NumBytesInNalUnit variables for access unit n (with n greater than 0) shall be less than or equal to FormatCapabilityFactor * MaxLumaSr * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ÷ MinCr, where MaxLumaSr is specified in Table A.5, and both MaxLumaSr and FormatCapabilityFactor are the values that apply to access unit n for the tier and level of the layer.

- The removal time of access unit 0 shall satisfy the constraint that the number of tiles in coded pictures in access unit 0 is less than or equal to Min( Max( 1, MaxTileCols * MaxTileRows * 120 * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) + MaxTileCols * MaxTileRows * PicSizeInSamplesY / MaxLumaPs ), MaxTileCols * MaxTileRows ), for the value of layerSizeInSamplesY of access unit 0, where MaxTileCols and MaxTileRows are the values specified in Table A.4 that apply to access unit 0 for the tier and level of the layer.

- The difference between consecutive CPB removal times of access units n and n − 1 (with n greater than 0) shall satisfy the constraint that the number of tiles in coded pictures in access unit n is less than or equal to Min( Max( 1, MaxTileCols * MaxTileRows * 120 * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ), MaxTileCols * MaxTileRows ), where MaxTileCols and MaxTileRows are the values specified in Table A.4 that apply to access unit n for the tier and level of the layer.

    - Decoder capabilities

When a decoder conforms to any profile specified in Annex A, it shall also have the INBLD capability specified in clause A.11.1.

Clause A.11.2 specifies requirements for a decoder conforming to any profile specified in Annex A.

  - Byte stream format

The specifications in clause A.12 apply.

  - Hypothetical reference decoder

The specifications in clause A.13 and its subclauses apply.

  - Supplemental enhancement information

The specifications in Annex D and clause A.14 and their subclauses apply.

  - Video usability information

The specifications in clause A.15 and its subclauses apply.