| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I1008 |
| --- | --- |

| Title: | MV-HEVC Conformance Draft 1 |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Draft |  |  |
| Author(s) or Contact(s): | Ying Chen Shinya Shimizu Teruhiko Suzuki | Tel: Email: | +1-858-845-6589 cheny@qti.qualcomm.com shimizu.shinya@lab.ntt.co.jp teruhikos@jp.sony.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document is a proposed draft 1 of conformance test of MV-HEVC.

The following text specifies the normative tests for verifying conformance of ITU-T H.265 | ISO/IEC 23008-2 video bitstreams and decoders conforming to the Multiview Main profile.

[Ed. Note: The conformance bitstreams are available at the following link (to be created):

http://wftp3.itu.int/av-arch/jct3v-site/bitstream_exchange/under_test/MV-HEVC/]

- MV-HEVC Conformance Draft 1

In subclause 6.5.7, add the following text at the end of the subclause:

A decoder that conforms to the Multiview Main profile at specific level shall be capable of decoding the specified bitstreams in Table 3. In addition to the bitstreams defined in Table 3, a decoder that conforms to the Multiview High profile shall be capable of decoding the Main and Main 10 profiles bitstreams specified in Table 1, and the Main 12, Main 4:2:2 10 profile bitstreams specified in Table 2.

In subclause 6.6, add the following text at the end of the subclause:

### 6.6.13	Test bitstreams – MV-HEVC

#### 6.6.13.1	Test bitstream #MVHEVCS-A

Specification: All slices are coded as I, P or B slices. Only the first picture of each view is coded as IDR picture. Each picture contains only one slice. NumViews is equal to 2. NumDirectRefLayers of the non-base view is equal to 1. For each picture in the non-base view, inter-view prediction is enabled. The two views are with the same spatial resolution. All NAL units are encapsulated into the byte stream format specified in ITU-T Rec. H.265 | ISO/IEC 23008-2.

Functional stage: Decoding of two views with inter-view prediction and inter-prediction.

Purpose: To conform the most normal case.

Contributor: Qualcomm

#### 6.6.13.2	Test bitstream #MVHEVCS-B

Specification: All slices of the base view are coded as I slices and all slices of the non-base view are coded with only inter-view prediction, thus P slices. Only the first picture of each view is coded as IDR picture. Each picture contains only one slice. NumViews is equal to 2. NumDirectRefLayers of the non-base view is equal to 1. The two views are with the same spatial resolution. All NAL units are encapsulated into the byte stream format specified in ITU-T Rec. H.265 | ISO/IEC 23008-2.

Functional stage: Decoding of two views with only inter-view prediction and intra-prediction.

Purpose: To conform the all-Intra case.

Contributor: Sharp.

#### 6.6.13.3	Test bitstream #MVHEVCS-C

Specification: All slices are coded as I, P or B slices. Only the first picture of each view is coded as IDR picture. Each picture contains only one slice. NumViews is equal to 2. NumDirectRefLayers is always equal to 0, meaning inter-view prediction is disabled. The two views are with the same spatial resolution. All NAL units are encapsulated into the byte stream format specified in ITU-T Rec. H.265 | ISO/IEC 23008-2.

Functional stage: Decoding of two views with only inter-prediction and intra-prediction.

Purpose: To conform the case of simulcast coding.

Contributor: Sony.

#### 6.6.13.4	Test bitstream #MVHEVCS-D

Specification: All slices are coded as I, P or B slices. Only the first picture of each view is coded as IDR picture. Each picture contains only one slice. NumViews is equal to 2. NumDirectRefLayers is always equal to 0, meaning inter-view prediction is disabled. The two views are with different spatial resolutions. All NAL units are encapsulated into the byte stream format specified in ITU-T Rec. H.265 | ISO/IEC 23008-2.

Functional stage: Decoding of two views with only inter-prediction and intra-prediction.

Purpose: To conform the case when different spatial resolutions for two views are used.

Contributor: NTT.

#### 6.6.13.5	Test bitstream #MVHEVCS-E

Specification: All slices are coded as I, P or B slices. Only the first picture of each view is coded as IDR picture and thus an IRAP access unit. In addition, every two GOPs start with an access unit which contains pictures that are all CRA pictures. Each picture contains only one slice. NumViews is equal to 2. NumDirectRefLayers of the non-base view is equal to 1. For each picture in the non-base view, inter-view prediction is enabled. The two views are with the same spatial resolution. All NAL units are encapsulated into the byte stream format specified in ITU-T Rec. H.265 | ISO/IEC 23008-2.

Functional stage: Decoding of two views with inter-view prediction and inter-prediction.

Purpose: To conform the random access hierarchical B case.

Contributor: Qualcomm.

#### 6.6.13.6	Test bitstream #MVHEVCS-F

Specification: All slices are coded as I, P or B slices. Only the first picture of each view is coded as IDR picture and thus an IRAP access unit. In addition, for each of every two following GOPs, an access unit which contains pictures that are all CRA pictures are requested. Other pictures are non-IRAP pictures. Each picture contains only one slice. NumViews is equal to 2. NumActiveRefLayerPics is equal to 1 for each picture in an IRAP access unit of the non-base view, and 0 otherwise, i.e., only for each picture in the non-base view in the IRAP access unit, inter-view prediction is enabled. The two views are with the same spatial resolution. All NAL units are encapsulated into the byte stream format specified in ITU-T Rec. H.265 | ISO/IEC 23008-2.

Functional stage: Decoding of two views with inter-view prediction only for random access points.

Purpose: To conform the flexibility of inter-view prediction applicability scope (inter-view pred. only for IRAP pictures).

Contributor: Qualcomm.

#### 6.6.13.7	Test bitstream #MVHEVCS-G

Specification: All slices are coded as I, P or B slices. Only the first picture of each view is coded as IDR picture and thus an IRAP access unit. In addition, every two GOPs start with an access unit which contains pictures that are all CRA pictures. Each picture contains only one slice. NumViews is equal to 3. NumDirectRefLayers of the non-base view is equal to 1. For each picture in the non-base view, inter-view prediction is enabled. The three views are with the same spatial resolution. All NAL units are encapsulated into the byte stream format specified in ITU-T Rec. H.265 | ISO/IEC 23008-2.

Functional stage: Decoding of two views with inter-view prediction and inter-prediction.

Purpose: To conform the random access hierarchical B case (random access 3-view case, PIP cfg.).

Contributor: NTT.

#### 6.6.13.8	Test bitstream #MVHEVCS-H

Specification: All slices are coded as I, P or B slices. Only the first picture of each view is coded as IDR picture and thus an IRAP access unit. In addition, every two GOPs start with an access unit which contains pictures that are all CRA pictures. Each picture contains only one slice. NumViews is equal to 3. NumDirectRefLayers of the first non-base view is equal to 1 and NumDirectRefLayers of the second non-base view is equal to 2. For each picture in the non-base view, inter-view prediction is enabled. The three views are with the same spatial resolution. All NAL units are encapsulated into the byte stream format specified in ITU-T Rec. H.265 | ISO/IEC 23008-2.

Functional stage: Decoding of two views with inter-view prediction and inter-prediction.

Purpose: To conform the random access hierarchical B case (H random access 3-view case, IBP cfg.).

Tentative contributor: LGE.

#### 6.6.13.9	Test bitstream #MVHEVCS-I

Specification: All slices are coded as I, P or B slices. Only the first picture of each view is coded as IDR picture and thus an IRAP access unit. In addition, every two GOPs start with an access unit which contains pictures that are all CRA pictures. Each picture contains only one slice. NumViews is equal to 3. NumDirectRefLayers of the first non-base view is equal to 1 and NumDirectRefLayers of the second non-base view is equal to 2. The P view is coded with viewOrderIdx equal to 1. For each picture in the non-base view, inter-view prediction is enabled. Each view contains both texture and depth and the views are with the same spatial resolution. All NAL units are encapsulated into the byte stream format specified in ITU-T Rec. H.265 | ISO/IEC 23008-2.

Functional stage: Decoding of three views with inter-view prediction and inter-prediction.

Purpose: To conform the random access hierarchical B case (random access 3-view case, IBP cfg. MV-HEVC plus depth).

Tentative contributor: Nokia.

Note that the vertical constraint applies only to the P view (for both texture and depth of this P view).

#### 6.6.13.10	Test bitstream #MVHEVCS-J

Specification: All slices are coded as I, P or B slices. For the non-base view, only the first picture of is coded as IDR picture and thus an IRAP access unit. In addition, every two GOPs start with an access unit which contains pictures that are all CRA pictures. Each picture contains only one slice. NumViews is equal to 2. NumDirectRefLayers of the non-base view is equal to 1. For each picture in the non-base view, inter-view prediction is enabled. The two views are with the same spatial resolution. The base view contains pictures derived by external means (may be set as dummy pictures). All NAL units are encapsulated into the byte stream format specified in ITU-T Rec. H.265 | ISO/IEC 23008-2.

Functional stage: Decoding of two views with the base view being external.

Purpose: To conform the functionality of hybrid scalability.

Tentative contributor: tbd.

| Table 3 – Bitstreams for Multiview Main profile |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Categories | Sub category | Bitstream | File name | Main | Main 10 | Main still picture |  | Main tier | Level | Frame rate (Frames/sec) |
| Prediction Structure (2-view) | Inter-view prediction | MVHEVCS-A | MVHEVCS_A_Qualcomm_5 | X | X |  |  | X | 5.1 and higher | tbd |
|  | All Intra | MVHEVCS-B | MVHEVCS_B_Sharp_5 | X | X |  |  | X | 5.1 and higher | tbd |
|  | Simulcast | MVHEVCS-C | MVHEVCS_C_Sony_5 | X | X |  |  | X | 5.1 and higher | tbd |
|  | Simulcast with asymmetric resolutions | MVHEVCS-D | MVHEVCS_D_NTT_5 | X | X |  |  | X | 5.1 and higher | tbd |
|  | Inter-view prediction and hierarchical B | MVHEVCS-E | MVHEVCS_E_Qualcomm_5 | X | X |  |  | X | 5.1 and higher | tbd |
|  | Inter-view prediction for IRAP AUs only | MVHEVCS-F | MVHEVCS_F_Qualcomm_5 | X | X |  |  | X | 5.1 and higher | tbd |
| Prediction Structure (3-view) | Inter-view prediction | MVHEVCS-G | MVHEVCS_G_NTT_5 | X | X |  |  | X | 5.1 and higher | tbd |
|  | Inter-view prediction with PIP view structure | MVHEVCS-H | MVHEVCS_H_LGE_5 | X | X |  |  | X | 5.1 and higher | tbd |
|  | Inter-view prediction with IBP view structure | MVHEVCS-I | MVHEVCS_I_Nokia_5 | X | X |  |  | X | 5.1 and higher | tbd |
| Hybrid scalability |  | MVHEVCS-J | MVHEVCS_J_XXX_5 | X | X |  |  | X | 5.1 and higher | tbd |

[Ed. (CY): the level values for the MV-HEVC bitstreams may need to be revisited.]