| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18-24 Oct. 2014 | Document: JCT3V-J0021 |
| --- | --- |

| Title: | Structured lookup table definition in DMM1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520, JAPAN | Tel: Email: | +81-43-299-8526 tsukuba.takeshi@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution proposes a structured LUT definition in DMM, which generate 16x16 to 32x32 pattern by scaling and position-shifting wedgelet pattern of 8x8 in order to reduce the storage size. Experimental result reportedly shows that 0.10% and 0.06% bdrate changes for synthesis in the CTC and all-intra configuration, respectively. It is asserted that this proposed method can achieve storage size to 5Kbytes without significant coding loss.

# Introduction

In the last meeting, a lookup table size reduction method for DMM1 was adopted [1], in which wedgelets pattern in 32x32 block can be generated on the fly in the decoding process by scaling the corresponding 16x16 wedgelet pattern. Although required storage was reduced to 40Kbytes, the size is considered still large. It is desirable to reduce the storage size of wedgelet pattern, which enable whole table being always in cache.

Table 1: Required storage in HTM12.0

| PU size | Number of wedgelet pattern | Number of bits for one wedgelet pattern | Total bits | Required Storage (Accumulated bits) |
| --- | --- | --- | --- | --- |
| 4×4 | 86 | 16 | 1,376 | 1,376 |
| 8×8 | 766 | 64 | 46,024 | 50,400 |
| 16×16 | 1,350 | 256 | 345,600 | 396,000 (=40Kbytes) |
| 32x32 | 1,350 | - | - | - |

# Proposed Method

It is proposed that

- Generate  16x16 and 32x32 wedgelet pattern by scaling 8x8 wedgelet pattern as shown in Figure 1 (a)

- Or Generate 16x16 and 32x32 wedgelet pattern by scaling and position-shifting 8x8 wedgelet pattern as shown in Figure 1(b).

Figure 1: Examples of generated wedgelet pattern

- Conceptually the derivation logic can be shown as follows:

- wedgePattern[ x ][ y ] = wedgePatternTable8x8[ wedge_full_tab_idx[ xTb ][ yTb ] ][ (x + offset) >> scale ][ (y + offset) >> scale ]

- where offset is 0 or scale, the variable scale is set to (log2(nTbs) – 3), and nTbs is 16 or 32.

As can be seen in Table 2, the storage size reduces from 40 Kbytes to 5 Kbytes, which can be stored in cache.

Table 2: Required storage in proposed method

| PU size | Number of wedgelet pattern | Number of bits for one wedgelet pattern | Total bits | Required Storage (Accumulated bits) |
| --- | --- | --- | --- | --- |
| 4×4 | 86 | 16 | 1,376 | 1,376 |
| 8×8 | 766 | 64 | 46,024 | 50,400 (=5Kbytes) |
| 16×16 | 1532(=766*2) | - | - | - |
| 32×32 | 1532(=766*2) | - | - | - |

# Experimental results

The common test condition specified in JCT3V-I1100 [2] is used for the evaluation. Table 1-2 show the simulation results of the proposed method, in which bdrate changes are 0.10% and 0.06% for synthesis in the CTC and all-intra configuration, suggesting the loss is small.

Table 2: Result under CTC

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.07% | -0.03% | -0.02% | -0.01% | 0.03% | 106.0% | 103.3% | 104.0% |
| Kendo | 0.00% | 0.04% | 0.05% | 0.03% | 0.05% | 0.10% | 101.9% | 100.5% | 100.1% |
| Newspaper_CC | 0.00% | -0.07% | 0.14% | 0.01% | 0.00% | 0.06% | 96.4% | 96.4% | 94.4% |
| GT_Fly | 0.00% | 0.15% | -0.05% | 0.01% | 0.00% | 0.02% | 101.5% | 99.8% | 99.1% |
| Poznan_Hall2 | 0.00% | 0.01% | 0.44% | 0.10% | 0.08% | 0.16% | 101.4% | 99.8% | 99.0% |
| Poznan_Street | 0.00% | -0.05% | 0.08% | 0.01% | 0.03% | 0.01% | 101.0% | 99.6% | 99.3% |
| Undo_Dancer | 0.00% | 0.17% | 0.15% | 0.04% | 0.08% | 0.15% | 96.2% | 96.6% | 94.7% |
| Shark | 0.00% | 0.31% | 0.30% | 0.06% | 0.09% | 0.30% | 100.9% | 100.7% | 98.8% |
| 1024x768 | 0.00% | -0.04% | 0.05% | 0.01% | 0.01% | 0.06% | 101.4% | 100.1% | 99.5% |
| 1920x1088 | 0.00% | 0.12% | 0.18% | 0.04% | 0.06% | 0.13% | 100.2% | 99.3% | 98.2% |
| average | 0.00% | 0.06% | 0.13% | 0.03% | 0.04% | 0.10% | 100.7% | 99.6% | 98.7% |

Table 3: Result under all-intra configuration

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.00% | 0.00% | 0.00% | 0.01% | 0.05% | 109.6% | 98.2% | 98.0% |
| Kendo | 0.00% | 0.00% | 0.00% | 0.00% | 0.02% | 0.05% | 112.3% | 98.9% | 99.4% |
| Newspaper_CC | 0.00% | 0.00% | 0.00% | 0.00% | 0.03% | 0.05% | 112.8% | 99.4% | 98.6% |
| GT_Fly | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.05% | 114.1% | 100.7% | 100.7% |
| Poznan_Hall2 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.01% | 104.9% | 95.7% | 94.6% |
| Poznan_Street | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.02% | 119.4% | 105.0% | 105.4% |
| Undo_Dancer | 0.00% | 0.00% | 0.00% | 0.00% | 0.04% | 0.11% | 115.0% | 101.1% | 100.7% |
| Shark | 0.00% | 0.00% | 0.00% | 0.00% | 0.04% | 0.17% | 107.1% | 97.7% | 97.0% |
| 1024x768 | 0.00% | 0.00% | 0.00% | 0.00% | 0.02% | 0.05% | 111.6% | 98.8% | 98.7% |
| 1920x1088 | 0.00% | 0.00% | 0.00% | 0.00% | 0.02% | 0.07% | 112.1% | 100.0% | 99.7% |
| average | 0.00% | 0.00% | 0.00% | 0.00% | 0.02% | 0.06% | 111.9% | 99.6% | 99.3% |

# Conclusion

It is asserted that the proposal can achieve storage size to 5Kbytes without significant coding loss. It is recommended to adopt it into the next version of 3D-HEVC.

# Patent rights declaration(s)

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- T. Tsukuba, T. Tomohiro, “Lookup table size reduction in DMM1”, JCT3V-I0110, July 2014.

- G. Tech, K. Wegner, Y. Chen and S. Yea, “3D-HEVC Draft Text 5,” JCT3V-I1001, July 2014.

- K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” JCT3V-I1101, July 2014.

- 3DV-HTM version 12.0: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-12.0/

# Text changes

- I.8.4.4.2.11 Specification of tables WedgePatternTable

NOTE 1 − Tables and values resulting from the processes specified in the following are independent of any information contained in the bitstream. Therefore the derivation process described in this subclause can be carried out once as part of the initialization of the decoding process. Alternatively, the tables and values can be stored within the decoder (read-only) memory as fixed lookup tables, such that the derivation process described in this subclause does not need to be implemented in the decoder at all.

The list WedgePatternTable[ log2BlkSize ] of binary partition patterns of size (1  <<  log2BlkSize )x(1  <<  log2BlkSize), the variable NumWedgePattern[ log2BlkSize ] specifying the number of binary partition patterns in list WedgePatternTable[ log2BlkSize ] are derived as specified in the following:

- For log2BlkSize ranging from 2 to 3  4, inclusive, the following applies:

  - The variable resShift is set equal to ( log2BlkSize  = =  4 ) ? 0 : 1.

  - The variable wBlkSize is set equal to ( 1  <<  ( log2BlkSize + resShift ) )

  - For wedgeOri in the range of 0 to 5, inclusive, the following ordered steps apply.

    - Depending on wedgeOri the variables xPosS, yPosS, xPosE, yPosE, xIncS, yIncS, xIncE, yIncE are derived as specified in Table Error: Reference source not found−3.

Table Error: Reference source not found−3 − Specification of xPosS, yPosS, xPosE, yPosE, xIncS, yIncS, xIncE, yIncE

| wedgeOri | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| xPosS | 0 | wBlkSize − 1 | wBlkSize − 1 | 0 | 0 | wBlkSize − 1 |
| yPosS | 0 | 0 | wBlkSize − 1 | wBlkSize − 1 | 0 | 0 |
| xPosE | 0 | wBlkSize − 1 | wBlkSize− 1 | 0 | 0 | 0 |
| yPosE | 0 | 0 | wBlkSize − 1 | wBlkSize − 1 | wBlkSize − 1 | 0 |
| xIncS | 1 | 0 | −1 | 0 | 1 | 0 |
| yIncS | 0 | 1 | 0 | −1 | 0 | 1 |
| xIncE | 0 | −1 | 0 | 1 | 1 | 0 |
| yIncE | 1 | 0 | −1 | 0 | 0 | 1 |

    - For m in the range of 0 to wBlkSize − 1, inclusive, the following applies:

      - For n in the range of 0 to wBlkSize − 1, inclusive,  the following applies:

        - The Wedgelet pattern generation process as specified in subclause Error: Reference source not found is invoked with patternSize being equal to ( 1  <<  log2BlkSize ), the variable resShift, variable wedgeOri, xS being equal to ( xPosS + m * xIncS ), yS being equal to ( yPosS + m * yIncS ), xE being equal to ( xPosE + n * xIncE ) and yE being equal to ( yPosE + n * yIncE ) as inputs, and the output is the binary array curWedgePattern.

        - The wedgelet pattern list insertion process as specified in subclause Error: Reference source not found is invoked with log2BlkSize, and the binary partition pattern curWedgePattern as inputs.

- NumWedgePattern[ 5 ] is set equal to NumWedgePattern[ 4 ].

- For k = 0..NumWedgePattern[ 5 ] − 1 the following applies:

  - For x, y =0..( 1  <<  5 ) − 1 the following applies:

    - WedgePatternTable[ 5 ][ k ][ x ][ y ] = WedgePatternTable[ 4 ][ k ][ x  >>  1 ][ y  >> 1 ]

NOTE 1 − WedgePatternTable[ 5 ] might be derived on-the-fly in the decoding process from WedgePatternTable[ 4 ].

- I.8.4.4.2.7 Specification of intra prediction mode INTRA_DMM_WFULL

Inputs to this process are:

- a sample location ( xTb, yTb ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

- the neighbouring samples p[ x ][ y ], with x = −1, y = − 1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

- a variable nTbS specifying the transform block size.

Output of this process is:

- the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1, are derived as specified by the following ordered steps:

  - The variable wedgePattern[ x ][ y ] with x, y =0..nTbS − 1, specifying a binary partition pattern is derived as.

      - wedgePattern = WedgePatternTable[ Log2( nTbS ) ][ wedge_full_tab_idx[ xTb ][ yTb ] ]	(Error: Reference source not found-1)

- If Log2( nTbS ) is less than 4,
The variable wedgePattern[ x ][ y ] with x, y =0..nTbS − 1, specifying a binary partition pattern is derived as:

- wedgePattern[ x ][ y ] = WedgePatternTable[ Log2( nTbS ) ][ wedge_full_tab_idx[ xTb ][ yTb ] ][ x ][ y ]

- Otherwise ( Log2( nTbs ) is equal to or greater than 4 ), the following applies:

- The variable scale, pelShiftFlag, ref_wedge_idx, offsetX, offsetY are derived as follows:

- scale = Log2( nTbs ) – 3

- pelShiftFlag = wedge_full_tab_idx[ xTb ][ yTb]>=NumWedgePattern[ 3 ] ? 1 : 0

- ref_wedge_idx = pelShiftFlag ? 
wedge_full_tab_idx – NumWedgePattern[3]: wedge_full_tab_idx[ xTb ][ yTb ]

- offsetX = pelShiftFlag ? scale : 0

- offsetY = pelShiftFlag ? scale : 0

- The variable wedgePattern[ x ][ y ] with x, y =0..nTbS − 1, specifying a binary partition pattern is derived as:

- wedgePattern[ x ][ y ] = WedgePatternTable[ 3 ][ ref_wedge_idx ][ x2 ][ y2 ] ,

- where x2 = clip3(0, nTbs-1, x + offsetX) >> scale, y2 = clip3(0, nTbs-1, y + offsetY) >> scale

  - The depth partition value derivation and assignment process as specified in subclause Error: Reference source not found is invoked with the neighbouring samples p[ x ][ y ], the binary pattern wedgePattern[ xTb ][ yTb ], the sample location ( xTb, yTb ), and the transform size nTbS as inputs, and the output is assigned to predSamples[ x ][ y ].