| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0025_r1 |
| --- | --- |

| Title: | Simplification of DMM table derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Takeshi Tsukuba 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520, JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to simplify the DMM table derivation process by rotating.  Experimental result reportedly shows 0.01 % and 0.00 % BD-rate changes in synthesis in CTC and all-intra configuration respectively.

Revision 1 fixed the all intra results in Undo Dancer and fixed a typo in the proposed WD.

# Introduction

The DMM table consists of six different orientations, in the range of wedgeOri =0..5. Currently each table has to be generated with a line segment process and orientation specific filling and sub-sampling process. However, since the table with the different orientation has the similar structure,  simple rotation is sufficient to produce. With this, we can remove orientation specific filling process and sub-sampling process

In addition to line segment and filling, pruning process is required for each generated pattern. If proposed rotating process is utilized, pruning process for rotating table can be exempted.

Taking account of the above problems, we propose to modify the table generation with

- reusing generated tables by rotation

- one filling

- one sub-sampling

- reduced pruning

# Proposed Method

It is proposes to generate as follows

- generate the tables with wedgeOri being 1 to 3 by rotating the table with wedgeOri being 0

- generate the tables with wedgeOri being 5 by rotating the table with wedgeOri being 4

- replace orientation specific sub-sampling with simple one sub-sampling in line segment

- replace orientation specific filling with one filling

- remove pruning for he table with wedgeOri being 1, 2, 3, and 5

- remove inverse pruning

# WD change (relative to JCT3V-I1001_v3)

- I.8.4.4.2.11 Specification of tables WedgePatternTable

NOTE 1 − Tables and values resulting from the processes specified in the following are independent of any information contained in the bitstream. Therefore the derivation process described in this subclause can be carried out once as part of the initialization of the decoding process. Alternatively, the tables and values can be stored within the decoder (read-only) memory as fixed lookup tables, such that the derivation process described in this subclause does not need to be implemented in the decoder at all.

The list WedgePatternTable[ log2BlkSize ] of binary partition patterns of size (1  <<  log2BlkSize )x(1  <<  log2BlkSize), the variable NumWedgePattern[ log2BlkSize ] specifying the number of binary partition patterns in list WedgePatternTable[ log2BlkSize ] are derived as specified in the following:

- For log2BlkSize ranging from 2 to 4, inclusive, the following applies:

  - The variable resShift is set equal to ( log2BlkSize  = =  4 ) ? 0 : 1.

  - The variable wBlkSize is set equal to ( 1  <<  ( log2BlkSize + resShift ) )

  - The patternSize is set equal to ( 1  <<  log2BlkSize )

  - The variables posWedgePattern is set equal to 0,

  - For wedgeOri in the range of 0 to 5, inclusive, the following ordered steps apply.

    - Depending on wedgeOri the variables xPosS, yPosS, xPosE, yPosE, xIncS, yIncS, xIncE, yIncE are derived as specified in Table Error: Reference source not found−1 − Specification of xPosS, yPosS, xPosE, yPosE, xIncS, yIncS, xIncE, yIncE.

- Table Error: Reference source not found−1 − Specification of xPosS, yPosS, xPosE, yPosE, xIncS, yIncS, xIncE, yIncE

| wedgeOri | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| xPosS | 0 | wBlkSize − 1 | wBlkSize − 1 | 0 | 0 | wBlkSize − 1 |
| yPosS | 0 | 0 | wBlkSize − 1 | wBlkSize − 1 | 0 | 0 |
| xPosE | 0 | wBlkSize − 1 | wBlkSize− 1 | 0 | 0 | 0 |
| yPosE | 0 | 0 | wBlkSize − 1 | wBlkSize − 1 | wBlkSize − 1 | 0 |
| xIncS | 1 | 0 | −1 | 0 | 1 | 0 |
| yIncS | 0 | 1 | 0 | −1 | 0 | 1 |
| xIncE | 0 | −1 | 0 | 1 | 1 | 0 |
| yIncE | 1 | 0 | −1 | 0 | 0 | 1 |

    - The variables startPos is set equal to posWedgePattern

    - If wedgeOri is equal to 0 or 4, the following applies:

      - For xSm in the range of 0 to wBlkSize − 1, inclusive, the following applies:

        - For n in the range of 0 to wBlkSize − 1, inclusive,  the following applies:

          - yS = 0

          - xE = (wedgeOri==0) ? 0 : n

          - yE = (wedgeOri==0) ? n : wBlkSize − 1

          - The Wedgelet pattern generation process as specified in subclause  is invoked with patternSize being equal to ( 1  <<  log2BlkSize ), the variable resShift, variable wedgeOri, xS, yS, xE, yExS being equal to ( xPosS + m * xIncS ), yS being equal to 0 ( yPosS + m * yIncS ), xE being equal to ( xPosE + n * xIncE ) and yE being equal to ( yPosE + n * yIncE ) as inputs, and the output is the binary array curWedgePattern.

          - The wedgelet pattern list insertion process as specified in subclause  is invoked with log2BlkSize, and the binary partition pattern curWedgePattern, wedgeOri and startPos as inputs, output is the variable posWedgePattern

      - The variables endPos is set equal to posWedgePattern - 1

    - Otherwise (wedgeOri is equal to 1..3 or wedgeOri is equal to 5), the following applies:

      - For pos in the range of startPos to endPos, inclusive, the following applies:

        - The Wedgelet pattern rotation process as specified in subclause  is invoked with the variable patternSize, wedgeOri, and the binary partition pattern WedgePatternTable[ log2BlkSize ][ pos ] as inputs, and the output is the binary array curWedgePattern.

        - The wedgelet pattern list insertion process as specified in subclause  is invoked with log2BlkSize, and the binary partition pattern curWedgePattern and wedgeOri as inputs, output is the variable posWedgePattern

- The wedgelet pattern list insertion process as specified in subclause  is invoked with log2BlkSize, and the binary partition pattern curWedgePattern as inputs.NumWedgePattern[ 5 ] is set equal to NumWedgePattern[ 4 ].

- For k = 0..NumWedgePattern[ 5 ] − 1 the following applies:

  - For x, y =0..( 1  <<  5 ) − 1 the following applies:

    - WedgePatternTable[ 5 ][ k ][ x ][ y ] = WedgePatternTable[ 4 ][ k ][ x  >>  1 ][ y  >> 1 ]

NOTE 1 − WedgePatternTable[ 5 ] might be derived on-the-fly in the decoding process from WedgePatternTable[ 4 ].

- I.8.4.4.2.12.1 Wedgelet pattern generation process

Inputs to this process are:

- a variable patternSize specifying the binary partition pattern size,

- a resolution shift value resShift specifying the precision of the wedgelet partition start and end positions relative to patternSize,

- a variable wedgeOri specifying the orientation identifier of the wedgelet pattern,

- a variable xS specifying the partition line start horizontal position,

- a variable yS specifying the partition line start vertical position,

- a variable xE specifying the partition line end horizontal position,

- a variable yE specifying the partition line end vertical position.

Output of this process is:

- binary array wedgePattern[ x ][ y ] of size ( patternSize )x( patternSize ).

- The variable curSize specifying the size of the current partition pattern is derived as follows:.

  - curSize = ( resShift  = =  1 ) ? ( patternSize  <<  1 ) : patternSize	(Error: Reference source not found-1)

- The values of variable wedgePatterncurPattern[ x ][ y ], are derived as specified by the following ordered steps.

  - For x, y = 0.. patternSizecurSize − 1, wedgePatterncurPattern[ x ][ y ] is set equal to 0.

  - The samples of the array wedgePatterncurPattern that form a line between ( xS, yS ) and ( xE, yE ) are set equal to 1 as specified in the following:

x0 = xS
		y0 = yS
		x1 = xE
		y1 = yE
		if( abs( yE − yS ) > abs( xE − xS ) ) {
			( x0, y0 ) = Swap( x0, y0 )
			( x1, y1 ) = Swap( x1, y1 )
		}
		if( x0 > x1 ) {
			( x0, x1 ) = Swap( x0, x1 )
			( y0, y1 ) = Swap( y0, y1 )
		}
		sumErr = 0
		posY = y0
		for( posX = x0; posX  <=  x1; posX ++ ) {
			if( abs( yE − yS ) > abs( xE − xS ) )  
				wedgePatterncurPattern[ posY  >> resShift ][ posX  >> resShift ] = 1 
			else 
				wedgePatterncurPattern[ posX >> resShift ][ posY   >> resShift ] = 1 
			sumErr  +=  ( abs( y1 − y0 )  <<  1 )
			if( sumErr  >=  ( x1 − x0 ) ) {
				posY  +=  ( y0 < y1 ) ? 1 : −1
				sumErr  −=  ( x1 − x0 )  <<  1
			}
		}

  - The samples of wedgePatterncurPattern belonging to the a smaller partition are set equal to 1 as specified in the following:

if( wedgeOri  = =  0 ) 
			for( iX = 0; iX < xS; iX ++ )  
				for( iY = 0; curPattern[ iX ][ iY ]  = =  0; iY++ )  
					curPattern[ iX ][ iY ] = 1
		else if( wedgeOri  = =  1 ) 
			for( iY = 0; iY < yS; iY++ )  
				for( iX = curSize − 1; curPattern[ iX ][ iY ]  = =  0 ; iX− − )
					curPattern[ iX ][ iY ] = 1
		else if( wedgeOri  = =  2 ) 
			for( iX = curSize − 1; iX > xS; iX− − ) 
				for( iY = curSize − 1; curPattern[ iX ][ iY ]  = =  0 ; iY− −)
					curPattern[ iX ][ iY ] = 1
		else if( wedgeOri  = =  3 ) 
			for( iY = curSize − 1; iY > yS; iY− − )  
				for( iX = 0; curPattern[ iX ][ iY ]  = =  0 ; iX++) 
					curPattern[ iX ][ iY ] = 1
		else if( wedgeOri  = =  4 ) && ( ( xS + xE ) < curSize ) ) 
			for( iY = 0; iY < curSize; iY++ ) 
				for( iX = 0; curPattern[ iX ][ iY ]  = = 0  ; iX++) 
					curPattern[ iX ][ iY ] = 1
		else if( wedgeOri  = =  4 )
		 	for( iY = 0; iY < curSize; iY++ )  
				for( iX = curSize − 1; curPattern[ iX ][ iY ]  = =  0 ; iX− −) 
					curPattern[ iX ][ iY ] = 1
		else if( wedgeOri  = =  5 )  &&  ( ( yS + yE ) < curSize ) )
			for( iX = 0; iX < curSize; iX ++ )  
				for( iY = 0; curPattern[ iX ][ iY ] = = 0 ; iY++ )
					curPattern[ iX ][ iY ] = 1
		else if( wedgeOri  = =  5 ) 
			for( iX = 0; iX < curSize; iX++ )  
				for( iY = curSize − 1; curPattern[ iX ][ iY ]  = =  0 ; iY− − )  
					curPattern[ iX ][ iY ] = 1

for( iY = 0; iY <= yE>> resShift; iY++ )
			for( iX = 0; iX <= patternSize − 1 || wedgePattern [ iX ][ iY ]  = =  0 ; iX++ )
				wedgePattern [ iX ][ iY ] = 1

  - The binary partition pattern wedgePattern[ x ][ y ], with x, y = 0..patternSize − 1, is derived as specified in the following:

    - If resShift is equal to 1, the following applies:

      - Depending on wedgeOri, the variables xOff and yOff are set as specified in Table Error: Reference source not found−2.

Table Error: Reference source not found−2 − Specification of xOff, yOff

| wedgeOri | ( xS + xE ) < curSize | xOff | yOff |
| --- | --- | --- | --- |
| 0 |  | 0 | 0 |
| 1 |  | 1 | 0 |
| 2 |  | 1 | 1 |
| 3 |  | 0 | 1 |
| 4 | 0 | 1 | 0 |
| 4 | 1 | 0 | 0 |
| 5 | 0 | 0 | 1 |
| 5 | 1 | 0 | 0 |

      - For x, y = 0 ..patternSize − 1 the following applies:

        - wedgePattern[ x ][ y ] = curPattern[ ( x  <<  1 ) + xOff ][ ( y  <<  1 ) + yOff ]	(Error: Reference source not found-2)

Otherwise (resShift is not equal to 1), wedgePattern is set equal to curPattern.

I.8.4.4.2.11.2 Wedgelet pattern rotation process

Inputs to this process are:

- a variable patternSize specifying the binary partition pattern size,

- a variable wedgeOri specifying the orientation identifier of the wedgelet pattern,

- a binary array refPattern[ x ][ y ] of size ( patternSize )x( patternSize ).

Output of this process is:

- binary array wedgePattern[ x ][ y ] of size ( patternSize )x( patternSize ).

- The values of variable wedgePattern[ x ][ y ] are derived as specified in the following:

- cInc = wedgeOri == 2 ? − 1 : 0;

- sInc = (wedgeOri == 1 ? − 1 : 1) * (1 + xInc);

- iOffset = cInc + sInc < 0 ? patternSize − 1 ? 0

- jOffset = −sInc + cInc < 0 ? patternSize − 1 ? 0

  - for( y = 0; y < patternSize; y++ )  
				for( x = 0; x < patternSize; x++ )  {

  - i = iOffset + cInc * x + sInc  * y

  - j = jOffset − sInc * x + cInc  * y

- wedgePattern[ x ][ y ] = refPattern [ iOffset ][ jOffset ] ? refPattern [ i ][ j ] : 1 -refPattern [ i ][ j ]

- }

- I.8.4.4.2.11.3 Wedgelet pattern list insertion process

Inputs to this process are:

- a variable log2BlkSize specifying the binary partition pattern size as ( 1  <<  log2BlkSize ),

- binary partition pattern wedgePattern[ x ][ y ], with x, y =0..( 1  <<  log2BlkSize ) − 1.

- a variable wedgeOri specifying the orientation identifier of the wedgelet pattern,

Output of this process is:

- a variable posWedgePattern.

- The variable isValidFlag specifying whether the binary partition pattern wedgePattern is added to the list WedgePatternTable[ log2BlkSize ] not is set 	equal to 0.

- If wedgeOri is equal to 0 or 4, the value of isValidFlag is derived as specified by the following ordered steps.

  - For x, y = 0..( 1  <<  log2BlkSize ) − 1 the following applies:

    - When wedgePattern[ x ][ y ] is not equal to wedgePattern[ 0 ][ 0 ] the flag isValidFlag is set to 1.

  - For k = 0..NumWedgePattern[ log2BlkSize ] − 1 the following applies:

    - The flag patIdenticalFlag is set equal to 1.

    - For x, y =0..( 1  <<  log2BlkSize ) − 1 the following applies:

      - When wedgePattern[ x ][ y ] is not equal to WedgePatternTable[ log2BlkSize ][ k ][ x ][ y ], patIdenticalFlag is set to 0.

    - When patIdenticalFlag is equal to 1, isValidFlag is set to 0.

  - For k = 0..NumWedgePattern[ log2BlkSize ] − 1 the following applies:

    - The flag patInvIdenticalFlag is set to 1.

    - For x, y =0..( 1  <<  log2BlkSize ) − 1 the following applies:

      - When wedgePattern[ x ][ y ] is equal to WedgePatternTable[ log2BlkSize ][ k ][ x ][ y ], patInvIdenticalFlag is set to 0.

    - When patInvIdenticalFlag is equal to 1, isValidFlag is set to 0.

- Otherwise (wedgeOri is equal to 1..3 or wedgeOri is equal to 5), the value of isValidFlag is set equal to 1.

- When isValidFlag is equal to 1, the following applies:

  - The pattern WedgePatternTable[ log2BlkSize ][ NumWedgePattern[ log2BlkSize ] ] is set equal to wedgePattern.

  - The value of NumWedgePattern[ log2BlkSize ] is increased by one.

The variable posWedgePattern is set equal to NumWedgePattern[ log2BlkSize ]

# Experimental results

CTC

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.1% | 0.01% | -0.01% | 0.04% | 100.0% | 102.0% | 100.0% |
| Kendo | 0.0% | -0.1% | 0.1% | 0.02% | 0.01% | 0.01% | 100.1% | 100.4% | 100.1% |
| Newspaper_CC | 0.0% | 0.2% | 0.0% | 0.00% | -0.02% | 0.02% | 100.2% | 101.2% | 100.7% |
| GT_Fly | 0.0% | 0.1% | -0.1% | 0.00% | 0.00% | -0.06% | 101.0% | 98.1% | 100.4% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.2% | 0.04% | 0.05% | 0.06% | 100.1% | 99.7% | 100.7% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.01% | 0.01% | 0.01% | 101.0% | 102.1% | 100.1% |
| Undo_Dancer | 0.0% | -0.1% | -0.1% | -0.02% | -0.02% | -0.06% | 100.3% | 100.6% | 99.9% |
| Shark | 0.0% | 0.1% | 0.1% | 0.02% | 0.02% | 0.03% | 101.2% | 100.1% | 99.5% |
| 1024x768 | 0.0% | 0.0% | 0.1% | 0.01% | -0.01% | 0.03% | 100.1% | 101.2% | 100.2% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.01% | 0.01% | 0.00% | 100.7% | 100.1% | 100.1% |
| average | 0.0% | 0.0% | 0.0% | 0.01% | 0.01% | 0.01% | 100.5% | 100.5% | 100.2% |

AI

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.02% | 99.7% | 98.9% | 100.2% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.00% | 0.01% | 0.01% | 100.4% | 94.6% | 94.5% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.04% | 101.1% | 100.2% | 96.3% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | -0.01% | 99.8% | 98.3% | 98.5% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.2% | 97.5% | 98.5% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 99.9% | 100.7% | 98.4% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | -0.02% | 99.9% | 100.8% | 100.1% |
| Shark | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.0% | 100.1% | 98.5% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.02% | 100.4% | 97.9% | 97.0% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | -0.01% | 100.0% | 99.5% | 98.8% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.1% | 98.9% | 98.1% |

# Conclusion

It is proposed to simplify the DMM table derivation process.  It is recommended to adopt this method in next 3D-HEVC and HTM.

# Patent rights declaration(s)

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).