| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0026 |
| --- | --- |

| Title: | VSP access improvement |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Takeshi Tsukuba 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520, JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to modify the VSP depth access position to improve VSP coding efficiency.

# Introduction

The adaptive depth access was proposed in JCT3V-I0105 but it was not accepted because the adaptive access increases complexity relatively largely compared with its benefit of 0.05 % average gain in synthesis. This is the follow up proposal which eliminates the adaptive part and may decrease complexity by reducing access and comparison. The benefit is almost the same at 0.04 % average gain.

# Proposed Method

It is proposed to use maximum of two depth in 8x4 / 4x8 subblock for VSP.

We argue that the rational of the horizontal two depths access are:

- Because 8x4 / 4x8 block is relatively small, four corner access may be too much for robustness but could harm the efficiency by suppressing the subblock characteristics.

- In VSP case, the vertical component of disparity vector is considered zero, so the plural vertical coordinates may not be needed for robustness. Note: Plural horizontal coordinate differences should curb the effect of inaccuracy of disparity vector.

# WD change (relative to JCT3V-I1001_v3)

- I.8.5.5.5.2 Derivation process for a disparity sample array

…

The array disparitySamples is derived as specified in the following:

- For sBy in the range of 0 to ( ( nPSH / nSubBlkH ) − 1 ), inclusive, the following applies:

  - For sBx in the range of 0 to ( ( nPSW / nSubBlkW ) − 1 ), inclusive, the following applies:

    - The variable maxDep is set equal to −1 and modified as specified in the following:

xSubB = sBx * nSubBlkW
			ySubB = sBy * nSubBlkH
			xP0 = Clip3( 0, pic_width_in_luma_samples − 1, xTL + xSubB ) 
			xP1 = Clip3( 0, pic_width_in_luma_samples − 1, yTL + xSubB + nSubBlkW − 1 )
			If partIdc is equal to 2, the following applies:
				yP0 = Clip3( 0, pic_height_in_luma_samples − 1, yTL + ySubB + nSubBlkH/2 − 1 ) 
				maxDep = Max(refDepPels[ xP0 ][ yP0 ], refDepPels[ xP1 ][ yP0 ] )
			Otherwise (partIdc is not equal to 2), the following applies:
				yP0 = Clip3( 0, pic_height_in_luma_samples − 1, yTL + ySubB ) 
				yP1 = Clip3( 0, pic_height_in_luma_samples − 1, yTL + ySubB + nSubBlkH − 1 )
				maxDep = Max( maxDep, refDepPels[ xP0 ][ yP0 ] )
				maxDep = Max( maxDep, refDepPels[ xP0 ][ yP1] )
				maxDep = Max( maxDep, refDepPels[ xP1][ yP0 ] )
				maxDep = Max( maxDep, refDepPels[ xP1][ yP1 ] )

    - The values of the array depthSamples are modified as specified in the following:

for ( yOff = 0; yOff < nSubBlkH; yOff++ )
				for( xOff = 0; xOff < nSubBlkW; xOff++ ) {
					x = xSubB + xOff 
					y = ySubB + yOff 
					disparitySamples[ x ][ y ] = DepthToDisparityB[ refViewIdx ][ maxDep ]
				}

# Experimental results

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.1% | -0.03% | -0.03% | -0.01% | 100.0% | 98.6% | 98.5% |
| Kendo | 0.0% | -0.1% | 0.0% | 0.01% | -0.01% | -0.01% | 99.9% | 101.1% | 99.8% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | -0.03% | -0.05% | -0.07% | 100.2% | 98.4% | 98.8% |
| GT_Fly | 0.0% | 0.1% | 0.0% | 0.01% | 0.00% | 0.02% | 100.3% | 99.3% | 99.7% |
| Poznan_Hall2 | 0.0% | -0.1% | 0.1% | -0.02% | -0.03% | 0.00% | 99.8% | 98.0% | 99.9% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.02% | 0.02% | -0.02% | 99.9% | 101.1% | 100.0% |
| Undo_Dancer | 0.0% | -1.3% | -1.0% | -0.32% | -0.31% | -0.23% | 100.0% | 101.1% | 99.8% |
| Shark | 0.0% | -0.2% | -0.4% | -0.07% | -0.06% | -0.03% | 100.1% | 99.2% | 99.6% |
| 1024x768 | 0.0% | -0.1% | 0.0% | -0.02% | -0.03% | -0.03% | 100.0% | 99.4% | 99.0% |
| 1920x1088 | 0.0% | -0.3% | -0.3% | -0.08% | -0.07% | -0.05% | 100.0% | 99.7% | 99.8% |
| average | 0.0% | -0.2% | -0.2% | -0.05% | -0.06% | -0.04% | 100.0% | 99.6% | 99.5% |

# Conclusion

It is proposed to improve VSP depth access, which shows slight coding gain with less complexity. It is recommended to adopt this method in the next 3D-HEVC and HTM.

# Patent rights declaration(s)

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- T. Ikai, T. Tsukuba, “Adaptive depth access for VSP enhancement”, JCT3V-I0105, 9th Meeting: Sapporo, JP, 3–9 July 2014.