| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0027 |
| --- | --- |

| Title: | Cleanup1: Derived DV derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Takeshi Tsukuba 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520, JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to remove SKIP mode specific decision in DV derivation process. It is reported that the BD-rate difference of the proposed method is 0.00 %, 0.00 % and 0.01 % in texture, video and synthesis respectively.

# Introduction

The current 3D-HEVC DV derivation process required a SKIP mode flag check. However it is considered that the typical HEVC implementation doesn’t have the flag, since there are no differences between SKIP and Inter merge in terms of motion parameter derivation process. However 3D-HEVC violates the above concept and increases implementation cost. We have tested that performance impact replacing SKIP mode check with merge mode check (SKIP or Merge), is negligible. It is strongly recommended to remove the special check.

# Proposed Method

It is proposed to use the merge mode check rather than SKIP mode check in derived disparity vector derivation process.

# WD change (relative to JCT3V-I1001_v3)

- I.8.5.5 Derivation process for disparity vectors

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable nCbS specifying the size of the current luma coding block,

The flag dvAvailFlag is set equal to 0, and both components of the disparity vector mvDisp are set equal to 0.

The variable checkDVParallelMergeFlag is derived as follows:

- If one or more of the following conditions are true, checkParallelMergeFlag is set equal to 1.

  - CuPredMode[ xCb ][ yCb ] is equal to MODE_SKIP.

  - CuPredMode[ xCb ][ yCb ] is equal to MODE_INTER and merge_flag[ xCb ][ yCb ] is equal to 1.

- Otherwise, checkParallelMergeFlag is set equal to 0.

The derivation process for a disparity vector from temporal neighbour block as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), and the variable nCbS as inputs, and the outputs are the flag dvAvailFlag, the disparity vector mvDisp and the reference view order index refViewIdx.

When dvAvailFlag is equal to 0, for each N being A1, B1 and ( xN, yN ) being ( xCb − 1,  yCb + nCbS − 1 ), ( xCb + nCbS − 1,  yCb − 1 ), respectively, the following ordered steps apply.

  - When yCb − 1 is less than ( ( yCb  >>  Log2CtbSizeY )  <<  Log2CtbSizeY ), the following applies:

        - xB1 = ( ( xB1  >>  3 )  <<  3 ) + ( ( xB1  >>  3 ) & 1) * 7	(Error: Reference source not found-1)

  - The derivation process for z-scan order block availability as specified in subclause 6.4.1 is invoked with ( xCurr, yCurr ) set equal to the ( xCb, yCb ) and the luma location ( xN, yN ) as the input and the output assigned to availableN.

  - When availableN is equal to 1 and CuPredMode[ xN ][ yN ] is equal to MODE_INTRA, availableN is set equal to 0.

  - When all of the following conditions are true, availableN is set equal to 0.

    - checkParallelMergeFlag is equal to 1

    - ( xCb  >>  ( log2_parallel_merge_level_minus2 + 2) ) is equal to
( xN  >>  ( log2_parallel_merge_level_minus2 + 2) )

    - ( yCb  >>  ( log2_parallel_merge_level_minus2 + 2) ) is equal to ( yN  >>  ( log2_parallel_merge_level_minus2 + 2) ).

  - The flag availableIvpMvSearchFlagN is set equal to availableN.

  - When one of the following conditions is true, N is equal to B1 and ( ( yN  >>  Log2CtbSizeY )  <<  Log2CtbSizeY ) is less than ( ( yCb >> Log2CtbSizeY )  <<  Log2CtbSizeY), availableIvpMvSearchFlagN is set equal to 0.

  - The flag availableFlagIvpMvN is set equal to 0.

  - For each X from 0 to 1, the following applies:

    - When dvAvailFlag is equal to 0, availableN is equal to 1, RefIdxLX[ xN ][ yN ] is greater than or equal to 0, and PredFlagLX[ xN ][ yN ] is equal to 1, the following applies:

      - If RefPicListX[ RefIdxLX[ xN ][ yN ] ] is an inter-view reference picture of the current picture, the following applies:

          - refViewIdx = ViewIdx( RefPicListX[ RefIdxLX[ xN ][ yN ] ] )	(Error: Reference source not found-2)

          - mvDisp = MvLXN[ xN ][ yN ]		(Error: Reference source not found-3)

          - dvAvailFlag = 1		(Error: Reference source not found-4)

      - Otherwise (RefPicListX[ RefIdxLX[ xN ][ yN ] ] is not an inter-view reference picture), the following applies:

        - The variable checkIvpMergeFlag is derived as follows:

          - If one or more of the following conditions are true, checkIvpMergeFlag is set equal to 1.

            - CuPredMode[ xN ][ xN ] is equal to MODE_SKIP.

            - CuPredMode[ xN ][ xN ] is equal to MODE_INTER and merge_flag[ xN ][ xN ] is equal to 1.

          - Otherwise, checkIvpMergeFlag is set equal to 0.

        - When availableIvpMvSearchFlagN is equal to 1, availableFlagIvpMvN is equal to 0, and CuPredMode[ xN ][ yN ] is equal to MODE_SKIPcheckIvpMergeFlag is equal to 1 and IvpMvFlag[ xN ][ yN ] is equal to 1, the following applies:

            - ivpMvDispN = MvRefinedDisp[ xN ][ yN ]	(Error: Reference source not found-5)

            - refViewIdxN = RefViewIdx[ xN ][ yN ]	(Error: Reference source not found-6)

            - availableFlagIvpMvN = 1		(Error: Reference source not found-7)

When dvAvailFlag is equal to 0 for each N being A1 and B1, the following applies:

  - When dvAvailFlag is equal to 0 and availableFlagIvpMvN is equal to 1, the following applies:

      - mvDisp = ivpMvDispN		(Error: Reference source not found-8)

      - refViewIdx = refViewIdxN		(Error: Reference source not found-9)

      - dvAvailFlag = 1		(Error: Reference source not found-10)

When dvAvailFlag is equal to 0, refViewIdx is set equal to DefaultRefViewIdx, and mvDisp is set equal to ( 0, 0 ).

Depending on depth_refinement_flag[ nuh_layer_id ], the following applies:

- If depth_refinement_flag[ nuh_layer_id ] is equal to 1, the following applies:

  - The derivation process for a disparity sample array as specified in subclause Error: Reference source not found is invoked with the luma locations xCb, yCb, the disparity vector mvDisp, the view identifier refViewIdx, the variable nPSW equal to nCbS, the variable nPSH equal to nCbS, and the variable partIdc equal to 0 as inputs, and the output is the array disparitySamples of size (nCbS)x(nCbS).

  - The disparity vector mvRefinedDisp is set equal to ( disparitySamples[ 0 ][ 0 ], 0 ).

- Otherwise (depth_refinement_flag[ nuh_layer_id ] is equal to 0), the disparity vector mvRefinedDisp is set equal to mvDisp.

For use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x = xCb.. ( xCb + nCbS − 1 ), y = yCb..( yCb + nCbS− 1 ):

  - MvDisp[ x ][ y ] = mvDisp		(Error: Reference source not found-11)

  - MvRefinedDisp[ x ][ y ] = mvRefinedDisp		(Error: Reference source not found-12)

  - RefViewIdx[ x ][ y ] = refViewIdx		(Error: Reference source not found-13)

  - DispAvailabilityIdc[ x ][ y ] = dvAvailFlag ? DISP_AVAILABLE : 
	( DefaultRefViewIdxAvailableFlag ? DISP_DEFAULT : DISP_NONE )	(Error: Reference source not found-14)

# Experimental results

CTC

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.00% | 0.01% | 0.03% | 100.5% | 100.2% | 98.6% |
| Kendo | 0.0% | -0.1% | -0.1% | -0.01% | -0.02% | -0.03% | 100.6% | 99.6% | 100.1% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | -0.01% | -0.01% | 0.02% | 100.9% | 100.0% | 99.2% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.01% | 100.2% | 99.3% | 100.4% |
| Poznan_Hall2 | 0.0% | 0.1% | 0.0% | 0.03% | 0.04% | 0.01% | 99.9% | 98.9% | 100.3% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.01% | 0.01% | -0.01% | 100.4% | 101.1% | 99.5% |
| Undo_Dancer | 0.0% | -0.2% | 0.1% | 0.00% | 0.00% | 0.02% | 100.4% | 101.0% | 99.9% |
| Shark | 0.0% | 0.0% | 0.0% | 0.00% | 0.01% | 0.01% | 100.1% | 99.9% | 99.8% |
| 1024x768 | 0.0% | 0.0% | 0.0% | -0.01% | 0.00% | 0.01% | 100.7% | 99.9% | 99.3% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.01% | 0.01% | 0.01% | 100.2% | 100.0% | 100.0% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.01% | 100.4% | 100.0% | 99.7% |

Depth off (--ViewSynthesisPred=0 --DepthRefinement=0 --DBBP=0)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | -0.1% | -0.02% | -0.02% | 0.00% | 99.7% | 102.8% | 100.3% |
| Kendo | 0.0% | 0.0% | -0.1% | -0.02% | -0.01% | 0.01% | 99.8% | 97.6% | 97.5% |
| Newspaper_CC | 0.0% | 0.0% | -0.1% | -0.01% | 0.00% | -0.02% | 100.1% | 105.5% | 101.8% |
| GT_Fly | 0.0% | 0.0% | -0.1% | -0.01% | 0.00% | 0.00% | 99.9% | 100.7% | 99.8% |
| Poznan_Hall2 | 0.0% | 0.1% | 0.0% | 0.02% | 0.01% | 0.02% | 99.4% | 100.9% | 100.6% |
| Poznan_Street | 0.0% | 0.0% | -0.1% | -0.01% | -0.01% | -0.02% | 99.6% | 99.8% | 100.6% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | -0.01% | 0.00% | 0.01% | 99.9% | 98.3% | 99.9% |
| Shark | 0.0% | 0.0% | -0.1% | -0.01% | 0.00% | -0.01% | 100.0% | 100.6% | 100.2% |
| 1024x768 | 0.0% | 0.0% | -0.1% | -0.02% | -0.01% | 0.00% | 99.9% | 102.0% | 99.9% |
| 1920x1088 | 0.0% | 0.0% | -0.1% | 0.00% | 0.00% | 0.00% | 99.7% | 100.1% | 100.2% |
| average | 0.0% | 0.0% | -0.1% | -0.01% | 0.00% | 0.00% | 99.8% | 100.8% | 100.1% |

# Conclusion

It is proposed to remove SKIP mode specific decision in DV derivation process. Since there is no specific coding performance impact, it is recommended to adopt this cleanup in the next 3D-HEVC and HTM.

# Patent rights declaration(s)

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).