| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0028 |
| --- | --- |

| Title: | Cleanup2: Default DV derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Takeshi Tsukuba 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520, JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to modify the default DV derivation process. It is reported that the difference was not observed in the simulation by the cleanup.

# Introduction

The default DV is derived at the slice level and used when there are no temporal or spatial neighbouring DVs. The current derivation process prioritizes the smaller ViewIdx to decide default reference view order index. However the prioritizing may harm coding efficiency and increase complexity.

# Proposed Method

It is proposing to search reference picture list without smaller ViewIdx prioritizing. With this, the earlier one in the reference picture list becomes the default reference.

# WD change (relative to JCT3V-I1001_v3)

      - Derivation process for the default reference view order index for disparity derivation

This process is invoked when the current slice is a P or B slice.

The variable DefaultRefViewIdx is set equal to −1, the variable DefaultRefViewIdxAvailableFlag is set equal to 0, and the following applies for curViewIdx in the range of 0 to ( ViewIdx − 1 ), inclusive:

- The following applies for X in the range of 0 to 1, inclusive:

  - When X is equal to 0 or the current slice is a B slice, the following applies for i in the range of 0 to NumRefPicsLX −1, inclusive:

    - When all of the following conditions are true, DefaultRefViewIdx is set equal to curViewIdx of the RefPicListX[ i ] and DefaultRefViewIdxAvailableFlag is set equal to 1.

      - DefaultRefViewIdxAvailableFlag is equal to 0.

      - ViewIdx( RefPicListX[ i ] ) is not equal to curViewIdx.

      - PicOrderCnt( RefPicListX[ i ] ) is equal to PicOrderCntVal.

# Experimental results

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.2% | 98.8% | 99.5% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.5% | 102.0% | 99.4% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.7% | 101.4% | 100.3% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.0% | 98.7% | 99.8% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.0% | 99.2% | 100.3% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 99.9% | 100.3% | 100.6% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 99.7% | 100.9% | 99.9% |
| Shark | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.2% | 100.7% | 99.6% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.5% | 100.7% | 99.7% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.0% | 99.9% | 100.0% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.2% | 100.2% | 99.9% |

# Conclusion

It is recommended to adopt this method in the next 3D-HEVC and HTM.

# Patent rights declaration(s)

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).