| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0030 |
| --- | --- |

| Title: | Cleanup4: Remove DDD |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Takeshi Tsukuba 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520, JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to remove DDD merge candidate. It is reported that the BD-rate difference of the proposed method is 0.00 %, -0.02 % and 0.01 % in texture, video and synthesis respectively.

# Introduction

DDD merge candidate has been adopted. However, after the adoption the gain of DDD decreases due to the several subsequent modifications, especially single depth mode. It seems single depth mode can cover most of DDD benefits.

# Proposed Method

It is proposed to remove DDD.

# WD change (relative to JCT3V-I1001_v3)

The following shows the highlight by the change. Other changes are also needed.

  - The merging candidate list, extMergeCandList, is constructed as follows:

i = 0
			if( availableFlagT )
				extMergeCandList[ i++ ] = T
			if( availableFlagD )
				extMergeCandList[ i++ ] = D
			if( availableFlagIvMC  &&  ( !availableFlagT  | |  differentMotion( T, IvMC ) ) )
				extMergeCandList[ i++ ] = IvMC
			N = DepthFlag ? T : IvMC
			if( availableFlagA1  &&  ( !availableFlagN  | |  differentMotion( N, A1 ) ) )
				extMergeCandList[ i++ ] = A1
			if( availableFlagB1  &&  ( !availableFlagN  | |  differentMotion( N, B1 ) ) )
				extMergeCandList[ i++ ] = B1
			if( availableFlagB0 )
				extMergeCandList[ i++ ] = B0	(Error: Reference source not found-1)
			if( availableFlagIvDC  &&  ( !availableFlagA1  | |  differentMotion( A1, IvDC ) )  &&  
				( !availableFlagB1  | |  differentMotion( B1, IvDC ) )  &&  ( i < ( 5 + NumExtraMergeCand ) ) )
				extMergeCandList[ i++ ] = IvDC
			if( availableFlagVSP  &&  !ic_flag  &&  iv_res_pred_weight_idx  = =  0  &&  
									i < ( 5 + NumExtraMergeCand ) )
				extMergeCandList[ i++ ] = VSP			if( availableFlagA0  &&  i < ( 5 + NumExtraMergeCand ) )
				extMergeCandList[ i++ ] = A0
			if( availableFlagB2  &&  i < ( 5 + NumExtraMergeCand ) )
				extMergeCandList[ i++ ] = B2
			if( availableFlagIvMCShift  &&  i < ( 5 + NumExtraMergeCand )  &&
								( !availableFlagIvMC  | |  differentMotion( IvMC, IvMCShift ) ) )
				extMergeCandList[ i++ ] = IvMCShift
			if( availableFlagIvDCShift  &&  i < ( 5 + NumExtraMergeCand ) )
				extMergeCandList[ i++ ] = IvDCShift
			j = 0
			while( i < MaxNumMergeCand ) {	(Error: Reference source not found-2)
				N = baseMergeCandList[ j++ ]
				if( N  !=  A1  &&  N  !=  B1  &&  N  !=  B0  &&  N  !=  A0  &&  N  !=  B2 )
					extMergeCandList[ i++ ] = N
			}

- I.8.5.3.2.20 Derivation process for the disparity derived merging candidate

This process is not invoked when mpi_flag[ nuh_layer_id ] is equal to 0.

Inputs to this process are:

- a luma location ( xPb, yPb ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- two variables nPbW and nPbH specifying the width and the height of the current prediction block,

Outputs of this process are (with X being replaced by 0 and 1):

- the flags availableFlagD specifying whether the disparity derived merging candidate is available,

- the prediction utilization flag predFlagLXD,

- the reference index refIdxLXD,

- the motion vector mvLXD,

- the depth value dispDerivedDepthVal.

The variable availableFlagD is set equal to 0. For X in the range of 0 and 1, inclusive, the variable predFlagLXD is set equal to 0, the variable refIdxLXD is set equal to −1 and both components of the motion vector mvLXD are set equal to 0.

The derivation process for a texture merging candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, and the variable mvAccFlag being equal to 1 as inputs, and the outputs are the flags availableFlagT, predFlagLXT, the reference index refIdxLXT, and the motion vector mvLXT (with X being replaced by 0 and 1).

For X in the range of 0 to 1, inclusive, when X is equal to 0 or the current slice is a B slice, the following applies:

- When predFlagLXT is equal to 1 and availableFlagD is equal to 0, and PicOrderCnt( RefPicListX[ refIdxLXT ] ) is equal to PicOrderCntVal, the disparity derived merging candidate is derived as follows:

    - mvLXD = mvLXT		(Error: Reference source not found-3)

    - refIdxLXD = refIdxLXT		(Error: Reference source not found-4)

    - predFlagLXD = 1		(Error: Reference source not found-5)

    - availableFlagD = 1		(Error: Reference source not found-6)

    - refViewIdx = ViewIdx( RefPicListX[ refIdxLXD ] )	(Error: Reference source not found-7)

    - dispVal = mvLXD[ xRef ][ yRef ][ 0 ] 		(Error: Reference source not found-8)

    - dispDerivedDepthVal = DispToDepthF( refViewIdx, dispVal )	(Error: Reference source not found-9)

# Experimental results

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.00% | -0.03% | -0.01% | 99.8% | 98.1% | 97.4% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.00% | -0.02% | 0.00% | 99.4% | 95.3% | 97.4% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.00% | -0.04% | -0.06% | 100.6% | 105.1% | 100.7% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.00% | -0.02% | 0.10% | 100.1% | 99.2% | 99.9% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.00% | -0.04% | -0.03% | 99.9% | 99.4% | 100.5% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.01% | 100.0% | 103.4% | 100.5% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.00% | -0.03% | 0.00% | 100.1% | 100.8% | 100.3% |
| Shark | 0.0% | 0.0% | 0.0% | 0.00% | -0.01% | 0.06% | 100.2% | 98.9% | 99.5% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | -0.03% | -0.02% | 100.0% | 99.5% | 98.5% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.00% | -0.02% | 0.03% | 100.1% | 100.3% | 100.1% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | -0.02% | 0.01% | 100.0% | 100.0% | 99.5% |

# Conclusion

It is proposed to remove DDD candidate. Since DDD doesn’t show the sufficient benefits, it is recommended to remove DDD in the next 3D-HEVC and HTM.

# Patent rights declaration(s)

SHARP Corporation does not have any current or pending patent rights relating to the technology described in this contribution.