| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18-24 Oct. 2014 | Document: JCT3V-J0031 |
| --- | --- |

| Title: | CE1: Cross-check results on Test 2 and Test 4 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520, JAPAN | Tel: Email: | +81-43-299-8526 tsukuba.takeshi@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-check results for Test 2 and Test 4 of JCT3V-J0032 on segmental prediction by MediaTek. Experimental results under CTC exactly matched the results provided by the proponent.

# Introduction

JCT3V-J0032 (CE1) proposes a multi segmental prediction method for depth coding with combing inter-SDC and additionally also intra-SDC. The source code provided by the proponents was inspected and matched the WD provided by the proponents. Simulations for Test 2 and Test 4 as shown in Table 1 were performed under CTC described in JCT3V-I1101.

Table 1: summary of Tests for CE1

| Test | N segments | Multi segmental prediction with Intra-SDC | Multi segmental prediction with Inter-SDC |
| --- | --- | --- | --- |
| Test 2 | 2 or 3 | Yes | Yes |
| Test 4 | 2 | No | Yes |

# Results

Table 2 and Table 3 summarizes results of Test 2 and Test 4, respectively. The results exactly matched those provided by the proponents.

Table 2: Result for Test 2 under CTC

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.02% | 0.03% | 0.01% | 0.12% | -0.32% | 108.9% | 102.7% | 102.0% |
| Kendo | 0.00% | -0.10% | 0.06% | 0.01% | 0.10% | -0.48% | 110.1% | 102.7% | 102.3% |
| Newspaper_CC | 0.00% | -0.08% | -0.05% | -0.03% | 0.13% | -0.45% | 103.8% | 98.4% | 96.3% |
| GT_Fly | 0.00% | 0.03% | -0.20% | -0.03% | 0.06% | -0.51% | 104.9% | 100.0% | 98.4% |
| Poznan_Hall2 | 0.00% | 0.14% | -0.02% | 0.02% | 0.20% | -0.69% | 106.1% | 102.0% | 100.1% |
| Poznan_Street | 0.00% | -0.01% | -0.10% | -0.02% | 0.03% | -0.09% | 111.9% | 104.6% | 103.5% |
| Undo_Dancer | 0.00% | -0.05% | -0.10% | -0.01% | 0.07% | -0.30% | 101.6% | 98.6% | 96.0% |
| Shark | 0.00% | -0.08% | -0.23% | -0.03% | 0.08% | -0.89% | 102.3% | 98.3% | 96.2% |
| 1024x768 | 0.00% | -0.06% | 0.01% | 0.00% | 0.12% | -0.41% | 107.6% | 101.3% | 100.2% |
| 1920x1088 | 0.00% | 0.01% | -0.13% | -0.01% | 0.09% | -0.49% | 105.4% | 100.7% | 98.8% |
| average | 0.00% | -0.02% | -0.08% | -0.01% | 0.10% | -0.46% | 106.2% | 100.9% | 99.3% |

Table 3: Result for Test 4 under CTC

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.15% | 0.08% | 0.04% | 0.10% | -0.13% | 105.4% | 101.2% | 101.4% |
| Kendo | 0.00% | -0.04% | 0.16% | 0.03% | 0.10% | -0.31% | 106.4% | 102.5% | 102.7% |
| Newspaper_CC | 0.00% | -0.08% | 0.00% | -0.01% | 0.04% | -0.34% | 100.0% | 97.9% | 96.1% |
| GT_Fly | 0.00% | -0.10% | -0.14% | -0.03% | 0.03% | -0.45% | 105.6% | 102.3% | 101.8% |
| Poznan_Hall2 | 0.00% | 0.22% | 0.10% | 0.05% | 0.17% | -0.31% | 102.4% | 100.3% | 99.3% |
| Poznan_Street | 0.00% | 0.06% | 0.03% | 0.01% | 0.02% | -0.03% | 105.3% | 100.8% | 100.3% |
| Undo_Dancer | 0.00% | -0.03% | -0.06% | -0.01% | 0.04% | -0.23% | 97.6% | 97.7% | 96.5% |
| Shark | 0.00% | -0.13% | -0.25% | -0.03% | 0.02% | -0.78% | 102.8% | 100.3% | 100.5% |
| 1024x768 | 0.00% | 0.01% | 0.08% | 0.02% | 0.08% | -0.26% | 103.9% | 100.5% | 100.1% |
| 1920x1088 | 0.00% | 0.00% | -0.06% | 0.00% | 0.06% | -0.36% | 102.7% | 100.3% | 99.7% |
| average | 0.00% | 0.00% | -0.01% | 0.01% | 0.07% | -0.32% | 103.2% | 100.4% | 99.8% |

# Conclusion

The source code and WD provided by the MediaTek were confirmed. Cross-check results for Test 2 and Test 4 exactly matched those provided by the proponent.