| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0032 |
| --- | --- |

| Title: | 3D-CE1: Segmental prediction in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Jicheng An1, Xianguo Zhang1, Han Huang1, Jian-Liang Lin2, Shawmin Lei2 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Kai Zhang +86-010-56900888 ext. 83890 {kai.zhang, jicheng.an, xianguo.zhang, h.huang, jl.lin, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents the results of CE1 in which a segmental prediction method for Inter-SDC and Intra-SDC is proposed for depth coding. Besides the current Inter-SDC mode, a segmental Inter-SDC mode with 2 or 3 segments is added. For each segment, a single depth value is derived and assigned to all reconstructed samples within that segment. As an extension, the same concept is also applied to Intra-SDC. The experimental results reportedly show that the segmental Inter-SDC achieves 0.5% BD-rate saving for the synthesized views in the common test condition and segmental Inter-SDC and segmental Intra-SDC together can achieve 0.6% BD-rate saving for the synthesized views.

# Introduction

Inter-SDC is a special prediction mode for depth coding in 3D-HEVC [1]. When Inter-SDC is applied, a normal inter-prediction is conducted first and a coded offset is added to all prediction samples to get the reconstructed sample values.

In a depth picture, the foreground and background usually hold very different brightness with sharp edges between them as two examples demonstrated in Fig. 1 and Fig. 2. Because the coding process loses high-frequency information generally, the quality of reconstruction is deteriorated more when there are acute pixel value changes. The coding efficiency of Inter-SDC will be reduced when such reconstructed pixels are referred to as the prediction samples.

Fig. 1 A block with two segments with sharp sample value changes.

Fig. 2 A block with three segments with sharp sample value changes.

# Proposed method

In this contribution, a segmental prediction method for Inter-SDC (segInter-SDC) is proposed. For a CU which is coded with Inter-SDC mode, a flag is signaled to indicate whether to apply the normal Inter-SDC or the segInter-SDC. When the segInter-SDC is enabled, two steps are applied to obtain the reconstructed block from the prediction block.

In the first step, samples in the current block are classified into one, two or three segments. When the current block is classified as one segment, all samples in the current block are classified as segment 0. When the prediction block is divided into two segments, a sample is classified as segment 0 if its value is smaller than threshold T which is the average value of all the samples in the prediction block. Otherwise, it is classified as segment 1. When the prediction block is divided into three segments, two thresholds T1 and T2 are calculated as T1= (T+Vmin)/2 and T2= (Vmax+T)/2, where Vmax and Vmin are the minimum and the maximum sample value in the prediction block, respectively. A sample is classified as segment 0 if its value is smaller than T1; it is classified as segment 2 if its value is larger than T2; otherwise it is classified as segment 1.

In the second step, a single value is derived for each segment in the reconstructed block. A single value denoted as V is calculated as V = E + O, where E is an estimated value for the segment of the prediction block and O is an offset transmitted to the decoder. If there is only one segment, E is set equal to average value of all the samples in the prediction block. Otherwise, E = Vsegpred, where Vsegpred is the sample value appearing most often in the segment of the prediction block. At encoder, the offset for each segment is calculated by subtracting the average value of all the original sample values in the segment of the current block and E. If segmental prediction is on, the number of segments, 1, 2 or 3 is signaled explicitly, and the offset is signaled in the same way as DcOffset in the current 3D-HEVC. An example of segInter-SDC with 2 segments for an 8x8 block is illustrated in Fig. 3.

The segmental prediction method can also be extended to Intra-SDC, termed as segIntra-SDC. The segIntra-SDC is performed in the same way as segInter-SDC except that the prediction block is produced by intra-prediction.

Fig. 3 An example of segInter-SDC with 2 segments for an 8x8 block.

# Experimental results

The proposed method is integrated on HTM-12.0 [2]. Six tests as described in Table 1 are conducted under the common test condition (CTC) [3], and the results are shown in Table 2-Table 7, respectively. The results show that the one segment mode does not contribute to the coding gain. The segInter-SDC with two and three segments could achieve the best performance with 0.5% BD-rate saving for the synthesized views. When segIntra-SDC is also applied, the coding gain goes up to 0.6% BD-rate saving.

Table 1.  Configuration for tests

|  | 1 segment | 2 segments | 3 segments | For Inter-SDC | For Intra-SDC |
| --- | --- | --- | --- | --- | --- |
| Test 1 | Yes | Yes | Yes | Yes | No |
| Test 2 | No | Yes | Yes | Yes | No |
| Test 3 | Yes | Yes | No | Yes | No |
| Test 4 | No | Yes | No | Yes | No |
| Test 5 | No | Yes | Yes | Yes | Yes |
| Test 6 | No | Yes | No | Yes | Yes |

Table 2.  Performance for the test 1 (CTC)

Table 3.  Performance for the test 2 (CTC)

Table 4.  Performance for the test 3 (CTC)

Table 5.  Performance for the test 4 (CTC)

Table 6.  Performance for the test 5 (CTC)

Table 7.  Performance for the test 6 (CTC)

# Conclusion

In the proposed segmental Inter-SDC and Intra-SDC modes, a single reconstructed sample value is derived and assigned to all reconstructed samples within a segment. The experimental results show that proposed method can achieve 0.6% BD-rate saving for the synthesized views in the common test condition.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 5,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-I1001, July. 2014.

- HTM-12.0, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-12.0.

- K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G1100, Jan. 2014.