| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0034 |
| --- | --- |

| Title: | Complexity reduction on illumination compensation for 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xianguo Zhang1, Kai Zhang1, Jicheng An1, Han Huang1, Jian-Liang Lin2, Shawmin Lei2 1MediaTek Beijing: Building 1-B, No. 6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {xianguo.zhang, kai.zhang, jicheng.an, h.huang, jl.lin, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, for each block coded with illumination compensation (IC), the LLS(linear least square) method is utilized to calculate α and β parameters for each color component. To reduce the usage of LLS which requires numbers of additional multiplication operations and memory accessing, this contribution proposes to disable bi-prediction for IC and set α as 1 for chroma components. Experimental results reportedly show the proposed simplifications only introduces 0.01% BD-rate increase whereas saves the operation number and bandwidth in worst case.

# Introduction

In the current working draft (WD) text 5 for 3D-HEVC [1] and HTM12.0 [2], IC utilizes LLS procedures to calculate α and β parameters for all luma components and >8x8 CU’s chroma components[5] and each prediction direction. The samples utilized for training α and β parameters are down-sampled 2N samples from the 2N left samples and 2N above samples.

Let x(i) and y(i) be the i-th training sample among the total 2N ones for α calculation in the reference block and current block respectively, let yi denote the i-th predicted sample in the 2Nx2N prediction block and xi denote the i-th predicted sample in the 2Nx2N reference block, the LLS procedure is shown as the following equations.

According to the WD text 5 and JCT3V-I0111[3], the operation analysis for the current IC procedures is listed in Table 1, where the CU size is 2Nx2N. The operation calculation procedure can be seen from Appendix I.

Table 1. Operation analysis for IC procedures in JCT3V-I0111 [3]

| Operations used for IC | ADD+CMP | MUL (most are general ones) |
| --- | --- | --- |
| for luma in one prediction direction | 4N2+8N+10 | 4N2+4N+5 |
| for bi-predicted 8x8 CU (chroma IC is disabled) | 8N2+16N+20 | 8N2+8N+10 |
| for bi-predicted >8x8 CUs | 12N2+32N+60 | 12N2+16N+30 |

Following the same complexity and bandwidth analysis in [3], we make a new analysis on the latest Draft Text 5. As shown in Table 2, IC becomes the worst case on operation complexity. Especially, in a general system where general multiplication is time-consuming, IC will be surely the worst case for the decoding time of a 3D-HEVC decoder.

Table 2. Complexity analysis of Draft Text 5

# Proposed simplifications

In this proposal, two simplifications are proposed

Simplification 1: Disable bi-prediction for IC.

Simplification 2: Set α as 1 for chroma components.

With these two simplifications, as shown in Table 3, 50% ADD/MUL operations can be saved. For the overall complexity analysis, IC turns to be a non-worst case, as shown in Table 4.

Table 3. Operation comparison for the proposed IC

| Operations used for proposed IC | ADD+CMP | MUL (most are general ones) |
| --- | --- | --- |
| for 8x8 CU (chroma and bi-prediction IC is disabled) | 4N2+8N+10 | 4N2+4N+5 |
| for >8x8 CUs (bi-prediction IC is disabled, only set alpha=32 for chroma) | 6N2+12N+12 (4N+2 is for chroma offset calculation) | 4N2+4N+5 |

Table 4. Complexity analysis for the WD text with the simplified IC

- Experimental results

The proposed method is conducted based on HTM-12.0 [2] under the common test condictions (CTC) [4]. As shown in Table 3, the experiments results show that the proposed simplifications on IC brings only 0.01% BD-rate increase, while the numbers of additional multiplication operations and memory accessing could be significantly reduced.

Table 3.  The BD-rate results of the proposed method over HTM12.0

- Text Specification Changes

I.7.4.9.6 Coding unit extension semantics

- It is a requirement of bitstream conformance that, when dbbp_flag[ x0 ][ y0 ] or ic_flag[ x0 ] [ y0 ] is equal to 1, inter_pred_idc[ x0 ][ y0 ] shall not be equal to PRED_BI.

I.8.5.3.2.2 Derivation process for luma motion vectors for merge mode

- 11.  The variable subPbMotionFlag is derived as specified in the following:

      - subPbMotionFlag = ( ( N  = =  IvMC )	  | |  vspModeFlag )  && !dbbp_flag  && !ic_flag		(Error: Reference source not found-1)

- 12.  The following assignments are made with X being replaced by 0 or 1:

      - mvLX = subPbMotionFlag ? 0 : mvLXN		(Error: Reference source not found-2)

      - refIdxLX = subPbMotionFlag ? −1 : refIdxLXN	(Error: Reference source not found-3)

      - predFlagLX = subPbMotionFlag ? 0 : predFlagLXN	(Error: Reference source not found-4)

- I.8.5.5.3.3.6.1 Derivation process for illumination compensation mode availability and parameters

- For X being replaced by 0 and 1, when puIcFlagLX is equal to 1, the variables icWeightLX, and icOffsetLX are derived by the following ordered steps:

- 1. ……

- 2. ……

- 3. The derivation process for illumination compensation parameters as specified in subclause  is invoked, with the list of neighbouring samples in the current picture curNeighSampleList, the list of neighbouring samples in the reference picture refNeighSample, a variable cIdx specifying colour component index and the number of neighbouring samples numNeighSamlesLX as inputs and the illumination parameters icWeightLX, and icOffsetLX as outputs.

- I.8.5.3.3.6.2 Derivation process for illumination compensation parameters

Inputs to this process are:

- a list curSampleList specifying the current samples,

- a list refSampleList specifying the reference samples,

- a variable numSamples specifying the number of elements in curSampleList and refSampleList,

- a bit depth of samples, bitDepth,

- a variable cIdx specifying colour component index.

The value of variable divCoeff is derived from Error: Reference source not found depending on sDenomDiv and the variables icWeight, and icOffset are derived as follows:

  - icWeight = cIdx ?1 << 5: ( sNumerDiv * divCoeff )  >>  ( shiftDenom − shiftNumer + 10 )	(Error: Reference source not found-5)

- icOffset = ( sumCur − ( ( icWeight * sumRef )  >>  5 ) + avgOffset )  >>  avgShift	(Error: Reference source not found-6)

# Patent rights declarations

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, "3D-HEVC Draft Text 5," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-I1001, July. 2014

- HTM-12.0, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-12.0

- Tomohiro Ikai, " AHG7: Complexity assessment of inter-prediction tools," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-I0111, July, 2014.

- D. Rusanovksyy, A. Vetro, and K. Müller, "Common Test Conditions for 3D Video Extensions Development," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-E1100, July, 2013.

- Tomohiro Ikai, "CE4-related: Resampling in IC parameter derivation and 4x4 Chroma removal," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-I0111, July, 2014.

# APPENDIX I: The operation calculation for Table 1

Table 1. Operation analysis for IC procedures in JCT3V-I0111 [3]

| Operations used for IC | ADD+CMP | MUL (most are general ones) |
| --- | --- | --- |
| for luma in one prediction direction | 4N2+8N+10 | 4N2+4N+5 |
| for bi-predicted 8x8 CU (chroma IC is disabled) | 8N2+16N+20 | 8N2+8N+10 |
| for bi-predicted >8x8 CUs | 12N2+32N+60 | 12N2+16N+30 |

- Counting procedures for the ( 4N2+8N+10) ADD+CMP of luma IC in one direction

- CU size is 2Nx2N, y(0)~y(2N-1) is the down-sampled current block’s left column and above row, x(0)~x(2N-1) is the down-sampled reference block’s left column and above row

- 4N2 indicates the “+” in  for the whole 2Nx2N matrix.

- 8N indicates the 4 kinds of ∑+ (size is 2N) operations in the above equations.

- 10 indicates the about 10 ADD+CMP operations in WD Text from I-212 to I-221

The variable avgShift and avgOffset are derived as follows:

  - avgShift = Log2( numSamples / 2)	 (Error: Reference source not found-7)

  - avgOffset = 1  <<  ( avgShift − 1 )	 (Error: Reference source not found-8)

The variables numerDiv and denomDiv are derived as follows:

  - denomDiv= ( ( sumRefSquare + ( sumRefSquare  >>  7 ) )  <<  avgShift )
	                                      − ( sumRef * sumRef )  >>  precShift	(Error: Reference source not found-9)

  - numerDiv= Clip3( 0, 2 * denomDiv, ( ( sumProdRefCur + ( sumRefSquare  >>  7 ) )  <<  avgShift )
	                                      − ( sumRef * sumCur )  >>  precShift )	(Error: Reference source not found-10)

The variables shiftNumer and shiftDenom are derived as follows:

  - shiftDenom = Max( 0, Floor( Log2( Abs( denomDiv ) ) ) − 5 )	(Error: Reference source not found-11)

  - shiftNumer = Max( 0, shiftDenom − 12 )	 (Error: Reference source not found-12)

The variables sNumerDiv and sDenomDiv are derived as follows:

  - sDenomDiv = denomDiv  >>  shiftDenom	 (Error: Reference source not found-13)

  - sNumerDiv = numerDiv  >>  shiftNumer	 (Error: Reference source not found-14)

The value of variable divCoeff is derived from Error: Reference source not found depending on sDenomDiv and the variables icWeight, and icOffset are derived as follows:

  - icWeight = ( sNumerDiv * divCoeff )  >>  ( shiftDenom − shiftNumer + 10 )	(Error: Reference source not found-15)

- icOffset = ( sumCur − ( ( icWeight * sumRef )  >>  5 ) + avgOffset )  >>  avgShift	(Error: Reference source not found-16)

- Counting procedures for the ( 4N2+4N+5) ADD+CMP of luma IC in one direction

- CU size is 2Nx2N, y(0)~y(2N-1) is the down-sampled current block’s left column and above row, x(0)~x(2N-1) is the down-sampled reference block’s left column and above row

- 4N2 indicates the “×” in  for the whole 2Nx2N matrix.

- 4N indicates the 2 ∑×(size is 2N) operations in the above equations.

- 5 indicates the 5 MUL operations in WD Text from I-212 to I-221

  - denomDiv= ( ( sumRefSquare + ( sumRefSquare  >>  7 ) )  <<  avgShift )
                        − ( sumRef * sumRef )  >>  precShift	(Error: Reference source not found-17)

  - numerDiv= Clip3( 0, 2 * denomDiv, ( ( sumProdRefCur + ( sumRefSquare  >>  7 ) )  <<  avgShift ) − ( sumRef * sumCur )  >>  precShift )	(Error: Reference source not found-18)

  - icWeight = ( sNumerDiv * divCoeff )  >>  ( shiftDenom − shiftNumer + 10 )         (Error: Reference source not found-19)

  - icOffset = ( sumCur − ( ( icWeight * sumRef )  >>  5 ) + avgOffset )  >>  avgShift  (Error: Reference source not found-20)

- Counting procedures for the (8N2+16N+20) ADD+CMP of 8x8 Bi-predicted IC in one direction

(4N2+8N+10)*2 = 8N2+16N+20

- Counting procedures for the (12N2+32N+60) ADD+CMP of >8x8 Bi-predicted IC in one direction

1)	For luma and chroma components, the results is

4N2 * 3/2 + 8N*2 +10*3 = 6N2+16N+30

2)	When utilizing bi-prediction

(6N2+16N+30)*2 = 12N2+32N+60

- Counting procedures for the (8N2+8N+10) ADD+CMP of 8x8 Bi-predicted IC in one direction

(4N2+4N+5)*2 = 8N2+8N+10

- Counting procedures for the (12N2+16N+30) ADD+CMP of >8x8 Bi-predicted IC in one direction

1)	For luma and chroma components, the results is

4N2 * 3/2 + 4N*2 +5*3 = 6N2+8N+15

2)	When utilizing bi-prediction

(6N2+8N+15)*2 = 12N2+16N+30