| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0035 |
| --- | --- |

| Title: | On Lookup Table Size Reduction for DMM1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xianguo Zhang1, Kai Zhang1, Jicheng An1, Han Huang1, Jian-Liang Lin2, Shawmin Lei2 1MediaTek Beijing: Building 1-B, No. 6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {xianguo.zhang, kai.zhang, jicheng.an, h.huang, jl.lin, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

For the DMM1 intra prediction, it requires 400k-bit storage to store the lookup table in the current design. In this contribution, it is proposed to downsample the wedgelet patterns for DMM1 to reduce the lookup table size. Experimental results reportedly show that the proposed simplification could achieve more than 50% storage saving of the wedgelet patterns with negligible BD-rate increase.

# Introduction

In order to reduce the size for the DMM1 lookup table in current 3D-HEVC [1], while accessing the wedgelet patterns, intra 32x32 PU re-uses the lookup table of 16x16 PU to obtain the starting and ending point of the wedgelet segmentation [4]. However, even with this simplification, the lookup table information for the current DMM1’s wedgelet patterns still requires about 400k bits storage as listed in Table 1.

Table 1. Number of bits for wedgelet pattern at different block size in the current HTM12

| PU size | Number of wedgelet pattern | Bins for decoding the index | Number of bits for one wedgelet pattern | Total Table Size(bits) | Accumulated Table Size (bits) | bits ratio [%] |
| --- | --- | --- | --- | --- | --- | --- |
| 4×4 | 86 | 7 | 16 | 1,376 | 1,376 | 0.35%=1376/396000 |
| 8×8 | 766 | 10 | 64 | 46,024 | 50,400 | 11.62%=46024/396000 |
| >=16×16 | 1,350 | 11 | 256 | 345,600 | 396,000 | 87.27%=345600/396000 |

# Proposed method

In this contribution, we propose to downsample the lookup table size for PU>=16x16. The proposed simplification is illustrated as Fig. 1 and described as follows:

- For the wedgelet patterns with starting point and ending point located in adjacent edges: only the ones with even-position (relative to the corresponding corner point) starting and ending points are kept.

- For the wedgelet patterns with starting point and ending point located in opposite edges: with even-x-or-y-coordinate starting points are kept.

Figure 1. Two categories of wedgelet segmentations

The number of wedgelet patterns with this proposed simplification for PU>=16x16 is shown in Table 3. As shown in the table, this proposed method could achieve 54.3% total table size saving.

Table 3. Table size for wedgelet pattern with this proposed simplificaiton

| PU size | Number of wedgelet pattern | Bins for decoding the index | Number of bits for one wedgelet pattern | Total Table Size(bits) | Accumulated Table Size (bits) | Table size saving [%] |
| --- | --- | --- | --- | --- | --- | --- |
| 4×4 | 86 | 7 | 16 | 1,376 | 1,376 | (396,000-180,960) /396,000 = 54.3% |
| 8×8 | 766 | 10 | 64 | 46,024 | 50,400 |  |
| >=16×16 | 510 | 9 | 256 | 130,560 | 180,960 |  |

- Experimental results

The proposed simplification is conducted based on HTM-12.0 [2]. The simulation results under the common test conditions (CTC) [3] and all intra cases are shown in Table 4 and Table 5, respectively. As shown in the results, the simplification only introduces 0.02% BD-rate loss for CTC and no loss for all intra cases.

Table 4.  The results under all intra test condictions

Table 5.  The results under CTC

# Text specification changes

Table Error: Reference source not found-1 –Values of wedgeFullTabIdxBits[ log2PUSize ]

| Initialization variable | wedgeFullTabIdxBits |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| log2PbSize | 2 | 3 | 4 | 5 | 6 |
| Value | 7 | 10 | 119 | 119 | 13 |

    - sizeScaleS = 1+ (log2BlkSize > 3 ) ;

    - sizeScaleE = wedgeOri<4 ?1+ (log2BlkSize > 3 ) : 1;

    - For m in the range of 0 to wBlkSize / sizeScaleS − 1, inclusive, the following applies:

      - For n in the range of 0 to wBlkSize/ sizeScaleE − 1, inclusive,  the following applies:

        - The Wedgelet pattern generation process as specified in subclause Error: Reference source not found is invoked with patternSize being equal to ( 1  <<  log2BlkSize ), the variable resShift, variable wedgeOri, xS being equal to ( xPosS + m*sizeScaleS* xIncS ), yS being equal to ( yPosS + m*sizeScaleS*yIncS ), xE being equal to ( xPosE + n*sizeScaleE * xIncE ) and yE being equal to ( yPosE + n*sizeScaleE  * yIncE ) as inputs, and the output is the binary array curWedgePattern.

        - The wedgelet pattern list insertion process as specified in subclause Error: Reference source not found is invoked with log2BlkSize, and the binary partition pattern curWedgePattern as inputs.

# Conclusion

It is proposed to further reduce the look up table size of DMM1 by partially down sampling. Experimental results show this method saves more than 50% of the total table size whereas produces negligible BD-rate increase.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, "3D-HEVC Draft Text 5," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-I1001, July. 2014

- HTM-12.0, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-12.0

- D. Rusanovksyy, A. Vetro, and K. Müller, "Common Test Conditions for 3D Video Extensions Development," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-E1100, July, 2013.

- Tomohiro Ikai, "Lookup table size reduction in DMM1," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-I0110, July, 2014.