| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0036 |
| --- | --- |

| Title: | Reduction of Worst Case Memory Bandwidth in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

Currently the external memory bandwidth requirement of 3D-HEVC in the worst case is larger than the worst case of HEVC because 3D-HEVC needs to additionally access a depth map compared to HEVC. The worst case of 3D-HEVC occurs when the bi-predictive 8x8 PU, which directly or indirectly uses a depth-based disparity vector (DoNBDV), is applied. Therefore, it is proposed to use NBDV instead of DoNBDV in 8x8 CU to avoid the additional external memory access compared to HEVC in the worst case. With the proposed method, the worst case memory bandwidth of 3D-HEVC becomes the same as the worst case of HEVC. The proposed method reportedly provides no coding loss.

# Introduction

In the current 3D-HEVC [1], the external memory bandwidth requirement of 3D-HEVC in the worst case is larger than the worst case of HEVC because 3D-HEVC needs to additionally access a depth map compared to HEVC. Table 1 shows the external memory bandwidth of 3D-HEVC in the worst case depending on used memory patterns. As can be seen in Table 1, the external memory bandwidth in the worst case is 114% compared to HEVC. That is, the additional depth access increases the memory bandwidth by 14%.

Table 1. Worst case memory bandwidth of 3D-HEVC (when compared to the worst case of HEVC)

| Memory Pattern | External Memory Bandwidth |
| --- | --- |
| 1x1 | 110% |
| 4x2 | 113% |
| 8x2 | 114% |

# Proposed Method

The worst case of 3D-HEVC occurs when the bi-predictive 8x8 PU, which directly or indirectly uses a depth-based disparity vector (DoNBDV), is applied. For instance, DoNBDV is directly used in the inter-view merge candidates (IVMC, IVDC, shifted IVMC, shifted IVDC) and DoNBDV is indirectly used in merge candidates followed by IVMC because the availability of IVMC, which is the first merge candidate, is decided by using DoNBDV. Therefore, in this contribution it is proposed, when the current CU size is 8x8, to use NBDV instead of DoNBDV to avoid the additional external memory bandwidth compared to HEVC in the worst case. With the proposed method, the worst case memory bandwidth of 3D-HEVC becomes the same as the worst case of HEVC.

The disparity derivation process in the 3D-HEVC working draft can be changed as follows. The proposed method only allows the depth-based disparity vector derivation when the current CU size (nCbS) is greater than 8.

I.8.5.5 Derivation process for disparity vectors … Depending on depth_refinement_flag[ nuh_layer_id ] and nCbS, the following applies: - If depth_refinement_flag[ nuh_layer_id ] is equal to 1 and nCbS > 8, the following applies: - The derivation process for a disparity sample array as specified in subclause Error: Reference source not found is invoked with the luma locations xCb, yCb, the disparity vector mvDisp, the view identifier refViewIdx, the variable nPSW equal to nCbS, the variable nPSH equal to nCbS, and the variable partIdc equal to 0 as inputs, and the output is the array disparitySamples of size (nCbS)x(nCbS). - The disparity vector mvRefinedDisp is set equal to ( disparitySamples[ 0 ][ 0 ], 0 ). - Otherwise (depth_refinement_flag[ nuh_layer_id ] is equal to 0), the disparity vector mvRefinedDisp is set equal to mvDisp. For use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x = xCb.. ( xCb + nCbS − 1 ), y = yCb..( yCb + nCbS− 1 ): - MvDisp[ x ][ y ] = mvDisp (Error: Reference source not found-1) - MvRefinedDisp[ x ][ y ] = mvRefinedDisp (Error: Reference source not found-2) - RefViewIdx[ x ][ y ] = refViewIdx (Error: Reference source not found-3) - DispAvailabilityIdc[ x ][ y ] = dvAvailFlag ? DISP_AVAILABLE : ( DefaultRefViewIdxAvailableFlag ? DISP_DEFAULT : DISP_NONE ) (Error: Reference source not found-4)

# Experimental Results

The simulation was carried out based on the common test condition [2] and HTM 12 reference software. Table 2 shows the performance comparison results between the current and the proposed method. And Table 3 shows the comparison of the worst case memory bandwidth between the current and the proposed method. The proposed method can reduce the worst case memory bandwidth by 14% while maintaining the coding performance.

Table 2. Performance comparison results between the CTC anchor and the proposed method

Table 3. Comparison of the worst case memory bandwidth between the current and the proposed method 
(when compared to the worst case of HEVC)

| Memory Pattern | External Memory Bandwidth (Current) | External Memory Bandwidth (Proposed) |
| --- | --- | --- |
| 1x1 | 110% | 100% |
| 4x2 | 113% | 100% |
| 8x2 | 114% | 100% |

Table 4 shows the comparison of the worst case memory bandwidth of each coding tool between the current and the proposed method. With the proposed method, it can be shown that the worst case memory bandwidth of all the 3D-HEVC coding tools is equal to or less than the worst case of HEVC.

Table 4. Comparison of the worst case memory bandwidth of each coding tool between the current and the proposed method (when compared to the worst case of HEVC)

| Tool | Memory Pattern (1x1) |  | Memory Pattern (4x2) |  | Memory Pattern (8x2) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Current | Proposed | Current | Proposed | Current | Proposed |
| HEVC | 100% | 100% | 100% | 100% | 100% | 100% |
| VSP | 50% | 50% | 66% | 66% | 81% | 81% |
| DBBP | 79% | 79% | 71% | 71% | 69% | 69% |
| IVMC | 110% | 100% | 113% | 100% | 114% | 100% |
| ARP | 97% | 97% | 95% | 88% | 108% | 94% |

# Conclusion

In this contribution, it is proposed to use NBDV instead of DoNBDV in 8x8 CU to avoid additional memory access in 3D-HEVC compared to HEVC. With the proposed method, the worst case memory bandwidth becomes the same as the worst case of HEVC without coding loss. Therefore, it is recommended to adopt the proposed method into next 3D-HEVC draft text.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 5,” JCT3V-I1001, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Jul. 2014, Sapporo, JP.

[2] D. Rusanovskyy, K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-G1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Jan. 2014, San Jose, USA.