| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0037_WD |
| --- | --- |

| Title: | Proposed text of 3D-HEVC Working Draft for JCT3V-J0037 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Byeongdoo Choi Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com b.d.choi@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document provides the proposed text of 3D-HEVC Working Draft. The changes are highlighted in green and strikethrough.

I.8.3.10  Derivation process for the target reference index for residual prediction

This process is invoked when the current slice is a P or B slice.

Let currPic be the current picture.

The variables RpRefIdxL0 and RpRefIdxL1 are set equal to −1, and the variables RpRefPicAvailFlagL0 and RpRefPicAvailFlagL1 are set equal to 0.

The following applies for X in the range of 0 to 1, inclusive:

- When X is equal to 0 or the current slice is a B slice the following applies:

  - The variable pocDiff is set equal to 215 − 1.

  - For i in the range of 0 to num_ref_idx_lX_active_minus1, inclusive, the following applies:

    - The variable currPocDiff is set equal to Abs( PicOrderCnt( RefPicListX[ i ] ) − PicOrderCntVal ).

    - When currPocDiff is not equal to 0 and currPocDiff is less than pocDiff, the following applies:

        - pocDiff = currPocDiff		(Error: Reference source not found-1)

        - RpRefIdxLX = i		(Error: Reference source not found-2)

        - RpRefPicAvailFlagLX = 1		(Error: Reference source not found-3)

- The variable RpRefPicAvailFlag is set equal to ( RpRefPicAvailFlagL0  | |  RpRefPicAvailFlagL1 ) && DefaultRefViewIdxAvailableFlag.

…