| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0037 |
| --- | --- |

| Title: | ARP, IC and DBBP Flags Signaling for 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Byeongdoo Choi Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com b.d.choi@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes methods for signaling of the syntax elements for ARP, IC and DBBP coding tools, which need to access a view reference picture, i.e. inter-view prediction coding tools. In the current 3D-HEVC, when the view reference picture is unavailable in the current reference picture lists, the CU level syntax elements of these coding tools are signaled in spite of the fact that these coding tools cannot be used in the current slice. Therefore, it is proposed not to signal the CU level syntax elements of these coding tools for the current slice when the view reference picture is unavailable and to additionally remove unnecessary encoder restriction of ARP, which checks whether the current DV is a default DV. With the proposed methods, unnecessary signaling and checking processes can be avoided. The proposed methods reportedly provide no coding loss (-0.01% and 0.00% for coded and synthesized views, respectively).

# Proposed Methods

## Item 1: Signaling of Syntax Element for ARP (iv_res_pred_weight_idx)

In the current ARP (advanced residual prediction), the CU (coding unit) level syntax element iv_res_pred_weight_idx is signaled when all of the following conditions are satisfied.

- VPS (view parameter set) level enabling flag (iv_res_pred_flag) is equal to ‘1’.

- RpRefPicAvailFlag, which indicates whether the current slice has at least one temporal reference picture in the current reference picture lists (both list 0 and list 1), is equal to ‘1’.

- The current CU is not coded with INTRA mode.

- The partition mode of the current CU is PART_2Nx2N mode.

However, when the view reference picture is unavailable in the current reference picture lists, even though ARP cannot be used in the current slice, iv_res_pred_weight_idx is always signaled. Therefore, it is proposed not to signal iv_res_pred_weight_idx when the view reference picture is unavailable in the current slice. The signaling condition of iv_res_pred_weight_idx can be changed as follows.

- VPS (view parameter set) level enabling flag (iv_res_pred_flag) is equal to ‘1’.

- RpRefPicAvailFlag, which indicates whether the current slice has at least one temporal reference picture and one view reference picture in the current reference picture lists (both list 0 and list 1), is equal to ‘1’.

- The current CU is not coded with INTRA mode.

- The partition mode of the current CU is PART_2Nx2N mode.

The RpRefPicAvailFlag derivation process in the 3D-HEVC working draft text can be changed as follows. DefaultRefViewIdxAvailableFlag, which indicates whether the current slice has at least one view reference picture, and which is already derived in slice level and is used to disable the merge candidates which use view reference picture such as VSP (view synthesis picture), IVMC (inter-view motion vector candidate, etc., can be added as highlighted in green.

I.8.3.10 Derivation process for the target reference index for residual prediction This process is invoked when the current slice is a P or B slice. Let currPic be the current picture. The variables RpRefIdxL0 and RpRefIdxL1 are set equal to −1, and the variables RpRefPicAvailFlagL0 and RpRefPicAvailFlagL1 are set equal to 0. The following applies for X in the range of 0 to 1, inclusive: - When X is equal to 0 or the current slice is a B slice the following applies: - The variable pocDiff is set equal to 215 − 1. - For i in the range of 0 to num_ref_idx_lX_active_minus1, inclusive, the following applies: - The variable currPocDiff is set equal to Abs( PicOrderCnt( RefPicListX[ i ] ) − PicOrderCntVal ). - When currPocDiff is not equal to 0 and currPocDiff is less than pocDiff, the following applies: - pocDiff = currPocDiff (Error: Reference source not found-1) - RpRefIdxLX = i (Error: Reference source not found-2) - RpRefPicAvailFlagLX = 1 (Error: Reference source not found-3) - The variable RpRefPicAvailFlag is set equal to ( RpRefPicAvailFlagL0 ¦ ¦ RpRefPicAvailFlagL1 ) && DefaultRefViewIdxAvailableFlag. …

## Item 2: Removal of Encoder Restriction of iv_res_pred_weight_idx

In the current ARP, there is a encoder restriction which checks whether the current derived DV (disparity vector) is a default DV. If the current DV is a default DV, iv_res_pred_weight_idx cannot have a non-zero value, which means ARP is always disabled. Moreover, for this encoder restriction, the DV status of the current CU needs to be stored; DISP_AVAILABLE, DISP_DEFAULT and DISP_NONE.

However, even if the current DV is a default DV, ARP can be enabled by using the default DV. Therefore, it is proposed to remove this encoder restriction of iv_res_pred_weigh_idx. The semantics of iv_res_pred_weight_idx in the 3D-HEVC working draft text can be changed as follows. With the proposed method, unnecessary encoder restriction can be avoided and the storage for the DV status of the current CU can be completely removed.

I.7.4.9.5.2 Coding unit extension semantics … The variable rpEnableFlag is derived as specified in the following: - rpEnableFlag = iv_res_pred_flag[ nuh_layer_id ] && RpRefPicAvailFlag ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ) && ( PartMode = = PART_2Nx2N ) (Error: Reference source not found-4) iv_res_pred_weight_idx specifies the index of the weighting factor used for residual prediction. iv_res_pred_weight_idx equal to 0 specifies that residual prediction is not used for the current coding unit. iv_res_pred_weight_idx not equal to 0 specifies that residual prediction is used for the current coding unit. When not present, the value of iv_res_pred_weight_idx is inferred to be equal to 0. When DispAvailabilityIdc[ x0 ][ y0 ] is not equal to DISP_AVAILABLE, iv_res_pred_weight_idx shall be equal to 0. …

## Item 3: Setting Slice Level Enabling Flag for IC (slice_ic_enable_flag)

In the current IC (illumination compensation), IC has a slice level enabling flag slice_ic_enable_flag, which indicates whether IC is used for the current slice. If slice_ic_enable_flag is set as enabled, the CU level syntax element ic_flag should be signaled. However, if there is no view reference picture in the current slice slice_ic_enable_flag might be set as enabled even though IC is never used. In order to avoid this case, it is proposed to impose the restriction, which enabling of slice_ic_enable_flag is not allowed when there is no view reference picture in the current reference picture lists, to slice_ic_enable_flag.

The semantics of slice_ic_enable_flag in the 3D-HEVC working draft text can be changed as follows. Here, DefaultRefViewIdxAvailableFlag indicates whether the current slice has at least one view reference picture, and is derived in slice level.

I.7.4.7.1 General slice segment header semantics … slice_ic_enable_flag equal to 1 specifies that illumination compensation is enabled for the current slice. slice_ic_enable_flag equal to 0 specifies that illumination compensation is disabled for the current slice, When not present, the value of slice_ic_enable_flag is inferred to be equal to 0. When DefaultRefViewIdxAvailableFlag is equal to 0, slice_ic_enable_flag shall be equal to 0. …

## Item 4: Signaling of Syntax Element for DBBP (dbbp_flag)

In the current DBBP (depth-based block partitioning), the CU (coding unit) level syntax element dbbp_flag is also signaled when the view reference picture is unavailable in the current reference picture lists. In order to avoid unnecessary signaling, DBBP is enabled when DefaultRefViewIdxAvailableFlag is true, that is, when the current slice has at least one view reference picture.

The coding unit syntax in the 3D-HEVC working draft text can be changed as follows.

I.7.3.8.5 Coding unit syntax coding_unit( x0, y0, log2CbSize , ctDepth) {; Descriptor. …; ae(v). if( depth_based_blk_part_flag[ nuh_layer_id ] && ( log2CbSize > 3 ) && ( PartMode = = PART_2NxN ¦ ¦ PartMode = = PART_Nx2N ) && DefaultRefViewIdxAvailableFlag). dbbp_flag[ x0 ][ y0 ]; ae(v). ….

# Experimental Results

The simulation was carried out based on the common test condition [2] and HTM 12 reference software. There are two test cases; one is for Item 1, 3 and 4 (Test 1: Not allowing CU level signaling for ARP, IC and DBBP when the view reference is unavailable) and the other is for Item 2 (Test 2: Removal of parsing condition of ARP).

Table 1 shows the performance comparison results of Test 1 (Item 1, 3, 4). There is no coding performance change since these proposed methods are not affected on the common test condition.

Table 1. Performance comparison results between CTC anchor and Test 1
(Not allowing CU level signaling for ARP, IC and DBBP when the view reference is unavailable)

Table 2 shows the performance comparison results of Test 2 (Item 2). The proposed method provides no coding loss (-0.01% and 0.00% for coded and synthesized views, respectively).

Table 2. Performance comparison results between CTC anchor and Test 2
(Removal of parsing condition of ARP)

# Conclusion

This contribution proposes methods for signaling of the syntax elements for ARP, IC and DBBP. With the proposed methods, unnecessary signaling and checking processes can be avoided. The proposed methods reportedly provide no coding loss (-0.01% and 0.00% for coded and synthesized views, respectively). Therefore, it is recommended to adopt the proposed methods into next 3D-HEVC draft text.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 5,” JCT3V-I1001, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Jul. 2014, Sapporo, JP.

[2] D. Rusanovskyy, K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-G1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Jan. 2014, San Jose, USA.