| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0038 |
| --- | --- |

| Title: | Depth intra skip coding (DISC) mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, and Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

At the last meeting, the single depth mode was adopted (I0095). When a CU is coded as the single depth mode, the reconstruction is performed by filling the CU with a single depth value, which is derived from the spatial neighboring pixels. This contribution proposes to use the pixels generated by one of the the conventional horizontal and vertical intra prediction modes for the reconstruction, instead of the single depth value. Since the proposed method does not send the residual after the prediction such as the inter skip mode, it is called a depth intra skip coding (DISC) mode in this contribution. The results shows that the DISC mode has better performance than the single depth mode. Especially, the bit saving of about -1.1% is achived in Poznan_Hall2. In addition, the single depth value derivation part (I.8.4.4.2.9) can be removed.

# Proposed Method

The single depth mode fills a CU with one single depth value for the reconstruction. To derive the single depth value, a sample candidate list is constructed by inserting the neighboring samples such as An/2, Bn/2, B0, A0, and B-1 in Fig. 1. The prunming process is performed between the sample candidates. Only two sample candidates in the list is checked through RDO and then the best single depth value is determined. A flag is signaled to distinguish the two different sample candidates. Since the signle depth mode does not code the residual information, only one bit flag signaling is required.

Figure 1. Single depth value derivation (I0095).

Even though the single depth mode is efficient, it introduces new functions such as the single depth value derivation and pruning processess. To avoid the new functions and further improve the coding effeciency, this contribution proposes to replace the single depth mode with the DISC mode. The main concept of the single depth mode and DISC mode is exactly same, but the DISC mode removes the single depth value derivation and pruning processess and instead exploits the exiting intra prediction modes. For example, the single depth mode uses two spatial neighboring pixel values for the prediction and reconstruction, while the DISC mode uses the vertical and horizontal prediction modes. The residual information is not compressed and the best prediction mode is determined through RDO. A flag is signaled to distinguish the two different predction modes.

# Test Results

Test results of the proposed method are summarized in Tables 1 (CTC) and 2 (All intra condition).

Table 1. Performance of the proposed method (CTC)

Table 2. Performance of the proposed method (All intra condition)

# Conclusion

By integrating the proposed method on the current 3D-HTM, it improves the coding performance. Hence, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).