| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0039 |
| --- | --- |

| Title: | Removal of redundant VSP candidates in Merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, and Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

At the last meeting, several proposals (I0053, I0087, I0112, I0106, and I0130) tried to restrict the VSP inheritances from the neighboring blocks for the simplification of the Merge mode construction. However, it showed that the restriction results in bit rate increase for dependent views, especially in CG sequences. This contribution proposes to restrict the VSP inheritance in terms of the potential coding performance. Since the inherited and default VSP candidates have same DV, the Merge mode can include the redundant VSP candidates. It is possible that all the candidates are VSP in the Merge mode. Therefore, the proposed method (a) disallows the VSP inheritance or (b) only allows the VSP inheritance from the left block and the default VSP is disabled if the inherited VSP exists. The results demonstrate that the proposed method removes the redundant VSP candidates without the coding loss. In addition, when the proposed method disallows the VSP inheritance, the VSP storage can be completely removed.

# Proposed Method

The multiple VSP candidates are inherited from the VSP flags (VspFlag) of the neighboring blocks such as A1, B1, B0, A0, and A2 in Fig. 1. However, these inherited VSP and the default VSP candidates have the same DV. Hence, it is possible that all the candidates are the VSP candidates in the Merge mode, which are redundant.

Fig 1. Multiple VSP candidates.

To remove the redundant VSP candidates,

(a) the proposed method removes the VSP inheritance from the neighboring blocks. In this case, there is no chance that the VSP candidate can be placed in the front, such as A1, B1, and B0, so the default VSP is placed between B1 and IVDC. Fig. 2 shows the Merge mode list without the VSP inheritance.

Fig 2. No VSP inheritance.

(b) the proposed method only allows the VSP inheritance from A1 and the default VSP is disabled if the inherited VSP exists. The default VSP is placed between B1 and IVDC. Fig. 3 shows the Merge mode list allowing only one VSP inheritance.

Fig 3. Only one VSP inheritance from A1.

# Test Results

Test results of the proposed method are summarized in Tables 1 and 2, respectively. Table 1 illustrates the performance when the proposed method removes the VSP inheritance from the neighboring blocks. Table 2 shows the performance when the proposed method only allows the VSP inheritance from A1.

Table 1. Performance when the proposed method removes the VSP inheritance

Table 2. Performance when the proposed method only allows the VSP inheritance from A1

# Conclusion

By integrating the proposed method on the current 3D-HTM, it removes the redundant VSP candidates in the Merge mode. Hence, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).