| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0040 |
| --- | --- |

| Title: | Simplification for single depth mode pruning process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Sehoon Yea 19, Yangjae-daero 11gil, Seocho-gu, Seoul 137-130 South Korea | Tel: Email: | +82-2-6912-6477 sunmi.yoo@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

- Abstract

Single depth mode for depth intra coding is adopted at the last meeting. In the single depth mode process, up to five samples on the fixed position from the neighbor samples are compared with each other to remove the duplicated candidates. Once the candidates are pruned, the first two available samples are used and one of them is signaled with an index. However, for most cases, the first two samples are available and they are chosen if they are not same from each other. Therefore, it is proposed to compare only first two samples and make them as the candidate samples. The experimental result reportedly shows that there is no impact on the coding efficiency.

# Introduction

At the last meeting, the single depth mode for depth intra coding is adopted. In this method, a depth block can be represented by one sample value and the sample value is derived from one of designated neighbor samples. To make the representative sample, the fixed neighbor samples are compared to remove the candidates with the duplicated values. After the pruning process, the first two available samples are regarded as final candidates, then the index is signaled to the decoder.

# Proposed method

The proposed method is to reduce the number of designated neighbor samples from five to two. The depth samples with the locations (-1, nTbS>>1) and (nTbS >>1, -1) positions are fixed for the depth intra mode. This method can reduce the number of comparison of the pruning process.

Figure 1 The target candidate pixels (red) and removed candidate positions (blue)

# Experimental result

The experimental result shows that there is minor effect on the current 3D-HEVC.

Table 1 Proposed method vs. HTM-12.0 (CTC)

Table 2 Proposed method vs. HTM-12.0 (AI)

# Conclusion

In this document, it is proposed to reduce the number of comparison neighbor samples for the depth intra mode. The experimental data reportedly shows that there is no impact on the coding efficiency. From the complexity point of view, the proposed method can reduce the number of comparisons. In conclusion, it is recommended to be adopted for the next version of 3D-HEVC.

# References

- K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments” Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, San Jose, USA, Jan. 2014.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).