| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-Jxxxx_WD |
| --- | --- |

| Title: | Simplification for single depth mode pruning process] |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Sehoon Yea 19, Yangjae-daero 11gil, Seocho-gu, Seoul 137-130 South Korea | Tel: Email: | +82-2-6912-6477 sunmi.yoo@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Working draft

The newly added parts compared to working draft are highlighted in green and the removed parts are marked with strikethrough.

- I.8.4.4.2.9	Specification of intra prediction mode INTRA_SINGLE_SAMPLE

Inputs to this process are:

- a sample location ( xTb, yTb ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

- the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

- a variable nTbS specifying the transform block size.

Output of this process is:

- the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The lists predSample[ i ] and availableFlag[ i ] are derived as specified in the following:

- For i in the range of 0 to 41, inclusive, the following applies:

  - Depending on i, the luma location ( xN, yN ) is specified in Error: Reference source not foundTable I-6.

  - The variable availableFlag[ i ] is derived as specified in the following:

    - If p[ xN, yN ] is marked as "available for intra prediction", availableFlag[ i ] is set equal to 1.

    - Otherwise (p[ xN, yN ] is marked as "not available for intra prediction"), availableFlag[ i ] is set equal to 0.

  - The variable predSample[ i ] is set equal to p[ xN, yN ].

Table Error: Reference source not found-1 – Specification of xN and yN depending on i

| i | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
| xN | nTbS >> 1−1 | 0 nTbS >> 1 | 0 | −1 | −1 |
| yN | 0 nTbS >> 1 | nTbS >> 1−1 | −1 | 0 | −1 |

- The list availableFlag[ i1 ] is modified as specified in the following:

- For i in the range of 1 to 4, inclusive, the following applies:

  - For j in the range of 0 to ( i − 1 ), inclusive, the following applies:

  - availableFlag[ i1 ] = availableFlag[ i1 ]  &&
	!( availableFlag[ j0 ]  &&  ( predSample[ j0 ]  = =  predSample[ i1 ] ) )	(Error: Reference source not found-1)

The list sampleCandList is derived as specified in the following:

numCand = 0
	for( i = 0; i < 52; i++ )
		if ( availableFlag[ i ]  && numCand < 2 ) 
			sampleCandList[ numCand++ ] = predSample[ i ]
	if( numCand  = =  0 )
		sampleCandList[ numCand++ ] = ( 1  <<  ( BitDepthY − 1 ) )
	if( numCand  = =  1 )
		sampleCandList[ numCand++ ] = sampleCandList[ 0 ] + 1

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1 are derived as follows:

  - predSamples[ x ][ y ] = sampleCandList[ single_sample_idx ]	(Error: Reference source not found-2)