| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0042 |
| --- | --- |

| Title: | Simplification of depth merge candidate list |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Sehoon Yea 19, Yangjaedaero-11gil Seocho-gu, Seoul 137-893 South Korea | Tel: Email: | +82-2-6912-6477 sunmi.yoo@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, DDD (Disparity derived depth) and IvMC are performed as the second and third depth merge candidates, respectively. For DDD, the disparity from the collocated texture is converted to depth data and included to the merge list. If the IvMC is selected, motion vectors in the reference view are derived by a converted disparity of the median depth value of the current bitdepth (1<<BitDepth-1). However, according to [1], DDD and IvMC brought only 0.12% and 0.09% of coding efficiency, respectively. Moreover, as single depth mode for depth is adopted, it is estimated that the coding performance of DDD may decrease. Finally, it is found out that there is only 0.01% of coding efficiency by DDD in the current 3D-HEVC, and 0.06% of coding gain is achieved from IvMC. Therefore, it is proposed to remove DDD and IvMC to reduce syntax elements and the complexity.

# Proposed method

For the current 3D-HEVC, DDD is performed and included as the second depth merge candidate. For DDD, the disparity in the collocated texture block is converted to the depth value (from 0 to 255) with the inverse conversion table. However, as mentioned above, it was shown that only 0.12% of coding efficiency was observed by DDD at the last meeting. Moreover, only for DDD process, the inverse conversion table (disparity to depth) shall be transmitted through a VPS or a slice segment header. If the DDD is removed, the inverse table transmission also can be removed. Therefore, in this proposal, it is confirmed that how much DDD brings coding efficiency and proposed to remove DDD for the next version of 3D-HEVC. Moreover, it is proposed to remove IvMC for depth merge coding. It was also reportedly shown that only 0.09% of coding gain of the last 3D-HEVC.

# Experimental result

## DDD Off

DDD can be disabled in the source code by setting the macro H_3D_DDD to be 0.

Table 1 DDD disabled vs. HTM-12.0

## Depth IvMC Off

In the configuration, IvMC can be separately disabled for texture and depth. In the proposed method, the IvMvPred is set to 1 0, 1 for texture and 0 for depth. Also, in the source code, when the current component is depth, it is disabled to get the inter-view motion vector.

Table 2 Depth IvMC Off vs. HTM-12.0

## DDD and Depth IvMC Off

Table 3 DDD & Depth IvMC Off vs. HTM-12.0

# Conclusion

In this document, it is shown that disabling DDD has no harm to the current 3D-HEVC. According to Table 1, DDD does not introduce a meaningful coding efficiency in the current 3D-HEVC at all. Furthermore, IvMC for depth coding achieves only 0.06% of coding gain. Therefore, it is proposed to adopt the proposed method, the removal of DDD and IvMC, for the next 3D-HEVC.

# References

- S. Yoo, S. Yea, “AHG5 : Performance evaluation for 3D-HEVC and its coding tools,” Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-I0115, Sapporo, JP, July 2014.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).