| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0043 |
| --- | --- |

| Title: | PU boundary deblocking restriction for DBBP blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Sehoon Yea 19, Yangjae-daero 11gil, Seocho-gu, Seoul 137-893 South Korea | Tel: Email: | +82-2-6912-6477 sunmi.yoo@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

- Abstract

This document proposes the restriction of deblocking filtering process of PU boundary when the current CU is a DBBP block. In the DBBP process, a split of PU does not represent for the motion compensation unit of the current block, but the carriage of motion vectors for each arbitrary-shaped segment. Since the DBBP performs an arbitrary shaped motion compensation, it is not necessary to be deblocked on the virtual PU boundary in the DBBP blocks. For the proposed method, the experimental result is reportedly shown that there is a minor effect on the current 3D-HEVC, with decreased decoding complexity.

# Proposed method

As described in the abstract, the DBBP block is motion compensated with the arbitrary-shaped segment derived from the corresponding depth block. Even though the DBBP-coded block has two motion vectors for each prediction unit (size of 2NxN or Nx2N), they no longer represent the motion vector of each PU, but for each segment for the current DBBP block. In this sense, applying the deblocking filter for the PU boundary for the DBBP block is less meaningful. Therefore, it is proposed to skip the PU boundary debloking filtering for the DBBP blocks. Since the DBBP is enabled only for CUs divided into two PUs with 2NxN or Nx2N, the DBBP block flag can be checked when those partitioned block is under deblocking filter process.

Figure 1 DBBP merging process [1]

(a)                           (b)

Figure 2 Filtered samples (a) Current 3D-HEVC (b) Proposed Method
(Red : DBBP segment filter, Yellow : PU Deblocking filter)

When a CU is under deblocking process, whether it is filtered or not is determined described in Figure 3. If the current CU is coded with DBBP mode and meets all the conditions, the boundary filtering process will be applied for the PU boundary. However, as metioned above, the current block is motion compensated using its segment mask. It means there is no PU boundary to be deblocked. Therefore, by using the proposed method, the a number of computations in the dashed process depicted in Figure 3 can be saved for the DBBP block.

Figure 3 A block diagram of deblocking filter process

In addition, in Tables below, the number of computations saved by the proposed method are counted for the 16x16 DBBP blocks (the minimum block size for DBBP). The computation analysis is based on HEVC Standard[2]. Please note that the strong deblocking filter is not applied since the strong filter is enabled in the condition that one or two of adjacent blocks is intra-coded.

Table 1 The number of saved computations for Bs
(8.7.2.4 in text)

|  | Compare | Add/Sub |
| --- | --- | --- |
| Bs calculation | 31 | 12 |

Table 2 The number of saved computations for decision process for luma block edges 
(8.7.2.5.3 and 8.7.2.5.6 in text)

| CU size | Add/Sub | LUT access | Compare |
| --- | --- | --- | --- |
| 16x16 | 184 | 2 | 80 |
| 32x32 | 736 | 2 | 320 |
| 64x64 | 2944 | 2 | 1280 |

Table 3 The number of saved computations for normal filter
(8.7.2.5.7 in text)

| CU size | Filter type | Add/Sub | Compare |
| --- | --- | --- | --- |
| 16x16 | Normal filter | 136 | 72 |
|  | Strong filter | 714 | 8 |
| 32x32 | Normal filter | 544 | 288 |
|  | Strong filter | 1344 | 32 |
| 64x64 | Normal filter | 2176 | 1152 |
|  | Strong filter | 5376 | 128 |

# Experimental result

The experiment is under common test conditions[3]. As a result, it is shown that the proposed method does not have a damaging influence on the coding efficiency.

Table 4 The proposed method vs. HTM-12.0 (CTC)

# Conclusion

In this proposal, it is proposed to restrict the deblocking filtering process for the DBBP blocks. The expeirmental result shows that there is no impact on the current 3D-HEVC from the coding effciciency point of view, with reduced decoding complexity. Therefore, it is recommended to be adopted for the next version of 3D-HEVC.

# References

- F. Jäger , “Cleanup of Depth-based Block Partitioning (DBBP),” Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-I0056, Sapporo, JP, July 2014.

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 10 (for FDIS & Last Call) ,” Joint Collaborative Team on Video Coding of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-L1003_v34, Geneva, CH, Jan. 2013.

- K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-G1100, San Jose, USA, Jan. 2014.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).