| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0043 |
| --- | --- |

| Title: | PU boundary deblocking restriction for DBBP blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Sehoon Yea 19, Yangjaedaero-11gil Seocho-gu, Seoul 137-893 South Korea | Tel: Email: | +82-2-6912-6477 sunmi.yoo@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Working draft

The newly added parts compared to working draft are highlighted in green and the removed parts are marked with strikethrough. The newly added parts in yellow are for the proposed method.

- I.8.7	In-loop filter process

The specifications in subclause 8.7 apply.

- I.8.7.1		General

The specifications in subclause 8.7.1 apply.

- I.8.7.2		Deblocking filter process

- I.8.7.2.1	General

The specifications in subclause 8.7.2.1 apply with the following modifications:

- All invocations of the process specified in subclause 8.7.2.3 are replaced with invocations of the process specified in subclause I.8.7.2.3 with dbbp_flag[ xCb ][ yCb ] as additional input.

- I.8.7.2.2	Derivation process of transform block boundary

The specifications in subclause 8.7.2.2 apply.

- I.8.7.2.3	Derivation process of transform block boundary

Inputs to this process are:

–	a variable log2CbSize specifying the luma coding block size,

–	a prediction partition mode PartMode,

–	a two-dimensional (nCbS)x(nCbS) array edgeFlags,

–	a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered,

- a flag dbbpFlag specifying whether the current block is coded with DBBP or not.

Output of this process is the modified two-dimensional (nCbS)x(nCbS) array edgeFlags.

Depending on the values of edgeType, PartMode, and the flag dbbpFlag, the following applies for k = 0..( 1  <<  log2CbSize ) − 1:

- If edgeType is equal to EDGE_VER, the following applies:

  - When PartMode is equal to PART_Nx2N and dbbpFlag is equal to 0, edgeFlags[ 1  <<  ( log2CbSize − 1 ) ][ k ] is set equal to 1.

  - When PartMode is equal to PART_NxN, edgeFlags[ 1  <<  ( log2CbSize − 1 ) ][ k ] is set equal to 1.

  - When PartMode is equal to PART_nLx2N, edgeFlags[ 1  <<  ( log2CbSize − 2 ) ][ k ] is set equal to 1.

  - When PartMode is equal to PART_nRx2N, edgeFlags[ 3 * ( 1  <<  ( log2CbSize − 2 ) ) ][ k ] is set equal to 1.

- Otherwise (edgeType is equal to EDGE_HOR), the following applies:

  - When PartMode is equal to PART_2NxN and dbbpFlag is equal to 0, edgeFlags [ k ][ 1  <<  ( log2CbSize − 1 ) ] is set equal to 1.

  - When PartMode is equal to PART_NxN, edgeFlags[ k ][ 1  <<  ( log2CbSize − 1 ) ] is set equal to 1.

  - When PartMode is equal to PART_2NxnU, edgeFlags[ k ][ 1  <<  ( log2CbSize − 2 ) ] is set equal to 1.

  - When PartMode is equal to PART_2NxnD, edgeFlags[ k ][ 3 * ( 1  <<  ( log2CbSize − 2 ) ) ] is set equal to 1.

- I.8.7.2.4	Derivation process of boundary filtering strength

The specifications in subclause 8.7.2.4 apply.

- I.8.7.2.5	Edge filtering process

The specifications in subclause 8.7.2.5 apply.

- I.8.7.3	Sample adaptive offset process

The specifications in subclause 8.7.3 apply.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).