| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0044 |
| --- | --- |

| Title: | On camera parameter transmission for 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Junghak Nam Sehoon Yea 19, Yangjaedaero-11gil Seocho-gu, Seoul 137-893 South Korea | Tel: Email: | +82-2-6912-6477 sunmi.yoo@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In this contribution, an efficient method for camera parameter transmission for 3D-HEVC is proposed. Currently, the camera parameters can be transmitted either in a VPS (video parameter set) or in a slice segment header. Whether the camera parameters are sent through the VPS or the slice segment header is determined if the camera parameters of the current view are variable to the slice or not. For instance, when a camera parameter set can be representative of the current view, it is sent through the VPS. On the other hand, if the camera parameter set is changeable as time goes on, it is sent through the slice header. However, the problem of this method is that the camera parameter set may be sent repeatedly when the camera parameter is changed only once. Therefore, it is proposed a prevention method for the duplicated camera parameter transmission.

# Problem statement

In the current 3D-HEVC, the camera parameters can be transmitted either in the VPS or in the slice segment header. When the camera parameters are sent through the VPS, it is determined that the camera parameters are not changed at all as time goes on. In contrast, if the camera parameters are different from each slice, they are sent through the slice header. In the current CTC, GT_Fly is the only sequence whose camera parameters are different from one slice to the other. For this sequence, the camera parameter set of the each slice is sent through the slice segment header, and the decoder receives 250 different camera parameter sets for each slice. In fact, in the current CTC, there is no redundant camera parameter transmission. However, if the number of the camera parameter changes is less than the number of slices, the redundant camera parameter sets are transmitted to the decoder. For example, if a sequence has two different camera parameter sets in a view, it has 100 slices, and the camera parameter change occurs at 50th slice, the first camera parameter set is sent 50 times and the other camera parameter set is transmitted last 50 times.

Figure 1 Redundant camera parameter transmission in 3D-HEVC

# Proposed method

In proposed methods, the transmitted camera parameters can be maintained until the new camera parameter set is given. In this document, two different methods are suggested.

Figure 2 Proposed method of camera parameter transmission

## Method 1 - VPS camera parameter transmission unchanged

It is unchanged that the camera parameter set is transmitted through the VPS when it is not changeable in a view. Also, when the camera parameter is variable, the decoder receives the camera parameter through the slice segment header. However, whether the decoder parses the camera parameter or not can be determined with a flag, cp_update_flag[ ViewIdx ], in the slice segment header. If it is equal to 0, the camera parameter for the current slice is not parsed, and the stored camera parameter set for the current view is unchanged and used for the current slice.

- I.7.3.6.1    General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( nuh_layer_id > 0 && cp_in_slice_segment_header_flag[ ViewIdx ] ) |  |
| cp_update_flag[ ViewIdx ] | u(1) |
| if ( cp_update_flag[ ViewIdx ] ) |  |
| for ( j = 0; j < ViewIdx; j++ ) { |  |
| cp_scale[ j ] | se(v) |
| cp_off[ j ] | se(v) |
| cp_inv_scale_plus_scale[ j ] | se(v) |
| cp_inv_off_plus_off[ j ] | se(v) |
| } |  |
| ... |  |
| } |  |

- I.7.4.7.1	General slice segment header semantics

...

cp_update_flag[ ViewIdx ] equal to 1 specifies that the syntax elements cp_scale[ ViewIdx ], cp_off[ ViewIdx ], cp_inv_scale_plus_scale[ ViewIdx ], and cp_inv_off_plus_off[ ViewIdx ] are present in the slice segment header. cp_update_flag[ ViewIdx ] equal to 0 specifies that there is no camera parameter update. When not present, cp_update_flag[ ViewIdx ] is inferred to be equal to 0.

cp_scale[ j ], cp_off[ j ], cp_inv_scale_plus_scale[ j ], and cp_inv_off_plus_off[ j ] specify conversion parameters for converting a depth value to a disparity value. When not present and cp_in_slice_segment_header_flag[ ViewIdx ] is equal to 0, the values of cp_scale[ j ], cp_off[ j ], cp_inv_scale_plus_scale[ j ], and cp_inv_off_plus_off[ j ], are inferred to be equal to vps_cp_scale[ ViewIdx ][ j ], vps_cp_off[ ViewIdx ][ j ], vps_cp_inv_scale_plus_scale[ ViewIdx ][ j ], and vps_cp_inv_off_plus_off[ ViewIdx ][ j ], respectively. It is a requirement of bitstream conformance, that the values of cp_scale[ j ], cp_off[ j ], cp_inv_scale_plus_scale[ j ], and cp_inv_off_plus_off[ j ] in a slice segment header having a ViewIdx equal to viewIdxA and the values of cp_scale[ j ], cp_off[ j ], cp_inv_scale_plus_scale[ j ], and cp_inv_off_plus_off[ j ] in a slice segment header having a ViewIdx equal to viewIdxB shall be the same, when viewIdxA is equal to viewIdxB.

When cp_in_slice_segment_header_flag[ ViewIdx ] is equal to 0 or cp_update_flag[ ViewIdx ] is equal to 1, the following applies :

CpScale[ ViewIdx ][ j ] = cp_scale[ j ]

CpOff[ ViewIdx ][ j ] = cp_off[ j ]

CpInvScalePlusScale[ ViewIdx ][ j ] = cp_inv_scale_plus_scale[ j ]

CpInvOffPlusOff[ ViewIdx ][ j] = cp_inv_off_plus_off[ j ]

The array DepthToDisparityB[ j ][ d ] specifying the disparity between the current view and the view with ViewIdx equal j corresponding to the depth value d in the view with ViewIdx equal to j and the array DepthToDisparityF[ j ][ d ] specifying the disparity between the view with ViewIdx equal j and the current view corresponding to the depth value d in the current view is derived as specified in the following:

- The variable log2Div is set equal to BitDepthY − 1 + cp_precision.

- For d in range of 0 to ( ( 1  <<  BitDepthY ) − 1), inclusive, the following applies:

  - For i in the range of 0 to ViewIdx − 1, inclusive, the following applies:

    - offset = ( cp_offCpOff[ ViewIdx ][ j ]  <<  BitDepthY ) + ( ( 1  <<  log2Div )  >>  1 )	(Error: Reference source not found-1)

    - scale = cp_scaleCpScale[ ViewIdx ][ j ]		(Error: Reference source not found-2)

    - DepthToDisparityB[ j ][ d ] = ( scale * d + offset )  >>  log2Div	(Error: Reference source not found-3)

    - invOffset = ( ( cp_inv_off_plus_offCpInvOffPlusOff[ ViewIdx ][ j ] − cp_offCpOff[ ViewIdx ][ j ] )  <<  BitDepthY ) + ( (  1 <<  log2Div )  >>  1 )	(Error: Reference source not found-4)

    - invScale = ( cp_inv_scale_plus_scaleCpInvScalePlusScale[ ViewIdx ][ j ] − cp_scaleCpScale[ ViewIdx ][ j ] )	(Error: Reference source not found-5)

    - DepthToDisparityF[ j ][ d ] = ( invScale * d + invOffset )  >>  log2Div 	(Error: Reference source not found-6)

The variables DispToDepthInvScale[ j ], DispToDepthInvShift[ j ], and DispToDepthInvOffset[ j ] used for derivation of a depth value in the current view from the disparity between the view with ViewIdx equal to j and the current view are derived as follows:

absCpScale = Abs( cp_scaleCpScale[ ViewIdx ][ j ] )
	signCpScale = Sign( cp_scaleCpScale[ ViewIdx ][ j ] )
	convShift = Ceil( Log2( cp_scaleCpScale[ ViewIdx ][ j ] ) ) + 9
	convPrec = ( 1 << convShift )
	for( d = 256, dMinError = 256, absMinError  = Abs( convPrec − absCpScale *256 ) ; d < 512; d++ ) {
		absCurrErr = Abs( convPrec − absCpScale * d )
		if( absCurrErr < absMinError ) {	(Error: Reference source not found-7)
			absMinError = absCurrErr
			dMinError = d
		}
	}
	signMinError = Sign( convPrec − absCpScale * dMinError )
	DispToDepthInvShift[ j ] = convShift
	DispToDepthInvScale[ j ] = signCpScale * ( dMinError  <<  ( BitDepthY + cp_precision − 1 ) )
	DispToDepthInvOffset[ j ] = −1 * signCpScale * dMinError * ( cp_offCpOff[ ViewIdx ][ j ]  <<  BitDepthY ) + 								(1  <<  ( convShift − 1 ) ) + signMinError * ( 1  <<  ( convShift − 4 ) )

The function DispToDepthF( j, d ) is defined as follows:

  - DispToDepthF( j, d )  = Clip3( 0, ( 1  <<  bitDepthY ) − 1, ( ( DispToDepthInvScale[ j ] * d + 		(Error: Reference source not found-8)
	DispToDepthInvOffset[ j ] ) >> DispToDepthInvShift[ j ] ) )

...

## Method 2 - VPS camera parameter set as default

In the method 2, the camera parameters are sent both in VPS and in the slice segment header when they are time-variant. At first, the default camera parameters are sent through VPS and maintained until the new camera parameters are transmitted in the slice segment header.

- I.7.3.2.1.2	Video parameter set extension 2 syntax

| vps_extension2( ) { | Descriptor |
| --- | --- |
| ... |  |
| cp_precision | ue(v) |
| for( i = 1; i < NumViews; i++ ) { |  |
| cp_present_flag[ i ] | u(1) |
| if( cp_present_flag[ i ] ) { |  |
| cp_in_slice_segment_header_flag[ i ] | u(1) |
| if( !cp_in_slice_segment_header_flag[ i ] ) |  |
| for( j = 0; j < i; j++ ) { |  |
| vps_cp_scale[ i ][ j ] | se(v) |
| vps_cp_off[ i ][ j ] | se(v) |
| vps_cp_inv_scale_plus_scale[ i ][ j ] | se(v) |
| vps_cp_inv_off_plus_off[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |

- I.7.4.3.1.2	Video parameter set extension 2 semantics

...

vps_cp_scale[ i ][ j ], vps_cp_off[ i ][ j ], vps_cp_inv_scale_plus_scale[ i ][ j ], and vps_cp_inv_off_plus_off[ i ][ j ] specify conversion parameters for converting a depth value to a disparity value and might be used to infer the values of cp_scale[ j ], cp_off[ j ], cp_inv_scale_plus_scale[ j ], and cp_inv_off_plus_off[ j ] for the i-th view specified in VPS. When the i-th view contains both a texture view and a depth view, the conversion parameters are associated with the texture view.

The variables CpScale[ i ][ j ], CpOff[ i ][ j ], CpInvScalePlusScale[ i ][ j ], and CpInvOffPlusOff[ i ][ j] are defined as follows :

CpScale[ i ][ j ] = vps_cp_scale[ i ][ j ]

CpOff[ i ][ j ] = vps_cp_off[ i ][ j ]

CpInvScalePlusScale[ i ][ j ] = vps_cp_inv_scale_plus_scale[ i ][ j ]

CpInvOffPlusOff[ i ][ j] = vps_cp_inv_off_plus_off[ i ][ j ]

...

- I.7.3.6.1    General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( nuh_layer_id > 0 && cp_in_slice_segment_header_flag[ ViewIdx ] ) |  |
| cp_update_flag [ ViewIdx ] | u(1) |
| if ( cp_update_flag [ViewIdx ] ) |  |
| for ( j = 0; j < ViewIdx; j++ ) { |  |
| cp_scale[ j ] | se(v) |
| cp_off[ j ] | se(v) |
| cp_inv_scale_plus_scale[ j ] | se(v) |
| cp_inv_off_plus_off[ j ] | se(v) |
| } |  |
| ... |  |
| } |  |

- I.7.4.7.1	General slice segment header semantics

...

cp_update_flag[ ViewIdx ] equal to 1 specifies that the syntax elements cp_scale[ ViewIdx ], cp_off[ ViewIdx ], cp_inv_scale_plus_scale[ ViewIdx ], and cp_inv_off_plus_off[ ViewIdx ] are present in the slice segment header. cp_update_flag[ ViewIdx ] equal to 0 specifies that there is no camera parameter update. When not present, cp_update_flag[ ViewIdx ] is inferred to be equal to 0.

cp_scale[ j ], cp_off[ j ], cp_inv_scale_plus_scale[ j ], and cp_inv_off_plus_off[ j ] specify conversion parameters for converting a depth value to a disparity value. When not present, the values of cp_scale[ j ], cp_off[ j ], cp_inv_scale_plus_scale[ j ], and cp_inv_off_plus_off[ j ], are inferred to be equal to vps_cp_scale[ ViewIdx ][ j ], vps_cp_off[ ViewIdx ][ j ], vps_cp_inv_scale_plus_scale[ ViewIdx ][ j ], and vps_cp_inv_off_plus_off[ ViewIdx ][ j ], respectively. It is a requirement of bitstream conformance, that the values of cp_scale[ j ], cp_off[ j ], cp_inv_scale_plus_scale[ j ], and cp_inv_off_plus_off[ j ] in a slice segment header having a ViewIdx equal to viewIdxA and the values of cp_scale[ j ], cp_off[ j ], cp_inv_scale_plus_scale[ j ], and cp_inv_off_plus_off[ j ] in a slice segment header having a ViewIdx equal to viewIdxB shall be the same, when viewIdxA is equal to viewIdxB.

When cp_update_flag[ ViewIdx ] is equal to 1, the following applies :

CpScale[ ViewIdx ][ j ] = cp_scale[ j ]

CpOff[ ViewIdx ][ j ] = cp_off[ j ]

CpInvScalePlusScale[ ViewIdx ][ j ] = cp_inv_scale_plus_scale[ j ]

CpInvOffPlusOff[ ViewIdx ][ j] = cp_inv_off_plus_off[ j ]

The array DepthToDisparityB[ j ][ d ] specifying the disparity between the current view and the view with ViewIdx equal j corresponding to the depth value d in the view with ViewIdx equal to j and the array DepthToDisparityF[ j ][ d ] specifying the disparity between the view with ViewIdx equal j and the current view corresponding to the depth value d in the current view is derived as specified in the following:

- The variable log2Div is set equal to BitDepthY − 1 + cp_precision.

- For d in range of 0 to ( ( 1  <<  BitDepthY ) − 1), inclusive, the following applies:

  - For i in the range of 0 to ViewIdx − 1, inclusive, the following applies:

    - offset = ( cp_offCpOff[ ViewIdx ][ j ]  <<  BitDepthY ) + ( ( 1  <<  log2Div )  >>  1 )	(Error: Reference source not found-9)

    - scale = cp_scaleCpScale[ ViewIdx ][ j ]		(Error: Reference source not found-10)

    - DepthToDisparityB[ j ][ d ] = ( scale * d + offset )  >>  log2Div	(Error: Reference source not found-11)

    - invOffset = ( ( cp_inv_off_plus_offCpInvOffPlusOff[ ViewIdx ][ j ] − cp_offCpOff[ ViewIdx ][ j ] )  <<  BitDepthY ) + ( (  1 <<  log2Div )  >>  1 )	(Error: Reference source not found-12)

    - invScale = ( cp_inv_scale_plus_scaleCpInvScalePlusScale[ ViewIdx ][ j ] − cp_scaleCpScale[ ViewIdx ][ j ] )	(Error: Reference source not found-13)

    - DepthToDisparityF[ j ][ d ] = ( invScale * d + invOffset )  >>  log2Div 	(Error: Reference source not found-14)

The variables DispToDepthInvScale[ j ], DispToDepthInvShift[ j ], and DispToDepthInvOffset[ j ] used for derivation of a depth value in the current view from the disparity between the view with ViewIdx equal to j and the current view are derived as follows:

absCpScale = Abs( cp_scaleCpScale[ ViewIdx ][ j ] )
	signCpScale = Sign( cp_scaleCpScale[ ViewIdx ][ j ] )
	convShift = Ceil( Log2( cp_scaleCpScale[ ViewIdx ][ j ] ) ) + 9
	convPrec = ( 1 << convShift )
	for( d = 256, dMinError = 256, absMinError  = Abs( convPrec − absCpScale *256 ) ; d < 512; d++ ) {
		absCurrErr = Abs( convPrec − absCpScale * d )
		if( absCurrErr < absMinError ) {	(Error: Reference source not found-15)
			absMinError = absCurrErr
			dMinError = d
		}
	}
	signMinError = Sign( convPrec − absCpScale * dMinError )
	DispToDepthInvShift[ j ] = convShift
	DispToDepthInvScale[ j ] = signCpScale * ( dMinError  <<  ( BitDepthY + cp_precision − 1 ) )
	DispToDepthInvOffset[ j ] = −1 * signCpScale * dMinError * ( cp_offCpOff[ ViewIdx ][ j ]  <<  BitDepthY ) + 								(1  <<  ( convShift − 1 ) ) + signMinError * ( 1  <<  ( convShift − 4 ) )

The function DispToDepthF( j, d ) is defined as follows:

  - DispToDepthF( j, d )  = Clip3( 0, ( 1  <<  bitDepthY ) − 1, ( ( DispToDepthInvScale[ j ] * d + 		(Error: Reference source not found-16)
	DispToDepthInvOffset[ j ] ) >> DispToDepthInvShift[ j ] ) )

# Conclusion

In this proposal, an efficient camera parameter transmission method is proposed. Currently, if there is a camera parameter change only once, the camera parameters are transmitted for each slice, which can be duplicated. Therefore, it is proposed to prevent the repetition of camera parameter transmission for the slice segment header with a flag, which indicates whether the current slice segment header has new camera parameters or not. In conclusion, it is recommended that this proposal is adopted for the next version of 3D-HEVC.

# References

- G. Tech, K. Wegner, Y. Chen, S.Yea, “3D-HEVC Draft Text 5”, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-I1001_v3, Sapporo, JP, July 2014.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).