| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0051 |
| --- | --- |

| Title: | Draft text for Advanced Boundary Chain coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungdong Seo Junghak Nam Sehoon Yea 19, Yangjae-daero 11gil, Seocho-gu Seoul 137-130, Korea | Tel: Email: | +82-2-6912-6477 jungdong.seo@lge.com junghak.nam@lge.com sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

The newly added parts compared to 3D-HEVC working draft 3 are highlighted in green and the removed parts are marked with strikethrough.

- I.7.3.8.5.1 Intra mode extension syntax

| intra_mode_ext( x0 , y0 , log2PbSize ) { | Descriptor |
| --- | --- |
| if( log2PbSize < 6 ) |  |
| dim_not_present_flag[ x0 ][ y0 ] | ae(v) |
| if ( !dim_not_present_flag[ x0 ][ y0 ] ) |  |
| depth_intra_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_DMM_WFULL ) |  |
| wedge_full_tab_idx[ x0 ][ y0 ] | ae(v) |
| else if ( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_CHAIN ) { |  |
| edge_start_side[ x0 ][ y0 ] | ae(v) |
| edge_start_position[ x0 ][ y0 ] | ae(v) |
| num_edge_codes[ x0 ][ y0 ] | ae(v) |
| for( k = 0; k < num_edge_codes; k++ ) |  |
| edge_code[ x0 ][ y0 ][ k ] | ae(v) |
| } |  |
| } |  |

depth_intra_mode_flag[ x0 ][ y0 ] is used to specify the depth intra mode of the current prediction unit.

The variable DepthIntraMode[ x0 ][ y0 ] is derived as specified in the following:

  - DepthIntraMode[ x0 ][ y0 ] = dim_not_present_flag[ x0 ][ y0 ] ? −1 : depth_intra_mode_flag[ x0 ][ y0 ]	(Error: Reference source not found-1)

Error: Reference source not found specifies the value for the depth intra mode and the associated names.

Table Error: Reference source not found-4 – Specification of DepthIntraMode and associated name

| DepthIntraMode | Associated name |
| --- | --- |
| −1 | INTRA_DEP_NONE |
| 0 | INTRA_DEP_DMM_WFULL |
| 1 | INTRA_DEP_DMM_CPREDTEX |
| 2 | INTRA_DEP_CHAIN |

edge_start_ side[ x0 ][ y0 ] equal to 0 specifies that the start point of region boundary chain coding is located on the top row boundary of the current block. edge_start_side[ x0 ][ y0 ] equal to 1 specifies that the start point of region boundary chain coding is located on the left column boundary of the current block. edge_start_side[ x0 ][ y0 ] equal to 2 specifies that the start point of region boundary chain coding is located on the right column boundary of the current block.

edge_start_position[ x0 ][ y0 ] specifies the column position of the start point of region boundary chain coding when edge_start_side[ x0 ][ y0 ] is equal to 0. edge_start_position[ x0 ][ y0 ] specifies the row position of the start point of region boundary chain coding when edge_start_side[ x0 ][ y0 ] is 1 or 2.

num_edge_codes[ x0 ][ y0 ] specifies the number of edges within the current block.

edge_code[ x0 ][ y0 ][ k ] shall be one of the values shown in Error: Reference source not found5. edge_code[ x0 ][ y0 ][ k ] is used to derive the edge direction of the k-th edge when edge_intra_flag[ x0 ][ y0 ] is equal to 1.

- Table I-5 – Interpretation of edge_code[ x0 ][ y0 ][ k ]

| edge_code[ x0 ][ y0 ][ k ] | edge direction |
| --- | --- |
| 0 | 0° |
| 1 | 45° |
| 2 | −45° |
| 3 | 90° |
| 4 | −90° |

- I.8.4.2	Derivation process for luma intra prediction mode

Input to this process is a luma location ( xPb, yPb ) specifying the top-left sample of the current luma prediction block relative to the top left luma sample of the current picture.

In this process, the luma intra prediction mode IntraPredModeY[ xPb ][ yPb ] is derived.

Error: Reference source not found specifies the value for the intra prediction mode and the associated names.

Table Error: Reference source not found-6 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated name |
| --- | --- |
| 0 | INTRA_PLANAR |
| 1 | INTRA_DC |
| 2..34 | INTRA_ANGULAR2..INTRA_ANGULAR34 |
| 35 | INTRA_DMM_WFULL |
| 36 | INTRA_DMM_CPREDTEX |
| 37 | INTRA_SINGLE_SAMPLE |
| 38 | INTRA_CHAIN |

IntraPredModeY[ xPb ][ yPb ] labelled 0..34 represents directions of predictions as illustrated in Figure 8 1.

- If single_sample_flag[ xPb ][ yPb ] is equal to 1, IntraPredModeY[ xPb ][ yPb ] is set equal to INTRA_SINGLE_SAMPLE.

- Otherwise, if DepthIntraMode[ xPb ][ yPb ] is equal to INTRA_DEP_DMM_WFULL, IntraPredModeY[ xPb ][ yPb ] is set equal to INTRA_DMM_WFULL.

- Otherwise if DepthIntraMode[ xPb ][ yPb ] is equal to INTRA_DEP_DMM_CPREDTEX, IntraPredModeY[ xPb ][ yPb ] is set equal to INTRA_DMM_CPREDTEX.

- Otherwise if DepthIntraMode[ xPb ][ yPb ] is equal to INTRA_DEP_CHAIN, IntraPredModeY[ xPb ][ yPb ] is set equal to INTRA_CHAIN.

- Otherwise (DepthIntraMode[ xPb ][ yPb ] is equal to INTRA_DEP_NONE), IntraPredModeY[ xPb ][ yPb ] is derived as the following ordered steps:

  - The neighbouring locations ( xNbA, yNbA ) and ( xNbB, yNbB ) are set equal to ( xPb − 1, yPb ) and ( xPb, yPb − 1 ), respectively.

  - For X being replaced by either A or B, the variables candIntraPredModeX are derived as follows:

    - The availability derivation process for a block in z-scan order as specified in subclause 6.4.2 is invoked with the location ( xCurr, yCurr ) set equal to ( xPb, yPb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbX, yNbX ) as inputs, and the output is assigned to availableX.

    - The candidate intra prediction mode candIntraPredModeX is derived as follows:

      - If availableX is equal to FALSE, candIntraPredModeX is set equal to INTRA_DC.

      - Otherwise, if CuPredMode[ xNbX ][ yNbX ] is not equal to MODE_INTRA or pcm_flag[ xNbX ][ yNbX ] is equal to 1, candIntraPredModeX is set equal to INTRA_DC,

      - Otherwise, if X is equal to B and yPb − 1 is less than ( ( yPb  >>  CtbLog2SizeY )  <<  CtbLog2SizeY ), candIntraPredModeB is set equal to INTRA_DC.

      - Otherwise, if candIntraPredModeX is larger than 34, candIntraPredModeX is set equal to INTRA_DC.

      - Otherwise, candIntraPredModeX is set equal to IntraPredModeY[ xNbX ][ yNbX ].

…

- I.8.4.4.2		Intra sample prediction

- I.8.4.4.2.1	General intra sample prediction

Inputs to this process are:

- a sample location ( xTbCmp, yTbCmp ) specifying the top-left sample of the current transform block relative to the top left sample of the current picture,

- a variable predModeIntra specifying the intra prediction mode,

- a variable nTbS specifying the transform block size,

- a variable cIdx specifying the colour component of the current block.

Output of this process is the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The nTbS * 4 + 1 neighbouring samples p[ x ][ y ] that are constructed samples prior to the deblocking filter process, with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1, are derived as follows:

- The neighbouring location ( xNbCmp, yNbCmp ) is specified by:

    - ( xNbCmp, yNbCmp ) = ( xTbCmp + x, yTbCmp + y )	(Error: Reference source not found-2)

- The current luma location ( xTbY, yTbY ) and the neighbouring luma location ( xNbY, yNbY ) are derived as follows:

    - ( xTbY, yTbY ) = ( cIdx  = =  0 ) ? ( xTbCmp, yTbCmp ) : ( xTbCmp  <<  1, yTbCmp  <<  1 )	(Error: Reference source not found-3)

    - ( xNbY, yNbY ) = ( cIdx  = =  0 ) ? ( xNbCmp, yNbCmp ) : ( xNbCmp  <<  1, yNbCmp  <<  1 )	(Error: Reference source not found-4)

- The availability derivation process for a block in z-scan order as specified in subclause 6.4.1 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xTbY, yTbY ) and the neighbouring luma location ( xNbY, yNbY ) as inputs, and the output is assigned to availableN.

- Each sample p[ x ][ y ] is derived as follows:

  - If one or more of the following conditions are true, the sample p[ x ][ y ] is marked as "not available for intra prediction":

    - The variable availableN is equal to FALSE.

    - CuPredMode[ xNbY ][ yNbY ] is not equal to MODE_INTRA and constrained_intra_pred_flag is equal to 1.

  - Otherwise, the sample p[ x ][ y ] is marked as "available for intra prediction" and the sample at the location ( xNbCmp, yNbCmp ) is assigned to p[ x ][ y ].

- When at least one sample p[ x ][ y ] with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1 is marked as "not available for intra prediction", the reference sample substitution process for intra sample prediction in subclause 8.4.4.2.2 is invoked with the samples p[ x ][ y ] with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1, nTbS, and cIdx as inputs, and the modified samples p[ x ][ y ] with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1 as output.

- Depending on the value of predModeIntra, the following ordered steps apply:

  - When cIdx is equal to 0 and predModeIntra is in the range of 0 to 34, the filtering process of neighbouring samples specified in subclause 8.4.4.2.3 is invoked with the sample array p and the transform block size nTbS as inputs, and the output is reassigned to the sample array p.

  - The intra sample prediction process according to predModeIntra applies as follows:

    - If predModeIntra is equal to INTRA_PLANAR, the corresponding intra prediction mode specified in subclause 8.4.4.2.4 is invoked with the sample array p and the transform block size nTbS as inputs, and the output is the predicted sample array predSamples.

    - Otherwise, if predModeIntra is equal to INTRA_DC, the corresponding intra prediction mode specified in subclause 8.4.4.2.5 is invoked with the sample array p, the transform block size nTbS, and the colour component index cIdx as inputs, and the output is the predicted sample array predSamples.

    - Otherwise, if predModeIntra is in the range of INTRA_ANGULAR2..INTRA_ANGULAR34, the corresponding intra prediction mode specified in subclause 8.4.4.2.6 is invoked with the intra prediction mode predModeIntra, the sample array p, the transform block size nTbS, and the colour component index cIdx as inputs, and the output is the predicted sample array predSamples.

    - Otherwise, if predModeIntra is equal to INTRA_DMM_WFULL, the corresponding intra prediction mode specified in subclause Error: Reference source not found is invoked with the location ( xTbY, yTbY ), the sample array p and the transform block size nTbS as inputs ,and the outputs are the predicted sample array predSamples.

    - Otherwise, if predModeIntra is equal to INTRA_DMM_CPREDTEX, the corresponding intra prediction mode specified in subclause Error: Reference source not found is invoked with the location ( xTbY, yTbY ), with the sample array p and the transform block size nTbS as inputs, and the outputs are the predicted sample array predSamples.

    - Otherwise, if predModeIntra is equal to INTRA_SINGLE_SAMPLE, the corresponding intra prediction mode specified in subclause I.8.4.4.2.9 is invoked with the location ( xTbY, yTbY ), with the sample array p and the transform block size nTbS as inputs, and the outputs are the predicted sample array predSamples.

    - Otherwise, if predModeIntra is equal to INTRA_CHAIN, the corresponding intra prediction mode specified in subclause Error: Reference source not found10 is invoked with the location ( xTbY, yTbY ), with the sample array p and the transform block size nTbS as inputs, and the outputs are the predicted sample array predSamples.

- I.8.4.4.2.10	Specification of intra prediction mode INTRA_CHAIN

Inputs to this process are:

- a sample location ( xTb, yTb ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

- the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

- a variable nTbS specifying the transform block size,

Output of this process is:

- the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1, are derived as specified in the following ordered steps.

  - The start point of the chain in the current prediction unit is derived from edge_start_side [ xTb ][ yTb ] and edge_start_position[ xTb ][ yTb ].

- If edge_start_side [ xTb ][ yTb ] is equal to 1, the start point is set as s[ 0 ][ yS ] where yS is equal to edge_start_position[ xTb ][ yTb ].

- If edge_start_side [ xTb ][ yTb ] is equal to 0, the start point is set as s[ xS ][ 0 ] where xS is equal to edge_start_position[ xTb ][ yTb ].

- Otherwise (edge_start_side [ xTb ][ yTb ] is equal to 2), the start point is set as s[ nTbS − 1][ yS ] where yS is equal to edge_start_position[ xTb ][ yTb ].

  - The number of edges in the current prediction unit is derived from num_edge_codes[ xTb ][ yTb ].

  - The direction of each edge is derived from edge_code[ xTb ][ yTb ][ k ], for k = 0.. num_edge_codes [ xTb ][ yTb ] − 1. The direction of the edge is derived from edge_code as shown in Error: Reference source not found5.

  - The region boundary generated by connecting each edge separates the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1 into two regions: the region rA that covers the top-left pixel (x0, y0) and the region rB that covers the remaining region. The derivation process of the binary contour pattern as specified in subclause H.8.4.4.2.9.1 is invoked with the sample location ( xTb, yTb ), and the variable nTbS as inputs, and the output is the binary partition pattern edgePattern[ x ][ y ], with x, y = 0..nTbS − 1.

  - The depth partition value derivation and assignment process as specified in subclause I.8.4.4.2.10 is invoked with the neighbouring samples p[ x ][ y ], the binary pattern edgePattern[ x ][ y ], the transform size nT, the dcOffsetAvailFlag set equal to depth_dc_Flag[ xTb ][ yTb ], and the DC Offsets DcOffset[ xTb ][ yTb ][ 0 ], and DcOffset[ xTb ][ yTb ][ 1 ] as inputs and the output is assigned to predSamples[ x ][ y ].

- I.8.4.4.2.10.1	Edge pattern generation process

Inputs to this process are:

- a variable patternSize specifying the binary partition pattern size,

- a variable edge_start_side specifying the location of start boundary of the current block,

- a variable edge_start_position specifying the position of the start point,

- a variable num_edge_codes specifying the number of edges within the current block,

- a variable edge_code[ k ] specifying the edge direction of the k-th edge,

Output to this process is:

- binary array partitionPattern[ x ][ y ], with x, y = 0..patternSize – 1, specifying a binary partition pattern.

- The variable curx, cury are derived as specified in the following:

- curx = (edge_start_side == 0 ?) edge_start_position : ( (edge_start_side == 1 ?) 0 : N - 1 )

- cury = (edge_start_side == 0 ?) 0 : ( (edge_start_side == 1 ?) edge_start_position : edge_start_position )

- The variable prevEdgeIdx is derived as specified in the following:

- prevEdgeIdx = (edge_start_side == 0 ?) 2 : ( (edge_start_side == 1 ?) 0 : 1 )

- The variable vertEdgeInfo[x][y] and horizEdgeInfo[y], with x, y = 0..patternSize – 1, are derived as specified in the following:

- For y in the range of 0 to ( N – 1 ), inclusive the following applies:

  - For x in the range of 0 to (N – 2), inclusive the following applies:

    - vertEdgeInfo [ x ] [ y ] = 0

- For y in the range of 0 to ( N – 2 ), inclusive the following applies:

  - horizEdgeInfo [ y ] = 0

- For i in the range of 0 to num_edge_codes, inclusive the following applies:

  - If i equal to num_edge_codes, the following applies:

    - If (  ( cury == 0, and PrevEdgeIdx == 1 ) or ( cury == N-1, and PrevEdgeIdx == 0 ) or 
( curx == 0, and PrevEdgeIdx == 2 ) or ( curx == N-1, and PrevEdgeIdx == 3 )  )

      - CurEdgeCode = 3

    - Otherwise,

      - CurEdgeCode = 0

  - Otherwise, the following applies:

    - CurEdgeCode = edge_code [ i ]

  - AbsoluteEdgeIdx = AbsoluteEdgeIdxTable [ PrevEdgeIdx ] [ CurEdgeCode ]

  - AbsoluteEdgeTable is specified in Table I - 7.

  - EdgePatternIncIdx_x = EdgePatternIncIdxTable [ AbsoluteEdgeIdx ] [ 0 ]

  - EdgePatternIncIdx_y = EdgePatternIncIdxTable [ AbsoluteEdgeIdx ] [ 1 ]

  - EdgePatternIncIdxTable is specified in Table I - 8.

  - EdgeMark = EdgeMarkingTable [ PrevEdgeIdx ] [ CurEdgeCode ]

  - EdgeMarkingTable is specified in Table I - 9.

  - If ( EdgeMark & 0x1 ) is equal to 1, the following applies:

    - vertEdgeInfo [ curx-1 ] [ cury ] = 1

  - If ( EdgeMark & 0x2 ) is equal to 1, and curx is equal to 0, the following applies:

    - horizEdgeInfo [ cury-1 ] = 1

  - If ( EdgeMark & 0x4 ) is equal to 1, the following applies:

    - vertEdgeInfo [ curx ] [ cury ] = 1

  - If (EdgeMark & 0x8 ) is equal to 1, and curx is equal to 0, the following applies:

    - horizEdgeInfo [ cury ] = 1

  - curx += EdgePatternIncIdx_x

  - cury += EdgePatternIncIdx_y

  - PrevEdgeIdx = AbsoluteEdgeIdx

- Binary array partitionPattern[ x ][ y ] is derived as specified in the following:

  - startPartitionIdx = 0

  - For y in the range of 0 to ( N – 1 ), inclusive the following applies:

    - partitionIdx = startPartitionIdx

    - If y is smaller than (N – 1), , the following applies:

      - If ( horizEdgeInfo[y] == 1 )

        - startPartitionIdx = ! startPartitionIdx

    - For x in the range of 0 to ( N – 1 ), inclusive the following applies:

      - partitionPattern [ x ] [ y ] = partitionIdx

      - If ( x < N – 1)

        - If ( vertEdgeInfo [ x ] [ y ] == 1 )

          - partitionIdx = ! partitionIdx

Table Error: Reference source not found−7 – Specification of AbsoluteEdgeIdxTable

| PrevEdgeIdx | CurEdgeIdx |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 |
| 0 | 0 | 6 | 4 | 3 | 2 |
| 1 | 1 | 5 | 7 | 2 | 3 |
| 2 | 2 | 4 | 5 | 0 | 1 |
| 3 | 3 | 7 | 6 | 1 | 0 |
| 4 | 4 | 0 | 2 | 6 | 5 |
| 5 | 5 | 2 | 1 | 4 | 7 |
| 6 | 6 | 3 | 0 | 7 | 4 |
| 7 | 7 | 1 | 3 | 5 | 6 |

Table Error: Reference source not found−8 – Specification of EdgePatternIncIdxTable

| AbsoluteEdgeIdx | EdgePatternIncIdx[AbsoluteEdgeIdx][0] | EdgePatternIncIdx[AbsoluteEdgeIdx][1] |
| --- | --- | --- |
| 0 | 1 | 0 |
| 1 | -1 | 0 |
| 2 | 0 | 1 |
| 3 | 0 | -1 |
| 4 | 1 | 1 |
| 5 | -1 | 1 |
| 6 | 1 | -1 |
| 7 | -1 | -1 |

Table Error: Reference source not found−9 – Specification of EdgeMarkingTable

| PrevEdgeIdx | CurEdgeCode |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 |
| 0 | 2 | 3 | 2 | 3 | 0 |
| 1 | 8 | 12 | 8 | 12 | 0 |
| 2 | 4 | 6 | 4 | 6 | 0 |
| 3 | 1 | 9 | 1 | 9 | 0 |
| 4 | 6 | 6 | 4 | 7 | 4 |
| 5 | 12 | 12 | 8 | 14 | 8 |
| 6 | 3 | 3 | 2 | 11 | 2 |
| 7 | 9 | 9 | 1 | 13 | 1 |

- I.9.3.3.2	Initialization process for context variables

The specifications in subclause 9.3.2.2 apply with the following modifications.

- All references to the process specified in subclauses 7.3.8.1 through 7.3.8.11 are replaced with references to the process specified in subclauses Error: Reference source not found to Error: Reference source not found.

- Error: Reference source not found is appended to the end of Table 9-4.

- Error: Reference source not found to Error: Reference source not found are appended to the end of the subclause.

Table Error: Reference source not found-1 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

| Syntax structure | Syntax element | ctxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| cu_extension( ) intra_mode_ext( ) | depth_intra_mode_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | depth_dc_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | depth_dc_abs | Error: Reference source not found | 0 | 1 | 2 |
|  | iv_res_pred_weight_idx | Error: Reference source not found |  | 0..2 | 3..5 |
|  | ic_flag | Error: Reference source not found |  | 0 | 1 |
|  | dbbp_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | sdc_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | dim_not_present_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | edge_code | Table I-21 | 0 | 1 | 2 |

Table Error: Reference source not found-2 – Values of initValue for depth_intra_mode_flag ctxIdx

| Initialization variable | ctxIdx of depth_intra_mode_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table I-21 – Values of initValue for edge_code ctxIdx

| Initialization variable | ctxIdx of edge_code |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 153 | 153 | 153 |

- I.9.3.3	Binarization process

- I.9.3.3.1	General

The specifications in subclause 9.3.3.1 apply with the following modifications.

- Error: Reference source not found is appended to the end of Table 9-32.

Table Error: Reference source not found-31 – Syntax elements and associated binarizations

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |
| cu_extension( ) | iv_res_pred_weight_idx | TR | cMax = 2, cRiceParam = 0 |
|  | ic_flag | FL | cMax = 1 |
|  | dbbp_flag | FL | cMax = 1 |
|  | sdc_flag | FL | cMax = 1 |
| intra_mode_ext( ) | dim_not_present_flag | FL | cMax = 1 |
|  | depth_intra_mode_flag | FL TR | cMax = 12, cRiceParam = 0 |
|  | wedge_full_tab_idx | FL | cMax = wedgeFullTabIdxBits[ log2PbSize ] (defined in Error: Reference source not found) |
|  | depth_dc_flag | FL | cMax = 1 |
|  | depth_dc_abs | I.9.3.3.12 | - |
|  | depth_dc_sign_flag | FL | cMax = 1 |
|  | edge_start_side | TR | cMax = 2, cRiceParam=0 |
|  | edge_start_position | FL | cMax = edgeStartPositionLenBits[log2PbSize] (defined in Table I-24) |
|  | num_edge_codes | I.9.3.3.13 | - |
|  | edge_code | TR | cMax = 5, cRiceParam=0 |

Table Error: Reference source not found-42 -Values of wedgeFullTabIdxBits[ log2PUSize ]

| Initialization variable | wedgeFullTabIdxBits |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| log2PbSize | 2 | 3 | 4 | 5 | 6 |
| Value | 7 | 10 | 11 | 11 | 13 |

Table I-23 –Values of edgeStartPositionLenBits[ log2PUSize ]

| Initialization variable | edgeStartPositionLenBits |  |  |  |
| --- | --- | --- | --- | --- |
| log2PbSize | 2 | 3 | 4 | 5 |
| Value | 2 | 3 | 4 | 5 |

- I.9.3.3.13 Binarization process for num_edge_codes

- Input to this process is a request for the a syntax element num_edge_codes,

- Output of this process is the binarization of the syntax element.

The variable EdgeNumber is the number of edges obtained by num_edge_code.

The value of StartEdgeNum and BlkSize is defined in Table I-26 and Table I-27, respectively.

Table I-24 –Values of StartEdgeNum[ log2PUSize ]

| Initialization variable | StartEdgeNum |  |  |  |
| --- | --- | --- | --- | --- |
| log2PbSize | 2 | 3 | 4 | 5 |
| Value | 2 | 4 | 8 | 16 |

Table I-25 –Values of BlkSize[ log2PUSize ]

| Initialization variable | BlkSize |  |  |  |
| --- | --- | --- | --- | --- |
| log2PbSize | 2 | 3 | 4 | 5 |
| Value | 4 | 8 | 16 | 32 |

The variable cMax is derived from StartEdgeNum as:

- cMax = ( StartEdgeNum [log2PUSize] <= EdgeNumber && 
       EdgeNumber < StartEdgeNum [log2PUSize]+BlkSize[log2PUSize]-1 )
       ? StartEdgeNum-1 : BlkSize -1

The bin string is derived as follows:

- If cMax is equal to StartEdgeNum-1

  - The variable EdgeNumSymbol is derived as:
 EdgeNumSymbol = ( EdgeNumber – BlkSize [ log2PUSize ] – 1 > 0 ) ? 
                  ( (EdgeNumber - BlkSize[ log2PUSize ] – 1 ) << 1 ) – 1 :
                  abs ( EdgeNumber – BlkSize [ log2PUSize ] – 1 ) << 1   
, and bin string is specified by invoking the FL binarization process as specified in clause 9.3.3.4 for EdgeNumSymbol with the variable cMax.

- Otherwise,

  - The variable EdgeNumSymbol is derived as:
EdgeNumSymbol = ( EdgeNumber >= StartEdgeNum [ log2PUSize ] + BlkSize [ log2PUSize ] – 1 ) ?
                 ( EdgeNumber – BlkSize [ log2PUSize ] ) : ( EdgeNumber - 1 )
, and bin string is specified by invoking the FL binarization process as specified in clause 9.3.3.4 for EdgeNumSymbol with the variable cMax.

- I.9.3.4.2	Derivation process for ctxTable, ctxIdx and bypassFlag

- I.9.3.4.2.1	General

The specifications in subclause 9.3.4.2.1 apply with the following modifications:

- Error: Reference source not found Table I-26 is appended to the end of Table 9-37.

Table Error: Reference source not found-5 26–Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >=5 |
| wedge_full_tab_idx | bypass | bypass | bypass | bypass | bypass | bypass |
| depth_dc_flag | 0 | na | na | na | na | na |
| depth_dc_abs | 0 | 0 | 0 | bypass | bypass | bypass |
| depth_dc_sign_flag | bypass | 0 | 0 | 0 | 0 | 0 |
| iv_res_pred_weight_idx | 0, 1 Error: Reference source not found27 | 2 | na | na | na | na |
| ic_flag | 0 | na | na | na | na | na |
| dbbp_flag | 0 | na | na | na | na | na |
| depth_intra_mode_flag | 0 | na0 | na | na | na | na |
| sdc_flag | 0 | na | na | na | na | na |
| dim_not_present_flag | 0 | na | na | na | na | na |
| edge_start_side | bypass | bypass | na | na | na | na |
| edge_start_position | bypass | bypass | bypass | bypass | bypass | bypass |
| num_edge_codes | bypass | bypass | bypass | bypass | bypass | bypass |
| edge_code | 0 | bypass | bypass | bypass | bypass | na |