| gmJoint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0051 |
| --- | --- |

| Title: | Advanced boundary chain coding for depth intra coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungdong Seo Junghak Nam Sehoon Yea 19, Yangjae-daero 11gil, Seocho-gu Seoul 137-130, Korea | Tel: Email: | +82-2-6912-6477 jungdong.seo@lge.com junghak.nam@lge.com sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This proposal provides results of the advanced boundary chain coding (ABC) based on HTM-12.0. The ABC is an improved coding tool based on the region boundary coding which separates a block into two partitions following an arbitrary contour described by a chain code. This proposal presents the experimental results of ABC without CABAC. The proposed method shows 0.2% and 0.2% BDR gains in synthesized views under CTC and AI configurations, respectively.

# Introduction

In this contribution, advanced boundary chain (ABC) coding method is proposed for depth intra coding. It partitions the current block into two regions by signaling the region boundaries with chain codes. The proposed method can remove the inter-component correlation between texture and depth in depth intra coding.

# Proposed method

The proposed ABC coding consists of three steps

Step 1: find internal edge

The internal edges within a depth block are calculated in the encoder. The Step 1 consists of several sub-steps.

- Calculate differences between vertically and horizontally adjacent pixels.

- Mark as edge candidates if the difference is greater than the threshold.

- Eliminate the edge candidates which have smaller differences than neighbouring edge candidates.

- Connect unlinked edges and prune unconnected edges.

- Check whether the block was separated into two regions exactly.

Step 2: code the edges using chain codes

The edge information is encoded by chain codes. Firstly, an edge start from the block boundaries and then the next edge is selected as an edge connected to the current one. The last edge ends at the block boundaries. In order to construct chain codes, if defines totally five traverse types (0, 45, -45, 90, and -90 degrees). Two entropy coding methods are proposed for the codes. One uses limited context coded bins and the other employs bypass coding for all syntax of the codes.

Step 3: make the prediction block

The representative pixel values for the current block are calculated in each region based on the neighbouring pixels. The calculation process is the same as the process of DMM series. Then, the current depth block is filled with the calculated predictors.

# Experimental results

The proposed methods were integrated into HTM-12.0 reference software and the tests were conducted under random access (CTC) and intra-only (AI) configuration.

## ABC with limited context coded bins for chain code

This method encodes the only MSB of chain code based on context models. The other bits are encoded as bypass. The experimental results show 0.2% BD-rate gain in synthesized views.

Table 1. ABC with limited context coded bins for chain code vs. HTM-12.0 on CTC

Table 2. ABC with limited context coded bins for chain code vs. HTM-12.0 on AI

## ABC without context coded bins

In this method, all syntax for ABC is encoded as bypass. The experimental results show 0.2% and 0.1% BD-rate gain in the synthesized views under CTC and AI configurations, respetively.

Table 3. ABC without context coded bins vs. HTM-12.0 on CTC

Table 4. ABC without context coded bins vs. HTM-12.0 on CTC

# Conclusion

This contribution proposes an advanced boundary chain (ABC) coding method for depth intra coding. The improved scheme provides 0.2% and 0.2% coding gains under common test condition and all-intra configuration, respectively.

# Patent rights declaration(s)

LG Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).