| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0054 |
| --- | --- |

| Title: | Simplification of single depth intra mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gun Bang, *Young Su Heo, *Woo Woen Gwun, *Gwang Hoon Park, Gwang Soon Lee, Nam Ho Hur ETRI, 218 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, Korea *Media Lab., Kyung Hee University 1, Seocheon, Giheung, Yongin, Gyeonggi, 446-701, Korea | Tel: Email: | +82-42-860-6791 {gbang, gslee, namho} @etri.re.kr +82-31-201-3680 {*medist, *woo_gwun, *ghpark} @khu.ac.kr |
| Source: | ETRI and Kyung Hee University |  |  |

_____________________________

# Abstract

This contribution proposes removing of derivation process for pruning and deriving virtual candidates during single depth intra prediction mode.

# Introduction

In the current version of 3D-HEVC, pruning process for single depth intra prediction mode is performed to insert two different candidate in candidate list which has length of two. In the worst case scenario comparison process needed to be performed 4 time to construct the list.

Also, after performing search process of candidate, if number of usable candidate is less than length of the list, induction process for virtual candidate is performed. First, if number of available candidate is 0, median value (in case of 8bit depth, 128 is used) is inserted as first entry in list. Second, if number of usable candidate is 1, offset is added on the list’s first entry candidate and inserted as a second entry.

However, processes which have mentioned above have minimal effect on coding efficiency. Therefore, this contribution suggests removing of pruning and virtual candidate derivation process as shown in table 1 to simplify the procedure. Proposed simplification method doesn’t create virtual candidate, so if there is a case where no usable candidate exists (Occurs only for the first unit in a frame), single sample flag is signaled as 0.

Table 1. Modified WD for single depth intra prediction mode

- Specification of intra prediction mode INTRA_SINGLE_SAMPLE Inputs to this process are: - a sample location ( xTb, yTb ) specifying the top-left sample of the current block relative to the top-left sample of the current picture, - the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1, - a variable nTbS specifying the transform block size. Output of this process is: - the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1. The lists predSample[ i ] and availableFlag[ i ] are derived as specified in the following: - For i in the range of 0 to 4, inclusive, the following applies: - Depending on i, the luma location ( xN, yN ) is specified in Error: Reference source not found. - The variable availableFlag[ i ] is derived as specified in the following: - If p[ xN, yN ] is marked as "available for intra prediction", availableFlag[ i ] is set equal to 1. - Otherwise (p[ xN, yN ] is marked as "not available for intra prediction"), availableFlag[ i ] is set equal to 0. - The variable predSample[ i ] is set equal to p[ xN, yN ]. Table I-6 – Specification of xN and yN depending on i i; 0; 1; 2; 3; 4. xN; nTbS >> 1; 0; 0; −1; −1. yN; 0; nTbS >> 1; −1; 0; −1. - The list availableFlag[ i ] is modified as specified in the following: - For i in the range of 1 to 4, inclusive, the following applies: - For j in the range of 0 to ( i − 1 ), inclusive, the following applies: - availableFlag[ i ] = availableFlag[ i ] && !( availableFlag[ j ] && ( predSample[ j ] = = predSample[ i ] ) ) (Error: Reference source not found-1) The list sampleCandList is derived as specified in the following: numCand = 0 for( i = 0; i < 5; i++ ) if ( availableFlag[ i ] && numCand < 2 ) sampleCandList[ numCand++ ] = predSample[ i ] if( numCand = = 0 ) sampleCandList[ numCand++ ] = ( 1 << ( BitDepthY − 1 ) ) if( numCand = = 1 ) sampleCandList[ numCand++ ] = sampleCandList[ 0 ] + 1 The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1 are derived as follows: - predSamples[ x ][ y ] = sampleCandList[ single_sample_idx ] (Error: Reference source not found-2)

# Simulation results

Table 2 shows simulation results for the proposed simplification method performed on HTM 12.0[1], common test conditions [2]. As shown in the results, simplification has almost no effect on coding efficiency. The tests are performed in different clusters, so complexity results may not be accurate.

Table 2. Proposed vs. HTM 12.0 CTC

# Conclusion

This contribution proposes simplification of single depth intra prediction mode by deleting pruning and derivation process for virtual candidates.

# References

- HTM-12, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-12.0

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G1100, Jan., 2014.

# Patent rights declaration(s)

ETRI and Kyung Hee University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).