| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0055 |
| --- | --- |

| Title: | Motion buffer reduction for depth |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gun Bang, *Young Su Heo, *Woo Woen Gwun, *Gwang Hoon Park, Gwang Soon Lee, Nam Ho Hur ETRI, 218 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, Korea *Media Lab., Kyung Hee University 1, Seocheon, Giheung, Yongin, Gyeonggi, 446-701, Korea | Tel: Email: | +82-42-860-6791 {gbang, gslee, namho} @etri.re.kr +82-31-201-3680 {*medist, *woo_gwun, *ghpark} @khu.ac.kr |
| Source: | ETRI and Kyung Hee University |  |  |

_____________________________

# Abstract

This contribution suggests reduction of buffer used on motion data for the depth map. The proposed method applies higher compression ratio on depth motion data.

# Introduction

Current version of the 3D-HEVC performs subsampling on motion data to reduce size of buffer which is used for storing motion data.

- After each picture is coded: In case of the 8x8 unit, top-left 4x4 block is used as representative of motion data, resulting reduction of motion data buffer in 1/4.

- After access unit (AU) is coded: In case of 16x16 unit, top-left 4x4 block is also used as representative of motion data, resulting reduction of motion data buffer in 1/16.

Fig. 1. Subsampling of motion data in current 3D-HEVC

# Proposed method

In the current version of 3D-HEVC, motion compression is performed in same ratio for the both texture and depth. This contribution proposes higher compression ratio to be used on the depth.

Motion data from the coded frame can be used for AMVP or merge mode. In the case of texture coding, motion prediction using inter-view motion is often performed. Since usage of stored motion data is needed, increasing too much subsampling ratio will cause efficiency of inter-view motion parameter prediction to be dropped, resulting reduction of coding efficiency.

However, in the case of the depth coding, accuracy for motion parameter inheritance which only uses motion data of a corresponding texture is pretty high. Which means that unlike texture coding, using higher compression ratio on depth motion data won’t cause problem on coding efficiency.

Table 1 proposes subsampling ratio which could be applied for depth motion data. For each unit, top-left sub-block is used for representative motion data in the same manner as the current 3D-HEVC. The cases (1, 3, 5) executes immediate subsampling after each depth is coded, while the cases (2, 4) executes progressive subsampling. Note that the ratio 1/256 is same as LCU level.

Table 1. The proposed subsampling ratio for depth motion data

|  | After each depth is coded | After AU is coded |
| --- | --- | --- |
| Anchor | 1/4 (8x8 unit) | 1/16 (16x16 unit) |
| Case 1 | 1/64 (32x32) | 1/64 (32x32) |
| Case 2 | 1/64 (32x32) | 1/256 (64x64, LCU) |
| Case 3 | 1/256 (64x64, LCU) | 1/256 (64x64, LCU) |
| Case 4 | 1/256 (64x64, LCU) | 1/1024 (128x128, 4LCU) |
| Case 5 | 1/1024 (128x128, 4LCU) | 1/1024 (128x128, 4LCU) |

The Fig. 2 describes the case 2 that utilizes selective subsample ratio on progressive motion data compression for depth.

Fig. 2. The proposed subsampling method for depth motion data (Case 2)

The Fig. 3 describes the case 3 that utilizes selective subsample ratio on immediate motion data compression for depth.

Fig. 3. The proposed subsampling method for depth motion data (Case 3)

# Experimental results

All the cases shown in Table 1 are experimented on common test condition [2], HTM-12.0[1]. Table 2 shows a summary results on the test cases. According to the simulation results, the proposed method achieves negligible coding loss on video and synthesized BD-rate while an amount of motion buffer is reduced.

Table 2. Summary results on the test cases

|  | Subsample ratio (after each depth is coded) | Subsample ratio (after AU is coded) | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR/total bitrate |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Anchor | 1/4 | 1/16 | - | - | - | - | - |
| Case 1 | 1/64 | 1/64 | 0.01 | 0.02 | 0.01 | 0.01 | 0.02 |
| Case 2 | 1/64 | 1/256 | -0.01 | 0.00 | 0.00 | 0.01 | 0.05 |
| Case 3 | 1/256 | 1/256 | -0.01 | 0.00 | 0.00 | 0.03 | 0.08 |
| Case 4 | 1/256 | 1/1024 | 0.02 | -0.01 | 0.00 | 0.04 | 0.12 |
| Case 5 | 1/1024 | 1/1024 | 0.02 | -0.01 | 0.00 | 0.06 | 0.16 |

Table 3. Case 1: Immediate (1/64, 1/64)

Table 4. Case 2: Progressive (1/64, 1/256)

Table 5. Case 3: Immediate (1/256, 1/256)

Table 6. Case 4: Progressive (1/256, 1/1024)

Table 7. Case 5: Immediate (1/1024, 1/1024)

# Conclusion

It is proposed to apply higher sampling ratio in depth motion data then texture. It is recommended to adopt this method in next 3D-HEVC an HTM.

# References

- HTM-12, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-12.0

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G1100, Jan., 2014.

# Patent rights declaration(s)

ETRI and Kyung Hee University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).