| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0056 |
| --- | --- |

| Title: | On MFC plus Depth: Reference Software Update |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lucia Grimaldi, Klaas Schueuer Rheinstrasse 45, Aufgang 1 12161 Berlin, Germany Peng Yin, Taoran Lu, Hariharan Ganapathy, Tao Chen, Walt Husak 432 Lakeside Dr. Sunnyvale CA 94085 USA | Tel: Email: | +1 408 330 3251 {pyin,kschu,wjh}@dolby.com |
| Source: | Dolby Laboratories Inc. |  |  |

_____________________________

# Abstract

This contribution provides an update on the MFC+D reference software source code and software manual. All the required features for MFC+D have been implemented and included in the software.

# Introduction

This contribution provides an update on the MFC+D reference software source code and software manual. Compared to the last version [1], the main features added are the support for interlaced coding. The software supports two different settings: 1) interlaced texture view components and interlace depth view components; 2) interlaced texture view components and progressive depth view components. Other added features: 1) flexible texture and depth view coding order. Three coding orders have been tested: T0D0D1T1, T0D0T1D1 and T0T1D0D1. 2) FPA SEI is only attached with the base texure view components. When the configuration files are configured as MVC+D case, the encoded bitstreams can be decoded by the 3D-AVC reference software in [2]. Therefore, all required features for MFC+D are supported in this software: 1) texture view components and depth view components can be coded at different resolutions; 2) interlaced texture view components and progressive depth view components; 3) interlaced texture view components and interlaced depth view components.

Note: for the interlaced coding, same as for the 3D-AVC reference software, there are some limitations due to memory requirement. For further details please refer to the attached software manual and example configuration files under bin\interlaced folder.

# Software Usage

The MFC+D reference software is attached within this contribution, including the MFC Depth software manual which conforms to MFC reference software manual.

A simple usage of the software is described below:

Example usage of the encoder: lencod_Release_x64.exe -d encoder_mfc_texture.cfg -depd encoder_mfc_depth.cfg

Example usage of the decoder:

- Decoding of texture view Base layer and Enhancement layer bitstreams and depth view View 0 and View 1 bitstream: ldecod_Release_x64.exe -d decoder_mfc_depth.cfg -p ExportViews=0

- Full reconstruction of left and right texture views from MFC and decoding of depth views View 0 and View 1: ldecod_Release_x64.exe -d decoder_mfc_depth.cfg -p ExportViews=1 -p DemuxMode=1

The software also provides example configurations for reduced depth resolution, interlaced texture and interlaced depth, and interlaced texture and progressive depth.

- lencod_Release_x64.exe -d encoder_mfc_texture.cfg -depd encoder_mfc_sub_depth.cfg

- lencod_Release_x64.exe	-d encoder_mfc_interlace_texture.cfg -depd  encoder_mfc_interlace_depth.cfg

- lencod_Release_x64.exe	-d encoder_mfc_interlace_texture.cfg -depd  encoder_mfc_progressive_depth.cfg

# Acknowledgement

We would like to thank Dong Tian from MERL, Ching-Chieh from ITRI and Menno Wildeboer from Philips for their help with the implementation and/or crosscheck of the software.

# References

- L. Grimaldi, K. Schueuer, P. Yin, T. Lu, H. Ganapathy, T. Chen, W. Husak, "On MFC plus Depth: Reference Software Update", JCT3V-I0127, Sapporo, JP, Mar. 2014

- D. Tian, A. Vetro and S. Shimizu, "AhG4: Software Improvements on 3DVATM", JCT3V-I0025, Sapporo, Japan, Jul. 2014

# Patent rights declaration(s)

Dolby Laboratories Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).