Information technology – Coding of audio-visual objects – Part 4: Conformance test, AMENDMENT X: Conformance test for MFC plus Depth extension of AVC

In 10.6.5.7, add the following text at the end of the subclause:

A decoder that conforms to the MFC Depth High profile at a specific level shall be capable of decoding all bitstreams in which all active MVCD sequence parameter sets have any of the following:

–	profile_idc is equal to 128

–	profile_idc is equal to 118 and constraint_set5_flag is equal to 1

–	profile_idc is equal to 134

–	profile_idc is equal to 135,

–	profile_idc is equal to 138,

–	profile_idc is equal to 100 or 77

–	profile_idc is equal to 88 and constraint_set1_flag equal to 1

–	profile_idc is equal to 66 and constraint_set1_flag equal to 1

and in which level_idc or the combination of level_idc and constraint_set3_flag for all active MVCD sequence parameter sets represent a level less than or equal to the specific level. In addition to the bitstreams defined in Table-AMDxx-1, a decoder that conforms to the MFC Depth High profile shall be capable of decoding the Main profile bitstreams specified in Table AMD6-1, the High profile bitstreams specified in Table AMD 9-2, the MVC High profile bitstreams specified in Table AMD xxx, the MFC High profile bitstreams specified in Table AMD xxx, and the MVC Depth High profile bitstreams specified in Table AMD xxx.

After 10.6.6.xx. x, add the following text:

- 10.6.6.36 Test bitstreams – MFC Depth High Profile

- 10.6.6.36.1 Test bitstream # MFCDDR-1

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an IDR access unit. Each view component contains only one slice. num_views_minus1 is equal to 1. NumDepthViews is equal to 2. The base layer texture view is in SbS frame compatible format. Both the texture view and depth view components are progressive. The coding order of texture view and depth view is specified as "T0D0D1T1", i.e., texture from view point 0, followed by depth from view point 0, followed by depth from view point 1, and followed by texture from view point 1. The width and the height of depth view components are equal to the texture view components. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T H.264 | ISO/IEC 14496-10.

Functional stage: Decoding of the base and enhancement texture view component and depth view component and reconstruction of the enhanced resolution stereo texture views.

Purpose: Check that the decoder can properly decode the base and enhancement texture view components and the same resolution depth view components, and reconstruct the enhanced resolution stereo texture views.

- 10.6.6.36.2 Test bitstream # MFCDDR-2

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an IDR access unit. Each view component contains only one slice. num_views_minus1 is equal to 1. NumDepthViews is equal to 2. The base layer texture view is in SbS frame compatible format. Both the texture view and depth view components are progressive. The coding order of texture view and depth view is specified as "T0D0D1T1", i.e., texture from view point 0, followed by depth from view point 0, followed by depth from view point 1, and followed by texture from view point 1. The width and the height of the depth view components are half of the texture view components. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T H.264 | ISO/IEC 14496-10.

Functional stage: Decoding of the base and enhancement texture view component and depth view component and reconstruction of the enhanced resolution stereo texture views.

Purpose: Check that the decoder can properly decode the base and enhancement texture view components and lower resolution depth view components, and reconstruct the enhanced resolution stereo texture views.

- 10.6.6.36.3 Test bitstream #MFCDFLD-1

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an IDR access unit. Each view component contains only one slice. num_views_minus1 is equal to 1. NumDepthViews is equal to 2. The base layer texture view is in SbS frame compatible format. The coding order of texture view and depth view is specified as "T0D0D1T1", i.e., texture from view point 0, followed by depth from view point 0, followed by depth from view point 1, and followed by texture from view point 1. . field_pic_flag is equal to 1 in texture view components. field_pic_flag is equal to 0 in depth view components. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T H.264 | ISO/IEC 14496-10.

Functional stage: Decoding of the base and enhancement interlaced texture view component and interlaced depth view component and reconstruction of the enhanced resolution stereo texture views.

Purpose: Check that the decoder can properly decode the interlaced base and enhancement texture view component and progressive depth components, and reconstruct the enhanced resolution stereo texture views.

- 10.6.6.36.4 Test bitstream #MFCDFLD-2

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an IDR access unit. Each view component contains only one slice. num_views_minus1 is equal to 1. NumDepthViews is equal to 2. The base layer texture view is in SbS frame compatible format. The coding order of texture view and depth view is specified as "T0D0D1T1", i.e., texture from view point 0, followed by depth from view point 0, followed by depth from view point 1, and followed by texture from view point 1. field_pic_flag is equal to 1 for both texture view components and depth view components. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T H.264 | ISO/IEC 14496-10.

Functional stage: Decoding of the base and enhancement interlaced texture view component and progressive depth view component and reconstruction of the enhanced resolution stereo texture views.

Purpose: Check that the decoder can properly decode the interlaced base and enhancement texture view component and interlaced depth view components, and reconstruct the enhanced resolution stereo texture views.

- 10.6.6.36.5 Test bitstream #MFCDTDC-1

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an IDR access unit. Each view component contains only one slice. num_views_minus1 is equal to 1. NumDepthViews is equal to 2. The base layer texture view is in SbS frame compatible format. Both the texture view and depth view components are progressive. The coding order of texture view and depth view is specified as "T0D0T1D1", i.e., texture from view point 0, followed by depth from view point 0, followed by texture from view point 1, followed by depth from view point 1. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T H.264 | ISO/IEC 14496-10.

Functional stage: Decoding of the base and enhancement texture view component and depth view component and reconstruction of the enhanced resolution stereo texture views.

Purpose: Check that the decoder can properly decode the base and enhancement texture view components and depth view components with various coding order, and reconstruct the enhanced resolution stereo texture views.

- 10.6.6.36.6 Test bitstream #MFCDTDC-2

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an IDR access unit. Each view component contains only one slice. num_views_minus1 is equal to 1. NumDepthViews is equal to 2. The base layer texture view is in SbS frame compatible format. Both the texture view and depth view components are progressive. The coding order of texture view and depth view is specified as "T0T1D0D1", i.e., texture from view point 0, followed by texture from view point 1, followed by depth from view point 0, followed by depth from view point 1. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T H.264 | ISO/IEC 14496-10.

Functional stage: Decoding of the base and enhancement texture view components and depth view component and reconstruction of the enhanced resolution stereo texture views.

Purpose: Check that the decoder can properly decode the base and enhancement texture view components and depth view components with various coding order, and reconstruct the enhanced resolution stereo texture views.

- 10.6.6.36.7 Test bitstream #MFCDMFC-1

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an IDR access unit. Each view component contains only one slice. num_views_minus1 is equal to 1. NumDepthViews is equal to 2. Both the texture view and depth view components are progressive. The coding order of texture view and depth view is specified as "T0D0D1T1", i.e., texture from view point 0, followed by depth from view point 0, followed by depth from view point 1, and followed by texture from view point 1. mfc_format_idc is set to 1, i.e., the base layer texture view is in TaB frame compatible format. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T H.264 | ISO/IEC 14496-10.

Functional stage: Decoding of the base and enhancement texture view component and depth view component and reconstruction of the enhanced resolution stereo texture views.

Purpose: Check that the decoder can properly decode the base and enhancement texture and depth view components with various base layer texture frame compatible format, and reconstruct the enhanced resolution stereo texture views.

Add the following after Table AMD4x.1:

Table AMD4x.1 – Bitstreams for the MFC Depth High profile

| Categories | Bitstream | Donated by | File Name | MFC Depth High | Level | Frame Rate (Frame/Sec) |
| --- | --- | --- | --- | --- | --- | --- |
| Depth Resolution | MFCDDR-1 | Dolby | MFCDDR-1 | X | 4.1 | 25p |
|  | MFCDDR-2 | Dolby | MFCDDR-2 | X | 4.1 | 25p |
| Interlaced Coding Tools | MFCDFLD-1 | Dolby | MFCDFLD-1 | X | 4.1 | 50i |
|  | MFCDFLD-2 | Dolby | MFCDFLD-2 | X | 4.1 | 50i |
| Texture and Depth Coding Order | MFCDTDC-2 | Dolby | MFCDTDC-2 | X | 4.1 | 25p |
|  | MFCDTDC-2 | Dolby | MFCDTDC-2 | X | 4.1 | 25p |
| MFC format | MFCDMFC-1 | Dolby | MFCDMFC-1 | X | 4.1 | 25p |