| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18 – 24 Oct. 2014 | Document: JCT3V-J0059 |
| --- | --- |

| Title: | A cleanup of the size derivation for 3D-HEVC merge candidate list |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian-Liang Lin, Yi-Wen Chen, Yu-Wen Huang, Shawmin Lei MediaTek Hsinchu, No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jl.lin, yiwen.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In 3D-HEVC, to improve the coding efficiency and explore the inter-view and inter-component correlation, 3D extra merge candidates are included in the merge candidate list. The enabling of those extra merge candidates are controlled by three flags transmitted in the VPS extension 2 including iv_mv_pred_flag, mpi_flag, and view_synthesis_pred_flag. In order to utilize those 3D extra merge candidates in the merge candidate list, the size of the 3D extra merge candidate list is increased by 1 for the dependent texture and depth map. However, in current Draft Text, the determination of whether to increase the merge candidate list size depends only on the iv_mv_pred_flag and mpi_flag. The view_synthesis_pred_flag is missed in the decision. To fix this issue, it is proposed to include the view_synthesis_pred_flag in the determination.

# Problem Statement

In 3D-HEVC [1], the merge mode of the independent texture view is coded using the original HEVC merge candidates. To improve the coding efficiency and explore the inter-view and inter-component correlation in 3D-HEVC, 3D extra merge candidates are added in the 3D-HEVC merge candidate set for the dependent texture and depth map. The 3D extra merge candidates include the IVMP, shift IVMP, DV, shift DV, VSP, MPI, DDD, and VSP inheritance candidates. The enabling of those extra merge candidates are controlled by three control flags transmitted in the Video Parameter Set extension 2 (VPS_extension2) as summarized as follows:

- iv_mv_pred_flag: enabling of the IVMP and DV candidates

- mpi_flag: enabling of the MPI candidate

- view_synthesis_pred_flag: enabling of the VSP and VSP inheritance candidates

In order to utilize those 3D extra merge candidates in the merge candidate list, the size of the 3D extra merge candidate list is set as the size of the HEVC merge candidate list plus 1. However, in current Draft Text as shown in Table 1, the determination of whether to increase the merge candidate list size is only depending on the iv_mv_pred_flag and mpi_flag. The view_synthesis_pred_flag is missed in the decision. To fix this issue, it is proposed to include the view_synthesis_pred_flag in the determination.

Table 1.  The derivation of merge candidate list size in current Draft Text

five_minus_max_num_merge_cand specifies the maximum number of merging MVP candidates supported in the slice subtracted from 5.

The variable NumExtraMergeCand is derived as specified in the following:

  - NumExtraMergeCand = iv_mv_pred_flag[ nuh_layer_id ]  | |  mpi_flag[ nuh_layer_id ]		(I-1)

The maximum number of merging MVP candidates, MaxNumMergeCand is derived as follows:

  - MaxNumMergeCand = 5 − five_minus_max_num_merge_cand + NumExtraMergeCand		(I-2)

The value of MaxNumMergeCand shall be in the range of 1 to ( 5 + NumExtraMergeCand ), inclusive.

# Proposed Text Changes

The added part is highlighted in yellow color.

five_minus_max_num_merge_cand specifies the maximum number of merging MVP candidates supported in the slice subtracted from 5.

The variable NumExtraMergeCand is derived as specified in the following:

  - NumExtraMergeCand = iv_mv_pred_flag[ nuh_layer_id ]  | |  mpi_flag[ nuh_layer_id ]  | |  view_synthesis_pred_flag[ nuh_layer_id ]		(I-3)

The maximum number of merging MVP candidates, MaxNumMergeCand is derived as follows:

  - MaxNumMergeCand = 5 − five_minus_max_num_merge_cand + NumExtraMergeCand		(I-4)

The value of MaxNumMergeCand shall be in the range of 1 to ( 5 + NumExtraMergeCand ), inclusive.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 5,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-I1001-v3, July, 2014.