| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18 – 24 Oct. 2014 | Document: JCT3V-J0060 |
| --- | --- |

| Title: | 3D-HEVC HLS: Single depth flag signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Jian-Liang Lin, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yiwen.chen, jl.lin, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In current 3D-HEVC, most of the 3D coding tools (e.g. IVMP, sub-PU IVMP, DONBDV, VSP, MPI, DMM, SDC. DBBP, residual prediction) have a control flag signaled in VPS to provide flexibility to enable/disable the associated coding tools. However, there is no such high-level control flag for single depth intra mode. In this proposal, we propose to add one control flag in VPS for single depth mode.

# Introduction

For the purpose of flexibility to enable/disable 3D-HEVC coding tools, most of the coding tools have a control flag signaled in VPS as shown in Table 1. When single depth mode was proposed [1], there was no such flag to control the single depth mode. In this proposal, we propose to add one control flag in VPS.

| vps_extension2( ) { | Descriptor |
| --- | --- |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| layerId = layer_id_in_nuh[ i ] |  |
| iv_mv_pred_flag[ layerId ] | u(1) |
| iv_mv_scaling_flag[ layerId ] | u(1) |
| if ( !VpsDepthFlag[ layerId ] ) { |  |
| log2_sub_pb_size_minus3[ layerId ] | ue(v) |
| iv_res_pred_flag[ layerId ] | u(1) |
| depth_refinement_flag[ layerId ] | u(1) |
| view_synthesis_pred_flag[ layerId ] | u(1) |
| depth_based_blk_part_flag[ layerId ] | u(1) |
| } else { |  |
| mpi_flag[ layerId ] | u(1) |
| log2_mpi_sub_pb_size_minus3[ layerId ] | ue(v) |
| dmm_cpredtex_flag[ layerId ] | u(1) |
| intra_sdc_dmm_wfull_flag[ layerId ] | u(1) |
| lim_qt_pred_flag[ layerId ] | u(1) |
| inter_sdc_flag[ layerId ] | u(1) |
| } |  |

Table 1 VPS in current 3D-HEVC

# Proposed Text

In this contribution, a new flag “single_depth_flag” is proposed to improve the flexibility of single depth mode. As shown in Table 2, the added syntax is highlighted in yellow color.

Table 2 Proposed control flag for single depth mode in VPS

| vps_extension2( ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| layerId = layer_id_in_nuh[ i ] |  |
| iv_mv_pred_flag[ layerId ] | u(1) |
| iv_mv_scaling_flag[ layerId ] | u(1) |
| if ( !VpsDepthFlag[ layerId ] ) { |  |
| log2_sub_pb_size_minus3[ layerId ] | ue(v) |
| iv_res_pred_flag[ layerId ] | u(1) |
| depth_refinement_flag[ layerId ] | u(1) |
| view_synthesis_pred_flag[ layerId ] | u(1) |
| depth_based_blk_part_flag[ layerId ] | u(1) |
| } else { |  |
| mpi_flag[ layerId ] | u(1) |
| log2_mpi_sub_pb_size_minus3[ layerId ] | ue(v) |
| dmm_cpredtex_flag[ layerId ] | u(1) |
| intra_sdc_dmm_wfull_flag[ layerId ] | u(1) |
| lim_qt_pred_flag[ layerId ] | u(1) |
| inter_sdc_flag[ layerId ] | u(1) |
| single_depth_flag[ layerId ] | u(1) |
| } |  |
| } |  |
| … |  |
| } |  |

| slice_segment_header_extension ( ) { | Descriptor |
| --- | --- |
| … |  |
| If(single_depth_flag[ nuh_layer_id ]){ |  |
| slice_single_sample_mode_enable_flag | u(1) |
| } |  |
| … |  |
| } |  |

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Y.-W. Chen, J.-L. Lin, Y.-W. Huang, S. Lei, “Single depth intra mode for 3D-HEVC,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-I0095, July, 2014.