| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0061 |
| --- | --- |

| Title: | AHG7: Draft Report of Complexity Assessment |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | G. G. Lee T. Ikai H. Liu G. Bang | Tel: Email: | +886-6-2360663 clee@mail.ncku.edu.tw ikai.tomohiro@sharp.co.jp gbang@etri.re.kr hongbinl@qti.qualcomm.com |
| Source: | AHG |  |  |

_____________________________

# Abstract

A summarized report of complexity assessment AHG in 3D-HEVC. This draft report re-states the metrics used to evaluate the coding tools in previous meetings and provides an example to guide the complexity assessment.

# Introduction to Intrinsic Complexity Metrics

The complexity metrics should be transparent to hardware and software implementations. In addition, the complexity metrics are also needed to be indicative of the software, hardware or system architecture requirements. In contrast to the above description, the complexity metrics should be known early in the design phase to avoid design changes in later stages. Hence, four complexity measures has been studied and described in complexity assessment Error: Reference source not found-[3]. In previous meetings, several contributions has been reported to evaluate the coding tools based on these metrics [4]-[11].

## Number of operations

This metric calculates the number of each type of operations, including (I) addition/subtraction/comparison, (II) logic operation, (III) multiplication, (IV) division and (V) shift, which are the basic operators used commonly in signal processing and video codec algorithms. Furthermore, (VI) Look-Up Table (LUT) access should be also assessed due to the fact that it is usually utilized for video codec.

Report of (I) addition/subtraction/comparison, (III) multiplication, (IV) division (if used) and (VI) LUT access (if used) is highly recommended. The report of (II) logic operation and (V) shift can be exempted since those are not significant in terms of computation.	Additions (+) and subtractions (−) are considered to be operators having equivalent complexities since they can be implemented using two's complement and the complexity in using a not operation and extra addition of one is almost negligible. A similar argument extends to describing logical operators such as larger than (>), smaller than (<), and the equality (=) operators as being equivalent in complexity to addition and subtraction.  The three previous logical operations can be performed by checking the sign bit after subtraction which again has negligible complexity. Multipliers may be more complicated than adders in hardware design as multiple additions are needed in the multiplication process. Shift operators are considered the least complex since they can be performed by moving the corresponding significant bits.

In addition, two operand operations with constant input and variable input should be differentiated to provide better accuracy in the estimation of complexity. E.g., X + Y is more complex than X + 5 since Y is a variable while the second operand 5 is a constant. Another example is that X × Y can be more complex than X × 5 since constant multiple can be decomposed into shifts and additions. Furthermore, bit precision of each operands discussed above should also be taken into account, since it can significantly influence the complexity especially in many standards having 4:4:4 data format with more than 10 bits precision.

In video system, CABAC and MC computation is mainly concerned. Therefore the number of context coded bins per pixel and the operation cost in MC-like scheme should be checked.

## Degree of parallelism

Based on the data dependency, this metric on the degree of parallelism characterizes the potential for which the operations can be executed concurrently. The metric defined here is capable of quantifying the degree of parallelism based on the dataflow graph and also provides a systematic methodology in the assessment of the complexity of algorithms using spectral graph theory [12].

This metric should be considered with metric 1, number of operations, concurrently since sometimes the evaluation between non-parallelizable coding tool with few operations and parallelizable coding tool with more operations should be made.

In the context of video system, low level (pixel level/ instruction level)*1 and module level parallelism*2 is very important. It is no need to say slice/tile/WPP level’s importance.

*1: SIMD operation may operate 8 pixels at once (16 bit * 8 pixels = 128 bits register, now we may use 256 bit registers).

*2: Some modules, i.e. CABAC, motion parameter derivation, residual, prediction (MC), loopfilter and some other modules can be operated parallel. We should keep the concept. For example, using the derived motion parameter or prediction data in CABAC may induce pursing dependency. And keeping the balance is also important, i.e. removing most CABAC doesn’t always mean we can use longer MC (vice versa).

## Data storage requirement

Data storage requirement is transparent to hardware/software but depends on the processing order or dataflow of the algorithm. This can be obtained by analyzing the lifetime of the required input data. For this category, a good measurement is to estimate the number of data accesses from the potential data storage device.

It is important to distinguish between data storage which is algorithmic intrinsic and the memory configuration or requirement which is dependent upon implementation issues. For example, different signal processing systems or platforms may use DRAM from different companies with different specifications for the cross page reference. Hence this may result in different design of clock speeds due to efficiency and real time concerns even for the same data size resulting in unfair complexity comparisons. Hence it is significant to stay with algorithmic intrinsic properties and also not to be biased by different platforms having different physical constraints.

## Data transfer rate

The data transfer rate needs to be distinguished from the bandwidth since bandwidth is a platform-dependent metric and it could be influenced by (I) types of data transaction, (II) memory hierarchy, (III) data alignment in memory (memory pattern), and (IV) datapath architecture. On the other hand, data transfer rate is simply accounting for the amount of data should be transferred between modules no matter the destination is internal or external storage and hence it is intrinsic complexity. It is noticeable that the input and output data transfer should be distinguished since the characteristics of writing and reading are different. With the help of the data transfer rate and bandwidth, the designer is provided with the insight of clock speed and bit widths of the interconnect bus

To provide more meaningful assessment, some typical memory patterns or data allocations, which restrict the minimum amount of data should be fetched once, could be involved in data transfer rate, such as 1×1, 2×1, 4×1, 8×1, 4×2 and 8×2. (M×N denotes that M pixels in horizontal direction and N pixels in vertical direction.) Under this consideration, the assessed data transfer rate would be more close to the real bandwidth. However, the number of data access should be investigated to give the penalty on the several burst numbers in accessing DRAM discretely.

It is important to note that:

- All four complexity measures are required during complexity assessment.

- The data granularity used in the dataflow during complexity assessment will result in different implementations

# Example of Complexity Assessment

In this draft report, a case study on bi-directional motion compensation (MC) in HEVC version 1 at decoder side is presented [4]. In the following assessment, the complexity of Prediction Unit (PU) would be assessed first and then the complexity as Coding Tree Unit (CTU) would be conducted based on the results in PU level. (Size of CTU in 3D-HEVC is set to be 64×64 and PU size in the following paragraphs is M×N, where M is width and N is height.)

## Number of operations:

Two major process in MC [13] is two-dimensional interpolation at non-integer pixel position, and in the worst, Luminance data require 8-tap filter and its coefficients are [-1, 4, -11, 40, 40, -11, 4, -1]. (When to-be-interpolated located at 2/4 pixel location.) On the other hand, the Chrominance data require the 4-tap filter, [-2, 58, 10, -2], to generate the non-integer pixel. (This filter is used for pixel located at 1/8 pixel location.) As a consequence, we could assess the number of operations required by one PU.

- Additions/Subtractions: (7×M×(N+7) +7×M×N) + 2×(3×(M/2)×(N/2+3)+3×(M/2)×(N/2))

- Multiplications: (8×(M+7)×N +8×M×N) + 2×(4×(M/2+3)×(N/2)+4×(M/2)×(N/2))

In non-weighting bi-prediction case, we could assess the number in one CU as follows.

- Additions/Subtractions: numAddInPU×2+ M×N

- Multiplications: numMultInPU×2

numAddInPU: number of additions/subtractions in one prediction unit
numMultInPU: number of multiplications in one prediction unit

## Data storage requirement

The data storage requirement denotes that the required data size for computing one data granularity, i.e., PU size here. Thus, for the MC, (I) reference texture data and (II) interpolated data. the following algebric expressions illustrate the requirement of data storage. Note that only luminance component is calculated due to the fact that the buffer could be reused for chrominance components.

- Reference texture data: (M+7)×(N+7) pixels

- Interpolated data: M×N pixels

## Data transfer rate

The data transfer rate accumulate the data transfer between MC and other modules; hence, input and output data transaction should be calculated. For MC, the reference texture data for interpolation and the interpolated data would be transferred. Hence, the algebric expressions of data transfer rate are illustrated as follows.

- Reference texture data: (M+7)×(N+7) + 2×(M/2+3)×(N/2+3) pixels

- Interpolated data: M×N + 2×(M/2)×(N/2) pixels

In the aspect of data transfer rate, we could furthermore consider different memory patterns that restrict the minimum amount of data should be fetched once; hence, the assessed data transfer rate under different memory patterns will be more close to the bandwidth in the real world scenario.

## Summary of complexity assessment at CTU level

CTU is commonly a fair level to evaluate the complexity of coding tools so the complexity at PU size in previous sections would be scaled up to the CTU level. Moreover, since the process of each MC is independent, the complexity at CTU level could be linearly scaled according to the corresponding PU size. The complexity for these three aspects, number of operations, data storage requirement, and data transfer rate, are shown in Table I and Table II in numeric manner; in addition, Table II shows the data transfer rate with various memory patterns.

Table I Complexity assessment of MC at CTU level

| PU size | Number of operations per 64x64 block |  | Data storage requirement (bytes) |
| --- | --- | --- | --- |
|  | Multiplications | Additions/Subtractions | Texture |
| 64×64 | 172,544 | 152,832 | 9,137 |
| 32×32 | 181,248 | 160,256 | 2,545 |
| 16×16 | 198,656 | 175,104 | 785 |
| 8×8 | 233,472 | 204,800 | 289 |
| 8×4 | 151,552 | 129,024 | 197 |
| 4×8 | 116,736 | 99,328 | 197 |

Table II Data transfer rate of MC at CTU level with various memory patterns

| PU size | Data transfer per 64x64 block (bytes) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1×1 | 2×1 | 4×1 | 4×2 | 4×4 | 8×2 | 8×4 | 8×8 |
| 64×64 | 14,982 | 15,264 | 16,112 | 16,416 | 17,632 | 17,280 | 18,560 | 20,480 |
| 32×32 | 17,944 | 18,560 | 20,416 | 21,120 | 23,936 | 23,040 | 26,112 | 30,720 |
| 16×16 | 24,672 | 26,112 | 30,464 | 32,256 | 39,424 | 36,864 | 45,056 | 57,344 |
| 8×8 | 41,344 | 45,056 | 56,320 | 61,440 | 81,920 | 73,728 | 98,304 | 122,880 |
| 8×4 | 30,080 | 32,768 | 40,960 | 46,080 | 61,440 | 55,296 | 73,728 | 122,880 |
| 4×8 | 30,080 | 33,792 | 45,056 | 49,152 | 65,536 | 73,728 | 98,304 | 122,880 |

# Conclusion

With assessed complexity, the development of video coding standard could consider algorithmic performance and architectural complexity simultaneously to avoid for over-emphasizing on algorithmic performance or complexity reduction alone. This report provides transparent complexity metrics to evaluate the complexity; thus, it is recommended to consider these metrics in developing video coding standards in the future.

# Reference

- G. G. (Chris) Lee, Y.-K. Chen, M. Mattavelli, and E. S. Jang, "Algorithm/Architecture Co-Exploration of Visual Computing: Overview and Future Perspectives," IEEE Trans. on Circuits and Systems for Video Technology. Vol. 19, Iss.11, pp. 1576-1587, Nov. 2009.

- G. G. Lee, H.-Y. Lin, and S.-Y. Kung, "Algorithm/Architecture Co-exploration," in Multimedia Image and Video Processing, Second Edition, ed: CRC Press, 2012, pp. 573-608.

- JCT3V-D0301, “AHG 10 Overview of Algorithmic Intrinsic Complexity Measures,” Inchon, KR, 20 – 26 Apr. 2013.

- JCT3V-I0111, “AHG7: Complexity assessment of inter-prediction tools,” Sapporo, JP, 3 – 9 Jul. 2014

- JCT3V-I0069, “AHG7: Complexity Assessment on Depth Intra Modes with Simplified Data Flows,” Sapporo, JP, 3 – 9 Jul. 2014

- JCT3V-H0127, “AHG8: Complexity Assessment on Depth Intra Modes,” Valencia, ES, 29 March – 4 April 2014

- JCT3V-G0114, “AHG 10: Complexity Assessment on Illumination Compensation (IC),” San Jose, USA, 11 – 17 Jan. 2014.

- JCT3V-G0208, “AHG10: Complexity assessment of motion compensation process,” San Jose, USA, 11 – 17 Jan. 2014.

- JCT3V-E0132, “AHG 10: Complexity Assessment on Simplified Depth Coding (SDC),” Vienna, AT, 27 July – 2 Aug. 2013.

- JCT3V-D0090, “AhG 10: Complexity Assessment on Adaptive Luminance Compensation and Plane Segmentation based Intra Prediction in 3D-AVC,” Inchon, KR, 20 – 26 Apr. 2013.

- JCT3V-D0089, “3D-HEVC: Adaptive Virtual Depth Block Partition for View Synthesis Prediction and Corresponding Complexity Analysis,” Inchon, KR, 20 – 26 Apr. 2013.

- G. G. Lee, H.-Y. Lin, C.-F. Chen, T.-Y. Huang, "Quantifying Intrinsic Parallelism Using Linear Algebra for Algorithm/Architecture Co-Exploration," IEEE Transactions on Parallel and Distributed Systems, Vol. 23, Iss. 5, pp. 944-957, May. 2012.

- JCTVC-L1003, “High Efficiency Video Coding (HEVC) text specification draft 10 (for FDIS & Consent),” Geneva, CH, 14-23 Jan. 2013.

# Patent rights declaration(s)

NCKU may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).