| - Joint Collaborative Team on 3D Video Coding Extensions - of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 - 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0062 |
| --- | --- |

| Title: | Software for Texture and Depth View Packing SEI message in ATM13.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takanori Senoh, Koki Wakunami, Yasuyuki Ichihashi, Hisayuki Sasaki, Ryutaro Oi, Kenji Yamamoto Masayuki Tanimoto | Tel: Email: | {senoh, k.wakunami, y-ichihashi, sasaki, oi, k.yamamoto}@nict.go.jp tanimototentative3@yahoo.co.jp |
| Source: | NICT and NISRI |  |  |

_____________________________

# Abstract

This contribution reports the integration of texture and depth view packing SEI message in ATM13.1. It is proposed to include this software in the reference software for 14496-10:201x/AMD1.

- 1 Introduction

At the 9th JCT-3V meeting, software improvement for texture and depth view packing SEI message for 3D-AVC was reported based on ATM10.0r1 (JCT3V-I0063). This contribution reports the integration of the same SEI message in ATM13.1. ATM13.1 software integrated with this SEI message is attached to this contribution, together with the GVD software which uses this SEI message.

- 2 Experimental results

The encoding, decoding and view-synthesis processes are same as reported in JCT3V-H0061. The SEI message decoder in ATM13.1 outputs GVDViewSynth.cfg file, GVDCameraParam.txt file and GVDCameraParam.CSV file. GVDViewSynth.cfg file contains parameter values for depth_type, num_residual_texture_views_minus1, residual_depth_flag, and other parameter values used for the view synthesis. GVDCameraParam.txt file contains parameter values for intrinsic camera parameters (focal length, principal point co-ordinates) and extrinsic camera parameters (rotation matrix and x-translation) for view synthesis. GVDCameraParam.CSV file contains parameter values for Znear, Zfar and x-translation for all views.

Table 1 shows the test results for 3-view case. The enc time includes global depth generation, its encoding, residual view generation and encoding of base view, residual view and global depth map. Ren time includes the generation of eight views (view 0, 0.25. 0.5, 0.75, 1.25, 1.5, 1.75 and 2). It was observed that the performance was almost same as the previous report (JCT3V-I0063).

Table 1 Summary of GVD performance comparing to ATMv.13.1 HP mode for 3-view case.

- 3 Recommendation

It is proposed to adopt this software for the reference software of 14496-10:201x/AMD1.

# Patent rights declarations

NICT (National Institute of Information and Communications Technology) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NISRI (Nagoya Industrial Science Research Institute) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).