| - Joint Collaborative Team on 3D Video Coding Extensions - of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 - 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0063r3 |
| --- | --- |

| Title: | Improvement of Alternative depth info SEI message in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takanori Senoh, Koki Wakunami, Yasuyuki Ichihashi, Hisayuki Sasaki, Ryutaro Oi, Kenji Yamamoto Masayuki Tanimoto | Tel: Email: | {senoh, k.wakunami, y-ichihashi, sasaki, oi.ryutaro, k.yamamoto}@nict.go.jp tanimototentative3@yahoo.co.jp |
| Source: | NICT, NISRI |  |  |

_____________________________

# Abstract

This contribution proposes the improvement of alternative depth info SEI message in 3D-HEVC DT5 according to the decision at the 8th JTC-3V meeting and the related voting results for ISO/IEC 14496-10:201x/PDAM1: Multi-Resolution Frame Compatible Stereoscopic Video with Depth Maps, and Additional Supplemental Enhancement Information.

# Relevant DAM text changes in 3D-HEVC

In I.14.2.1 Alternative depth information SEI message syntax, add the following view_synthesis_parameter() after residual_depth_flag.

| alternative_depth_info( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| if( depth_type = = 0 ) { |  |  |
| num_residual_texture_views_minus1 | 5 | ue(v) |
| residual_depth_flag | 5 | u(1) |
| view_synthesis_parameter( ) | 5 |  |
| } |  |  |
| } |  |  |
| } |  |  |

Add a new subclause L.14.2.1.1 “view_synthesis_parameter syntax” as follows

        - View synthesis parameter syntax

| view_synthesis_parameter( ) { | C | Descriptor |
| --- | --- | --- |
| z_flag | 5 | u(1) |
| intrinsic_param_flag | 5 | u(1) |
| rotation_flag | 5 | u(1) |
| translation_flag | 5 | u(1) |
| if( z_flag ) |  |  |
| for( i = 0; i <= num_residual_texture_views_minus1 + 1; i++ ) { |  |  |
| z_near_sign_flag[ i ] | 5 | u(1) |
| z_near_exponent[ i ] | 5 | u(7) |
| z_near_mantissa_len_minus1[ i ] | 5 | u(5) |
| z_near mantissa[ i ] | 5 | u(v) |
| z_far_sign_flag[ i ] | 5 | u(1) |
| z_far_exponent[ i ] | 5 | u(7) |
| z_far_mantissa_len_minus1[ i ] | 5 | u(5) |
| z_far_mantissa[ i ] | 5 | u(v) |
| } |  |  |
| if( intrinsic_param_flag ) { |  |  |
| prec_focal_length | 5 | ue(v) |
| prec_principal_point | 5 | ue(v) |
| } |  |  |
| if( rotation_flag ) |  |  |
| prec_rotation_param | 5 | ue(v) |
| if ( translation_flag ) |  |  |
| prec_translation_param | 5 | ue(v) |
| for( i = 0; i <= num_residual_texture_views_minus1 + 1; i++ ) { |  |  |
| if( intrinsic_param_flag ) { |  |  |
| sign_focal_length_x[ i ] | 5 | u(1) |
| exponent_focal_length_x[ i ] | 5 | u(6) |
| mantissa_focal_length_x[ i ] | 5 | u(v) |
| sign_focal_length_y[ i ] | 5 | u(1) |
| exponent_focal_length_y[ i ] | 5 | u(6) |
| mantissa_focal_length_y[ i ] | 5 | u(v) |
| sign_principal_point_x[ i ] | 5 | u(1) |
| exponent_principal_point_x[ i ] | 5 | u(6) |
| mantissa_principal_point_x[ i ] | 5 | u(v) |
| sign_principal_point_y[ i ] | 5 | u(1) |
| exponent_principal_point_y[ i ] | 5 | u(6) |
| mantissa_principal_point_y[ i ] | 5 | u(v) |
| } |  |  |
| if( rotation_flag ) |  |  |
| for ( j = 1; j <= 3; j++ ) /* row */ |  |  |
| for( k = 1; k <= 3; k++ ) { /* column */ |  |  |
| sign_r[ i ][ j ][ k ] | 5 | u(1) |
| exponent_r[ i ][ j ][ k ] | 5 | u(6) |
| mantissa_r[ i ][ j ][ k ] | 5 | u(v) |
| } |  |  |
| if ( translation_flag ) { |  |  |
| sign_t_x[ i ] | 5 | u(1) |
| exponent_t_x[ i ] | 5 | u(6) |
| mantissa_t_x[ i ] | 5 | u(v) |
| } |  |  |
| } |  |  |
| } |  |  |

In I.14.3.1 Alternative depth information SEI message semantics, replace the whole text beginning with num_residual_texture_views_minus1 by the following text.

When depth_type = 0, alternative depth information SEI message informs the global view and depth (GVD) information.

The GVD information informs the decoder that view components of one output view contain samples of multiple distinct spatially packed constituent residual pictures. The related output view is not suitable to be displayed directly and may be converted to displayable views by using the view synthesis parameters. This information can be used after the decoder output to appropriately rearrange the samples to produce displayable views that are appropriate for display or other purposes (which are outside the scope of this Recommendation | International Standard).

The GVD information, when present, shall be associated with an IDR access unit. The information signaled in this SEI message applies to all the access units in the coded video sequence. This SEI message conveys information associated with the coded video sequence that outputs the views that may be further processed to generate the displayable views.

In such a coded video sequence, there are at least two output views, wherein the base view is coded as a displayable view and a non-base view is a residual view. A residual view is not a displayable view. Instead it contains a sequence of difference pictures which are decoded view components of the residual view. Each difference picture is created by frame packing arrangement containing up to four constituent residual pictures.

Each constituent residual picture represents a picture corresponding to a 2:1 decimation from the residual picture horizontally and vertically, starting from the top-left-most sample.

Each constituent residual picture is considered as being produced by the pictures of two displayable views, wherein one, which is denoted as view A, belongs to the view that is coded as the base view and is used to predict another picture in another view, which is referred to herein as a view B. The residual picture of such a process is considered as the residual picture associated with view B. The residual picture consists of the samples which may not be predictable by the base view, namely occluded samples and out-of-picture samples relative to the base picture. After decimation from the residual picture associated with view B, a constituent residual picture associated with view B is generated.

The coded depth views follow the same picture structure as described above.

num_residual_texture_views_minus1 plus 1 specifies the number of constituent residual texture pictures packed into each texture view component of a non-base view. num_residual_texture_views_minus1 shall be in the range of 0 to 3, inclusive.

residual_depth_flag equal to 1 specifies that residual depth pictures are packed into the depth components of the non-base view, with a packing scheme that is identical to the one used for the residual texture pictures. The depth components of the base view shall always be present, independent of the value of residual_depth_flag.

When this SEI message is present, it is desirable to disable inter-view and inter-component prediction.

Furthermore, the coded video sequence shall contain two texture views, wherein the non-base texture view is the only residual view containing the texture information and is composited by spatially packed residual pictures for texture. The number of the coded depth views in the coded video sequence shall be equal to 1 + residual_depth_flag.

The view number (with a value from 0 to num_residual_texture_views_minus1 + 1, inclusive) in the view synthesis parameter syntax table indicates the location of the constituent residual pictures in the non-base texture view as shown in Table I-xx. In the view synthesis parameter syntax table, i equal to 0 indicates the base texture view.

The horizontally and vertically 2:1 subsampled constituent residual pictures are packed in the view components of the non-base texture view as shown in Table I-xx. Let the output non-base texture view have a width equal to W and a height equal to H for the luma component. The texture and depth view packing SEI message shall not be present unless W is a multiple of 2 * SubWidthC and H shall be a multiple of is a multiple of 2 * SubHeightC. Each constituent residual depth picture of the non-base depth view component is associated with a constituent residual picture of the non-base texture view component in the same relative location.

- Table I-xx – Top-left corner coordinates of constituent residual pictures packed in a residual view of width W and height H.

| i | Top-left corner coordinate |
| --- | --- |
| 1 | (0, 0) |
| 2 | (H/2, 0) |
| 3 | (0, W/2) |
| 4 | (H/2, W/2) |

Add a new subclause I.14.3.1.1 “View synthesis parameter semantics” as follows:

      - View synthesis parameter semantics

The syntax structure specifies parameters of view synthesis parameters which are used to synthesize views from the decoded base view and constituent residual pictures.

z_flag equal to 1 indicates the presence of the syntax elements z_near_sign_flag[ i ], z_near_exponent[ i ],  z_near_mantissa_len_minus1[ i ], z_near mantissa[ i ], z_far_sign_flag[ i ], z_far_exponent[ i ], z_far_mantissa_len_minus1[ i ], and z_far_mantissa[ i ], for i ranging from 0 to num_residual_texture_views_minus1 + 1. z_flag equal to 0 indicates that these syntax elements are not present.

intrinsic_param_flag equal to 1 indicates the presence of intrinsic camera parameter syntax elements. intrinsic_param_flag equal to 0 indicates that these syntax elements are not present.

rotation_flag equal to 1 indicates the presence of rotation camera parameter syntax elements. rotation_flag equal to 0 indicates that these syntax elements are not present. When rotation_flag is 0, a default rotation camera parameter of a unit matrix value is inferred.

translation_flag equal to 1 indicates the presence of horizontal translation camera parameter syntax elements. translation_flag equal to 0 indicates that these syntax elements are not present.

z_near_sign_flag[ i ] equal to 0 indicates that the sign of the nearest depth value of the i-th camera is positive. z_near_sign_flag[ i ] equal to 1 indicates that the sign of the nearest depth value of the i-th camera is negative.

z_near_exponent[ i ] specifies the exponent part of the nearest depth value of the i-th camera. The value of z_near_exponent[ i ] shall be in the range of 0 to 27−2, inclusive. The value 27−1 is reserved for future use by ITU-T | ISO/IEC. When z_near_exponent[ i ] is equal to 27−1, the value of z near[ i ] is unspecified.

z_near_mantissa_len_minus1[ i ] + 1 specifies the number of bits in the mantissa of the nearest depth value of the i-th camera. The value of z_near_mantissa_len_minus1 [ i ] shall be in the range of 0 to 31, inclusive.

z_near_mantissa[ i ] specifies the mantissa part of the nearest depth value of the i-th camera.

z_far_sign_flag[ i ] equal to 0 indicates that the sign of the farthest depth value of the i-th camera is positive. z_far_sign_flag[ i ] equal to 1 indicates that the sign of the farthest depth value of the i-th camera is negative.

z_far_exponent[ i ] specifies the exponent part of the farthest depth value of the i-th camera. The value of z_far_exponent[ i ] shall be in the range of 0 to 27−2, inclusive. The value 27−1 is reserved for future use by ITU-T | ISO/IEC. When z_far_exponent[ i ] is equal to 27−1, the value of z far[ i ] is unspecified.

z_far_mantissa_len_minus1[ i ] + 1 specifies the number of bits in the mantissa of the farthest depth value of the i-th camera. The value of mantissa_len_minus1_z_far[ i ] shall be in the range of 0 to 31, inclusive.

z_far_mantissa[ i ] specifies the mantissa part of the farthest depth value of the i-th camera.

prec_focal_length specifies the exponent of the maximum allowable truncation error for focal_length_x[ i ] and focal_length_y[ i ] as given by 2−prec_focal_length. The value of prec_focal_length shall be in the range of 0 to 31, inclusive.

prec_principal_point specifies the exponent of the maximum allowable truncation error for principal_point_x[ i ] and principal_point_y[ i ] as given by 2−prec_principal_point. The value of prec_principal_point shall be in the range of 0 to 31, inclusive.

prec_rotation_param specifies the exponent of the maximum allowable truncation error for r[ i ][ j ][ k ] as given by 2−prec_rotation_param. The value of prec_rotation_param shall be in the range of 0 to 31, inclusive.

prec_translation_param specifies the exponent of the maximum allowable truncation error for t_x[i] as given by 2−prec_translation_param. The value of prec_ translation_param shall be in the range of 0 to 31, inclusive.

sign_focal_length_x[ i ] equal to 0 indicates that the sign of the focal length of the i-th camera in the horizontal direction is positive. sign_focal_length_x[ i ] equal to 1 indicates that the sign of the focal length of the i-th camera in the horizontal direction is negative.

exponent_focal_length_x[ i ] specifies the exponent part of the focal length of the i-th camera in the horizontal direction. The value of exponent_focal_length_x[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. When exponent_focal_length_x[ i ] is equal to 63, the value of focal length of the horizontal direction for the i-th camera is unspecified.

mantissa_focal_length_x[ i ] specifies the mantissa part of the focal length of the i-th camera in the horizontal direction. The length of the mantissa_focal_length_x[ i ] syntax element is determined as follows:

−	If exponent_focal_length_x[ i ] = = 0, the length is Max( 0, prec_focal_length − 30 ).

−	Otherwise (0 < exponent_focal_length_x[ i ] < 63), the length is Max( 0, exponent_focal_length_x[ i ] + prec_focal_length − 31 ).

sign_focal_length_y[ i ] equal to 0 indicates that the sign of the focal length of the i-th camera in the vertical direction is positive. sign_focal_length_y[ i ] equal to 1 indicates that the sign of the focal length of the i-th camera in the vertical direction is negative.

exponent_focal_length_y[ i ] specifies the exponent part of the focal length of the i-th camera in the vertical direction. The value of exponent_focal_length_y[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. When exponent_focal_length_y[ i ] is equal to 63, the value of focal length of the vertical direction is unspecified.

mantissa_focal_length_y[ i ] specifies the mantissa part of the focal length of the i-th camera in the vertical direction.

The length of the mantissa_focal_length_y[ i ] syntax element is determined as follows:

–	If exponent_focal_length_y[ i ] = = 0, the length is Max( 0, prec_focal_length − 30 ).

–	Otherwise (0 < exponent_focal_length_y[ i ] < 63), the length is Max( 0, exponent_focal_length_y[ i ] + prec_focal_length − 31 ).

sign_principal_point_x[ i ] equal to 0 indicates that the sign of the principal point of the i-th camera in the horizontal direction is positive. sign_principal_point_x[ i ] equal to 1 indicates that the sign of the principal point of the i-th camera in the horizontal direction is negative.

exponent_principal_point_x[ i ] specifies the exponent part of the principal point of the i-th camera in the horizontal direction. The value of exponent_principal_point_x[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. When exponent_principal_point_x[ i ] is equal to 63, the value of principal point in the horizontal direction for the i-th camera is unspecified.

mantissa_principal_point_x[ i ] specifies the mantissa part of the principal point of the i-th camera in the horizontal direction The length of the mantissa_principal_point_x[ i ] syntax element in units of bits is determined as follows:

–	If exponent_principal_point_x[ i ] = = 0, the length is Max( 0, prec_principal_point − 30 ).

–	Otherwise (0 < exponent_principal_point_x[ i ] < 63), the length is Max( 0, exponent_principal_point_x[ i ] + prec_principal_point − 31 ).

sign_principal_point_y[ i ] equal to 0 indicates that the sign of the principal point of the i-th camera in the vertical direction is positive. sign_principal_point_y[ i ] equal to 1 indicates that the sign of the principal point of the i-th camera in the vertical direction is negative.

exponent_principal_point_y[ i ] specifies the exponent part of the principal point of the i-th camera in the vertical direction. The value of exponent_principal_point_y[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. When exponent_principal_point_y[ i ] is equal to 63, the value of principal point in the vertical direction for the i-th camera is unspecified.

mantissa_principal_point_y[ i ] specifies the mantissa part of the principal point of the i-th camera in the vertical direction. The length of the mantissa_principal_point_y[ i ] syntax element in units of bits is determined as follows:

–	If exponent_principal_point_y[ i ] = = 0, the length is Max( 0, prec_principal_point − 30 ).

–	Otherwise (0 < exponent_principal_point_y[ i ] < 63), the length is Max( 0, exponent_principal_point_y[ i ] + prec_principal_point − 31 ).

sign_r[ i ][ j ][ k ] equal to 0 indicates that the sign of (j, k) component of the rotation matrix for the i-th camera is positive. sign_r[ i ][ j ][ k ] equal to 1 indicates that the sign of (j, k) component of the rotation matrix for the i-th camera is negative.

exponent_r[ i ][ j ][ k ] specifies the exponent part of (j, k) component of the rotation matrix for the i-th camera. The value of exponent_r[ i ][ j ][ k ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. When exponent_r[ i ][ j ][ k ] is equal to 63, the value of rotation matrix is unspecified.

mantissa_r[ i ][ j ][ k ] specifies the mantissa part of (j, k) component of the rotation matrix for the i-th camera.

The length of the mantissa_r[ i ][ j ][ k ] syntax element in units of bits is determined as follows:

–	If exponent_r[ i ] = = 0, the length is Max( 0, prec_rotation_param − 30 ).

–	Otherwise (0 < exponent_r[ i ] < 63), the length is Max( 0, exponent_r[ i ] + prec_rotation_param − 31 ).

The rotation matrix R[ i ] for i-th camera is represented as follows:

r[i][0][0]r[i][0][1]r[i][0][2]r[i][1][0]r[i][1][1]r[i][1][2]r[i][2][0]r[i][2][1]r[i][2][2]		(I-XX)

sign_t_x[ i ] equal to 0 indicates that the sign of horizontal component of the translation vector for the i-th camera is positive. sign_t_x[ i ] equal to 1 indicates that the sign of horizontal component of the translation vector for the i-th camera is negative.

exponent_t_x[ i ] specifies the exponent part of horizontal component of the translation vector for the i-th camera. The value of exponent_t_x[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. When exponent_t_x[ i ] is equal to 63, the value of the translation vector is unspecified.

mantissa_t_x[ i ] specifies the mantissa part of horizontal component of the translation vector for the i-th camera.

The length v of the mantissa_t_x[ i ] syntax element in units of bits is determined as follows:

–	If exponent_t_x[ i ] = = 0, the length v = Max( 0, prec_translation_param − 30 ).

–	Otherwise (0 < exponent_t_x[ i ] < 63), the length v = Max( 0, exponent_t_x[ i ] + prec_translation_param − 31 ).

The variable x, where x refers to a focal length, principal point, rotation, or translation is computed as follows, where e = exponent, s = sign, n = mantissa and v = mantissa_len_minus1 + 1:

–	If 0 < e < 63, x = (−1)s * 2e−31 * (1 + n  2v).

–	Otherwise (e is equal to 0), x = (−1)s * 2−(30+v) * n.

NOTE – The above specification is similar to what found in IEC 60559:1989, Binary floating-point arithmetic for microprocessor systems.

Add a new subclause I.14.3.1.2 “Reconstruction of the displayable texture views (Informative)” as follows:

      - Reconstruction of the displayable texture views (Informative)

The base view is denoted as the 0-th view, corresponding to the 0-th camera, and the view B is denoted as the i-th view corresponding to the i-th camera. i is in the range of 1 through 4, inclusive. An informative process to reconstruct the displayable texture views from the constituent residual pictures in the non-base view as well as the pictures in the base view is summarized in the following paragraph for each view B of the packed views in the non-base view, where view B is one of the i-th views, packed as a constituent residual picture in the non-base view.

–	When residual_depth_flag is equal to 1, the constituent residual depth picture associated with a view B is extracted out of the non-base depth view. The constituent residual depth picture is 1:2 upsampled horizontally and vertically to a residual depth view B. By using the view synthesis parameters, the base depth view is projected to the coordinates of the residual depth view B. Samples of residual depth view B which have smaller depth values are overwritten by the larger values of the projected depth samples. Remaining holes are inpainted.

–	When residual_depth_flag is equal to 0, the base depth view is projected to the residual depth view B by using the view synthesis parameters and the depth values of the base depth view. In case of duplicated projections, the larger depth value is retained. The remaining holes are inpainted. When num_residual_texture_views_minus1 + 2 is an even number, the base depth view is projected to the coordinates of the residual depth view B from its center location between base view and view B (for i equal to 1).

–	The resulted depth views and the base depth view are upsampled to the same resolution of the base-texture view.

–	The constituent residual texture picture associated with view B is extracted from the non-base texture view and is 1:2 upsampled horizontally and vertically to a residual picture view B.

–	Using the resulted depth view B and the view synthesis parameters, the base texture view is projected to the coordinates of the residual picture B. The remaining holes are inpainted to generate the displayable view.

# Recommendation

It is recommended to replace the depth_type=0 part of the description for alternative depth info SEI message in 3D-HEVC DT5 with this document.

# Patent rights declarations

NICT (National Institute of Information and Communications Technology) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NISRI (Nagoya Industrial Science Research Institute) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).