| - Joint Collaborative Team on 3D Video Coding Extensions - of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 - 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0064r1 |
| --- | --- |

| Title: | CShift in HTM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Takanori Senoh, Koki Wakunami, Yasuyuki Ichihashi, Hisayuki Sasaki, Ryutaro Oi, Kenji Yamamoto | Tel: Email: | {senoh, k.wakunami, y-ichihashi, sasaki, oi, k.yamamoto}@nict.go.jp |
| Source: | NICT |  |  |

_____________________________

# Abstract

This contribution requests to solve the mismatch of CShift (principal point X) between HTM vs. DERS (Depth Estimation Reference Software) and VSRS (View Synthesis Reference Software), including VSRS-1D-Fast for the future FTV standard.

- 1 Introduction

FTV AHG was established at the 105th MPEG meeting in July, 2013. Since then, active EEs (Exploration Experiments ) have been continued. At the 109th MPEG meeting, it was agreed to issue a CfP (Call for Proposal) at February 2016. For this purpose, 3D-HEVC reference software HTM, DERS6 (Depth Setimation Reference Software) and VSRS4 (View Synthesis Reference Software), including VSRS-1D-Fast are anticipated as the fundamental technologies for the future FTV standard, which encode and decode tens and hundreds of texture and depth views. For such purpose, the coding configuration parameter, CShift (which is the principal point X in camera parameter) is indispensable for each camera to shift the principal point X in order to keep the target in its image frame. However, it was observed CShift direction according to Position (translation X in camera parameter) is opposite between HTM and DERS, VSRS or VSRS-1D-Fast.

- 2 Experimental results

HTM11.2 safely runs with the CShift values as shown in Fig.1(a) but crashes with the ones as shown in Fig.1(b). VSRS-1D-Fast synthesizes a double images with Fig.1(a) cfg file. The difference between two camera parameters is CShift direction. HTM requires increasing CShift when Position decreases.

#Bee

- cam_param_bee_for HTM_temp (HTM11.2 safely runs with this CShift).

(b) cam_param_bee_for_HTM (HTM11.2 crashes with this CShift).

Fig.1 Camera parameter bee for HTM

On the contrary, VSRS6 yields double images with the principal point x values shown in Fig.2(a), which are identical to the CShift values in Fig.1(a).  VSRS6 synthesizes correct views with the principal point x values shown in Fig.2(b) which are identical to the CShift values in Fig.1(b). VSRS-1D-Fast also synthesizes correct views with the cfg file in Fig.1(b).

(a) cam_param_bee_for_VSRS_NG 	(b) cam_param_bee_for _VSRS

(VSRS6 yiels double images with this principal point x.) 	(VSRS6 generates correct views with this principal point x.)

Fig.2 Camera parameter bee for VSRS

As shown in Fig.3, when a camera translates to left from cam ID=101 to 102, its image sensor is shifted to right in order to keep the target in the image frame. Consequently, the principal point, which is the center of camera lens, looks moved to left at the principal point x= 950.113801 from the previous point of x=960 since the principal point x is measured from the origin (top left corner) of the image sensor. Generally, the origin of image sensor is the top left corner of images.

Consequently, cam_param_bee_for HTM in Fig.1(b) or cam_param_bee_for VSRS in Fig.2(b) seems natural. DERS is also using the same camera parameter as in Fig2(b).

Fig.3 camera layout

- 3 Recommendation

It is recommended HTM software is improved to be inter-operable with DERS and VSRS including VSRS-1D-Fast in terms of the camera parameter, specifically CShift (principal point x) since it is expected these software are fundamental technologies for the future FTV standard.

# Patent rights declarations

NICT (National Institute of Information and Communications Technology) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).