| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Strasbourg, FR, 18–24 Oct. 2014 | Document: JCT3V-J0066 |
| --- | --- |

| Title: | Simplification and improvement of sub-PU |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng， Yongbing Lin, Xu Chen, Jianhua Zheng | Tel: Email: | +86-755-28425082 zhengxiaozhen@huawei.com |
| Source: | Hisilicon Technologies |  |  |

_____________________________

# Abstract

This contribution proposes two simplification methods for sub-PU technique. Both of them can remove the irregular sub-PU partitions. Compared to htm12 anchor, method 1 doesn’t have the impact on coding performance, method 2 can achieve about 0.1% gain on video.

# Introduction

In current 3D-HEVC, sub-PU prediction is used for IVMC and MPI. When AMP is used, the irregular motion compensation prediction (MCP) block sizes might be used which are not existed in HEVC version1. For an example, if the CU size is 16x16 and SubPbSize is set at 8 in VPS extension2, 8x12 and 12x8 MCP block sizes are allowed. This phenomenon is illustrated in Figure 1. In the 8th and 9th JCT-3V meetings, several proposals, e.g. G0077, H0066, H0074 and H0133, suggest restricting sub-PU prediction on 2Nx2N PU that can avoid using the irregular MCP block sizes. Considering the lack of the flexibility for the use of sub-PU, the suggestions in those proposals haven’t been adopted. This contribution proposes two alternative solutions that can also avoid the use of irregular sub-PU and still reserve the flexibility for sub-PU prediction.

Figure 1: Illustration of current sub-PU partitioning

# Proposed Methods

## Method 1: Restrict the use of sub-PU in particular AMP block sizes

The irregular sub-PU block sizes are only generated at particular AMP block sizes. If one of the AMP block’s edge isn’t the multiple of SubPbSize, the irregular sub-PU block size is used. To avoid those block sizes, method 1 suggests checking the size of a prediction block’s two edge when the sub-PU block size is calculated. If one of the edge is not the multiple of SubPbSize or less than SubPbSize, the sub-PU block sizes are set equal to the prediction block sizes. The checking condition is shown as below (the words marked by the yellow colour are the new added parts compared to current specification).

The variables minSize, nSbW and nSbH are derived as:

  - minSize = DepthFlag ? MpiSubPbSize[ nuh_layer_id ] : SubPbSize[ nuh_layer_id ]	(Error: Reference source not found-1)

  - nSbW = ( nPbW % SubPbSize[ nuh_layer_id ]  !=  0 || nPbH /%SubPbSize[ nuh_layer_id ]  !=  0 ) ? nPbW : minSize		(Error: Reference source not found-2)

  - nSbH = (nPbW % SubPbSize[ nuh_layer_id ]  !=  0 || nPbH % SubPbSize[ nuh_layer_id ]  !=  0 ) ? nPbH : minSize		(Error: Reference source not found-3)

The partitioning process is shown at Figure 2.

Figure 2: Illustration of sub-PU partitioning proposed by method 1

## Method 2: Use non-square sub-PU for non-square prediction block

Method 2 suggests using non-square sub-PU for the non-square prediction block. If the prediction block uses horizontal partitioning modes, e.g. PART_2NxN, PART_2NxnL, PART_2NxnR, the sub-PU size is set to SubPbSize<<1 x SubPbSize>>1; otherwise if the prediction block uses vertical partitioning modes, e.g. PART_Nx2N, PART_nLx2N, PART_nRx2N, the sub-PU size is set to SubPbSize>>1 x SubPbSize<<1; for other partitioning modes, e.g. PART_2Nx2N, PART_NxN, the sub-PU size is still set to SubPbSize x SubPbSize. This modification can guarantee that the shape of sub-PU is matched with the shape of prediction block, and the irregular sub-PU block sizes can be removed. For an example, 16x4 and 16x12 prediction blocks are split to one 16x4 sub-PU and three 16x4 sub-PU respectively based on the solution provided by method 2. Figure 3 illustrates the sub-PU partitioning proposed by method 2.

Figure 3: Illustration of sub-PU partitioning for method 2

# Experimental Results

The proposed two methods are implemented on HTM12.0. Table 1 shows the experimental results of method 1. The results show that the proposed method 1 doesn’t have the coding performance degradation compared to HTM12.0 anchor. The experimental results of method 2 are shown at table 2. The proposed method 2 can get about 0.1% coding gain on video.

Table 1: Results for method 1

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.1% | 0.0% | 0.01% | 0.03% | 0.06% | 99.3% | 96.6% | 98.6% |
| Kendo | 0.0% | -0.1% | 0.0% | 0.00% | 0.00% | -0.02% | 104.3% | 123.2% | 102.1% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | -0.01% | -0.04% | -0.09% | 103.9% | 93.2% | 99.4% |
| GT_Fly | 0.0% | -0.1% | -0.1% | 0.00% | -0.01% | 0.00% | 100.0% | 93.7% | 102.8% |
| Poznan_Hall2 | 0.0% | 0.2% | 0.1% | 0.04% | 0.07% | 0.08% | 98.6% | 96.0% | 96.0% |
| Poznan_Street | 0.0% | 0.0% | 0.1% | 0.00% | 0.00% | 0.00% | 98.7% | 108.5% | 104.5% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | -0.05% | 100.9% | 112.2% | 99.6% |
| Shark | 0.0% | 0.0% | 0.0% | 0.01% | 0.01% | 0.00% | 100.2% | 106.5% | 99.2% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | -0.02% | 102.5% | 104.3% | 100.0% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.01% | 0.01% | 0.00% | 99.7% | 103.4% | 100.4% |
| average | 0.0% | 0.0% | 0.0% | 0.01% | 0.01% | 0.00% | 100.7% | 103.7% | 100.3% |

Table 2: Results for method 2

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | -0.1% | -0.08% | -0.10% | 0.01% | 99.9% | 95.2% | 95.9% |
| Kendo | 0.0% | -0.2% | 0.0% | -0.07% | -0.08% | 0.03% | 99.1% | 99.5% | 100.9% |
| Newspaper_CC | 0.0% | -0.1% | -0.1% | -0.05% | -0.06% | -0.07% | 100.1% | 102.9% | 100.2% |
| GT_Fly | 0.0% | -0.2% | 0.0% | -0.07% | -0.08% | -0.04% | 99.5% | 82.2% | 102.0% |
| Poznan_Hall2 | 0.0% | -0.1% | 0.2% | -0.02% | -0.03% | 0.07% | 100.2% | 100.8% | 99.2% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | -0.02% | -0.03% | -0.02% | 98.9% | 122.0% | 102.0% |
| Undo_Dancer | 0.0% | -0.2% | -0.4% | -0.12% | -0.12% | -0.10% | 100.4% | 107.2% | 96.4% |
| Shark | 0.0% | 0.1% | -0.1% | -0.02% | -0.02% | 0.00% | 100.0% | 96.3% | 99.8% |
| 1024x768 | 0.0% | -0.2% | 0.0% | -0.07% | -0.08% | -0.01% | 99.7% | 99.2% | 99.0% |
| 1920x1088 | 0.0% | -0.1% | -0.1% | -0.05% | -0.06% | -0.02% | 99.8% | 101.7% | 99.9% |
| average | 0.0% | -0.1% | -0.1% | -0.06% | -0.07% | -0.01% | 99.8% | 100.8% | 99.5% |

# Conclusion

This contribution provides two methods that can be used to avoid the use of irregular sub-PU block sizes. Since the proposed methods only forbid applying sub-PU concept to the prediction blocks that can be split into irregular sub-PU block sizes, these two methods still reserve the flexibility of the use of sub-PU. Compare to HTM12.0 anchor, method 1 has the same coding performance, and method 2 can get about 0.1% gain.

# Changes to the description of WD

Method 1

I.8.5.3.2.17

The variables minSize, nSbW and nSbH are derived as:

  - minSize = DepthFlag ? MpiSubPbSize[ nuh_layer_id ] : SubPbSize[ nuh_layer_id ]	(Error: Reference source not found-4)

  - nSbW = ( nPbW % SubPbSize[ nuh_layer_id ]  !=  0 || nPbH /%SubPbSize[ nuh_layer_id ]  !=  0 ) ? nPbW : minSize		(Error: Reference source not found-5)

  - nSbH = (nPbW % SubPbSize[ nuh_layer_id ]  !=  0 || nPbH % SubPbSize[ nuh_layer_id ]  !=  0 ) ? nPbH : minSize		(Error: Reference source not found-6)

Method 2

I.8.5.3.2.17

The variables minSize, nSbW and nSbH are derived as:

if (part_mode == SIZE_2NxN || part_mode == SIZE_2NxnU || part_mode == SIZE_2NxnD )

{

minSizeW = DepthFlag ? ( MpiSubPbSize[ nuh_layer_id ] << 1 ) : ( SubPbSize[ nuh_layer_id ] << 1)

minSizeH = DepthFlag ? ( MpiSubPbSize[ nuh_layer_id ] >> 1 ) : ( SubPbSize[ nuh_layer_id ] >> 1)

}

else if (part_mode == SIZE_Nx2N || part_mode == SIZE_nLx2N || part_mode == SIZE_nRx2N)

{

minSizeW = DepthFlag ? ( MpiSubPbSize[ nuh_layer_id ] >> 1 ) : ( SubPbSize[ nuh_layer_id ] >> 1)

minSizeH = DepthFlag ? ( MpiSubPbSize[ nuh_layer_id ] << 1 ) : ( SubPbSize[ nuh_layer_id ] << 1)

}

else

{

minSizeW = DepthFlag ? MpiSubPbSize[ nuh_layer_id ] : SubPbSize[ nuh_layer_id ]

minSizeH = DepthFlag ? MpiSubPbSize[ nuh_layer_id ] : SubPbSize[ nuh_layer_id ]

}

  - nSbW = ( nPbW / minSizeW  <=  1 ) ? nPbW : minSizeW	(Error: Reference source not found-7)

  - nSbH = ( nPbH / minSizeH  <=  1 ) ? nPbH : minSizeH	(Error: Reference source not found-8)

# Patent rights declaration(s)

Hisilicon Technologies and Huawei Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).